Instruction sheet for candidate

| Qualification | National Diploma Level 05 Artificial Intelligence Supervisor in Artificial Intelligence Technology |
| --- | --- |
| Competency Standard | - Develop Object Detection system - Develop Chat bot - Develop Self Driving Vehicle |
| Purpose of Assessment | SummativeAssessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. Time frame (for practical demonstration & assessment): - Develop Robot by following task - Develop the Input Unit - Object Classification Object Localization & Detection - Voice Controlled using NLTK - Setup RC Car Control Unit |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Develop the Input Unit - Connect raspberry pi with camera module and HC-SR04 ultrasonic sensor - Develop the two-client program on raspberry pi - Run the program to stream color video - Run the program to get ultrasonic sensor data to the computer - Set up the local Wi-Fi with computer - Set the video to Low latency video streaming Object Classification Object Localization& Detection - Take a pre-trained imagenet from the following URL http://download.tensorflow.org/models/object_detection/ - Re-train the last fully connected layer with the objects that is the image contains a car vs. the image doesn’t contain anobject. - Get all proposals(=~2000 p/image), resize them to match the CNN input, then save to disk. - Train SVM to classify between object and background (One binary SVM for each class) - Train a linear regression classifier that will output some correction factor - After detecting that objectin image localize its position - Mark the object with a rectangle or bounding box - Show score represent how level of confidence for each of the object or not. - Score is shown on the result image, together with the class label. - Visualization of the results of a detection. &localization. Voice Controlled using NLTK - Install Pyaudio - Create instance of Microphone class - Using listen () to Capture Microphone Input - Set the error for Unrecognizable Speech - Calibrate the speech recognition engine. - Create lists with your preset question’s answers - Use a loop code with random and Wikipedia - Set the pre trained data set and implement it. - Take as input the tokens and return normalized tokens - if a user’s input is a greeting, the bot shall return a voice greeting response Setup RC Car Control Unit - Setup multithread TCP Server on computer to get data from raspberry pi and ultrasonic sensor - Connect Arduino to the computer via using USB - Set computer outputs commands to Arduino using serial interface. - Install miniconda (Python3) on your computer - Create auto-RCcar environment with all necessary libraries for this project - Activate auto-RC car environment - RC car must successfully navigate on the track with the ability to avoid front collision, and respond to stop sign and traffic light accordingly. - Develop a Technical project report on MS Word according to the pattern given in the Annexure-I - Develop a Project Presentation on MS PowerPoint according to the pattern given in the Annexure-II |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Diploma Level 05 Artificial Supervisor in Artificial Intelligence Technology |
| Competency Standard | - Develop Object Detection system - Develop Chat bot - Develop Self Driving Vehicle |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | - Develop the Input Unit - Object Classification Object Localization & Detection - Voice Controlled using NLTK - Setup RC Car Control Unit - Setup RC Car Control Unit |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Connect raspberry pi with camera module and HC-SR04 ultrasonic sensor |  |  |
| - Develop the two-client program on raspberry pi |  |  |
| - Run the program to stream color video |  |  |
| - Run the program to get ultrasonic sensor data to the computer |  |  |
| - Set up the local Wi-Fi with computer |  |  |
| - Set the video to Low latency video streaming |  |  |
| - Take a pre-trained imagenet from the following URL |  |  |
| - http://download.tensorflow.org/models/object_detection/ |  |  |
| - Re-train the last fully connected layer with the objects that is the image contains a car vs. the image doesn’t contain an object. |  |  |
| - Get all proposals(=~2000 p/image), resize them to match the CNN input, then save to disk. |  |  |
| - Train SVM to classify between object and background (One binary SVM for each class) |  |  |
| - Train a linear regression classifier that will output some correction factor |  |  |
| - After detecting that object in image localize its position |  |  |
| - Mark the object with a rectangle or bounding box |  |  |
| - Show score represent how level of confidence for each of the object or not. |  |  |
| - Score is shown on the result image, together with the class label. |  |  |
| - Visualization of the results of a detection. &localization. |  |  |
| - Install Pyaudio |  |  |
| - Create instance of Microphone class |  |  |
| - Using listen () to Capture Microphone Input |  |  |
| - Set the error for Unrecognizable Speech |  |  |
| - Calibrate the speech recognition engine. |  |  |
| - Create lists with your preset question’s answers |  |  |
| - Use a loop code with random and Wikipedia |  |  |
| - Set the pre trained data set and implement it. |  |  |
| - Take as input the tokens and return normalized tokens |  |  |
| - if a user’s input is a greeting, the bot shall return a voice greeting response |  |  |
| - Setup multithread TCP Server on computer to get data from raspberry pi and ultrasonic sensor |  |  |
| - Connect Arduino to the computer via using USB |  |  |
| - Set computer outputs commands to Arduino using serial interface. |  |  |
| - Install miniconda (Python3) on your computer |  |  |
| - Create auto-RCcar environment with all necessary libraries for this project |  |  |
| - Activate auto-RC car environment |  |  |
| - RC car must successfully navigate on the track with the ability to avoid front collision, and respond to stop sign and traffic light accordingly. |  |  |
| - Develop a Technical project report on MS Word according to the pattern given in the Annexure-I |  |  |
| - Develop a Project Presentation on MS PowerPoint according to the pattern given in the Annexure-II |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Diploma Level 05 Artificial Supervisor in Artificial Intelligence Technology |
| --- | --- |
| Competency Standard | - Develop Object Detection system - Develop Chat bot - Develop Self Driving Vehicle |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Develop the Input Unit - Object Classification Object Localization & Detection - Voice Controlled using NLTK - Setup RC Car Control Unit |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Connect raspberry pi with camera module and HC-SR04 ultrasonic sensor |  |  |  |  |  |
|  | Develop the two-client program on raspberry pi |  |  |  |  |  |
|  | Run the program to stream color video |  |  |  |  |  |
|  | Run the program to get ultrasonic sensor data to the computer |  |  |  |  |  |
|  | Set up the local Wi-Fi with computer |  |  |  |  |  |
|  | Set the video to Low latency video streaming |  |  |  |  |  |
|  | Take a pre-trained imagenet from the following URL http://download.tensorflow.org/models/object_detection/ |  |  |  |  |  |
|  | Re-train the last fully connected layer with the objects that is the image contains a car vs. the image doesn’t contain an object. |  |  |  |  |  |
|  | Get all proposals(=~2000 p/image), resize them to match the CNN input, then save to disk. |  |  |  |  |  |
|  | Train SVM to classify between object and background (One binary SVM for each class) |  |  |  |  |  |
|  | Train a linear regression classifier that will output some correction factor |  |  |  |  |  |
|  | After detecting that object in image localize its position |  |  |  |  |  |
|  | Mark the object with a rectangle or bounding box |  |  |  |  |  |
|  | Show score represent how level of confidence for each of the object or not. |  |  |  |  |  |
|  | Score is shown on the result image, together with the class label. |  |  |  |  |  |
|  | Visualization of the results of a detection. &localization. |  |  |  |  |  |
|  | Install Pyaudio |  |  |  |  |  |
|  | Create instance of Microphone class |  |  |  |  |  |
|  | Using listen () to Capture Microphone Input |  |  |  |  |  |
|  | Set the error for Unrecognizable Speech |  |  |  |  |  |
|  | Calibrate the speech recognition engine. |  |  |  |  |  |
|  | Create lists with your preset question’s answers |  |  |  |  |  |
|  | Use a loop code with random and Wikipedia |  |  |  |  |  |
|  | Set the pre trained data set and implement it. |  |  |  |  |  |
|  | Take as input the tokens and return normalized tokens |  |  |  |  |  |
|  | if a user’s input is a greeting, the bot shall return a voice greeting response |  |  |  |  |  |
|  | Setup multithread TCP Server on computer to get data from raspberry pi and ultrasonic sensor |  |  |  |  |  |
|  | Connect Arduino to the computer via using USB |  |  |  |  |  |
|  | Set computer outputs commands to Arduino using serial interface. |  |  |  |  |  |
|  | Install miniconda (Python3) on your computer |  |  |  |  |  |
|  | Create auto-RCcar environment with all necessary libraries for this project |  |  |  |  |  |
|  | Activate auto-RC car environment |  |  |  |  |  |
|  | RC car must successfully navigate on the track with the ability to avoid front collision, and respond to stop sign and traffic light accordingly. |  |  |  |  |  |
|  | Develop a Technical project report on MS Word according to the pattern given in the Annexure-I |  |  |  |  |  |
|  | Develop a Project Presentation on MS PowerPoint according to the pattern given in the Annexure-II |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Diploma Level 05 Artificial Supervisor in Artificial Intelligence Technology |
| --- | --- |
| Competency Standard | - Develop Object Detection system - Develop Chat bot - Develop Self Driving Vehicle |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is chatbot technology? |  |  |
|  | Why do we need chatbot? |  |  |
|  | What is object detection in image processing? |  |  |
|  | How do you train models for object detection? |  |  |
|  | What is object detection API? |  |  |
|  | What is real time object detection? |  |  |
|  | How does a driverless car work? Sketch a wok flow diagram. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________

Annexure-A

Project Report Format

Table of Content

Introduction

  - Why your topic is important (convince us!)

  - Where is it used? Applications

  - What you will talk about/do

  - Overview of the rest of your project (section 2 covers...section 3 presents...)

Background

  - Any relevant and specific info

e.g. hardware statistics, equipment used

  - What other people had to say on this topic(s)

(be sure to cite your references, and quote as appropriate)

  - You are expected to discuss the books and papers that you include in your references. You must also cite them. If nothing else, include a brief rationale explaining why you thought it was useful.

  - What other people did on this topic (or related topics)

  - Problems and shortcomings of their work

  - How your work is different and better

Project

  - Your approach to the problem

  - What you did

  - Design

- what you already had (and where it came from)
- what you added/changed
- for parts, include close-up drawings (e.g. screenshots)

  - What did/didn't work?

  - Include graphs, equations, pictures, etc. as appropriate

  - Results

Include relevant observations, measurements, and statistics. For example, for the VLSI Class: Include statistics such as timing information if available by simulation, or if not, your own analysis about critical path, delays, and clock cycles. Be sure to include size information: the total size of the circuit measured (X lambda by Y lambda), and the transistor count.

Summary/Result

  - Try to draw together the intro, background, and project sections.

  - How do they all relate together? (They may appear to be disjoint sections to an unfamiliar reader).

  - Restate important results

Budget

Working timeline

Conclusions and Future work

  - What was accomplished / learned

  - What you would have done differently

  - Future work

References

  - You should include a number of books and papers that were useful. If no number if specified, then include at least 5 books or papers or website. (If this is a group project, include at least 5 per person.) Wikipedia is not appropriate, and you will be penalized if you include it.

  - Cite the papers/books that you used

  - Anything you found useful

  - Include textbooks from class if you want

## Things to include in the report:

- Pictures

- Your observations and measurements

- Equations

- Graphs

- Figures

- Simulation, model

Annexure -II

Presentation Format

- Introduction the project

- Objective

- Problem Statement

- Project Methodology

- Budget

- Working timeline

- Result

- Conclusion

## Things to include in the report:

- Pictures

- Your observations and measurements

- Equations

- Graphs

- Figures

- Simulation, model

- Working demo