Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 2 in Automation & Process Control |
| --- | --- |
| Competency Standard | Implement Electromagnetism to See Various Effects & Verify Faradays Law (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Demonstrate Electromagnetism - Implement circuit to determine the effect on current carrying conductor in magnetic field. - Determine the effect on conductor by varying the current with the help of rheostat. Single phase AC Power - Plot magnetic lines of forces of bar magnet. - Verify Faradays law by moving magnet in side coil. - Verify Faradays law by moving coil near the magnet field. - Verify EMF through induction. |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Demonstrate Electromagnetism - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. - Connect dry cell battery with coil wound on the iron nail. - Bring iron nail near the iron pieces and demonstrate the observation. Implement circuit to determine the effect on current carrying conductor in magnetic field. - Take copper rod (5 cm in length) and connect wires across it. - Give DC supply to copper rod through rheostat. - Place current carrying copper rod inside the horseshoe magnet. - Observe the rod movement - Change the polarity of supply and observe the rod again Determine the effect on conductor by varying the current with the help of rheostat.single phase AC Power - Reduce the rheostat resistance - Record the effect on copper rod. - Increase the rheostat resistance - Record the effect on copper rod. Plot magnetic lines of forces of bar magnet. - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet. - Mark points on paper where the compass needle stop. - Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4. Verify Faradays law by moving magnet inside coil. - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. Verify Faradays law by moving coil near the magnet field. - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it - Connect Galvanometer with coil - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvanometer - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvanometer Verify EMF through induction. - Take step down transformer and connect its secondary with a Galvanometer and primary winding with a DC battery through a rheostat - Continuously variate the rheostat and observe the reading on the Galvanometer |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 2 in Automation & Process Control |
| Competency Standard | Implement Electromagnetism to See Various Effects & Verify Faradays Law (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Demonstrate Electromagnetism - Implement circuit to determine the effect on current carrying conductor in magnetic field. - Determine the effect on conductor by varying the current with the help of rheostat. Single phase AC Power - Plot magnetic lines of forces of bar magnet. - Verify Faradays law by moving magnet in side coil. - Verify Faradays law by moving coil near the magnet field. - Verify EMF through induction. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. |  |  |
| - Connect dry cell battery with coil wound on the iron nail. |  |  |
| - Bring iron nail near the iron pieces and demonstrate the observation. |  |  |
| - Take copper rod (5 cm in length) and connect wires across it. |  |  |
| - Give DC supply to copper rod through rheostat. |  |  |
| - Place current carrying copper rod inside the horseshoe magnet. |  |  |
| - Observe the rod movement |  |  |
| - Change the polarity of supply and observe the rod again |  |  |
| - Reduce the rheostat resistance |  |  |
| - Record the effect on copper rod. |  |  |
| - Increase the rheostat resistance |  |  |
| - Record the effect on copper rod. |  |  |
| - Place a bar magnet on paper and outline its boundary with the help of lead pencil. |  |  |
| - Place a compass needle at one side the magnet. |  |  |
| - Mark points on paper where the compass needle stop. |  |  |
| - Repeat the same procedure till compass reach at the other end of magnet. |  |  |
| - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4. |  |  |
| - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. |  |  |
| - Connect Galvanometer with coil. |  |  |
| - Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. |  |  |
| - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. |  |  |
| - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it |  |  |
| - Connect Galvanometer with coil |  |  |
| - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvanometer |  |  |
| - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvanometer |  |  |
| - Take step down transformer and connect its secondary with a Galvanometer and primary winding with a DC battery through a rheostat |  |  |
| - Continuously variate the rheostat and observe the reading on the Galvanometer |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 2 in Automation & Process Control |
| --- | --- |
| Competency Standard | Implement Electromagnetism to See Various Effects & Verify Faradays Law (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Demonstrate Electromagnetism - Implement circuit to determine the effect on current carrying conductor in magnetic field. - Determine the effect on conductor by varying the current with the help of rheostat. Single phase AC Power - Plot magnetic lines of forces of bar magnet. - Verify Faradays law by moving magnet in side coil. - Verify Faradays law by moving coil near the magnet field. - Verify EMF through induction. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Took iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. |  |  |  |  |  |
|  | Connected dry cell battery with coil wound on the iron nail. |  |  |  |  |  |
|  | Bring iron nail near the iron pieces and demonstrated the observation. |  |  |  |  |  |
|  | Took copper rod (5 cm in length) and connect wires across it. |  |  |  |  |  |
|  | Gave DC supply to copper rod through rheostat. |  |  |  |  |  |
|  | Placed current carrying copper rod inside the horse shoe magnet. |  |  |  |  |  |
|  | Observe the rod movement |  |  |  |  |  |
|  | Change the polarity of supply and observe the rod again |  |  |  |  |  |
|  | Reduced the rheostat resistance |  |  |  |  |  |
|  | Recorded the effect on copper rod. |  |  |  |  |  |
|  | Increased the rheostat resistance |  |  |  |  |  |
|  | Recorded the effect on copper rod. |  |  |  |  |  |
|  | Placed a bar magnet on paper and outline its boundary with the help of lead pencil. |  |  |  |  |  |
|  | Placed a compass needle at one side the magnet. |  |  |  |  |  |
|  | Marked points on paper where the compass needle stop. |  |  |  |  |  |
|  | Repeated the same procedure till compass reach at the other end of magnet. |  |  |  |  |  |
|  | Changed the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4. |  |  |  |  |  |
|  | Constructed a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. |  |  |  |  |  |
|  | Connected Galvanometer with coil. |  |  |  |  |  |
|  | Moved permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. |  |  |  |  |  |
|  | Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. |  |  |  |  |  |
|  | Constructed a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it |  |  |  |  |  |
|  | Connected Galvanometer with coil |  |  |  |  |  |
|  | Fixed permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvanometer |  |  |  |  |  |
|  | Hold the coil near the magnetic field do not move, now record the effect on reading of Galvanometer |  |  |  |  |  |
|  | Took step down transformer and connect its secondary with a Galvanometer and primary winding with a DC battery through a rheostat |  |  |  |  |  |
|  | Continuously variated the rheostat and observe the reading on the Galvanometer |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 2 in Automation & Process Control |
| --- | --- |
| Competency Standard | Implement Electromagnetism to See Various Effects & Verify Faradays Law (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe magnetic Material |  |  |
|  | Unit of magnetic flux density is_______ |  |  |
|  | - Ohm - Henry - Farad - Tesla |  |  |
|  | Iron is a ferromagnetic material |  |  |
|  | - TRUE - FALSE |  |  |
|  | state Fleming’s left-hand rule |  |  |
|  | Magnetic lines of forces travel from North to south through Air. |  |  |
|  | a) TRUE b) FALSE |  |  |
|  | Describe function of iron core |  |  |
|  | Rheostat is used to control current in circuit for various applications. |  |  |
|  | a) TRUE b) FALSE |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________