Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Construct & Verify Combinational logic circuit (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Apply Karnaugh mapping & Boolean algebra to simplify logic expressions - Construct & verify the truth table of Half adder - Construct & verify the truth table of Full adder - Construct & verify the truth table of Half Subtraction - Construct & verify the truth table of Full Subtraction - Verify Decoder - Operate seven segment display with seven segment decoder. - Verify Encoder - Verify multiplexer and DE- multiplexer |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Apply Karnaugh mapping & Boolean algebra to simplify logic expressions - Identify the SOP & POS - Apply Boolean algebra &Karnaugh mapping to simplify SOP & POS. - Construct logic circuits with simplified SOP & POS. Construct & verify the truth table of Half adder - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a half-adder circuit using one XOR gate and two NAND gates. Construct & verify the truth table of Full adder - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a full-adder circuit using two ICS, &7486 and &7400. Construct & verify the truth table of Half Subtraction - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input Construct & verify the truth table of Full Subtraction - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. Verify Decoder - Place (Decoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. Operate seven segment display with seven segment decoder. - Insert (7 segment decoder IC) and 7 segment display on bread board. - Identify the input, output, Vcc and ground pin. - Connect segment display with seven segment decoder input output pins. - Record & verify the output result against each given input. Verify Encoder - Place (Encoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. Verify multiplexer and DE- multiplexer - Implement following function with multiplexer F(ABC)=∑ (0,2,3,4,5,6): - Implement 4-to-1 mux and one 2-to-1 mux. - Implement 1-to-4 dmux using 1-to-2 dmux. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| Competency Standard | Construct & Verify Combinational logic circuit (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Apply Karnaugh mapping & Boolean algebra to simplify logic expressions - Construct & verify the truth table of Half adder - Construct & verify the truth table of Full adder - Construct & verify the truth table of Half Subtraction - Construct & verify the truth table of Full Subtraction - Verify Decoder - Operate seven segment display with seven segment decoder. - Verify Encoder - Verify multiplexer and DE- multiplexer |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify the SOP & POS |  |  |
| - Apply Boolean algebra &Karnaugh mapping to simplify SOP & POS. |  |  |
| - Construct logic circuits with simplified SOP & POS. |  |  |
| - Place (AND gate IC) & (XOR gate IC) on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect LED to the output pin of IC and apply different logics at input pins. |  |  |
| - Record & verify the output result against each given input |  |  |
| - Design, Construct, and test a half-adder circuit using one XOR gate and two NAND gates. |  |  |
| - Place (AND gate IC) & (XOR gate IC) on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect LED to the output pin of IC and apply different logics at input pins. |  |  |
| - Record & verify the output result against each given input |  |  |
| - Design, Construct, and test a full-adder circuit using two ICS, &7486 and &7400. |  |  |
| - Place (AND, NOT&XOR gate IC) on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect LED to the output pin of IC and apply different logics at input pins. |  |  |
| - Record & verify the output result against each given input |  |  |
| - Place (AND, NOT&XOR gate IC) on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect LED to the output pin of IC and apply different logics at input pins. |  |  |
| - Record & verify the output result against each given input. |  |  |
| - Place (Decoder IC) on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect LED to the output pin of IC and apply different logics at input pins. |  |  |
| - Record & verify the output result against each given input. |  |  |
| - Insert (7 segment decoder IC) and 7 segment display on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect segment display with seven segment decoder input output pins. |  |  |
| - Record & verify the output result against each given input. |  |  |
| - Place (Encoder IC) on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect LED to the output pin of IC and apply different logics at input pins. |  |  |
| - Record & verify the output result against each given input. |  |  |
| - Implement following function with multiplexer F(ABC)=∑ (0,2,3,4,5,6): |  |  |
| - Implement 4-to-1 mux and one 2-to-1 mux. |  |  |
| - Implement 1-to-4 dmux using 1-to-2 dmux. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Construct & Verify Combinational logic circuit (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Apply Karnaugh mapping & Boolean algebra to simplify logic expressions - Construct & verify the truth table of Half adder - Construct & verify the truth table of Full adder - Construct & verify the truth table of Half Subtraction - Construct & verify the truth table of Full Subtraction - Verify Decoder - Operate seven segment display with seven segment decoder. - Verify Encoder - Verify multiplexer and DE- multiplexer |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identified the SOP & POS |  |  |  |  |  |
|  | Applied Boolean algebra &Karnaugh mapping to simplify SOP & POS. |  |  |  |  |  |
|  | Constructed logic circuits with simplified SOP & POS. |  |  |  |  |  |
|  | Placed (AND gate IC) & (XOR gate IC) on bread board. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected LED to the output pin of IC and applied different logics at input pins. |  |  |  |  |  |
|  | Recorded& verified the output result against each given input |  |  |  |  |  |
|  | Constructed and tested a half-adder circuit using one XOR gate and two NAND gates. |  |  |  |  |  |
|  | Placed (AND gate IC) & (XOR gate IC) on bread board. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected LED to the output pin of IC and applied different logics at input pins. |  |  |  |  |  |
|  | Recorded& verified the output result against each given input |  |  |  |  |  |
|  | Constructed, and tested a full-adder circuit using two ICS, &7486 and &7400. |  |  |  |  |  |
|  | Placed (AND, NOT&XOR gate IC) on bread board. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected LED to the output pin of IC and applied different logics at input pins. |  |  |  |  |  |
|  | Recorded& verified the output result against each given input |  |  |  |  |  |
|  | Placed (AND, NOT&XOR gate IC) on bread board. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected LED to the output pin of IC and applied different logics at input pins. |  |  |  |  |  |
|  | Recorded& verified the output result against each given input. |  |  |  |  |  |
|  | Placed (Decoder IC) on bread board. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected LED to the output pin of IC and applied different logics at input pins. |  |  |  |  |  |
|  | Recorded& verified the output result against each given input. |  |  |  |  |  |
|  | Inserted (7 segment decoder IC) and 7 segment display on bread board. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected segment display with seven segment decoder input output pins. |  |  |  |  |  |
|  | Recorded& verified the output result against each given input. |  |  |  |  |  |
|  | Placed (Encoder IC) on bread board. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected LED to the output pin of IC and applied different logics at input pins. |  |  |  |  |  |
|  | Recorded& verified the output result against each given input. |  |  |  |  |  |
|  | Implemented following function with multiplexer F(ABC)=∑ (0,2,3,4,5,6): |  |  |  |  |  |
|  | Implemented 4-to-1 mux and one 2-to-1 mux. |  |  |  |  |  |
|  | Implemented 1-to-4 dmux using 1-to-2 dmux. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Construct & Verify Combinational logic circuit (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | A half adder is a ________ circuit. |  |  |
|  | - Combinational - Sequential - Both A and B - None |  |  |
|  | A full adder has |  |  |
|  | - 2 inputs, 2 outputs - 3 inputs, 2 outputs - 2 inputs, 1 output - 3 inputs, 1 output |  |  |
|  | What is the requirement of fullsubstractor circuit? |  |  |
|  | - Three inputs and two outputs - Three inputs and one output - Two inputs and two outputs - Two inputs and three outputs |  |  |
|  | How many input and output required for multiplexer? |  |  |
|  | - Many input and one output - One input and many output - One input and one output - None of the above |  |  |
|  | A demultiplexer is a |  |  |
|  | - 1-to-N device - N-to-1 device - 1-to-1 device - N-to-N device |  |  |
|  | Why decoder is used in the digital electronics? |  |  |
|  | - To convert coded information into non coded form - To convert non coded information into coded form - It is used to separate address bus and data bus - None of the above |  |  |
|  | Why encoder is used in the digital electronics? |  |  |
|  | - To convert non coded information into coded - To convert coded information into non coded form - It is used to separate address bus and data bus - None of the above |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________