Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Construct and Verify function of Flip Flops (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Construct and verify the truth table of RS latch - Construct and verify the truth table of clocked RS latch using NAND gat - Verify function of D flip flop. - Verify function of JK/T flip flop |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Construct and verify the truth table of RS latch - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. Construct and verify the truth table of clocked RS latch using NAND gate - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. Verify function of D flip flop. - Insert (D flip flop) IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. Verify function of JK/T flip flop - Insert 74112 (JK flip flop) IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| Competency Standard | Construct and Verify function of Flip Flops (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Construct and verify the truth table of RS latch - Construct and verify the truth table of clocked RS latch using NAND gat - Verify function of D flip flop. - Verify function of JK/T flip flop |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Place (NAND gate IC) on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect LEDs to outputs pins. |  |  |
| - Apply different logic inputs to Record & verify the output result against each given input. |  |  |
| - Place (NAND gate IC) on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect LEDs to outputs pins. |  |  |
| - Apply different logic inputs to Record & verify the output result against each given input. |  |  |
| - Insert (D flip flop) IC on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect LEDs\ Scope to outputs pins. |  |  |
| - Apply different logic inputs to Record & verify the output result against each given input. |  |  |
| - Insert 74112 (JK flip flop) IC on bread board. |  |  |
| - Identify the input, output, Vcc and ground pin. |  |  |
| - Connect LEDs\ Scope to outputs pins. |  |  |
| 16. Apply different logic inputs to Record & verify the output result against each given input. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Construct and Verify function of Flip Flops (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Construct and verify the truth table of RS latch - Construct and verify the truth table of clocked RS latch using NAND gat - Verify function of D flip flop. - Verify function of JK/T flip flop |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Placed (NAND gate IC) on bread board |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected LEDs to outputs pins. |  |  |  |  |  |
|  | Applied different logic inputs to Record & verified the output result against each given input. |  |  |  |  |  |
|  | Placed (NAND gate IC) on bread board. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected LEDs to outputs pins. |  |  |  |  |  |
|  | Applied different logic inputs to Record & verified the output result against each given input. |  |  |  |  |  |
|  | Inserted (D flip flop) IC on bread board. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected LEDs\ Scope to outputs pins. |  |  |  |  |  |
|  | Applied different logic inputs to Record & verified the output result against each given input. |  |  |  |  |  |
|  | Inserted 74112 (JK flip flop) IC on bread board. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin. |  |  |  |  |  |
|  | Connected LEDs\ Scope to outputs pins. |  |  |  |  |  |
|  | Applied different logic inputs to Record & verified the output result against each given input. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Construct and Verify function of Flip Flops (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is difference between Latch and flip flop? |  |  |
|  | Define flip flop |  |  |
|  | A flip flop is a _______ circuit. |  |  |
|  | - Combinational - Sequential - Both A and B - None |  |  |
|  | The truth table for an S-R flip-flop has how many VALID entries? |  |  |
|  | - 1 - 2 - 3 d) 4 |  |  |
|  | When both inputs of a J-K flip-flop cycle, the output will ___________ |  |  |
|  | - Be invalid - Change - Not change - Toggle |  |  |
|  | A basic S-R flip-flop can be constructed by cross-coupling of which basic logic gates? |  |  |
|  | - AND or OR gates - XOR or XNOR gates - NOR or NAND gates - d) AND or NOR gates |  |  |
|  | What is a trigger pulse? |  |  |
|  | - A pulse that starts a cycle of operation - A pulse that reverses the cycle of operation - A pulse that prevents a cycle of operation - A pulse that enhances a cycle of operation |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________