Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Analyse Three Phase Motors (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Identify and operate 3 phase Squirrel cage motor. - Verify rotating magnetic field - Measure the starting and running current of three phase induction motor. - Draw torque slip curve of three phase induction motor - Determine the slip of 3 phase induction motor by stroboscope - Starts 3 phase Induction motor with direct on line method. - Starts 3 phase Induction motor with Auto transformer starter. - Starts 3 phase Induction motor with Star Delta starter. - Make connection to reverse the direction of 3 phase induction motor. - Determine the efficiency of three phase induction motor - Control the speed of three phase induction motor by changing input voltage - Control the speed of three phase induction motor by rotor resistance control method - Operate two motors in cascade - Start the synchronous motor using external prime mover. - Identify the effect of field current on three phase synchronous motor. |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Identify and operate 3 phase Squirrel cage motor. - Identify and select 3 phase squirrel cage motor. - Make connection as per diagram. - Connect the supply and interpret the result. Verify rotating magnetic field - Connect the three terminals of stator winding with each other and with ground. - Apply three phase supply to the other terminals. - Place a compass between the stator and note the direction of rotating magnetic field. - Change the two phases with each other. - Note the direction of rotating magnetic field. Measure the starting and running current of three phase induction motor. - Connect the stator winding in star. - Connect an ammeter in series with stator. - Note the ammeter reading at instant of just starting. - Note the reading of ammeter at running condition. Draw torque slip curve of three phase induction motor - Connect three phase induction motor with electro dynamo meter with Belt. - Start the motor and run at rated speed - Measure the RPM. - Change the torque with electro dynamo meter and note the speed - Calculate slip with given values of speed. - Draw graph between slip and torque. Determine the slip of 3 phase induction motor by stroboscope - Connect the stator of three phase induction motor in star - Apply single phase AC supply to the stroboscope. - Mark dot on motor shaft with marker. - Apply three phase supply to the motor - Note the RPM of the motor with the help of stroboscope. - Calculate slip. Starts 3 phase Induction motor with direct on line method. - Draw line diagram of direct-on 3-phase motor Starter. - Identify and select 3 phase Induction motor. - Make Connection of motor with DOL as per diagram. - Switch on the 3 phase breaker and interpret the result. Starts 3 phase Induction motor with Auto transformer starter. - Connect power circuit with the help of diagram. - Connect control circuit with the help of diagram. - Apply rated voltage to the motor. - Gradually reduce the taping of auto transformer with the help of control circuit. Starts 3 phase Induction motor with Star Delta starter. - Draw line diagram of a starter-delta starter. - Identify and select 3 phase Induction motor. - Make Connection of motor with star delta starter as per diagram. - Switch On the 3 phase breaker and interpret the result. Make connection to reverse the direction of 3 phase induction motor. - Draw line diagram to reverse the direction of induction motor - Identify and select 3 phase Induction motor. - Make Connection of motor through 3 pole 2 way switch as per diagram. - Move the handle of 3Pole 2 Way switch from off position to position 1. - Move the handle of 3 Pole 2 Way switch from Position 1 to off position and wait for stopping of motor - Move the handle of 3Pole 2 way switch from off position to position 2 and interpret the result. Determine the efficiency of three phase induction motor - Connect wattmeter, ammeter with the stator of three phase induction motor. - Run the motor at rated speed. - Measure the reading of wattmeter and voltmeter. - Stop the motor - Couple three phase induction motor and electro dynamo meter with belt. - Run the motor. - Supply single phase supply to the dynamo meter. - Adjust the supply of electro dynamo meter so that the maximum current passes through the motor. - Read the meters reading. - Calculate the efficiency of the motor by using these reading. Control the speed of three phase induction motor by changing input voltage - Connect the stator of the motor in star connection. - Insert rheostat in series with stator of the motor. - Set the rheostat resistance to its max value. - Run the motor - Change rheostat resistance and note the effect of speed changes. Control the speed of three phase induction motor by rotor resistance control method - Select wound rotor induction motor. - Connect the stator of motor in star connection. - Connect a star connected starter with motor rotor. - Set rotor resistance to its max value. - Run the motor. - Change the rotor resistance and note the effect of speed change. Operate two motors in cascade - Select two wound rotor induction motor. - Couple shaft of the both motors. - Connect the stator of motor 2 with the rotor of motor 1. - Connect the motor 2 rotor in star connection. - Apply three phase voltage to the stator of motor 1. - Control the speed of both motors by changing the rotor resistance of motor 2. Start the synchronous motor using external prime mover. - Couple the shaft of the synchronous motor with external prime mover (DC Shunt motor) - Connect the stator winding of synchronous motor in star connection. - Apply rated voltage to the stator. - Run the prime mover, which results in locking of rotor with stator magnetic field Identify the effect of field current on three phase synchronous motor. - Apply rated voltage to stator. - Excite the field winding. - Change the field winding voltage and measure the effect on Armature current. - Draw the V curve between Armature Current and Field current. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| Competency Standard | Analyse Three Phase Motors (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Identify and operate 3 phase Squirrel cage motor. - Verify rotating magnetic field - Measure the starting and running current of three phase induction motor. - Draw torque slip curve of three phase induction motor - Determine the slip of 3 phase induction motor by stroboscope - Starts 3 phase Induction motor with direct on line method. - Starts 3 phase Induction motor with Auto transformer starter. - Starts 3 phase Induction motor with Star Delta starter. - Make connection to reverse the direction of 3 phase induction motor. - Determine the efficiency of three phase induction motor - Control the speed of three phase induction motor by changing input voltage - Control the speed of three phase induction motor by rotor resistance control method - Operate two motors in cascade - Start the synchronous motor using external prime mover. - Identify the effect of field current on three phase synchronous motor. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify and select 3 phase squirrel cage motor. |  |  |
| - Make connection as per diagram. |  |  |
| - Connect the supply and interpret the result. |  |  |
| - Connect the three terminals of stator winding with each other and with ground. |  |  |
| - Apply three phase supply to the other terminals. |  |  |
| - Place a compass between the stator and note the direction of rotating magnetic field. |  |  |
| - Change the two phases with each other. |  |  |
| - Note the direction of rotating magnetic field. |  |  |
| - Connect the stator winding in star. |  |  |
| - Connect an ammeter in series with stator. |  |  |
| - Note the ammeter reading at instant of just starting. |  |  |
| - Note the reading of ammeter at running condition. |  |  |
| - Connect three phase induction motor with electro dynamo meter with Belt. |  |  |
| - Start the motor and run at rated speed |  |  |
| - Measure the RPM. |  |  |
| - Change the torque with electro dynamo meter and note the speed |  |  |
| - Calculate slip with given values of speed. |  |  |
| - Draw graph between slip and torque. |  |  |
| - Connect the stator of three phase induction motor in star |  |  |
| - Apply single phase AC supply to the stroboscope. |  |  |
| - Mark dot on motor shaft with marker. |  |  |
| - Apply three phase supply to the motor |  |  |
| - Note the RPM of the motor with the help of stroboscope. |  |  |
| - Calculate slip. |  |  |
| - Draw line diagram of direct-on 3-phase motor Starter. |  |  |
| - Identify and select 3 phase Induction motor. |  |  |
| - Make Connection of motor with DOL as per diagram. |  |  |
| - Switch on the 3 phase breaker and interpret the result. |  |  |
| - Connect power circuit with the help of diagram. |  |  |
| - Connect control circuit with the help of diagram. |  |  |
| - Apply rated voltage to the motor. |  |  |
| - Gradually reduce the taping of auto transformer with the help of control circuit. |  |  |
| - Draw line diagram of a starter-delta starter. |  |  |
| - Identify and select 3 phase Induction motor. |  |  |
| - Make Connection of motor with star delta starter as per diagram. |  |  |
| - Switch On the 3 phase breaker and interpret the result. |  |  |
| - Draw line diagram to reverse the direction of induction motor |  |  |
| - Identify and select 3 phase Induction motor. |  |  |
| - Make Connection of motor through 3 pole 2 way switch as per diagram. |  |  |
| - Move the handle of 3Pole 2 Way switch from off position to position 1. |  |  |
| - Move the handle of 3 Pole 2 Way switch from Position 1 to off position and wait for stopping of motor |  |  |
| - Move the handle of 3Pole 2 way switch from off position to position 2 and interpret the result. |  |  |
| - Connect wattmeter, ammeter with the stator of three phase induction motor. |  |  |
| - Run the motor at rated speed. |  |  |
| - Measure the reading of wattmeter and voltmeter. |  |  |
| - Stop the motor |  |  |
| - Couple three phase induction motor and electro dynamo meter with belt. |  |  |
| - Run the motor. |  |  |
| - Supply single phase supply to the dynamo meter. |  |  |
| - Adjust the supply of electro dynamo meter so that the maximum current passes through the motor. |  |  |
| - Read the meters reading. |  |  |
| - Calculate the efficiency of the motor by using these reading. |  |  |
| - Connect the stator of the motor in star connection. |  |  |
| - Insert rheostat in series with stator of the motor. |  |  |
| - Set the rheostat resistance to its max value. |  |  |
| - Run the motor |  |  |
| - Change rheostat resistance and note the effect of speed changes. |  |  |
| - Select wound rotor induction motor. |  |  |
| - Connect the stator of motor in star connection. |  |  |
| - Connect a star connected starter with motor rotor. |  |  |
| - Set rotor resistance to its max value. |  |  |
| - Run the motor. |  |  |
| - Change the rotor resistance and note the effect of speed change. |  |  |
| - Select two wound rotor induction motor. |  |  |
| - Couple shaft of the both motors. |  |  |
| - Connect the stator of motor 2 with the rotor of motor 1. |  |  |
| - Connect the motor 2 rotor in star connection. |  |  |
| - Apply three phase voltage to the stator of motor 1. |  |  |
| - Control the speed of both motors by changing the rotor resistance of motor 2. |  |  |
| - Couple the shaft of the synchronous motor with external prime mover (DC Shunt motor) |  |  |
| - Connect the stator winding of synchronous motor in star connection. |  |  |
| - Apply rated voltage to the stator. |  |  |
| - Run the prime mover, which results in locking of rotor with stator magnetic field |  |  |
| - Apply rated voltage to stator. |  |  |
| - Excite the field winding. |  |  |
| - Change the field winding voltage and measure the effect on Armature current. |  |  |
| - Draw the V curve between Armature Current and Field current. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Analyse Three Phase Motors (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Identify and operate 3 phase Squirrel cage motor. - Verify rotating magnetic field - Measure the starting and running current of three phase induction motor. - Draw torque slip curve of three phase induction motor - Determine the slip of 3 phase induction motor by stroboscope - Starts 3 phase Induction motor with direct on line method. - Starts 3 phase Induction motor with Auto transformer starter. - Starts 3 phase Induction motor with Star Delta starter. - Make connection to reverse the direction of 3 phase induction motor. - Determine the efficiency of three phase induction motor - Control the speed of three phase induction motor by changing input voltage - Control the speed of three phase induction motor by rotor resistance control method - Operate two motors in cascade - Start the synchronous motor using external prime mover. - Identify the effect of field current on three phase synchronous motor. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identified and select 3 phase squirrel cage motor. |  |  |  |  |  |
|  | Made connection as per diagram. |  |  |  |  |  |
|  | Connected the supply and interpret the result. |  |  |  |  |  |
|  | Connected the three terminals of stator winding with each other and with ground. |  |  |  |  |  |
|  | Applied three phase supply to the other terminals. |  |  |  |  |  |
|  | Placed a compass between the stator and note the direction of rotating magnetic field. |  |  |  |  |  |
|  | Changed the two phases with each other. |  |  |  |  |  |
|  | Noted the direction of rotating magnetic field. |  |  |  |  |  |
|  | Connected the stator winding in star. |  |  |  |  |  |
|  | Connected an ammeter in series with stator. |  |  |  |  |  |
|  | Noted the ammeter reading at instant of just starting. |  |  |  |  |  |
|  | Noted the reading of ammeter at running condition. |  |  |  |  |  |
|  | Connected three phase induction motor with electro dynamo meter with Belt. |  |  |  |  |  |
|  | Started the motor and run at rated speed |  |  |  |  |  |
|  | Measured the RPM. |  |  |  |  |  |
|  | Changed the torque with electro dynamo meter and note the speed |  |  |  |  |  |
|  | Calculated slip with given values of speed. |  |  |  |  |  |
|  | Drew graph between slip and torque. |  |  |  |  |  |
|  | Connected the stator of three phase induction motor in star |  |  |  |  |  |
|  | Applied single phase AC supply to the stroboscope. |  |  |  |  |  |
|  | Marked dot on motor shaft with marker. |  |  |  |  |  |
|  | Applied three phase supply to the motor |  |  |  |  |  |
|  | Noted the RPM of the motor with the help of stroboscope. |  |  |  |  |  |
|  | Calculated slip. |  |  |  |  |  |
|  | Drew line diagram of direct-on 3-phase motor Starter. |  |  |  |  |  |
|  | Identified and select 3 phase Induction motor. |  |  |  |  |  |
|  | Made Connection of motor with DOL as per diagram. |  |  |  |  |  |
|  | Switched on the 3 phase breaker and interpret the result. |  |  |  |  |  |
|  | Connected power circuit with the help of diagram. |  |  |  |  |  |
|  | Connected control circuit with the help of diagram. |  |  |  |  |  |
|  | Applied rated voltage to the motor. |  |  |  |  |  |
|  | Gradually reduced the taping of auto transformer with the help of control circuit. |  |  |  |  |  |
|  | Drew line diagram of a starter-delta starter. |  |  |  |  |  |
|  | Identified and select 3 phase Induction motor. |  |  |  |  |  |
|  | Made Connection of motor with star delta starter as per diagram. |  |  |  |  |  |
|  | Switched On the 3 phase breaker and interpret the result. |  |  |  |  |  |
|  | Drew line diagram to reverse the direction of induction motor |  |  |  |  |  |
|  | Identified and select 3 phase Induction motor. |  |  |  |  |  |
|  | Made Connection of motor through 3 pole 2 way switch as per diagram. |  |  |  |  |  |
|  | Moved the handle of 3Pole 2 Way switch from off position to position 1. |  |  |  |  |  |
|  | Moved the handle of 3 Pole 2 Way switch from Position 1 to off position and wait for stopping of motor |  |  |  |  |  |
|  | Moved the handle of 3Pole 2 way switch from off position to position 2 and interpret the result. |  |  |  |  |  |
|  | Connected wattmeter, ammeter with the stator of three phase induction motor. |  |  |  |  |  |
|  | Ran the motor at rated speed. |  |  |  |  |  |
|  | Measured the reading of wattmeter and voltmeter. |  |  |  |  |  |
|  | Stopped the motor |  |  |  |  |  |
|  | Coupled three phase induction motor and electro dynamo meter with belt. |  |  |  |  |  |
|  | Ran the motor. |  |  |  |  |  |
|  | Supplied single phase supply to the dynamo meter. |  |  |  |  |  |
|  | Adjusted the supply of electro dynamo meter so that the maximum current passes through the motor. |  |  |  |  |  |
|  | Read the meters reading. |  |  |  |  |  |
|  | Calculated the efficiency of the motor by using these reading. |  |  |  |  |  |
|  | Connected the stator of the motor in star connection. |  |  |  |  |  |
|  | Inserted rheostat in series with stator of the motor. |  |  |  |  |  |
|  | Sets the rheostat resistance to its max value. |  |  |  |  |  |
|  | Ran the motor |  |  |  |  |  |
|  | Changed rheostat resistance and note the effect of speed changes. |  |  |  |  |  |
|  | Selected wound rotor induction motor. |  |  |  |  |  |
|  | Connected the stator of motor in star connection. |  |  |  |  |  |
|  | Connected a star connected starter with motor rotor. |  |  |  |  |  |
|  | Sets rotor resistance to its max value. |  |  |  |  |  |
|  | Ran the motor. |  |  |  |  |  |
|  | Changed the rotor resistance and note the effect of speed change. |  |  |  |  |  |
|  | Selected two wound rotor induction motor. |  |  |  |  |  |
|  | Coupled shaft of the both motors. |  |  |  |  |  |
|  | Connected the stator of motor 2 with the rotor of motor 1. |  |  |  |  |  |
|  | Connected the motor 2 rotor in star connection. |  |  |  |  |  |
|  | Applied three phase voltage to the stator of motor 1. |  |  |  |  |  |
|  | Controlled the speed of both motors by changing the rotor resistance of motor 2. |  |  |  |  |  |
|  | Coupled the shaft of the synchronous motor with external prime mover (DC Shunt motor) |  |  |  |  |  |
|  | Connected the stator winding of synchronous motor in star connection. |  |  |  |  |  |
|  | Applied rated voltage to the stator. |  |  |  |  |  |
|  | Ran the prime mover, which results in locking of rotor with stator magnetic field |  |  |  |  |  |
|  | Applied rated voltage to stator. |  |  |  |  |  |
|  | Excited the field winding. |  |  |  |  |  |
|  | Changed the field winding voltage and measure the effect on Armature current. |  |  |  |  |  |
|  | Drew the V curve between Armature Current and Field current. |  |  |  |  |  |
| COMPETENT |  |  | NOT YETCOMPETENT |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Analyse Three Phase Motors (Level 4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Draw Star delta conversion diagram for 3-phase motor. |  |  |
|  | Define how to change the direction of rotation of 3-phase motor. |  |  |
|  | What is Stroboscope? |  |  |
|  | State 3 advantages of 3-phase motor. |  |  |
|  | Define Auto transformer. |  |  |
|  | What is magnetic contactor? |  |  |
|  | What is back EMF? |  |  |
|  | which type of connection is use at the starting of motor? |  |  |
|  | Which type of connection is use at the running of motor? |  |  |
|  | How to reverse the direction of three phase motor? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________