Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Operate Special Purpose Motors (Level-4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Operate and speed control of AC series motor. - Operate miniature (reluctance and hysteresis) single phase AC motors. - Construct and operate stepper motor. - Construct, Operate and develop control circuit of servo motor. |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Operate and speed control of AC series motor. - Connect the armature and field winding in series. - Apply rated voltage to the motor. - Measure the effect on motor speed by change in applied voltage. Operate miniature (reluctance and hysteresis) single phase AC motors. - Connect the stator of motor with single phase supply. - Apply mechanical load by hand - Measure the effect of load change on motor speed. - Draw torque speed curves. Construct and operate stepper motor. - Identify the pin configuration of stepper motor driver circuit. - Wire the stepper motor and driver circuit with the help of wiring diagram. - Connect the stepper motor controller ( Arduino/microcontroller based controller) - Connect the power source with driver circuit and controller circuit - upload the code in controller - Operate motor by given commands for specific controlled movement of motor shaft - Send command for directional control Construct, Operate and develop control circuit of servo motor - Identify the pin configuration of DC servo motor driver circuit. - Wire the servo motor and driver circuit with the help of wiring diagram. - Connect the servo motor controller ( Arduino/microcontroller based controller) - Connect the power source with driver circuit and controller circuit - upload the code in controller to operate position servo control - Operate motor by given commands for specific controlled movement of motor shaft with the help of potentiometer |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| Competency Standard | Operate Special Purpose Motors (Level-4) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Operate and speed control of AC series motor. - Operate miniature (reluctance and hysteresis) single phase AC motors. - Construct and operate stepper motor. - Construct, Operate and develop control circuit of servo motor. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Connect the armature and field winding in series. |  |  |
| - Apply rated voltage to the motor. |  |  |
| - Measure the effect on motor speed by change in applied voltage. |  |  |
| - Connect the stator of motor with single phase supply. |  |  |
| - Apply mechanical load by hand |  |  |
| - Measure the effect of load change on motor speed. |  |  |
| - Draw torque speed curves. |  |  |
| - Identify the pin configuration of stepper motor driver circuit. |  |  |
| - Wire the stepper motor and driver circuit with the help of wiring diagram. |  |  |
| - Connect the stepper motor controller (Arduino/microcontroller based controller) |  |  |
| - Connect the power source with driver circuit and controller circuit |  |  |
| - upload the code in controller |  |  |
| - Operate motor by given commands for specific controlled movement of motor shaft |  |  |
| - Send command for directional control |  |  |
| - Identify the pin configuration of DC servo motor driver circuit. |  |  |
| - Wire the servo motor and driver circuit with the help of wiring diagram. |  |  |
| - Connect the servo motor controller (Arduino/microcontroller based controller) |  |  |
| - Connect the power source with driver circuit and controller circuit |  |  |
| - upload the code in controller to operate position servo control |  |  |
| - Operate motor by given commands for specific controlled movement of motor shaft with the help of potentiometer |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Operate Special Purpose Motors (Level-4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Operate and speed control of AC series motor. - Operate miniature (reluctance and hysteresis) single phase AC motors. - Construct and operate stepper motor. - Construct, Operate and develop control circuit of servo motor. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Connected the armature and field winding in series. |  |  |  |  |  |
|  | Applied rated voltage to the motor. |  |  |  |  |  |
|  | Measured the effect on motor speed by change in applied voltage. |  |  |  |  |  |
|  | Connected the stator of motor with single phase supply. |  |  |  |  |  |
|  | Applied mechanical load by hand |  |  |  |  |  |
|  | Measured the effect of load change on motor speed. |  |  |  |  |  |
|  | Drew torque speed curves. |  |  |  |  |  |
|  | Identified the pin configuration of stepper motor driver circuit. |  |  |  |  |  |
|  | Wired the stepper motor and driver circuit with the help of wiring diagram. |  |  |  |  |  |
|  | Connected the stepper motor controller (Arduino/microcontroller based controller) |  |  |  |  |  |
|  | Connected the power source with driver circuit and controller circuit |  |  |  |  |  |
|  | uploaded the code in controller |  |  |  |  |  |
|  | Operated motor by given commands for specific controlled movement of motor shaft |  |  |  |  |  |
|  | Sent command for directional control |  |  |  |  |  |
|  | Identified the pin configuration of DC servo motor driver circuit. |  |  |  |  |  |
|  | Wired the servo motor and driver circuit with the help of wiring diagram. |  |  |  |  |  |
|  | Connected the servo motor controller (Arduino/microcontroller based controller) |  |  |  |  |  |
|  | Connected the power source with driver circuit and controller circuit |  |  |  |  |  |
|  | uploaded the code in controller to operate position servo control |  |  |  |  |  |
|  | Operated motor by given commands for specific controlled movement of motor shaft with the help of potentiometer |  |  |  |  |  |
| COMPETENT |  |  | NOT YETCOMPETENT |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control |
| --- | --- |
| Competency Standard | Operate Special Purpose Motors (Level-4) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define AC Series Motor. |  |  |
|  | What is stepper motor? |  |  |
|  | Define control unit of Stepper motor. |  |  |
|  | Define Step angle of a stepper motor. |  |  |
|  | Explain the connection of ammeter and volt meter. |  |  |
|  | Difference between permanent magnet & variable reluctance. |  |  |
|  | Universal motors are used in |  |  |
|  | - Conveyor - food mixer - elevator - Refrigerator |  |  |
|  | When the motor speed of the stepper motor increases, the torque will decrease |  |  |
|  | A shaded pole motor is one of the types of single phase induction motor. |  |  |
|  | - True - False |  |  |
|  | When the capacitor of a capacitor start induction motor is short circuited, the motor will __________________. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________