Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control Digital Electronics (Level 4) |
| --- | --- |
| Competency Standard | - Verify truth tables using digital gates - Construct & Verify Combinational logic circuit - Construct and Verify function of Flip Flops - Use 555 IC as Multivibrator |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Operate 7 Segment display with BCD to 7 Segment Decoder IC. - Construct a 4-bit shift register by Using D- Flip Flops - Construct a simple PWM Generator Circuit using 555 Timer |
| Time: 04Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Operate 7 Segment display with BCD to 7 Segment Decoder IC. - Insert 7 segment decoder IC, 7 segment display and switches on bread board. - Make connection as per given circuit diagram. - Identify the input, output, Vcc and ground pin and supply the circuit. - Draw table of BCD table. - Apply the input to BCD to 7 Segment Decoder IC by switch on/off the switches according to table. - Record & verify the output result against each given input. Construct a 4-bit shift register by Using D- Flip Flops - Draw circuit diagram 4-bit register. - Make connection of D-Flip Flop as per diagram to construct 4-bit shift register. - Apply data at the input of register and give clock pulse - Recode the output according to the input. Construct a simple PWM Generator Circuitusing 555 Timer - Place 555 IC on bread board/trainer - Make connection as per diagram. - Output frequency of the PWM signal controlled by selecting resistor RV1 and capacitor C1. - We have used a variable resistor in place of fixed resistor for changing duty cycle of the output signal. - Capacitor Charging through D1 diode and Discharge through D2 diode will generates PWM signal at 555 timer's output pin. - Record the output signal wave shape from oscilloscope. |

.

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 4 in Automation & Process Control Digital Electronics (Level 4) |
| Competency Standard | - Verify truth tables using digital gates - Construct & Verify Combinational logic circuit - Construct and Verify function of Flip Flops - Use 555 IC as Multivibrator |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Operate 7 Segment display with BCD to 7 Segment Decoder IC. - Construct a 4-bit shift register by Using D- Flip Flops - Construct a simple PWM Generator Circuit using 555 Timer |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Insert 7 segment decoder IC, 7 segment display and switches on bread board. |  |  |
| - Make connection as per given circuit diagram. |  |  |
| - Identify the input, output, Vcc and ground pin and supply the circuit. |  |  |
| - Draw table of BCD table. |  |  |
| - Apply the input to BCD to 7 Segment Decoder IC by switch on/off the switches according to table. |  |  |
| - Record & verify the output result against each given input. |  |  |
| - Draw circuit diagram 4-bit register. |  |  |
| - Make connection of D-Flip Flop as per diagram to construct 4-bit shift register. |  |  |
| - Apply data at the input of register and give clock pulse |  |  |
| - Recode the output according to the input. |  |  |
| - Place 555 IC on bread board/trainer |  |  |
| - Make connection as per diagram. |  |  |
| - Output frequency of the PWM signal controlled by selecting resistor RV1 and capacitor C1. |  |  |
| - We have used a variable resistor in place of fixed resistor for changing duty cycle of the output signal. |  |  |
| - Capacitor Charging through D1 diode and Discharge through D2 diode will generates PWM signal at 555 timer's output pin. |  |  |
| 16. Recode the output signal wave shape from oscilloscope. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control Digital Electronics (Level 4) |
| --- | --- |
| Competency Standard | - Verify truth tables using digital gates - Construct & Verify Combinational logic circuit - Construct and Verify function of Flip Flops - Use 555 IC as Multivibrator |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Operate 7 Segment display with BCD to 7 Segment Decoder IC. - Construct a 4-bit shift register by Using D- Flip Flops - Construct a simple PWM Generator Circuit using 555 Timer |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Inserted 7 segment decoder IC, 7 segment display and switches on bread board. |  |  |  |  |  |
|  | Made connection as per given circuit diagram. |  |  |  |  |  |
|  | Identified the input, output, Vcc and ground pin and supply the circuit. |  |  |  |  |  |
|  | Drew table of BCD table. |  |  |  |  |  |
|  | Applied the input to BCD to 7 Segment Decoder IC by switch on/off the switches according to table. |  |  |  |  |  |
|  | Recorded& verified the output result against each given input. |  |  |  |  |  |
|  | Drew circuit diagram 4-bit register. |  |  |  |  |  |
|  | Made connection of D-Flip Flop as per diagram to construct 4-bit shift register. |  |  |  |  |  |
|  | Applied data at the input of register and give clock pulse |  |  |  |  |  |
|  | Recoded the output according to the input. |  |  |  |  |  |
|  | Placed 555 IC on bread board/trainer |  |  |  |  |  |
|  | Made connection as per diagram. |  |  |  |  |  |
|  | Output frequency of the PWM signal controlled by selecting resistor RV1 and capacitor C1. |  |  |  |  |  |
|  | Used a variable resistor in place of fixed resistor for changing duty cycle of the output signal. |  |  |  |  |  |
|  | Capacitor Charging through D1 diode and Discharge through D2 diode will generates PWM signal at 555 timer's output pin. |  |  |  |  |  |
|  | Recoded the output signal wave shape from oscilloscope. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control Digital Electronics (Level 4) |
| --- | --- |
| Competency Standard | - Verify truth tables using digital gates - Construct & Verify Combinational logic circuit - Construct and Verify function of Flip Flops - Use 555 IC as Multivibrator |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | 7 segment generates output |  |  |
|  | - a to b - a to f - a to g - a to z |  |  |
|  | BCD to 7 segment is |  |  |
|  | - decoder - encoder - Multiplexer - Demultiplexer |  |  |
|  | In BCD, number 1010 has |  |  |
|  | - meaning - no meaning - value - Vcc |  |  |
|  | In BCD, number 0011diplay |  |  |
|  | - 1 - 2 - 3 - 4 |  |  |
|  | In D flip-flop, D stands for _____________ |  |  |
|  | - Distant - Data - Desired - Delay |  |  |
|  | The D flip-flop has _______ input. |  |  |
|  | - 1 - 2 - 3 - 4 |  |  |
|  | The D flip-flop has ______ output/outputs. |  |  |
|  | - 1 - 2 - 3 - 4 |  |  |
|  | Which of these sets of logic gates are designated as universal gates? |  |  |
|  | - NOR, NAND. - XOR, NOR, NAND. - OR, NOT, AND. - NOR, NAND, XNOR |  |  |
|  | How many types of multivibrators are? |  |  |
|  | - 2 - 4 - 5 - 3 |  |  |
|  | Astable multivibrator is ________ in any state. |  |  |
|  | - Stable - Unstable - Saturated - Both Stable & Saturated |  |  |
|  | Bistable multivibrator is ________ in any state. |  |  |
|  | - Stable - Unstable - Saturated - Independent |  |  |
|  | Bistable multivibrator has ___________ |  |  |
|  | - two stable states - one stable state - no stable state - three stable state |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________