Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control Industrial Electronic (Level 4) |
| --- | --- |
| Competency Standard | - Implement Diode and Thyristor in power control application. - Construct Single Phase and Three Phase Inverter - Operate Digital Sensor Switches - Control speed of DC/AC Motor |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Make a full wave rectifier circuit using SCR - Construct three phase bridge inverter and observe its output - Operate Digital Proximity Switch - Implement Field control method & armature control method to control the speed of DC motor with 3 phase bridge control rectifier |
| Time: 04Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Make a full wave rectifier circuit using SCR - Select the required components. - Make connection as per given circuit diagram. - Switch on the power supply and vary triggering pulse at the gate of SCR using potentiometer. - One SCR conducts during the positive half cycle and the other during the negative half cycle and thus unidirectional current flows in the load circuit - Connect oscilloscope across the load resistor and record the output wave shape the output wave shape. Construct three phase bridge inverter and observe its output - Draw circuit diagram of three-phase bridge inverter. - Make connection as per diagram. - Apply output of controller IC to the gates of MOSFET’s for triggering pulses. - Connect oscilloscope across the load resistor and record the output wave shape Proximity Switch - Identify the type of proximity switch. - Identify the pins of the unit - Make connection to the controller and supply the switch. - Place the switch with the prescribed dimensions of diameter and length - Test the output of the switch (On and Off Condition) Implement Field control method & armature control method to control the speed of DC motor with 3 phase bridge rectifier. - Draw circuit diagram of field control & armature control method to control the speed of DC motor. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Now changes the triggering of SCR’s and record the effect on speed of DC motor. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 4 in Automation & Process Control Industrial Electronic (Level 4) |
| Competency Standard | - ImplementDiode and Thyristor in power control application. - Construct Single Phase and Three Phase Inverter - Operate Digital Sensor Switches - Control speed of DC/AC Motor |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | - Make a full wave rectifier circuit using SCR - Construct three phase bridge inverter and observe its output - Operate Digital Proximity Switch - Implement Field control method & armature control method to control the speed of DC motor with 3 phase bridge control rectifier |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select the required components. |  |  |
| - Make connection as per given circuit diagram. |  |  |
| - Switch on the power supply and vary triggering pulse at the gate of SCR using potentiometer. |  |  |
| - One SCR conducts during the positive half cycle and the other during the negative half cycle and thus unidirectional current flows in the load circuit |  |  |
| - Connect oscilloscope across the load resistor and record the output wave shape the output wave shape |  |  |
| - Draw circuit diagram of three-phase bridge inverter. |  |  |
| - Make connection as per diagram. |  |  |
| - Apply output of controller IC to the gates of MOSFET’s for triggering pulses. |  |  |
| - Connect oscilloscope across the load resistor and record the output wave shape |  |  |
| - Identify the type of proximity switch. |  |  |
| - Identify the pins of the unit |  |  |
| - Make connection to the controller and supply the switch. |  |  |
| - Place the switch with the prescribed dimensions of diameter and length |  |  |
| - Test the output of the switch (On and Off Condition) |  |  |
| - Draw circuit diagram of field control & armature control method to control the speed of DC motor. |  |  |
| - Make connection as per diagram. |  |  |
| - Apply output of controller IC to the gates of all SCR’s |  |  |
| - Now changes the triggering of SCR’s and record the effect on speed of DC motor. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control Industrial Electronic (Level 4) |
| --- | --- |
| Competency Standard | - Implement Diode and Thyristor in power control application. - Construct Single Phase and Three Phase Inverter - Operate Digital Sensor Switches - Control speed of DC/AC Motor |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Make a full wave rectifier circuit using SCR - Construct three phase bridge inverter and observe its output - Operate Digital Proximity Switch - Implement Field control method & armature control method to control the speed of DC motor with 3 phase bridge control rectifier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Selected the required components. |  |  |  |  |  |
|  | Made connection as per given circuit diagram. |  |  |  |  |  |
|  | Switched on the power supply and vary triggering pulse at the gate of SCR using potentiometer. |  |  |  |  |  |
|  | One SCR conducts during the positive half cycle and the other during the negative half cycle and thus unidirectional current flows in the load circuit |  |  |  |  |  |
|  | Connected oscilloscope across the load resistor and record the output wave shape the output wave shape |  |  |  |  |  |
|  | Drew circuit diagram of three-phase bridge inverter. |  |  |  |  |  |
|  | Made connection as per diagram. |  |  |  |  |  |
|  | Applied output of controller IC to the gates of MOSFET’s for triggering pulses. |  |  |  |  |  |
|  | Connected oscilloscope across the load resistor and record the output wave shape |  |  |  |  |  |
|  | Identified the type of proximity switch. |  |  |  |  |  |
|  | Identified the pins of the unit |  |  |  |  |  |
|  | Made connection to the controller and supply the switch. |  |  |  |  |  |
|  | Placed the switch with the prescribed dimensions of diameter and length |  |  |  |  |  |
|  | Tested the output of the switch (On and Off Condition) |  |  |  |  |  |
|  | Drew circuit diagram of field control & armature control method to control the speed of DC motor. |  |  |  |  |  |
|  | Made connection as per diagram. |  |  |  |  |  |
|  | Applied output of controller IC to the gates of all SCR’s |  |  |  |  |  |
|  | Changed the triggering of SCR’s and record the effect on speed of DC motor. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 4 in Automation & Process Control Industrial Electronic (Level 4) |
| --- | --- |
| Competency Standard | - Implement Diode and Thyristor in power control application. - Construct Single Phase and Three Phase Inverter - Operate Digital Sensor Switches - Control speed of DC/AC Motor |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | SCR cannot be used for full wave rectifier |  |  |
|  | - TRUE - FALSE |  |  |
|  | Draw symbol of SCR |  |  |
|  | SCR is three terminal device |  |  |
|  | - TRUE - FALSE |  |  |
|  | Gate terminal of SCR in control rectifier is used to. |  |  |
|  | - Turn on SCR - Turn off SCR - Firing angle control - Stable the SCR |  |  |
|  | In three-phase bridge inverter we used ______________ number of switching devices. |  |  |
|  | - 3 - 6 - 9 - 12 |  |  |
|  | Power inverter converts |  |  |
|  | - DC to AC - AC to DC - AC to AC - DC to DC |  |  |
|  | In full bridge inverter (1 phase), _____________ number of devices switch on simultaneously at a time. |  |  |
|  | - 1 - 2 - 3 - 4 |  |  |
|  | Name any two method of speed control of DC motor commonly used. |  |  |
|  | In armature voltage control method for speed control in d.c. shunt motor, a variable resistance is |  |  |
|  | - Added in series with the armature - Added in series with the field - Connected across armature - Both in series with armature and field |  |  |
|  | Increase in field resistance will |  |  |
|  | - decrease the speed of motor - increase the speed of motor - not have significant effect on speed - no effect |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________