|  | Assessment Evidence Guide For 4. Pro Auto Electrician Level-3 Formative Assessment 30. Repair Battery Charging System |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Technology L-3 (4. Pro Auto Electrician) |
| --- | --- |
| Competency Standard(s) | 30. Repair Battery Charging System |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Repair charging system - Replace alternator |
| Time: 03 hrs. | During a practical assessment, under observation by an assessor, you are required to - Inspect vehicle charging system and diagnose the following. Battery warning light indication on instrument panel, test output voltage of alternator, drive belt condition, alternator bearing noise, wire harness, fuse, and proper insulated wires. - Dismantle/service of alternator as following; Repair connector, replace carbon brushes, Replace Rectifier Bridge, replace voltage regulator and replace stator winding... - Perform safe working practice to remove the alternator from the vehicle and refit under the specified procedure. - Perform test of alternator on work bench to confirm the operation |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-3 (4. Pro Auto Electrician) |
| Competency Standards | 30. Repair Battery Charging System |
| Assessment Task | 1. Repair charging system 2. Replace alternator |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Inspect vehicle charging system and diagnose the following. Battery warning light indication on instrument panel, test output voltage of alternator, drive belt condition, alternator bearing noise, wire harness, fuse, and proper insulated wires. |  |  |
| - Dismantle/service of alternator as following; Repair connector, replace carbon brushes, Replace Rectifier Bridge, replace voltage regulator and replace stator winding... |  |  |
| - Perform safe working practice to remove the alternator from the vehicle and refit under the specified procedure. |  |  |
| - Perform test of alternator on work bench to confirm the operation |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-3 (4. Pro Auto Electrician) |
| --- | --- |
| Competency Standard(s) | 30. Repair Battery Charging System |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Repair charging system 2. Replace alternator |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Inspect vehicle charging system and diagnose the following. Battery warning light indication on instrument panel, test output voltage of alternator, drive belt condition, alternator bearing noise, wire harness, fuse, and proper insulated wires. |  |  |  |  |  |
|  | Dismantle/service of alternator as following; Repair connector, replace carbon brushes, Replace Rectifier Bridge, replace voltage regulator and replace stator winding... |  |  |  |  |  |
| 3. | Perform safe working practice to remove the alternator from the vehicle and refit under the specified procedure. |  |  |  |  |  |
| 4. | Perform test of alternator on work bench to confirm the operation |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-3 (4. Pro Auto Electrician) |
| --- | --- |
| Competency Standard(s) | 30. Repair Battery Charging System |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define rotor? |  |  |
|  | Define stator windings? |  |  |
|  | The device which regulates the voltages is called …… |  |  |
|  | Rectifier bridge converts alternating current into ……. |  |  |
|  | Alternator converts mechanical energy into electrical energy T/F |  |  |
|  | RPM stands for rough per minute T/F |  |  |
|  | Bridge rectifier consists of Diodes Resistors LED |  |  |
|  | In alternator current produce in stator windings is passed to Rectifier bridge Rotor Slip rings |  |  |
|  | Step up and step down are the types of Transformer Transistor |  |  |
|  | Transformer consists of |  |  |

Feedback to the Candidate

Candidate’s Signature______________________ Assessor’s Signature _________________________