|  | Assessment Evidence Guide For 5. Diagnostic Technician Level-3 Formative Assessment 33. Diagnose Engine Fault |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| --- | --- |
| Competency Standard(s) | 33. Diagnose Engine Fault |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Engine fault adjustment - Engine fault diagnosing |
| Time: 03 hrs. | During a practical assessment, under observation by an assessor, you are required to - Visually inspect and replace the air filter if required - Inspect the oil level of the engine. - Check the water level in the cooling system. - Check and replace the ignition leads of the engine if required. - Check the timing belt if it is not shredded or broken. - Check and inspect the fuel pressure. - Replacement of the spark plugs. - Check if there is any malfunction light on the instrument cluster. - Visually inspect the engine compartment if there is anything worn, broken, burnt or missing. - Check the voltage (if 12 volts are present in the system). - Try to find any short circuit in the wiring harness. - Check and replace crank shaft sensor if not working. - Spin the engine and try to diagnose the problem - If the engine is not spinning than replace the starter motor. - Demonstrate how to check the compression of an engine. |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| Competency Standards | 33. Diagnose Engine Fault |
| Assessment Task | 1. Engine fault adjustment 2. Engine fault diagnosing |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Visually inspect and replace the air filter if required |  |  |
| - Inspect the oil level of the engine. |  |  |
| - Check the water level in the cooling system. |  |  |
| - Check and replace the ignition leads of the engine if required. |  |  |
| - Check the timing belt if it is not shredded or broken. |  |  |
| - Check and inspect the fuel pressure. |  |  |
| - Replacement of the spark plugs. |  |  |
| - Check if there is any malfunction light on the instrument cluster. |  |  |
| - Visually inspect the engine compartment if there is anything worn, broken, burnt or missing. |  |  |
| - Check the voltage (if 12 volts are present in the system). |  |  |
| - Try to find any short circuit in the wiring harness. |  |  |
| - Check and replace crank shaft sensor if not working. |  |  |
| - Spin the engine and try to diagnose the problem |  |  |
| - If the engine is not spinning than replace the starter motor. |  |  |
| - Demonstrate how to check the compression of an engine. |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| --- | --- |
| Competency Standard(s) | 33. Diagnose Engine Fault |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Engine fault adjustment 2. Engine fault diagnosing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Visually inspect and replace the air filter if required |  |  |  |  |  |
|  | Inspect the oil level of the engine. |  |  |  |  |  |
|  | Check the water level in the cooling system. |  |  |  |  |  |
|  | Check and replace the ignition leads of the engine if required. |  |  |  |  |  |
|  | Check the timing belt if it is not shredded or broken. |  |  |  |  |  |
|  | Check and inspect the fuel pressure. |  |  |  |  |  |
|  | Replacement of the spark plugs. |  |  |  |  |  |
|  | Check if there is any malfunction light on the instrument cluster. |  |  |  |  |  |
|  | Visually inspect the engine compartment if there is anything worn, broken, burnt or missing. |  |  |  |  |  |
|  | Check the voltage (if 12 volts are present in the system). |  |  |  |  |  |
|  | Try to find any short circuit in the wiring harness. |  |  |  |  |  |
|  | Check and replace crank shaft sensor if not working. |  |  |  |  |  |
| 13. | Spin the engine and try to diagnose the problem |  |  |  |  |  |
| 14. | If the engine is not spinning than replace the starter motor. |  |  |  |  |  |
| 15. | Demonstrate how to check the compression of an engine. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent. |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| --- | --- |
| Competency Standard(s) | 33. Diagnose Engine Fault |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define types of belts? |  |  |
|  | Describe the importance/function of timing belt? |  |  |
|  | When engine is cranked than electrical energy is converted to. - Mechanical Energy. - Heat Energy. - Thermal energy. - Potential energy. |  |  |
|  | Ideally how many volts are required to charge the battery? |  |  |
|  | Illustrate what will happen if the crank shaft sensor is faulty? |  |  |
|  | Please describe the necessary steps to find out if the fuel pump is faulty? |  |  |
|  | What is the role of a spark plug in an engine? - To ignite the air/fuel mixture Internal Expanding Brake. - Start the engine - All of above. |  |  |
|  | What are the possible causes of engine overheat? - Thermostat Faulty - Water Leakage - Temperature Sensor Faulty - Chocked Radiator - All of these |  |  |
|  | If the stuck of Radiator fan may cause of low oil pressure in the engine? - True - False |  |  |
|  | Enlist the faults of a non-starter engine? |  |  |

Feedback to the Candidate

Candidate’s Signature ________________________Assessor’s Signature_________________________