|  | Assessment Evidence Guide For 5. Diagnostic Technician Level-3 Formative Assessment 34. Perform Engine Overhauling (Petrol/Diesel) |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| --- | --- |
| Competency Standard(s) | 34. Perform Engine Overhauling (Petrol/Diesel) |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Engine sub-assemblies - Engine measurement |
| Time: 03 hrs. | During a practical assessment, under observation by an assessor, you are required to - Inspect the whole engine and check for the possible leaks and broken or worn off or missing parts. - Take off the cylinder head. - Check if the rings or piston are broken or massively worn off. - Dismantle the lower part(block) of the engine and check if the crankshaft or the connecting rods are o.k. - Take the measurements and bore the cylinders. - Assemble the engine and make sure you put the new gaskets and the seals. - Set the timing and set the tappets adjust the spark plugs gap. - Spin the engine manually before cranking to know if there is any resistance of blockage in the cycle. |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| Competency Standards | 34. Perform Engine Overhauling (Petrol/Diesel) |
| Assessment Task | 1. Engine sub-assemblies 2. Engine measurement |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Inspect the whole engine and check for the possible leaks and broken or worn off or missing parts. |  |  |
| - Take off the cylinder head. |  |  |
| - Check if the rings or piston are broken or massively worn off. |  |  |
| - Dismantle the lower part(block) of the engine and check if the crankshaft or the connecting rods are o.k. |  |  |
| - Take the measurements and bore the cylinders. |  |  |
| - Assemble the engine and make sure you put the new gaskets and the seals. |  |  |
| - Set the timing and set the tappets adjust the spark plugs gap. |  |  |
| - Spin the engine manually before cranking to know if there is any resistance of blockage in the cycle. |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| --- | --- |
| Competency Standard(s) | 34. Perform Engine Overhauling (Petrol/Diesel) |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor ________________________Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Engine sub-assemblies 2. Engine measurement |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| 1 | Inspect the whole engine and check for the possible leaks and broken or worn off or missing parts. |  |  |  |  |  |
| 2 | Take off the cylinder head. |  |  |  |  |  |
| 3 | Check if the rings or piston are broken or massively worn off. |  |  |  |  |  |
| 4 | Dismantle the lower part(block) of the engine and check if the crankshaft or the connecting rods are o.k. |  |  |  |  |  |
| 5 | Take the measurements and bore the cylinders. |  |  |  |  |  |
| 6 | Assemble the engine and make sure you put the new gaskets and the seals. |  |  |  |  |  |
| 7 | Set the timing and set the tappets adjust the spark plugs gap. |  |  |  |  |  |
| 8 | Spin the engine manually before cranking to know if there is any resistance of blockage in the cycle. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent. |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| --- | --- |
| Competency Standard(s) | 34. Perform Engine Overhauling (Petrol/Diesel) |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
| 1 | Define the function of the valves and Describe the importance/function valve seals? |  |  |
| 2 | What is the first step if we are about to pull out the engine of the car? Disconnect the earth wire Take off the wheel Check the timing Drain the gear oil |  |  |
| 3 | Ring clip is used to - Remove the piston rings - Install the piston into the cylinder - Broke the piston. - Repair the piston rings. |  |  |
| 4 | What tool is required to tight the cylinder head bolts? |  |  |
| 5 | What is the function of timing gun. |  |  |
| 6 | Name the possible causes of engine failure. |  |  |
| 7 | What is the minimum oil pressure required in an engine while adle? - 5 psi. - 7.5 psi - 10 psi. |  |  |
| 8 | Describe the necessary steps to find out if the oil pump is faulty? |  |  |
| 9 | Please tell the possible causes if the engine block or cylinder head needs to be faced? |  |  |
| 10 | Enlist the step-by-step removal of an engine from the vehicle? |  |  |

Feedback to the Candidate

Candidate’s Signature ________________________Assessor’s Signature_________________________