|  | Assessment Evidence Guide For 5. Diagnostic Technician Level-3 Formative Assessment 39. Diagnose Faults of Ignition System |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| --- | --- |
| Competency Standard(s) | 39. Diagnose Faults of Ignition System |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Repair Ignition System - Adjust ignition timing |
| Time: 03 hrs. | During a practical assessment, under observation by an assessor, you are required to - Inspect the vehicle Ignition system and diagnose the following (Note: follow safe working procedure). - Crank the engine and test presence of spark in a system, identify faulty fuse in fuse box, test the performance of plug cable and plugs, inspect distributor cap/rotor for the performance, assess the correct ignition timing, inspect vacuum advancer, assess C.B Point condition/gap and test condenser, test ignition coil performance, and test CDI of distributor. - Installation of distributer assembly under specified procedure and start the engine, adjust ignition timing with the help of timing gun. - Inspect/Replace ignition switch, inspect/replace ignition coil, and Repair/replace electrical connectors under specified procedure. |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| Competency Standards | 39. Diagnose Faults of Ignition System |
| Assessment Task | 1. Repair Ignition System 2. Adjust ignition timing |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Inspect the vehicle Ignition system and diagnose the following (Note: follow safe working procedure). |  |  |
| - Crank the engine and test presence of spark in a system, identify faulty fuse in fuse box, test the performance of plug cable and plugs, inspect distributor cap/rotor for the performance, assess the correct ignition timing, inspect vacuum advancer, assess C.B Point condition/gap and test condenser, test ignition coil performance, and test CDI of distributor. |  |  |
| - Installation of distributer assembly under specified procedure and start the engine, adjust ignition timing with the help of timing gun. |  |  |
| - Inspect/Replace ignition switch, inspect/replace ignition coil, and Repair/replace electrical connectors under specified procedure. |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| --- | --- |
| Competency Standard(s) | 39. Diagnose Faults of Ignition System |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor ________________________Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Repair Ignition System 2. Adjust ignition timing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| 1 | Inspect the vehicle Ignition system and diagnose the following (Note: follow safe working procedure). |  |  |  |  |  |
| 2 | Crank the engine and test presence of spark in a system, identify faulty fuse in fuse box, test the performance of plug cable and plugs, inspect distributor cap/rotor for the performance, assess the correct ignition timing, inspect vacuum advancer, assess C.B Point condition/gap and test condenser, test ignition coil performance, and test CDI of distributor. |  |  |  |  |  |
| 3 | Installation of distributer assembly under specified procedure and start the engine, adjust ignition timing with the help of timing gun. |  |  |  |  |  |
| 4 | Inspect/Replace ignition switch, inspect/replace ignition coil, and Repair/replace electrical connectors under specified procedure. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent. |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-3 (5. Diagnostic Technician) |
| --- | --- |
| Competency Standard(s) | 39. Diagnose Faults of Ignition System |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
| 1 | Describe the purpose of ignition system? |  |  |
| 2 | what is ignition switch? |  |  |
| 3 | what tools are required to set the distributor timing? - Timing gun - Torque Tod - Impact gun |  |  |
| 4 | What is the function of vacuum advancer? |  |  |
| 5 | what is the abbreviation of CDI? |  |  |
| 6 | What is the usage of timing gun while installing distributor? - Set the timing - Set the timing belt - Set the fan belt |  |  |

Feedback to the Candidate

Candidate’s Signature ________________________ Assessor’s Signature______________________