|  | Assessment Evidence Guide For 9. Auto Body & Paint Technician Level-4(Part-I) Formative Assessment 73. Maintain Paint Booth and Stores |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 73. Maintain Paint Booth and Stores |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Maintain paint booth filters - Maintain cleanliness of paint booth - Replace paint booth filter - Fill fuel in paint booth furnace / burner - Store materials - Maintain paint stores - Store tools and equipment |
| Time: 05 hrs. | During a practical assessment, under observation by an assessor, you are required to - Fixes the air filter blowing air to clean it - Operate air filter in the paint booth for at least half an hour - Clean the interior of the paint booth using a vacuum cleaner - Close doors to prevent the entry of dust - Gather tools and equipment - Remove filters from their attachments - Clean the places where filters and their supports were fixed using a vacuum cleaner - Select filter according to the accessory number of paint booth manufacturer - Fix that filter according to the instructions of manufacturer - Test the functioning of the filter - Fill the fuel tanks with not more than the required quantity of fuel - Fix fire extinguishing equipment - Fix alarm sign - label all the materials for easy identification (using suitable diagrams or names) and store them in the correct places - Store inflammable materials separately from other materials - Keep records of receipts and issues of materials - label paint and other chemicals and store them separately - Store tools and equipment enabling easy access - Enter information about tools and equipment in the stock-book - Maintain data about malfunctioning tools and equipment |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| Competency Standards | 73. Maintain Paint Booth and Stores |
| Assessment Task | - Maintain paint booth filters - Maintain cleanliness of paint booth - Replace paint booth filter - Fill fuel in paint booth furnace / burner - Store materials - Maintain paint stores - Store tools and equipment |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Fixes the air filter blowing air to clean it |  |  |
| - Operate air filter in the paint booth for at least half an hour |  |  |
| - Clean the interior of the paint booth using a vacuum cleaner |  |  |
| - Close doors to prevent the entry of dust |  |  |
| - Gather tools and equipment |  |  |
| - Remove filters from their attachments |  |  |
| - Clean the places where filters and their supports were fixed using a vacuum cleaner |  |  |
| - Select filter according to the accessory number of paint booth manufacturer |  |  |
| - Fix that filter according to the instructions of manufacturer |  |  |
| - Test the functioning of the filter |  |  |
| - Fill the fuel tanks with not more than the required quantity of fuel |  |  |
| - Fix fire extinguishing equipment |  |  |
| - Fix alarm sign |  |  |
| - label all the materials for easy identification (using suitable diagrams or names) and store them in the correct places |  |  |
| - Store inflammable materials separately from other materials |  |  |
| - Keep records of receipts and issues of materials |  |  |
| - label paint and other chemicals and store them separately |  |  |
| - Store tools and equipment enabling easy access |  |  |
| - Enter information about tools and equipment in the stock-book |  |  |
| - Maintain data about malfunctioning tools and equipment |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 73. Maintain Paint Booth and Stores |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor ________________________Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Maintain paint booth filters - Maintain cleanliness of paint booth - Replace paint booth filter - Fill fuel in paint booth furnace / burner - Store materials - Maintain paint stores - Store tools and equipment |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Fixes the air filter blowing air to clean it |  |  |  |  |  |
|  | Operate air filter in the paint booth for at least half an hour |  |  |  |  |  |
|  | Clean the interior of the paint booth using a vacuum cleaner |  |  |  |  |  |
|  | Close doors to prevent the entry of dust |  |  |  |  |  |
|  | Gather tools and equipment |  |  |  |  |  |
|  | Remove filters from their attachments |  |  |  |  |  |
|  | Clean the places where filters and their supports were fixed using a vacuum cleaner |  |  |  |  |  |
|  | Select filter according to the accessory number of paint booth manufacturer |  |  |  |  |  |
|  | Fix that filter according to the instructions of manufacturer |  |  |  |  |  |
|  | Test the functioning of the filter |  |  |  |  |  |
|  | Fill the fuel tanks with not more than the required quantity of fuel |  |  |  |  |  |
|  | Fix fire extinguishing equipment |  |  |  |  |  |
|  | Fix alarm sign |  |  |  |  |  |
|  | label all the materials for easy identification (using suitable diagrams or names) and store them in the correct places |  |  |  |  |  |
|  | Store inflammable materials separately from other materials |  |  |  |  |  |
|  | Keep records of receipts and issues of materials |  |  |  |  |  |
|  | label paint and other chemicals and store them separately |  |  |  |  |  |
|  | Store tools and equipment enabling easy access |  |  |  |  |  |
|  | Enter information about tools and equipment in the stock-book |  |  |  |  |  |
|  | Maintain data about malfunctioning tools and equipment |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 73. Maintain Paint Booth and Stores |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Why use the paint booth for paint. |  |  |
|  | Describe the benefit of paint booth. |  |  |
|  | Describe the importance of air filter in paint booth. |  |  |
|  | Describe the fuel tank and its maintenance. |  |  |
|  | Describe the types of paint that use in auto body shop. |  |  |
|  | Describe the procedure store the paint. |  |  |
|  | Describe the safety precaution for paint process. |  |  |

Feedback to the Candidate

Candidate’s Signature ______________________Assessor’s Signature_________________________