|  | Assessment Evidence Guide For 9. Auto Body & Paint Technician Level-4(Part-I) Formative Assessment 75. Prepare Paint Booth and Vehicle |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 75. Prepare Paint Booth and Vehicle |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Mask required body parts - Remove dust on painted surface - Adjust paint booth temperature - Fix water separating filter - Adjust paint booth burner / furnace - Adjust paint booth heating timer - Operate paint booth ventilator feed air fan |
| Time: 03 hrs. | During a practical assessment, under observation by an assessor, you are required to - Select coating to be applied - Cover such body parts with masking tape or some other material to prevent paint dripping on to them when applying - Scrub off the dust layer - Check whether the sensors in the paint booth were kept in the places mentioned by the manufacturer - Adjust the temperature regulator in the room to the required temperature - Unfix and remove the water separating filter - Select water separating filter of the particular type - Fix it at the correct location - Locate the vehicle at the center of the paint booth - Close all the doors of paint booth - Check whether the fire extinguishing equipment was functioning - Adjust both / all the plugs in the burners to emit a high temperature - Operate all the burners - Adjust both / all the timers as to indicate the same time - Adjust the relevant timers according to the required time - Prepare the vehicle to be painted/ paint and were you ready - Check whether the feed / exhaust air fan in paint booth was functioning - Switch on the fan - Switch off the feed / exhaust fan in the paint booth after painting |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| Competency Standards | 75. Prepare Paint Booth and Vehicle |
| Assessment Task | 1. Mask required body parts 2. Remove dust on painted surface 3. Adjust paint booth temperature 4. Fix water separating filter 5. Adjust paint booth burner / furnace 6. Adjust paint booth heating timer 7. Operate paint booth ventilator feed air fan |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select coating to be applied |  |  |
| - Cover such body parts with masking tape or some other material to prevent paint dripping on to them when applying |  |  |
| - Scrub off the dust layer |  |  |
| - Check whether the sensors in the paint booth were kept in the places mentioned by the manufacturer |  |  |
| - Adjust the temperature regulator in the room to the required temperature |  |  |
| - Unfix and remove the water separating filter |  |  |
| - Select water separating filter of the particular type |  |  |
| - Fix it at the correct location |  |  |
| - Locate the vehicle at the center of the paint booth |  |  |
| - Close all the doors of paint booth |  |  |
| - Check whether the fire extinguishing equipment was functioning |  |  |
| - Adjust both / all the plugs in the burners to emit a high temperature |  |  |
| - Operate all the burners |  |  |
| - Adjust both / all the timers as to indicate the same time |  |  |
| - Adjust the relevant timers according to the required time |  |  |
| - Prepare the vehicle to be painted/ paint and were you ready |  |  |
| - Check whether the feed / exhaust air fan in paint booth was functioning |  |  |
| - Switch on the fan |  |  |
| - Switch off the feed / exhaust fan in the paint booth after painting |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 75. Prepare Paint Booth and Vehicle |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor ________________________Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Mask required body parts 2. Remove dust on painted surface 3. Adjust paint booth temperature 4. Fix water separating filter 5. Adjust paint booth burner / furnace 6. Adjust paint booth heating timer 7. Operate paint booth ventilator feed air fan |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select coating to be applied |  |  |  |  |  |
|  | Cover such body parts with masking tape or some other material to prevent paint dripping on to them when applying |  |  |  |  |  |
|  | Scrub off the dust layer |  |  |  |  |  |
|  | Check whether the sensors in the paint booth were kept in the places mentioned by the manufacturer |  |  |  |  |  |
|  | Adjust the temperature regulator in the room to the required temperature |  |  |  |  |  |
|  | Unfix and remove the water separating filter |  |  |  |  |  |
|  | Select water separating filter of the particular type |  |  |  |  |  |
|  | Fix it at the correct location |  |  |  |  |  |
|  | Locate the vehicle at the center of the paint booth |  |  |  |  |  |
|  | Close all the doors of paint booth |  |  |  |  |  |
|  | Check whether the fire extinguishing equipment was functioning |  |  |  |  |  |
|  | Adjust both/all the plugs in the burners to emit a high temperature |  |  |  |  |  |
|  | Operate all the burners |  |  |  |  |  |
|  | Adjust both / all the timers as to indicate the same time |  |  |  |  |  |
|  | Adjust the relevant timers according to the required time |  |  |  |  |  |
|  | Prepare the vehicle to be painted/ paint and were you ready |  |  |  |  |  |
|  | Check whether the feed / exhaust air fan in paint booth was functioning |  |  |  |  |  |
|  | Switch on the fan |  |  |  |  |  |
|  | Switch off the feed / exhaust fan in the paint booth after painting |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 75. Prepare Paint Booth and Vehicle |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Why masking is necessary during paint process? |  |  |
|  | Why remove the dust from surface before paint process? |  |  |
|  | State importance of heat (Baking) for paint process. |  |  |
|  | Write importance fix water separating filter. |  |  |
|  | Describe importance adjustment paint booth burner / furnace during paint process. |  |  |
|  | Describe importance Adjustment paint booth heating timer during paint process. |  |  |
|  | Describe the importance Operate paint booth ventilator feed air fan. |  |  |
| 8. | A paint booth should be perfectly clean before pulling any vehicle into the booth. - True. - False. |  |  |
| 9. | If turn on the circulation fans in a paint booth____ must closed. |  |  |

Feedback to the Candidate

Candidate’s Signature ________________________Assessor’s Signature_________________