|  | Assessment Evidence Guide For 9. Auto Body & Paint Technician Level-4(Part-I) Formative Assessment 78. Apply Body Sealer |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 78. Apply Body Sealer |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Scuffing for body sealing - Nozzle processing - Cleaning & degreasing - Application of body sealer |
| Time: 03 hrs. | During a practical assessment, under observation by an assessor, you are required to - Collect tools & equipment for scuffing - Wear PPE according to instruction - Scuff the area with double action sander use 600 grit sand paper - Scuff with 600 grit scuffing material in locations where it is impossible to use a dual action sander - Scuff the area, as it will be difficult to scuff after applying body sealer the paint manufacturer - Collect tools & equipment for nozzle processing - cut the tip of nozzle according to required shape of the sealer - cutting both sizes to form a peak shape - cutting one side of a peak shape - Press with pliers - cut the other side so that it is flat - Decide on the areas to be clean - Clean and degrease the surface where body sealer is to be applied - Blow air across the surface to remove dirt and dust - Wipe the surface with a shop cloth-soaked id degreaser - Clean tools & equipment and store them properly - Grasp the lever with your dominant hand, and hold your other in the vicinity of the nozzle - Adjust the air pressure Use low air pressure to apply the sealer in a narrow and thin manner - Adjust the air pressure Use high air pressure to apply the sealer in a wide and thick manner - Adjust the angle of sealing gun - Adjust speed and movement of sealer gun - Find a position that allows you to stand in one place and apply the sealer from start to finish without moving - Use a rubber spatula to shape the body sealer |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| Competency Standards | 78. Apply Body Sealer |
| Assessment Task | 1. Scuffing for body sealing 2. Nozzle processing 3. Cleaning & degreasing 4. Application of body sealer |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Collect tools & equipment for scuffing |  |  |
| - Wear PPE according to instruction |  |  |
| - Scuff the area with double action sander use 600 grit sand paper |  |  |
| - Scuff with 600 grit scuffing material in locations where it is impossible to use a dual action sander |  |  |
| - Scuff the area, as it will be difficult to scuff after applying body sealer the paint manufacturer |  |  |
| - Collect tools & equipment for nozzle processing |  |  |
| - cut the tip of nozzle according to required shape of the sealer |  |  |
| - cutting both sizes to form a peak shape |  |  |
| - cutting one side of a peak shape |  |  |
| - Press with pliers |  |  |
| - cut the other side so that it is flat |  |  |
| - Decide on the areas to be clean |  |  |
| - Clean and degrease the surface where body sealer is to be applied |  |  |
| - Blow air across the surface to remove dirt and dust |  |  |
| - Wipe the surface with a shop cloth-soaked id degreaser |  |  |
| - Clean tools & equipment and store them properly |  |  |
| - Grasp the lever with your dominant hand, and hold your other in the vicinity of the nozzle |  |  |
| - Adjust the air pressure Use low air pressure to apply the sealer in a narrow and thin manner |  |  |
| - Adjust the air pressure Use high air pressure to apply the sealer in a wide and thick manner |  |  |
| - Adjust the angle of sealing gun |  |  |
| - Adjust speed and movement of sealer gun |  |  |
| - Find a position that allows you to stand in one place and apply the sealer from start to finish without moving |  |  |
| - Use a rubber spatula to shape the body sealer |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 78. Apply Body Sealer |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor ________________________Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Scuffing for body sealing 2. Nozzle processing 3. Cleaning & degreasing 4. Application of body sealer |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Collect tools & equipment for scuffing |  |  |  |  |  |
|  | Wear PPE according to instruction |  |  |  |  |  |
|  | Scuff the area with double action sander use 600 grit sand paper |  |  |  |  |  |
|  | Scuff with 600 grit scuffing material in locations where it is impossible to use a dual action sander |  |  |  |  |  |
|  | Scuff the area, as it will be difficult to scuff after applying body sealer the paint manufacturer |  |  |  |  |  |
|  | Collect tools & equipment for nozzle processing |  |  |  |  |  |
|  | cut the tip of nozzle according to required shape of the sealer |  |  |  |  |  |
|  | cutting both sizes to form a peak shape |  |  |  |  |  |
|  | cutting one side of a peak shape |  |  |  |  |  |
|  | Press with pliers |  |  |  |  |  |
|  | cut the other side so that it is flat |  |  |  |  |  |
|  | Decide on the areas to be clean |  |  |  |  |  |
|  | Clean and degrease the surface where body sealer is to be applied |  |  |  |  |  |
|  | Blow air across the surface to remove dirt and dust |  |  |  |  |  |
|  | Wipe the surface with a shop cloth-soaked id degreaser |  |  |  |  |  |
|  | Clean tools & equipment and store them properly |  |  |  |  |  |
|  | Grasp the lever with your dominant hand, and hold your other in the vicinity of the nozzle |  |  |  |  |  |
|  | Adjust the air pressure Use low air pressure to apply the sealer in a narrow and thin manner |  |  |  |  |  |
|  | Adjust the air pressure Use high air pressure to apply the sealer in a wide and thick manner |  |  |  |  |  |
|  | Adjust the angle of sealing gun |  |  |  |  |  |
|  | Adjust speed and movement of sealer gun |  |  |  |  |  |
|  | Find a position that allows you to stand in one place and apply the sealer from start to finish without moving |  |  |  |  |  |
|  | Use a rubber spatula to shape the body sealer |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 78. Apply Body Sealer |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | List types of body sealer. |  |  |
|  | How to preparation body sealers. |  |  |
|  | State the purpose of body sealer application. |  |  |
|  | The ____ provides a solid color over which to apply the color coat. |  |  |
|  | All sealants must be paintable and have good adhesion to bare and primed metal. - True. - False. |  |  |
|  | A ________ should be allowed to adequately dry before painting. |  |  |
|  | Write safety precautions to be followed in the application of body sealer. |  |  |
| 8. | List the location in auto body where to apply body sealer. |  |  |

Feedback to the Candidate

Candidate’s Signature _______________________Assessor’s Signature_________________