|  | Assessment Evidence Guide For 9. Auto Body & Paint Technician Level-4(Part-I) Formative Assessment 79. Perform Masking |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 79. Perform Masking |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Preparation before masking - Masking on opening area - Masking on gaps - Masking on fitting & glass |
| Time: 05 hrs. | During a practical assessment, under observation by an assessor, you are required to - Collect the necessary tools / equipment and materials - Wear PPE according to instruction - Wipe the area with shop cloth which is socked in degreaser - Blow air on the surface to dry the degreaser - Clean work area - Clean tools & equipment and store them properly - Pull out the desired length of tape - Mask the openings left after fittings are removed - Make sure that masking tape will not be peeled off by the air pressure - Mask the gaps between the panels use gap tape - Pay extra attention to the upper and lower edges when masking the front and rear door gaps - Mask the borders of the fittings last using 18mm masking tape - Leave a gap the thickness of the paint coat between the panel surface and the masking tape - Mask the areas that require fine line tape first - Mask the glass moldings that come into direct contact with the paint surface - After painting, remove the masking tape at a 90-degree angle to the vehicle’s surface. |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| Competency Standards | 79. Perform Masking |
| Assessment Task | 1. Preparation before masking 2. Masking on opening area 3. Masking on gaps 4. Masking on fitting & glass |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Collect the necessary tools / equipment and materials |  |  |
| - Wear PPE according to instruction |  |  |
| - Wipe the area with shop cloth which is socked in degreaser |  |  |
| - Blow air on the surface to dry the degreaser |  |  |
| - Clean work area |  |  |
| - Clean tools & equipment and store them properly |  |  |
| - Pull out the desired length of tape |  |  |
| - Mask the openings left after fittings are removed |  |  |
| - Make sure that masking tape will not be peeled off by the air pressure |  |  |
| - Mask the gaps between the panels use gap tape |  |  |
| - Pay extra attention to the upper and lower edges when masking the front and rear door gaps |  |  |
| - Mask the borders of the fittings last using 18mm masking tape |  |  |
| - Leave a gap the thickness of the paint coat between the panel surface and the masking tape |  |  |
| - Mask the areas that require fine line tape first |  |  |
| - Mask the glass moldings that come into direct contact with the paint surface |  |  |
| - After painting, remove the masking tape at a 90-degree angle to the vehicle’s surface. |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 79. Perform Masking |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor ________________________Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Preparation before masking 2. Masking on opening area 3. Masking on gaps 4. Masking on fitting & glass |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Collect the necessary tools / equipment and materials |  |  |  |  |  |
|  | Wear PPE according to instruction |  |  |  |  |  |
|  | Wipe the area with shop cloth which is socked in degreaser |  |  |  |  |  |
|  | Blow air on the surface to dry the degreaser |  |  |  |  |  |
|  | Clean work area |  |  |  |  |  |
|  | Clean tools & equipment and store them properly |  |  |  |  |  |
|  | Pull out the desired length of tape |  |  |  |  |  |
|  | Mask the openings left after fittings are removed |  |  |  |  |  |
|  | Make sure that masking tape will not be peeled off by the air pressure |  |  |  |  |  |
|  | Mask the gaps between the panels use gap tape |  |  |  |  |  |
|  | Pay extra attention to the upper and lower edges when masking the front and rear door gaps |  |  |  |  |  |
|  | Mask the borders of the fittings last using 18mm masking tape |  |  |  |  |  |
|  | Leave a gap the thickness of the paint coat between the panel surface and the masking tape |  |  |  |  |  |
|  | Mask the areas that require fine line tape first |  |  |  |  |  |
|  | Mask the glass moldings that come into direct contact with the paint surface |  |  |  |  |  |
|  | After painting, remove the masking tape at a 90-degree angle to the vehicle’s surface. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 79. Perform Masking |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define Purpose of masking. |  |  |
|  | The ______ should be removed slowly so that it comes off evenly. |  |  |
|  | Write difference between masking and tapping |  |  |
|  | How is masking tape purchased? |  |  |
|  | How is masking material removed from painted surface? |  |  |
|  | What is advantage of masking tape over newspaper? |  |  |
|  | Use newspaper for masking a vehicle because it meets all of masking requirement. - True. - False. |  |  |
|  | Before masking materials are applied the vehicle must be _____ A. Cleaned B. Covered. C. Closed. D. All of these. |  |  |
|  | How do you mask a vehicle using the liquid masking system? |  |  |
|  | Automotive masking paper is heat-resistant so that it can be used safely in baking ovens. - True. - False. |  |  |

Feedback to the Candidate

Candidate’s Signature _______________________Assessor’s Signature_________________________