|  | Assessment Evidence Guide For 9. Auto Body & Paint Technician Level-4(Part-I) Formative Assessment 80. Apply Under Coat |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 80. Apply Under Coat |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Apply undercoats - Apply one pack solid color paint - Apply two pack metallic pearl color paint - Apply two pack solid color paint - Apply transparent colored silver paint - Apply one pack metallic pearl color paint |
| Time: 03 hrs. | During a practical assessment, under observation by an assessor, you are required to - Collect necessary tools & equipment - Wear PPE according to instruction - Mask all the body parts of the vehicle except those that were to be painted - Mix filler or primer with thinner according to the instructions of the paint manufacturer - Measure the viscosity of the mixture According to instruction of the paint manufacture - Adjust the nozzle of the spray gun and test the uniformity of the spray - Start the spray gun and apply the undercoat uniformly follow the method of application - Check the under coat was uniformly applied covering all the areas - Add paint to the spray gun (to 3/4 of its volume) - Prepare the paint mixture according to the instructions of the paint manufacturer (4 parts of paint, 1 part of hardener and 10% of thinner). - Prepare the surface to be applied with 2K solid color paint - Select 1.2 mm or 1.5 mm nozzle spray gun - Maintain the required temperature in the paint booth - Apply paint on the vehicle body follow the prescribed method - Check that the painting was in good condition - Apply the second paint color follow the above steps - Add and mix thinner according to the standard proportion - Adjust the temperature to suit the paint - Adjust the flame of the spray gun as required - Perform spraying by holding the spray gun at the appropriate distance - Allow the body to dry for an appropriate period - Bake the vehicle body well in the paint booth |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| Competency Standards | 80. Apply Under Coat |
| Assessment Task | 1. Apply undercoats 2. Apply one pack solid color paint 3. Apply two pack metallic pearl color paint 4. Apply two pack solid color paint 5. Apply transparent colored silver paint 6. Apply one pack metallic pearl color paint |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Collect necessary tools & equipment |  |  |
| - Wear PPE according to instruction |  |  |
| - Mask all the body parts of the vehicle except those that were to be painted |  |  |
| - Mix filler or primer with thinner according to the instructions of the paint manufacturer |  |  |
| - Measure the viscosity of the mixture According to instruction of the paint manufacture |  |  |
| - Adjust the nozzle of the spray gun and test the uniformity of the spray |  |  |
| - Start the spray gun and apply the undercoat uniformly follow the method of application |  |  |
| - Check the under coat was uniformly applied covering all the areas |  |  |
| - Add paint to the spray gun (to 3/4 of its volume) |  |  |
| - Prepare the paint mixture according to the instructions of the paint manufacturer (4 parts of paint, 1 part of hardener and 10% of thinner). |  |  |
| - Prepare the surface to be applied with 2K solid color paint |  |  |
| - Select 1.2 mm or 1.5 mm nozzle spray gun |  |  |
| - Maintain the required temperature in the paint booth |  |  |
| - Apply paint on the vehicle body follow the prescribed method |  |  |
| - Check that the painting was in good condition |  |  |
| - Apply the second paint color follow the above steps |  |  |
| - Add and mix thinner according to the standard proportion |  |  |
| - Adjust the temperature to suit the paint |  |  |
| - Adjust the flame of the spray gun as required |  |  |
| - Perform spraying by holding the spray gun at the appropriate distance |  |  |
| - Allow the body to dry for an appropriate period |  |  |
| - Bake the vehicle body well in the paint booth |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 80. Apply Under Coat |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor ________________________Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Apply undercoats 2. Apply one pack solid color paint 3. Apply two pack metallic pearl color paint 4. Apply two pack solid color paint 5. Apply transparent colored silver paint 6. Apply one pack metallic pearl color paint |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Collect necessary tools & equipment |  |  |  |  |  |
|  | Wear PPE according to instruction |  |  |  |  |  |
|  | Mask all the body parts of the vehicle except those that were to be painted |  |  |  |  |  |
|  | Mix filler or primer with thinner according to the instructions of the paint manufacturer |  |  |  |  |  |
|  | Measure the viscosity of the mixture According to instruction of the paint manufacture |  |  |  |  |  |
|  | Adjust the nozzle of the spray gun and test the uniformity of the spray |  |  |  |  |  |
|  | Start the spray gun and apply the undercoat uniformly follow the method of application |  |  |  |  |  |
|  | Check the under coat was uniformly applied covering all the areas |  |  |  |  |  |
|  | Add paint to the spray gun (to 3/4 of its volume) |  |  |  |  |  |
|  | Prepare the paint mixture according to the instructions of the paint manufacturer (4 parts of paint, 1 part of hardener and 10% of thinner). |  |  |  |  |  |
|  | Prepare the surface to be applied with 2K solid color paint |  |  |  |  |  |
|  | Select 1.2 mm or 1.5 mm nozzle spray gun |  |  |  |  |  |
|  | Maintain the required temperature in the paint booth |  |  |  |  |  |
|  | Apply paint on the vehicle body follow the prescribed method |  |  |  |  |  |
|  | Check that the painting was in good condition |  |  |  |  |  |
|  | Apply the second paint color follow the above steps |  |  |  |  |  |
|  | Add and mix thinner according to the standard proportion |  |  |  |  |  |
|  | Adjust the temperature to suit the paint |  |  |  |  |  |
|  | Adjust the flame of the spray gun as required |  |  |  |  |  |
|  | Perform spraying by holding the spray gun at the appropriate distance |  |  |  |  |  |
|  | Allow the body to dry for an appropriate period |  |  |  |  |  |
|  | Bake the vehicle body well in the paint booth |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 80. Apply Under Coat |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Write the function of primer. |  |  |
|  | Use 100-part putty over the top of primer or paint to fill small pinholes or sand scratches. - True. - False. |  |  |
|  | Primer is applied to bare metal and body filler before________. |  |  |
|  | Under coat is necessary because paint alone will not properly adhere to or stick to bare metal. - True. - False. |  |  |
|  | A ______ is used to apply primer to all exposed metal surfaces. A. Brush B. Spry gun. C. Scraper. D. With hand. |  |  |
|  | The primer (under coat) is used to improve adhesion (sticking) of the topcoat. - True. - False. |  |  |
|  | Write types of primers. |  |  |
| 8. | Why you must “sandwich” a primer between the substrate and the topcoat. |  |  |
| 9. | Primers are usually enamel or epoxy-based products because they provide better adhesion and corrosion resistance than older lacquers. - True. - False. |  |  |
| 10. | Types of Primers are ________ A. Plain B. Etching. C. Water-based. D. All of these. |  |  |

Feedback to the Candidate

Candidate’s Signature _______________________Assessor’s Signature_________________________