|  | Assessment Evidence Guide For 9. Auto Body & Paint Technician Level-4(Part-I) Formative Assessment 82. Perform Polishing to Body Surface |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 82. Perform Polishing to Body Surface |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Assess and repair the paint surface - Perform compound polishing - Paste body stickers - Apply wax - Clean body moldings - Clean buffers and tires - Re-fix emblem & logo |
| Time:05hrs. | During a practical assessment, under observation by an assessor, you are required to - Identify the areas to be cut using compound - Collect the required tools & equipment and materials - Wear PPE according to instruction - Press the paint coat with a finger to determine whether the surface is dry enough. - Wipe any dirt and dust off the panel with a clean shop cloth - Sand the surface with P1500 – P3000 grit sand paper. - Mask the fittings on the bordering section and panel edges - Attach the buff pad in the center of the polisher - Spread the compound without spinning the polisher to prevent spattering - Move the polisher so that each pass of the polisher overlaps the previous pass by 2/3 - Polish the surface using the polishing compounds in order from fine to ultra-fine - Use a polisher with a sponge buff pad to perform finish polishing. - Use the color matching light to check whether there are any remaining swirl marks - Decide the parts where stickers have to be pasted - Cut the relevant parts according to required shapes - Spray shampoo on the side where the sticker was pasted - Clean and store tools and equipment - Decide the parts to be waxed - Remove dust in areas where wax was to be applied - Apply wax on the required parts - Use a clean piece of cloth to scrub the wax applied - Decide the type of beadings to be cleaned - Clean the beadings and other components without damaging them - Wash the buffers and tires to be painted with a detergent (a kind of shampoo) and clean them - Select special points to fix badges - Mark the points where the badges were to be fixed - Paste double side tape on the inner sides of the badges - Paste the inner sides of the badges with excess tape - Apply primer paint on the bored holes - Paste / fix the badges as required |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| Competency Standards | 82. Perform Polishing to Body Surface |
| Assessment Task | 1. Assess and repair the paint surface 2. Perform compound polishing 3. Paste body stickers 4. Apply wax 5. Clean body moldings 6. Clean buffers and tires 7. Re-fix emblem & logo |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify the areas to be cut using compound |  |  |
| - Collect the required tools & equipment and materials |  |  |
| - Wear PPE according to instruction |  |  |
| - Press the paint coat with a finger to determine whether the surface is dry enough. |  |  |
| - Wipe any dirt and dust off the panel with a clean shop cloth |  |  |
| - Sand the surface with P1500 – P3000 grit sand paper. |  |  |
| - Mask the fittings on the bordering section and panel edges |  |  |
| - Attach the buff pad in the center of the polisher |  |  |
| - Spread the compound without spinning the polisher to prevent spattering |  |  |
| - Move the polisher so that each pass of the polisher overlaps the previous pass by 2/3 |  |  |
| - Polish the surface using the polishing compounds in order from fine to ultra-fine |  |  |
| - Use a polisher with a sponge buff pad to perform finish polishing. |  |  |
| - Use the color matching light to check whether there are any remaining swirl marks |  |  |
| - Decide the parts where stickers have to be pasted |  |  |
| - Cut the relevant parts according to required shapes |  |  |
| - Spray shampoo on the side where the sticker was pasted |  |  |
| - Clean and store tools and equipment |  |  |
| - Decide the parts to be waxed |  |  |
| - Remove dust in areas where wax was to be applied |  |  |
| - Apply wax on the required parts |  |  |
| - Use a clean piece of cloth to scrub the wax applied |  |  |
| - Decide the type of beadings to be cleaned |  |  |
| - Clean the beadings and other components without damaging them |  |  |
| - Wash the buffers and tires to be painted with a detergent (a kind of shampoo) and clean them |  |  |
| - Select special points to fix badges |  |  |
| - Mark the points where the badges were to be fixed |  |  |
| - Paste double side tape on the inner sides of the badges |  |  |
| - Paste the inner sides of the badges with excess tape |  |  |
| - Apply primer paint on the bored holes |  |  |
| - Paste / fix the badges as required |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 82. Perform Polishing to Body Surface |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor ________________________Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Assess and repair the paint surface 2. Perform compound polishing 3. Paste body stickers 4. Apply wax 5. Clean body moldings 6. Clean buffers and tires 7. Re-fix emblem & logo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identify the areas to be cut using compound |  |  |  |  |  |
|  | Collect the required tools & equipment and materials |  |  |  |  |  |
|  | Wear PPE according to instruction |  |  |  |  |  |
|  | Press the paint coat with a finger to determine whether the surface is dry enough. |  |  |  |  |  |
|  | Wipe any dirt and dust off the panel with a clean shop cloth |  |  |  |  |  |
|  | Sand the surface with P1500 – P3000 grit sand paper. |  |  |  |  |  |
|  | Mask the fittings on the bordering section and panel edges |  |  |  |  |  |
|  | Attach the buff pad in the center of the polisher |  |  |  |  |  |
|  | Spread the compound without spinning the polisher to prevent spattering |  |  |  |  |  |
|  | Move the polisher so that each pass of the polisher overlaps the previous pass by 2/3 |  |  |  |  |  |
|  | Polish the surface using the polishing compounds in order from fine to ultra-fine |  |  |  |  |  |
|  | Use a polisher with a sponge buff pad to perform finish polishing. |  |  |  |  |  |
|  | Use the color matching light to check whether there are any remaining swirl marks |  |  |  |  |  |
|  | Decide the parts where stickers have to be pasted |  |  |  |  |  |
|  | Cut the relevant parts according to required shapes |  |  |  |  |  |
|  | Spray shampoo on the side where the sticker was pasted |  |  |  |  |  |
|  | Clean and store tools and equipment |  |  |  |  |  |
|  | Decide the parts to be waxed |  |  |  |  |  |
|  | Remove dust in areas where wax was to be applied |  |  |  |  |  |
|  | Apply wax on the required parts |  |  |  |  |  |
|  | Use a clean piece of cloth to scrub the wax applied |  |  |  |  |  |
|  | Decide the type of beadings to be cleaned |  |  |  |  |  |
|  | Clean the beadings and other components without damaging them |  |  |  |  |  |
|  | Wash the buffers and tires to be painted with a detergent (a kind of shampoo) and clean them |  |  |  |  |  |
|  | Select special points to fix badges |  |  |  |  |  |
|  | Mark the points where the badges were to be fixed |  |  |  |  |  |
|  | Paste double side tape on the inner sides of the badges |  |  |  |  |  |
|  | Paste the inner sides of the badges with excess tape |  |  |  |  |  |
|  | Apply primer paint on the bored holes |  |  |  |  |  |
|  | Paste / fix the badges as required |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-4 (P-I) (9. Auto Body & Paint Technician) |
| --- | --- |
| Competency Standard(s) | 82. Perform Polishing to Body Surface |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Write the purpose of polishing. |  |  |
| 2. | Polishing involves using very fine grit compound to bring the paint surface up to full gloss. - True. - False. |  |  |
| 3. | Polishing is usually done after ________. |  |  |
| 4. | Slight defects in the topcoat can be repaired by __________. |  |  |
| 5. | To care for a new finish avoiding waxing and polishing for up to ____months. (A) 1 (B) 2 (C) 3 (D) 4 |  |  |
| 6. | Polishing machine is power tool that uses an orbital action to bring out the full paint gloss or shine. - True. - False. |  |  |
| 7. | Differentiate between sanding and polishing. |  |  |

Feedback to the Candidate

Candidate’s Signature ______________________Assessor’s Signature___________________