|  | Assessment Evidence Guide For 18. Vehicle Examiner Level-5 (Part-II) Formative Assessment 142. Inspect Vehicle Steering System and Wheel Alignment |
| --- | --- |
|  | Date 15-19 September, 2020 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame (for practical demonstration & assessment) - Inspect and check the performance of steering system and wheel alignment - Inspect and check the wheel alignment |
| Time: 03 hrs. | During a practical assessment, under observation by an assessor, you are required to - Inspect vehicle steering system for rubber boots on (inner and outer ball joint), Tie road condition, and inner/outer ball joints play, steering rack and pinion box, Mounting for the steering box under the specified procedure. - Replacement/servicing the Outer tie road, inner tie road, steering mountings, steering boots (inner and outer ball joint), under the specified procedure - Servicing steering gear (Rack and pinion) box under the specified procedure. - Inspect and check the following components of steering system - Lose steering Column - Defective energy absorbing steering column - Steering wheel play - Steering linkage play - Steering gear lash - Steering wheel Hardness - Lose Ball joint - Front wheel bearing - Check for the vehicle pre alignment for proper Tire pressure/inflation, loose ball joints, wheel bearings, tie road ends, axle bushes, faulty spring, tire deformation, vehicle ride height, and loose stabilizer bar under specified procedure. - Drove the vehicle and load/offload it on wheel alignment machine following safety rules as specified in the manual. - Perform wheel alignment using the wheel alignment machine as described procedure in the machine manual and adjusts Toe, camber of front and rear wheels under specified procedure. |
| Minimum Evidence Required |  |

| Qualification | Automobile Technology L-5 (P-II) (18. Vehicle Examiner) |
| --- | --- |
| Competency Standard(s) | 142. Inspect Vehicle Steering System and Wheel Alignment |
| Purpose of Assessment | Formative Assessment |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Automobile Technology L-5 (P-II) (18. Vehicle Examiner) |
| Competency Standards | 142. Inspect Vehicle Steering System and Wheel Alignment |
| Assessment Task | - Inspect and check the performance of steering system and wheel alignment - Inspect and check the wheel alignment |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Inspect vehicle steering system for rubber boots on (inner and outer ball joint), Tie road condition, and inner/outer ball joints play, steering rack and pinion box, Mounting for the steering box under the specified procedure. |  |  |
| - Replacement/servicing the Outer tie road, inner tie road, steering mountings, steering boots (inner and outer ball joint), under the specified procedure |  |  |
| - Servicing steering gear (Rack and pinion) box under the specified procedure. |  |  |
| - Inspect and check the following components of steering system |  |  |
| - Lose steering Column |  |  |
| - Defective energy absorbing steering column |  |  |
| - Steering wheel play |  |  |
| - Steering linkage play |  |  |
| - Steering gear lash |  |  |
| - Steering wheel Hardness |  |  |
| - Lose Ball joint |  |  |
| - Front wheel bearing |  |  |
| - Check for the vehicle pre alignment for proper Tire pressure/inflation, loose ball joints, wheel bearings, tie road ends, axle bushes, faulty spring, tire deformation, vehicle ride height, and loose stabilizer bar under specified procedure. |  |  |
| - Drove the vehicle and load/offload it on wheel alignment machine following safety rules as specified in the manual. |  |  |
| - Perform wheel alignment using the wheel alignment machine as described procedure in the machine manual and adjusts Toe, camber of front and rear wheels under specified procedure. |  |  |

Candidate’s Signature___________________	Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Automobile Technology L-5 (P-II) (18. Vehicle Examiner) |
| --- | --- |
| Competency Standard(s) | 142. Inspect Vehicle Steering System and Wheel Alignment |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Inspect and check the performance of steering system and wheel alignment - Inspect and check the wheel alignment |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Inspect vehicle steering system for rubber boots on (inner and outer ball joint), Tie road condition, and inner/outer ball joints play, steering rack and pinion box, Mounting for the steering box under the specified procedure. |  |  |  |  |  |
|  | Replacement/servicing the Outer tie road, inner tie road, steering mountings, steering boots (inner and outer ball joint), under the specified procedure |  |  |  |  |  |
|  | Servicing steering gear (Rack and pinion) box under the specified procedure. |  |  |  |  |  |
|  | Inspect and check the following components of steering system - Lose steering Column - Defective energy absorbing steering column - Steering wheel play - Steering linkage play - Steering gear lash - Steering wheel Hardness - Lose Ball joint - Front wheel bearing |  |  |  |  |  |
|  | Check for the vehicle pre alignment for proper Tire pressure/inflation, loose ball joints, wheel bearings, tie road ends, axle bushes, faulty spring, tire deformation, vehicle ride height, and loose stabilizer bar under specified procedure. |  |  |  |  |  |
|  | Drove the vehicle and load/offload it on wheel alignment machine following safety rules as specified in the manual. |  |  |  |  |  |
|  | Perform wheel alignment using the wheel alignment machine as described procedure in the machine manual and adjusts Toe, camber of front and rear wheels under specified procedure. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Automobile Technology L-5 (P-II) (18. Vehicle Examiner) |
| --- | --- |
| Competency Standard(s) | 142. Inspect Vehicle Steering System and Wheel Alignment |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe the purpose and function of steering systems (non-assisted/manual), power assisted system, electronic steering system with a motor |  |  |
|  | True rolling is obtained only when the wheels roll on arcs which have a 'common center' or common axis - True - False |  |  |
|  | Define steering ratio |  |  |
|  | The converts the rotary movement of the steering wheel in to the angular turning of the front wheels - Steering gear box - Struts - Rack and pinion - steering arm |  |  |
|  | Enlist tools and equipment used for wheel alignment |  |  |
|  | If tie rod ends become worn, they may cause the steering wheel to feel loose - True - False |  |  |
|  | How do you diagnose and rectify the steering system problems |  |  |
|  | is implemented so that when the car is cornering and the wheel is rolling over and gaining - Negative camber - Positive camber |  |  |
|  | How does ride height affect vehicle performance |  |  |

Feedback to the Candidate

Candidate’s Signature_____________________ Assessor’s Signature ____________________