|  | Assessment Evidence Guide For NVC LEVEL-3 “BANANA BIOMASS RECYCLING” (Junior Technician) Summative Assessment |
| --- | --- |
|  | Date: 18th – 24th May 2026 Karachi |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate manage

| Qualification | NVC Level 3 Banana Biomass Recycling (Junior Technician) |
| --- | --- |
| Competency Standard | 0031PSD02 Perform Basic Communication 0211BBR02 Comply with Personal Health and Safety Guidelines 0813BBR301 Execute Digital Literacy 0813BBR302 Perform Eco Product Development 0813BBR303 Perform Sub-Surface composting 0813BBR304 Assist Quality Assurance system for Recycled Products 0613BBR305 Green Entrepreneurship |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration Number_______________________________________ Date______________________________ |
| Guidance for Candidate | Section A1: Guidance for Candidate To meet this standard, you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): Activity:2 Perform Sub-Surface Composting process from Banana biomass |
| Time: 5 Hours. | During a practical assessment, under observation by an assessor, you are required to: |
| Minimum Evidence Required | Assess suitability of soil type and water table depth Remove non-biodegradable items. Manage aeration by coarse material layering Install perforated pipes if forced aeration required Categorize biomass into green; N (Nitrogen) and brown; C (Carbon) Evaluate moisture level of raw biomass before mixing Manage C:N ratio, balancing required amount of animal manure. Prevent biomass from hazardous chemicals Spread biomass in uniform layers within standard thickness Apply starter culture with manure layers between biomass Shape windrow into trapezoidal structure for drainage if needed Perform hand squeeze test for moisture evaluation Monitor digital moisture sensors placed at different bed depths. Select UV-resistant plastic/ biodegradable waste/crop residue Overlap cover edges preventing pests Verify expiry date and storage conditions of bio-decomposers. Mix microbial consortia with lukewarm water to initiate microbial activity Spray solution evenly during the turning process. Maintain optimal pH levels (6.5–7.5) for microbial survival. Dispose empty containers according to OHS rules. Clean collection area for next cycle |

(Bold Statements in the above performance Criteria are critical)

Section A2: Self-Assessment Checklist

| Assessment Task | Activity:1 Perform Sub-Surface Composting process from Banana biomass |
| --- | --- |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Assess suitability of soil type and water table depth |  |  |
| - Remove non-biodegradable items. |  |  |
| - Manage aeration by coarse material layering |  |  |
| - Install perforated pipes if forced aeration required |  |  |
| - Categorize biomass into green; N (Nitrogen) and brown; C (Carbon) |  |  |
| - Evaluate moisture level of raw biomass before mixing |  |  |
| - Manage C:N ratio, balancing required amount of animal manure. |  |  |
| - Prevent biomass from hazardous chemicals |  |  |
| - Spread biomass in uniform layers within standard thickness |  |  |
| - Apply starter culture with manure layers between biomass |  |  |
| - Shape windrow into trapezoidal structure for drainage if needed |  |  |
| - Perform hand squeeze test for moisture evaluation |  |  |
| - Monitor digital moisture sensors placed at different bed depths. |  |  |
| - Select UV-resistant plastic/ biodegradable waste/crop residue |  |  |
| - Overlap cover edges preventing pests |  |  |
| - Verify expiry date and storage conditions of bio-decomposers. |  |  |
| - Mix microbial consortia with lukewarm water to initiate microbial activity |  |  |
| - Spray solution evenly during the turning process. |  |  |
| - Maintain optimal pH levels (6.5–7.5) for microbial survival. |  |  |
| - Dispose empty containers according to OHS rules. |  |  |
| - Clean collection area for next cycle |  |  |

(Bold Statements in the above performance Criteria are critical)

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Section B : Assessors Judgment Guide

| Qualification | NVC Level-3 Banana Biomass Recycling (Junior Technician) |
| --- | --- |
| Competency Standard | 0031PSD02 Perform Basic Communication 0211BBR02 Comply with Personal Health and Safety Guidelines 0813BBR301 Execute Digital Literacy 0813BBR302 Perform Eco Product Development 0813BBR303 Perform Sub-Surface composting 0813BBR304 Assist Quality Assurance system for Recycled Products 0613BBR305 Green Entrepreneurship |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: _____________________________________________________ Registration Number_________________ Signature: ______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor____________________________________ Assessor’s code: _____________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Section B1: Observation Checklist

| Assessment Task |  | Activity:1 Perform Sub-Surface Composting process from Banana biomass |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Assess suitability of soil type and water table depth |  |  |  |  |  |
|  | Remove non-biodegradable items. |  |  |  |  |  |
|  | Manage aeration by coarse material layering |  |  |  |  |  |
|  | Install perforated pipes if forced aeration required |  |  |  |  |  |
|  | Categorize biomass into green; N (Nitrogen) and brown; C (Carbon) |  |  |  |  |  |
|  | Evaluate moisture level of raw biomass before mixing |  |  |  |  |  |
|  | Manage C:N ratio, balancing required amount of animal manure. |  |  |  |  |  |
|  | Prevent biomass from hazardous chemicals |  |  |  |  |  |
|  | Spread biomass in uniform layers within standard thickness |  |  |  |  |  |
|  | Apply starter culture with manure layers between biomass |  |  |  |  |  |
|  | Shape windrow into trapezoidal structure for drainage if needed |  |  |  |  |  |
|  | Perform hand squeeze test for moisture evaluation |  |  |  |  |  |
|  | Monitor digital moisture sensors placed at different bed depths. |  |  |  |  |  |
|  | Select UV-resistant plastic/ biodegradable waste/crop residue |  |  |  |  |  |
|  | Overlap cover edges preventing pests |  |  |  |  |  |
|  | Verify expiry date and storage conditions of bio-decomposers. |  |  |  |  |  |
|  | Mix microbial consortia with lukewarm water to initiate microbial activity |  |  |  |  |  |
|  | Spray solution evenly during the turning process. |  |  |  |  |  |
|  | Maintain optimal pH levels (6.5–7.5) for microbial survival. |  |  |  |  |  |
|  | Dispose empty containers according to OHS rules. |  |  |  |  |  |
|  | Clean collection area for next cycle |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

(Bold Statements in the above performance Criteria are critical)

Section B2: Knowledge Assessment

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | How long does banana residue composting usually take? |  |  |
|  | What does ammonia smell indicate in a compost pile? |  |  |
|  | Why should you wash sieving equipment immediately after compost processing? |  |  |
|  | Why should you store finished compost in shaded area away from direct sun? |  |  |
|  | What is the main advantage of sub-surface composting for farmers? |  |  |
|  | How can product quality be improved? |  |  |
|  | What is the benefit of implementing a quality assurance system? |  |  |
|  | What materials are included in banana residue composting? |  |  |
|  | Why is turning of compost important? |  |  |
|  | How does sub-surface composting benefit soil? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________