Instruction Sheet for the Candidate

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Measure Angles by Theodolite |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the giventime frame (for practical demonstration & assessment): - Find out Horizontal Angle measurement - Measure Horizontal Angle by Repetition Method - Measure Horizontal Angle by Reiteration Method - Measure Vertical Angle measurement |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Identify the Equipment - Set Theodolite on tripod at station point O - Perform temporary adjustment & Focus Theodolite. - Set Vernier 0⁰-0’-0”at given point A - Range other object B - Take Vernier reading for angle AOB - Repeat method B to A - Check the difference - Bisect taken angle - Repeat method BOA - Set the zero of vertical zero of the vertical circle clamp and tangent screw. - Bring bubble of the altitude level in the central position - Use clip screw for adjustment - Loose vertical circle clamp screw - Set telescope towards object and sight - Set vertical circle tangent screw. - Observe both verniers on the vertical circle - Check mean of two vernier readings - Change “face” instrument and repeat process. - Calculate mean of two vernier readings (vertical angle). - Check instrumental errors. |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Surveying |
| Competency Standards | Measure Angles by Theodolite |
| Assessment Task | - Find out Horizontal Angle measurement - Measure Horizontal Angle by Repetition Method - Measure Horizontal Angle by Reiteration Method - Measure Vertical Angle measurement |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify the Equipment |  |  |
| - Set Theodolite on tripod at station point |  |  |
| - Perform temporary adjustment & Focus Theodolite. |  |  |
| - Set 0⁰-0’-0” Vernier |  |  |
| - Bisect given points |  |  |
| - Sight other given points |  |  |
| - Read Vernier reading |  |  |
| - Identify the Equipment |  |  |
| - Set Theodolite on tripod at station point O |  |  |
| - Perform temporary adjustment & Focus Theodolite. |  |  |
| - Set Vernier 0⁰-0’-0”at given point A |  |  |
| - Range other object B |  |  |
| - Take Vernier reading for angle AOB |  |  |
| - Repeat method B to A |  |  |
| - Check the difference |  |  |
| - Identify the Equipment |  |  |
| - Set Theodolite on tripod at station point O |  |  |
| - Perform temporary adjustment & Focus Theodolite. |  |  |
| - Set Vernier 0⁰0’0” at given point A |  |  |
| - Range other object B |  |  |
| - Take Vernier Angle AOB |  |  |
| - Bisect taken angle |  |  |
| - Repeat method BOA |  |  |
| - Check the difference |  |  |
| - Perform permanent adjustment of theodolite at given station point |  |  |
| - Set the zero of vertical zero of the vertical circle clamp and tangent screw. |  |  |
| - Bring bubble of the altitude level in the central position |  |  |
| - Use clip screw for adjustment |  |  |
| - Loose vertical circle clamp screw |  |  |
| - Set telescope towards object and sight |  |  |
| - Set vertical circle tangent screw. |  |  |
| - Observe both verniers on the vertical circle |  |  |
| - Check mean of two vernier readings |  |  |
| - Change “face” instrument and repeat process. |  |  |
| - Calculate mean of two vernier readings (vertical angle). |  |  |
| - Check instrumental errors. |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Measure Angles by Theodolite |
| Candidate Details | Name:______________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Find out Horizontal Angle measurement - Measure Horizontal Angle by Repetition Method - Measure Horizontal Angle by Reiteration Method - Measure Vertical Angle measurement |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identified the Equipment |  |  |  |  |  |
|  | Set Theodolite on tripod at station point |  |  |  |  |  |
|  | Performed temporary adjustment & Focus Theodolite. |  |  |  |  |  |
|  | Set 0⁰-0’-0” Vernier |  |  |  |  |  |
|  | Bisected given points |  |  |  |  |  |
|  | Sighted other given points |  |  |  |  |  |
|  | Read Vernier reading |  |  |  |  |  |
|  | Identified the Equipment |  |  |  |  |  |
|  | Set Theodolite on tripod at station point O |  |  |  |  |  |
|  | Performed temporary adjustment & Focus Theodolite. |  |  |  |  |  |
|  | Set Vernier 0⁰-0’-0”at given point A |  |  |  |  |  |
|  | Ranged other object B |  |  |  |  |  |
|  | Took Vernier reading for angle AOB |  |  |  |  |  |
|  | Repeated method B to A |  |  |  |  |  |
|  | Checked the difference |  |  |  |  |  |
|  | Identified the Equipment |  |  |  |  |  |
|  | Set Theodolite on tripod at station point O |  |  |  |  |  |
|  | Performed temporary adjustment & Focus Theodolite. |  |  |  |  |  |
|  | Set Vernier 0⁰0’0” at given point A |  |  |  |  |  |
|  | Ranged other object B |  |  |  |  |  |
|  | Took Vernier Angle AOB |  |  |  |  |  |
|  | Bisected taken angle |  |  |  |  |  |
|  | Repeated method BOA |  |  |  |  |  |
|  | Checked the difference |  |  |  |  |  |
|  | Performed permanent adjustment of theodolite at given station point |  |  |  |  |  |
|  | Set the zero of vertical zero of the vertical circle clamp and tangent screw. |  |  |  |  |  |
|  | Brought bubble of the altitude level in the central position |  |  |  |  |  |
|  | Used clip screw for adjustment |  |  |  |  |  |
|  | Loosed vertical circle clamp screw |  |  |  |  |  |
|  | Set telescope towards object and sight |  |  |  |  |  |
|  | Set vertical circle tangent screw. |  |  |  |  |  |
|  | Observed both verniers on the vertical circle |  |  |  |  |  |
|  | Checked mean of two vernier readings |  |  |  |  |  |
|  | Changed “face” instrument and repeat process. |  |  |  |  |  |
|  | Calculated mean of two vernier readings (vertical angle). |  |  |  |  |  |
|  | Checked instrumental errors. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Measure Angles by Theodolite |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is theodolite? |  |  |
|  | Name the types of theodolite. |  |  |
|  | Name the parts of theodolite. |  |  |
|  | What is temporary adjustment of instrument? |  |  |
|  | Define centering of instrument. |  |  |
|  | Define leveling of instrument. |  |  |
|  | What is meant by non transit theodolite? |  |  |
|  | What is meant by transit theodolite? |  |  |
|  | What is meant by transiting? |  |  |
|  | Define left face of theodolite. |  |  |
|  | Define right face of theodolite. |  |  |
|  | What is meant by swinging of telescope? |  |  |
|  | What is difference between normal and inverted telescope? |  |  |
|  | Define trunion axis. |  |  |
|  | Define line of collimation. |  |  |
|  | What is Azimuth? |  |  |
|  | What is vertical circle? |  |  |
|  | What is meant by axis of telescope? |  |  |
|  | What is the function of tangent screw? |  |  |
|  | What is meant by least count? |  |  |
|  | What is parallex? |  |  |
|  | How parallex is removed? |  |  |
|  | Name the methods of taking horizontal angles with theodolite. |  |  |
|  | What is focusing of theodolite? |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________