Instruction Sheet for the Candidate

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Perform Temporary Adjustment of Total Station |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the giventime frame (for practical demonstration & assessment): - Adjust temporarily total station on a given point - Set the Total station Electronically - Set co-ordinates of occupied station |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Identify the equipment - Unfold tripod - Perform centering of total station at station point - Perform leveling of total station - Adjust foot screws & center the bubble - Focus the telescope - Set instrument on station point - Fix battery - Turn on instrument - Show opening screen as “MEAS” screen. - Adjust foot screw - Rotate instrument 90 degree - Repeat method and check - Set instrument on station point - Turn on instrument - Select co-ordinate menu - Input co-ordinates - Check Result |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Surveying |
| Competency Standards | Perform Temporary Adjustment of Total Station |
| Assessment Task | - Adjust temporarily total station on a given point - Set the Total station Electronically - Set co-ordinates of occupied station |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify the equipment |  |  |
| - Unfold tripod |  |  |
| - Perform centering of total station at station point |  |  |
| - Perform leveling of total station |  |  |
| - Adjust foot screws & center the bubble |  |  |
| - Focus the telescope |  |  |
| - Set instrument on station point |  |  |
| - Fix battery |  |  |
| - Turn on instrument |  |  |
| - Show opening screen as “MEAS” screen. |  |  |
| - Adjust foot screw |  |  |
| - Rotate instrument 90 degree |  |  |
| - Repeat method and check |  |  |
| - Set instrument on station point |  |  |
| - Turn on instrument |  |  |
| - Select co-ordinate menu |  |  |
| - Input co-ordinates |  |  |
| - Check Result |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Perform Temporary Adjustment of Total Station |
| Candidate Details | Name:______________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Adjust temporarily total station on a given point - Set the Total station Electronically - Set co-ordinates of occupied station |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identify the equipment |  |  |  |  |  |
|  | Unfold tripod |  |  |  |  |  |
|  | Perform centering of total station at station point |  |  |  |  |  |
|  | Perform leveling of total station |  |  |  |  |  |
|  | Adjust foot screws & center the bubble |  |  |  |  |  |
|  | Focus the telescope |  |  |  |  |  |
|  | Set instrument on station point |  |  |  |  |  |
|  | Fix battery |  |  |  |  |  |
|  | Turn on instrument |  |  |  |  |  |
|  | Show opening screen as “MEAS” screen. |  |  |  |  |  |
|  | Adjust foot screw |  |  |  |  |  |
|  | Rotate instrument 90 degree |  |  |  |  |  |
|  | Repeat method and check |  |  |  |  |  |
|  | Set instrument on station point |  |  |  |  |  |
|  | Turn on instrument |  |  |  |  |  |
|  | Select co-ordinate menu |  |  |  |  |  |
|  | Input co-ordinates |  |  |  |  |  |
|  | Check Result |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Perform Temporary Adjustment of Total Station |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is total station? |  |  |
|  | Name the types of total station. |  |  |
|  | Name the parts of total station. |  |  |
|  | What is temporary adjustment of instrument? |  |  |
|  | Define centering of instrument. |  |  |
|  | Define leveling of instrument. |  |  |
|  | What is prism? |  |  |
|  | Name the types of prism. |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________