Instruction Sheet for the Candidate

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Measure Angles by Total Station |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the giventime frame (for practical demonstration & assessment): - Measure horizontal angles by Total Station - Measure vertical angles by Total Station - Measure the height of an object with total station by using REM - Measure the distance between different station points with the help of occupied point (MLM) by using measure data or by using coordinate data |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Identify station points - Set total station - Fix ranging rod on point - Sight the point - Set 0˚-0’-0” on total station for horizontal angle - Set 0˚-0’-0” on total station for vertical angle - Sight the prism - Record measurement for vertical angle - Set percentage for Angle - Place the mirror target on the ground at the corner of the building - Record the mirror target height from the range pole scale - Set target height by setting target height - Lock position with horizontal and vertical clamps - Take observation using REM menu - Unlock the vertical clamp and sight in the top of the building corner - Re-lock vertical clamp - Record height BY using REM - Place the mirror on reference point - Measure the reference point - Record results |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Surveying |
| Competency Standards | Measure Angles by Total Station |
| Assessment Task | - Measure horizontal angles by Total Station - Measure vertical angles by Total Station - Measure the height of an object with total station by using REM - Measure the distance between different station points with the help of occupied point (MLM) by using measure data or by using coordinate data |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify station points |  |  |
| - Set total station on intermediate station point |  |  |
| - Fix ranging rod on the first point |  |  |
| - Fix ranging rod on the other point |  |  |
| - Sight first point |  |  |
| - Set 0˚-0’-0” on total station for horizontal angle |  |  |
| - Sight the other point |  |  |
| - Record the measurement |  |  |
| - Repeat by changing face |  |  |
| - Select the station point |  |  |
| - Set total station on station point |  |  |
| - Sight the first point |  |  |
| - Set 0˚-0’-0” on total station for vertical angle |  |  |
| - Sight the prism |  |  |
| - Record measurement for vertical angle |  |  |
| - Set percentage for Angle |  |  |
| - Set up total station on station point |  |  |
| - Place the mirror target on the ground at the corner of the building |  |  |
| - Record the mirror target height from the range pole scale |  |  |
| - Set target height by setting target height |  |  |
| - Lock position with horizontal and vertical clamps |  |  |
| - Take observation using REM menu |  |  |
| - Unlock the vertical clamp and sight in the top of the building corner |  |  |
| - Re-lock vertical clamp |  |  |
| - Record height BY using REM |  |  |
| - Set up total station on station point |  |  |
| - Place the mirror on reference point |  |  |
| - Measure the reference point |  |  |
| - Aim at the first target point |  |  |
| - Measure the target point |  |  |
| - Aim at the second target point |  |  |
| - Measure the second target point |  |  |
| - Record results |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Measure Angles by Total Station |
| Candidate Details | Name:______________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Measure horizontal angles by Total Station - Measure vertical angles by Total Station - Measure the height of an object with total station by using REM - Measure the distance between different station points with the help of occupied point (MLM) by using measure data or by using coordinate data |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identify station points |  |  |  |  |  |
|  | Set total station on intermediate station point |  |  |  |  |  |
|  | Fix ranging rod on the first point |  |  |  |  |  |
|  | Fix ranging rod on the other point |  |  |  |  |  |
|  | Sight first point |  |  |  |  |  |
|  | Set 0˚-0’-0” on total station for horizontal angle |  |  |  |  |  |
|  | Sight the other point |  |  |  |  |  |
|  | Record the measurement |  |  |  |  |  |
|  | Repeat by changing face |  |  |  |  |  |
|  | Select the station point |  |  |  |  |  |
|  | Set total station on station point |  |  |  |  |  |
|  | Sight the first point |  |  |  |  |  |
|  | Set 0˚-0’-0” on total station for vertical angle |  |  |  |  |  |
|  | Sight the prism |  |  |  |  |  |
|  | Record measurement for vertical angle |  |  |  |  |  |
|  | Set percentage for Angle |  |  |  |  |  |
|  | Set up total station on station point |  |  |  |  |  |
|  | Place the mirror target on the ground at the corner of the building |  |  |  |  |  |
|  | Record the mirror target height from the range pole scale |  |  |  |  |  |
|  | Set target height by setting target height |  |  |  |  |  |
|  | Lock position with horizontal and vertical clamps |  |  |  |  |  |
|  | Take observation using REM menu |  |  |  |  |  |
|  | Unlock the vertical clamp and sight in the top of the building corner |  |  |  |  |  |
|  | Re-lock vertical clamp |  |  |  |  |  |
|  | Record height BY using REM |  |  |  |  |  |
|  | Set up total station on station point |  |  |  |  |  |
|  | Place the mirror on reference point |  |  |  |  |  |
|  | Measure the reference point |  |  |  |  |  |
|  | Aim at the first target point |  |  |  |  |  |
|  | Measure the target point |  |  |  |  |  |
|  | Aim at the second target point |  |  |  |  |  |
|  | Measure the second target point |  |  |  |  |  |
|  | Record results |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Measure Angles by Total Station |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Which mode is used for angle measurements? |  |  |
|  | What is line of collimation? |  |  |
|  | Name the methods to measure horizontal angles. |  |  |
|  | Define repetition method for measurement of angle. |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________