Instruction Sheet for the Candidate

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Perform Long & Cross Sectioning |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name____________________________________________________ Registration/Roll Number_______________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the giventime frame (for practical demonstration & assessment): - Perform Cross Sectioning - Draw Cross Sections - Perform Long Sectioning - Draw Long Sections |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Adjust Level &Locate temporary bench mark. - Take readings every 25 feet interval of existing track (Transversely) - Observe first reading on BM and the center line - Observe staff reading on specific interval starting from center line to right & left sides and record in the field book - Continue staff readings on every RD of 25 ft with same pattern (Left & Right) of given line - Calculate Reduced Levels & Recheck by formula - Take the all readings with offset to plot the cross section - Design the proposed levels on each RD - Calculate mean of existing levels of each RD - Calculate height by subtracting the mean existing level from proposed levels - Calculate earth work by measuring the cross sectional area of each RD - Draw cross section - Establish stations every 30m /100 ft. interval of existing track (Longitudinally) - Observe reading on specific interval (30m /100 ft) and record in the field book - Take all readings with offset to plot the Long section - Draw Long section on graph paper / Sheet |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Surveying |
| Competency Standards | Perform Long & Cross Sectioning |
| Assessment Task | - Perform Cross Sectioning - Draw Cross Sections - Perform Long Sectioning - Draw Long Sections |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Adjust Level &Locate temporary bench mark. |  |  |
| - Take readings every 25 feet interval of existing track (Transversely) |  |  |
| - Observe first reading on BM and the center line |  |  |
| - Observe staff reading on specific interval starting from center line to right & left sides and record in the field book |  |  |
| - Continue staff readings on every RD of 25 ft with same pattern (Left & Right) of given line |  |  |
| - Calculate Reduced Levels & Recheck by formula |  |  |
| - Take the all readings with offset to plot the cross section |  |  |
| - Design the proposed levels on each RD |  |  |
| - Calculate mean of existing levels of each RD |  |  |
| - Calculate height by subtracting the mean existing level from proposed levels |  |  |
| - Calculate earth work by measuring the cross sectional area of each RD |  |  |
| - Adjust Level &Locate temporary bench mark. |  |  |
| - Take readings every 25 feet interval of existing track (Transversely) |  |  |
| - Observe first reading on BM and the center line |  |  |
| - Observe staff reading on specific interval starting from center line to right & left sides and record in the field book |  |  |
| - Continue staff readings on every RD of 25 ft with same pattern (Left & Right) of given line |  |  |
| - Calculate Reduced Levels & Recheck by formula |  |  |
| - Take the all readings with offset to plot the cross section |  |  |
| - Design the proposed levels on each RD |  |  |
| - Calculate mean of existing levels of each RD |  |  |
| - Draw cross section |  |  |
| - Calculate height by subtracting the mean existing level from proposed levels |  |  |
| - Locate temporary bench mark. |  |  |
| - Establish stations every 30m /100 ft. interval of existing track (Longitudinally) |  |  |
| - Observe first reading on temporary bench mark. |  |  |
| - Observe reading on specific interval (30m /100 ft) and record in the field book |  |  |
| - Calculate Reduced Levels at the center line readings to plot the Longitudinal section |  |  |
| - Design proposed levels on each RD |  |  |
| - Calculate mean of existing levels of each RD |  |  |
| - Calculate height by subtracting the mean existing level from proposed levels |  |  |
| - Adjust Level &Locate temporary bench mark. |  |  |
| - Take readings every 100 feet interval of existing track (Longitudinally) |  |  |
| - Observe first reading on BM and the center line |  |  |
| - Observe staff reading on specific interval starting from center line to right & left sides and record in the field book |  |  |
| - Continue staff readings on every RD of 100 feet on given line |  |  |
| - Calculate Reduced Levels & Recheck by formula |  |  |
| - Take all readings with offset to plot the Long section |  |  |
| - Design the proposed levels on each RD |  |  |
| - Calculate mean of existing levels of each RD |  |  |
| - Draw Long section on graph paper / Sheet |  |  |
| - Calculate height by subtracting the mean existing level from proposed levels |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Perform Long & Cross Sectioning |
| Candidate Details | Name:______________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Perform Cross Sectioning - Draw Cross Sections - Perform Long Sectioning - Draw Long Sections |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Adjusted Level &Locate temporary bench mark. |  |  |  |  |  |
|  | Took readings every 25 feet interval of existing track (Transversely) |  |  |  |  |  |
|  | Observed first reading on BM and the center line |  |  |  |  |  |
|  | Observed staff reading on specific interval starting from center line to right & left sides and record in the field book |  |  |  |  |  |
|  | Continued staff readings on every RD of 25 ft with same pattern (Left & Right) of given line |  |  |  |  |  |
|  | Calculated Reduced Levels & Recheck by formula |  |  |  |  |  |
|  | Took the all readings with offset to plot the cross section |  |  |  |  |  |
|  | Designed the proposed levels on each RD |  |  |  |  |  |
|  | Calculated mean of existing levels of each RD |  |  |  |  |  |
|  | Calculated height by subtracting the mean existing level from proposed levels |  |  |  |  |  |
|  | Calculated earth work by measuring the cross sectional area of each RD |  |  |  |  |  |
|  | Adjusted Level &Locate temporary bench mark. |  |  |  |  |  |
|  | Took readings every 25 feet interval of existing track (Transversely) |  |  |  |  |  |
|  | Observed first reading on BM and the center line |  |  |  |  |  |
|  | Observed staff reading on specific interval starting from center line to right & left sides and record in the field book |  |  |  |  |  |
|  | Continued staff readings on every RD of 25 ft with same pattern (Left & Right) of given line |  |  |  |  |  |
|  | Calculated Reduced Levels & Recheck by formula |  |  |  |  |  |
|  | Took the all readings with offset to plot the cross section |  |  |  |  |  |
|  | Designed the proposed levels on each RD |  |  |  |  |  |
|  | Calculated mean of existing levels of each RD |  |  |  |  |  |
|  | Drew cross section |  |  |  |  |  |
|  | Calculated height by subtracting the mean existing level from proposed levels |  |  |  |  |  |
|  | Located temporary bench mark. |  |  |  |  |  |
|  | Established stations every 30m /100 ft. interval of existing track (Longitudinally) |  |  |  |  |  |
|  | Observed first reading on temporary bench mark. |  |  |  |  |  |
|  | Observed reading on specific interval (30m /100 ft) and record in the field book |  |  |  |  |  |
|  | Calculated Reduced Levels at the canter line readings to plot the Longitudinal section |  |  |  |  |  |
|  | Designed proposed levels on each RD |  |  |  |  |  |
|  | Calculated mean of existing levels of each RD |  |  |  |  |  |
|  | Calculated height by subtracting the mean existing level from proposed levels |  |  |  |  |  |
|  | Adjusted Level &Locate temporary bench mark. |  |  |  |  |  |
|  | Took readings every 100 feet interval of existing track (Longitudinally) |  |  |  |  |  |
|  | Observed first reading on BM and the center line |  |  |  |  |  |
|  | Observed staff reading on specific interval starting from center line to right & left sides and record in the field book |  |  |  |  |  |
|  | Continued staff readings on every RD of 100 feet on given line |  |  |  |  |  |
|  | Calculated Reduced Levels & Recheck by formula |  |  |  |  |  |
|  | Took all readings with offset to plot the Long section |  |  |  |  |  |
|  | Designed the proposed levels on each RD |  |  |  |  |  |
|  | Calculated mean of existing levels of each RD |  |  |  |  |  |
|  | Drew Long section on graph paper / Sheet |  |  |  |  |  |
|  | Calculated height by subtracting the mean existing level from proposed levels |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Surveying |
| --- | --- |
| Competency Standard(s) | Perform Long & Cross Sectioning |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define profile levelling. |  |  |
|  | Define cross section. |  |  |
|  | Define longitudinal section. |  |  |
|  | Define chainage. |  |  |
|  | Differentiate between design level and reduced level. |  |  |
|  | Define camber. |  |  |
|  | Define gradient. |  |  |
|  | Define RD. |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________