| Instruction Sheet for the Candidate |  |
| --- | --- |
| Qualification | Quantity Surveying |
| Competency Standard(s) | Produce of rough cost estimate of Sewerage scheme. |
| Purpose of Assessment | Formative |
| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): - Prepare Rough cost Estimate of sewer lines - Prepare Rough cost Estimate of manhole - Prepare Rough cost Estimate of Septic Tank. - Prepare hydraulic statement for sewerage scheme. |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Take out lengths of each dia pipe. - Prepare analysis of rate for sewer line. - Calculate amount of sewer line. - Calculate quantities of excavation & foundation concrete. - Calculate the quantities of brick work & RCC work. - Calculate the quantities of flooring concrete & plaster. - Calculate the quantities of iron steps, manhole cover. - Prepare abstract of cost. - Calculate quantities of excavation & foundation concrete. - Calculate the quantities of brick work & RCC work. - Calculate the quantities of flooring concrete & plaster. - Calculate the quantities of iron steps, manhole cover vent pipe, inlet and outlet pipes. - Prepare abstract of cost. - Note the name of sewerage pipe lines and their lengths. - Calculate population based Area to be served for each pipe line (own, other & Total). - Calculate the Present Population for sewer line on the basis of Projected Population Density, area. - Calculate maximum flow for sewer line by considering average daily flow and peak ratio. - Calculate the total discharge for sewer line (total=”by adding max. flow, infiltration & storm water allowance”, min “by considering present population, peak ration & infiltration”, and max “by adding industrial waste (self & other) & total discharge”). - Calculate the velocity of sewage at full flow by considering dia of sewer and its relevant slope. - Calculate the ratio to velocity for min discharge and velocity at min flow by multiplying ratio to velocity for min discharge with velocity at full flow. - Calculate the ratio to velocity for max discharge and velocity at max flow by multiplying ratio to velocity for max discharge with velocity at full flow. - Calculate the fall in line due to gradient, due to change in dia of pipe, and due to drop in line. - Note the ground level & invert level at upper and lower end of line. - Calculate the bedding + wall thickness of line according to dia of line. - Calculate bedding level for upper & lower ends of line. - Calculate depth of cutting for upper & lower ends of line and the average depth of cutting. - Calculate the depth of man holes and number of man holes for the line. - Calculate the mean depth of man holes for each dia sewer line. |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Quantity Surveying |
| Competency Standards | Produce of rough cost estimate of Sewerage scheme. |
| Assessment Task | Prepare Rough cost Estimate of sewer lines |
|  | Prepare Rough cost Estimate of manhole |
|  | Prepare Rough cost Estimate of Septic Tank. |
|  | Prepare hydraulic statement for sewerage scheme. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Take out lengths of each dia pipe. |  |  |
| - Prepare analysis of rate for sewer line. |  |  |
| - Calculate amount of sewer line. |  |  |
| - Calculate quantities of excavation & foundation concrete. |  |  |
| - Calculate the quantities of brick work & RCC work. |  |  |
| - Calculate the quantities of flooring concrete & plaster. |  |  |
| - Calculate the quantities of iron steps, manhole cover. |  |  |
| - Prepare abstract of cost. |  |  |
| - Calculate quantities of excavation & foundation concrete. |  |  |
| - Calculate the quantities of brick work & RCC work. |  |  |
| - Calculate the quantities of flooring concrete & plaster. |  |  |
| - Calculate the quantities of iron steps, manhole cover vent pipe, inlet and outlet pipes. |  |  |
| - Prepare abstract of cost. |  |  |
| - Note the name of sewerage pipe lines and their lengths. |  |  |
| - Calculate population based Area to be served for each pipe line (own, other & Total). |  |  |
| - Calculate the Present Population for sewer line on the basis of Projected Population Density, area. |  |  |
| - Calculate maximum flow for sewer line by considering average daily flow and peak ratio. |  |  |
| - Calculate the total discharge for sewer line (total=”by adding max. flow, infiltration & storm water allowance”, min “by considering present population, peak ration & infiltration”, and max “by adding industrial waste (self & other) & total discharge”). |  |  |
| - Calculate the velocity of sewage at full flow by considering dia of sewer and its relevant slope. |  |  |
| - Calculate the ratio to velocity for min discharge and velocity at min flow by multiplying ratio to velocity for min discharge with velocity at full flow. |  |  |
| - Calculate the ratio to velocity for max discharge and velocity at max flow by multiplying ratio to velocity for max discharge with velocity at full flow. |  |  |
| - Calculate the fall in line due to gradient, due to change in dia of pipe, and due to drop in line. |  |  |
| - Note the ground level & invert level at upper and lower end of line. |  |  |
| - Calculate the bedding + wall thickness of line according to dia of line. |  |  |
| - Calculate bedding level for upper & lower ends of line. |  |  |
| - Calculate depth of cutting for upper & lower ends of line and the average depth of cutting. |  |  |
| - Calculate the depth of man holes and number of man holes for the line. |  |  |
| - Calculate the mean depth of man holes for each dia sewer line. |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Quantity Surveying |
| --- | --- |
| Competency Standard(s) | Produce of rough cost estimate of Sewerage scheme. |
| Candidate Details | Name:______________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare Rough cost Estimate of sewer lines - Prepare Rough cost Estimate of manhole - Prepare Rough cost Estimate of Septic Tank. - Prepare hydraulic statement for sewerage scheme. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Took out lengths of each dia pipe. |  |  |  |  |  |
|  | Prepared analysis of rate for sewer line. |  |  |  |  |  |
|  | Calculated amount of sewer line. |  |  |  |  |  |
|  | Calculated quantities of excavation & foundation concrete. |  |  |  |  |  |
|  | Calculated the quantities of brick work & RCC work. |  |  |  |  |  |
|  | Calculated the quantities of flooring concrete & plaster. |  |  |  |  |  |
|  | Calculated the quantities of iron steps, manhole cover. |  |  |  |  |  |
|  | Prepared abstract of cost. |  |  |  |  |  |
|  | Calculated quantities of excavation & foundation concrete. |  |  |  |  |  |
|  | Calculated the quantities of brick work & RCC work. |  |  |  |  |  |
|  | Calculated the quantities of flooring concrete & plaster. |  |  |  |  |  |
|  | Calculated the quantities of iron steps, manhole cover vent pipe, inlet and outlet pipes. |  |  |  |  |  |
|  | Prepared abstract of cost. |  |  |  |  |  |
|  | Note the name of sewerage pipe lines and their lengths. |  |  |  |  |  |
|  | Calculated population based Area to be served for each pipe line (own, other & Total). |  |  |  |  |  |
|  | Calculated the Present Population for sewer line on the basis of Projected Population Density, area. |  |  |  |  |  |
|  | Calculated maximum flow for sewer line by considering average daily flow and peak ratio. |  |  |  |  |  |
|  | Calculated the total discharge for sewer line (total=”by adding max. flow, infiltration & storm water allowance”, min “by considering present population, peak ration & infiltration”, and max “by adding industrial waste (self & other) & total discharge”). |  |  |  |  |  |
|  | Calculated the velocity of sewage at full flow by considering dia of sewer and its relevant slope. |  |  |  |  |  |
|  | Calculated the ratio to velocity for min discharge and velocity at min flow by multiplying ratio to velocity for min discharge with velocity at full flow. |  |  |  |  |  |
|  | Calculated the ratio to velocity for max discharge and velocity at max flow by multiplying ratio to velocity for max discharge with velocity at full flow. |  |  |  |  |  |
|  | Calculated the fall in line due to gradient, due to change in dia of pipe, and due to drop in line. |  |  |  |  |  |
|  | Noted the ground level & invert level at upper and lower end of line. |  |  |  |  |  |
|  | Calculated the bedding + wall thickness of line according to dia of line. |  |  |  |  |  |
|  | Calculated bedding level for upper & lower ends of line. |  |  |  |  |  |
|  | Calculated depth of cutting for upper & lower ends of line and the average depth of cutting. |  |  |  |  |  |
|  | Calculated the depth of man holes and number of man holes for the line. |  |  |  |  |  |
|  | Calculated the mean depth of man holes for each dia sewer line. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Quantity Surveying |
| --- | --- |
| Competency Standard(s) | Produce of rough cost estimate of Sewerage scheme. |
| Candidate Details | Name:_________________________________________________________ Registration/Roll Number: ______Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ___________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define sewerage line? |  |  |
|  | Enlist components of sewerage scheme? |  |  |
|  | Draw table of hydraulic Statement? |  |  |
|  | List all items of works for sewer line and their measurement units? |  |  |
|  | List all items of work for a manhole and their measurement units? |  |  |
|  | List all items of work for a septic tank and their measurement units? |  |  |
|  | State the elements of hydraulic statement for water sewerage scheme? |  |  |
|  | How present population for a sewer line is calculated? |  |  |
|  | What is meant by full discharge, minimum discharge & maximum discharge? |  |  |
|  | How full discharge, minimum discharge & maximum discharge for sewer line are calculated? |  |  |
|  | How ratio to velocity for minimum discharge & maximum discharge for sewer line are calculated? |  |  |
|  | How velocity for full discharge, minimum discharge & maximum discharge for sewer line are calculated? |  |  |
|  | How fall due to gradient, change of dia and due to drop for sewer line are calculated? |  |  |
|  | What is meant by peak discharge & flow? |  |  |
|  | What is the value of velocity for flow in sewer line? |  |  |
|  | State the formula to calculate the head losses in pipes? |  |  |
|  | Why ground and invert, bedding levels are calculated? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ____________________