Instruction Sheet for the Candidate

| Qualification | Plumbing/Public Health & water supply |
| --- | --- |
| Competency Standard(s) | 9.8.6 Install plumbing fixtures |
| Purpose of Assessment | Formative |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): 1. Install Wash Hand Basin 2. Install English water closet 3. Install Flushing Cistern 4. Install Urinal 5. Install Sink 6. Install Bath Tub |
| Time: | During a practical assessment, under observation by an assessor, you are required to 1. Identify safety requirements for installation 2. Identify tools for work 3. Identify Fixtures 4. Perform measurements 5. Mark out Installation point 6. Turn the water supply off of the concerned section 7. Mark and drill pilot holes for the floor fixing screws. 8. Mark, drill and plug the wall holes for basin fixing screw 9. Install tee stop cock at the end of water supply pipe (hot and cold) 10. Connect the flexible rubber connection pipes with pillar cocks or mixture 11. Fit the compline in the hole of basin. Fit floor trap under the waste pipe and adjust waste pipe with it. 12. Re-position the pedestal and screw it into proper place. 13. Position the basin and check its level then screw the basin to wall 14. Inject silicon sealant for sealing. 15. Tighten the pedestal screws. 16. Turn on the water supply and check for leaks.17. Mark out Installation point 17. Mark out Installation point 18. Compact and Level the floor at the place of installation of water closet. 19. Place P-trap at compacted and levelled surface. 20. Place the W.C pot on the trap in leveled surface and provide support to it. |
| Minimum Evidence Required |  |

| Minimum Evidence Required | 21. Fill the P-trap with cement and cloth bandage soaked in cement solution. 22. Connect soil pipe to P-trap 23. Connect tee with soil pipe outside the toilet wall. 24. Joint the vent pipe with small end of tee. 25. Connect larger end of soil pipe to septic tank. 26. Attach flush pipe in to the pot hole. 27. Drill holes at the required place and height for flushing cistern 28. Install flushing with screws and joint it with flush pipe. 29. Fill the earth to required level. 30. Finish conglomerate floor or tile flooring 31. Mark out Installation point 32. Measure height for flushing cistern 33. Drill holes at the required place and height for flushing cistern. 34. Install flushing with screws and joint it with flush pipe 35. Fix flushing pipe 36. Connect inlet pipe with water supply 37. Install adequate support for wall hangers and install wall hangers if needed. 38. Drill the holes at two feet height 39. Determine the position for waste pipe nipple 40. Apply thread sealant tape. 41. Install female collar to the waste pipe. 42. Place the gasket on the female collar with the beveled surface facing away from the collar. 43. Position the urinal on the wall hangers and screwed them 44. Secure the urinal bottle trap and waste pipe to female collar using the screws. 45. Install the flush valve according to manufacturer instructions. 46. Connect the water supply and complete waste pipe connection. 47. Turn on water supply. 48. Flush and check the entire installation for leakage 49. Measure height for sink 50. Install adequate support for wall hangers and install wall hangers if needed 51. Fix sink fat measured height with drain pipe 52. Connect inlet pipe with water supply 53. Perform final quality inspection 54. Check Level 55. Conduct Operational Testing 56. Set and level Bath Tub at designated place 57. Install a ledger board for supporting tub 58. Secure the tub to studs according to manufacturer instructions. 59. Attach drain to tub 60. Connect the overflow drain 61. Connect the drain pipe with P-trap 62. Measure Height for bath tub 63. Fix Bath Tub with drain pipe 64. Connect inlet pipe with water supply 65. Perform leakage test 66. Perform Safety Precaution 67. Use PPEs |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Plumbing/Public Health & water supply |
| Competency Standards | 9.8.6 Install plumbing fixtures |
| Assessment Task | - Install Wash Hand Basin - Install English water closet - Install Flushing Cistern - Install Urinal - Install Sink - Install Bath Tub |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify safety requirements for installation |  |  |
| - Identify tools for work |  |  |
| - Identify Fixtures |  |  |
| - Perform measurements |  |  |
| - Mark out Installation point |  |  |
| - Turn the water supply off of the concerned section |  |  |
| - Mark and drill pilot holes for the floor fixing screws. |  |  |
| - Mark, drill and plug the wall holes for basin fixing screw |  |  |
| - Install tee stop cock at the end of water supply pipe (hot and cold) |  |  |
| - Connect the flexible rubber connection pipes with pillar cocks or mixture |  |  |
| - Fit the compline in the hole of basin. Fit floor trap under the waste pipe and adjust waste pipe with it. |  |  |
| - Re-position the pedestal and screw it into proper place. |  |  |
| - Position the basin and check its level then screw the basin to wall |  |  |
| - Inject silicon sealant for sealing. |  |  |
| - Tighten the pedestal screws. |  |  |
| - Turn on the water supply and check for leaks. |  |  |
| - Mark out Installation point |  |  |
| - Compact and Level the floor at the place of installation of water closet. |  |  |
| - Place P-trap at compacted and levelled surface. |  |  |
| - Place the W.C pot on the trap in leveled surface and provide support to it. |  |  |
| - Fill the P-trap with cement and cloth bandage soaked in cement solution. |  |  |
| - Connect soil pipe to P-trap |  |  |
| - Connect tee with soil pipe outside the toilet wall. |  |  |
| - Joint the vent pipe with small end of tee. |  |  |
| - Connect larger end of soil pipe to septic tank. |  |  |
| - Attach flush pipe in to the pot hole. |  |  |
| - Drill holes at the required place and height for flushing cistern |  |  |
| - Install flushing with screws and joint it with flush pipe. |  |  |
| - Fill the earth to required level. |  |  |
| - Finish conglomerate floor or tile flooring |  |  |
| - Mark out Installation point |  |  |
| - Measure height for flushing cistern |  |  |
| - Drill holes at the required place and height for flushing cistern. |  |  |
| - Install flushing with screws and joint it with flush pipe |  |  |
| - Fix flushing pipe |  |  |
| - Connect inlet pipe with water supply |  |  |
| - Perform final quality inspection |  |  |
| - Check Level |  |  |
| - Use PPEs |  |  |
| - Mark out Installation point |  |  |
| - Turn the water supply off of the concerned section. |  |  |
| - Install adequate support for wall hangers and install wall hangers if needed. |  |  |
| - Drill the holes at two feet height |  |  |
| - Determine the position for waste pipe nipple |  |  |
| - Apply thread sealant tape. |  |  |
| - Install female collar to the waste pipe. |  |  |
| - Place the gasket on the female collar with the beveled surface facing away from the collar. |  |  |
| - Position the urinal on the wall hangers and screwed them |  |  |
| - Secure the urinal bottle trap and waste pipe to female collar using the screws. |  |  |
| - Install the flush valve according to manufacturer instructions. |  |  |
| - Connect the water supply and complete waste pipe connection. |  |  |
| - Turn on water supply. |  |  |
| - Flush and check the entire installation for leakage |  |  |
| - Mark out Installation point |  |  |
| - Measure height for sink |  |  |
| - Install adequate support for wall hangers and install wall hangers if needed |  |  |
| - Fix sink fat measured height with drain pipe |  |  |
| - Connect inlet pipe with water supply |  |  |
| - Perform final quality inspection |  |  |
| - Check Level |  |  |
| - Conduct Operational Testing |  |  |
| - Use PPEs |  |  |
| - Mark out Installation point |  |  |
| - Set and level Bath Tub at designated place |  |  |
| - Install a ledger board for supporting tub |  |  |
| - Secure the tub to studs according to manufacturer instructions. |  |  |
| - Attach drain to tub |  |  |
| - Connect the overflow drain |  |  |
| - Connect the drain pipe with P-trap |  |  |
| - Measure Height for bath tub |  |  |
| - Fix Bath Tub with drain pipe |  |  |
| - Connect inlet pipe with water supply |  |  |
| - Perform leakage test |  |  |
| - Perform Safety Precaution |  |  |
| - Use PPEs |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Plumbing/Public Health & water supply |
| --- | --- |
| Competency Standard(s) | 9.8.6 Install plumbing fixtures |
| Candidate Details | Name:______________________________________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Install Wash Hand Basin - Install English water closet - Install Flushing Cistern - Install Urinal - Install Sink - Install Bath Tub |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identified safety requirements for installation |  |  |  |  |  |
|  | Identified tools for work |  |  |  |  |  |
|  | Identified Fixtures |  |  |  |  |  |
|  | Performed measurements |  |  |  |  |  |
|  | Marked out Installation point |  |  |  |  |  |
|  | Turned the water supply off of the concerned section. |  |  |  |  |  |
|  | Marked and drilled pilot holes for the floor fixing screws. |  |  |  |  |  |
|  | Marked, drilled and plugged the wall holes for basin fixing screw |  |  |  |  |  |
|  | Installed tee stop cock at the end of water supply pipe (hot and cold) |  |  |  |  |  |
|  | Connected the flexible rubber connection pipes with pillar cocks or mixture |  |  |  |  |  |
|  | Fitted the couplene in the hole of basin. Fit floor trap under the waste pipe and adjust waste pipe with it. |  |  |  |  |  |
|  | Re-positioned the pedestal and screw it into proper place. |  |  |  |  |  |
|  | Positioned the basin and check its level then screw the basin to wall. |  |  |  |  |  |
|  | Injected silicon sealant for sealing. |  |  |  |  |  |
|  | Tighten the pedestal screws. |  |  |  |  |  |
|  | Turned on the water supply and checked for leaks |  |  |  |  |  |
|  | Mark out Installation point |  |  |  |  |  |
|  | Compacted and Level the floor at the place of installation of water closet. |  |  |  |  |  |
|  | Placed P-trap at compacted and levelled surface. |  |  |  |  |  |
|  | Placed the W.C pot on the trap in leveled surface and provide support to it. |  |  |  |  |  |
|  | Filled the P-trap with cement and cloth bandage soaked in cement solution. |  |  |  |  |  |
|  | Connected soil pipe to P-trap |  |  |  |  |  |
|  | Connected tee with soil pipe outside the toilet wall. |  |  |  |  |  |
|  | Jointed the vent pipe with small end of tee. |  |  |  |  |  |
|  | Connected larger end of soil pipe to septic tank. |  |  |  |  |  |
|  | Attached flush pipe in to the pot hole. |  |  |  |  |  |
|  | Drilled holes at the required place and height for flushing cistern. |  |  |  |  |  |
|  | Installed flushing with screws and joint it with flush pipe. |  |  |  |  |  |
|  | Filled the earth to required level. |  |  |  |  |  |
|  | Finished conglomerate floor or tile flooring |  |  |  |  |  |
|  | Marked out Installation point |  |  |  |  |  |
|  | Measured height for flushing cistern |  |  |  |  |  |
|  | Drilled holes at the required place and height for flushing cistern. |  |  |  |  |  |
|  | Installed flushing with screws and joint it with flush pipe |  |  |  |  |  |
|  | Fixed flushing pipe |  |  |  |  |  |
|  | Connected inlet pipe with water supply |  |  |  |  |  |
|  | Performed final quality inspection |  |  |  |  |  |
|  | Checked Level |  |  |  |  |  |
|  | Used PPEs |  |  |  |  |  |
|  | Marked out Installation point |  |  |  |  |  |
|  | Turned the water supply off of the concerned section. |  |  |  |  |  |
|  | Installed adequate support for wall hangers and installed wall hangers if needed. |  |  |  |  |  |
|  | Drilled the holes at two feet height |  |  |  |  |  |
|  | Determined the position for waste pipe nipple |  |  |  |  |  |
|  | Applied thread sealant tape. |  |  |  |  |  |
|  | Installed female collar to the waste pipe. |  |  |  |  |  |
|  | Placed the gasket on the female collar with the beveled surface facing away from the collar. |  |  |  |  |  |
|  | Positioned the urinal on the wall hangers and screwed them. |  |  |  |  |  |
|  | Secured the urinal bottle trap and waste pipe to female collar using the screws. |  |  |  |  |  |
|  | Installed the flush valve according to manufacturer instructions. |  |  |  |  |  |
|  | Connected the water supply and complete waste pipe connection. |  |  |  |  |  |
|  | Turned on water supply. |  |  |  |  |  |
|  | Flushed and checked the entire installation for leakage |  |  |  |  |  |
|  | Marked out Installation point |  |  |  |  |  |
|  | Measured height for sink |  |  |  |  |  |
|  | Installed adequate support for wall hangers and installed wall hangers if needed. |  |  |  |  |  |
|  | Fixed sink fat measured height with drain pipe |  |  |  |  |  |
|  | Connected inlet pipe with water supply |  |  |  |  |  |
|  | Performed final quality inspection |  |  |  |  |  |
|  | Checked Level |  |  |  |  |  |
|  | Conducted Operational Testing |  |  |  |  |  |
|  | Used PPEs |  |  |  |  |  |
|  | Marked out Installation point |  |  |  |  |  |
|  | Measured Height for bath tub |  |  |  |  |  |
|  | Set and levelled Bath Tub at designated place |  |  |  |  |  |
|  | Installed a ledger board for supporting tub |  |  |  |  |  |
|  | Installed a ledger board for supporting tub |  |  |  |  |  |
|  | Attached drain to tub |  |  |  |  |  |
|  | Connected the overflow drain |  |  |  |  |  |
|  | Connected the drain pipe with P-trap |  |  |  |  |  |
|  | Fixed Bath Tub with drain pipe |  |  |  |  |  |
|  | Connected inlet pipe with water supply |  |  |  |  |  |
|  | Performed leakage test |  |  |  |  |  |
|  | Performed Safety Precaution |  |  |  |  |  |
|  | Used PPEs |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Plumbing/Public Health & water supply |
| --- | --- |
| Competency Standard(s) | Install plumbing fixtures |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define water closet |  |  |
|  | Name the types of water closet |  |  |
|  | Define Flushing Cistern |  |  |
|  | Name the types of flushing cistern |  |  |
|  | State levelling of WC |  |  |
|  | State fixing of WC |  |  |
|  | Define WC with its types |  |  |
|  | Define wash hand basin Pedestal type |  |  |
|  | Define trap fitting |  |  |
|  | Name the tool’s and equipment’s used for installing flushing cistern, WC and hand basin |  |  |

Feedback to the Candidate

Candidate’s Signature___________________ Assessor’s Signature ____________________