Assessors Judgment Guide

| Qualification | R.C.C & Concrete Technology |
| --- | --- |
| Competency Standard(s) | Destructive Tests on Hardened Concrete |
| Candidate Details | Name:____________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Calculate compressive strength of concrete (1:2:4) using cubes and cylinders - Perform modules of rupture test on standard size beams. - Perform Split cylinder test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Measured the cement and sand by weight. |  |  |  |  |  |
|  | Mixed the material thoroughly. |  |  |  |  |  |
|  | Added specified %age of water |  |  |  |  |  |
|  | Filled mould in specific manner. |  |  |  |  |  |
|  | Placed the sample in atmosphere for specific time, then dip in fresh water. |  |  |  |  |  |
|  | Tested the prepared samples of 3,7,14, and 28 days |  |  |  |  |  |
|  | Recorded readings. |  |  |  |  |  |
|  | Plotted the graph. |  |  |  |  |  |
|  | Prepared the sample of concrete beam as per specification |  |  |  |  |  |
|  | Placed the specimen on loading points. |  |  |  |  |  |
|  | Applied load at a constant rate. |  |  |  |  |  |
|  | Measured the cross section of the tested specimen at each end and at center to calculate average depth and height. |  |  |  |  |  |
|  | Calculated Flexural strength by using formula. |  |  |  |  |  |
|  | Prepared the sample of concrete cylinder as per specifications. |  |  |  |  |  |
|  | Recorded the weight and dimensions of sample. |  |  |  |  |  |
|  | Placed the specimen on compression testing machine With ply wood strips above and below. |  |  |  |  |  |
|  | Applied the continuous load within the range of 0.72 to 1.4 Mpa / Min. |  |  |  |  |  |
|  | Observed the failure pattern |  |  |  |  |  |
|  | Recorded the Flexural strength. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | R.C.C & Concrete Technology |
| --- | --- |
| Competency Standard(s) | Destructive Tests on Hardened Concrete |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Explain the function of admixtures in concrete. |  |  |
|  | Explain the function of plasticizer in concrete. |  |  |
|  | What is meant by segregation of concrete. |  |  |
|  | With increase in the W/C ratio the compressive strength of concrete increases or decreases. |  |  |
|  | What is the ratio of dry and wet concrete mix. |  |  |
|  | Describe the relation factor of compressive strength of cube and Cylinder. |  |  |
|  | Explain the Tensile strength (split cylinder test), and Flexural strength (Beam test), |  |  |
|  | Describe the difference between destructive and non-destructive tests. |  |  |
|  | Why concrete strength is taken mostly at 28 days. |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | R.C.C & Concrete Technology |
| Competency Standards | Destructive Tests on Hardened Concrete |
| Assessment Task | - Calculate compressive strength of concrete (1:2:4) using cubes and cylinders - Perform modules of rupture test on standard size beams. - Perform Split cylinder test |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Measure the cement and sand by weight. |  |  |
| - Mix the material thoroughly. |  |  |
| - Add specified %age of water |  |  |
| - Fill mould in specific manner. |  |  |
| - Place the sample in atmosphere for specific time, then dip in fresh water. |  |  |
| - Test the prepared samples of 3,7,14, and 28 days |  |  |
| - Record readings. |  |  |
| - Plot the graph. |  |  |
| - Prepare the sample of concrete beam as per specification |  |  |
| - Place the specimen on loading points. |  |  |
| - Apply load at a constant rate. |  |  |
| - Measure the cross section of the tested specimen at each end and at center to calculate average depth and height. |  |  |
| - Calculate Flexural strength by using formula. |  |  |
| - Prepare the sample of concrete cylinder as per specifications. |  |  |
| - Record the weight and dimensions of sample. |  |  |
| - Place the specimen on compression testing machine with ply wood strips above and below. |  |  |
| - Apply the continuous load within the range of 0.72 to 1.4 Mpa / Min. |  |  |
| - Observe the failure pattern |  |  |
| - Record the Flexural strength. |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Instruction Sheet for the Candidate

| Qualification | R.C.C & Concrete Technology |
| --- | --- |
| Competency Standard(s) | Destructive Tests on Hardened Concrete |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): - Calculate compressive strength of concrete (1:2:4) using cubes and cylinders - Perform modules of rupture test on standard size beams. - Perform Split cylinder test |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Measure the cement and sand by weight. - Mix the material thoroughly. - Add specified %age of water - Fill mould in specific manner. - Place the sample in atmosphere for specific time, then dip in fresh water. - Test the prepared samples of 3,7,14, and 28 days - Record readings. - Plot the graph. - Prepare the sample of concrete beam as per specification - Place the specimen on loading points. - Apply load at a constant rate. - Measure the cross section of the tested specimen at each end and at center to calculate average depth and height. - Calculate Flexural strength by using formula. - Prepare the sample of concrete cylinder as per specifications. - Record the weight and dimensions of sample. - Place the specimen on compression testing machine With ply wood strips above and below. - Apply the continuous load within the range of 0.72 to 1.4 Mpa / Min. - Observe the failure pattern - Record the Flexural strength. |
| Minimum Evidence Required |  |

Integrated Approach of Assessment

Following are the competencies in the qualification of welding.  You are required to design the assessment task(s) by following integrated approach and clustering of competencies:

- Take measurements

- Cut the material as per specifications

- Perform Welding of the material

- Perform Finishing of the object

- Perform fabrication/designing on object

TASK(S):

__________________________________________________________________________________