Assessors Judgment Guide

| Qualification | Environmental Technology |
| --- | --- |
| Competency Standard(s) | Determine Water Pollution |
| Candidate Details | Name:______________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Determine Dissolved Oxygen in water 2. Determine Turbidity of water 3. Determine Total Suspended Solids in water 4. Find out the PH value of water 5. Find out the concentration of Nitrates in water 6. Determine Biochemical Oxygen Demand for 5 days |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Idenified dissolved oxygen meter |  |  |  |  |  |
|  | Calibrated the equipment for measurement |  |  |  |  |  |
|  | Recorded the result of dissolved oxygen in water |  |  |  |  |  |
|  | Identified turbidity meter |  |  |  |  |  |
|  | Calibrated the equipment for measurement |  |  |  |  |  |
|  | Evaluated the result of turbidity |  |  |  |  |  |
|  | Identified the equipment Total Suspended Solids |  |  |  |  |  |
|  | Recorded volume of water sample |  |  |  |  |  |
|  | Filtered the water sample |  |  |  |  |  |
|  | Dried the filtered material |  |  |  |  |  |
|  | Weighed the material |  |  |  |  |  |
|  | Determined the quantitative TSS |  |  |  |  |  |
|  | Identified litmus paper for pH |  |  |  |  |  |
|  | Wetted the litmus paper with water |  |  |  |  |  |
|  | Compared the color with the standard key of pH |  |  |  |  |  |
|  | Recorded the pH value |  |  |  |  |  |
|  | Identified Spectrophotometer |  |  |  |  |  |
|  | Prepared standard NO3 nitrate solution |  |  |  |  |  |
|  | Recorded absorbance or transmittance against redistilled water set at zero absorbance |  |  |  |  |  |
|  | Obtained absorbance of standard nitrate solution |  |  |  |  |  |
|  | Drawn the relationship between concentration and absorbance |  |  |  |  |  |
|  | Calculated the factor “m” ( slope of line) |  |  |  |  |  |
|  | Calculated the concentration of nitrate from the standard curve or from equation (conc. = m* abs) |  |  |  |  |  |
|  | Determined the amount of sample to be analyzed |  |  |  |  |  |
|  | Placed a clean, calibrated thermometer into the constant temperature chamber |  |  |  |  |  |
|  | Turned on the constant temperature chamber to allow the controlled temperature |  |  |  |  |  |
|  | Turned on the DO instrument |  |  |  |  |  |
|  | Aerated diluted water before adding nutrient solutions |  |  |  |  |  |
|  | Added nutrient solution in diluted water |  |  |  |  |  |
|  | Shaked the container of diluted water for about 1 minute |  |  |  |  |  |
|  | Placed the diluted water in the constant temperature chamber |  |  |  |  |  |
|  | Determined the initial and final (after 5 days ± 4 hours) DO tests of the diluted water |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Environmental Technology |
| --- | --- |
| Competency Standard(s) | Determine Water Pollution |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Name Sources of Water Pollution |  |  |
|  | Define Properties of Sewage (physical, chemical, biological) |  |  |
|  | Name the Tests for Sewage (physical) |  |  |
|  | Name the Tests for Sewage (chemical) |  |  |
|  | Name the Tests for Sewage (biological) |  |  |
|  | Define Sewage treatment |  |  |
|  | Define Biochemistry |  |  |
|  | Name methods of sewage disposal |  |  |
|  | Name Water standards for drinking purpose |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Environmental Technology |
| Competency Standards | Determine Water Pollution |
| Assessment Task | 1. Determine Dissolved Oxygen in water 2. Determine Turbidity of water 3. Determine Total Suspended Solids in water 4. Find out the PH value of water 5. Find out the concentration of Nitrates in water 6. Determine Biochemical Oxygen Demand for 5 days |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Idenify dissolved oxygen meter |  |  |
| - Calibrate the equipment for measurement |  |  |
| - Record the result of dissolved oxygen in water |  |  |
| - Identify turbidity meter |  |  |
| - Calibrate the equipment for measurement |  |  |
| - Evaluate the result of turbidity |  |  |
| - Identify the equipment Total Suspended Solids |  |  |
| - Record volume of water sample |  |  |
| - Filter the water sample |  |  |
| - Dry the filtered material |  |  |
| - Weigh the material |  |  |
| - Determine the quantitative TSS |  |  |
| - Identify litmus paper for pH |  |  |
| - Wet the litmus paper with water |  |  |
| - Compare the color with the standard key for pH |  |  |
| - Record the PH value |  |  |
| - Identify Spectrophotometer |  |  |
| - Prepare standard NO3 nitrate solution |  |  |
| - Record absorbance or transmittance against redistilled water set at zero absorbance |  |  |
| - Obtain absorbance of standard nitrate solution |  |  |
| - Draw the relationship between concentration and absorbance |  |  |
| - Calculate the factor “m” ( slope of line) |  |  |
| - Calculate the concentration of nitrate from the standard curve or from equation (conc. = m* abs) |  |  |
| - Determine the amount of sample to be analyzed |  |  |
| - Place a clean, calibrated thermometer into the constant temperature chamber |  |  |
| - Turn on the constant temperature chamber to allow the controlled temperature |  |  |
| - Turn on the DO instrument |  |  |
| - Aerate diluted water before adding nutrient solutions |  |  |
| - Add to diluted water |  |  |
| - Shake the container of diluted water for about 1 minute |  |  |
| - Place the diluted water in the constant temperature chamber |  |  |
| - Determine the initial and final (after 5 days ± 4 hours) DO tests of the diluted water |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Instruction Sheet for the Candidate

| Qualification | Environmental Technology |
| --- | --- |
| Competency Standard(s) | Determine Water Pollution |
| Purpose of Assessment | Formative |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): 1. Determine Dissolved Oxygen in water 2. Determine Turbidity of water 3. Determine Total Suspended Solids in water 4. Find out the PH value of water 5. Find out the concentration of Nitrates in water 6. Determine Biochemical Oxygen Demand for 5 days |
| Time: 2 hours | During a practical assessment, under observation by an assessor, you are required to - Idenify dissolved oxygen meter - Calibrate the equipment for measurement - Record the result of dissolved oxygen in water - Identify turbidity meter - Calibrate the equipment for measurement - Evaluate the result of turbidity - Identify the equipment Total Suspended Solids - Record volume of water sample - Filter the water sample - Dry the filtered material - Weigh the material - Determine the quantitative TSS - Identify litmus paper for pH - Wet the litmus paper with water - Compare the color with the standard key for pH - Record the PH value - Identify Spectrophotometer - Prepare standard NO3 nitrate solution - Record absorbance or transmittance against redistilled water set at zero absorbance - Obtain absorbance of standard nitrate solution - Draw the relationship between concentration and absorbance - Calculate the factor “m” ( slope of line) - Calculate the concentration of nitrate from the standard curve or from equation (conc. = m* abs) - Determine the amount of sample to be analyzed - Place a clean, calibrated thermometer into the constant temperature chamber - Turn on the constant temperature chamber to allow the controlled temperature - Turn on the DO instrument - Aerate diluted water before adding nutrient solutions - Add to diluted water - Shake the container of diluted water for about 1 minute - Place the diluted water in the constant temperature chamber - Determined the initial and final (after 5 days ± 4 hours) DO tests of the diluted water |
| Minimum Evidence Required |  |