Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level-5 in Civil Technology (Surveying – Digital Surveying) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – Digital Surveying |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): - Measure the area of a traverse by direct field observations using total station and GPS. - Knowledge Assessment |
| Time: 05 Hrs | - During a practical assessment, under observation by an assessor, you are required to “Measure the area of a traverse by direct field observations using total station and GPS”. demonstrating the following criteria: |
| Minimum Evidence Required | - Set up total station on station point - Create a file name using file management - Enter instrument height and prism code after setting zero against point number or name - Measure coordinates of points and save it - Calculate area by using functions of total station - Set up GPS by performing initialization - Acquire the coordinates of the points of boundaries - Calculate area by using functions of GPS |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level-5 in Civil Technology (Surveying – Digital Surveying) |
| Purpose of Assessment | Summative Assessment – Digital Surveying |
| Assessment Task | Compute volume of a pond by contouring using auto level and measure distances by tacheometery. |

I can……………….

| Performance Criteria// | Yes | No |
| --- | --- | --- |
| - Set up total station on station point |  |  |
| - Create a file name using file management |  |  |
| - Enter instrument height and prism code after setting zero against point number or name |  |  |
| - Measure first point and save it |  |  |
| - Turn the total station to second point |  |  |
| - Repeat the steps to the rest of the stations |  |  |
| - Calculate area by using functions of total station |  |  |
| - Set up GPS by performing initialization |  |  |
| - Acquire the coordinates of the points of boundaries |  |  |
| - Calculate area by using functions of GPS |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level-5 in Civil Technology (Surveying – Digital Surveying) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – Digital Surveying |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Compute volume of a pond by contouring using auto level and measure distances by tacheometery. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Set up total station on station point |  |  |  |  |  |
|  | Created a file name using file management |  |  |  |  |  |
|  | Entered instrument height and prism code after setting zero against point number or name |  |  |  |  |  |
|  | Measured first point and save it |  |  |  |  |  |
|  | Turned the total station to second point |  |  |  |  |  |
|  | Repeated the steps to the rest of the stations |  |  |  |  |  |
|  | Calculated area by using functions of total station |  |  |  |  |  |
|  | Set up GPS by performing initialization |  |  |  |  |  |
|  | Acquired the coordinates of the points of boundaries |  |  |  |  |  |
|  | Calculated area by using functions of GPS |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level-5 in Civil Technology (Surveying – Digital Surveying) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – Digital Surveying |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________Signature of the Assessor:_______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is total station? |  |  |
|  | Name the types of total station. |  |  |
|  | Name the parts of total station. |  |  |
|  | What is temporary adjustment of instrument? |  |  |
|  | Define centering of instrument. |  |  |
|  | Define leveling of instrument. |  |  |
|  | What is prism? |  |  |
|  | Name the types of prism. |  |  |
|  | Which mode is used to measure distance? |  |  |
|  | Name the types of correction. |  |  |
|  | What is atmospheric correction? |  |  |
|  | What is correction for prism constant? |  |  |
|  | What is vertical and horizontal angle tilt correction? |  |  |
|  | Differentiate between fine, tracking and coarse mode. |  |  |
|  | What is meant by stack out measurement of distances. |  |  |
|  | What is meant by offset mode of measurement? |  |  |
|  | Which mode is used for angle measurements? |  |  |
|  | What is line of collimation? |  |  |
|  | Name the methods to measure horizontal angles. |  |  |
|  | Define repetition method for measurement of angle. |  |  |
|  | How many types of figures can be formed? |  |  |
|  | What is irregular figure? |  |  |
|  | Define regular figure. |  |  |
|  | Give formula to calculate area of irregular figure. |  |  |
|  | What is meant by GPS? |  |  |
|  | Name the types of GPS. |  |  |
|  | Name the different modes of GPS. |  |  |
|  | Enlist the functions of menu mode of GPS. |  |  |
|  | Enlist the main functions of compass mode of GPS. |  |  |
|  | Enlist the main functions of map mode of GPS. |  |  |
|  | Define initializing of GPS. |  |  |
|  | Enlist the basic functions of location mode of GPS. |  |  |
|  | Define point positioning. |  |  |
|  | Define relative positioning. |  |  |
|  | Define static survey. |  |  |
|  | Define Real Time Kinematic (RTK) GPS positioning. |  |  |
|  | Define space segment. |  |  |
|  | Define satellite geometry. |  |  |
|  | Define Global Navigation Satellite. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________Assessor’s Signature ___________________