Instruction Sheet for the Candidate

| Qualification | 0611ICT09 National Vocational Certificate Level-2 in civil technology ( building materials and construction technique ) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – ( Construction techniques I) |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): - Construct a curved wall (header bond) as given in annexure A of 9” thickness and 1.5’ height, apply damp proof course (1:2:4) with 0.6 W/C at the top and also apply paint through brush. Use Margalla crush, Chenab sand and tap water in construction. - Knowledge Assessment |
| Time: 05 Hrs | During a practical assessment, under observation by an assessor, you are required to “Construct a curved wall (header bond) given in annexure A of 9” thickness and 1.5’ height with 1:6 cement sand mortar, apply damp proof course (1:2:4) with 0.6 w/c at the top and also apply paint through brush. Use Margalla crush, Chenab sand and tap water in construction “ demonstrating the following criteria: |
| Minimum Evidence Required | - Select and check the tools and equipment as required, check for serviceability and fix/report any faults. - Calculate material quantity requirements in accordance with specifications. - Obtain and check materials needed for compliance and prepare it. - Plot planned curve points from job drawings and establish all trammel centres on footing slab. - Plan curve of wall to specified location from trammel or plotted points and mark on the footing slab - Mix Mortar to specifications. - Lay Bricks for first course to planned set out for line and specified bond according to specifications. - Maintain Gauge within standard tolerance at every course level. - Maintain Vertical face alignment - Remove unwanted mortar Brickwork prior to drying to remove unwanted mortar and face is cleaned. - Remove Excess mortar from brick surfaces and cavities are cleaned free of mortar and debris. - Identify Type of flashing and damp proofing material in accordance with and state of structure. - Identify Area of structure for damp proofing from or site inspection and inspect for defects. - Correct Defects and carry out surface preparation of structure requiring damp proofing to manufacturer specifications - Apply Damp proof material with a consistent mortar bed on top and bottom. - Fold Damp proof course to follow shape of surrounding structures. - Prepare Flashing materials for application to surrounding structures in accordance with requirements. - Lay, lap and join Flashing material to follow shape of surrounding structure. - Form Flashings or moisture barrier material and seal around openings. - Install Flashing and damp proof course to project outside of mortar joint on external surface, and outside of brickwork in accordance with specifications. - Protect surfaces not to be painted by drop sheets, masking or removal of objects. - Provide adequate ventilation to maintain a safe environment. - Identify base colour by sample analysis or specification, and correct tint base, paint type and sheen level determined. - Select colourants suitable for the colour match, mix with the base, allow to dry and match against the sample/specification for accuracy. - Mix materials for specified paint finish in accordance with manufacturer recommendations. - Prepare correct amounts of paint material to specified ratio and drying time in accordance with manufacturer specification. - Select brush and/or roller to suit specified surface profile, size of area, type of paint and finish specified. - Apply sealant, undercoat, intermediate coat and finish coat by brush/roller to achieve the required level of opacity, finish, texture and sheen, in accordance with specifications. - Cure Finished paint surface and test using curing method in accordance with manufacturer recommendations. - Clean Work area and disposed of, reused or recycled waste in accordance with work specifications. - Clean Tools plant and equipment ,check for faults and maintained and store in accordance with workplace practices |

Annexure A

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | 0611ICT09 National Vocational Certificate Level-2 in civil technology ( building materials and construction technique ) |
| Purpose of Assessment | Summative Assessment – ( Construction techniques I ) |
| Assessment Task | Construct a curved wall (header bond) given in annexure A of 9” thickness and 1.5’ height with 1:6 cement sand mortar, apply damp proof course (1:2:4) at the top with 0.6 W/C and also apply paint through brush. Use Margalla crush, Chenab sand and tap water in construction. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| Select and check the tools and equipment as required, check for serviceability and fix/report any faults. |  |  |
| Calculate material quantity requirements in accordance with specifications. |  |  |
| Obtain and check materials needed for compliance and prepare it. |  |  |
| Plot planned curve points from job drawings and establish all trammel centres on footing slab. |  |  |
| Plan curve of wall to specified location from trammel or plotted points and mark on the footing slab. |  |  |
| Mix Mortar to specifications. |  |  |
| Lay Bricks for first course to planned set out for line and specified bond according to specifications. |  |  |
| Maintain Gauge within standard tolerance at every course level. |  |  |
| Maintain Vertical face alignment |  |  |
| Remove unwanted mortar Brickwork prior to drying to remove unwanted mortar and face is cleaned. |  |  |
| Remove Excess mortar from brick surfaces and cavities are cleaned free of mortar and debris. |  |  |
| Identify Area of structure for damp proofing from or site inspection and inspect for defects. |  |  |
| Correct Defects and carry out surface preparation of structure requiring damp proofing to manufacturer specifications |  |  |
| Apply Damp proof material with a consistent mortar bed on top and bottom. |  |  |
| Fold Damp proof course to follow shape of surrounding structures |  |  |
| Prepare Flashing materials for application to surrounding structures in accordance with requirements. |  |  |
| Lay, lap and join Flashing material to follow shape of surrounding structure. |  |  |
| Form Flashings or moisture barrier material and seal around openings. |  |  |
| Install Flashing and damp proof course to project outside of mortar joint on external surface, and outside of brickwork in accordance with specifications. |  |  |
| Identify Type of flashing and damp proofing material in accordance with and state of structure. |  |  |
| Protect surfaces not to be painted by drop sheets, masking or removal of objects. |  |  |
| Provide adequate ventilation to maintain a safe environment. |  |  |
| Identify base colour by sample analysis or specification, and correct tint base, paint type and sheen level determined. |  |  |
| Select colourants suitable for the colour match, mix with the base, allow to dry and match against the sample/specification for accuracy. |  |  |
| Mix materials for specified paint finish in accordance with manufacturer recommendations.. |  |  |
| Prepare correct amounts of paint material to specified ratio and drying time in accordance with manufacturer specification. |  |  |
| Select brush to suit specified surface profile, size of area, type of paint and finish specified. |  |  |
| Apply sealant, undercoat, intermediate coat and finish coat by brush/roller to achieve the required level of opacity, finish, texture and sheen, in accordance with specifications. |  |  |
| Cure Finisd paint sface and test using curing method in accordance with manufacturer recommendations. |  |  |
| Clean Work area and disposed of, reused or recycled waste in accordance with work specifications. |  |  |
| Clean Tools plant and equipment ,check for faults and maintained and store in accordance with workplace practices |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | 0611ICT09 National Vocational Certificate Level-2 in civil technology ( building materials and construction technique ) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – ( Construction techniques I ) |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Construct a curved wall(header bond) of 9” thickness and 1.5’ height with 1:6 cement sand mortar, apply damp proof course (1:2:4) with 0.6 W/C at the top and also apply tiles on its face. Use Margalla crush, Chenab sand and tap water in construction. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select and check the tools and equipment as required, check for serviceability and fix/report any faults. |  |  |  |  |  |
|  | Calculate material quantity requirements in accordance with specifications. |  |  |  |  |  |
|  | Obtain and check materials needed for compliance and prepare it. |  |  |  |  |  |
|  | Plot planned curve points from job drawings and establish all trammel centres on footing slab. |  |  |  |  |  |
|  | Plan curve of wall to specified location from trammel or plotted points and mark on the footing slab. |  |  |  |  |  |
|  | Mix Mortar to specifications. |  |  |  |  |  |
|  | Lay Bricks for first course to planned set out for line and specified bond according to specifications. |  |  |  |  |  |
|  | Maintain Gauge within standard tolerance at every course level. |  |  |  |  |  |
|  | Maintain Vertical face alignment |  |  |  |  |  |
|  | Remove unwanted mortar Brickwork prior to drying to remove unwanted mortar and face is cleaned. |  |  |  |  |  |
|  | Remove Excess mortar from brick surfaces and cavities are cleaned free of mortar and debris. |  |  |  |  |  |
|  | Identify Area of structure for damp proofing from or site inspection and inspect for defects. |  |  |  |  |  |
|  | Correct Defects and carry out surface preparation of structure requiring damp proofing to manufacturer specifications |  |  |  |  |  |
|  | Apply Damp proof material with a consistent mortar bed on top and bottom. |  |  |  |  |  |
|  | Fold Damp proof course to follow shape of surrounding structures |  |  |  |  |  |
|  | Prepare Flashing materials for application to surrounding structures in accordance with requirements. |  |  |  |  |  |
|  | Lay, lap and join Flashing material to follow shape of surrounding structure. |  |  |  |  |  |
|  | Form Flashings or moisture barrier material and seal around openings. |  |  |  |  |  |
|  | Install Flashing and damp proof course to project outside of mortar joint on external surface, and outside of brickwork in accordance with specifications. |  |  |  |  |  |
|  | Identify Type of flashing and damp proofing material in accordance with and state of structure. |  |  |  |  |  |
|  | Protect surfaces not to be painted by drop sheets, masking or removal of objects. |  |  |  |  |  |
|  | Provide adequate ventilation to maintain a safe environment. |  |  |  |  |  |
|  | Identify base colour by sample analysis or specification, and correct tint base, paint type and sheen level determined. |  |  |  |  |  |
|  | Select colourants suitable for the colour match, mix with the base, allow to dry and match against the sample/specification for accuracy. |  |  |  |  |  |
|  | Mix materials for specified paint finish in accordance with manufacturer recommendations.. |  |  |  |  |  |
|  | Prepare correct amounts of paint material to specified ratio and drying time in accordance with manufacturer specification. |  |  |  |  |  |
|  | Select brush to suit specified surface profile, size of area, type of paint and finish specified. |  |  |  |  |  |
|  | Apply sealant, undercoat, intermediate coat and finish coat by brush/roller to achieve the required level of opacity, finish, texture and sheen, in accordance with specifications. |  |  |  |  |  |
|  | Cure Finished paint surface and test using curing method in accordance with manufacturer recommendations. |  |  |  |  |  |
|  | Clean Work area and disposed of, reused or recycled waste in accordance with work specifications. |  |  |  |  |  |
|  | Clean Tools plant and equipment ,check for faults and maintained and store in accordance with workplace practices |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | 0611ICT09 National Vocational Certificate Level-2 in civil technology ( building materials and construction technique ) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – ( Construction techniques II ) |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define grouting. |  |  |
|  | Define solvent. |  |  |
|  | How horizontal alignment is checked? |  |  |
|  | Define scaffolding. |  |  |
|  | Define key stone. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________