National Qualification for Certificate Level-5 in Civil Technology

Competency Standards

Competency Standards

Competency Standards

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

Contents

A.	Occupation Packaging Of Civil Technology	9

B.    Qualification Packaging Of Civil Technology	9

1.     Introduction	10

2.     Purpose Of The Qualification	10

3.     Core Competencies Of The Qualification	10

4.	Date Of Validation	22

5.	Codes Of Qualifications	22

6.	Members Of Qualifications Development Committee	23

7.	Entry Requirements	24

8.	Proposed Scheme Of Studies	25

9.	Detail Of Qualifications And Its Competency Standards	28

9.1.	Surveying	28

9.1.1.	Perform Chain Survey	28

9.1.2.	Perform Compass Traversing	30

9.1.3.	Carry out Plane Table Survey	32

9.1.4.	Perform Levelling	34

9.1.5.	Perform Reciprocal Levelling	36

9.1.6.	Perform Long & Cross Sectioning	38

9.1.7.	Perform Tacheometry	42

9.1.8.	Perform Contouring	44

9.1.9.	Perform Hydrographic Survey	46

9.1.10.	Compute Volume by Contouring	48

9.1.11.	Measure Angles by Theodolite	50

9.1.12.	Perform Trigonometric Levelling by Theodolite	53

9.1.13.	Perform Traversing by Theodolite	55

9.1.14.	Set Out Horizontal Curves by Theodolite	57

9.1.15.	Set out Vertical Curves by Theodolite	59

9.1.16.	Carry out Triangulation by Theodolite	61

9.1.17.  	Perform Temporary Adjustment of Total Station	63

9.1.18.      Measure Distance by Total Station	65

9.1.19.      Measure Angles by Total Station	67

9.1.20.	Measure Area by Total Station	70

9.1.21.      Set out Curves by Total Station	72

9.1.22.      Perform Integration of GPS	74

9.1.23.  	Measure Co-ordinates by GPS	77

9.1.24.  	Set out Civil Engineering Structures by GPS	79

9.2.	Quantity Surveying	82

9.2.1.	Produce rough cost estimates of buildings	82

9.2.2.	Evaluate items of work for straight wall, T, L, H, U shaped and circular walls.	84

9.2.3.	Produce detailed estimate of a single storey building.	86

9.2.4.	Produce Annual/Special repair estimates-Building and Road.	87

9.2.5.	Evaluate Earth work for Civil Engineering structures	88

9.2.6.	Produce detailed estimate of Arterial roads	90

9.2.7.	Produce material statements	92

9.2.8.	Analyse Rates of different items of work for building	94

9.2.9.	Produce of rough cost estimate of water supply scheme.	96

9.2.10.	Produce of rough cost estimate of Sewerage scheme.	99

9.2.11.	Evaluate value & standard rent of an existing building	101

9.2.12.	Produce Detailed Estimate of Culverts.	103

9.2.13.	Produce Detailed Estimate of Bridge.	105

9.2.14.	Produce Detailed Estimate of Water Tanks.	107

9.2.15.	Produce Detailed Estimate of Wells.	110

9.2.16.	Produce Detailed Estimate of Steel Structures	112

9.2.17.	Produce Detailed Estimate of RCC Framed Structure	114

9.2.18.	Produce Detailed Estimate of Building Services.	116

9.2.19.	Produce Detailed Estimates Using Computer Software -Excel.	118

9.2.20.	Produce Project of Road in AutoCAD Civil 3D software	123

9.3.	Civil Engineering. Drawing	126

9.3.1.	Produce Templates, Title Block/ Strip & Draw Lines	126

9.3.2.	Perform Lettering And Printing On Sheet.	129

9.3.3.	Construct Scales for Drawing	130

9.3.4.	Construct Geometrical Figures	133

9.3.5.	Produce Orthographic Projections	136

9.3.6.	Perform Sectioning	138

9.3.7.	Perform Dimensioning On Drawings	140

9.3.8.	Produce Pictorial Drawings	142

9.3.9.	Produce Auxiliary Views	144

9.3.10.	Perform Development of Surfaces	146

9.3.11.	Develop Symbols of Engineering Drawings	148

9.3.12.	Produce Cross Section of Wall.	151

9.3.13.	Develop Building Drawings	153

9.3.14.	Perform House Planning	156

9.3.15.	Produce Layout Plans for Building Services.	158

9.3.16.	Produce Drawings of Sanitary Structures	160

9.3.17.	Produce Drawing of Framed Structure Building	162

9.3.18.	Develop Sectioning Drawing for foundations	165

9.3.19.	Develop Drawings for Doors & Windows	167

9.3.20.	Produce Drawings of Stairs and Stair Cases	169

9.3.21.	Produce Drawings of Water Tanks	171

9.3.22.	Produce Drawings Of Cross Drainage Structures	173

9.3.23.	Perform Basic Computer Operations	175

9.3.24.	Perform Drafting Settings	178

9.3.25.	Develop 2d Drawings	180

9.3.26.	Perform Dimensioning, and Printing	182

9.3.27.	Develop 3d Drawings	184

9.3.28.	Produce Civil Engineering Drawings using AutoCAD	186

9.4.	Transportation Engineering	189

9.4.1.	Plan Route Alignment of Highway	189

9.4.2.	Design Highway Geometrics	190

9.4.3.	Perform tests on stone metal for pavement construction	192

9.4.4.	Perform tests on bitumen	193

9.4.5.	Prepare drawings of flexible/rigid pavement	196

9.4.6.	Plan highway drainage	198

9.4.7.	Develop maintenance plan for roads	198

9.4.8.	Develop construction plan of Tunnels	200

9.4.9.	Produce sketches of an airport	201

9.4.10.	Produce sketches of permanent Way	203

9.4.11.	Produce sketches of points and crossings	205

9.4.12.	Produce sketches of station yards and signals	207

9.4.13.	Produce sketches of harbour	209

9.5.	Environment Health and safety	211

9.5.1.	Determine Water Pollution	211

9.5.2.	Evaluate Air Pollution	213

9.5.3.	Determine Noise Pollution	215

9.5.4.	Evaluate Solid Waste Pollution	216

9.5.5.	Adopt occupational health and safety	217

9.5.6.	Respond to Natural Disaster	219

9.6.	Soil mechanics and Bridge Engineering.	220

9.6.1	Determine moisture content and specific gravity of soil	220

9.6.2.	Determine Atterberg’s Limits	222

9.6.3.        Identify Type of Soil	224

9.6.4.        Conduct soil compaction tests	226

9.6.5.        Determine permeability of soil	229

9.6.6.        Investigate shear strength of soil	231

9.6.7.        Find out bearing capacity of soil	233

9.6.8.        Produce sketches of bridges w.r.t. functions	235

9.6.9.        Produce sketches of bridges w.r.t. material of construction	237

9.6.10.	Produce sketches of bridges w.r.t. type of super structure	239

9.6.11.      Produce sketches of bridges w.r.t. floor type	241

9.6.12.      Produce sketches of culverts, low level and high level causeway	243

9.7.	Mechanics of structures	245

9.7.1.	Locate centroid of different sections	245

9.7.2.	Find out moment of inertia of different sections	247

9.7.3.	Produce Shear Force and Bending Moment Diagrams	249

9.7.4.	Find hardness of metals	252

9.7.5.	Conduct Tensile Strength Test	254

9.7.6.	Design Homogeneous Beam	256

9.7.7.	Determine Deflection of Beams	258

9.7.8.	Determine load carrying capacity of columns	260

9.7.9.	Evaluate Efficiency of Joints-riveted and welded	262

9.7.10.	Analyze forces in truss members	264

9.7.11.	Determine stability of retaining wall	266

9.8.	Plumbing/Public Health & water supply	267

9.8.1.	Identify plumbing tools & equipment	267

9.8.2.	Perform cutting, threading & reaming	269

9.8.3.	Make water connection	271

9.8.4.	Layout for domestic water supply	273

9.8.5.	Join pipes & fittings	275

9.8.6.	Install plumbing fixtures	278

9.8.7.	Install & repair water pumps	281

9.8.8.	Install turbine pumps	283

9.8.9.	Estimate demand for Water Supply	285

9.8.10.	Prepare sketches of aqueducts & water pipes	287

9.8.11.	Draw sketches of intakes	289

9.8.12.	Plan water treatment (Clarification)	291

9.8.13.	Plan water treatment (Filtration)	293

9.8.14.	Plan water distribution system	295

9.9.	R.C.C & concrete technology	297

9.9.1.	Test the Properties of Cement	297

9.9.2.	Find Out Strength of Cement	299

9.9.3.	Test the Properties of Aggregate	301

9.9.4.	Test the Properties of Fresh Concrete	303

9.9.5.	Destructive Tests on Hardened Concrete	305

9.9.6.	Perform Non-Destructive Tests on Hardened Concrete	307

9.9.7.	Design and Draw Reinforcement for Beams	308

9.9.8.	Design and Draw Reinforcement for Slabs	310

9.9.9.	Design and Draw Reinforcement for Columns	311

9.9.10.	Design and Draw Reinforcement for Footings	313

9.9.11.	Design and Draw Reinforcement for Stairs	314

9.10.	Building Materials & Construction Techniques	315

9.10.1.	Identify bricks	315

9.10.2.	Dress stone for masonry	318

9.10.3.	Prepare slaked lime	320

9.10.4.	Identify and store cement for construction	322

9.10.5.	Select sand for construction	324

9.10.6.	Perform concreting	326

9.10.7.	Cut and core concrete	328

9.10.8.	Identify ferrous and non-ferrous metals	330

9.10.9.	Identify advanced construction materials	332

9.10.10.	Work Safely in Construction Industry	334

9.10.11.	Interpret Simple Building Plans	336

9.10.12.	Read and Interpret plans and Specifications	338

9.10.13.	Apply power and hand tools in construction.	341

9.10.14.	Perform Simple Levelling	343

9.10.15.	Lay out Building plans	345

9.10.16.	Carryout excavation and install supports	348

9.10.17.	Place Concrete in Foundations	351

9.10.18.	Prepare for construction Process (Brick/ block laying)	353

9.10.19.	Mix Cementitious materials (Mortar & Concrete)	355

9.10.20.	Install flashing and damp proof course	357

9.10.21.	Carry out brick work	359

9.10.22.	Construct Cavity Wall	362

9.10.23.	Construct Curved Wall	367

9.10.24.	Construct Masonry arches (semi-circular and segmental)	369

9.10.25.	Construct Scaffolding	372

9.10.26.	Install doors and window frames	375

9.10.27.	Prepare Surface and fix tiles	378

9.10.28.	Prepare Surface for Painting	383

9.10.29.	Paint by spray, brush and roller	386

9.10.30.	Apply stains and clear timber finishes	389

9.10.31.	Prepare site layout of civil engineering projects	392

9.10.32.	Construct masonry steps and stairs	395

9.10.33.	Install stairs	398

9.10.34.	Prepare for solid dry wall plastering	401

9.10.35.	Apply plaster	404

9.10.36.	Prepare subgrade, base, and bedding course for paving	407

9.10.37.	Lay paving	410

9.10.38.	Perform concrete, tiles and mosaic flooring	412

9.10.39.	Perform marble/granite flooring	415

9.10.40.	Apply water proofing remedial processes	417

9.10.41.	Apply and install sealants	420

9.10.42.	Perform water proofing works	422

9.10.43.	Apply water proofing tools and equipment	426

9.10.44.	Perform water proofing of basements	430

9.10.45.	Apply water proofing to internal wet areas	435

9.10.46.	Install Physical termite barriers	440

9.10.47.	Erect and dismantle slip formwork	444

9.10.48.	Install trench support	447

9.10.49.	Construct underpinning	449

9.10.50.	Carry out reinforced concrete work	452

9.10.51.	Work safely on roofs	455

9.10.52.	Install roof components	457

9.10.53.	Install roof drainage	460

9.10.54.	Lay roof tiles	462

9.10.55.	Erect roof trusses	464

9.10.56.	Erect and dismantle formwork to suspended slabs, columns, beams and walls	466

9.10.57.	Operate elevated platforms	468

9.10.58.	Construct, erect and dismantle formwork for stairs and ramps	471

9.10.59.	Evaluate materials for multi-storey buildings	475

9.10.60.	Apply structural principles to the construction of large, high rise and complex buildings	477

9.10.61.	Install fire and smoke containment systems	480

9.10.62.	Inform clients about thermal performance of residential buildings	484

9.10.63.	Design air conditioning and ventilation system	487

9.10.64.	Design acoustic of an auditorium	490

9.10.65.	Insulate sound insulation products in building	491

9.10.66.	Install ceiling insulation	493

9.10.67.	Design an earthquake resistant building	498

9.10.68.	Coordinate maintenance and repair of buildings	500

9.10.69.	Assess construction faults in buildings	503

9.10.70.	Repair and rectify concrete	505

9.10.71.	Repair wall and floor tiles	508

9.10.72.	Apply building codes and standards to building projects	511

9.11.	Workshop Practice-I (wood working)	514

9.11.1.	Carry out OH & S requirement in workshop	514

9.11.2.	Determine properties and types of locally manufactured timber	516

9.11.3.	Sharpen carpentry tools	518

9.11.4.	Apply hand tools	519

9.11.5.	Perform wooden joinery work	521

9.11.6.	Apply fastenings	523

9.11.7.	Apply traditional spirits polishing techniques	524

9.11.8.	Apply portable power tools	527

3.11.9.	Make simple calculations of timber	529

3.11.10.	Construct a basic timber product	530

9.12.	Construction of project planning and management	532

9.12.1.	Develop Project Management Life Cycle	532

9.12.2.	Develop Preliminary Project Plan	535

9.12.3.	Develop Construction Project Plan	537

9.12.4.	Conduct on-site supervision of construction projects	540

9.12.5.	Perform Basic operation in Primavera P6	543

9.12.6.	Perform road Project Activities Scheduling in Primavera P6	545

9.12.7.	Perform road Project Resources Costing &Planning in Primavera P6	548

9.12.8.	Manage construction Project in Primavera P6	551

9.13.	Hydraulics and Irrigation Engineering	552

9.13.1.	Determine Hydrostatic pressure	552

9.13.2.	Apply Hydro kinematic Principles	554

9.13.3.	Produce sketches of canals and head works	557

9.13.4.	Design Canals	560

9.13.5.	Produce sketches of Hydraulic structures	562

9.14.	Workshop Practice-I (electrical wiring)	564

9.14.1.	Carry out OH & S requirement in workshop	564

9.14.2.	Maintain safety	566

9.14.3.	Perform electrical wiring	568

9.14.4.	Install electrical system	571

9.14.5.	Construct circuits	574

9.14.6.	Measure properties of electricity	576

9.14.7.	Perform electrical wiring of a single room	577

9.15.	Sanitary Engineering	580

9.15.1.	Design sanitary sewers	580

9.15.2.	Design storm water sewers	582

9.15.3.	Layout of sewers	584

9.15.4.	Prepare sketches of sewer appurtenance	586

9.15.5.	Construct Sewers	589

9.15.6.	Maintain Sewers	591

9.15.7.	Install Sewage Pumps	593

9.15.8.	Produce sketches of sewage treatment plant	595

9.15.9.	Dispose sewage	597

9.16.	Tendering and Accounting	599

9.16.1.	Produce Tender Documents & Collect Proposals	599

9.16.2.	Perform Tender Evaluation & Award of Contract	602

9.16.3.	Maintain record of measurements and payments.	604

9.16.4.	Maintain record of Stores.	606

9.17.	Civil Engineering Project	608

9.17.1.	Develop Project for construction of RCC framed Structure Class Room.	608

9.17.2.	Develop Project plan for construction of RCC OHR of 10000 gal (UK) capacity.	611

9.17.3.	Develop Project plan for construction of Water Supply Scheme.	613

9.17.4.	Develop Project plan for construction of sewerage scheme	615

9.17.5.	Develop Project plan for construction of Highway	617

9.17.6.	Develop Project plan for construction of Irrigation Canal	619

9.18.	Entrepreneurial Skill	621

9.18.1.	Investigate micro business opportunities	621

9.18.2.	Develop a micro business proposal	624

9.18.3.	Develop a marketing plan	626

9.18.4.	Develop and review a business plan.	628

9.18.5.	Organise finances for the micro business	630

9.18.6.	Manage human resources	632

9.18.7.	Market products and services	635

9.18.8.	Monitor and review business performance	638

9.18.9.	Negotiate for resolving business issues	640

9.19.	Soft Skills	642

9.19.1.	Develop workplace policy and procedures for sustainability	642

9.19.2.	Manage meetings	644

9.19.3.	Manage recruitment selection and induction processes	646

9.19.4.	Manage personal work priorities and professional development	648

9.19.5.	Manage workforce planning	650

9.19.6.	Undertake project work	652

9.19.7.	Identify and communicate trends in career development	655

9.19.8.	Apply specialist interpersonal and counseling interview skills	657

9.19.9.	Work safely in an office environment	661

9.19.10.	Develop workplace documents	663

9.19.11.	Prepare and implement negotiation	666

9.19.12.	Maintain professionalism in the workplace	669

9.19.13.	Maintain professional development and career professionalism	671

9.19.14.	Organize schedules	673

9.20.	Digital Skills	675

9.20.1.	Install computer operating systems and hardware	675

9.20.2.	Operate digital media technology	677

9.20.3.	Perform computer operations	679

9.20.4.	Use computer applications	681

9.20.5.	Create user documentation	684

9.20.6.	Create technical documentation	685

9.20.7.	Create basic databases	687

9.20.8.	Use social media tools for collaboration and engagement	689

9.20.9.	E-Commerce- SEO (Search Engine Optimization)	691

9.20.10.	E-Commerce- SCM (Supply Chain Management)	692

9.20.11.	E-Commerce- Social Media Marketing	693

9.20.12.	Use digital devices	694

9.20.13.	Operate word-processing applications	696

9.20.14.	Operate spreadsheet applications	698

9.20.15.	Operate presentation packages	700

9.20.16.	Perform writing and editing tasks	702

9.20.17.	Write and Edit Copy	704

9.20.18.	Build Java applets	706

9.20.19.	Manipulate Images (Illustrator)	708

9.20.20.	Manipulate Images (Photoshop)	710

9.20.21.	Design simple web page layouts	712

9.20.22.	Apply introductory programming techniques	713

# Occupation Packaging of Civil Technology

| S.# | Name of Occupation | Level | Digital Skills | Soft Skills | Technical Skills |
| --- | --- | --- | --- | --- | --- |
|  | Civil Surveyor | 3 | 9.20.1 to 9.20.22 | 9.19.1 to 9.19.14 | 9.1.1 to 9.1.24 9.3.1 to 9.3.11 |
|  | Draftsman | 3 | 9.20.1 to 9.20.22 | 9.19.1 to 9.19.14 | 9.3.1 to 9.3.28 |
|  | Quantity Surveyor | 5 | 9.20.1 to 9.20.22 | 9.19.1 to 9.19.14 | 9.2.1 to 9.2.20 9.3.1 to 9.3.28 9.1.6,9.1.8,9.1.10 9.16.1 to 9.16.4 |
|  | Quantity Surveyor after Draftsman | 5 | ----- | ----- | 9.2.1 to 9.2.20 9.1.6,9.1.8,9.1.10 9.16.1 to 9.16.4 |
|  | Materials Technician | 5 | 9.20.1 to 9.20.22 | 9.19.1 to 9.19.14 | 9.4.3, 9.4.4 9.6.1 to 9.6.7 9.7.4, 9.7.5 9.9.1 to 9.9.6 9.10.1 |
|  | Construction Quality control Supervisor | 5 | 9.20.1 to 9.20.22 | 9.19.1 to 9.19.14 | 9.1.1 to 9.1.24 9.4.3, 9.4.4 9.6.1 to 9.6.7 9.7.4, 9.7.5 9.9.1 to 9.9.6 9.10.1 9.12.1 to 9.12.8 |
|  | Mason | 2 | ------ | ------ | 9.10.10 to 9.10.58 |
|  | Plumber | 2 | ----- | --- | 9.8.1 to 9.8.8 9.15.3, 9.15.5, 9.15.7 |
|  | Domestic Electrician | 2 | ------ | ------ | - to 9.14.7 |

# B. Qualification Packaging of Civil Technology

| S.# | Name of Qualification | Level | Digital Skills | Soft Skills | Technical Skills |
| --- | --- | --- | --- | --- | --- |
| 1 | Site supervisor | 4 | 9.20.1 to 9.20.22 | 9.19.1 to 9.19.14 | 9.1.1 to 9.1.10 9.3.1 to 9.3.22 9.4.1 to 9.4.16 9.8.1 to 9.8.14 9.9.1 to 9.9.4 9.10.1 to 9.10.72 9.14.1 to 9.14.7 9.11.1 to 9.11.10 9.15.1 to 9.15.9 |

# 1. Introduction

Civil engineering is considered as the first discipline of the various branches of engineering after military engineering, and includes the designing, planning, construction, and maintenance of the infrastructure. The works include roads, bridges, buildings, dams, canals, water supply and numerous other facilities that affect the life of human beings. Civil engineering is intimately associated with the private and public sectors, including the individual homeowners and international enterprises. It is one of the oldest engineering professions, and ancient engineering achievements due to civil engineering include the pyramids of Egypt and road systems developed by the Romans.

Civil engineering has a significant role in the life of every human being, though one may not truly sense its importance in our daily routine. The function of civil engineering commences with the start of the day when we take a shower, since the water is delivered through a water supply system including a well-designed network of pipes, water treatment plant and other numerous associated services. The network of roads on which we drive while proceeding to school or work, the huge structural bridges we come across and the tall buildings where we work, all have been designed and constructed by civil engineers. Even the benefits of electricity we use are available to us through the contribution of civil engineers who constructed the towers for the transmission lines. In fact, no sphere of life may be identified that does not include the contribution of civil engineering. Thus, the importance of civil engineering may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Civil Technology under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# 2. Purpose of the Qualification

The purpose of this qualifications is to set high professional standards for civil technology sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for civil technology industry

- Enable the existing workforce to capacitate themselves in new technologies and methods

# 3.  Core competencies of the Qualification

The Civil Technology qualifications of level 5 consists 40% Theory and 60% Practical. The Core competencies of the qualification are as follows:

| 9.1 Surveying |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
|  | 9.1a- Basic Engineering Survey | 29 | 51 | 80 | 8 |  |  |
| 9.1.1. | Perform Chain Survey | 8 | 12 | 20 | 2 | Technical | 3 |
| 9.1.2. | Perform Compass Traversing | 9 | 21 | 30 | 3 | Technical | 3 |
| 9.1.3. | Carry out Plane Table Survey | 12 | 18 | 30 | 3 | Technical | 3 |
|  | 9.1b- Contouring and Levelling | 45 | 85 | 130 | 13 |  |  |
| 9.1.4. | Perform Levelling | 9 | 21 | 30 | 3 | Technical | 3 |
| 9.1.5. | Perform Reciprocal Levelling | 4 | 6 | 10 | 1 | Technical | 3 |
| 9.1.6. | Perform Long & Cross Sectioning | 8 | 12 | 20 | 2 | Technical | 3 |
| 9.1.7. | Perform Tacheometry | 6 | 12 | 18 | 1.8 | Technical | 3 |
| 9.1.8. | Perform Contouring | 8 | 12 | 20 | 2 | Technical | 4 |
| 9.1.9. | Perform Hydrographic Survey | 6 | 12 | 18 | 1.8 | Technical | 4 |
| 9.1.10. | Compute Volume by Contouring | 4 | 10 | 14 | 1.4 | Technical | 4 |
|  | 9.1c- Theodolite Surveying | 32 | 98 | 130 | 13 |  |  |
| 9.1.11. | Measure Angles by Theodolite | 8 | 24 | 32 | 3.2 | Technical | 4 |
| 9.1.12. | Perform Trigonometric Levelling by Theodolite | 4 | 12 | 16 | 1.6 | Technical | 5 |
| 9.1.13. | Perform Traversing by Theodolite | 8 | 18 | 26 | 2.6 | Technical | 5 |
| 9.1.14. | Set Out Horizontal Curves by Theodolite | 6 | 20 | 26 | 2.6 | Technical | 5 |
| 9.1.15. | Set out Vertical Curves by Theodolite | 3 | 12 | 15 | 1.5 | Technical | 5 |
| 9.1.16. | Carry out Triangulation by Theodolite | 3 | 12 | 15 | 1.5 | Technical | 5 |
|  | 9.1d- Digital Surveying | 32 | 88 | 120 | 12 |  |  |
| 9.1.17. | Perform Temporary Adjustment of Total Station | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.1.18. | Measure Distance by Total Station | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.1.19. | Measure Angles by Total Station | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.1.20. | Measure Area by Total Station | 4 | 10 | 14 | 1.4 | Technical | 5 |
| 9.1.21. | Set out Curves by Total Station | 4 | 12 | 16 | 1.6 | Technical | 5 |
| 9.1.22. | Perform Integration of GPS | 4 | 12 | 16 | 1.6 | Technical | 5 |
| 9.1.23. | Measure Co-ordinates by GPS | 4 | 12 | 16 | 1.6 | Technical | 5 |
| 9.1.24. | Set out Civil Engineering Structures by GPS | 4 | 24 | 28 | 2.8 | Technical | 5 |
| 9.2 Quantity Surveying. |  |  |  |  |  |  |  |
| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
|  | 9.2a Quantity Surveying-I | 18 | 82 | 100 | 10 |  |  |
| 9.2.1 | Produce rough cost estimates of buildings | 5 | 15 | 20 | 2 | Technical | 5 |
| 9.2.2 | Evaluate items of work for straight wall, T, L, H, U shaped and circular walls. | 1 | 9 | 10 | 1 | Technical | 5 |
| 9.2.3 | Produce detailed estimate of a single storey building. | 2 | 13 | 15 | 1.5 | Technical | 5 |
| 9.2.4 | Produce Annual/special repair estimates-Building and Road. | 2 | 8 | 10 | 1 | Technical | 5 |
| 9.2.5 | Evaluate Earth work for Civil Engineering structures | 2 | 18 | 20 | 2 | Technical | 5 |
| 9.2.6 | Produce detailed estimate of Arterial roads | 3 | 12 | 15 | 1.5 | Technical | 5 |
| 9.2.7 | Produce material statements | 3 | 7 | 10 | 1 | Technical | 5 |
|  | 9.2b Quantity Surveying-II | 15 | 55 | 70 | 7 |  |  |
| 9.2.8 | Analyze Rates of different items of work for building | 4 | 16 | 20 | 2 | Technical | 5 |
| 9.2.9 | Produce of rough cost estimate of water supply scheme. | 2 | 8 | 10 | 1 | Technical | 5 |
| 9.2.10 | Produce of rough cost estimate of Sewerage scheme. | 2 | 8 | 10 | 1 | Technical | 5 |
| 9.2.11 | Evaluate Value & Standard Rent of Buildings | 2 | 8 | 10 | 1 | Technical | 5 |
| 9.2.12 | Produce Detailed Estimate of Culverts | 5 | 15 | 20 | 2 | Technical | 5 |
|  | 9.2c Quantity Surveying-III | 17 | 103 | 120 | 12 |  |  |
| 9.2.13 | Produce Detailed Estimate of Bridges. | 1 | 9 | 10 | 1 | Technical | 5 |
| 9.2.14 | Produce Detailed Estimate of Water Tanks. | 2 | 18 | 20 | 2 | Technical | 5 |
| 9.2.15 | Produce Detailed Estimate of Wells. | 2 | 18 | 20 | 2 | Technical | 5 |
| 9.2.16 | Produce Detailed Estimate of Steel Structures | 2 | 8 | 10 | 1 | Technical | 5 |
| 9.2.17 | Produce Detailed Estimate of RCC Framed Structure | 3 | 17 | 20 | 2 | Technical | 5 |
| 9.2.18 | Produce Detailed Estimate of Building Services. | 2 | 8 | 10 | 1 | Technical | 5 |
| 9.2.19 | Produce Detailed Estimates Using Computer Software -Excel. | 1 | 9 | 10 | 1 | Technical | 5 |
| 9.2.20 | Produce Project of Road in AutoCAD Civil 3D software | 4 | 16 | 20 | 2 | Technical | 5 |
| 9.3 Civil Engineering Drawing |  |  |  |  |  |  |  |
| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
|  | 9.3-a Basic Engineering Drawing | 10 | 90 | 100 | 10 |  |  |
| 9.3.1. | Produce Templates, Title Block/ Strip & Draw Lines | 1 | 9 | 10 | 1 | Technical | 4 |
| 9.3.2. | Perform Lettering And Printing On Sheet. | 1 | 9 | 10 | 1 | Technical | 4 |
| 9.3.3. | Construct Scales for Drawing | 1 | 9 | 10 | 1 | Technical | 4 |
| 9.3.4. | Construct Geometrical Figures | 1 | 9 | 10 | 1 | Technical | 4 |
| 9.3.5. | Produce Orthographic Projections | 1 | 9 | 10 | 1 | Technical | 5 |
| 9.3.6. | Perform Sectioning | 1 | 9 | 10 | 1 | Technical | 4 |
| 9.3.7. | Perform Dimensioning On Drawings | 1 | 9 | 10 | 1 | Technical | 4 |
| 9.3.8. | Produce Pictorial Drawings | 1 | 9 | 10 | 1 | Technical | 5 |
| 9.3.9. | Produce Auxiliary Views | 1 | 9 | 10 | 1 | Technical | 5 |
| 9.3.10. | Perform Development of Surfaces | 1 | 9 | 10 | 1 | Technical | 4 |
|  | 9.3-b Building Drawing | 10 | 90 | 100 | 10 |  |  |
| 9.3.11. | Develop Symbols of Engineering Drawings | 1 | 9 | 10 | 1 | Technical | 4 |
| 9.3.12. | Produce Cross Section of Wall. | 1 | 9 | 10 | 1 | Technical | 5 |
| 9.3.13. | Develop Building Drawings | 4 | 36 | 40 | 4 | Technical | 5 |
| 9.3.14. | Perform House Planning | 1 | 9 | 10 | 1 | Technical | 5 |
| 9.3.15. | Produce Layout Plans for Building Services. | 2 | 18 | 20 | 2 | Technical | 5 |
| 9.3.16. | Produce Drawings of Sanitary Structures | 1 | 9 | 10 | 1 | Technical | 5 |
|  | 9.3-c Structural Engineering Drawing | 10 | 90 | 100 | 10 |  |  |
| 9.3.17. | Produce Drawing of Framed Structure Building | 3 | 27 | 30 | 3 | Technical | 5 |
| 9.3.18. | Develop Sectioning Drawing for foundations | 2 | 18 | 20 | 2 | Technical | 5 |
| 9.3.19. | Develop Drawings for Doors & Windows | 1 | 9 | 10 | 1 | Technical | 5 |
| 9.3.20. | Produce Drawings of Stairs and Stair Cases | 1 | 9 | 10 | 1 | Technical | 5 |
| 9.3.21. | Produce Drawings of Water Tanks | 2 | 18 | 20 | 2 | Technical | 5 |
| 9.3.22. | Produce Drawings Of Cross Drainage Works | 1 | 9 | 10 | 1 | Technical | 5 |
|  | 9.3-d AutoCAD Civil | 20 | 60 | 80 | 8 |  |  |
| 9.3.23. | Perform Basic Computer Operations | 4 | 6 | 10 | 1 | Technical | 4 |
| 9.3.24. | Perform Drafting Settings and Formatting | 4 | 6 | 10 | 1 | Technical | 4 |
| 9.3.25. | Develop 2d Drawings of Structures | 2 | 18 | 20 | 2 | Technical | 5 |
| 9.3.26. | Perform Dimensioning, Lettering & Plotting | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.3.27. | Develop 3d Drawings | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.3.28. | Produce Civil Technology Drawings | 2 | 18 | 20 | 2 | Technical | 5 |
| 9.4 Transportation Engineering |  |  |  |  |  |  |  |
| S# | Competency Standard | Th | Practical | Total | Credit Hour | Category | Level |
|  | 9.4-a Highways, Air Ports and Traffic Engineering | 49 | 51 | 100 | 10 |  |  |
| 9.4.1. | Plan Route Alignment of Highway | 6 | 6 | 12 | 1.2 | Technical | 5 |
| 9.4.2. | Design Highway Geometrics | 6 | 6 | 12 | 1.2 | Technical | 5 |
| 9.4.3. | Perform tests on stone metal for pavement construction | 4 | 6 | 10 | 1.0 | Technical | 5 |
| 9.4.4. | Perform tests on bitumen | 6 | 6 | 12 | 1.2 | Technical | 5 |
| 9.4.5. | Prepare drawings of flexible/rigid pavement | 6 | 6 | 12 | 1.2 | Technical | 5 |
| 9.4.6. | Plan highway drainage | 6 | 6 | 12 | 1.2 | Technical | 5 |
| 9.4.7. | Develop maintenance plan for roads | 6 | 6 | 12 | 1.2 | Technical | 5 |
| 9.4.8. | Develop construction plan of Tunnels | 6 | 6 | 12 | 1.2 | Technical | 5 |
| 9.4.9. | Produce sketches of an airport | 3 | 3 | 6 | 0.6 | Technical | 5 |
|  | 9.4-b Railways, Docks and Harbours | 49 | 51 | 100 | 10 |  |  |
| 9.4.10. | Produce sketches of permanent Way | 12 | 12 | 24 | 2.4 | Technical | 5 |
| 9.4.11. | Produce sketches of points and crossings | 13 | 15 | 28 | 2.8 | Technical | 5 |
| 9.4.12. | Produce sketches of station yards and signals | 12 | 12 | 24 | 2.4 | Technical | 5 |
| 9.4.13. | Produce sketches of harbour | 12 | 12 | 24 | 2.4 | Technical | 5 |
| 9.5 Environment Health and Safety |  |  |  |  |  |  |  |
| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
|  |  | 32 | 48 | 80 | 8 |  |  |
| 9.5.1. | Determine Water Pollution | 6 | 9 | 15 | 1.5 | Generic | 5 |
| 9.5.2. | Evaluate Air Pollution | 4 | 6 | 10 | 1 | Generic | 5 |
| 9.5.3. | Determine Noise Pollution | 6 | 9 | 15 | 1.5 | Generic | 5 |
| 9.5.4. | Evaluate Solid Waste Pollution | 4 | 6 | 10 | 1 | Generic | 5 |
| 9.5.5. | Adopt occupational health and safety | 6 | 9 | 15 | 1.5 | Generic | 4 |
| 9.5.6. | Respond to Natural Disaster | 6 | 9 | 15 | 1.5 | Generic | 5 |
| 9.6 Soil Mechanics and Bridge Engineering |  |  |  |  |  |  |  |
| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
|  | 9.6-a Soil Mechanics | 32 | 48 | 80 | 8 |  |  |
| 9.6.1. | Determine moisture content and specific gravity of soil | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.6.2. | Determine Atterberg’s Limits | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.6.3. | Identify type of soil | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.6.4. | Conduct soil compaction tests | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.6.5. | Determine permeability of soil | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.6.6. | Investigate shear strength of soil | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.6.7. | Find out bearing capacity of soil | 4 | 6 | 10 | 1 | Technical | 5 |
|  | 9.6-b Bridge Engineering | 32 | 48 | 80 | 8 |  |  |
| 9.6.8. | Produce sketches of bridges w.r.t. functions | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.6.9. | Produce sketches of bridges w.r.t. material of construction | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.6.10. | Produce sketches of bridges w.r.t. type of super structure | 8 | 12 | 20 | 2 | Technical | 5 |
| 9.6.11. | Produce sketches of bridges w.r.t. floor type | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.6.12. | Produce sketches of culverts, low level and high level causeway | 6 | 9 | 15 | 1.5 | Technical | 5 |

| 9.7 Mechanics of Structures |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
|  | 9.7-a Strength of Materials | 32 | 48 | 80 | 8 |  |  |
| 9.7.1. | Locate centroid of different sections | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.7.2. | Find out moment of inertia of different sections | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.7.3. | Produce Shear Force and Bending Moment Diagrams | 8 | 12 | 20 | 2 | Technical | 5 |
| 9.7.4. | Find hardness of metals | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.7.5. | Conduct Tensile Strength Test | 6 | 9 | 15 | 1.5 | Technical | 5 |
|  | 9.7-b Engineering Mechanics | 32 | 48 | 80 | 8 |  |  |
| 9.7.6. | Design Homogeneous Beam | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.7.7. | Determine Deflection of Beams | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.7.8. | Determine load carrying capacity of columns | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.7.9. | Evaluate Efficiency of Joints-riveted and welded | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.7.10. | Analyze forces in truss members | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.7.11. | Determine stability of retaining wall | 6 | 9 | 15 | 1.5 | Technical | 5 |

| 9.8 Plumbing Public Health and Water Supply |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
|  | 9.8-a Plumbing | 12 | 48 | 60 | 6 |  |  |
| 9.8.1. | Identify plumbing tools & equipment | 2 | 6 | 8 | 0.8 | Technical | 2 |
| 9.8.2. | Perform cutting, threading & reaming | 2 | 6 | 8 | 0.8 | Technical | 3 |
| 9.8.3. | Make water connection | 1 | 6 | 7 | 0.7 | Technical | 3 |
| 9.8.4. | Layout for domestic water supply | 1 | 6 | 7 | 0.7 | Technical | 4 |
| 9.8.5. | Join pipes & fittings | 1 | 6 | 7 | 0.7 | Technical | 3 |
| 9.8.6. | Install plumbing fixtures | 2 | 6 | 8 | 0.8 | Technical | 3 |
| 9.8.7. | Install & repair water pumps | 1 | 6 | 7 | 0.7 | Technical | 3 |
| 9.8.8. | Install turbine pumps | 2 | 6 | 8 | 0.8 | Technical | 5 |
|  | 9.8-b Water Supply | 32 | 48 | 80 | 8 |  |  |
| 9.8.9. | Estimate demand for Water Supply | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.8.10. | Prepare sketches of aqueducts & water pipes | 4 | 6 | 10 | 1 | Technical | 4 |
| 9.8.11. | Draw sketches of intakes | 4 | 6 | 10 | 1 | Technical | 4 |
| 9.8.12. | Plan water treatment (Clarification) | 8 | 12 | 20 | 2 | Technical | 5 |
| 9.8.13. | Plan water treatment (Filtration) | 8 | 12 | 20 | 2 | Technical | 5 |
| 9.8.14. | Plan water distribution system | 4 | 6 | 10 | 1 | Technical | 5 |

| 9.9 R.C.C. design and Concrete Technology |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
|  | 9.9-a Concrete Technology | 32 | 48 | 80 | 8 |  |  |
| 9.9.1. | Test the Properties of Cement | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.9.2. | Find Out Strength of Cement | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.9.3. | Test the Properties of Aggregate | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.9.4. | Test the Properties of Fresh Concrete | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.9.5. | Destructive Tests on Hardened Concrete | 6 | 9 | 15 | 1.5 | Technical | 5 |
| 9.9.6. | Perform Non-Destructive Tests on Hardened Concrete | 4 | 6 | 10 | 1 | Technical | 5 |
|  | 9.9-b RCC Design | 49 | 51 | 100 | 10 |  |  |
| 9.9.7. | Design and Draw Reinforcement for Beams | 16 | 18 | 34 | 3.4 | Technical | 5 |
| 9.9.8. | Design and Draw Reinforcement for Slabs | 12 | 12 | 24 | 2.4 | Technical | 5 |
| 9.9.9. | Design and Draw Reinforcement for Columns | 6 | 6 | 12 | 1.2 | Technical | 5 |
| 9.9.10. | Design and Draw Reinforcement for Footings | 6 | 6 | 12 | 1.2 | Technical | 5 |
| 9.9.11. | Design and Draw Reinforcement for Stairs | 9 | 9 | 18 | 1.8 | Technical | 5 |

| 9.10. Building Materials & Construction Techniques |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
|  | 9.10-a Building Materials | 32 | 48 | 80 | 8 |  |  |
| 9.10.1. | Identify bricks | 4 | 6 | 10 | 1 | Technical | 2 |
| 9.10.2. | Dress stone for masonry | 4 | 6 | 10 | 1 | Technical | 3 |
| 9.10.3. | Prepare slaked lime | 2 | 3 | 5 | 0.5 | Technical | 2 |
| 9.10.4. | Identify and store cement for construction | 4 | 6 | 10 | 1 | Technical | 3 |
| 9.10.5. | Select sand for construction | 2 | 3 | 5 | 0.5 | Technical | 2 |
| 9.10.6. | Perform concreting | 6 | 9 | 15 | 1.5 | Technical | 3 |
| 9.10.7. | Cut and core concrete | 2 | 3 | 5 | 0.5 | Technical | 3 |
| 9.10.8. | Identify ferrous and non-ferrous metals | 4 | 6 | 10 | 1 | Technical | 2 |
| 9.10.9. | Identify advanced construction materials | 4 | 6 | 10 | 1 | Technical | 3 |
|  | 9.10-b Construction Techniques-I | 37 | 63 | 100 | 10 |  |  |
| 9.10.10 | Work Safely in Construction Industry | 2 | 3 | 5 | 0.5 | Generic | 3 |
| 9.10.11 | Interpret Simple Building Plans | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.12 | Read and Interpret plans and Specifications | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.13 | Apply power and hand tools in construction. | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.14 | Perform Simple Levelling | 2 | 3 | 5 | 0.5 | Technical | 3 |
| 9.10.15 | Lay out Building plans | 2 | 3 | 5 | 0.5 | Technical | 5 |
| 9.10.16 | Carryout excavation and install supports | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.17 | Place Concrete in Foundations | 2 | 3 | 5 | 0.5 | Technical | 5 |
| 9.10.18 | Prepare for construction Process (Brick/ block laying) | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.19 | Mix Cementitious materials (Mortar & Concrete) | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.10.20 | Install flashing and damp proof course | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.21. | Carry out brick work | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.10.22. | Construct Cavity Wall | 2 | 3 | 5 | 0.5 | Technical | 3 |
| 9.10.23. | Construct Curved Wall | 1 | 3 | 4 | 0.4 | Technical | 4 |
| 9.10.24. | Construct Masonry arches (semi-circular and segmental) | 2 | 3 | 5 | 0.5 | Technical | 5 |
| 9.10.25. | Construct Scaffolding | 2 | 3 | 5 | 0.5 | Technical | 3 |
| 9.10.26. | Install doors and window frames | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.27. | Prepare Surface and fix tiles | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.10.28. | Prepare Surface for Painting | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.10.29. | Paint by spray, brush and roller | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.30. | Apply stains and clear timber finishes | 2 | 3 | 5 | 0.5 | Technical | 4 |
|  | 9.10-c Construction Techniques-II | 37 | 63 | 100 | 10 |  |  |
| 9.10.31. | Prepare site layout of civil engineering projects | 2 | 3 | 5 | 0.5 | Technical | 5 |
| 9.10.32. | Construct masonry steps and stairs | 2 | 6 | 8 | 0.8 | Technical | 4 |
| 9.10.33. | Install stairs | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.34. | Prepare for solid dry wall plastering | 2 | 3 | 5 | 0.5 | Technical | 3 |
| 9.10.35. | Apply plaster | 2 | 3 | 5 | 0.5 | Technical | 3 |
| 9.10.36. | Prepare subgrade, base, and bedding course for paving | 2 | 3 | 5 | 0.5 | Technical | 3 |
| 9.10.37. | Lay paving | 2 | 3 | 5 | 0.5 | Technical | 3 |
| 9.10.38. | Perform concrete, tiles and mosaic flooring | 4 | 6 | 10 | 1 | Technical | 3 |
| 9.10.39. | Perform marble/granite flooring | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.40. | Apply water proofing remedial processes | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.41. | Apply and install sealants | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.42. | Perform water proofing works | 2 | 3 | 5 | 0.5 | Technical | 5 |
| 9.10.43. | Apply water proofing tools and equipment | 2 | 3 | 5 | 0.5 | Technical | 3 |
| 9.10.44. | Perform water proofing of basements | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.45. | Apply water proofing to internal wet areas | 1 | 3 | 4 | 0.4 | Technical | 4 |
| 9.10.46. | Install Physical termite barriers | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.10.47. | Erect and dismantle slip formwork | 2 | 3 | 5 | 0.5 | Technical | 5 |
| 9.10.48. | Install trench support | 1 | 3 | 4 | 0.4 | Technical | 4 |
| 9.10.49. | Construct underpinning | 2 | 3 | 5 | 0.5 | Technical | 4 |
|  | 9.10-d Construction Techniques-III | 31 | 69 | 100 | 10 |  |  |
| 9.10.50. | Carry out reinforced concrete work | 2 | 3 | 5 | 0.5 | Technical | 5 |
| 9.10.51. | Work safely on roofs | 1 | 3 | 4 | 0.4 | Technical | 4 |
| 9.10.52. | Install roof components | 2 | 3 | 5 | 0.5 | Technical | 5 |
| 9.10.53. | Install roof drainage | 2 | 3 | 5 | 0.5 | Technical | 3 |
| 9.10.54. | Lay roof tiles | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.10.55. | Erect roof trusses | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.10.56. | Erect and dismantle formwork to suspended slabs, columns, beams and walls | 1 | 3 | 4 | 0.4 | Technical | 5 |
| 9.10.57. | Operate elevated platforms | 3 | 6 | 9 | 0.9 | Technical | 4 |
| 9.10.58. | Construct, erect and dismantle formwork for stairs and ramps | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.59. | Evaluate materials for multi-storey buildings | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.10.60. | Apply structural principles to the construction of large, high rise and complex buildings | 1 | 3 | 4 | 0.4 | Technical | 5 |
| 9.10.61. | Install fire and smoke containment systems | 1 | 3 | 4 | 0.4 | Technical | 5 |
| 9.10.62. | Inform clients about thermal performance of residential buildings | 2 | 3 | 5 | 0.5 | Technical | 5 |
| 9.10.63. | Design air conditioning and ventilation system | 1 | 3 | 4 | 0.4 | Technical | 5 |
| 9.10.64. | Design acoustic of an auditorium | 1 | 3 | 4 | 0.4 | Technical | 5 |
| 9.10.65. | Insulate sound insulation products in building | 1 | 3 | 4 | 0.4 | Technical | 4 |
| 9.10.66. | Install ceiling insulation | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.10.67. | Design an earthquake resistant building | 1 | 3 | 4 | 0.4 | Technical | 4 |
| 9.10.68. | Coordinate maintenance and repair of buildings | 1 | 3 | 4 | 0.4 | Technical | 4 |
| 9.10.69. | Assess construction faults in buildings | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.10.70. | Repair and rectify concrete | 1 | 1.5 | 2.5 | 0.25 | Technical | 3 |
| 9.10.71. | Repair wall and floor tiles | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.10.72. | Apply building codes and standards to building projects | 1 | 1.5 | 2.5 | 0.25 | Technical | 5 |

| 9.11. Workshop Practice(Wood Working) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
|  |  | 15 | 51 | 66 | 14.6 |  |  |
| 9.11.1. | Carry out OH & S requirement in workshop | 2 | 3 | 5 | 0.5 | Generic | 2 |
| 9.11.2. | Determine properties and types of locally manufactured timber | 2 | 6 | 8 | 0.8 | Generic | 3 |
| 9.11.3. | Sharpen carpentry tools | 1 | 6 | 7 | 0.7 | Technical | 3 |
| 9.11.4. | Apply hand tools | 1 | 6 | 7 | 0.7 | Technical | 3 |
| 9.11.5. | Perform wooden joinery work | 2 | 6 | 8 | 0.8 | Technical | 3 |
| 9.11.6. | Apply fastenings | 1 | 3 | 4 | 0.4 | Technical | 3 |
| 9.11.7. | Apply traditional spirits polishing techniques | 2 | 3 | 5 | 0.5 | Technical | 4 |
| 9.11.8. | Apply portable power tools | 1 | 3 | 4 | 0.4 | Technical | 4 |
| 9.11.9. | Make simple calculations of timber | 2 | 6 | 8 | 0.8 | Technical | 4 |
| 9.11.10. | Construct a basic timber product | 1 | 9 | 10 | 1 | Technical | 5 |

9.12. Construction Project Planning & Management

| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 32 | 48 | 80 | 8 |  |  |
| 9.12.1. | Develop Project Management Life Cycle | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.12.2. | Develop Preliminary Project Plan | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.12.3. | Develop Construction Project Plan | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.12.4. | Conduct on-site supervision of construction projects | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.12.5. | Perform Basic operation in Primavera P6 | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.12.6. | Perform road Project Activities Scheduling in Primavera P6 | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.12.7. | Perform road Project Resources Costing &Planning in Primavera P6 | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.12.8. | Manage construction Project in Primavera P6 | 4 | 6 | 10 | 1 | Technical | 5 |

9.13. Hydraulics and Irrigation Engineering

| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9.13-a Hydraulics Engineering | 32 | 48 | 80 | 8 |  |  |
| 9.13.1. | Determine Hydrostatic pressure | 14 | 21 | 35 | 3.5 | Technical | 5 |
| 9.13.2. | Apply Hydro kinematic Principles | 18 | 27 | 45 | 4.5 | Technical | 5 |
|  | 9.13-b Irrigation Engineering | 32 | 48 | 80 | 8 |  |  |
| 9.13.3. | Produce sketches of canals and head works | 12 | 18 | 30 | 3 | Technical | 4 |
| 9.13.4. | Design Canals | 10 | 15 | 25 | 2.5 | Technical | 5 |
| 9.13.5. | Produce sketches of Hydraulic structures | 10 | 15 | 25 | 2.5 | Technical | 5 |

9.14. Workshop Practice (Electrical Wiring)

| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 16 | 48 | 64 | 6.4 |  |  |
| 9.14.1. | Carry out OH & S requirement in workshop | 2 | 6 | 8 | 0.8 | Generic | 4 |
| 9.14.2. | Maintain safety | 2 | 6 | 8 | 0.8 | Technical | 4 |
| 9.14.3. | Perform electrical wiring | 3 | 9 | 12 | 1.2 | Technical | 4 |
| 9.14.4. | Install electrical system | 3 | 9 | 12 | 1.2 | Technical | 3 |
| 9.14.5. | Construct Circuits | 2 | 6 | 8 | 0.8 | Technical | 3 |
| 9.14.6. | Measure properties of electricity | 2 | 6 | 8 | 0.8 | Technical | 3 |
| 9.14.7. | Perform electrical wiring of a single room | 2 | 6 | 8 | 0.8 | Technical | 3 |

9.15. Sanitary Engineering

| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 32 | 48 | 80 | 8 |  |  |
| 9.15.1. | Design sanitary sewers | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.15.2. | Design storm water sewers | 3 | 5 | 8 | 0.8 | Technical | 5 |
| 9.15.3. | Layout of sewers | 4 | 6 | 10 | 1 | Technical | 4 |
| 9.15.4. | Prepare sketches of sewer appurtenance | 4 | 6 | 10 | 1 | Technical | 5 |
| 9.15.5. | Construct Sewers | 3 | 5 | 8 | 0.8 | Technical | 5 |
| 9.15.6. | Maintain Sewers | 3 | 4 | 7 | 0.7 | Technical | 4 |
| 9.15.7. | Install Sewage Pumps | 4 | 6 | 10 | 1 | Technical | 3 |
| 9.15.8. | Produce sketches of sewage treatment plant | 3 | 4 | 7 | 0.7 | Technical | 4 |
| 9.15.9. | Dispose sewage | 4 | 6 | 10 | 1 | Technical | 5 |

9.16. Tendering and Accounting

| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 32 | 48 | 80 | 8 |  |  |
| 9.16.1. | Produce Tender Documents & Collect Proposals | 8 | 12 | 20 | 2 | Generic | 5 |
| 9.16.2. | Perform Tender Evaluation & Award of Contract | 8 | 12 | 20 | 2 | Generic | 5 |
| 9.16.3. | Maintain record of measurements and payments. | 8 | 12 | 20 | 2 | Technical | 5 |
| 9.16.4. | Maintain record of Stores. | 8 | 12 | 20 | 2 | Technical | 5 |

9.17. Civil Engineering Project

| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 100 | 100 | 10 |  |  |
| 9.17.1. | Develop Project for construction of RCC framed Structure Class Room. | 0 | 18 | 18 | 1.8 | Technical | 5 |
| 9.17.2. | Develop Project plan for construction of RCC OHR of 10000 gal (UK) capacity. | 0 | 18 | 18 | 1.8 | Technical | 5 |
| 9.17.3. | Develop Project plan for construction of Water Supply Scheme. | 0 | 16 | 16 | 1.6 | Technical | 5 |
| 9.17.4. | Develop Project plan for construction of sewerage scheme | 0 | 16 | 16 | 1.6 | Technical | 5 |
| 9.17.5. | Develop Project plan for construction of Highway | 0 | 16 | 16 | 1.6 | Technical | 5 |
| 9.17.6. | Develop Project plan for construction of Irrigation Canal | 0 | 16 | 16 | 1.6 | Technical | 5 |

9.18. Entrepreneurship

| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 | 45 | 60 | 6 |  |  |
| 9.18.1. | Investigate micro business opportunities | 1 | 3 | 4 | 0.4 | Generic | 5 |
| 9.18.2. | Develop a micro business proposal | 2 | 6 | 8 | 0.8 | Generic | 5 |
| 9.18.3. | Develop a marketing plan | 2 | 6 | 8 | 0.8 | Generic | 4 |
| 9.18.4. | Develop and review a business plan. | 2 | 6 | 8 | 0.8 | Generic | 4 |
| 9.18.5. | Organize finances for the micro business | 2 | 6 | 8 | 0.8 | Generic | 5 |
| 9.18.6. | Manage human resources | 1 | 3 | 4 | 0.4 | Generic | 5 |
| 9.18.7. | Market products and services | 2 | 6 | 8 | 0.8 | Generic | 5 |
| 9.18.8. | Monitor and review business performance | 2 | 6 | 8 | 0.8 | Generic | 5 |
| 9.18.9. | Negotiate for resolving business issues | 1 | 3 | 4 | 0.4 | Generic | 5 |

9.19. Soft Skills

| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 | 46 | 60 | 6 |  |  |
| 9.19.1 | Develop workplace policy and procedures for sustainability | 1 | 6 | 7 | 0.7 | Generic | 3 |
| 9.19.2 | Manage meetings | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.3 | Manage recruitment selection and induction processes | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.4 | Manage personal work priorities and professional development | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.5 | Manage workforce planning | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.6 | Undertake project work | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.7 | Identify and communicate trends in career development | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.8 | Apply specialist interpersonal and counseling interview skills | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.9 | Work safely in an office environment | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.10 | Develop workplace documents | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.11 | Prepare and implement negotiation | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.12 | Maintain professionalism in the workplace | 1 | 3 | 4 | 0.4 | Generic | 3 |
| 9.19.13 | Maintain professional development and career professionalism | 1 | 4 | 5 | 0.5 | Generic | 3 |
| 9.19.14 | Organize schedules | 1 | 3 | 4 | 0.4 | Generic | 3 |

9.20. Digital Skills

| S# | Competency Standard | Theory | Practical | Total | Credit Hour | Category | Level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 12 | 48 | 60 | 6 |  |  |
| 9.20.1. | Use computer operating systems and hardware | 0.5 | 3 | 3.5 | 0.35 | Generic | 3 |
| 9.20.2. | Operate digital media technology | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.3. | Perform computer operations | 0.5 | 3 | 3.5 | 0.35 | Generic | 3 |
| 9.20.4. | Use computer applications | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.5. | Create user documentation | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.6. | Create technical documentation | 0.5 | 3 | 3.5 | 0.35 | Generic | 3 |
| 9.20.7. | Create basic databases | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.8. | Use social media tools for collaboration and engagement | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.9. | E-Commerce- SEO (Search Engine Optimization) | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.10. | E-Commerce- SCM (Supply Chain Management) | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.11 | E-Commerce- Social Media Marketing | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.12 | Use digital devices | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.13. | Operate word-processing applications | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.14. | Operate spreadsheet applications | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.15. | Operate presentation packages | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.16. | Perform writing and editing tasks | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.17. | Write and Edit Copy | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.18. | Build Java applets | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.19. | Manipulate Images (Illustrator) | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.20. | Manipulate Images (Photoshop) | 0.5 | 2 | 2.5 | 0.25 | Generic | 3 |
| 9.20.21. | Design simple web page layouts | 1 | 2 | 3 | 0.3 | Generic | 3 |
| 9.20.22. | Apply introductory programming techniques | 1 | 3 | 4 | 0.4 | Generic | 3 |

# Date of Validation

The level 5 of National DAE qualification on Civil Technology has been validated by the Qualifications Development Committee (QDC) members on 27-29 May 2019 and will remain in currency until May, 2022

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Civil Technology level-5 |  |
| --- | --- |
| Code | Description |
| 000000000 | 1st Semester National Qualification Certificate of level-5, in “ Civil Technology” |
| 000000000 | 2nd Semester National Qualification Certificate of level-5, in “ Civil Technology” |
| 000000000 | 3rd Semester National Qualification Certificate of level-5, in “ Civil Technology” |
| 000000000 | 4th Semester National Qualification Certificate of level-5, in “ Civil Technology” |
| 000000000 | 5th Semester National Qualification Certificate of level-5, in “ Civil Technology” |
| 000000000 | 6th Semester National Qualification Certificate of level-5, in “ Civil Technology” |

# Members of Qualifications Development Committee

The following members participated in the qualifications development and validation of these qualifications:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Azhar Iqbal Shad | Chief Instructor (Civil) | GCT Raiwnd Road, LHR |
| 2 | Engr. Mazhar Abbas Naqvi | Principal | GSTC, Faisalabad |
| 3 | Engr. Naveed Ahmad Ajmal | Sr. Instructor (Civil) | GCT Bahalpur |
| 4 | Imtiaz Ahmed Awan | Sr. Instructor (Civil) | GCT Rasul (M.B.Din) |
| 5 | Muhammad Amjad Rafique | Sr. Instructor (Civil) | GCT Rasul (M.B.Din) |
| 6 | Engr.Zuneera Ashfaq | Asstt Director | PITAC Lahore |
| 7 | Engr Rehan Younis | Asstt Director | FWO- Rawalpindi |
| 8 | Engr. Mohsin Jahanzeb | Instructor (Civil) | GSTC, Faisalabad |
| 9 | Muhammad Asad Mahmood | Contractor | G.Five Construction Co. Faisal Abad |
| 10 | Engr. Nosheen Amina | Consultant | Allied Engg. Services Lahore |
| 11 | Hira Ishtiaq | Coordinator Engr. | Allied Engg. Services Lahore |
| 12 | Engr. Hafsah Khalid | Project Manager | ANS Associates Lahore |
| 13 | Engr. Habiba Mohsin | Project Manager | Jers Engg. Co. Lahore |
| 14 | Mian Muhammad Naeem | Consultant Engr. | Mian Associates Lahore |
| 15 | Engr. Rana Haroon Mujahid | SDO, Bridges & Roads | C&W Faisalabad |
| 16 | Bushra Asghar | Instructor | GCT (w) Lahore |
| 17 | Engr. Inayet ur Rehman | DACUM, Facilitator | Peshawar |
| 18 | Muhammad Ishaq | QDC Coordinator | NAVTTC, Islamabad |

Members of Qualifications Validation Committee

The following members participated in the qualifications development and validation of these qualifications:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Azhar Iqbal Shad | Chief Instructor (Civil) | GCT Raiwnd Road, LHR |
| 2 | Engr. Mazhar Abbas Naqvi | Principal | GSTC, Faisalabad |
| 3 | Mr. Imtiaz Ahmed Awan | Sr. Instructor (Civil) | GCT Rasul (M.B.Din) |
| 4 | Mr. Tariq Ali | A / Professor (Civil) | GPI Batkhela KP-TEVTA |
| 5 | Mr. Fida Hussain Mirjat | Principal | GMI Matli Hyderabad |
| 5 | Mr. Muhammad Khurram Murad | DCE PBTE | Lahore |
| 6 | Engr. Inayat ur Rehmen | DACUM Facilitator | Ex. KP-TEVTA |
| 7 | Mr. Sikander Masood | Director SS & C. | NAVTTC Islamabad |

# Entry Requirements

The entry for D.A. E National Certificate level 5, in Civil Technology are

- A person having National Vocational Certificate level 4, in Civil Technology.

- A person having Matric certificate with Science subjects

# Proposed Scheme of Studies

1st Semester (000000000) National Qualification Certificate of level-5, in “Civil Technology”

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  |
| 000000000 | Civil Engineering Practice (W.W & ET) | Technical | 31 | 99 | 130 | 13 |
| 000000000 | Plumbing | Technical | 12 | 48 | 60 | 6 |
| 000000000 | Building Materials | Technical | 32 | 48 | 80 | 8 |
| 000000000 | Basic Engineering Survey | Technical | 29 | 51 | 80 | 8 |
| 000000000 | Basic Engineering Drawing | Technical | 10 | 90 | 100 | 10 |
| 000000000 | Digital Skills | Generic | 12 | 48 | 60 | 6 |
| 000000000 |  | Generic |  |  |  |  |
| 000000000 |  | Functional |  |  |  |  |
| Total |  |  | 126 | 384 | 510 | 51 |

2nd Semester (000000000) National Qualification Certificate of level-5, in “Civil Technology”

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  |
| 000000000 | Construction Techniques-I | Technical | 37 | 63 | 100 | 10 |
| 000000000 | Contouring & Levelling | Technical | 45 | 85 | 130 | 13 |
| 000000000 | Building Drawing | Technical | 10 | 90 | 100 | 10 |
| 000000000 | Quantity Surveying-I | Technical | 18 | 82 | 100 | 10 |
| 000000000 | AutoCAD Civil | Technical | 20 | 60 | 80 | 8 |
| 000000000 | Soft Skills | Generic | 14 | 46 | 60 | 6 |
| 000000000 |  | Generic |  |  |  |  |
| 000000000 |  | Functional |  |  |  |  |
| Total |  |  | 144 | 426 | 570 | 57 |

3rd Semester (000000000) National Qualification Certificate of level-5, in “Civil Technology”

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  |
| 000000000 | Theodolite Surveying | Technical | 32 | 98 | 130 | 13 |
| 000000000 | Construction Techniques-II | Technical | 37 | 63 | 100 | 10 |
| 000000000 | Water Supply | Technical | 32 | 48 | 80 | 8 |
| 000000000 | Quantity Surveying-II | Technical | 15 | 55 | 70 | 7 |
| 000000000 | Structural Engineering Drawing | Technical | 10 | 90 | 100 | 10 |
| 000000000 | Strength of Materials | Technical | 32 | 48 | 80 | 8 |
| 000000000 |  | Generic |  |  |  |  |
| 000000000 |  | Functional |  |  |  |  |
| Total |  |  | 158 | 402 | 560 | 56 |

4th Semester (000000000) National Qualification Certificate of level-5, in “Civil Technology”

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  |
| 000000000 | Construction Techniques-III | Technical | 31 | 69 | 100 | 10 |
| 000000000 | Hydraulics Engineering | Technical | 32 | 48 | 80 | 8 |
| 000000000 | Quantity Surveying III | Technical | 17 | 103 | 120 | 12 |
| 000000000 | Digital Surveying | Technical | 32 | 88 | 120 | 12 |
| 000000000 | Engineering Mechanics | Technical | 32 | 48 | 80 | 8 |
| 000000000 | Construction Project Planning Management | Technical | 32 | 48 | 80 | 8 |
| 000000000 |  | Generic |  |  |  |  |
| 000000000 |  | Functional |  |  |  |  |
| Total |  |  | 176 | 404 | 580 | 58 |

5th Semester (000000000) National Qualification Certificate of level-5, in “Civil Technology”

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  |
| 000000000 | Sanitary Engineering | Technical | 32 | 48 | 80 | 8 |
| 000000000 | Concrete Technology | Technical | 32 | 48 | 80 | 8 |
| 000000000 | Tendering & Accounting | Technical | 32 | 48 | 80 | 8 |
| 000000000 | Environment Health & Safety | Generic | 32 | 48 | 80 | 8 |
| 000000000 | Bridge Engineering | Technical | 32 | 48 | 80 | 8 |
| 000000000 | Soil Mechanics | Technical | 32 | 48 | 80 | 8 |
| 000000000 |  | Generic |  |  |  |  |
| 000000000 |  | Functional |  |  |  |  |
| Total |  |  | 192 | 288 | 480 | 48 |

6th Semester (000000000) National Qualification Certificate of level-5, in “Civil Technology”

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  |
| 000000000 | RCC Design | Technical | 49 | 51 | 100 | 10 |
| 000000000 | Highways, Airport & Traffic Engg | Technical | 49 | 51 | 100 | 10 |
| 000000000 | Railways, Dock & Harbours | Technical | 49 | 51 | 100 | 10 |
| 000000000 | Civil Engineering Project | Technical | 0 | 100 | 100 | 10 |
| 000000000 | Irrigation Engineering | Technical | 32 | 48 | 80 | 8 |
| 000000000 | Entrepreneurship | Generic | 15 | 45 | 60 | 6 |
| 000000000 |  | Generic |  |  |  |  |
| 000000000 |  | Functional |  |  |  |  |
| Total |  |  | 194 | 346 | 540 | 54 |

- Building Materials & Construction Techniques

### Identify bricks

Overview: This competency standard covers the skills and knowledge required to interpret brick is a man-made building material used to make walls and make places to walk. It is a single unit of a kneaded clay-bearing soil, sand and lime, or concrete material, fire-hardened or air-dried, used in masonry construction. Bricks are made mostly of clay. Bricks used outdoors on the ground are called "pavers".

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1. Inspect Brick Kiln | P1. Identify constituents materials P2. Ensure mixing proportions P3. Inspect brick making method P4. Prepare Report. |
| CU-2. Inspect Pre-Cast block unit factory | P1. Identify manufacturing items P2. Identify constituents materials P3. Ensure mixing proportions P4. Record making process |
| CU-3. Inspect a quarry / crusher | P1. Inspect raw material. P2. Separate aggregate types P3. Record the crushing procedure. P4. Prepare Report |
| CU-4. Inrerpret brick manufacturing process | P1. Identify raw materials P2. Mixed raw material P3. Check ingredients. P4. Clean & filled mould P5. Empty mould P6. Dry bricks P7. Cook bricks in kiln P8. Use PPEs |
| CU-5. Test bricks | P1. Select bricks P2. Find compressive strength of selected bricks P3. Find water absorption of selected bricks P4. Check dimensional tolerance of selected bricks P5. Find efflorescence of selected bricks P6. Perform hardness test on selected bricks P7. Perform abrasion test on selected bricks P8. Check structure of selected bricks |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard.

This includes the knowledge of:

K1.  Explain Uses of different building materials.

K2.  Describe and explain the Method to prepare brick kiln

K3.  Describe essential need for aggregate

K4.  Explain the quarry and its working

K5. Explain demand of water for different building materials

K6. Explain variation in size of bricks

K7. Describe optimizing quality of crush

K8. Explain network for different building materials

K9. Explain safety precautions for laying, preparing brick & aggregate

K10. Explain quarry and its type.

K11. Explain blasting method

K12. Explain manufacturing of bricks

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare layout of brick kiln

- Preparation plan to laying of brick at construction site as per standards

- Apply method to prepare a kiln

- Identify Method to carry out quarry process

- Check construction specification for bricks and aggregates.

- Apply variation for brick manufacturing

- Apply brick manufacturing process

- Apply safety precautions

- Apply sieving process at quarry

- Be able to

  - Find compressive strength of selected bricks

  - Find water absorption of selected bricks Check ingredients.

  - Check dimensional tolerance of selected bricks

  - Find efflorescence of selected bricks

  - Perform hardness test on selected bricks

  - Perform abrasion test on selected bricks

  - Check structure of selected bricks

TOOLS AND EQUIPMENT’S

- Brick Making Machine

- Brick Molds

- Hand roller

- Thermometer

- Umbrella with stand

- Spirit level

- PPEs

### Dress stone for masonry

Overview: This competency standard covers the skills and knowledge required to perform roughly dressed stones are laid in a mortar the result is a stone rubble masonry and Ashlar Masonry. Stone masonry using dressed (cut) stones is known as ashlar masonry. Stone Veneer. Stone veneer is used as a protective and decorative covering for interior or exterior walls and surfaces.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Inspect dressing of natural stone | P1. Identify materials P2. Observe dressing process P3. Record the process |
| CU-2. Interpret Stone masonry (Rubble) | P1. Identify Materials P2. Pick Rough stones P3. Prepare cement sand mortar P4. Perform alignment P5. Lay stones in layers P6. Carry out Joints filling with mortar P7. Perform safety precautions |
| CU-3. Interpret Stone masonry (Ashlar) | P1. Identify Materials P2. Sharpen edge of Rough stones P3. Prepare cement sand mortar P4. Perform alignment P5. Lay stones in proper bond P6. Perform safety precautions |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1.Comprehend the purpose and application of stone masonry

K2 Knowledge for method to apply rough stone masonry

K3. Explain method to apply ashlar stone masonry

K4- Describe working of stone masonry precautions.

K5.  Explain different type of stones used in masonry

K6. Describe different jointing methods of stones

K7. Describe safety procedure for joining of stones

K8.  Explain filling holes method in rough stone masonry

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Define Joints in stone masonry.

- Draw sketch of different stone masonry work.

- Interpret different stone masonry procedures

- Calculate quantity for stone masonry

- Plan safety procedure during construction of stone masonry

- Plan the steps to joining of stones in layers as Rubble

- Adopt safety precautions while construction of sewers.

TOOLS AND EQUIPMENT’S

- Calculator

- Thermometer

- Stone masonry instruments

- Scissor

- trowel

- Umbrella with stand

- Spirit level

- PPEs

- Hammer

- Chisels

### Prepare slaked lime

Overview:  This competency standard covers the skills and knowledge required apply white alkaline substance consisting of calcium hydroxide, made by adding water to quicklime and used in traditional building methods to make plaster, mortar, and lime wash it Quicklime reacts with water to form slaked lime (calcium hydroxide). Slaked lime is used to reduce the acidity of lakes and soils. Lakes and soils can become acidic because of acid rain.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Prepare slacked lime | P1. Identify requisite material P2. Spread lime at ponds P3. Add water P4. Administer change in lime P5. Check strength |
| CU-2. store quick lime | P1. Classify lime as per use P2. Identify go down for storing lime P3. Prepare lime putty to store lime |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard.

This includes the knowledge of:

K1-Describe lime with its properties

K2- Explain Method to convert quick lime into slaked lime.

K3- Explain procedure to get lime from quarry.

K4. Describe design procedure for maintaining a quarry

K5. Explain safety precaution to get lime from quarry

K6. Explain PPEs

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions at quarry

- Maintain safety precautions to convert lime property

- Handling the quarry process

- Design the lime properties

- Execute and plan to change lime property

TOOLS AND EQUIPMENT’S

- Spirit level

- PPEs

- Kassie

- Thermometer

- Lime beds

- Umbrella with stand

### Identify and store cement for construction

Overview: This competency standard covers the skills and knowledge required for basics as construction material powdery substance made by calcining lime and clay, mixed with water to form mortar or mixed with sand, gravel, and water to make concrete. Store cement in a building which is dry, leak proof and as moisture proof as possible. There should be minimum number of windows in the storage building. Stack the cement bags off the floor on wooden planks in such a way, so that it is about 150 mm to 200 mm above the floor.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Develop Flow diagram for manufacturing of cement | P1. Identify drawing instruments P2. Fix Drawing sheet on board P3. Select scale P4. Use safety precautions for drawing P5. Draw flow diagram |
| CU-2. Manage storage process for cement | P1. Identify material P2. Use PPEs P3. Locate moisture proof place P4. Stacked cement bags P5. Cover with plastic sheet |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard.

This includes the knowledge of:

K1-Describe cement storage process

K2- Explain Method to cover the cement in moisture proof place.

K3- Explain procedure to get cement from moisture proof room.

K4. Describe design procedure for maintaining a moisture proof room

K5. Explain safety precaution to get and use of cement from moisture proof room

K6. Explain PPEs

K7. Describe line / block diagram for manufacturing of cement

K8. Explain parts of cement manufacturing plant

K9. Explain working of rotary kiln

K10. Explain temperature zone at rotary kiln.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions at quarry

- Maintain safety precautions at cement manufacturing plant

- Handling the quarry process

- Design the cement process at plant

- Execute and plan to change cement into different categories

- Manage and Plan different type of cement at plant

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Calculator,

- Measuring scale,

- Drawing sheet,

- Measuring Tape,

- Compass,

- Spirit level

- PPEs

- Thermometer

- Umbrella with stand

### Select sand for construction

Overview: This competency standard covers the skills and knowledge required for basics as construction granular material composed of finely divided rock and mineral particles. It is defined by size, being finer than gravel and coarser than silt. Desert sand, although plentiful, is not suitable for concrete, and 50 billion tons of beach sand and fossil sand is needed each year for construction.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Interpret acquire Sand | P1. Identify material P2. Select quarry P3. Manage quarrying process P4. Use safety precautions for drawing P5. Produce sand |
| CU-2. Test sand for use | P1. Select sand from heap P2. Find moisture content in sand P3. Determine bulking of sand P4. Find clay content in selected sand P5. Find salts in sand Find size of sand grains.? |
| CU-3. Manage storage process of sand at site | P1. Identify material P2. Use PPEs P3. Locate safe place P4. Dump sand P5. Cover with plastic sheet |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard.

This includes the knowledge of:

K1-Describe sand storage process

K2- Explain Method to cover the sand and water sprinkling on sand.

K3- Explain procedure to get sand from quarry

K4. Describe design procedure for maintaining quarry for sand

K5. Explain safety precaution to get and use of sand at construction site

K6. Explain PPEs

K7. Describe type of sand ingredients.

K8. Explain sieving of sand

K9. Explain working of sand in concrete

K10. Explain working temperature of concrete.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions at quarry

- Maintain safety precautions at sand quarrying site

- Handling the quarry process

- Design the sand ingredients size

- Execute and plan to change concrete ratio with sand

- Manage and Plan different type of sand at site

TOOLS AND EQUIPMENT’S

NIL

### Perform concreting

Overview: This competency standard covers the skills and knowledge required for basics as construction material made from a mixture of broken stone or gravel, sand, cement, and water, which can be spread or poured into moulds and forms a mass resembling stone on hardening. It is an important construction material used extensively in buildings, bridges, roads and dams. Its uses range from structural applications, to paviors, kerbs, pipes and drains. Concrete is a composite material, consisting mainly of Portland cement, water and aggregate

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Prepare concrete mixture | P1. Read, and interpret drawing / layout and specification according to the job requirements P2. Select materials according to job requirements. P3. Select and use PPE as per the job requirements. P4. Select and use required tools and equipment as per the job requirements. P5. Prepare materials as per the job requirement following standard procedures. P6. Mix materials as per the job requirement following standard procedures. P7 Transport concrete mix as per the job requirement following standard procedures |
| CU-2. Place, screed and level concrete | P1. Place concrete in horizontal layers into location to marked out levels. P2. Minimize the height of vertical drop of concrete to avoid segregation of concrete materials. P3. Consolidate poured concrete during process using compaction or vibration method P4. Check finished levels using appropriate levelling device. P5. Screed concrete to correct levels and grades using appropriate straight edged tool/formwork-mounted screed. |
| CU-3. Clean-up | P1. Clean work area and waste disposed of, reused or recycled in accordance with work specifications P2. Clean tools and equipment, check for faults and maintain and store in accordance with workplace practices. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1-Describe water cement ratio

K2- Explain Method to mix the concrete.

K3- Explain procedure to get cement concrete from mixer.

K4. Describe design procedure of shoot to transfer the concrete

K5. Explain safety precaution to spread and mix the concrete

K6. Explain PPEs

K7. Describe line diagram to prepare concrete

K8. Explain type of curing for concrete

K9. Explain safe working with concrete at site

K10. Explain temperature zone for concreting.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions for concreting at site

- Maintain safety precautions while preparing of concrete

- Handling the concrete process

- Design the concrete ingredients

- Execute and plan to change platform and shoots for concreting at site

- Manage and Plan different type of concrete at site

TOOLS AND EQUIPMENT’S

- Float Blades

- Troweling Blades

- Groove Cutter

- Jointer Tool

- Edger

- Brooms

- Polishers

- Grinders

- Trowel

- Float

- Vibrator

- Form work

- Moulds

- Workability test apparatus

- Compass,

16.             Spirit level

- PPEs

- Thermometer

- Umbrella with stand

- Vibrating Rod

### Cut and core concrete

Overview: This competency standard covers the skills and knowledge required for Concrete cutting process of controlled sawing, drilling and removal of concrete performed by skilled operators using special saws that use diamond impregnated blades. There are many different kinds of concrete cutting but the most common are wall sawing, core drilling and slab or flat sawing. Concrete cutting involves a variety of techniques used by professionals to efficiently and safely cut large, square-or rectangle-shaped incisions into wall concrete or slab concrete. Core drilling is the process of making precise circular cuts in concrete to make smooth openings for a variety of applications.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Perform cutting of concrete | P1. Identify materials P2. Identify instruments P3. Adopt safety precautions P4. Measure size as per design P5. Use saw & drill P6. Test results P7. Use PPEs |
| CU-2. Manage core concrete | P1. Identify materials P2. Identify instruments P3. Adopt safety precautions P4. Measure size P5. Use saw & drill P6. Test results P7. Use PPEs |
| CU-3 Clean-up | P1. Clear work are by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1-Describe cutting of concrete methods

K2- Explain Method to fix the saw and drill to concrete.

K3- Explain procedure to get fix concrete part to cutting.

K4. Describe design procedure for cutting of concrete

K5. Explain safety precaution to cut the concrete

K6. Explain PPEs

K7. Explain type of curing for concrete

K9. Explain safe working process to cut concrete at site

K10. Explain temperature zone for concreting.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions for concrete cutting

- Maintain safety precautions while cutting of concrete

- Handling the concrete cutting tools and equipment

- Design the concrete ingredients

- Execute and plan to change tools for cutting design while working at site

- Manage and Plan different type cutting instrument for concrete at site

TOOLS AND EQUIPMENT’S

- Float Blades

- Core cutting machine

- Groove Cutter

- Jointer Tool

- Core cutting saw

- Drill Machines

- Polishers

- apparatus

- Compass,

- Spirit level

- PPEs

- Thermometer

- Umbrella with stand

### Identify ferrous and non-ferrous metals

Overview: This competency standard covers the skills and knowledge required for Non-Ferrous Metals do not contain Iron, are not magnetic and are usually more resistant to corrosion than ferrous metals. Some examples of Non-Ferrous Metals we deal which are Aluminum, Aluminum Alloys and Copper.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Identify ferrous metals | P1. Identify materials P2. Select ferrous metals P3. Check iron particles P4. Perform magnetic operation P5. Use PPEs P6. Perform results P7. Use PPEs |
| CU-2. Identify Non-ferrous metals | P1. Identify materials P2. Select ferrous metals P3. Check iron particles P4. Perform magnetic operation P5. Use PPEs P6. Perform results |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1-Describe ferrous and nonferrous metals

K2- Explain Method to perform strength for metals.

K3- Explain different alloys

K4. Describe design procedure for manufacturing of alloys

K5. Explain safety precaution to manufacturing alloys

K6. Explain PPEs

K7. Explain type of nonferrous metals

K9. Explain safe working process to cut the metals

K10. Explain temperature zone for cutting metal.

K11.  Explain Annealing process

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions for cutting of metals

- Maintain safety precautions while alloys process

- Handling the alloy process at plant

- Design the alloy for better performance

- Execute and plan to change the heat while alloying

- Manage and Plan different type alloy processing at plant

TOOLS AND EQUIPMENT’S

  - Sheet plate

  - Plate bar

  - Plate rod

  - Channels

  - Forgings area

  - Casting platform

  - Furnace

  - Handling equipment

  - Float Blades

  - Core cutting machine

  - Groove Cutter

  - Jointer Tool

  - Drill Machines

  - Polishers

  - PPEs

  - Thermometer

  - Umbrella with stand

### Identify advanced construction materials

Overview: This competency standard covers the skills and knowledge required Traditional construction materials, such as timber, steel, asphalt and Portland cement concrete are often used in many construction projects. Modern materials, such as. Polymers and composites are making headway into the construction industry.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Identify Timber | P1. Identify materials P2. Select color for wood P3. Count annual rings P4. Check pith & rays P5. Select bark & sap wood P6. Perform results |
| CU-2. Identify steel | P1. Identify materials - P2. Perform Tensile Strength. - P3. Perform Ductility. - P4. Check Malleability. - P5. Monitor Durability. - P6. Identify Conductivity. - P7. Check Luster P8. Use PPEs |
| CU-3. Identify asphalt | P1. Identify materials - P2. Perform Brittleness - P3. Perform Ductility - P4. Check Hardening - P5. Monitor Viscosity - P6. Identify Conductivity - P7. Check Luster P8. Use PPEs |
| CU-4. Identify cement | - P1. Indicate material - P2. Check Fineness - P3. Administer Soundness. - P4. Monitor Consistency. - P5. Check Strength. - P6. Supervise Setting time. - P7. Check Heat of hydration. - P8. Study Loss of ignition. - P9. Perform Bulk density. P10.Use PPES |
| CU-5. Identify glass | P1. Identify material P2. Check transparency, P3. Perform heat resistance, P4. Check pressure P5. Indicate breakage resistance P6. Check chemical resistance. P7. Use PPEs |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1-Describe different construction material

K2- Explain Method to perform strength for different materials

K3- Explain different material properties

K4. Describe design procedure for manufacturing different materials

K5. Explain safety precaution to manufacturing and finishing of materials

K6. Explain PPEs

K7. Explain type of nonferrous metals

K9. Explain safe working process to cut the metals and timber

K10. Explain temperature zone for cutting different materials

K11.  Explain Annealing process for metals

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions for cutting of metals

- Maintain safety precautions while alloys process

- Handling the alloy process at plant

- Design the alloy for better performance

- Execute and plan to change the heat while alloying

- Manage and Plan different type alloy processing at plant

- Manage to cut the timber

- Plan Designing in timber process

### Work Safely in Construction Industry

Overview: This competency standard covers the skills and knowledge required to identify OHS legislative requirements, identify construction hazards and control measures, identify OHS communication and reporting processes and identify OHS incident response procedures.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Identify OHS legislative requirements. | Identify and explain applicable OHS legislative requirements relevant to own work, role and responsibilities. Identify duty of care requirements. Identify and explain Own responsibilities to comply with safe work practices |
| - Identify construction hazards and control measures. | Identify basic principles of risk management. Identify & discuss Common construction hazards. Identify Measures for controlling hazards and risks. |
| - Identify OHS communication and reporting processes. | Identify & discuss OHS communication processes, information and documentation. Identify & explain role of designated OHS personnel. Identify & explain Safety signs and symbols. Identify & discuss Procedures and relevant authorities for reporting hazards, incidents and injuries are identified. |
| - Identify OHS incident response procedures. | Identify & explain General procedures for responding to incidents and emergencies. Identify procedures for accessing first aid. Identify & demonstrate requirements for the selection and use of relevant personal protective equipment. Identify & discuss Fire safety equipment. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe applicable Commonwealth, State or Territory OHS legislation, regulations, standards, codes of practice and industry standards/guidance notes relevant to own work, role and responsibilities

- Describe basic principles of risk management and assessment for construction work.

- Describe common construction hazards, common construction safety signage and its meanings

- Describe general construction emergency response and evacuation procedures.

- Describe general construction work activities that require licenses, tickets or certificates of competency.

- Describe general first aid response requirements and general procedures for raising OHS issues.

- Explain general procedures for reporting OHS hazards, accidents, incidents, emergencies, injuries, near misses and dangerous occurrences.

- Describe general procedures for responding to hazards, incidents and injuries.

- Describe general workers' compensation and injury management requirements.

- Describe OHS hierarchy of controls, OHS responsibilities and rights of duty holders, including:

- Describe persons in control of construction work/projects- employers and self-employed persons, supervisors, employees, designers, inspectors and manufacturers and suppliers

- Describe own responsibilities to comply with safe work practices relating to: housekeeping, identification of hazards, preventing bullying or harassment, smoking, use of amenities, use of drugs and alcohol

- Describe role of OHS committees and representatives

- Describe types of common personal protective equipment and fire safety equipment

- Explain types of OHS information and documentation.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- applicable OHS legislative and safety requirements for construction work including duty of care

- the range of common construction hazards and procedures for the assessment of risk and application of the hierarchy of control

- OHS communication processes, information and documentation including the role of OHS committees and representatives, the meaning of common safety signs and symbols, and procedures for reporting hazards, incidents and injuries

- general procedures for responding to incidents and emergencies including evacuation, first aid, fire safety equipment and PPE.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |

### Interpret Simple Building Plans

Overview: This competency standard covers the skills and knowledge required to locate, interpret and apply relevant information standards of simple building plans,

| Competency Unit | Performance Criteria |
| --- | --- |
| - Identify types of drawings and their functions. | - Main types of plans and drawings used in the construction sector of the industry are identified. - Key features and functions of each type of drawing are identified. - Quality requirements of company operations are recognized and adhered to. - Environmental requirements and controls are identified from job plans, specifications and environmental plan. |
| - Recognize amendments. | - Title panel of project documentation is checked to verify latest amendments to drawing. - Amendments to specifications are checked to ensure currency of information and conveyed to others where appropriate. |
| - Recognize commonly used symbols and abbreviations. | - Construction symbols and abbreviations are recognized. - Legend is located on project drawings, and symbols and abbreviations are correctly interpreted. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- basic calculations of heights, areas, volumes and grades

- commonly used construction symbols and abbreviations

- construction terminology

- drawing conventions

- features of plans and elevations, including direction, scale, key, contours, symbols and abbreviations

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- locate, interpret and apply relevant information, standards and specifications

- comply with site safety plan, OHS regulations and state and territory legislation applicable to workplace operations

- communicate and work effectively and safely with others

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Calculator |
| 2 | Ruler |
| 3 | Pencil |

### Read and Interpret plans and Specifications

Overview: This competency standard covers the skills and knowledge required to read, interpret plans and specifications.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Identify types of drawings and their functions. | - Main types of plans and drawings used in the construction sector of the industry are identified. - Key features and functions of each type of drawing are identified. - Quality requirements of company operations are recognized and adhered to. - Environmental requirements and controls are identified from job plans, specifications and environmental plan. |
| - Recognize amendments. | - Title panel of project documentation is checked to verify latest amendments to drawing. - Amendments to specifications are checked to ensure accuracy of information and conveyed to others where appropriate. |
| - Recognize commonly used symbols and abbreviations. | - Construction symbols and abbreviations are recognized. - Legend is located on project drawings, and symbols and abbreviations are correctly interpreted. |
| - Locate and identify key features on a site plan. | - Orientation of the plan with the site is achieved. - Key features of the site are identified and located. - Access to site is gained and services, main features, contours and datum are identified. |
| - Identify project requirements. | - Dimensions for project and nominated locations are identified. - Construction types and dimensions for nominated locations are identified. - Environmental controls and locations are identified. - Location, dimensions and tolerances for ancillary works are identified. |

| - Read and interpret job specifications. | - Job specifications are identified from drawings, notes and descriptions. - Standards of work, finishes and tolerances are identified from the project specifications. - Material attributes are identified from specifications. |
| --- | --- |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- basic calculations of heights, areas, volumes and grades

- commonly used construction symbols and abbreviations

- construction terminology

- drawing conventions

- features of plans and elevations, including direction, scale, key, contours, symbols and abbreviations

- job safety analysis (JSA) and safe work method statements

- key features of formal job specifications

- processes for application of scales in plan preparation and interpretation

- project quality requirements

- site and equipment safety (OHS) requirements

- Techniques for orienting/confirming the orientation of a plan.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- locate, interpret and apply relevant information, standards and specifications

- comply with site safety plan, OHS regulations and state and territory legislation applicable to workplace operations

- communicate and work effectively and safely with others for a minimum of two different projects, read and interpret the project plans, including:

- confirmation of amendment status and drawings confirmed 'for construction'

- orientation of plans to the ground

- six key features on both the plan and the site

- confirmation of six items of information from the title block of the project plans

- six construction dimensions, levels and locations from the project plans

- Six ancillary works dimensions, levels and locations from the project plans for a minimum of two formal specifications, identify the dimensions, material requirements and processes to be followed.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Calculator |
| 2 | Ruler |
| 3 | Pencil |

### Apply power and hand tools in construction.

Overview: This competency standard covers the skills and knowledge required to identify and select hand, power and pneumatic tools for given tasks and safely use and maintain a minimum of rule, tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock and crow bar and pinch bar for given tasks.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Safety requirements, and warnings, including use of personal protective equipment, are observed throughout the work. |
| - Identify and select hand, power and pneumatic tools | - Hand tools and power and pneumatic tools, their functions, operations and limitations are identified and selected - Safety requirements are applied when using hand, power and pneumatic tools. - Pre-operational checks, including lubricants, hydraulic fluid and water, are completed according to manufacturer recommendations. |
| - Use tools safely and effectively | - Hand tools used are appropriate to the task and used effectively - Power and pneumatic tools are safely and effectively used in accordance with manufacturer recommendations. - Tools are sharpened and maintained. |
| - Identify, select and use plant and equipment | - Plant and equipment are selected and used consistent with OHS requirements and the needs of the job. - Pre-operational checks, including lubricants, hydraulic fluid and water, are completed. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain need for store in a project

- State the classification of stores- stock, tools and plants, Road metal and materials charged direct to the work.

- Construction terminology, particularly names of tools and equipment

- Manual handling and correct posture for tool and equipment use

- Personal protective equipment and reasons for its use

- Safety techniques for tool and equipment use

- Workplace and equipment safety requirements

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Identify and select hand, power and pneumatic tools for given tasks.

- safely use and maintain a minimum of rule, tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock and crow bar and pinch bar for given tasks.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 |  |

### Perform Simple Levelling

Overview: This competency standard covers the skills and knowledge required to Prepare for work, Perform manual leveling and Cleanup work area.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Prepare for work. | - Requirements of the job are determined and various levels are obtained. - Work health and safety (WHS) and environmental requirements associated with levelling activities are adhered to throughout the work. - Quality assurance requirements are identified and adhered to according to workplace requirements. - Tools and equipment, including personal protective equipment, are selected and checked for serviceability. |
| - Perform manual leveling. | - Height to be transferred is identified from drawings, plans or instructions. - Carryout hand levelling and operate according to manufacturer instructions. - Levels are taken and marked according to job requirements. |
| - Clean up work area. | - Work area is cleaned, and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- different types of levelling equipment, their applications and their method of operation

- how to access relevant information, including codes and technical standards

- job safety analysis (JSA) and safe work method statements (SWMS)

- process of establishing, and checking levels and alignment

- simple calculations relating to carrying out levelling

- SI system of measurements

- workplace and equipment safety requirements

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- locating, interpreting and applying relevant information, standards and specifications for the establishment, and checking of levels

- applying safety requirements throughout the work sequence, including electrical safety requirements and the use of personal protective clothing and equipment

- grading a pipe (or equivalent) over 10 meters on a grade to a tolerance of + or - 5mm, ensuring:

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 |  |

### Lay out Building plans

Overview: This competency standard covers the skills and knowledge required to identify and select hand, power and pneumatic tools for given tasks and safely use and maintain a minimum of rule, tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock and crow bar and pinch bar for given tasks.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare | - Work instructions, including plans, specifications, quality requirements and operational details, are obtained, confirmed and applied from relevant information for planning and preparation. - Safety (OHS) requirements are followed in accordance with safety plans and policies. - Signage and barricade requirements are identified and implemented. - Tools and equipment selected to carry out tasks are consistent with job requirements, checked for serviceability, and any faults are rectified or reported prior to commencement. - Material quantity requirements are calculated in accordance with plans, specifications and quality requirements. - Materials appropriate to the work application are identified, obtained, prepared, safely handled and located ready for use. - Environmental requirements are identified for the project in accordance with environmental plans and statutory and regulatory authority obligations, and are applied. |
| - Identify and indicate site boundaries. | - Survey pegs at corners of site are located and identified in accordance with job drawings, specifications and site topography. - String lines are set accurately into position to identify site boundary markings in accordance with site plan and survey pegs. |
| - Set out first line for building alignment. | - Measurements of building line from boundary or existing building are determined from site drawings for setting out. - Approximate position and length of line, plus building clearance measurement at each end, are determined for hurdle location in accordance with site plan and survey pegs. - Pegs and hurdles/profiles are installed approximately level across and between one another with adequate provision to mark footing width on hurdle/profile in accordance with job drawings and specifications. - Location for line is accurately marked with nails on hurdles/profiles and line is set taut into position to true alignment with boundary in accordance with job drawings and specifications without error |
| - Set out right angled corners. | - Corner of building is determined on set building line to true measurement from adjacent boundary and marked with peg in accordance with job drawings and specifications. - Right angle is set up to line from corner peg using triangulation principles. - Hurdles/profiles are installed to approximate level of other hurdles and line is set taut to right angled alignment. |
| - Install other building lines. | - Hurdles for remaining building lines are installed to appropriate locations, approximately level with established hurdles in accordance with job drawings and specifications. - Measurements for remaining building lines are accurately marked and nailed on hurdles to dimensions from site drawings. - String lines are set taut into position to nailed locations on hurdles in accordance with job drawings and specifications. |
| - Building lines are checked for square. | - Diagonal measurements are checked for square and lines are adjusted to provide square relationship within 5mm tolerance over minimum diagonal length of 15m. - Measurements are checked for accuracy. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- application and requirements for line, level and plumb in construction projects

- basic construction processes

- basic mathematical techniques associated with setting out

- construction plan, symbols and construction terminology

- construction terminology

- job safety analysis (JSA) and safe work method statements

- processes for interpreting engineering drawings and sketches

- processes for setting out

- project quality requirements

- setting out techniques

- site and equipment safety (OHS) requirements

- site isolation and traffic control responsibilities and authorities

- types, characteristics, technical capabilities and limitations of setting out devices.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- locate, interpret and apply relevant information, standards and specifications

- comply with site safety plan, OHS regulations and state and territory legislation applicable to workplace operations

- comply with organisational policies and procedures, including quality requirements

- safely and effectively use tools and equipment

- communicate and work effectively and safely with others

- Set out a full size L shape building on a relatively level site to specifications.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 |  |

### Carryout excavation and install supports

Overview: This competency standard covers the skills and knowledge required to excavate and install trench and excavation support on a new or existing site, in order to make provisions for footings/slabs or to provide/repair/divert services.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare | - Work instructions are confirmed with supervisor. - Material quantity requirements are calculated in accordance with specifications. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared |
| - Locate excavation site and erect safety equipment | - Excavation site is located, and line and depth are established from site plans and instructions. - Service points and the excavation limits are set and marked. - Temporary drainage system is established to divert surface and sub-surface water. - Service markers are identified and interference with underground services avoided. |
| - Dig Excavations | - Excavations are safely dug with hand tools to ensure correct route, line and depth. - Machine operator is assisted with excavation to ensure correct route, line and depth. - Trench or excavation support is installed if required. |
| - Perform Clean up. | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |
| - Plan and prepare for installing trench support | - Access, interpret and apply trench support documentation, and ensure the work activity is compliant - Obtain, read, interpret, clarify and confirm work requirements - Identify and address potential risks, hazards and environmental issues, and implement control measures - Select and wear personal protective equipment appropriate for work activities - Identify, obtain and implement traffic signage requirements Select, and check for faults, equipment and/or attachments for work activities - Obtain and interpret emergency procedures, and be prepared for fire/accident/emergency |
| - Install trench shoring | - Communicate with plant operator and ensure the excavation of trenches complies with site plan, line and depth - Determine and prepare shoring method - Set out positioning of shoring - Position/erect shoring within the trench - Secure shoring in position and ensure structural conformity with regulations - Clean out excavation - Locate ladders for safe access and egress |
| - Remove trench shoring | - Release jacking mechanisms and remove ladders - Check shoring and prepare it for lifting from the trench - Remove shoring from trench and store it |
| - Conduct housekeeping activities | - Clear work area and dispose of or recycle materials - Clean and maintain condition of equipment, ensure suitability for use, and address/report issues - Manage/report hazards, and maintain a safe working environment - Process written records |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Apply common skills and knowledge specified in the introduction to these RMCS Commonly used in-ground services

- Describe excavation materials

- Describe excavation techniques

- Interpret Plans, specifications and drawings

- Define Shoring

- Describe types & components of shoring

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- effectively use tools, plant and equipment; and

- Mark out and excavate site as to without damaging services.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Battens, props, planks for shoring |

### Place Concrete in Foundations

.

Overview: This competency standard covers the skills and knowledge required to identify and select hand, power and pneumatic tools for given tasks and safely use and maintain a minimum of rule, tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock and crow bar and pinch bar for given tasks.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Define and prepare work area. | - Concrete is received, checked for debris and discharged into wheelbarrow, kibble, pump or hopper. - Location of concrete placement is determined from plans and specifications and is checked to be free of debris and waste. |
| - Place, screed and level concrete | - Concrete is placed in horizontal layers into location to mark out levels. - Height of vertical drop of concrete is minimized to avoid - Segregation of concrete materials. - Poured concrete is consolidated during process using compaction or vibration method. - Finished levels are checked using appropriate levelling device. - Concrete is screened to correct levels and grades using appropriate straight edged tool/formwork-mounted screed. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Concrete ingredients ratio

- Batching and finishing techniques

- Concrete materials

- Concrete placement

- Curing times

- Levelling techniques

- Types, characteristics, uses and limitations of plant, tools and equipment

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- use a hand trowel and power trowel to finish a minimum of 10 square metres of concrete in foundation.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Chutes - Line pumps - Measuring tapes and rules, mechanized dumpers - Rakes - Screed boards, shovels, stipple devices - Trowels troweling machines - Vibrators - Wheelbarrows |

### Prepare for construction Process (Brick/ block laying)

Overview: This competency standard covers the skills and knowledge required to Plan work, Set out brickwork/ blockwork, Lay bricks/blocks forming steps and wing walls and Clean up

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Set out brickwork/ blockwork | - Work platform is erected in accordance with workplace requirements. - Location and relative level of prepared footing are checked from job drawings and specifications. - Rise and going of flight and individual steps are calculated to codes and workplace requirements. - Flight and individual steps are set out from calculations and job drawings. |
| - Lay bricks/blocks forming steps and wing walls | - Mortar mix is prepared to required conformity and bricks/blocks are laid to set out according to specifications. - Base brickwork is constructed to specifications. - Steps are formed square, level, plumb and true and laid to specified bond. - Profile of steps is constructed to bond and design, aligned and plumb to specifications. - Parallel wing walls are formed to step alignment in accordance with specifications. - Jointing is carried out to work specifications. - Brickwork/blockwork is laid to line, set out with gauge and completed to work specifications. - Brick/block faces are cleaned free of mortar. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Brick and block expansion and growth, control and articulation joints

- Brick bond patterns

- Characteristics and applications of materials for masonry steps and stairs construction

- Plans, specifications and drawings

- Techniques of masonry steps and stairs construction

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- construct a flight of stairs with a minimum of three pre-cast treads, to specifications; and

- Construct a flight of three solid masonry treads with a parallel wing wall on one side to specifications.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Bolsters, brick buggies and brick grabs, brooms, buckets, builders’ lines Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, materials hoists, measuring tapes and rules, mortar boards, Pallet trolleys, plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressors, spirit levels, straight edges, string lines, Trowels, Wheelbarrows |

### Mix Cementitious materials (Mortar & Concrete)

Overview: This competency standard covers the skills and knowledge required to Plan work, Set out brickwork/ blockwork, Lay bricks/blocks forming steps, wing walls and Clean up

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Set out brickwork/ blockwork | - Work platform is erected in accordance with workplace requirements. - Location and relative level of prepared footing are checked from job drawings and specifications. - Rise and going of flight and individual steps are calculated to codes and workplace requirements. - Flight and individual steps are set out from calculations and job drawings. |
| - Lay bricks/blocks forming steps and wing walls | - Mortar mix is prepared to required conformity and bricks/blocks are laid to set out according to specifications. - Base brickwork is constructed to specifications. - Steps are formed square, level, plumb and true and laid to specified bond. - Profile of steps is constructed to bond and design, aligned and plumb to specifications. - Parallel wing walls are formed to step alignment in accordance with specifications. - Jointing is carried out to work specifications. - Brickwork/blockwork is laid to line, set out with gauge and completed to work specifications. - Brick/block faces are cleaned free of mortar. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Brick and block expansion and growth, control and articulation joints

- Brick bond patterns

- Characteristics and applications of materials for masonry steps and stairs construction

- Plans, specifications and drawings

- Techniques of masonry steps and stairs construction

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- construct a flight of stairs with a minimum of three pre-cast treads, to specifications; and

- Construct a flight of three solid masonry treads with a parallel wing wall on one side to specifications.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Bolsters, brick buggies and brick grabs, brooms, buckets, builders’ lines Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, materials hoists, measuring tapes and rules, mortar boards, Pallet trolleys, plumb rules, profiles, Scaffolds, shovels, small petrol or diesel engines and compressors, spirit levels, straight edges, string lines, Trowels Wheelbarrows |

### Install flashing and damp proof course

Overview: This competency standard covers the skills and knowledge required install flashings and damp proofing products to different types and styles of buildings.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Prepare for work | - Type of flashing and damp proofing material are identified in accordance with and state of structure. - Area of structure for damp proofing is identified from or site inspection and inspected for defects. - Defects are corrected and surface preparation of structure requiring damp proofing is carried out to manufacturer specifications |
| - Install damp proof course | - Damp proof is installed in accordance with work specifications. - Flashing or damp proof course is laid and lapped in accordance with specifications. - Damp proof material is applied with a consistent mortar bed on top and bottom. - Damp proof course is folded to follow shape of surrounding structures. |
| - Install moisture proof barrier or flashings | - Flashing materials are prepared for application to surrounding structures in accordance with requirements. - Flashing material is laid, lapped and joined to follow shape of surrounding structure. - Flashings or moisture barrier material is formed and sealed around openings. - Flashing and damp proof course are installed to project outside of mortar joint on external surface, and outside of brickwork in accordance with specifications. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Characteristics and applications of materials to install flashings and DPC

- Techniques for installing flashings and DPC

- Waterproofing methods

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Install, to specifications, flashings of suitable materials for cavity brickwork including internal and external corners, window and door heads and vertical flashings; and stepped and tray flashings to gables and/or parapets.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Bolsters, brick buggies and brick grabs, brooms, buckets, builders’ lines Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, materials hoists, measuring tapes and rules, mortar boards, Pallet trolleys, plumb rules, profiles, Scaffolds, shovels, small petrol or diesel engines and compressors, spirit levels, straight edges, string lines, Trowels Wheelbarrows |

### Carry out brick work

Overview: This competency standard covers the skills and knowledge required to lay and construct brick and masonry buildings.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Set out brickwork/ blockwork | - Bricks/blocks are identified, selected and checked for conformity with specifications. - Work platform is erected in accordance with workplace requirements. - Location and structural details of brickwork/blockwork are identified from plans. - Base brickwork construction, below and above floor is set out to location, dimensions and specifications. |
| - Install damp proof course | - Mortar mix is prepared and checked for conformity and bricks/blocks laid to set out to specification. - Brickwork/blockwork gauge is determined and set out rod is prepared. - Base brickwork/blockwork is constructed for veneer construction to requirements of regulations and specifications. - Sub-floor ventilation is installed in accordance with specifications. |
| - Construct Veneer Walls | - Structural frame is checked to ensure it is ready for brick or block veneer construction maintaining minimum cavity. - Damp proof courses are installed to specifications. - Ventilation for veneer construction is built to specifications. - Wall ties are positioned and correctly fixed to framework to specifications. - Openings are constructed and flashings installed. - Cavities are kept clear of mortar droppings and bridging. - Lintels are installed. - Top brickwork/blockwork is constructed to eaves/gable level in accordance with standards. - Veneer gable is constructed as required by plans and specifications. - Walls are built to gauge straight and true in plumb, line and level within standards tolerance. - Control joints are formed in accordance with locations on job drawings and specifications. - Weep holes, brick/block reinforcing and wall flashing are located and built in to. - Sill bricks are cut where required and laid to line in accordance with work specifications. |
| - Construct Veneer Walls | - Joints of laid brickwork/blockwork are raked or ruled to correct depth and profile in accordance with work specifications. - Brickwork/blockwork is brushed down prior to drying to remove unwanted mortar and face is cleaned. - Excess mortar is removed from brick/blockwork surfaces and cavities are cleaned free of mortar and debris. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Brick and block expansion and growth, control and articulation joints

- Brick bond patterns

- Characteristics and applications of materials for masonry veneer construction

- Techniques of masonry veneer construction

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- construct a section of a brick veneer building, including set out and gauge brickwork to a base; damp proof course (DPC) if applicable, weep holes, air vents, control joints and veneer ties; lintels and flashings; brick gables; and brick sills and flashings, completing all to specifications.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Bolsters, brick buggies and brick grabs, brooms, buckets, builders’ lines Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, materials hoists, measuring tapes and rules, mortar boards, Pallet trolleys, plumb rules, profiles, Scaffolds, shovels, small petrol or diesel engines and compressors, spirit levels, straight edges, string lines, Trowels Wheelbarrows |

### Construct Cavity Wall

Overview: This competency standard covers the skills and knowledge required to prepare for work, set out brickwork/ blockwork, construct base of brickwork/ blockwork, position door and window frames, construct brick walls, rake or rule joints, and clean up.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Prepare for work | - Work instructions, including plans, specifications, quality requirements and operational details are obtained from relevant information, confirmed and applied to determine bricklaying and block laying tasks. - Safety (OHS) requirements are followed in accordance with safety plans and policies. - Signage and barricade requirements are identified and implemented. - Plant, tools and equipment selected to carry out tasks are consistent with job requirements, checked for serviceability, and any faults are rectified or reported prior to commencement. - Material quantity requirements are calculated in accordance with plans, specifications and quality requirements. - Materials appropriate to the work application, including required fire resistance rating, are identified, obtained, prepared, safely handled and located ready for use. - Environmental requirements are identified for the project in accordance with environmental plans and statutory and regulatory authority obligations, and are applied. |
| - Set out brickwork/ blockwork | - Bricks and/or blocks are identified, selected and checked for conformity with specifications and compliance with standards. - Work platform is erected in accordance with regulatory and workplace requirements. - Location and structural details of brickwork/blockwork are identified from job drawings and job specifications. - Base brickwork below floor construction is set out to location, dimensions and specifications in compliance with standards. - Load bearing brickwork, including engaged piers, dwarf walls, isolated piers and corbelling are set out to job drawings and specifications. - Cavity brick wall is set out to requirements of job drawings. |
| - Construct Base of brickwork/ blockwork. | - Mortar mix is prepared and checked for conformity and bricks/blocks are laid to set out according to specifications and standards. - Brickwork/blockwork gauge is determined and set out rod is prepared. - Base brickwork/blockwork is constructed for cavity construction to requirements of regulations and specifications. |
| - Position door and window frames. | - Window frames are located and built in to cavity walls to specification, and are protected from mortar droppings during construction. - Door jambs are located, built in and fixed to cavity walls and single leaf walls according to job drawings and specifications. |
| - Construct brick walls. | - Brick or block cavity construction walls are constructed to job specifications and standards. - Damp proof courses and flashings are laid/built in to job specifications. - Ventilation for solid brick construction is built to requirements of job specification and regulations. - Walls are to be straight and true in plumb, line and level within standard tolerances. - Wall ties are positioned to comply with standards. - Openings are constructed and flashing is installed to job specifications. - Lintels are installed to job specifications. - Control joints are formed in accordance with locations on job drawings and standards. - Weep holes, brick reinforcing, vermin proofing and wall flashings are located and built in to job specifications. - Gables and parapets are constructed by plans and specifications. - Sill bricks are cut and laid to line in accordance with job specifications. - Tie down and lateral support systems for ceiling/roof structures are installed to walls in accordance with plans, specifications and standards. |
| - Rake or rule joints. | - Joints of laid brickwork/blockwork are raked or ruled to correct depth and profile in accordance with job specifications. - Brickwork/blockwork is brushed down prior to drying to remove unwanted mortar. - Excess mortar is removed from brick/blockwork surfaces and cavities are cleaned free of mortar and debris in accordance with manufacturer recommendations, job specifications and standards. |
| - Clean up | - Work area is cleaned, and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- brick and block expansion and growth, control and articulation joints

- brick bond patterns

- characteristics and applications of materials for cavity brick construction, including fire control and separation required by the Building Code of Australia (BCA) and other legislation

- construction terminology

- job safety analysis (JSA) and safe work method statements

- material safety data sheets (MSDS)

- materials storage and environmentally friendly waste management

- plans, specifications and drawings

- processes for the calculation of material requirements, quality requirements

- techniques of cavity brick construction, including:- anti-termite measures, closing of cavities and capping systems, damp proofing, floor, wall and roof members, gable and eaves construction, lintels and load bearing components, stepped and level flashing for parapets and gables, sub-floor construction, tying components, ventilation, vermin control

- types, characteristics, uses and limitations of plant, tools and equipment

- Workplace and equipment safety requirements.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.locate, interpret and apply relevant information, standards and specifications

- comply with site safety plan and OHS legislation, regulations and codes of practice applicable to workplace operations

- comply with organizational policies and procedures, including quality requirements

- safely and effectively use tools, plant and equipment

- communicate and work effectively and safely with others

- as a minimum, given the plans and specifications, construct a section of a cavity brick building, including sub-floor walling; damp proof course (DPC) and ventilation; attached and isolated piers; openings (window and door) and lintels; brick cavity ties and roof tie-downs; and parapets (stepped flashing), ensuring:

- correct identification of requirement and finishing of the task

- correct selection and use of appropriate processes, tools and equipment

- completing all work to specification ensuring correct selection and use of fire-rated materials and methods of construction

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Bolsters, brick buggies and brick grabs, brooms, buckets, builders’ lines Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, materials hoists, measuring tapes and rules, mortar boards, Pallet trolleys, plumb rules, profiles, Scaffolds, shovels, small petrol or diesel engines and compressors, spirit levels, straight edges, string lines, Trowels Wheelbarrows |

### Construct Curved Wall

Overview: This competency standard covers the skills and knowledge required to Plan work, Set out curve for construction of curved masonry wall, Lay curved wall, Finish Joints and Clean up

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Set out curve for construction of curved masonry wall | - Planned curve points are plotted from job drawings and all trammel centres are established on footing slab . - Curve of wall is planned to specified location from trammel or plotted points and marked on the footing slab. |
| - Lay curved wall. | - Mortar is mixed to specifications. - Bricks/blocks are laid for first course to planned set out for line and specified bond according to specifications. - Gauge is maintained within standard tolerance at every course level. - Vertical face alignment is maintained. - Bricks/blocks are laid level over the length of the wall to the established plan profile. - Bricks/blocks are laid to specified bond with perpendicular joints (perpends) maintained in a vertical line - Construction is completed to requirements of job drawings and specifications. |
| - Finish Joints. | - Excess mortar is removed from brick/blockwork surfaces in accordance with work specifications. - Joints of laid brickwork/blockwork are raked or ruled to correct profile and depth to. - Brickwork/blockwork is brushed down prior to drying. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Brick and block expansion and growth, control and articulation joints

- Brick bond patterns, types of joints and finishing

- Characteristics and applications of materials for masonry arch construction

- Construction and arch terminology

- Geometric calculations and drawing

- Plans, specifications and drawings

- Types of masonry arches and techniques of construction

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Construct a curved wall using an arc centre and trammel

- Construct a serpentine curved wall using the plotted points and template technique

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Bolsters, brick buggies and brick grabs, brooms, buckets, builders’ lines Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, materials hoists, measuring tapes and rules, mortar boards, Pallet trolleys, plumb rules, profiles, Scaffolds, shovels, small petrol or diesel engines and compressors, spirit levels, straight edges, string lines, Trowels Wheelbarrows |

### Construct Masonry arches (semi-circular and segmental)

Overview: This competency standard covers the skills and knowledge required to Plan work, Set out first course for masonry arch, Prepare for arch construction, Construct arch centre, Set up arch centre, Cut and lay brick/ block to arch, Finish Joints and Clean up

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Set out first course for masonry arch | - Location and line of brickwork/blockwork wall are set out on concrete footing/slab to job drawings. - Pan of arch is determined from prepared allotted arch centre plus 4mm. - Arch spans are set out to location for first course. |
| - Prepare for arch construction. | - Mortar mix is prepared and bricks/blocks are laid to form wall to set out. - Gauge of abutting walls is maintained within standard tolerance at each course level. - Plumb and alignment of vertical wall face are maintained. - Bricks are cut and laid level and to line over length of wall. - Abutment jambs/piers are laid vertical up to springing line. - Bricks/blocks are laid in stretcher bond to springing line of arch with perpendicular joints maintained in vertical line. |
| - Construct arch centre. | - Arch centre is set out and curve is drawn up in accordance with specifications and plan. - Plan is transferred to material and cut to shape. |
| - Set up arch centre. | - Height to springing line is determined and height to crown of arch is confirmed to be within standard tolerance. - Height of toms and wedges or adjustable metal props are determined to set up and support timber arch centre. - Supports are adjusted to ensure arch centre is level at right angles to wall face and level across springing line. - Props, toms, packers and wedges are located for easy removal. - Position of central key brick/block is established for gauged arch and tape used to mark gauge. |
| - Cut and lay brick/ block to arch. | - Bricks and blocks are cut and laid on centre to form arch to specifications. - Joints are maintained to equal size and parallel on the extrados of an arch. - Same size wedge shape is maintained on face. - Centreline of key brick/block wedge is maintained through vertical centre line of arch. - Even joint thickness is maintained around extrados for cut brickwork and blockwork. - All bricks are cut and laid to maintain even joints. - All joints are struck evenly to depth and shape to specifications. |
| - Finish Joints. | - Excess mortar is removed from brick/blockwork surfaces in accordance with work specifications. - Joints of laid brickwork/blockwork are raked or ruled to correct profile and depth to. - Brickwork/blockwork is brushed down prior to drying. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Brick and block expansion and growth, control and articulation joints

- Brick bond patterns, types of joints and finishing

- Characteristics and applications of materials for masonry arch construction

- Construction and arch terminology

- Geometric calculations and drawing

- Plans, specifications and drawings

- Types of masonry arches and techniques of construction

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Construct five arches - three are in brick and two in block, including bull's eye, segmental and Gothic with the construction of the arch centre for one of the arches

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Bolsters, brick buggies and brick grabs, brooms, buckets, builders’ lines Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, materials hoists, measuring tapes and rules, mortar boards, Pallet trolleys, plumb rules, profiles, Scaffolds, shovels, small petrol or diesel engines and compressors, spirit levels, straight edges, string lines, Trowels Wheelbarrows |

### Construct Scaffolding

Overview: This competency standard covers the skills and knowledge required to erect and dismantle a range of modular scaffolding systems to provide work platforms for construction purposes.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Whip, tie, splice and inspect ropes | - Ropes and cords are inspected for damage and wear. - Designated rope ends are whipped and spliced in accordance with regulations and project specifications. - Bends and hitches are applied and inspected in accordance with project specifications. |
| - Erect scaffolding | - Purpose for scaffolding is confirmed and associated work tasks are identified. - Expected loading on scaffold and supporting structure is determined using load tables and manufacturer specifications. - Scaffolding and components are selected and inspected, and damaged components are labelled and rejected. - Sole board/base plate is selected in accordance with regulations, legislation, codes of practice and manufacturer specifications. - Scaffolding is set out and erected in accordance with regulatory and manufacturer requirements. - Static lines are erected and installed where specified in accordance with regulatory requirements. - Lifting device is assembled and erected where specified. |
| - Inspect, repair and alter erected scaffolding | - Erected modular scaffolding is inspected for damage, corrosion, wear and compatibility. - Current use of scaffolding is checked against original design and is in accordance with regulations and specifications - Scaffolding stability is inspected and confirmed. - Alteration or repair is carried out where specified. - Inspection log and handover is completed and dated, ready for signing by a certified scaffolder. |
| - Dismantle scaffolding | - Scaffolding is isolated and appropriately signed and barricaded to ensure safe dismantling. - Scaffolding is dismantled using reverse procedure as for erection. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Scaffolding equipment and techniques

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Complete erect and dismantle a modular scaffolding system, in accordance with pecifications, including a minimum of:

- Three bays with an internal and external return

- Three lifts, including ties

- Ladder and stair access

- Fall and edge protection

- Complete whipping, splicing, tying and inspecting of five fiber ropes in accordance with regulations.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Adjustable base plates - Bends and hitches, box spanners, braces, bracket scaffolds (tank and formwork) - Cantilevered hoists (materials only with maximum capacity of 500kg) - Couplers and accessories - Fibre ropes - Gin wheels, guard rails - Hammers - Ledgers - Mesh guards, mid rails, modular and prefabricated scaffolds - Podgers hammers, prefabricated components - Safety nets, scaffold belts, scaffolding planks shovels, spanners, spirit levels - Stairs or ladders, standards, static lines, steel and aluminium tubes - Tape measures, torpedo levels, transoms - Wire nips, wrenches. - Pallet trolleys, plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressors, spirit levels, straight edges, string lines Trowels Wheelbarrows |

### Install doors and window frames

Overview: This competency standard covers the skills and knowledge required set out and install timber and metal window and door units of different types, with appropriate weather proofing, smooth access and good security.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Install window units to frame | - Window opening size is checked to be greater than overall window frame. - Reveals are joined and fixed securely to frames where specified. - Window unit is located to suit brickwork and eaves finish for veneer construction, whichever is applicable. - Window unit is positioned in place so that head/sill is level and stiles are plumb and in wind, ensuring reveals or frame are finished flush with face of inside wall lining. - Window is packed and fixed to wall frame through/to studs, in accordance with specified fixing and fastening methods. |
| - Replace window units/door frames | - Architraves and nosing are removed. - Sill bricks or cladding are removed where specified. - Fasteners are cut, packing removed and flashing detached from frame. - Window unit/ doorframe is removed. - Window unit/door frame and window components are installed to plumb, level and wind. |
| - Architraves and moldings are replaced | - Prepare door opening, and construct and fix jamb. - Doorframe opening size is checked to be greater than the overall door jamb width and height, allowing for plumbing of stiles, thickness of floor covering, levelling of door head and level of floor. - Jamb stiles are marked and cut to length allowing for clearances according to specifications. - Head is trenched to accommodate jamb stiles allowing for clearance according to specification. - Jamb frame is assembled, squared and braced with rebates flush. - Joints and rebates are cleaned and finished. |
| - Install door and door unit | - Door unit is positioned in place so that the head/sill is level and stiles are plumb and in wind, ensuring reveals or frame finished flush with face of inside wall lining. - Door is fitted to jamb allowing for clearances according to specifications with lock stile door backed off to facilitate correct operation. - Hinges are marked out on door and jamb. - Hinges are fitted to door and jamb. - Final adjustments of door are made. - Door furniture components are fitted and fixed to manufacturer specifications |
| - Perform clean up | - Work area is cleaned, and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Flashing requirements and installation techniques

- Processes for setting out

- Window and door installation and replacement techniques

- Window and door materials

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- construct and fit one standard external rebated door jamb/frame to specifications;

- fit and hang one standard door, including the door furniture and a pair of doors with door jambs to specifications;

- install one standard window or glazed sliding door unit to specifications; and

- Replace one standard window or glazed sliding door unit to specifications.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Air compressors and hoses  Bevels  Chisels  Drills  Hammers, hand/power saws  Marking equipment, measuring tapes and rules  Nail guns  Protractors  Spirit levels, squares (combination/tri), steel squares and fences, string lines Materials may include:  Barrel bolts  Cabin hooks closers  Dead bolts  Flash bolts, flashings  Handles hinges (butt and parliament)  Latches, locks  Metal  Night latches  Passage sets  Timber Windows may include:  All size windows  Casement  Double hung  Glazed sliding doors, curved and bay  Hopper  Straight and sliding |

### Prepare Surface and fix tiles

Overview: This competency standard covers the skills and knowledge required set out and install timber and metal window and door units of different types, with appropriate weather proofing, smooth access and good security, and fix wall tiles

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Prepare materials for tiling application | - Floor and wall tiling materials are checked to ensure they are suitable and meet specifications. - Material preparation is carried out to satisfy the requirements of the specified application process. |
| - Prepare underlay and sheeting substrate | - Assistance with underlay preparation is provided under supervision. - Substrate surface is finished to specification, with joints flush and sealed. |
| - Prepare render substrate surface | - Surface-mounted construction units and attachments are safely detached. - Surfaces of substrate structure are cleaned to remove all loose material. - Materials for splash coat are proportioned and mixed to instructions ready for application to wet surface. - Horizontal and vertical surrounds are prepared for tiling process in accordance with type of tile and specified finish. - Materials for render coat are proportioned and mixed to instructions ready for application. - Surface is scratched, rendered, cured and dried in accordance with specifications for tile application. |
| - Cut tiles as required. | - Tiles are cut without jagged, flaid edges or damage to tile surfaces or finish, in accordance with workplace procedures and manufacturer recommendations. - Recess hole or curve is cut by hand or machine to shape and size and to specified tolerance. - Tile jolly is edged to form a mitre so that biscuit is not exposed at the joint in accordance with workplace procedures and manufacturer recommendations. |
| - Fix wall tiles | - Mortar and/or adhesive is prepared and applied to tile surface in accordance with manufacturer recommendations. - Tiles are prepared and fixed, with pad tiles set to level alignment. - Horizontal joint is checked for straightness, and tile edges and surface alignment are checked for conformity. - Tiles are fixed to alignment maintaining designed pattern to specification. - Even margins are shown around openings, frames and fittings to specification. - Bottom course is cut and fixed to create a rake or square corner in accordance with drawings and specifications. - Splayed, manufactured, formed coves are fixed in accordance with drawings and specifications. - Vertical tiles are finished plumb and true to square corners. - Joints are maintained straight and uniform in width with due allowance for tolerance of tile sizes. - Control joints are built in, in accordance with specifications and manufacturer recommendations. - Mitre joints are made, maintaining glazing on mitre without damage to tile surfaces or finish and maintaining uniformity of mitre in accordance with company procedures and manufacturer recommendations |
| - Tile external corners. | - Setting out for plumb, level and square is checked to be within specified tolerance. - External corners are checked to ensure surface intersections are straight. - Curved bead angle trim or tiles are fixed so that external return and bead are square and measurements are accurate to junction with tiles and set out, where applicable. - Tiles are fixed with minimum voids in tile bed while maintaining fully bedded alignment to specifications. - Corner is kept square within specified tolerance and finish to specifications. |
| - Tile internal corners | - Internal corner is checked to ensure surfaces are flat and intersection is straight. - Tiles are cut where required and fixed to one wall to maintain alignment in accordance with set out and specifications. - Tiles are cut where required and fixed abutting adjacent wall tiles to line, set out and specifications. - Joints for abutting tiles are made in accordance with designed margin for grouting or for expansion joint, where applicable, to specifications. - Coved tile or trim is installed to coved internal wall or wall and floor junctions. - Coved tile or trim is installed so that line is straight and, where applicable, aligned with set out. - Tiles are fixed to cove tile or trim and finish to alignment and specifications. |
| - Grout wall tile face | - Joints are cleaned and prepared to receive grout according to manufacturer specifications. - Grout is mixed and applied in accordance with workplace and manufacturer specifications and to meet job requirements. - Tiles are cleaned and polished with dry cloth to specifications, removing all dust from surface and joints. |
| - Perform clean up | - Work area is cleaned, and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Surface preparation materials and techniques.

- Control joints

- Techniques in fixing tiles to walls and corners, including preparation of substrates

- Tiling materials, including tiles, adhesives, mortar, grouting and substrates, their qualities, characteristics, preparation, techniques, applications, limitations and finishing.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Prepare a concrete bathroom floor and pointed fibre cement sheet wall for tiling

- Prepare a timber bathroom floor and pointed fibre cement sheet wall for tiling

- Tile the front and return of a standard framed or bricked bath, including a straight hob between the bath and wall, using adhesive or mortar to fix the tiles; tiles are not to be proud of the bath and coving to floor is to allow for control joints as required.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Brooms, Brushes, Buckets Caulking guns, Cement sheet cutters, Concrete mixers Electrical leads Hammers, Hose and water sprays Ladders, Levelling equipment, Lump hammers Measuring tapes and rules, mortar boards Nippers Pointed grouters, Power drills, power screwdrivers, power grinders, power sanders Power grinders and sanders Rags, rubber mallets Sanding blocks, saws, scrapers, shovels, spacers and wedges Spatulas, sponges, squares, squeegees, straight edges, stringlines Trowels Wet and dry diamond saws, wheelbarrows, wire brushes, wooden floats Work platforms Sanding blocks, saws, scrapers, shovels, spacers and wedges, spatulas, sponges, squares squeegees, straight edges, stringlines Tile cutters and scribes, trowels Wet and dry diamond saws, wheelbarrows, wire brushes, wooden floats Materials may include: Acoustic underlay material, adhesives Caulking compound, cement mortar (with and without additives) Clouts, cornice adhesive, crack suppression membrane Fixings and fasteners Patching materials, plasterboard nails, pre-mixed and mixed fillers Sand and cement, self-tapping screws, soft sheet nails Wall board adhesive |

### Prepare Surface for Painting

Overview: This competency standard covers the skills and knowledge required to prepare new or uncoated surfaces for painting or clear finish, prepare previously coated surfaces for painting or clear finish, remove wallpaper and prepare surface for painting and perform clean up

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Prepare new or uncoated surfaces for painting or clear finish | - Suitability of surface for painting or clear finish application is determined in accordance with manufacturer recommendations. - Surface preparation method is correctly selected in accordance with the environment, finish and substrate requirements. - Surface is prepared to manufacturer specifications in compliance with substrate requirements. - Surface imperfections are stopped, filled and sanded to a smooth finish ready for painting. |
| - Prepare previously coated surfaces for painting or clear finish | - Condition and nature of existing substrate and surface material are determined and tested. - Surface preparation method is correctly selected in accordance with the environment, finish and substrate requirements. - Surfaces are prepared by removing unwanted coatings and loose debris. - Surface defects are repaired, filled and sanded to smooth finish ready for painting in accordance with specifications. |
| - Remove wallpaper and prepare surface for painting | - Type, condition and nature of existing type of wallpaper are determined prior to removal. - Surface preparation method is correctly selected in accordance with the environment, finish and substrate requirements. - Wallpaper is removed using the most appropriate method. - Surfaces are prepared for paint application by removing loose debris. - Surface defects are repaired and imperfections stopped, to smooth finish ready for painting in accordance with specifications. |
| - Perform clean up | - Work area is cleaned, and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Paint application testing procedures

- Prevention and/or rectification procedures for surface coating defects

- Procedures, products and techniques associated with preparation of surfaces and wallpaper removal

- Properties and surface preparation requirements of new substrates

- Required protection for application of clear or stained finishes

- Surface coating technology

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- A new gyprock surface of a minimum 8 square metres and one new surface of a minimum of 1 square metre or 8 lineal metres

- Remove wallpaper from a room, cubicle or equivalent and prepare the surface for painting

- Three previously coated surfaces with one an external timber surface, one an internal surface and one a metal or masonry surface

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Drop sheets, duster brushes Filling knives and blades Hammers, hand sanders, heat removal equipment Mechanical sanders Nail punches Putty knives Scrapers Water blasters, wire brushes |

### Paint by spray, brush and roller

Overview: This competency standard covers the skills and knowledge required to apply brushed, rolled or sprayed paint coatings to different materials to form a protective and decorative finish.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Prepare work area | - Materials and substrate surfaces are prepared in accordance with manufacturer recommendations. - Surfaces not to be painted are protected by drop sheets, masking or removal of objects. - Adequate ventilation is provided to maintain a safe environment. |
| - Mix paint colour | - Base colour is identified by sample analysis or specification, and correct tint base, paint type and sheen level determined. - Colourants suitable for the colour match are selected, mixed with the base, allowed to dry and matched against the sample/specification for accuracy. - Materials for specified paint finish are mixed in accordance with manufacturer recommendations. - Correct amounts of paint material are prepared to specified ratio and drying time in accordance with manufacturer specification. |
| - Apply paint with brush or roller | - Brush and/or roller is selected to suit specified surface profile, size of area, type of paint and finish specified. - Sealant, undercoat, intermediate coat and finish coat are applied by brush/roller to achieve the required level of opacity, finish, texture and sheen, in accordance with specifications. |
| - Apply paint by spray | - Appropriate spray equipment, accessories and lines are selected and set up for operation in accordance with the manufacturer’s recommendations. - Paint is mixed and the viscosity adjusted to allow for spray painting. - Paint is applied to achieve correct finish specifications. |
| - Finish the application | - Finished paint surface is cured and tested using curing method in accordance with manufacturer recommendations. - Doors, windows and furniture removed for painting application are re-installed correctly and without damage to finished |
| - Perform clean up | - Work area is cleaned, and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Compatibility of preparatory materials and paint systems

- Hazards associated with solvents, chemicals and dust

- Surface coating technology, including specification of paint systems for interior and exterior painting projects to maximise durability, protection and aesthetic considerations

- Testing techniques and support materials

- Theoretical principles relating to adhesion and cohesion of paint

- Theoretical principles relating to pigmentation and colouring agents, drying and curing processes and the role of solvents

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- A minimum application of six square metres brushed or rolled

- A timber panel door

- A timber window, including architrave and frame with moving parts

- Four different surface type.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Brushware, brushware accessories  Compressor, conventional spray unit  Diaphragm, drop sheets, duster brushes  Filling knives and blades filters  Hand sanders, heat and flame paint removal equipment  High-volume low-pressure (hvlp) spray application, hose  Masking equipment, mechanical sanders  Nail punches  Paint pots and buckets, paint stirrers  Piston airless spray unit (electrical, pneumatic and petrol), putty knives  Regulator, roller frames  Scrapers, spray equipment, spray guns, spray tips  Wire brushes |

### Apply stains and clear timber finishes

Overview: This competency standard covers the skills and knowledge required to apply stains and clear timber finishes to different material surfaces, to form a protective and decorative finish.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. |
| - Prepare materials and application area | - Adjoining surfaces are protected by masking off or covering. - Adequate ventilation is provided to maintain a safe environment. - Measures are taken to ensure application area remains free of dust and foreign matter. - Existing stained or finished surfaces for application are stripped using appropriate techniques. |
| - Stain bare timber surface. | - Stain is selected for type of timber in accordance with manufacturer recommendations. - Stain is prepared to proportions and consistency in accordance with manufacturer recommendations. - Application method for the specified surface, area size and type of finish is selected. - Stain is applied to bare timber surface to specifications. - Wood filler and putty are selected, mixed, colour-matched and applied to timber in accordance with specifications. |
| - Apply finish surface | - Coats of selected clear finish are applied to achieve required level of opacity, finish and sheen in accordance with specifications. - Drying time is allowed between coats and clear finish surfaces are cured in accordance with manufacturer recommendations. |
| - Perform clean up | - Work area is cleaned, and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Hazards associated with solvents, chemicals and dust

- Surface preparation techniques for clear wood finishing

- Testing techniques and support materials

- Theoretical principles relating to pigmentation and colouring agents, drying and curing processes and the role of solvents

- Types, properties, uses and limitations of clear timber finishes and timber stains

- Types, uses and limitations of commonly used brushes, rollers and other applicators

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- prepare and or strip surfaces; and

- select and apply stains and finishes to timber in accordance with specifications and to suit the timber type, condition and finish required.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Brush ware and accessories, buckets Drop sheets, duster brushes Filling knives and blades Hammers, heat guns Nail punches Paint pots and buckets, paint stirrers, putty knives Roller frames and accessories Sanders – hand and mechanical, scrapers, spray equipment |

Tools and Equipment

| Sr. # | Description |
| --- | --- |
|  | Calculator |
|  | Ruler |
|  | Pencil |
|  | Adjustable metal props Bolsters, brick buggies and brick grabs, brooms, buckets, builders’ lines Concrete mixers Dumpy levels Elevators Forklifts Hammers (bricks, club and scutch), hoses Jig saws, jointing tools Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, materials hoists, measuring tapes and rules, mortar boards Pallet trolleys, plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressors, spirit levels, straight edges, string lines Trowels Wheelbarrows Brush ware and accessories, buckets Drop sheets, duster brushes Filling knives and blades Hammers, heat guns Nail punches Paint pots and buckets, paint stirrers, putty knives Roller frames and accessories Sanders – hand and mechanical, scrapers, spray equipment |

### Prepare site layout of civil engineering projects

. Overview: This competency standard covers the skills and knowledge required to Prepare for work, Prepare site for temporary fencing and structures, Co-ordinate the erection of site security fencing/hoarding, Co-ordinate construction of temporary site structures and Co-ordinate storage of tools and machine accessories

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare for work | - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details. - Follow Safety (OHS) requirements in Accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Select plant, tools and equipment to carry out tasks consistent with job requirements, check for serviceability, and any faults - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Identify, obtain, prepare, and safely handle materials appropriate to the work application |
| CU2. Prepare site for temporary fencing and structures | P1. Identify services, location and work areas are accurately P2. Place adequate warning signs and safeguards to support and protect services and substructures identified in work area. P3. Set up and secure temporary signs, control systems and protective barriers in accordance with job and safety instructions. |
| CU3. Co-ordinate the erection of site security fencing/hoarding | P1, Select appropriate type fencing materials in adequate quantities. P2. Conform dimensions of installed fence components to job specifications and/or work instructions. P3. Check erected fence as meeting security and enclosure requirements. |
| CU4. Co-ordinate construction of temporary site structures | P1. Interpret instruction/specification for temporary site structures accurately to the relevant persons. P2. Lay out temporary site structures to facilitate ease of access and to prevent obstruction of other site activities. P3. Check erected temporary site structures as conforming to job specification/ instruction given. |
| CU5. Co-ordinate storage of tools and machine accessories | P1. Lay out Storage facilities correctly and built to specifications. P2. Correctly demonstrate working knowledge of the type of items to be stored P3. Organize store for proper delivery, checking and control of materials, tools and equipment. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- types and purpose of job information/instructions

- OH&S requirements for preparing construction sites

- physical conditions of site

- tools, machines and equipment

- site fencing and hoarding

- relevant statutory requirements relating to site boundaries

- access and traffic routes

- storage and control of tools and materials

- convenient, visible and safe storage

- storage space and zone

- quality standards and specifications

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- compliance with occupational health and safety regulations applicable to workplace operations

- application of organizational management policies and procedures including quality assurance requirements where applicable

- demonstrated ability to read and interpret job related documentations

- demonstrated ability to co-ordinate the clearing of construction sites

- demonstrated ability to co-ordinate the preparation of access route, and the construction of temporary fencing/hoarding and structures

- demonstrated ability to organize the storage of tools, equipment and materials on site

- relate to people from varied social, cultural and ethnic background

- evidence is best gathered through a holistic assessment activity, which integrates the elements of competency

Tools and equipment:

If workplace based, the resources should relate specifically to organization policies, guidelines, requirements, resources and equipment. If an off the job or simulated work environment is used then resources should be generic and be applicable to a variety of work environments. Resources may include:

• A real or simulated environment

• Project resources

• Organization policies and guidelines

• Access to relevant resource personnel

• Relevant legal and statutory documentation

### Construct masonry steps and stairs

. Overview: This competency standard covers the skills and knowledge required to construct masonry steps, stairs and wing walls for different types and styles of buildings. It includes planning, types and styles of buildings. Set out and installation of the masonry.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare for work. | P1. Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details, from relevant information to determine bricklaying and block laying tasks. P2. Follow safety (OHS) requirements in accordance with safety plans and policies. P3. Identify and implement signage and barricade requirements. P4. Select plant, tools and equipment to carry out tasks consistent with job requirements, check for serviceability, and any faults P5. Calculate material quantity requirements in accordance with plans, specifications and quality requirements. P6. Identify, obtain prepare, and safely handle Materials appropriate to the work application are and located ready for use. |
| CU2.Set out brickwork/ block work. | P1. Erect work platform in accordance with regulatory and workplace requirements and brick and block stair construction requirements. P2. Check location and relative level of prepared footing from job drawings and specifications. P3. Calculate rise and going of flight and individual steps as per codes and workplace requirements. |
| CU3.Lay bricks/blocks forming steps and wing walls. | - Prepare mortar mix to required conformity and specifications. - Construct base brickwork to specifications and requirements of standards and codes. - Construct square steps, level, and plumb and lay to specified bond. - Construct design and profile of steps and aligned to bond and plumb to specifications. - Carry out Joints to work specifications. - Set out Brickwork/block work laid to line, with gauge and complete to work specifications. - Clean brick/block faces free of mortar |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- brick bond patterns characteristics and applications of materials for masonry steps and stairs construction

- brick and block expansion and growth, control and articulation joints

- construction terminology

- materials storage and environmentally friendly waste management

- plans, specifications and drawings

- processes for the calculation of material requirements

- quality requirements

- techniques of masonry steps and stairs construction

- types, characteristics, uses and limitations of plant, tools and equipment

- workplace and equipment safety requirements

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to:

- locate, interpret and apply relevant information, standards and specifications

- safely and effectively use tools, plant and equipment

- communicate and work effectively and safely with others

- as a minimum, construct a flight of stairs with a minimum of three pre-cast treads, and

- construct a flight of three solid masonry treads with a parallel wing wall on one side, ensuring:

  - correct identification of requirement and finishing of the tasks

  - correct selection and use of appropriate processes, tools and equipment

  - Completing all work to specification.

Tools and equipment include:

- bolsters

- brick grabs

- brooms

- buckets

- builders' lines

- concrete mixers

- dumpy levels

- elevators

- hammers (bricks, club and scotch)

- hoses

- jig saws

- jointing tools

- line blocks

- line pins

- mason's squares

- masonry saws

- measuring tapes and rules

- mortar boards

- plumb rules

- margin or raking tools

- profiles

- shovels

- spirit levels

- straight edges

- string lines

- trowels

- wheelbarrows

### Install stairs

. Overview: This competency standard covers the skills and knowledge required to construct masonry steps, stairs and wing walls for different types and styles of buildings. It includes planning, types and styles of buildings. It includes planning, preparation, set out and installation of the masonry.

| Competency Units | Performance Criteria |
| --- | --- |
| - Plan and prepare | - Confirm Work instructions in accordance supervisor. - Select required tools and equipment, serviceability and any faults. - Calculate materials quantity requirements accordance with specifications. - Obtain materials needed for compliance, prepared quality requirements. |
| - Select and prepare materials and components | P1. Identify methods of assembling erected for stairs P2. Check components for appropriate locations in stair structure. P3. Determine method of assembling and fixing in accordance with stair design and location |
| CU3. Assemble strings and newels | P1. Identify Specific position for stairs P2. Check measurements and adjustments if applicable. P3. Assemble Strings and newels to design and specification. P4. Fix strings to walls temporarily supported or directly fixed in position /specification. |
| CU4. Install treads and risers | - Assemble strings and newels temporarily braced in vertical position. - Fix treads and risers about newels to assembly - Check flight is true and square. - Fit intermediate treads and risers. |
| CU5. Assemble and install landings | P1. Fix bearers, joists and level according to specification. P2. Fix nosing and floor landing according to finish specification |
| CU6. Install handrail and balustrade | - Fix balusters/intermediate railing and handrails to form stair balustrade according to specification, - Check and ensure plumb fit of Newels prior to final fixing - Fix hand railings fitted to wall in accordance with specifications |
| CU7.Install spiral stair and curved strings | P1. Mark location of stair and first step on floor and erect central post is into true position, fixed at floor and temporarily braced at top. P2. Support initial string section temporarily supported in place for assembly, and fit treads and risers into position to specification. P3. Develop stair progressively with the extending, supporting and fixing of curved string, and complete with head secured to floor/landing, balustrade installed and central post fixed to specifications. |
| CU8. Secure stair to structure and line spandrel area | P1. Carry out securing of stair to building during/on completion of assembly. P2. Frame, line and fix spandrel, where applicable, to specified finish. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Adhesives, fixings and fasteners related to stair construction

- Assembling procedures for stairs

- Leveling techniques

- marking of components

- Materials and their characteristics, relevant to stair construction

- Measuring and setting out related to assembling and installing stairs

- Stair construction and joining methods

- Types of stairs and

- select and use safe and efficient procedures in installing treads, risers, flooring and nosing

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Accurately set out stair location and check levels for adjustments on newels;

- adopt and use appropriate techniques to fit and fix balustrades;

- complete installation with stair true to plumb and level, and fixed securely to structure with

- surfaces finished free of marks;

- demonstrate safe and effective procedures in assembling strings and newels and installing

- landing bearers and joists;

- select and use appropriate processes, tools and equipment to assemble stair components;

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: - Mason hammer - Pointing towels - Measuring tape - Mason thread - Plumb bob - Leveling tools (water level, spirit level) - Bucket - Mortar pan - Wooden float - Steel float - Shovel - Spade - Wire brush - Screeds (wooden/steel). |

### Prepare for solid dry wall plastering

.

Overview: This competency standard covers the skills and knowledge required to effectively carry out the preparation process of dry wall plastering, and applies to effectively prepare the process for carrying out solid plastering work, and applies to individuals working in masonry in the construction industry.

| Competency Units | Performance Criteria |
| --- | --- |
| CUI. Plan for construction proces | P1. Assure Quality requirements of company’s construction operations P2. Identify Preparation and planning requirements from drawings/work location and/or supervisor’s instructions. P3. Observe OH&S requirements and accordance with application tasks and workplace environment. P4. Identify Safety hazards and correct procedures in order to minimize risk to self and others. P5. Select Materials to supervisor’s instructions, P6.Select Appropriate personal protective equipment, correctly. P6.Select Tools and equipment consistent with the job requirements. P7. Select Fixtures/fasteners consistent with job requirements and check for specifications, quality requirements and operational details, P8. Calculate Materials quantity requirements in accordance with plans, specifications and quality requirements. P9. Identify Materials appropriate to the work application are, obtained, prepared, safely handled and located ready for use. |
| CU2. Prepare work area suitable forconstruction process | - Identify activities to be carried out in work area from surfaces to be lined and height to be accessed. - Prepare work area for construction process to supervisor’s instructions |
| CU3. Use tools, plant and equipmentappropriate for construction process | P1. Use regular hand and power tools suitable for application process identified to job requirements. P2. carry out hand and power tools used safely and effectively processes |
| CU4. Use tools, plant and equipmentappropriate for constructionprocess | P1. Identified Regular hand and power tools suitable for application process to job requirements. P2. Use Hand and power tools safely and effectively to carry out processes. |
| CU5. Prepare background of brick,concrete or blockwork for solid plastering | P1. Identify Structure and surface prepared. Depressions patched with suitable material to supervisor’s instructions. P2 Apply adhesive Concrete surface where appropriate rough. P3 MIX materials for scratch coat to instructions ready for application to wet surface. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- workplace and equipment safety requirements

- drawings and specifications portable power tools and tools and equipment materials relative to dry wall plastering materials handling

  - measurement relative to dry wall plastering

  - fixing and fasteners consistent with dry wall plastering requirements

  - workplace communication sand and cement

  - · plaster

  - · cornice adhesive

  - · caulking compounds

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate compliance with occupational health and safety regulations applicable to work operations at all times.

- Mix materials as per the job requirement following standard procedures.

- Plaster to required alignments and thickness following standard practices.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: - Mason hammer - Pointing towels - Measuring tape - Mason thread - Plumb bob - Leveling tools (water level, spirit level) - Bucket - Mortar pan - Wooden float - Steel float - Shovel - Spade - Wire brush - Screeds (wooden/steel). |

### Apply plaster

. Overview: This competency standard covers the skills and knowledge required to construct masonry steps, stairs and wing walls for different types and styles of buildings. It includes planning, types and styles of buildings. It includes planning, preparation, set out and installation of the masonry.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare for work | - Work instructions, including plans, specifications, quality requirements and operational details, are obtained, confirmed and applied from relevant information to determine bricklaying and block laying tasks. - Safety (OHS) requirements are followed in Accordance with safety plans and policies. - Signage and barricade requirements are identified and implemented. - Plant, tools and equipment selected to carry out tasks are consistent with job requirements, checked for serviceability, and any faults are rectified or reported prior to commencement. - Material quantity requirements are calculated in accordance with plans, specifications and quality requirements. - Materials appropriate to the work application are identified, obtained, prepared, safely handled and located ready for use. |
| CU2.Prepare mortar mix | P1. Select PPE, tools and equipment in accordance with the job requirements. P2. Estimate materials required as per job specifications. P3. Prepare the surface/mixing machines for mixing of mortar as per the job requirement. P4. Mix materials as per the job requirement following standard procedures. P5. Transport mortar mix as per the job requirement following standard procedures. |
| CU3. Prepare scaffolding | P1. Select and use PPE according to the job requirements. P2. Select and use tools and equipment as per the job requirement following standard procedures. P3. Prepare estimate of materials required based on the job requirements. P4. Prepare scaffolds components as per the job requirement following standard procedures P5. Erect and assemble scaffolds components as per the job requirements following standard procedures. P6. Dismantle scaffolding as per the job requirement following standard procedures. P7. Store or stockpile components following standard procedures. |
| CU4. Perform plastering work | P1. Prepare the surface to be plastered as per the job requirement following standard. P2. Prepare and apply cement slurry as per the job requirement following standard practices. P3. Apply mortar mix as per the job requirement to the required specifications following standard procedures. P4. Cure the plastered surfaces as per the job requirement following standard practices. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Occupational Health and Safety (OHS) Regulations

- Composition, properties and specifications of plastering materials

- Economic use of materials

- Estimation of materials

- Purpose of plastering/pointing

- Types of plaster/pointing

- Methods of plastering/pointing

- Defects in plastering/pointing

- Basic applied mathematics

- Curing

- Repair and maintenance work

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate compliance with occupational health and safety regulations applicable to work operations at all times.

- Mix materials as per the job requirement following standard procedures.

- Plaster to required alignments and thickness following standard practices.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: - Mason hammer - Pointing towels - Measuring tape - Mason thread - Plumb bob - Leveling tools (water level, spirit level) - Bucket - Mortar pan - Wooden float - Steel float - Shovel - Spade - Wire brush - Screeds (wooden/steel). |

### Prepare subgrade, base, and bedding course for paving

Overview: This competency standard covers the skills and knowledge required to prepare subgrade, base andbedding course in preparation for laying paving.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Prepare for work | P1 confirm work instructions with supervisor. P2. Select required tools and equipment, check for serviceability and any faults. P3. Calculate materials quantity requirements in accordance with specifications. P4. Obtain Materials needed and check for compliance |
| CU2. Identify soil | P1 Identify class of soil used as per soil classification charts. P2. Determine properties of soil assessed to its class. P3.Determine subgrade requirements of penetration tests. |
| CU3.Prepare subgrade | - Identify drainage and other features are within the area to be paved. - Identify drains put in place where required - Identify services in work area - Identify damp proof courses and paving is constructed so that it does not compromise them. - Components are cleaned, stacked and stored for reuse or bundled for removal. |
| CU4. Install base course materials | P1. Calculate quantity of base course materials is based upon the subgrade type and purpose of the paved area. P2. Distribute material over area, allowing for compaction. P3. Compact area taking care not to over-compact base materials. |
| CU5. Install bedding course materials | P1. Select bedding course material P2. Calculate quantity of layer course materials P3. Distribute material over the area to be paved within tolerances stipulated by relevant standards. P4. Handle compacting machinery correctly. P5. Screed base materials to levels as determined, and set out by string lines or other mechanisms. P6. Clean the work area.. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- principles of California bearing ratio (CBR)

- properties of bedding course materials

- properties of geotextile materials

- relevant Australian standards

- stabilising bedding sand

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- locate, interpret and apply relevant information, standards and specifications

- comply with site safety plan and OHS legislation, regulations and codes of practice applicable to workplace operations

- comply with organizational policies and procedures including quality requirements

- safely and effectively operate and use tools, plant and equipment

- Communicate and work effectively and safely with others.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | - Tools and equipment: compactors concrete mixer levelling devices rakes screed shovels string lines Wheelbarrows. |
|  | - Materials required: aggregates bedding sand cement concrete crushed rock road base sand. |

### Lay paving

Overview: This competency standard covers the skills and knowledge required to lay pavers on level and inclined surfaces.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Plan work | P1. Confirm work instructions with supervisor. P2. Select tools and equipment are selected as required, check for serviceability and any faults P3. Calculate material quantity requirements in accordance with specifications. P4. Obtain and prepare materials needed for compliance |
| CU2 Set out work area | P1. Identify location and area of paved area from plans and specifications. P2. Identify sub-soil and footing types according to standards. P3. Locate underground services P4. Set out location and shape of paving area to dimensions from plans and specifications. |
| CU3. Lay paving | P1. Excavate area is to required depth, allowing for base and thickness of unit and specified finish level. P2. Position drainage pipes in sub soil in accordance with plans and specifications. P3. Mix mortar for masonry paving to specifications and standards. P4. Spread substrate base material and compact to specifications. P5. Spread and screed bedding material is spread to designed level and alignment. P6. Position edge boards to set out and specifications. P7. Grade paving surface to fall evenly, without ponding, to outlets or surface run-off system. P8. Determine initial starting line is and lay pavers to line conforming to specified pattern. P9. Cut paving units and segments and lay to designed pattern and specifications, with joints to specifications and surface finish aligned. P10. Maintain finished level is across junctions between different finishes. P11. Complete paving installation with joints finished to specifications. P12. Clean paving surface is on completion to specifications. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Characteristics and applications of materials for laying pavers

- Corner geometry

- Paving bonds and patterns, joints and finishing

- Plans, specifications and drawings

- Techniques for laying pavers

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- pave a level area of 3m x 5m with clay pavers, using sand as a bedding material on compacted crushed rock (closed finish with sand brushed in); and

- Pave an area of 3m x 5m over a fall with clay bricks, incorporating control joints and using mortar as a bedding material on concrete (mortar joints finish).

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: Bolsters, brick buggies, buckets, builders' lines, builders squares, Concrete mixers Forklifts, Hammers, hoses Line blocks, line pins Masonry saws, mason's squares measuring tapes and rules, mortar boards Pallet trolleys Rakes and brooms, rubber mallets Screed boards, shovels, small petrol or diesel engines, compressors or mixers, spirit levels, straight edges, string lines Trowels Vibrating plates, Wheelbarrows |

### Perform concrete, tiles and mosaic flooring

Overview: This competency standard covers the skills and knowledge required to supervising construction works using concrete. It requires candidates to be able to assess concrete materials in their raw and finished states, identify and evaluate the characteristics of different concrete types, identify defects and impurities in concrete and understand the effect of impurities in concrete. It is appropriate for supervisors of concrete constructions.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1 Prepare concrete mixture | P1. Read, and interpret drawing / layout and specification according to the job requirements. P2. Select materials according to job requirements. P3. Select and use PPE as per the job requirements. P4. Select and use required tools and equipment as per the job requirements P5. Prepare materials as per the job requirement following standard procedures. P6. Mix materials as per the job requirement following standard procedures. Transport concrete mix as per the job requirement following standard procedures. |
| CU2 Perform concrete flooring | P1. Prepare surfaces as per the job requirement following standard procedure P2. Lay the concrete mixtures as per the job requirement following standard procedure P3. Compact the concrete mixtures/surfaces following standard procedures. P4. Plaster the floor surfaces following standard practices P5. Prepare and apply cement slurry to the surfaces as per the job requirement following standard procedures. P6. Cure the floor surfaces following standard practices. |
| CU3 Perform tiles flooring | P1. Prepare surfaces as per the job requirement following standard procedures. P2. Prepare tiles as per the job requirement following standard procedures. P3. Plaster the floor surfaces as per the job requirement following standard procedures. P4. Prepare and apply cement slurry as per the job requirement following standard procedures. P5. Lay floor tiles as per the job requirement following standard procedures. P6. Cure the floor surfaces following standard practices. P7. Finish the floor surfaces as per the job requirement following standard procedures. |
| CU4 Perform mosaic flooring | P1. Prepare surfaces as per the job requirement following standard procedure. P2. Place the stripes as per the job requirement following standard practices. P3. Pour the concrete mixtures as per the job requirement following standard procedure. P4. Compact the concrete mixtures/surfaces of following standard procedures. P5. Prepare mosaic mix as per the job requirement following standard practices. P6. Pour and compact mosaic mix as per the job requirement following standard procedures. P7. Finish the floor surfaces as per the job requirement following standard procedures. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Occupational Health and Safety (OHS) Regulations

- Composition, properties and specifications of flooring mixtures

- Economic use of materials

- Estimation of materials

- Basic applied mathematics

- Repair and maintenance work

- Tiles design and decorations

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate compliance with hygiene and safety regulations applicable to work operations at all times.

- Prepare the flooring materials as per the job requirements.

- Finish the floor surfaces as per the job requirement following standard procedures

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: - Mason hammer - Trowel - Chisel - Measuring tape - Try square - Mason thread - Sprite level - Bucket - Mortar pan Sponge/form - Wooden/steel float - Shovel - Spade - Wire brush - Screeds(wooden/steel) - Grinder - Wooden/steel edge |

### Perform marble/granite flooring

Overview: This competency standard covers the skills and knowledge required to prepare concrete

Mixture and Lay marbles/granite

| Competency Units | Performance Criteria |
| --- | --- |
| CU1 Prepare concrete mixture | P1 Read, and interpret drawing / layout and Specification according to the job requirements. P2 Select materials according to job requirements. P3 Select and use PPE as per the job requirements. P4 Select and use required tools and equipment as per job requirements P5 Prepare materials as per the job requirement following standard procedures. P6 Mix materials as per the job requirement P7 Transport concrete mix as per the job requirement following standard procedures. |
| CU2. Lay marbles/granite | P1. Prepare surfaces as per the job requirement following standard procedure P2. Prepare marble/granite slabs as per the Standard procedure. P3 Plaster the floor surfaces as per the standard P4. Prepare and apply cement slurry as per the job requirement standard procedure. P5. Lay marble/granite tiles as per the job requirement following standard procedure. P6. Finish the floor surfaces as per the job requirement following standard procedures. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of: Fundamentals of masonry.

- Types of flooring

- Types of bondage

- Safe use of tools and equipment

- Communication

- First aid

- Estimation and costing.

- Curing procedures

- Economic use of materials

- Grinding and moulding procedures

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate compliance with hygiene and safety regulations applicable to work operations at all times.

- Demonstrate compliance with occupational health and safety regulations applicable to work operations at all times.

- Interpret drawings and layouts.

- Lay marble/granite tiles as per the job requirement following standard procedure.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: - Straight edge - Plumb bob - Sprite level - Grinding tools - Trowel - Pans shovels - Moulding tools - Cutter - Float |

### Apply water proofing remedial processes

Overview: This competency standard covers the skills and knowledge required to apply remedial waterproofing processes to external and below ground level wet areas, using injection epoxy, cement crystallisation or hydrostatic coating methods. It includes identification of the waterproofing system to be used, its preparation and its application.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare | - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details from relevant information for planning and preparation purposes. - Follow safety (OHS) requirements in accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Calculate material quantity in accordance with plans, specifications and quality requirements. - Identify environmental requirements for project in accordance with environmental plans and statutory and regulatory authority obligations. |
| CU2. Identify waterproofing system. | P1. Identify area to be waterproofed from a technical report or diagnosed damp fault area and inspected for defects and soundness in accordance with job and manufacturer specifications. P2. Inspect area of structure for waterproofing surface application for defects and soundness in accordance with job and manufacturer specifications. P3. Identify, analyze and select appropriate remedial waterproofing systems and products in accordance with job and manufacturer specifications P4. Check range of waterproofing materials for product suitability, conformity to specification and compatibility with surface material, preparation and waterproofing installation technique. P5. Select type of waterproofing material is identified in accordance with type of substrate, job specification, state of structure and job safety requirements. |
| CU3. Prepare for waterproofing process. | P1. Confirm site to be repaired in accordance with drawings, specifications and manufacturers' specification. P2. Set out construction/installation site to specified location, structure and dimensions in accordance with drawings and specifications. |
| CU4. Apply waterproofing using injection. | P1. Determine and measure Injection hole set-out spacing and in accordance with manufacturer recommendations. P2. Apply materials prepared for injection method application in accordance with manufacturer recommendations. P3. Prepare and set up Injection equipment for injection process in accordance with manufacturer recommendations. P4. Test and flush injection system in accordance with manufacturer specifications. P5. Make damp course water resistant and capped using injection method in accordance with manufacturer recommendations and to specifications. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of: characteristics and applications of remedial waterproofing materials

- construction systems and waterproofing considerations

- general construction terminology

- job safety analysis (JSA) and safe work method statements

- materials storage and environmentally friendly waste management

- plans, drawings and specifications

- plant, tools and equipment types, characteristics, uses and limitations

- principles and considerations of water exclusion

- processes for the calculation of material requirements

- quality requirements

- remedial waterproofing materials, processes and testing techniques

- Workplace and equipment safety requirements.

Critical Evidence(s) Required

The candidate needs must be able to provide evidence of the ability to:

- locate, interpret and apply relevant information, standards and specifications

- comply with site safety plan and OHS legislation, regulations and codes of practice applicable to workplace operations

- comply with organizational policies and procedures including quality requirements

- safely and effectively operate and use tools, plant and equipment

- communicate and work effectively and safely with others as a minimum:

- repair a crack in a concrete basement wall or a suspended car park slab, identifying remedial waterproofing method to be used, determining the type of material to be used and the repair requirement (e.g. spacing of injection ports/packers if using an injection method), and testing the application

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: angle grinders chisels electric drills and screwdrivers hammers impact drills measuring tapes and rules mixing equipment pressure injection equipment, including: - cartridge applications - compressors - hoses - pumps - vacuum pumps spirit levels straight edges. |

### Apply and install sealants

Overview: This competency standard covers the skills and knowledge required to apply and install sealants

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare for work | P1. Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details, for planning and preparation purposes P2. Follow safety (OHS) requirements in Accordance with safety plans and policies. P3. Identify and implement signage and barricade requirements P4. Select plant, tools and equipment to carry out tasks consistent with job requirements, check for serviceability, and any faults P5. Sealant material quantity requirements are calculated in accordance with plans, specifications and quality requirements. P6. Identify, obtain, prepare, safely handle and locate materials appropriate to the work application |
| - Prepare surface to receive sealants. | P1. Clean surface free of contaminants such as oil, grease, dust or moisture. P2. Surface is prepared by sanding and/or cleaning in accordance with preparation specifications. |
| CU3.Apply sealant. | P1. Apply sealant materials to manufacturer recommendations and specifications. P2. Take care to ensure no air is trapped within applied sealant |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- K1. safe materials handling techniques and requirements, including hazardous materials relevant to sealant application work

- K2. safe use of scaffolding and working platforms

- K3. types and performance of sealants used in buildings

- K4. types and use of hand tools and equipment relevant to sealant application and installation work

- K5. Workplace and equipment safety requirements.

Critical Evidence(s) Required

The candidate who demonstrates competency in this unit must be able to handle and use all the sealant materials and application devices listed in the range statement, providing evidence of the ability to:

- comply with OHS regulations and state and territory legislation applicable to workplace operations

- comply with organizational policies and procedures, including quality assurance requirements

- select and use appropriate processes, tools and equipment to carry out tasks

- adopt and carry out correct procedures prior to and during handling and application of materials

- demonstrate safe and effective operational use of tools and equipment

- demonstrate safe application in the process of cleaning up application area and cleaning equipment

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: brooms brushes cartridge applicators putty knives and paring knives rollers sanders spray equipment. |

### Perform water proofing works

Overview: This competency standard covers the skills and knowledge required to safely and effectively use tools and equipment used in waterproofing. It includes identification, selection and use of hand and power tools, plant and equipment used in masonry work.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare for work. | - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details from relevant information for planning and preparation purposes. - Follow safety (OHS) requirements in accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Calculate material quantity in accordance with plans, specifications and quality requirements. - Identify environmental requirements for project in accordance with environmental plans and statutory and regulatory authority obligations. |
| - Identify waterproofing system. | - Identify basement, water tank area to be waterproofed from job drawings and specifications. - Inspect area of structure to be waterproofed for defects and soundness in accordance with job and manufacturer specifications. - Identify, analyse and select appropriate waterproofing systems and products for water exclusion in accordance with job and manufacturer specifications, and with material safety data sheet (MSDS) directions. - Check range of waterproofing materials for product suitability; conformity to specification; and compatibility with surface material, preparation and waterproofing installation technique. - Determine termination detailing. - Identify type of waterproofing material in accordance with job specification, state of structure, and job safety requirements |
| - Prepare water proofing installation. | - Check basement, tank and wet area fitment or fixtures for specific measurements and set out in accordance with drawings and specifications. - Check substrate for soundness of fit, curing compounds, moisture content and other contaminants, and reported or remedied as required. - Check flooring installation levels and falls to waste outlets for conformity to specification. - Install corner flashing in accordance with manufacturer recommendations. - Check points of connection, termination detailing and over flashings as required to be in place and secure. - Check rebates for baths and basins for compliance with standards. |
| - Prepare substrate | - Correct defects and make good in accordance with manufacturer specifications, to bring substrate to a smooth and uniform finish. - Prepare surface of structure to be waterproofed to manufacturers' specification, including fixings and belling out around taps. - Install water stops and hobs in required location in compliance with standards and good building practice. - Perform priming coat on prepared surface of structure to manufacturers' specification, where applicable. |
| - Identify and select plant and equipment | - Apply waterproofing membrane to primed surface of structure to correct thickness and in accordance with manufacturers' job specification. - Apply appropriate bond breakers and fillets as required in accordance with manufacturer specifications. - Cure waterproofing membrane in accordance with manufacturers' specification and workplace requirements. - Conduct flood testing of installation if required. - Protect waterproofing system and materials using methods and materials consistent with manufacturers' specification, workplace requirements and good building practice. - Under take final inspection of site in accordance with certifying authority's requirements and sign-off and handover of work is carried out in accordance with workplace requirements. |
| - Clean up. | - Clear work area and dispose of, reuse or recycle materials in accordance with legislation, regulations, and codes of practice and job specification. - Clean, check, maintain and store plant, tools and equipment in accordance with manufacturer recommendations and standard work practices. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe applications, limitations and method of operation and maintenance of hand and power tools, plant and equipment applicable to waterproofing tasks

- State communication processes - verbal and signaling

- State general construction terminology

- Recognize and observe job safety analysis (JSA) and safe work method statements

Critical Evidence(s) Required

- The candidate needs to produce following critical evidences in order to competent in this competency standard.

A person who demonstrates competency in this unit must be able to provide evidence of the ability to:

Water proofing of water tank.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment angle grinders brooms, brushes buckets caulking guns chisels, including cold chisels cutting blades electric drills and screwdrivers fans floor scrapers hammers heat welders, hot air welders, lights measuring tapes and rules moisture meters nylon rollers pressure rollers fusion rollers scissors seam probes solvent applicators spirit levels straight edges Vacuum cleaners. |

### Apply water proofing tools and equipment

Overview: This competency standard covers the skills and knowledge required to safely and effectively use tools and equipment used in waterproofing. It includes identification, selection and use of hand and power tools, plant and equipment used in masonry work.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare for work. | - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details from relevant information for planning and preparation purposes. - Follow safety (OHS) requirements in accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Calculate material quantity in accordance with plans, specifications and quality requirements. - Identify environmental requirements for project in accordance with environmental plans and statutory and regulatory authority obligations. |
| - Identify hand and power tools. | - Identify type and function of hand and power tools to be used in the waterproofing work. - Methods of operation of hand and power tools are identified from specifications, standards and manufacturers' instructions. - Specific OHS requirements for specific hand and power tools are identified and applied. - Personal protective equipment (PPE) required for operation of the tools is identified in accordance with regulatory and workplace requirements. |
| - Select Tools for work | - Select tools and equipment consistent with job requirements. - Check tools, including leads and hoses, for tags, serviceability and report safety and any faults to supervisor. - Check power tools guards, guides and controls and maintained in accordance with manufacturer recommendations. - Check equipment to hold or support material during operation and inspect for faults. - Complete pre-operational checks, including lubricants, hydraulic fluid and water, in accordance with manufacturer recommendations. |
| - Use Tools | - Connect power and compressed air supply to work area to the area of work in accordance with regulatory and workplace requirements and codes of practice. - Observe start-up and shut-down procedures. - Use tools safely and effectively according to manufacturer recommendations and regulatory requirements. - Switch off tools safely and locate when not in use. |
| - Identify and select plant and equipment | - Identify function and limitations of waterproofing hand and power tools, plant and equipment used for waterproofing systems and waterproofing tasks. - Identify method of operation of plant and equipment from specifications, standards and manufacturers' instructions. - Identify and apply OHS requirements for specific plant and equipment, including requirements for guarding, guiding and controls. - Identify PPE required for the operation of plant and equipment in accordance with regulatory and workplace requirements. - Items of plant and equipment are selected consistent with hazard minimisation and needs of the job. - Check plant and equipment for safety, and report faults to supervisor in accordance with workplace procedures. - Recognise and adhere to OHS requirements for operating and using plant and equipment |
| - Use plant and equipment | - Observe start-up and shut-down procedures - Use plant and equipment safely and effectively according to manufacturer recommendations and regulatory requirements. - Switch off plant and equipment safely and locate when not in use. |
| - Clean up. | - Clear work area and dispose of, reuse or recycle materials in accordance with legislation, regulations, codes of practice and job specification. - Clean, check, maintain and store plant, tools and equipment in accordance with manufacturer recommendations and standard work practices. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe applications, limitations and method of operation and maintenance of hand and power tools, plant and equipment applicable to waterproofing tasks

- State communication processes - verbal and signaling

- State general construction terminology

- Recognise and observe job safety analysis (JSA) and safe work method statements

Critical Evidence(s) Required

- The candidate needs to produce following critical evidences in order to competent in this competency standard. A person who demonstrates competency in this unit must be able to provide evidence of the ability to:

locate, interpret and apply relevant information, standards and specifications

comply with site safety plan and OHS legislation, regulations and codes of practice applicable to workplace operations

comply with organizational policies and procedures including quality requirements

safely and effectively operate and use tools, plant and equipment

communicate and work effectively and safely with others as a minimum, follow work instructions and operating procedures to safely and effectively use appropriate tools, plant and equipment for the preparation of the substrate for waterproofing and the application of at least one of the following applications:- an internal wet area, an external wet area, a below ground level wet area, a remedial waterproofing project ensuring:

correct selection- and use of appropriate processes, tools and equipment

no damage to tools, plant and equipment, completing all work to specification

- •	Compliance with regulations, standards and organizational quality procedures and processes.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment angle grinders brooms, brushes buckets caulking guns chisels, including cold chisels cutting blades electric drills and screwdrivers fans floor scrapers hammers heat welders, hot air welders, lights measuring tapes and rules moisture meters nylon rollers pressure rollers fusion rollers scissors seam probes solvent applicators spirit levels straight edges vacuum cleaners. |

### Perform water proofing of basements

Overview: This competency standard covers the skills and knowledge required to identify of the waterproofing system to be used, prepare water proofing system and perform water proofing system in basements.

| Competency Units | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details from relevant information for planning and preparation purposes. - Follow safety (OHS) requirements in accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Select plant, tools and equipment to carry out tasks in consistent with job requirements, check for serviceability, and rectify or report any faults prior to commencement. - Calculate material quantity in accordance with plans, specifications and quality requirements. - Identify, obtain, prepare, and safely handle materials appropriate to the work application, including required fire resistance rating, ready for use. - Identify environmental requirements for project in accordance with environmental plans and statutory and regulatory authority obligations. |
| - Identify water proofing system. | - Identify basement area to be waterproofed from job drawings and specifications. - Inspect area of structure to be waterproofed for defects and soundness in accordance with job and manufacturer specifications. - Identify, analyse and select appropriate waterproofing systems and products for water exclusion in accordance with job and manufacturer specifications, and with material safety data sheet (MSDS) directions. - Check range of waterproofing materials for product suitability; conformity to specification; and compatibility with surface material, preparation and waterproofing installation technique. - Determine termination detailing. - Identify type of waterproofing material in accordance with job specification, state of structure, and job safety requirements with MSDS directions |
| - Prepare for water proofing installation | - Check basement area and wet area fitment or fixtures for specific measurements and set out in accordance with drawings and specifications. - Check substrate for soundness of fit, curing compounds, moisture content and other contaminants, and reported or remedied as required. - Check flooring installation levels and falls to waste outlets for conformity to specification. - Install corner flashing in accordance with manufacturer recommendations. - Check points of connection, termination detailing and over flashings as required to be in place and secure. - Check rebates for baths and basins for compliance with standards. |
| - Apply water proofing to basement. | - Correct defects and make good in accordance with manufacturer specifications, to bring substrate to a smooth and uniform finish. - Prepare surface of structure to be waterproofed to manufacturers' specification, including fixings and belling out around taps. - Install water stops and hobs in required location in compliance with standards and good building practice. - Perform priming coat on prepared surface of structure to manufacturers' specification, where applicable. - Apply waterproofing membrane to primed surface of structure to correct thickness and in accordance with manufacturers' job specification. - Apply appropriate bond breakers and fillets as required in accordance with manufacturer specifications. - Cure waterproofing membrane in accordance with manufacturers' specification and workplace requirements. - Conduct flood testing of installation if required. - Protect waterproofing system and materials using methods and materials consistent with manufacturers' specification, workplace requirements and good building practice. - Under take final inspection of site in accordance with certifying authority's requirements and sign-off and handover of work is carried out in accordance with workplace requirements. |
| - Clean up. | - Clear work area and dispose of, reuse or recycle materials in accordance with legislation, regulations, codes of practice and job specification. - Clean, check, maintain and store plant, tools and equipment in accordance with manufacturer recommendations and standard work practices. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe assessment and appreciation of moisture content in substrate materials

- State characteristics and applications of waterproofing materials and adhesives

- Describe construction systems and waterproofing considerations

- Define general construction terminology

- Describe internal waterproofing materials, processes and techniques

- Describe job safety analysis (JSA) and safe work method statements

- State materials storage and environmentally friendly waste management

- Describe MSDS

- Read and interpret plans, drawings and specifications

- Describe plant, tools and equipment types, characteristics, uses and limitations

- Describe principles and considerations of water exclusion

- State processes for the calculation of material requirements

- Describe quality requirements

- Explain termination and flashing principals

- Describe workplace and equipment safety requirements.

Critical Evidence(s) Required

- The candidate needs to produce following critical evidences in order to competent in this competency standard.

locate, interpret and apply relevant information, standards and specifications

comply with site safety plan and OHS legislation, regulations and codes of practice applicable to workplace operations

comply with organizational policies and procedures including quality requirements

safely and effectively operate and use tools, plant and equipment

communicate and work effectively and safely with others as a minimum:

waterproof a bathroom incorporating: lap up a wall, appropriate penetrations, wastes and hobs

install and detail a hob less frame shower enclosure and a bath that abuts a masonry connection wall, ensuring: correct identification of requirement and installation of the waterproofing

Correct selection and use of appropriate processes, tools and equipment

- •    Completing all work to specification, correct termination and over flashing.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab equipment angle grinders brooms, brushes buckets caulking guns chisels, including cold chisels cutting blades electric drills and screwdrivers fans floor scrapers hammers heat welders, hot air welders, lights measuring tapes and rules moisture meters nylon rollers pressure rollers fusion rollers scissors seam probes solvent applicators spirit levels straight edges Vacuum cleaners. |

### Apply water proofing to internal wet areas

Overview: This competency standard covers the skills and knowledge required to identify of the waterproofing system to be used, prepare water proofing system and apply water proofing system.

| Competency Units | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details from relevant information for planning and preparation purposes. - Follow safety (OHS) requirements in accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Select plant, tools and equipment to carry out tasks in consistent with job requirements, check for serviceability, and rectify or report any faults prior to commencement. - Calculate material quantity in accordance with plans, specifications and quality requirements. - Identify, obtain, prepare, and safely handle materials appropriate to the work application, including required fire resistance rating, ready for use. - Identify environmental requirements for project in accordance with environmental plans and statutory and regulatory authority obligations. |
| - Identify water proofing system. | - Identify internal area to be waterproofed from job drawings and specifications. - Inspect area of structure to be waterproofed for defects and soundness in accordance with job and manufacturer specifications. - Identify, analyse and select appropriate waterproofing systems and products for water exclusion in accordance with job and manufacturer specifications, and with material safety data sheet (MSDS) directions. - Check range of waterproofing materials for product suitability; conformity to specification; and compatibility with surface material, preparation and waterproofing installation technique. - Determine termination detailing. - Identify type of waterproofing material in accordance with job specification, state of structure, and job safety requirements with MSDS directions |
| - Prepare water proofing installation | - Check internal wet area and wet area fitment or fixtures for specific measurements and set out in accordance with drawings and specifications. - Check substrate for soundness of fit, curing compounds, moisture content and other contaminants, and reported or remedied as required. - Check flooring installation levels and falls to waste outlets for conformity to specification. - Install corner flashing in accordance with manufacturer recommendations. - Check points of connection, termination detailing and over flashings as required to be in place and secure. - Check rebates for baths and basins for compliance with standards. |
| - Prepare substrate | - Correct defects and make good in accordance with manufacturer specifications, to bring substrate to a smooth and uniform finish. - Prepare surface of structure to be waterproofed to manufacturers' specification, including fixings and belling out around taps. - Install water stops and hobs in required location in compliance with standards and good building practice. - Perform priming coat on prepared surface of structure to manufacturers' specification, where applicable. |
| - Apply water proofing. | - Apply waterproofing membrane to primed surface of structure to correct thickness and in accordance with manufacturers' job specification. - Apply appropriate bond breakers and fillets as required in accordance with manufacturer specifications. - Cure waterproofing membrane in accordance with manufacturers' specification and workplace requirements. - Conduct flood testing of installation if required. - Protect waterproofing system and materials using methods and materials consistent with manufacturers' specification, workplace requirements and good building practice. - Under take final inspection of site in accordance with certifying authority's requirements and sign-off and handover of work is carried out in accordance with workplace requirements. |
| - Clean up. | - Clear work area and dispose of, reuse or recycle materials in accordance with legislation, regulations, and codes of practice and job specification. - Clean, check, maintain and store plant, tools and equipment in accordance with manufacturer recommendations and standard work practices. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe assessment and appreciation of moisture content in substrate materials

- State characteristics and applications of waterproofing materials and adhesives

- Describe construction systems and waterproofing considerations

- Define general construction terminology

- Describe internal waterproofing materials, processes and techniques

- Describe job safety analysis (JSA) and safe work method statements

- State materials storage and environmentally friendly waste management

- Describe MSDS

- Read and interpret plans, drawings and specifications

- Describe plant, tools and equipment types, characteristics, uses and limitations

- Describe principles and considerations of water exclusion

- State processes for the calculation of material requirements

- Describe quality requirements

- Explain termination and flashing principals

- Describe workplace and equipment safety requirements.

Critical Evidence(s) Required

- The candidate needs to produce following critical evidences in order to competent in this competency standard.

locate, interpret and apply relevant information, standards and specifications

comply with site safety plan and OHS legislation, regulations and codes of practice applicable to workplace operations

comply with organizational policies and procedures including quality requirements

safely and effectively operate and use tools, plant and equipment

communicate and work effectively and safely with others as a minimum:

waterproof a bathroom incorporating: lap up a wall, appropriate penetrations, wastes and hobs

install and detail a hob less frame shower enclosure and a bath that abuts a masonry connection wall, ensuring: correct identification of requirement and installation of the waterproofing

correct selection and use of appropriate processes, tools and equipment

- •     Completing all work to specification, correct termination and over flashing.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment angle grinders brooms, brushes buckets caulking guns chisels, including cold chisels cutting blades electric drills and screwdrivers fans floor scrapers hammers heat welders, hot air welders, lights measuring tapes and rules moisture meters nylon rollers pressure rollers fusion rollers scissors seam probes solvent applicators spirit levels straight edges |

### Install Physical termite barriers

Overview: This competency standard covers the skills and knowledge required to install physical termite barriers, including barriers that combine physical and chemical characteristics. The unit requires the ability to assess techniques and methods of construction, as well as techniques required to prevent termites gaining concealed access to a completed construction. It requires a thorough understanding of client requirements, and the application of company information in order to perform the task. The selection of appropriate equipment, materials and methods is essential for performing the task safely and efficiently.

| Competency Units | Performance Criteria |
| --- | --- |
| - Plan for construction process | - Identify installation requirements for termite barrier from work order, drawings and supervisor’s instructions. - Review work site plan and select installation specification to prevent concealed termite access according to work order and legislative, occupational health and safety (OHS) and company requirements. - Advise client of installation procedure according to work order and company requirements. - Recognise and adhere to quality assurance requirements according to company requirements. - Select and locate materials in readiness for installation according to manufacturer specifications and OHS and company requirements. - Select equipment and tools consistent with installation requirements, check serviceability, and report faults to appropriate persons according to manufacturer specifications and OHS and company requirements. |
| - Access and assess site. | - Access work site according to client and company requirements. - Review installation method according to work order and company requirements, and clarify issues with appropriate persons. - Check previous trades work for possible effect on proposed installation of termite barrier according to company requirements. - Check previous trades work for possible effect on proposed installation of termite barrier according to company requirements. - Assess work site for access and safety issues according to legislative, OHS and company requirements. |
| - Prepare site for installation | - Identify hazards and control risks at the work site according to company, legislative, OHS and public health and safety requirements. - Determine usage pattern of work site to ensure safety of personnel and efficient use of equipment and tools. - Identify factors affecting completion of work order and advise appropriate persons promptly. |
| - Prepare equipment | - Select suitable personal protective equipment (PPE) and use according to manufacturer specifications and legislative, OHS and company requirements. - Select equipment and tools and check operational effectiveness according to manufacturer specifications and legislative, OHS and company requirements. - Set up equipment and tools according to manufacturer specifications and legislative, OHS and company requirements. - Adjust equipment and tools according to manufacturer specifications to suit operator’s requirements. |
| - Install physical barrier | - Install termite barrier to concealed termite access points according to work order, manufacturer specifications and company requirements. - Check installation of termite barrier according to company requirements. - Conduct work using safe operating practices according to manufacturer specifications and environmental, legislative, OHS and company requirements. |
| - Restore work site and complete documentation | - Clean equipment, tools and PPE according to manufacturer specifications and environmental, OHS and company requirements. - Check equipment, tools and PPE safely and record required maintenance according to manufacturer specifications and OHS and company requirements. - Store cleaned equipment, tools and PPE according to company requirements. - Dispose of collected waste according to work order, manufacturer specifications and environmental, legislative, OHS and company requirements. - Update client records regulatory and complete company documentation and distribute according to legislative and company requirements. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe legislation, regulations, codes of practice and industry advisory standards that apply to the installation of physical termite barriers, including: -OHS legislation, relevant Australian standards and relevant license requirements

- State company management structure and procedures, including: - emergency response procedures, manual-handling techniques, OHS procedures, and reporting channels

- Describe construction methods and materials, such as:- concrete slabs, footings and wall frame materials

- State hazardous substances in the workplace

- State installation equipment

- Describe product knowledge, including manufacturer specifications for equipment and products being used

- Describe subterranean termite ecology as related to termite barrier installation

- Explain work order specifications

Critical Evidence(s) Required

- The candidate needs to produce following critical evidences in order to competent in this competency standard.

carry out correct procedures prior to and during installation of physical termite barrier

communicate to support team and ensure safe and effective workplace operations

comply with:- company policies and procedures, including quality assurance, legislative and regulatory requirements, product installation specifications

identify hazards and risks associated with the installation

safely and effectively use hand and power tools, plant and equipment

achieve outcomes in relation to work order.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab equipment adhesive applicators angle grinders bolsters brushes and brooms drills: - power - rechargeable generators hammers hoses knives measuring tapes or rules power leads rollers safety switch equipment screwdrivers sheet metal or utility cutters or snips shovels. |

### Erect and dismantle slip formwork

Overview: This competency standard covers the skills and knowledge required to plan and prepare, set out formwork, assemble core form system, locate and install penetrations conduct slip system and clean up.

| Competency Units | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details from relevant information for planning and preparation purposes. - Follow safety (OHS) requirements in accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Select plant, tools and equipment to carry out tasks in consistent with job requirements, check for serviceability, and rectify or report any faults prior to commencement. - Calculate material quantity in accordance with plans, specifications and quality requirements. - Identify, obtain, prepare, and safely handle materials appropriate to the work application, including required fire resistance rating, ready for use. - Identify environmental requirements for project in accordance with environmental plans and statutory and regulatory authority obligations. |
| - Set out formwork. | - Locate set out points and lines according to engineer's drawings, survey datum points and site plan for formwork erection. - Locate slip form formwork positioning to predetermine set out. |
| - Assemble core form system. | - Erect and fix internal prefabricated system wall form shutters into location according to engineer's drawings. - Fit system to concrete nib walls to heights consistent with engineer's requirements. - Install shear key feet to manufacturer specifications. - Fit platforms and assembly into core to manufacturer specifications. - Fit structural steel system grid work and hydraulic hose lines to manufacturer specifications. - Suspend wall form shutters and fit & complete system cladding and platforms to manufacturer specifications and engineer's requirements. |
| - Locate and install penetrations. | - Set out locations and dimensions of penetrations in accordance with designated tolerance from engineering drawings. - Construct penetrations, block-outs and cast-in services where required to engineering drawings. - Install penetrations to requirements of engineering drawings. |
| - Conduct slip system. | - Erect and fix external prefabricated system wall shutters to engineers and manufacturer specifications. - Loosen and strip shutters according to manufacturer's and OHS requirements. - Communicate with rigger to activate electrical/ hydraulic jacking of the system to the new position. - Install trailing platforms to engineer's specifications. - Dismantle formwork in accordance with manufacturer and engineer's specifications. |
| - Clean up | - Clear work area and dispose of or recycle materials in accordance with project environmental management plan - Clean, check, maintain and store plant, tools and equipment |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of: common formwork faults, problems and suitable rectifications

- Describe common formwork faults, problems and suitable rectifications

- State concrete properties

- Define construction terminology

- Describe electrical/hydraulic jacking systems

- Describe formwork materials and techniques

- Describe hydraulic pressures applied to formwork

- Explain job safety analysis (JSA) and safe work method statements

- State material safety data sheets (MSDS)

- Describe materials storage and environmentally friendly waste management

- Read, Interpret plans, specifications and drawings

- Describe plant, tools and equipment types, characteristics, uses and limitation

- Describe processes for the calculation of material requirements

- State quality requirements for slip form formwork

- Describe workplace and equipment safety requirements.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

Locate, interpret and apply relevant information, standards and specifications

Comply with site safety plan, OHS regulations and state and territory legislation applicable to workplace operations

Comply with organizational policies and procedures, including quality requirements

Safely and effectively use tools, plant and equipment

Communicate and work effectively and safely with others

Complete, as part of a team, the erection and dismantling of the slip form formwork for one operation.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab equipment air compressors and hoses chisels hammers levelling equipment measuring tapes and rules nail bags pneumatic wrenches power drills power leads proprietary slip form formwork spanners spirit levels. |

### Install trench support

Overview: This competency standard covers the skills and knowledge required to install trench and excavation support on a new or existing site

| Competency Units | Performance Criteria |
| --- | --- |
| - Plan and prepare for installing trench support | - Access, interpret and apply trench support documentation, and ensure the work activity is compliant - Obtain, read, interpret, clarify and confirm work requirements - Identify and address potential risks, hazards and environmental issues, and implement control measures - Select and wear personal protective equipment appropriate for work activities - Identify, obtain and implement traffic signage requirements - Select, and check for faults, equipment and/or attachments for work activities - Obtain and interpret emergency procedures, and be prepared for fire/accident/emergency |
| - Install trench shoring | - Communicate with plant operator and ensure the excavation of trenches complies with site plan, line and depth - Determine and prepare shoring method - Set out positioning of shoring - Position/erect shoring within the trench - Secure shoring in position and ensure structural conformity with regulations - Clean out excavation - Locate ladders for safe access and egress |
| - Remove trench shoring | - Release jacking mechanisms and remove ladders - Check shoring and prepare it for lifting from the trench - Remove shoring from trench and store it |
| - Conduct housekeeping activities | - Clear work area and dispose of or recycle materials - Clean and maintain condition of equipment, ensure suitability for use, and address/report issues - Manage/report hazards, and maintain a safe working environment - Process written records |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe excavation techniques

- Interpret Plans, specifications and drawings

- Define Shoring

- Describe types & components of shoring

- Workplace and equipment safety requirements.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- effectively use tools, plant and equipment; and

- mark out and excavate site as to without damaging services.

- Support trench – timbering in trenches.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab equipment Battens, props, planks for shoring sheet piling trench shields timber sets |

### Construct underpinning

Overview: This competency standard covers the skills and knowledge required to It includes planning and preparing, setting out and preparing sections, excavating sections, constructing underpinnings, and cleaning up. Licensing, legislative, regulatory and certification requirements that apply to this unit can vary between states, territories, and industry sectors. Relevant information must be sourced prior to application of the unit.

| Competency Units | Performance Criteria |
| --- | --- |
| - Plan and prepare | - Access, interpret and apply compliance documentation relevant to the work activity - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details to the allotted task - Obtain and confirm safety requirements from the site safety plan and organisational policies and procedures, and apply to the allotted task - Identify, obtain and implement signage requirements from the project traffic management plan - Select tools and equipment to carry out tasks consistent with the requirements of the job, check for serviceability and rectify or report any faults - Identify environmental protection requirements from the project environmental management plan, and confirm and apply to the allotted task |
| - Set out and prepare sections | - Identify and record underpinning requirements for adjoining properties and roadways from site drawings and surveys of the surrounding construction - Identify and implement sections and sequence of excavations from the planned schedule for underpinning - Check existing shoring system for soundness and conformation to specifications and design requirements - Set out sections for excavation to underpinning specifications and schedule - Identify and protect areas for excavation and surrounding working space requirements to provide a safe working area - Erect barricades and signage, where required, in accordance with the site safety plan - Position plant and equipment for excavations |
| - Excavate sections | - Excavate designated sections to the designed depth of footings - Install trench and excavation support in accordance with soil characteristics, safety considerations and the job safety analysis |
| - Construct underpinning | - Construct underpinning in accordance with job specifications - Backfill excavated sections - Underpin alternate sections in sequence according to underpinning schedule |
| - Clean up | - Clear work area and disposed of or recycled materials in accordance with project environmental management plan - Clean, check, maintain and store plant, tools and equipment |

Knowledge & Understanding

- site and equipment safety requirements

- foundation work

- civil construction terminology

- underpinning techniques

- construction principles

- processes for interpreting engineering drawings

- soil, sand, rock, clay, shale, gravel and silt types and characteristics

- water erosion

- equipment types, characteristics, technical capabilities and limitations

- operational, maintenance and basic diagnostic procedures

- site isolation and traffic control responsibilities and authorities

- materials safety data sheets and materials handling methods

- project quality requirements

- JSAs/safe work method statement

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- knowledge of the requirements, procedures and instructions for constructing underpinnings

- implementation of requirements, procedures and techniques for the safe, effective and efficient completion of underpinning construction

- working with others to undertake and complete the construction of underpinnings that meets all of the required outcomes

- consistent timely completion of underpinning construction that safely, effectively and efficiently meets the required outcomes

- construction of underpinning for a minimum of two projects, one using drilling techniques and one using excavation techniques to the required job specifications

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab equipment compressors fittings and hoses pneumatic picks and jack hammers pumps winches sheet piling trench shields timber sets |

### Carry out reinforced concrete work

Overview: this competency standard covers the skills and knowledge required to plan and prepare work, place and fix reinforcement, carry out concrete placement and clean-up work site area.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare work | - Recognize Quality Assurance requirements of construction operations and adhered to. - Recognize OH&S requirements for application tasks and workplace environment adhered to. - Identify reinforcement, type of fixing and locations from instructions/reinforcement schedule, job drawings and specifications. - Check formwork/excavation for completion and conformity to receive reinforcement. - Select, fit and use appropriate personal protective equipment correctly. - Select tools and equipment consistent with job requirements - Check delivered reinforcement for correct size type and quantities against reinforcement schedule/details shown in job detail drawings |
| CU2.Place and fix reinforcement | - Cut and bend reinforcement bars to required set -out and drawing details. - Tie bars to designed configuration from drawings. - Cut reinforcement sheets to required sizes, where applicable. - Attach stiffening rods to panels to instructions as required to facilitate handling processes - Locate bar chairs/spacers to requirements of reinforcement schedule and job drawings. |
| CU3.Carry out concrete placement | - Provide assistance provide with the undertaking of relevant concrete tests. - Transport concrete correctly and safely with wheelbarrow and discharge into formwork using correct manual handling techniques. - Place concrete to instruction, minimizing spillage. - Compact concrete to specification and instruction using immersion vibrator or other specified method. - Screed concrete to specified levels/grades as per instructions. - Finish concrete to instruction to specified surface finish. - Identify and apply curing process. - Cover concrete surface adequately with appropriate material to support curing process and protect it from damage. |
| CU4.Clean-up work site area | P1. Clear work are by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to plan and prepare work, place and fix reinforcement, carry out concrete placement and clean-up work site area required to carry out tasks covered in this competency standard. This includes the knowledge of:

- workplace and equipment safety requirements

- formwork for concrete

- portable power tools, hand tools

- plant and equipment

- materials relevant to steel-fixing

- materials handling

- measurement and calculation

- drawings and specifications

- reinforcement schedule

- appropriate steel-fixing procedures and legislative requirements

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- demonstrate compliance with Occupational Health and Safety regulations applicable to workplace operations

- select and use appropriate processes, tools and equipment

- apply quality procedures and processes within the context of fixing steel reinforcing

- check materials for conformity with specifications and job requirements

- Identify and follow assembly location and placement sequence

- demonstrate safe and effective use of tools and equipment and handling of materials

- place and tie/weld reinforcement to specification

- interactively communicate with others to ensure safe and effective operations in fixing the reinforcing

Tools and Equipment

- shovels and rakes

- wooden floats

- steel floats

- bull floats

- immersion vibrator or vibrating table

- tarpaulins/covers

- curing agent applicator

- steam generator

- wheelbarrow

- tamping rods

- screed boards

- edging tool

- brooms

- bolt cutters

- hacksaw

- wire nippers

- tie wire spool

- measuring tape/rule

- reinforcement bender

### Work safely on roofs

Overview: This competency standard covers the skills and knowledge required to plan and prepare, prepare for work, perform work on roof and clean-up work site area

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare | - Select tools and equipment as required, - Check for serviceability and fix/report any faults - Calculate material quantity requirements in accordance with specifications - Obtain needed materials needed and check for compliance |
| CU2.Prepare for work | - Verify suitability of the roof structure to support the roof safety system. - Inspect and install fall protection and perimeter protection equipment for conformance |
| CU3.Perform work on roof | - Check access from ground to the roofing work area to ensure it is safe and clear of obstructions and hazards - Secure Roof materials and equipment safely on roof and distribute to eliminate risk of distorting or collapsing the building framework. - Check roof safety system is checked periodically for optimum performance, and correct/report faults |
| CU4.Clean-up work site area | P1. Clear work are by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to plan and prepare, prepare for work, perform work on roof and clean-up work site area required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Working safely on roofs

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- erect, maintain and dismantle, for a roof corner area, fall and perimeter protection,
incorporating handrails and foot walk or harnesses and harness fixing points for safe access
to the roof, stores and equipment locations

Tools and Equipment

- Fall protection

- Ladders

- Chain blocks, cranes

- Elevated work platforms

- Forklifts

- Hand trolleys, hoists and jacks

- Rollers

- Scaffolds

- Perimeter protection

### Install roof components

Overview: This competency standard covers the skills and knowledge required to install roof components. Site location for work application may be either domestic or commercial and may be a new work site or an existing structure being renovated, extended, restored or maintained.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Prepare for work | - Obtain plans and specifications - Select and install industrial roof components according to Work health and safety (WHS) and environmental - requirements - Identified and adhere quality assurance requirements to according to workplace requirements. - Plan and sequence tasks in conjunction with others involved in or work. - Select and check tools and equipment, including personal protective equipment for serviceability. - Prepare work area to support efficient selection and installation of industrial roof components. |
| CU2.Identify Installation requirements. | - Identify Industrial roof components from plans and specifications and are compatible with adjacent materials and other relevant information. - Select and check proposed sealant, fixing materials, and roofing and flashing materials for compliance with plans, specifications and relevant standards. - Identify, order and collect Materials according to workplace procedures. - Check materials and equipment for compliance with docket and order form and for acceptable condition, and report faults - Observe sustainability principles and concepts when preparing for and undertaking work process. - Identify location of installation and opening size from plans and manufacturer specifications. - Check location for obstructions that may affect installation. |
| CU3.Install roof components. | - Erect industrial component to comply with manufacturer recommendations. - Set out industrial roof component to comply with plans, specifications and site measurements - Install structural supports to comply with plans and specifications. - Prepare openings to comply with plans. - Locate and fix Mullions, frames, baffles, sheeting and louver blades to comply with manufacturer recommendations. - Install flashing and waterproofing to comply with relevant standards, plans, specifications, manufacturer recommendations and regulatory authorities' requirements. - Test installation performance. |
| CU4.Clean-up work site area | P1. Clear work are by disposing of materials. P2. Clean, check, maintain and store tools and equipment’s according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- capillary action, thermal expansion and fabrication techniques to prevent leaking

- installations

- corrosion prevention treatment requirements of cut sheets

- electrolysis and problems associated with the use of dissimilar metals

- job safety analysis (JSA) and safe work method statements (SWMS)

- processes of erecting and installing industrial roof components

- relevant WHS regulations and fall protection codes and requirements

- relevant statutory requirements related to installing roof components

- SI system of measurement

- types of fasteners, fixings and sealants and their application to the installation of roof components

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- locating, interpreting and applying relevant information, relevant  standards and  specifications to determine requirements, plan and install industrial components in roofs

- applying safety requirements throughout the work sequence, including electrical safety requirements and the use of personal protective clothing and equipment

- given the plans and specifications of an industrial roof, determine the material requirements, selecting and installing two of the following components in aroof (one on a ridge line and another in roof sheeting, which may include insulation):

- thermal vent or a skylight

- non-mechanised ventilator unit

- manual box-type Louvre unit

- continuous roof ventilator (ridge or slope mounted), ensuring:

  - application of sustainability principles and concepts

  - correct identification of requirements and details of proposed installations

  - completed installation must fit

  - correct selection and use of appropriate processes, tools and equipment

  - completing all work to specification

  - compliance with regulations, relevant standards and organizational quality procedures and processes

  - communicating and working effectively and safely with others

Tools and Equipment

- cranes

- fall protection equipment

- hand and power tools

- ladders

- levelling equipment

- lifting and load shifting equipment, including:

- chain blocks

- elevated work platforms

- forklifts

- hand trolleys

- hoists and jacks

- restricted height scaffolds

- rollers

- measuring equipment

- Perimeter guard rails.

### Install roof drainage

Overview: This competency standard covers the skills and knowledge required to install roof drainage components and rainwater goods for commercial and residential roof systems

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare | - Select tools and equipment as per specifications, - Check for serviceability and any faults - Calculate material quantity requirements in accordance with specifications - Obtain needed materials and check for compliance |
| CU2.Identify installation requirements | - Identify roof drainage components required for installation from drawings and specifications. - Draw fabrication patterns based on design and drawing of roof drainage. - Calculate quantity, type and sizing of drainage components, rain water materials and accessories required from drawings and specifications. - Install gutter support system to comply with engineering advice for the building. - Mark out materials for fabricating drainage components, as specified in drawings |
| CU3.Fabricate roof drainage components | - Determine methods of fabrication, tools and materials requirements based on drawings and job requirement - Fabricate roof drainage components in compliance with drawings and specifications |
| Set out and install roof drainage components | - Arrange roof drainage components in order of installation and to comply with and site measurements. - Install structural supports and join roof drainage components securely and watertight. - Install flashing and waterproofing to comply with the plans - Test the performance of the roof-drainage system to determine satisfactory installation and remedy any faults or leaks |
| CU4.Clean-up work site area | P1. Clear work are by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Capillary action, thermal expansion and fabrication techniques to prevent leaking
installations

- Characteristics of various metals and finishes

- Corrosion prevention treatment requirements of cut sheets

- Electrolysis and problems associated with the use of dissimilar metals

- Joining of materials

- Processes of fabricating, jointing and fixing roof drainage components

- Types of fasteners, fixings and sealants and their application to the fabrication and
installation of roof coverings

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- For a roofed area of at least 4 square metres, determine requirements, select, fabricate and install valley gutter, box gutter, eaves gutter and downpipe system, with gutter supports, expansion joints and caps.

Tools and Equipment

- Drawing equipment

- Fall protection equipment

- Hand and power tools

- Ladders

- Lifting and load shifting equipment, including:

- Chain blocks

- Cranes

- Elevated work platforms

- Forklifts

- Hand trolleys

- Hoists and jacks

- Restricted height scaffolds

- Rollers

- Measuring equipment.

### Lay roof tiles

Overview: This competency standard covers the skills and knowledge required to Plan and prepare, Prepare roof face, Cut and fix battens, Install tiles and Clean-up work site area.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare | - Select tools and equipment as per requirement. - Check for serviceability and any faults - Calculate material quantity requirements in accordance with specifications - Obtain needed materials and check for compliance |
| CU2.Prepare roof face | - Install fall arrest system to roof perimeter and roof surface and structure and check for stability and safe access - Assemble tile elevator and operate in accordance with site conditions and the manufacturers’ instructions. - Remove existing roofing material without damage to the roof frame - Clean areas already tiled with high-pressure water cleaners to the new installation’s condition. - Obtain required materials and calculate required tiles minimizing quantity by planning efficient cuts at the hip and valley joints. - Set out roof surface to manufacturer recommendations for specified tile. |
| CU3.Cut and fix battens | - Measure battens, cut and fixe using selected fasteners at specified centres. - Check completed sarking and batten work for correct alignment and fixing |
| Install tiles | - Load tiles onto roof, support and distribute evenly. - Spread, cut, secure and install roof tiles to manufacturer specifications. - Install flashing and waterproofing to comply with the plans and manufacturers’ recommendations. - Bed roof tiles using correct mortar mix, maintaining alignment. - Point roof tiles with mortar to a flush, smooth finish and install to manufacturer specifications |
| CU4.Clean-up work site area | P1. Clear work are by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding = required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Characteristics and applications of roof tiling materials

- Roof construction systems and structures and tiling considerations

- Roof tiling techniques and procedures

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Tile 45 square metres of a roof frame, incorporating a 1.5m hip, a 1.5m valley, a 1.5mgable and a 1.5m top ridge, including the appropriate sarking, battens, tile cutting, bedding and pointing

Tools and Equipment

- Air compressors and hoses

- Bedding frames, brooms, buckets

- Calculators, chalk lines, chisels, concrete mixers

- Elevators

- Fall safety devices

- Gutter protectors

- Hammers, hand saws, high pressure water cleaners

- Ladders

- Measuring tapes and rules

- Nail guns

- Pinchers, power drills, power saws

- Shovels, squares, string lines

- Tile cutters, trowels

### Erect roof trusses

Overview: This competency standard covers the skills and knowledge required to Plan and prepare, Locate roof trusses and Clean-up work site area.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare | - Select tools and equipment as per requirement. - Check for serviceability and any faults - Calculate material quantity requirements in accordance with specifications - Obtain needed materials and check for compliance |
| CU2.Locate roof trusses | - Set out location of roof trusses for hip and valley roofs on wall top plates to plan layout and specifications. - Earth steel frames temporarily during erection and connect to permanent earthlings system upon completion. - Erect and fix roof trusses including temporary bracing, to set out positions in correct sequence to line at apex and plumb. - Install top chord above wall plate to be constant height above wall plate. - Fix ceiling trimming and creeper rafter member to specifications. - Use bottom chord of truss to provide lateral support for internal walls. - Provide roof bracing through h construction, valley construction, diagonal metal tension or timber bracing or a combination of these, and fixed to specification. - Fix lateral restraints to truss chords in position to manufacturer specifications. |
| CU3.Clean-up work site area | P1. Clear work area by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Plans, specifications and drawings

- For roof trusses

- Roof bevels

- Roof calculations for lengths, quantities and pitch

- Roof load transfer

- Roof shape and geometry

- Roof truss erection and construction techniques

- Roof truss materials and installation, including fire control and separation materials

- Roof types and truss components

- Techniques for lifting and positioning of trusses

- Temporary and permanent bracing

- Timber types, structural properties and uses, including engineered timber products

- Truss set out

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Set out and erect a roof comprising a minimum of one hip end, a valley and enough
standard trusses to incorporate bracing for a full size roof in timber and metal.

Tools and Equipment

- Air compressors and hoses

- Clamps

- Hammers

- Marking equipment, measuring tapes and rules

- Nail bags, nail guns

- Power drills Power leads Power saws

- Saw stools, scaffolding, screwdrivers, spanners, spirit levels

- Squares (combination/tri), string lines

- Welding equipment.

### Erect and dismantle formwork to suspended slabs, columns, beams and walls

Overview: This competency standard covers the skills and knowledge required to Plan and prepare, erect formwork, Strip formwork and Clean-up work site area

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare | - Select tools and equipment as per requirement. - Check for serviceability and any faults - Calculate material quantity requirements in accordance with specifications - Obtain needed materials and check for compliance |
| CU2.Erect formwork | - Clear work area and prepare surface for safe erection of form work for suspended slabs, piers and shutters. - Set out formwork to requirements of plans and specifications. - Assemble formwork according to plans, specifications and class of surface finish, with support system set to correct height level and line. - Place bracing of formwork according to support plans and specifications to maintain rigidity and stability. - Erect formwork support system sequentially according to initial set out. - Construct formwork shutters form requirements and specified dimensions. - Install Block-outs and cast-in services to specified locations. - Remove debris, sawdust and other waste materials from completed formwork in accordance with waste management policy for the site. - Apply release agent to formwork face to manufacturer specifications where specified. |
| CU3.Strip formwork | - Remove formwork and bracing/strutting support sequentially and safely. - De-nail, clean and store or stack timber components safely for reuse or removal from site. - Clean, oil and store or stack steel components to manufacturers' maintenance recommendations. - Discard damaged formwork components safely after stripping. |
| CU4.Clean-up work site area | P1. Clear work are by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Formwork materials and techniques

- Hydraulic pressures applied to formwork

- Plans, specifications and drawings

- Purpose, application and properties of commonly used release agents

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Set out and erect suspended slab formwork (slab size a minimum of 30 square metres), incorporating a beam and two different types of columns with a specified formwork system at a minimum height of 2.4 metres.

Tools and Equipment

- Air compressors and hoses, automatic levels

- Chisels

- Formwork equipment

- Hammers, hand saws

- Ladders and scaffolding

- Marking equipment, measuring tapes and rules

- Nail bags, nail guns

- Pinch bars, power drills power grinders, power leads, power saws

- Saw stools, shovels, spanners spirit levels, squares (combination/tri)

- Steel squares, string lines

### Operate elevated platforms

Overview: This competency standard covers the skills and knowledge required to Plan and prepare, Conduct routine checks of Platform, Locate equipment in place for work application, Elevate platform to work location, Lower platform and shut down and Clean-up work site area

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare | - Recognize and OH&S requirements associated with operating elevating work platforms and workplace environment - Identify workplace operations plan in accordance with job requirements and surrounding activities and environment - Select appropriate personal protective equipment. - Select equipment consistent with job requirements and check for serviceability - Identify safety hazards and use procedures correctly to minimize risks to self and others. - Select materials in accordance with job drawings and/or from specifications/supervisor’s instructions |
| CU2.Conduct routine checks of Platform | - Determine power source where applicable and connect platform equipment to manufacturer's specifications. - Carry out routine pre-operational equipment checks in accordance with checklist from operator's manual. - Switch equipment on, in accordance with start-up procedures and check controls for correct operation and ease of movement. - Check emergency safety devices to instructions from operator's manual. - Check work location for level ground and floor surface to determine stabilizing and safe working area requirements. |
| CU3.Locate equipment in place for work application | - Locate platform in position for work application and engage stabilizers to set equipment base level into place. - Erect barricades and signage to isolate safe working area where applicable. - Place tools, equipment and materials into bucket/platform to job application requirements |
| CU4. Elevate platform to work location | - Operate controls to manufacturer’s recommendations and elevate platform to work position. - Switch power off and engage locking devices to operator's manual. - Carry out work to job specification and safety requirements of operator's manual. |
| CU5. Lower platform and shut down | - Operate controls to manufacturer's recommendations and lower platform to down position. - Carry out shut down procedures to operator's manual and switch off equipment |
| CU6.Clean-up work site area | - Remove and dispose of waste material of safely - Seal and store/stack unused materials - Remove, clean, maintain and store tools and equipment - Disengage stabilizers, store and secure equipment and remove unit from location - Carry out routine post-operational checks in accordance with checklist from operator's manual and report any faults |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- workplace and equipment safety

- requirements relative to EWP’s

- a range of EWP's

- operation and maintenance of equipment

- use of hand and power tools

- drawings and specifications

- communication processes – verbal and signalling

- materials and material fixing

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- demonstrate compliance with Occupational Health and Safety regulations applicable to workplace operations

- show compliance with organizational policies and procedures including Quality Assurance requirements

- carry out correct procedures prior to, during and after use of elevating work platform

- demonstrate safe and effective operational use of plant, tools and equipment

- demonstrate safe and effective work application while in set elevated position

- interactively communicate with others to ensure safe and effective workplace operations

Tools and Equipment

Elevating work platforms include but are not limited to:

- scissor type

- extending arm (cherry picker)

Personal protective equipment may include:

- coveralls

- safety boots

- hard hat/cap

- gloves

- safety glasses/goggles

- ear plugs/muffs

- dust masks/respirator

### Construct, erect and dismantle formwork for stairs and ramps

Overview: This competency standard covers the skills and knowledge required to Plan and prepare, Set out formwork, Assemble and erect formwork, Strip formwork and Clean up

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare | - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details, from relevant information for planning and preparation purposes. - Follow safety (OHS) requirements in accordance with safety plans and policies. - Identify and implement signage and barricade requirements - Select plant, tools and equipment to carry out tasks consistent with job requirements, check for serviceability, and rectify any faults or report prior to commencement. - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Identify, obtain, prepare, handle safely and locate materials appropriate to the work application ready for use. - Identify and apply environmental requirements for project in accordance with environmental plans and statutory and regulatory authority obligations |
| CU2.Set out formwork | - Identify design of stairs and/or ramps and method of joining formwork from job drawings and specifications, and check to be in accordance with legislation, regulations and codes of practice. - Determine exit and ground finish levels from plans, specifications and site inspection. - Determine rise, going and pitch of stairs and ramp from plans, specifications, site inspection, rise measurements and requirements of Building Code. - Calculate full size set out of stairs and ramp and make to determine rise, going and pitch of stairs to provide location of landings, stringers, treads and posts where specified. - Determine location of stair, ramp and newels from plans, specifications and pitch of stairs. - Set out location of footings to layout of designed stairs and ramp from plans, specifications - Select and set out materials for formwork, including stringers, to pitch of stairs with rises not exceeding specified space between treads |
| CU3.Assemble and erect formwork | - Place, fix and brace landing bearers and joists according to plans and specifications, where specified. - Check footings for accuracy to requirements of plans and specifications. - Erect and brace stairs and/or ramp formwork soffit is in accordance with plans and specifications. - Cut square formwork stringers to length and shape in accordance with set out and junction with newel posts, and install and brace. - Screw/bolt metal angle brackets to formwork strings to set out locations of tread support. - Locate and fix formwork strings for stairs and ramps according to plans and specifications. - Set out, cut square material for face of treads length and fix to metal angle bracket according to plans and specifications. - Brace risers at mid-span to prevent deflection under the load of wet concrete. - Erect and brace newels temporarily to plumb position, where specified. - Located and Secured formwork tie bolts to plans and specifications to maintain stair width where specified |
| CU4.Strip formwork. | - Remove formwork and bracing/strutting support sequentially and safely. - De-nail, clean and store or stack timber components safely for reuse or removal from site. - Clean, oil and store or stack steel components to manufacturers' maintenance recommendations. - Discard damaged formwork components safely after stripping. |
| CU5.Clean up | P1. Clear work are by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- application and requirements for line, level and plumb in construction projects

- construction terminology

- formwork materials and techniques

- job safety analysis (JSA) and safe work method statements

- material safety data sheets (MSDS)

- materials storage and environmentally friendly waste management

- plans, specifications and drawings

- plant, tools and equipment types, characteristics, uses and limitation

- processes for setting out and measuring

- processes for calculating material requirements

- quality requirements for formwork for stairs and ramps

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- locate, interpret and apply relevant information, standards and specifications

- comply with site safety plan, OHS regulations and state and territory legislation applicable to workplace operations

- comply with organizational policies and procedures, including quality requirements

- safely and effectively use tools, plant and equipment

- communicate and work effectively and safely with others

- complete a site assessment to determine levels and stair dimensions

- apply the height and going of stairs to construct, erect and dismantle the form work for a flight of stairs (free standing or against a wall), including a landing, with a minimum rise of flight of 1800mm

- Complete the construction, erection and dismantling of a ramp with a rise of 400mm.

Tools and Equipment

- air compressors and hoses

- automatic levels

- chisels

- explosive power tools

- grinders

- hammers

- hand saws

- laser levels

- marking equipment

- measuring tapes and rules

- nail bags

- nail guns

- optical levelling equipment

- pinch bars

- power drills

- power leads

- power saws

- props

- saw stools

- scaffolding

- shovels

- spanners

- spirit levels

- squares (combination/tri)

- steel squares and bevels

- string lines

### Evaluate materials for multi-storey buildings

Overview: This competency standard covers the skills and knowledge required to assess the nature and performance of concrete for use in Multi-storey buildings, Assess application of concrete used in

Multi-storey buildings, Evaluate methods undertaken to repair concrete, Evaluate effects of fire and heat on concrete used in multi-storey buildings and Monitor Environmental impacts of building materials used

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Assess the nature and performance of concrete for use in Multi-storey buildings. | - Identify and record plastic and hardened concrete properties for building types. - Enlist sources of aggregate and properties of each described. - Describe and record effects of impurities - Conduct manufacture and testing of concrete in accordance with relevant standards. |
| CU2.Assess application of concrete used in Multi-storey buildings. | - Determine selection and distribution methods of concrete following analysis of site access. - Demonstrate and maintain correct distribution and placement methods of concrete - Identify reasons and effects of compaction on both plastic and hardened concrete - Compare immersion, surface and form vibration - Maintain accurate records relating to the application of concrete - Identify and record types of curing methods and detrimental effects of poor or no curing |
| CU3.Evaluate methods undertaken to repair concrete | - Identify live and dormant cracks - Describe and record repair methods and causes of cracked concrete and concrete cancer in accordance with organizational procedures. - Diagnose and record faults in concrete in accordance with organizational procedures. |
| CU4.Evaluate effects of fire and heat on concrete used in multi-storey buildings. | - Test reinforced concrete for effects of fire and heat. - Identify and apply methods of fire protection for concrete elements |
| CU5.Monitor Environmental impacts of building materials used | - Monitor and apply new technologies in concrete for construction of multi-storey buildings in accordance with organizational policies and guidelines. - Identify and apply performance requirements of concrete in fire resistance construction in accordance with acceptable standard construction practices. - Identify cost-effectiveness of using recycled materials in accordance with acceptable standard construction practices. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- application of Building Code and standards

- applications of structural principles in buildings

- design principles and behaviour of structural members undergoing stress, strain,

- compression, bending or combined actions

- grading process and grade markings used to categorise timber and timber products

- OHS requirements, legislative codes and practices

- types and nature of materials and effect of their performance, including properties and uses of cement and types of hydraulic cement

- Work drawings and specifications.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- documentation, including design brief drawings, specifications, codes, design concepts, construction schedules and other necessary supporting documents

- research resources, including timber product information and samples

- access to relevant legislation, regulations and codes of practice

- Relevant computer software package and suitable hardware.

### Apply structural principles to the construction of large, high rise and complex buildings

Overview: This competency standard covers the skills and knowledge required to apply structural principles to the construction of large, high rise and complex buildings

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Apply structural principles to the planning of the erection or demolition of a structure. | - Identify main structural principles that apply to the erection of large, high rise and complex structures - Identify, analyse and apply performance characteristics of the structural elements, including materials identified in the building plan, to the planning of the construction work. - Undertake demolition of existing structures in accordance with legislative and planning requirements and safe work practices. |
| CU2.Coordinate and manage the site assessment and job set-up | - Put processes in place to analyse stability of soils and capacity of the site to support the construction loads. - Identify requirements for retaining walls in conjunction with related industry professionals and apply to the planning process. - Analyse and apply structural function and requirements for temporary structural elements to the planning process. |
| CU3.Coordinate and manage construction of footing systems | - Undertake coordination of the set-out of the building in accordance with documented building plans, following the full assessment of the site. - Assess structural performance of the footings specified in the building plan for compliance with relevant codes and accepted industry construction principles. - Lay and check footings, as specified in the building plan, are for compliance with standards and accepted industry construction principles. - Plan, implement and check damp coursing and the provision of termite barriers and other relevant techniques in accordance with codes, standards and industry practice |
| CU4.Coordinate and manage structural elements of the construction process. | - Identify and analyse technical construction principles and performance characteristics of construction materials in the planning of project. - Identify, implement and check processes for construction of all structural elements for compliance with manufacturer specifications and relevant standards and codes. - Identify and implement building plans and relevant standards and codes to ensure appropriate allowances have been made for plumbing, electrical conduits and other services to be installed. |
| CU5.Analyze and plan for Structural integrity of buildings. | - Consult relevant industry professional to provide advice regarding the structural integrity of proposed building. - Assess structural requirements and loads of the building design. - Conduct analysis of the effects of force and movements on structural elements. - Conduct analysis of properties and behaviours of structural materials is conducted. - Conduct analysis of section properties of structural elements using standard industry formulas and performance comparisons. - Evaluate performance characteristics of columns using standard industry techniques. - Assess methods used for stress distribution in connections between structural elements - Assess impact of various loads on the building structure - Consider design impact of high performance structural elements |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- applications of structural principles in buildings

- Building Code and standards

- design principles and behaviour of structural members undergoing stress, strain,

- compression, bending or combined actions

- interpretation and analysis of work drawings and specifications

- Nature of materials and effect of performance.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- coordinate professional input to evaluate structural integrity of large and complex buildings

- clearly analyse structural impact of design decisions

- comply with OHS and organisational quality procedures and processes

- apply and interpret relevant documentation and codes

- apply design principles relating to performance

- Identify typical faults and problems and the action required to rectify such faults.

### Install fire and smoke containment systems

Overview: This competency standard covers the skills and knowledge required to install fire and smoke containment systems

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Apply rules and regulations to installation operations | - Confirm and apply requirements of relevant rules and regulations to work procedures. - Check compliance requirements and take action according to organisational policies and procedures. |
| CU2.Prepare for system Installation | - Obtain work permits are prior to entering customer premises. - Discuss customer requirements and select a suitable fire and smoke containment system according to organisational and customer requirements. - Apply relevant measurement processes to installation site and passive fire and smoke containment system components. - Identify and assemble components, tools, equipment and materials required for installation - Identify and assemble fireproofing or fire-stopping materials required for installation of system |
| CU3.Preparefireproofingsubstrate or fire stopping opening for the system | - Check structural or penetrated elements for compliance with required smoke resistance or fire resistance level. - Prepare fireproofing substrate or fire stopping opening - Measure and prepare Materials for installation of fireproofing or fire stopping system |
| CU4.Install passive fire and smoke containment system | - Install fireproofing material or fire stopping system according to manufacturers’ installation instructions. - Inspect and test passive fire and smoke containment system for defects and check to ensure that installation is sound and aesthetically pleasing, according to standard testing procedures and acceptable workmanship - Rectify and report impediments according to organisational requirements. |
| CU5.Label or mark and certify passive fire and smoke containment system | - Assess and label or mark installation according to provisions of relevant standard and manufacturer, Building Code and organisational requirements. - Complete relevant schedule of evidence of compliance set out in relevant standard or logbook is completed according to relevant standard, and organisational and customer requirements. |
| CU6.Completeinstallation. | - Left customer premises in a clean and tidy condition on completion of installation functions. - Complete and distribute documentation according to organisational and customer requirements. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- action to take when a breach of health, safety or other policy occurs

- common technical terminology used in the installation of passive fire and smoke containment systems, including:

- plumb, level, square and perpendicular

- mix ratio, density, wet film thickness, dry film thickness, section factor (Hp/A), exposed surface to mass (ESM) ratio and critical temperature

- fire-rated walls, masonry, concrete, plasterboard and shaft wall

- fire-rated floors and floor ceiling systems

- penetration dimensions, i.e. width, height, depth and orientation

- ambient and cold smoke, medium smoke and elevated and hot smoke

- intumescent products and ablative products

- clearance, field of application, fire resistance levels, incipient spread of fire, spread of flame indices, fire test report, assessments and letters of opinion

- dangers of incorrect installation of passive fire and smoke containment systems

- key features of legislation, and building code requirements relating to:

- installing passive fire and smoke containment systems

- passive fire and smoke containment

- implications of not applying legislative requirements to job functions

- methods of converting imperial units of measurement to metric

- organisational requirements for labelling and certifying passive fire and smoke containment system installations

- passive fire and smoke containment labelling types

- principles of passive fire protection related to overall fire safety and containment systems specific to passive fire and smoke containment systems

- procedures for determining fire resistance levels and smoke resistance levels

- purpose and interpretation of Australian standards and technical documentation relating to passive fire and smoke containment systems

- technical terms relating to passive fire and smoke containment system installation

- Fireproofing and fire stopping variables, including types, components and fire barrier types.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- install different types of fireproofing to structural steel and ductwork:

  - spray,

  - intumescent paint

  - board

- install fire stopping systems comply with the requirements of:

- standards identified as relevant to work procedures

- building Acts, regulations and codes

- environmental regulations

- manufacturers’ specifications

- OHS legislation, codes and regulations

- organisational requirements, including OHS policies and procedures

Tools and Equipment

Fire and smoke containment products, including:

- fabric and timber protection, impregnated or sprayed

- fire-rated air transfer grilles fire-rated sealants, caulks and putties:

- acrylic

- polyurethane

- silicone

- fire resistant boards:

- calcium silicate

- vermiculite fire resistant mortars and grouts:

- load bearing

- non-load bearing fire resistant pillows:

- intumescent

- non-intumescent fire stop collars:

- cast in

- retrofit

- wall

- intumescent paints and coatings (other than those used for fireproofing structural steel)linear gaps seals:

- impregnated foams

- tapes

- intumescent gaskets

- sealants materials into or onto which passive fire and smoke containment products are installed, including:

- concrete

- fire resistant board

- masonry

- plasterboard

- steel

- timber

- UPVC

- mechanical and intumescent fire dampers

- smoke dampers passive fire and smoke containment solutions, including:

- access panels and hatches

- ceiling and partition systems

- control joints and expansion joints

- duct fireproofing and protection

- duct and damper penetration protection

- fire-rated sealants

- industrial fire protection

- intumescent dampers and seals

- openings in structural elements

- penetration solutions

- protection against spread of flame

- structural steel fireproofing and protection

- UPVC and plumbing penetrations

### Inform clients about thermal performance of residential buildings

Overview: This competency standard covers the skills and knowledge required to inform clients about thermal performance of residential buildings

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Explain thermal performance to clients | - Explain building thermal performance to clients in line with organisational procedures. - Identify impact of building design and materials on thermal performance of building and communicate to clients in line with organisational procedures. - Explain relationship between building thermal performance, heating and cooling, and energy consumption to clients in line with organisational procedures. - Explain main uses of energy in buildings and the proportion attributed to heating and cooling to clients in line with organisational procedures. - Apply correct industry terminology for energy use when communicating with clients. - Apply correct units of measurement when describing power and energy. - Explain environmental and social impacts of energy use to clients in line with organisational procedures. |
| CU2.Explain the process of heat transfer in residential buildings to clients | - Explain heat transfer to clients in line with organisational procedures. - Explain meanings of key terms used to describe heat transfer to clients in line with organisational procedures. - Explain ways in which residential buildings lose and gain heat to clients in line with organizational procedures |
| CU3.Explain factors that impact on thermal comfort to clients. | - Explain thermal comfort and its use in building thermal performance assessments to clients in line with organisational procedures. - Explain physical factors that influence thermal comfort to clients in line with organisational procedures. - Establish requirements for thermal comfort based on occupant needs and explain to clients in line with organisational requirements |
| CU4.Explain how climatic conditions influence thermal Performance of residential buildings to clients. | - Identify major climatic zones and explain their key characteristics relevant to thermal performance assessments - Explain types and uses of climate data used in carrying out thermal performance assessments to clients in line with organisational procedures - Explain impact of local climatic conditions on building thermal performance to clients in line with organisational procedures - Explain ways of improving thermal performance appropriate to the relevant climatic zone to clients in line with organisational procedures |
| CU5.Explain principles of passive design to clients. | - Explain principles of passive design to clients in line with organisational requirements. - Explain application of principles of passive design to plan and existing residential buildings to clients in line with client and organisational requirements. - Explain factors that diminish the thermal performance of buildings to clients in line with organisational procedures |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- climatic zones: characteristics of climatic zones

- climate data used in thermal performance assessment: diurnal temperature range, humidity,  irradiance, maximum and minimum temperature, solar geometry, wind speed and direction and impact of climate on thermal performance of residential buildings

- environmental and social impacts of energy use: climate change, energy cost, energy security, global warming, greenhouse gas emissions, operational costs and resource depletion

- heat transfer in residential buildings: impact on heating and cooling

- mechanisms: radiation, convection, conduction, modelling

- U and R-values of building materials

- R-values of wall constructions

- passive building design principles: building envelope, construction materials, construction methods, glazing, house plan ratio, insulation, orientation, thermal mass, ventilation, zoning, sources of information on climate

- thermal comfort: definition of thermal comfort, importance

- influences on thermal comfort: air movement, humidity, radiation, temperature, models of thermal comfort, requirements for thermal comfort, role of thermal comfort in building thermal performance assessments, thermal sensitivity of individuals

- thermal performance: factors that diminish and enhance the thermal performance of buildings,  impact of local climatic conditions on building thermal performance principles, relationship between building thermal performance, heating and cooling and energy consumption

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- explain thermal performance to clients using correct terminology

- explain the process of heat transfer in residential buildings to clients, including the ways in which residential buildings loose and gain heat

- explain factors that impact on thermal comfort to clients

- explain how climatic conditions influence thermal performance of residential buildings to clients, including the ways of improving thermal performance appropriate to particular climatic zones

- Explain principles of passive design to clients, their application in planned and existing residences, and the factors that diminish thermal performance.

### Design air conditioning and ventilation system

Overview: This competency standard covers the skills and knowledge required to design air conditioning and ventilation system

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Evaluate design parameters. | - Determine client requirements from plans, specifications and client briefs to determine work scope. - Evaluate user comfort conditions and specific use conditions and calculate psychometric evaluation and heat loads - Evaluate building heat losses. - Specify distribution requirements for air conditioning and ventilation system applications - Evaluate range of air conditioning and ventilation systems in accordance with given applications. - Evaluate mechanical services drawings and interpret. - Evaluate health risks that may arise due to poor maintenance of air conditioning and ventilation systems - Specify minimum performance requirements for several different air conditioning and ventilation systems. - Evaluate manufacturer specifications and technical manuals for suitability for a range of design applications. |
| CU2.Plan system components | - Plan appropriate zoning of air conditioning and ventilation systems to comply with given specific performance objectives. - Specify air conditioning units for optimum performance with reference to legislation, standards and other relevant documentation. - Specify fan types for a range of applications with reference to legislation, standards and other relevant documentation. - Plan air diversion systems, including registers, to ensure a balanced system with reference to legislation, standards and other relevant documentation. - Plan required ventilation and duct work components and specify locations with reference to legislation, standards and other relevant documentation. - Specify approved materials for air conditioning and ventilation systems are specified. - Complete plans and drawings using standard drawing symbols related to air conditioning and ventilation in accordance with legislation, standards and other relevant documentation. |
| CU3.Design and size systems | - Calculate volume air changes per hour from given floor plans and details. - Specify methods for eliminating health risks from existing or proposed systems with reference to legislation, standards and other relevant documentation. - Design and size range of air conditioning and ventilation systems for given applications. |
| CU4.Test systems. | - Evaluate test procedures for air conditioning and ventilation systems are evaluated. - Conduct tests using appropriate testing equipment, record results and prepare report. - Plan, design and specify adjustments required as a result of testing. |
| CU5.Prepare documentation. | - Develop appropriate checklist, including formulas required to carry out an air balance to a given specification. - Prepare plans for a range of air conditioning and ventilation systems. - Prepare specification for an air conditioning and ventilation system. - Prepare testing and commissioning schedule. - Produce operation and maintenance manual. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- air psychometrics and the use of psychometric charts

- standards, manufacturer specifications, Building Code and other applicable codes or standard operating procedures relevant to the sector

- compliance with OHS and organisational quality procedures and processes

- hazards associated with devices and systems used in the hydraulic sector

- heat load calculations

- installation methods used in hydraulic systems

- nature of materials and effect of performance

- terminology and definitions used in hydraulic design

- Work drawings and specifications.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- establishing and evaluating design parameters for a range of air conditioning and ventilation systems

- planning system components for a range of air-conditioning and ventilation systems, including:

  - air conditioning units

  - ducting systems

  - fans

  - designing and sizing a range of air conditioning and ventilation systems for given applications

- developing an appropriate checklist for tests tube carried out on the system, including formulas required to carry out an air balance

- conducting tests, recording and evaluating test results and preparing a report

- preparing plans for a range of air conditioning and ventilation systems

- preparing specifications for air conditioning and ventilation systems

- preparing testing and commissioning schedules for air conditioning and ventilation systems

- preparing operation and maintenance manuals for air conditioning and ventilation systems

Tools and Equipment

- air pressure gauge

- air velocity gauge

- air volume gauge

- humidity gauge

- Pitot tubes

- sound power levels

- Temperature.

### Design acoustic of an auditorium

Overview: This competency standard covers the skills and knowledge required to design acoustic of an auditorium

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify dimensions of hall for acoustic design | - Identify site of hall. - Make orientation, layout and structural design to keep noise within specification. - Set floor area and height to get volume of hall as per specification. - Set the floor plan as per use of hall |
| CU2. Design the seating plan | P1. Identify the distance of first line as per specified angle. P2. Set the width of seat and distance between two seats P3. Set the distance of last line from curtain line. P4. Set the height of adjacent seats. |
| CU2. Design surfaces of hall | P1. Design internal surface of roof false ceiling as per specification. P2. Design internal surfaces of walls as per specifications. P3. Set the reverberation as per use of hall according to specifications. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define terms: sound, pitch, loudness, tone, intensity of sound, reflection of sound, reverberation, line of reverberation, transmission of sound, and absorption of sound.

- Factors to be considered in acoustics of building

- Characteristics of various types of sound absorbing materials

- Design and requirements of an auditorium

- Principles to be observed in the acoustics design of an auditorium

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Drawing of acoustically designed hall

Tools and Equipment:

- Drawing tools

- Drawing sheets

### Insulate sound insulation products in building

Overview: This competency standard covers the skills and knowledge required to insulate sound insulation products in building

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Plan and prepare for installing insulation material | P1. Obtain and confirm work instructions and relevant information for planning and preparation purposes. P2. Undertake risk assessment to identify existing risks and hazards in the work area, including electrical risks and hazards. P3. Document identified risks and undertake appropriate response according to safety requirements. P4. Confirm insulating material and insulation requirements in accordance with work specifications. P5. Identify, fit correctly and use appropriate personal protective equipment (PPE) and clothing according to organisational policies and procedures. P6. Select tools and equipment appropriate to the requirements of the work, confirm for serviceability, and report for repair or replacement where not serviceable. P7. Determine associated material and organise ready for use according to quality requirements and work plans and specifications |
| CU2.Install insulation | - Measure insulation material accurately - Install insulation using approved processes and handling techniques - Install insulation safely without damage or distortion of the surrounding environment, electrical and other services and in a manner that maximises safety of self and others. |
| CU3.Complete installation | - Conduct final inspections to ensure installed ceiling insulation conforms to job and manufacturer specifications. - Clean work area and disposed of materials, reused or recycled according to organisational, safety and environmental requirements. - Clean, check, maintain and store tools and equipment according to manufacturer specifications and organisational procedures. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Sound insulation

- Physical measurement of sound

- Reverberation of echo

- Use of ray diagram and echo

- Common acoustical defects and remedies of conference hall

- Acoustic materials

- Explain methods of sound insulation of buildings

- Recommendations of different types of buildings for good acoustics

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Sound insulated building designed as per specifications

Tools and Equipment

- Drawing tools

- Drawing sheets

### Install ceiling insulation

Overview: This competency standard covers the skills and knowledge required to install ceiling insulation

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Identify legislative, regulatory, and organisational requirements. | - Identify and comply with safety and applicable legislative requirements - Identify and comply with organisational environmental and safety plans and policies according to organisational requirements. - Identify and apply safe work methods and practices according to organisational safety plans and policies. - Identify and apply environmental requirements are according to organisational environmental plans and regulatory requirements. - Identify and carry out emergency response and evacuation procedures when required. |
| CU2.Plan and prepare for installing insulation | - Obtain and confirm work instructions and relevant information for planning and preparation purposes. - Undertake risk assessment to identify existing risks and hazards in the work area, including electrical risks and hazards. - Document identified risks and undertake appropriate response according to safety requirements. - Confirm ceiling insulating material and insulation requirements in accordance with work specifications. - Identify, fit correctly and use appropriate personal protective equipment (PPE) and clothing according to organisational policies and procedures. - Select tools and equipment appropriate to the requirements of the work, confirm for serviceability, and report for repair or replacement where not serviceable. - Determine associated material and organise ready for use according to quality requirements and work plans and specifications |
| CU3.Install ceiling insulation | - Measure insulation material accurately to minimise waste. - Install insulation using approved processes and handling techniques according to manufacturer specifications and relevant electrical and building regulations - Use dust-suppression procedures to minimise health risk in work area to self and others. - Install insulation safely without damage or distortion of the surrounding environment, electrical and other services and in a manner that maximises safety of self and others. - Identify and report variations and difficulties affecting performance or quality requirements of own work. |
| CU4.Complete installation. | - Conduct final inspections to ensure installed ceiling insulation conforms to job and manufacturer specifications. - Make notification of work completion to designated personnel according to organisational procedures. - Clean work area and disposed of materials, reused or recycled according to organisational, safety and environmental requirements. - Clean, check, maintain and store tools and equipment according to manufacturer specifications and organisational procedures. - Document and report malfunctions, faults, wear or damage to tools, equipment and site accurately for repair or replacement according to organisational procedures |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- appropriate PPE and its use to reduce injury and electric shock

- ceiling insulation material types and quality requirements

- common health and safety risks associated with handling ceiling insulation

- common workplace safety hazards and risks, and procedures for reporting these to designated personnel

- emergency response and evacuation procedures

- environmental requirements, including waste management and recycling

- hierarchy of hazard control

- legislation, regulation and building codes related to ceiling insulation

- MSDS, JSA and safe work method statements

- methods for calculating insulation material quantities

- organisational requirements and procedures relating to ceiling insulation installation, including requirements for a systematic approach to planning own work

- procedures to safely use equipment, shift and handle products and materials, and work at heights and in enclosed areas

- product and process knowledge to identify problems and predict consequences

- relationships of 'R' rating with Building Code and standards' requirements and energy ratings

- tools and equipment prohibited for use near identified asbestos-containing materials (ACM)

- type and purpose of tags and logs of use for equipment

- types, characteristics, uses and limitations of installation tools and equipment

- reason for the operating temperature limit of electrical cables

- effect on cables partially surrounded by thermal insulation and fully surrounded by thermal insulation

- common wiring systems used in domestic premises indicating the age of the installation

- wiring not likely to be adversely affected by the retrospective installation of  thermal insulation

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Apply knowledge of industry products to identify:

- common faults and problems

- manufacturer specifications in relation to insulation

- properties of the available product

- manufacturer components and materials

Apply knowledge for the safe installation of thermal insulation in relation to electrical equipment, including:

- effects of thermal insulation on cables

- wiring likely to be adversely affected by the

- retrospective installation of thermal insulation

- clearance of thermal insulation from recessed own lights and ancillary equipment complete a risk assessment sheet for each installation which documents:

  - whether the wiring system is compatible with thermal insulation

  - number of recess luminaires in ceiling and how the clearances are to be met

  - electrical hazards, and measures taken to eliminate them

  - relevant work instructions

  - communicate and work effectively and safely with others

  - comply with organisational policies and procedures, including quality requirements

  - comply with site safety plan and OHS legislation, regulations and codes of practice applicable to workplace operations

  - follow work instructions, operating procedures and inspection practices to:

  - maintain workplace records in relation to materials, plant and equipment use

  - modify work activities to cater for variations in workplace procedures, personnel, contexts and environment prevent damage to the environment, equipment, products, or site select and use appropriate PPE

  - work effectively alone or with others and operate with minimal supervision

  - select and use non-conductive and insulated tools and materials to minimise electrical hazards

  - locate, interpret and apply relevant information, standards and specifications

  - select and install ceiling insulation on at least two occasions within agreed timeframes and standards using safe handling methods for materials and equipment.

Tools and Equipment

- broad knives

- brooms

- caulking guns

- electric screw guns

- hammers

- hand saws

- keyhole saws

- ladders

- manual levelling devices

- measuring tapes and rules

- nail bags

- power drills

- power leads

- power saws

- spanners

- spirit levels

- T squares

- taping knives

- tin snips

- trestles

- trowels

Non-conductive and insulated tools may include:

- air compressors and hoses

- C clamps

- docking saw and drop saws

- laser levelling devices

- masonry drills

- nail guns

- pop riveters

- saw stools

- Scaffolding and planks.

### Design an earthquake resistant building

Overview: This competency standard covers the skills and knowledge required to design of earthquake resistant building

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Identify principles of earthquake resistant buildings | P1. Obtain plans and specifications P2. Identify building configuration like lightness standards, symmetry, projections and size of building P3. Identify enclosed area P4. Design slopes, foundation and ductility |
| CU2.Identify seismic strengthening arrangements in building | P1. Obtain plans and specifications P2. Identify building material as per specifications P3. Identify horizontal reinforcement at lintel, plinth, gable and roof level P4. Identify vertical reinforcement as per specification P5. Prepare drawing |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- State Reid’s elastic theory, theory of plate tectonics, seismic waves, earthquake size local site effects, internal structure of earth, classification of earth quake tsunami

- Describe different seismic zones.

- Explain seismic design parameters.

- Explain seism resistant building architecture.

- State ductility consideration in earthquake resistant design of building.

- Explain construction of project in different seismic zones

- Describe methods and materials of construction.

- Describe principles to be observed while planning, designing and construction of earthquake resisting buildings.

- Describe precautions to be taken in planning, designing and construction of buildings in seismic zones.

Critical Evidence(s) Required

- The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

  - Drawing of seismic resistant building designed as per specifications

Tools and Equipment

- Drawing tools

- Drawing sheets

### Coordinate maintenance and repair of buildings

Overview: This competency standard covers the skills and knowledge required to coordinate maintenance and repair of buildings

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan repair or maintenance operation | - Review and confirm repair or maintenance requirements with relevant people according to organisational requirements. - Determine and arrange repair or maintenance resource requirements according to work order and budgetary requirements. - Identify site access and specific site requirements are identified and make appropriate arrangements according to client, organisational and legislative requirements. - Prepare work order and plan contingencies to ensure client and organisational requirements are met in an efficient and effective manner. - Recognise and satisfy client and tenant safety and security requirements in provision of services. - Establish communication channels and processes and agree with relevant people according to organisational requirements. |
| CU2.Coordinate repair or maintenance operation. | - Repair or maintenance operation is monitored against work schedules to ensure completion occurs within designated timeframes. - Factors affecting achievement of scheduled work are identified and required variations to schedules are negotiated and agreed with relevant people. - Established communication channels and processes are used to ensure accurate exchange of information throughout operation. - Situations requiring specialist advice are identified and assistance is sought as required according to organisational requirements. |
| CU3.Check and record completed operation. | - Receive notification of completed repair or maintenance and check against client and organisational requirements. - Arrange inspection to confirm repair or maintenance is fully operational and meets performance specifications and industry standards. - Verify faults, errors or omissions and determine and arrange prompt remedial action with relevant people according to organisational requirements. - Use business equipment and technology to maintain relevant documentation according to applicable OHS, legislative and organisational requirements. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- basic knowledge of property contracts and administrative requirements

- building codes and relevant standards

- building construction practice

- building control legislation

- common hazards to personal and public safety associated with particular types of maintenance work in buildings

- limitations of work role, responsibility and professional abilities

- occupancy rates and special conditions to be considered with particular services provided to clients and tenants

- OHS issues and requirements

- organisational and professional procedures, ethical practices and business standards

- points of contact with emergency service agencies

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- inspecting completed work against work order and ensuring remedial action for faults, errors or omissions

- monitoring maintenance and repair work ensuring effective communication exchange and identification of factors requiring variations to work schedules

- knowledge of organisation's practices, ethical standards and legislative requirements associated with planning and coordinating the repair, maintenance and modification of properties and facilities

- planning maintenance and repair work ensuring client and tenant safety and security, and resource requirements are according to budgetary parameters

- Responding appropriately to requests for maintenance and repair work and maintaining adequate records and reports.

### Assess construction faults in buildings

Overview: This competency standard covers the skills and knowledge required to assess construction faults in buildings

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Identify and analyse construction faults on building sites up to three storeys | - Collect information relating to the specific building construction problem. - Identify construction problem for the type of construction relative to original specifications and building material. - Communicate construction faults to appropriate personnel and document in accordance with standard work practices. - Use problem solving techniques, identify typical faults and problems and action to rectify is deemed to be in accordance with the Building codes. |
| CU2. Identify construction techniques, methods and materials nominated in relevant legislation in the Building codes and standard | - Use building terminology accurately in the communication of issues. - Examine working drawings and specifications and identify and evaluate existing or designed construction problems - Prepare alternative methods and materials to meet construction aims and objectives to the specification nominated in relevant legislation in the Building codes and standards. - Prepare detailed sketches of available alternative methods and materials available to meet the construction aims and objectives to specification. |
| CU3.Resolve construction faults in construction techniques and methods | - Evaluate project working drawings and specifications identifying existing or designed construction problems. - Prepare report identifying available alternative methods and materials to meet the construction aims and objectives to specification. - Prepare detailed sketches of available alternative methods and materials available to meet the construction aims and objectives to specification. |
| CU4. Resolve construction faults using alternative construction methods. | - Evaluate suitable methods from available alternative solutions and recommend to resolve the problem in accordance with project aims and objectives. - Integrate selected methods into the project in order to resolve the construction problems in accordance with project aims. - Carry out evaluation of available alternative forms of construction is carried out in accordance with project aims. |
| CU5.Resolve common on-site faults with building materials. | - Evaluate commonly occurring on-site problems with building materials and their causes. - Identify and implement corrective and preventative measures. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- access requirements for people with a disability and the requirements of the Disability Discrimination Act (DDA) with regard to access

- authorities and powers of a building surveyor

- design and construction principles of buildings

- nature of materials and effect on performance

- processes for the administration and preparation of documentation

- processes for the interpretation of reports, working drawings and specifications

- relevant federal, state or territory legislation and local government policy and procedures

- Research methods.

- Terminology, definitions and hazard identification.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- comply with OHS regulations applicable to workplace operations

- apply organisational management policies and procedures, including quality assurance requirements where appropriate

- assess construction faults in buildings, determine a rectification strategy and consider alternative construction methods; and the associated reporting of data, findings, recommendations and strategies for at least one residential building project and one commercial building project or equivalent in compliance with relevant legislation

- Provide reports to appropriate body/individual as determined by the project brief.

### Repair and rectify concrete

Overview: This competency standard covers the skills and knowledge required to repair and rectify concrete

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare | - Identify, confirm and apply work instructions, work health and safety (WHS) requirements and other information relevant to the work for planning and preparation purposes. - Select plant, tools and equipment consistent with job - Requirements check for serviceability, and rectify faults and report before work begins. - Identify and calculate material quantity requirements according to plans, specifications and quality requirements. - Obtain, prepare, handle safely and locate materials appropriate to the work application ready for use. - Identify environmental requirements for the project according to environmental plans and regulatory requirements. |
| CU2.Carry out minor repairs. | - Identify Types of surfaces that may require repair and categorise repair work as major repairs or minor repairs. - Repair colour variations by applying a concrete staining agent. - Repair effects of dusting by applying an appropriate surface hardener/dust inhibiting agent or removing the weak top layer. - Repair damaged or blistered concrete either by grinding or topping. - Apply repair mortars and self-levelling floor compounds according to manufacturer specifications. - Apply sealers and coloured paints to concrete surfaces according to manufacturer specifications. |
| CU3.Repair cracks and other major defects. | - Determine and rectify root cause of defect. - Prepare concrete and apply flexible epoxy resins to manufacturer specification. - Apply toppings to concrete using correct materials and techniques. - Apply acid etching and cleaning safely according to manufacturer requirements |
| CU4.Clean-up work site area | P1. Clear work are by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- types, location and use of relevant safety resources and information:

- job safety analyses (JSA) and safe work method statements (SWMS)

- safety data sheets (SDS)

- signage and barricades

- principles and processes for general concreting work:

- materials and techniques for repairing concrete

- preparation and placement of formwork and reinforcement

- use of control joints

- levelling techniques

- mortar mix composition and additives

- placing and finishing of concrete to ensure timely completion of job within quality requirements and specifications

- general construction terminology

- processes for material storage and environmentally friendly waste management

- processes for calculating material requirements

- quality requirements relevant to the repair and rectification of concrete

- types, characteristics, uses and limitations of concreting plant, tools and equipment:

- safe operating procedures

- operational, maintenance and basic diagnostic procedures

- workplace and equipment safety requirements

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- identify and apply the correct repair method and top coat to a minimum of one square metre of affected area for three minor repairs and one major repair listed in the range statement

- remove a section or whole defective area of concrete, cleaning and preparing the affected area and applying the necessary bonding

- locate, interpret and apply relevant information, standards and specifications relating to repairing and rectifying concrete

- comply with site safety plans and procedures

- comply with organisational policies and procedures relating to repairing and rectifying concrete while maintaining quality requirements

- safely and effectively operate and use plant, tools and equipment required to repair and rectify concrete

- Communicate and work effectively and safely with others at each stage of the process to repair concrete.

Tools and Equipment

- brooms

- concrete mixers

- floats

- grinders

- hoses

- polishers

- rollers

- screeds

- shovels

- trowels, including power trowels

- water blasters

- Wheel barrows.

### Repair wall and floor tiles

Overview: This competency standard covers the skills and knowledge required to repair wall and floor tiles

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Plan and prepare | - Identify, confirm and apply work instructions, work health and safety (WHS) requirements and other information relevant to the work for planning and preparation purposes. - Select plant, tools and equipment consistent with job - Requirements check for serviceability, and rectify faults and report before work begins. - Identify and calculate material quantity requirements according to plans, specifications and quality requirements. - Obtain, prepare, handle safely and locate materials appropriate to the work application ready for use. - Identify environmental requirements for the project according to environmental plans and regulatory requirements. |
| CU2.Repair damaged Tile work | - Remove damaged or defective tiles avoiding damage to surrounding tiles. - Clean old bedding and clear to allow placement of replacement tile. - Select replacement tiles and cut where applicable to match existing face, size, colour and pattern. - Fit tiles are fit and fix to maintain alignment with joints to uniform spacing. - Carry out grouting and clean tile face to specified finish. |
| CU4.Clean-up work site area | P1. Clear work are by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- general construction terminology

- job safety analysis (JSA) and safe work method statements

- material safety data sheets (MSDS) and hazards associated with solvents, adhesives and cement or epoxy-based grouts

- materials storage and environmentally friendly waste management

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- locate, interpret and apply relevant information, standards and specifications

- comply with site safety plan and OHS legislation, regulations and codes of practice applicable to workplace operations

- comply with organisational policies and procedures including quality requirements

- safely and effectively operate and use tools, plant and equipment

- communicate and work effectively and safely with others

- as a minimum remove and replace a quantity of damaged wall tiles, which are fixed with adhesive bedding; and remove and replace a quantity of damaged floor tiles, which are fixed with mortar bedding, ensuring:

  - correct identification of requirement and replacement of the tiles

  - correct selection and use of appropriate processes, tools and equipment

  - completing all work to specification

  - Compliance with regulations, standards and organisational quality procedures and processes.

Tools and Equipment

- Broom

- brushes

- buckets

- caulking guns

- cement sheet cutters

- hammers

- hose and water sprays

- ladders

- levelling equipment

- lump hammers

- measuring tapes and rules

- mortar boards

- nippers

- pointed grouters

- power drills

- power leads

- rags

- rubber mallets

- sanding blocks

- saws

- scrapers

- shovels

- spacers and wedges

- spatulas

- sponges

- squares

- squeegees

- straight edges

- string lines

- tile cutters and scribes

- trowels

- wet and dry diamond saws

- wheelbarrows

- wire brushes

- wooden floats may include:

- concrete mixers

- masonry drill bits

- power grinders

- power sander

### Apply building codes and standards to building projects

Overview: This competency standard covers the skills and knowledge required to apply building codes and standards to building projects

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Access and interpret relevant codes and standard requirements | - Identify relevant performance requirements from the Building codes that apply to individual projects (described as low rise). - Determine requirements of relevant Building codes deemed-to-satisfy (DTS) provisions. - Assess and interpret requirements of relevant standards referenced in the Building codes accordingly. |
| CU2.2. Classify buildings | - Determine nature of a building according to its use and arrangement. - Apply building codes criteria to determine the defined classification - Identify and interpret building codes requirements for multiple classification |
| CU3. Analyse and apply a range of solutions to a construction problem for compliance with the building codes. | - Determine range of criteria that will ensure that construction methods comply with building codes performance requirements. - Discuss and propose alternative solutions to a design or construction problem that will comply with building codes requirements in accordance with company policies and procedures. - Identify performance-based solutions and document in accordance with Building codes requirements. - Analyse and apply assessment methods referenced in the building codes to determine whether a building solution complies with performance requirements or DTS provision of the building codes - Identify and complete relevant documentation in accordance with building codes requirements. |
| CU4.Apply fire protection Requirements. | - Identify and apply passive and active fire control elements for low rise building required by the building codes and other legislation - Determine level of fire resistance required for the construction of various low rise buildings. - Carry out check of existing buildings for compliance with passive and active fire protection requirements in accordance with building codes requirements. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- basic design principles and the behaviour of structures under stress, strain, compression, bending or combined actions

- Building codes performance hierarchy

- definitions and common technical terms or usage specified under general provisions of building codes

- general nature of materials and the effects of performance

- relevant LDA and other development authorities standards

- relevant legislative and OHS requirements, codes and practices

- types of working drawings and specifications

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- comply with organisational quality procedures and processes

- apply and interpret relevant documentation and codes

- accurately apply building codes performance requirements relating to the design and construction of a building

- understand assessment methods available to determine compliance with the BCA

- Identify faults and problems and proposed action to rectify.

Tools and Equipment

Different building codes like LDA,  and other development authorities standards.

### Install electrical system

Overview: This competency standard covers the skills and knowledge required to Interpret Drawing, Arrange Resources for electrical wiring, Perform Wiring (Domestic & Industrial), termination and tagging, Perform Wire Dressing, and Perform Checking & Testing of electrical wiring.

| Competency Units | Performance Criteria |
| --- | --- |
| - Interpret Electrical layout | P1. Interpret layout of the job for installations P2. Read/interpret electrical drawing for electrical wirings P3. Follow drawing to connect components of equipment |
| - Arrange Resources (e.g. Power Supply, Tools & Equipment) for electrical equipment installation/dismantling | P1. Enlist and arrange tools and material as per job P2. Arrange work permit for the wiring task P3. Arrange backup resources for lighting, power and safety purposes as per job requirement |
| - Perform Electrical Panel Installation | P1. Extract information from layout regarding panel installations P2. Place the panel according to layout P3. Connect input and output of cables in relevant panels as per electrical layouts |
| - Perform Cable Installation | P1. Interpret layouts for cable route P2. Handle cable for laying with the help of stacker P3. Tag both ends of cables for cable tracing P4. Lay cables in cable trays according to route plan P5. Perform testing of cables e.g. insulation, continuity etc |
| - Perform Cable Dismantling | P1. Interpret layouts for cable route P2. Disconnect electric supply from the job P3. Handle cable for laying with the help of stacker P4. Tag both ends of cables for cable tracing P5. Remove cables in cable trays according to route plan |
| - Perform Electrical Appliances Installation | P1. Interpret vendor’s user manual for electrical installations P2. Coordinate with concerned departments before installations for feedback P3. Connect cables with machines as per operation manual P4.Verify the connections with respect to color coding/tagging/numbering |
| - Perform Electrical equipment Dismantling | P1. Extract information from layouts regarding electrical equipment dismantling P2. Disconnect input/output cables of relevant equipment P3. Remove equipment from location as per SOPs P4. Handle removed equipment as per manufacturers’ instructions P5. Place tags on dismantled items as per SOPs P6. Store the dismantled equipment at designated place |
| - Perform Earthing | P1. Perform earthing of cables according to calculated load P2. Perform earthing of electrical appliances according to calculated load P3. Perform earth resistance test as per standards |
| - Provide Power Supply to machine | P1. Interpret electrical drawings for power supply distribution P2. Perform power distribution according to drawings P3. Perform verification of electrical supply as per operations manual P4. Perform communication with all concerned persons for electrical appliance dry-run |

Knowledge & Understanding

The candidate must be able to develop skills and understanding o Interpret Drawing, Arrange Resources for electrical wiring, Perform Wiring (Domestic & Industrial), termination and tagging, Perform Wire Dressing, Perform Checking & Testing of electrical wiring required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define electrical symbols to be used in drawings

- Describe types of drawings (civil, electrical and communication) for the job

- Explain connection scheme of the job

- Explain all types of drawings (civil, electrical and communication)

- Explain the hierarchy of the organization to rectify the issues (if occurred)

- Describe the quality standards of original products

- Describe the quantity of products against purchase requisition

- Define electrical symbols to be used in drawings

- Describe types of drawings (civil, electrical) for machine

- Explain handling techniques for placement

- Describe all the types of drawings (civil and electrical)

- Describe the safe handling techniques of cable routing

- Explain the different methods of cable testing

- Describe different the types of drawings e.g. civil and electrical etc.

- Describe the safe handling techniques of cable routing

- Describe methods of installing the electrical appliances

- Describe basic knowledge of electrical appliances (A/C, water dispensers, microwave oven etc)

- Describe the different types of coding procedures (e.g. colour coding/tagging/numbering)

- Describe different the types of drawings e.g. civil and electrical etc.

- Enlist appropriate positions of tags for visual controls

- Explain earthing and testing procedures

- Describe working principle of earth tester

- Explain Describe different the types of drawings e.g. civil and electrical etc.

- Explain verification parameters according to standards

- Explain the importance of communication with relevant persons

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of PPE

- Use of tools

- Proper storage of tools and equipment

Tools and Equipment

Drawings, operations manuals

Layouts, ICT, measuring tools (calipers, cable chart, hand hacksaw, measuring tape, laser gun for length), quality standards, vendor certificate, grinding cutter, drill machines, hammer

Layouts, lifters, chain-pulley, jacks, sprit level

Layouts, measuring tools, cutting tools like wire cutter, side cutter, cable cutter, thimble presser, megger, stacker, tags, glands, shrouds, conduits, cable tray etc

Layouts, cutting tools e.g. wire cutter, side cutter, cable cutter, stacker, tags etc.

Drawing, layout, manual, fork lifter, drill machine, PPE

Layouts, lifters, chain-pulley, jacks, tags

Soldering, earth tester, thimble presser,

Drawings, multi meters,

### Construct circuits

Overview: This competency standard covers the skills and knowledge required to construct parallel circuit, series circuit and tunnel circuit.

| Competency Units | Performance Criteria |
| --- | --- |
| C1. Construct parallel circuit for two lamps | - Observe OHS requirements - Identify tools and equipment - Draw diagram of parallel circuits for two lamps. - Perform preliminary marking before termination of cables. - Follow drawing to perform wiring and for reporting/ record purpose. |
| C2. Construct series circuit for two lamps. | - Observe OHS requirements - Identify tools and equipment - Draw diagram of series circuits for two lamps. - Perform preliminary marking before termination of cables. - Follow drawing to perform wiring and for reporting/ record purpose. |
| C3. Construct tunnel circuit for multi storey building stair case. | - Observe OHS requirements - Identify tools and equipment - Draw diagram of tunnel circuit for multi storey staircase. - Perform preliminary marking before termination of cables. - Follow drawing to perform wiring and for reporting/ record purpose. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain electrical symbols

- Describe different types of drawings parallel circuits, series circuit and tunnel circuit.

- Describe how to prepare drawing if not available

- Describe types of breakers, contactors, relays etc.

- Describe standard tools used in wiring

- Explain different types of wiring techniques including: • Concealed • Conduit • Busway• Open • Duct

- Describe purpose of color coding and tagging during wiring

- Describe purpose of cable alignment

- Describe open, close circuits and testing procedures

- Describe Importance of controlled live test

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of PPE

- Use of tools

- Proper storage of tools and equipment

- Wiring of

Tools and Equipment

- Electrical drawing

- Computer

- Emergency light

- Ladder

- drill machine tool kit

- Thimble puncher

- Shroud

- PVC tape

- cable knife

- wire stripper

- drill machine

- PPE

- Cable tie

- Clamps

- Plier

- Cutter

- spiral/ flexible pipes

- tie base

- ravel bolt

- ravel plug

- hilty bolt

- wooden screw

- AVO meter

- test lamp

- earth tester

- line tester

- megger

    - Manipulate Images (Photoshop)

Overview: This unit describes the skills and knowledge required to compile and manipulate images. It applies to individuals involved in software, image editing and who are required to manipulate images, using Illustrator.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Performs basic image edits using Photoshop | 1.1 Able to describe differences between raster and vector imaging and uses for both 1.2 Edit Photos non-destructively 1.3 Able to comfortably navigate Photoshop 1.4 Crop picture effectively 1.5 Straighten the picture 1.6 Use the layering process in Photoshop |
| 2. Use masks and combine images | 2.1 Make and edit masks 2.2 Make selection 2.3 Make adjustment layers with masks 2.4 Use masks and layers to combine photos |
| 3. Merge and heal photos | 3.1 Use Photo merge 3.2 Fill selections with content aware fill 3.3 Use healing tools correctly 3.4 Apply Filters to photos |
| 4. Use non-photo elements in Photoshop designs | 4.1 Use text tool 4.2 Use brushes 4.3 Create shapes 4.4 Apply colors and gradients 4.5 Apply Layer Effects 4.6 Combine elements in pleasing ways |

Knowledge & Understanding

K1: State the names of the basic tools

K2: Explain the organization of layer method

K3: Elaborate the effects of masking, brush, cropping, and adjusting layers

K4: Describe the short keys for tools

K5: Explain the process of opening and saving image

K6: Describe the ways to add effects on an image

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to use Photoshop that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- get familiarize with Photoshop interface

- master all the tools in depth

- blend Images seamlessly

- decorate an Image

- create reflection effect

- create awesome text effect

- create rain effect , cloud effect

- create pixel perfect designs

- create blur backgrounds , polygon backgrounds

- Create whirlpool Effect, Bokeh Effect

    - Design simple web page layouts

Overview: This unit is about writing and editing copy (i.e. final version of text) for a range of formats in different media

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Identify main layout sections from design specification | 1.1 Identify the required sections of the web page 1.2 Create the web page structure |
| 2. Layout the web page to match design specification | 2.1 Position document elements 2.2 Style web page elements to match the design specifications |
| 3. Test and validate the layout | 3.1 Test the website in various browsers 3.2 Validate the web pages against industry standards |

Knowledge & Understanding

K1: Summaries standard web and web scripting design principles

K2: define and explain the application of the following for website development:

- cascading style sheets (CSS)

- the hypertext transfer protocol (HTTP)

- a markup language, such as hypertext markup language (HTML), or extensible hypertext markup language (XHTML)

- World Wide Web Consortium (W3C) standards.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to Design simple web page layouts that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- develop a website according to design specifications

- lay out page elements

- test the web pages in a variety of browsers

- Validate the web pages, against industry standards.

    - Apply introductory programming techniques

Overview: This unit describes the performance outcomes, skills and knowledge required to undertake introductory programming tasks in the development of a game or application. This unit addresses the knowledge, processes and techniques necessary to develop skills to create simple applications or games.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Apply language syntax and layout | 1.1 Apply basic language syntax rules 1.2 Use language data types, operators and expressions to create clear and concise code 1.3 Apply variables and variable scope 1.4 Use library functions in a program 1.5 Use commenting to create clear meaning to code |
| 2. Apply control structures | 2.1 Apply language syntax for sequence, selection and iteration constructs 2.2 Use logical operators to create expressions for use in selection and iteration constructs |
| 3. Code using standard algorithms | 3.1 Develop algorithms that use the sequence, selection and iteration constructs 3.2 Create and use arrays 3.3 Code standard sequential access algorithms for reading and writing text files, including end-of-file detection loops 3.4 Apply string manipulation |
| 4. Test code | 4.1 Use debugging techniques to trace code execution and examine variable contents to detect and correct errors 4.2 Create and conduct simple tests to confirm code meets design specification 4.3 Document the tests performed and results achieved |
| 5. Create an application or game | 5.1 Design an algorithm in response to basic program specifications 5.2 Develop the application or game to meet the program specification 5.3 Test and confirm that application or game meets the initial specifications |

Knowledge & Understanding

K1: Programming language syntax and command structure

K3: Small-sized application or game development.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to undertake introductory programming tasks in the development of a game or application. This unit addresses the knowledge, processes and techniques necessary to develop skills to create simple applications or games. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Apply programming language syntax, sequence, selection and iteration control structures to the development of an application or game

- Produce an application or game that is designed and built from a provided program specification

- Confirm that the created application or game meets the original program specifications and obtain user sign-off for completed program.