Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Building Materials & Construction Techniques |
| Competency Standards | Construct masonry arches(semi-circular and segmental) |
| Assessment Task | 1. Plan for work 2- Set out first course for masonry arch. 3- Prepare for arch construction. 4- Construct arch centre. 5- Set up arch centre. 6- Cut and lay brick/ block to arch. 7- Finish Joints. 8- Clean up |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Confirm Work instructions with supervisor. |  |  |
| - Select, check andTools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. |  |  |
| - Calculate Material quantity requirements in accordance with specifications. |  |  |
| - Materials needed are obtained, checked for compliance and prepared. |  |  |
| - Set out Location and line of brickwork/blockwork wall are set out on concrete footing/slab to job drawings. |  |  |
| - Determine Pan of arch is determined from prepared allotted arch centre plus 4mm. |  |  |
| - Set out Arch spans to location for first course.. |  |  |
| - Mix Mortar to specifications. |  |  |
| - Lay Bricks/blocks for first course to planned set out for line and specified bond according to specifications. |  |  |
| - Maintain Gauge within standard tolerance at every course level. |  |  |
| - Maintain Vertical face alignment |  |  |
| - Lay Abutment jambs/piers vertical up to springing line. |  |  |
| - Lay Bricks/blocks in stretcher bond to springing line of arch with perpendicular joints maintained in vertical line. |  |  |
| - Set out Arch centre and curve is drawn up in accordance with specifications and plan. |  |  |
| - Transfer Plan to material and cut to shape.Brush down |  |  |
| - Determine Height to springing line and confirm height to crown of archto be within standard tolerance. |  |  |
| - Determine Height of toms and wedges or adjustable metal props to set up and support timber arch centre. |  |  |
| - Adjust Supports to ensure arch centre is level at right angles to wall face and level across springing line. |  |  |
| - Locate Props, toms, packers and wedges for easy removal. |  |  |
| - Position of central key brick/block is established for gauged arch and tape used to mark gauge. |  |  |
| - Cut and lay Bricks and blocks on centre to form arch to specifications |  |  |
| - Maintain Joints to equal size and parallel on the extrados of an arch. |  |  |
| - Maintain Same size wedge shape is maintained on face. |  |  |
| - M,aintain Centreline of key brick/block wedge is maintained through vertical centre line of arch. |  |  |
| - Maintain Even joint thickness around extrados for cut brickwork and blockwork. |  |  |
| - Cut All bricks and laid to maintain even joints. |  |  |
| - Struck All joints evenly to depth and shape to specifications.. |  |  |
| - Cut Sill brickswhere required and laid to line in accordance with work specifications. |  |  |
| - Brush down Brickwork/blockwork prior to drying to remove unwanted mortar and face is cleaned. |  |  |
| - Remove Excess mortar from brick/blockwork surfaces and cavities are cleaned free of mortar and debris. |  |  |
| - Clean Work area and dispose of, reuse or recycle waste in accordance with work specifications. |  |  |
| - Clean Tools plant and equipment, check for faults and maintained and store in accordance with workplace practices |  |  |

Candidate’s Signature___________________Assessor’s Signature_____________________

Date: ________________________________

Instruction Sheet for the Candidate

| Qualification | Building Materials & Construction Techniques |
| --- | --- |
| Competency Standard(s) | Construct masonry arches(semi-circular and segmental) |
| Purpose of Assessment | Formative assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): 1. Plan for work 2- Set out first course for masonry arch. 3- Prepare for arch construction. 4- Construct arch centre. 5- Set up arch centre. 6- Cut and lay brick/ block to arch. 7- Finish Joints. 8- Clean up |
| Time: 3 hours | During a practical assessment, under observation by an assessor, you are required to - Confirm Work instructions with supervisor. - Select, check Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Calculate Material quantity requirements in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. - Set out Location and line of brickwork/blockwork wall are set out on concrete footing/slab to job drawings. - Determine Pan of arch is determined from prepared allotted arch centre plus 4mm. - Set out Arch spans to location for first course.. - Mix Mortar to specifications. - Lay Bricks/blocks for first course to planned set out for line and specified bond according to specifications. - Maintain Gauge within standard tolerance at every course level. - Maintain Vertical face alignment - Lay Abutment jambs/piers vertical up to springing line. - Lay Bricks/blocks in stretcher bond to springing line of arch with perpendicular joints maintained in vertical line. - Set out Arch centre and curve is drawn up in accordance with specifications and plan. - Transfer Plan to material and cut to shape. - Determine Height to springing line and confirm height to crown of archto be within standard tolerance. - Determine Height of toms and wedges or adjustable metal props to set up and support timber arch centre. - Adjust Supports to ensure arch centre is level at right angles to wall face and level across springing line. - Locate Props, toms, packers and wedges for easy removal. - Position of central key brick/block is established for gauged arch and tape used to mark gauge. - Cut and lay Bricks and blocks on centre to form arch to specifications - Maintain Joints to equal size and parallel on the extrados of an arch. - Maintain Same size wedge shape is maintained on face. - M,aintain Centreline of key brick/block wedge is maintained through vertical centre line of arch. - Maintain Even joint thickness around extrados for cut brickwork and blockwork. - Cut All bricks and laid to maintain even joints. - Struck All joints evenly to depth and shape to specifications. - Remove unwanted mortar Brickwork/blockwork prior to drying to remove unwanted mortar and face is cleaned. - Remove Excess mortar from brick/blockwork surfaces and cavities are cleaned free of mortar and debris. - Clean Work area and disposed of, reused or recycled waste in accordance with work specifications. - Clean Tools plant and equipment ,check for faults and maintained and store in accordance with workplace practices |
| Minimum Evidence Required |  |

Assessors Judgment Guide

| Qualification | Building Materials & Construction Techniques |
| --- | --- |
| Competency Standard(s) | Construct masonry arches(semi-circular and segmental) |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Plan for work 2- Set out first course for masonry arch. 3- Prepare for arch construction. 4- Construct arch centre. 5- Set up arch centre. 6- Cut and lay brick/ block to arch. 7- Finish Joints. 8- Clean up |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
| 1 | Confirm Work instructions with supervisor. |  |  |  |  |  |
| 2 | Select, check andTools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. |  |  |  |  |  |
| 3 | Calculate Material quantity requirements in accordance with specifications. |  |  |  |  |  |
| 4 | Materials needed are obtained, checked for compliance and prepared. |  |  |  |  |  |
| 5 | Set out Location and line of brickwork/blockwork wall are set out on concrete footing/slab to job drawings. |  |  |  |  |  |
| 6 | Determine Pan of arch is determined from prepared allotted arch centre plus 4mm. |  |  |  |  |  |
| 7 | Set out Arch spans to location for first course.. |  |  |  |  |  |
| 8 | Mix Mortar to specifications. |  |  |  |  |  |
| 9 | Lay Bricks/blocks for first course to planned set out for line and specified bond according to specifications. |  |  |  |  |  |
| 10 | Maintain Gauge within standard tolerance at every course level. |  |  |  |  |  |
| 11 | Maintain Vertical face alignment |  |  |  |  |  |
| 12 | Lay Abutment jambs/piers vertical up to springing line. |  |  |  |  |  |
| 13 | Lay Bricks/blocks in stretcher bond to springing line of arch with perpendicular joints maintained in vertical line. |  |  |  |  |  |
| 14 | Set out Arch centre and curve is drawn up in accordance with specifications and plan. |  |  |  |  |  |
| 15 | Transfer Plan to material and cut to shape. |  |  |  |  |  |
| 16 | Determine Height to springing line and confirm height to crown of arch to be within standard tolerance. |  |  |  |  |  |
| 17 | Determine Height of toms and wedges or adjustable metal props to set up and support timber arch centre. |  |  |  |  |  |
| 18 | Adjust Supports to ensure arch centre is level at right angles to wall face and level across springing line. |  |  |  |  |  |
| 19 | Locate Props, toms, packers and wedges for easy removal. |  |  |  |  |  |
| 20 | Position of central key brick/block is established for gauged arch and tape used to mark gauge. |  |  |  |  |  |
| 21 | Cut and lay Bricks and blocks on centre to form arch to specifications |  |  |  |  |  |
| 22 | Maintain Joints to equal size and parallel on the extrados of an arch. |  |  |  |  |  |
| 23 | Maintain Same size wedge shape is maintained on face. |  |  |  |  |  |
| 24 | M,aintain Centreline of key brick/block wedge is maintained through vertical centre line of arch. |  |  |  |  |  |
| 25 | Maintain Even joint thickness around extrados for cut brickwork and blockwork. |  |  |  |  |  |
| 26 | Cut All bricks and laid to maintain even joints. |  |  |  |  |  |
| 27 | Struck All joints evenly to depth and shape to specifications.. |  |  |  |  |  |
| 28 | Cut Sill brickswhere required and laid to line in accordance with work specifications. |  |  |  |  |  |
| 29 | Brush down Brickwork/blockwork prior to drying to remove unwanted mortar and face is cleaned. |  |  |  |  |  |
| 30 | Remove Excess mortar from brick/blockwork surfaces and cavities are cleaned free of mortar and debris. |  |  |  |  |  |
| 31 | Clean Work area and dispose of, reuse or recycle waste in accordance with work specifications. |  |  |  |  |  |
| 32 | Clean Tools plant and equipment, check for faults and maintained and store in accordance with workplace practices |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Building Materials & Construction Techniques |
| --- | --- |
| Competency Standard(s) | Construct masonry arches(semi-circular and segmental) |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
| 1 | Define arch. |  |  |
| 2 | What is the purpose of arch |  |  |
| 3 | What is the position of key stone. |  |  |
| 4 | Name the types of arches w.r.t construction. |  |  |
| 5 | What is the intrados. |  |  |
| 6 | What is the a span. |  |  |
| 7 | What is the minimum bearing of lintel on door? |  |  |
| 8 | The temporary structure to support arches is called? |  |  |
| 9 | The wedge shaped blocks are called? |  |  |
| 10 | What is a springing line. |  |  |
| 11 | Define abutment of an arch. |  |  |
| 12 | What traffic signage used during construction activity. |  |  |
| 13 | How to minimize work hazards. |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________

Integrated Approach of Assessment

Following are the competencies in the qualification of welding.  You are required to design the assessment task(s) by following integrated approach and clustering of competencies:

- Take measurements

- Cut the material as per specifications

- Perform Welding of the material

- Perform Finishing of the object

- Perform fabrication/designing on object

TASK(S):

_________________________________________________________________________________