Instruction Sheet for the Candidate

| Qualification | 0732B&C02 National Vocational Certificate Level-4 in DAE Civil (Quantity Surveying) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – Quantity Surveying Level 4 |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Detailed Estimate for culvert and road. - Materials Statement and Rate Analysis. - Rent of Building - Rough cost estimate for water supply and sewerage pipe lines. - Knowledge Assessment |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to: - Produce Detailed estimate of Arterial Road having RCC Slab culvert 5’ span Annexure A. Land width is 110 ft; formation width is 32 ft with average height of embankment = 3 ft with side slope 1:3 (V:H). Carriage way 20 ft wide having 1-inch TST, 6-inch base, 8-inch sub-base and length of road =5280 ft. - Produce Materials statement for RCC work and brick work in C/S 1:3 for above culvert. Also, analyse rate for brick work. - A building has 1145 sft area with plinth area rate of Rs. 1750/sft. Area of plot is 8 marla with a rate of Rs. 50000 per marla. Calculate rent for private building. - Produce Rough cost estimate for parts of water supply 6 in dia pipe line 540 ft long, 4in dia pipe line 790 ft long. - Produce Rough cost estimate for parts of sewerage 12 in dia pipe line 760 ft and 9 in dia pipe line 970 ft long. demonstrating the following criteria: |
| Minimum Evidence Required | - Calculate the quantities of Jungle clearance, clearing & Grubbing and Uprooting of trees as per requirement of site. - Calculate area of site for surveying and levelling. - Calculate area of permanent land. - Calculate the volume of earth work. - Calculate area of temporary land. - Calculate volume of sub-base and base course of road. - Calculate area of surfacing (TST or carpet). - Measure items of boundary pillar, mile post, furlong posts, sign boards. - Calculate the quantities of all items of work for substructure for single span RCC slab culvert. - Calculate the quantities of all items of work for superstructure for single span RCC slab culvert. - Calculate the quantities of all items of work for road structure for single span RCC slab culvert. - Incorporate quantities of all items of work and relevant rates Abstract of Quantities table for single span RCC slab culvert. - Calculate the amount of each item of work and Sum up the amounts for single span RCC slab culvert. - Add premium/ contractor’s profit on amounts of relevant rates for single span RCC slab culvert. - Identify materials required for brick work - Evaluate the proportion required for mortar - Calculate the number of bricks - Calculate proportional quantity of cement and sand - Incorporate quantities in schedule of materials table. - Identify materials required for RCC work. - Convert wet volume of concrete to dry volume. - Calculate proportional quantity of cement, sand, bajri and wood for shuttering. - Calculate the quantity of steel bars & binding wire by weight per unit volume. - Identify unit of rate, materials, and their ratio for brick work in foundations and plinth. - Calculate materials and their cost by input rates (MRS) for brick work in foundations and plinth. - Calculate the labour charges for the brick work in foundations & plinth. - Calculate the carriage charges for sand, cement & bricks. - Add sundries lump sum charges (tools plant, overheads, taxes etc.). - Sum up the amounts and add contractor’s profit. - Add water charges. - Calculate value of building. - Calculate the annual rent on construction cost and for cost of land. - Calculate the monthly rent. - Take out lengths of each diameter water supply pipe. - Prepare analysis of rate for water supply pipe line. - Calculate amount of water supply pipe line. - Take out lengths of each dia sewer line pipe. - Prepare analysis of rate for sewer line. - Calculate amount of sewer line. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | 0732B&C02 National Vocational Certificate Level-4 in DAE Civil (Quantity Surveying) |
| Purpose of Assessment | Summative Assessment – Quantity Surveying Level 3 |
| Assessment Task | - Produce Detailed estimate of Arterial Road having RCC Slab culvert 5’ span Annexure A. Land width is 110 ft; formation width is 32 ft with average height of embankment = 3 ft with side slope 1:3 (V:H). Carriage way 20 ft wide having 1-inch TST, 6-inch base, 8-inch sub-base and length of road =5280 ft. - Produce Materials statement for RCC work and brick work in C/S 1:3 for above culvert. Also, analyse rate for brick work. - A building has 1145 sft area with plinth area rate of Rs. 1750/sft. Area of plot is 8 marla with a rate of Rs. 50000 per marla. Calculate rent for private building. - Produce Rough cost estimate for parts of water supply 6 in dia pipe line 540 ft long, 4in dia pipe line 790 ft long. - Produce Rough cost estimate for parts of sewerage 12 in dia pipe line 760 ft and 9 in dia pipe line 970 ft long. demonstrating the following criteria: |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Calculate the quantities of Jungle clearance, clearing & Grubbing and Uprooting of trees as per requirement of site. |  |  |
| - Calculate area of site for surveying and levelling. |  |  |
| - Calculate area of permanent land. |  |  |
| - Calculate the volume of earth work. |  |  |
| - Calculate area of temporary land. |  |  |
| - Calculate volume of sub-base and base course of road. |  |  |
| - Calculate area of surfacing (TST or carpet). |  |  |
| - Measure items of boundary pillar, mile post, furlong posts, sign boards. |  |  |
| - Calculate the quantities of all items of work for substructure for single span RCC slab culvert. |  |  |
| - Calculate the quantities of all items of work for superstructure for single span RCC slab culvert. |  |  |
| - Calculate the quantities of all items of work for road structure for single span RCC slab culvert. |  |  |
| - Incorporate quantities of all items of work and relevant rates Abstract of Quantities table for single span RCC slab culvert. |  |  |
| - Calculate the amount of each item of work and Sum up the amounts for single span RCC slab culvert. |  |  |
| - Add premium/ contractor’s profit on amounts of relevant rates for single span RCC slab culvert. |  |  |
| - Identify materials required for brick work |  |  |
| - Evaluate the proportion required for mortar |  |  |
| - Calculate the number of bricks |  |  |
| - Calculate proportional quantity of cement and sand |  |  |
| - Incorporate quantities in schedule of materials table. |  |  |
| - Identify materials required for RCC work. |  |  |
| - Convert wet volume of concrete to dry volume. |  |  |
| - Calculate proportional quantity of cement, sand, bajri and wood for shuttering. |  |  |
| - Calculate the quantity of steel bars & binding wire by weight per unit volume. |  |  |
| - Identify unit of rate, materials, and their ratio for brick work in foundations and plinth. |  |  |
| - Calculate materials and their cost by input rates (MRS) for brick work in foundations and plinth. |  |  |
| - Calculate the labour charges for the brick work in foundations & plinth. |  |  |
| - Calculate the carriage charges for sand, cement & bricks. |  |  |
| - Add sundries lump sum charges (tools plant, overheads, taxes etc.). |  |  |
| - Sum up the amounts and add contractor’s profit. |  |  |
| - Add water charges. |  |  |
| - Calculate value of building. |  |  |
| - Calculate the annual rent on construction cost and for cost of land. |  |  |
| - Calculate the monthly rent. |  |  |
| - Take out lengths of each diameter water supply pipe. |  |  |
| - Prepare analysis of rate for water supply pipe line. |  |  |
| - Calculate amount of water supply pipe line. |  |  |
| - Take out lengths of each dia sewer line pipe. |  |  |
| - Prepare analysis of rate for sewer line. |  |  |
| - Calculate amount of sewer line. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

ANNUXURE-A

Assessors Judgment Guide

| Qualification | 0732B&C02 National Vocational Certificate Level-4 in DAE Civil (Quantity Surveying) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – Quantity Surveying Level 4 |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________ Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Produce Detailed estimate of Arterial Road having RCC Slab culvert 5’ span Annexure A. Land width is 110 ft; formation width is 32 ft with average height of embankment = 3 ft with side slope 1:3 (V:H). Carriage way 20 ft wide having 1-inch TST, 6-inch base, 8-inch sub-base and length of road =5280 ft. - Produce Materials statement for RCC work and brick work in C/S 1:3 for above culvert. Also, analyse rate for brick work. - A building has 1145 sft area with plinth area rate of Rs. 1750/sft. Area of plot is 8 marla with a rate of Rs. 50000 per marla. Calculate rent for private building. - Produce Rough cost estimate for parts of water supply 6 in dia pipe line 540 ft long, 4in dia pipe line 790 ft long. - Produce Rough cost estimate for parts of sewerage 12 in dia pipe line 760 ft and 9 in dia pipe line 970 ft long. demonstrating the following criteria: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Calculated the quantities of Jungle clearance, clearing & Grubbing and Uprooting of trees as per requirement of site. |  |  |  |  |  |
|  | Calculated area of site for surveying and levelling. |  |  |  |  |  |
|  | Calculated area of permanent land. |  |  |  |  |  |
|  | Calculated the volume of earth work. |  |  |  |  |  |
|  | Calculated area of temporary land. |  |  |  |  |  |
|  | Calculated volume of sub-base and base course of road. |  |  |  |  |  |
|  | Calculated area of surfacing (TST or carpet). |  |  |  |  |  |
|  | Measured items of boundary pillar, mile post, furlong posts, sign boards. |  |  |  |  |  |
|  | Calculated the quantities of all items of work for substructure for single span RCC slab culvert. |  |  |  |  |  |
|  | Calculated the quantities of all items of work for superstructure for single span RCC slab culvert. |  |  |  |  |  |
|  | Calculated the quantities of all items of work for road structure for single span RCC slab culvert. |  |  |  |  |  |
|  | Incorporated quantities of all items of work and relevant rates Abstract of Quantities table for single span RCC slab culvert. |  |  |  |  |  |
|  | Calculated the amount of each item of work and Summed up the amounts for single span RCC slab culvert. |  |  |  |  |  |
|  | Added premium/ contractor’s profit on amounts of relevant rates for single span RCC slab culvert. |  |  |  |  |  |
|  | Identified materials required for brick work |  |  |  |  |  |
|  | Evaluated the proportion required for mortar |  |  |  |  |  |
|  | Calculate the number of bricks |  |  |  |  |  |
|  | Calculated proportional quantity of cement and sand |  |  |  |  |  |
|  | Incorporated quantities in schedule of materials table. |  |  |  |  |  |
|  | Identified materials required for RCC work. |  |  |  |  |  |
|  | Converted wet volume of concrete to dry volume. |  |  |  |  |  |
|  | Calculated proportional quantity of cement, sand, bajri and wood for shuttering. |  |  |  |  |  |
|  | Calculated the quantity of steel bars & binding wire by weight per unit volume. |  |  |  |  |  |
|  | Identified unit of rate, materials, and their ratio for brick work in foundations and plinth. |  |  |  |  |  |
|  | Calculated materials and their cost by input rates (MRS) for brick work in foundations and plinth. |  |  |  |  |  |
|  | Calculated the labour charges for the brick work. |  |  |  |  |  |
|  | Calculated the carriage charges for sand, cement & bricks. |  |  |  |  |  |
|  | Added sundries lump sum charges (tools plant, overheads, taxes etc.). |  |  |  |  |  |
|  | Summed up the amounts and add contractor’s profit. |  |  |  |  |  |
|  | Added water charges. |  |  |  |  |  |
|  | Calculated value of building. |  |  |  |  |  |
|  | Calculated the annual rent on construction cost and for cost of land. |  |  |  |  |  |
|  | Calculated the monthly rent. |  |  |  |  |  |
|  | Took out lengths of each diameter water supply pipe. |  |  |  |  |  |
|  | Prepared analysis of rate for water supply pipe line. |  |  |  |  |  |
|  | Calculated amount of water supply pipe line. |  |  |  |  |  |
|  | Took out lengths of each dia sewer line pipe. |  |  |  |  |  |
|  | Prepared analysis of rate for sewer line. |  |  |  |  |  |
|  | Calculated amount of sewer line. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | 0732B&C02 National Vocational Certificate Level-4 in DAE Civil (Quantity Surveying) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – Quantity Surveying Level 4 |
| Candidate Details | Name: _____________________________________________________________ Registration Number: _________________ Candidate Signature: _____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor: ______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What are mandatory sign boards? |  |  |
|  | What is a Crete way? |  |  |
|  | State how materials are calculated for cement pointing? |  |  |
|  | State the materials required for 100 sft DPC 1.5” thick of CC (1:2:4) with 2 coats of bitumen? |  |  |
|  | What is the purpose of rate analysis? |  |  |
|  | What is meant by out-turn of labour? |  |  |
|  | What is the value of velocity for flow in pipeline? |  |  |
|  | What is meant by peak discharge & flow? |  |  |
|  | Define the term year’s purchase? |  |  |
|  | State the formula to calculate the radius of segmental arch? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________