Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level-3 in Civil Technology (Concrete Technology) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – Concrete Technology |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |
| Guidance for Candidate | To meet this standard you are required to complete the following within 05 Hrs. time frame (for practical demonstration & assessment): - Prepare the cement paste of standard consistence and find out the initial, final setting time along with compressive and tensile strength of cement. - Draw the Gradation curve as par the particle size distribution of aggregate. - Arrange the apparatus to check the workability of concrete. - Knowledge Assessment |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to “Check the key properties of basic ingredients of fresh concrete with its limitation to use at site “ demonstrating the following criteria: |
| Minimum Evidence Required | - Take standard weigh of cement sample - Check fineness of cement - Prepare cement paste of standard consistence - Find out Initial setting time - Find out final setting time - Find soundness of cement - Test the prepared samples of 3 days,7 days and 14 days to check the Compressive and tensile strength of cement. - Plotted the graph for compressive strength and tensile strength - Find out particle size distribution of fine and coarse aggregates with sieve analysis - Find out flakiness index of coarse aggregates - Find out elongation index of coarse aggregates - Find out of specific gravity of aggregates - Mixed the material properly to check workability of concrete - Take water cement ratio of 0.4, 0.5, 0.55 and 0.6 to perform slump test. - Removed the cone and measured the slump. - Filled the cylinder in specified layers in compacting factor apparatus. - Calculated the compacting factor by using formula. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level-3 in Civil Technology (Concrete Technology) |
| Purpose of Assessment | Summative Assessment – Concrete Technology |
| Assessment Task | - Prepare the cement paste of standard consistence and find out the initial, final setting time along with compressive and tensile strength of cement. - Draw the Gradation curve as par the particle size distribution of aggregate. - Arrange the apparatus to check the workability of concrete. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Take standard weigh of cement sample |  |  |
| - Check fineness of cement |  |  |
| - Prepare cement paste of standard consistence |  |  |
| - Find out Initial setting time |  |  |
| - Find out final setting time |  |  |
| - Find soundness of cement |  |  |
| - Test the prepared samples of 3 days,7days and 14 days to check the Compressive and tensile strength of cement. |  |  |
| - Plotted the graph for compressive strength and tensile strength |  |  |
| - Find out particle size distribution of fine and coarse aggregates with sieve analysis |  |  |
| - Find out flakiness index of coarse aggregates |  |  |
| - Find out elongation index of coarse aggregates |  |  |
| - Find out of specific gravity of aggregates |  |  |
| - Mixed the material properly to check workability of concrete |  |  |
| - Take water cement ratio of 0.4, 0.5, 0.55 and 0.6 to perform slump test. |  |  |
| - Removed the cone and measured the slump. |  |  |
| - Filled the cylinder in specified layers in compacting factor apparatus. |  |  |
| - Calculated the compacting factor by using formula. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level-3 in Civil Technology (Concrete Technology) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – Concrete Technology |
| Candidate Details | Name:______________________________________________________________ Registration Number: ________________________Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:___________________Signature:________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare the cement paste of standard consistence and find out the initial, final setting time along with compressive and tensile strength of cement. - Draw the Gradation curve as par the particle size distribution of aggregate. - Arrange the apparatus to check the workability of concrete. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Take standard weigh of cement sample |  |  |  |  |  |
|  | Check fineness of cement |  |  |  |  |  |
|  | Prepare cement paste of standard consistence |  |  |  |  |  |
|  | Find out Initial setting time |  |  |  |  |  |
|  | Find out final setting time |  |  |  |  |  |
|  | Find soundness of cement |  |  |  |  |  |
|  | Test the prepared samples of 3 days,7days and 14 days to check the Compressive and tensile strength of cement. |  |  |  |  |  |
|  | Plotted the graph for compressive strength and tensile strength |  |  |  |  |  |
|  | Find out particle size distribution of fine and coarse aggregates with sieve analysis |  |  |  |  |  |
|  | Find out flakiness index of coarse aggregates |  |  |  |  |  |
|  | Find out elongation index of coarse aggregates |  |  |  |  |  |
|  | Find out of specific gravity of aggregates |  |  |  |  |  |
|  | Mixed the material properly to check workability of concrete |  |  |  |  |  |
|  | Take water cement ratio of 0.4, 0.5, 0.55 and 0.6 to perform slump test. |  |  |  |  |  |
|  | Removed the cone and measured the slump. |  |  |  |  |  |
|  | Filled the cylinder in specified layers in compacting factor apparatus. |  |  |  |  |  |
|  | Calculate the compacting factor by using formula. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level-3 in Civil Technology (Concrete Technology) |
| --- | --- |
| Purpose of Assessment | Summative Assessment – Concrete Technology |
| Candidate Details | Name:______________________________________________________________ Registration Number: __________________ Candidate Signature:_____________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________Signature of the Assessor:_______________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the use of Vicat Apparatus. |  |  |
|  | What is the diameter and length of Plunger in vicat apparatus. |  |  |
|  | Standard consistency is achieved when distance from the end of the plunger from the bottom of the container is ---- mm? |  |  |
|  | # The minimum initial setting time in field should not be less than how much minutes. |  |  |
|  | What is the importance of gradation of aggregate. |  |  |
|  | Which type of aggregate should use in construction, angular or rounded. |  |  |
|  | How the workability of concrete can be increased without using more and more water. |  |  |
|  | What is the standard range of slump value allowable to use at site. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________Assessor’s Signature ___________________