National Competency Standards Level-2 for “Civil Technology”

Competency Standards

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

ACKNOWLEDGEMENTS

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to many representatives of business, industry, academia, government agencies, Provincial TEVTAs, Sector Skill Councils and trade associations who speared their time and expertise to the development and validation of these National Vocational Qualifications (Competency Standards, Curricula, Assessments Packs and related material). This work would not have been possible without the financial and technical support of the TVET Sector Support Programme co-funded by European Union, Norwegian and German Governments implemented by GIZ Pakistan. NAVTTC is especially indebted to Dr. Muqeem ul Islam, who lead the project from the front. The core team was comprised on:

- Dr. Muqeem ul Islam, Director General (Skills, Standards and Curricula) NAVTTC

- Mr. Muhammad Naeem Akhtar, Senior Technical Advisor TSSP-GIZ

- Mr. Muhammad Yasir, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Ishaq, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Fayaz Soomro, Deputy Director (SS&C Wing) NAVTTC

NAVTTC team under the leadership of Dr. Muqeem ul Islam initiated development of CBT & A based qualifications of diploma level-5 as a reform project of TVET sector in November 2018 and completed 27 NVQF diplomas of Level-5 in September, 2019. It seems worth highlighting that during this endeavor apart from developing competency standards/curricula in conventional trades new dimensions containing high-tech trades in TVET sector in the context of generation IR 4.0 trades have also been developed which inter alia includes Robotics, Mechatronics, artificial intelligence, industrial automation, instrumentation and process control. Moreover, trades like entrepreneurship, green/environmental skills and variety of soft/digital skill have also been developed to equip the Pakistani youth with skills set as per requirement of the global trends. These skills have been made integral part of all the 27 diplomas.

Nobody has been more important in the pursuit of this project than Dr. Nasir Khan, Executive Director, NAVTTC, whose patronage and support remain there throughout the development process and lastly to thanks specially to Syed Javed Hassan, Chairman NAVTTC and Raja Saad Khan, Deputy Team Lead TSSP-GIZ who made it happened in this challenging time.

Contents

1. Introduction	5

2. Purpose of the Qualification	5

3.  Core competencies of the Qualification	5

4. Date of Validation	22

5. Codes of Qualifications	22

6. Members of Qualifications Development Committee	23

7. Members of Qualifications Validation Committee	23

8. Entry Requirements	24

9. Proposed Scheme of Studies	24

9. A- Occupation Packaging of Civil Technology	26

9.	B- Qualification Packaging of Civil Technology	27

10.	Detail of Qualifications and its Competency Standards Level Wise	27

10.	A. National Qualification Certificate in Civil Technology Level 2	27

10.1.	Basic Civil Surveying	27

0732B&CE-1	. Perform Chain Survey	27

0732B&CE-2	. Perform Compass Traversing	29

0732B&CE-3	. Carry out Plane Table Survey	31

0732B&CE-4	. Perform Levelling	33

0732B&CE-5	. Perform Reciprocal Levelling	35

0732B&CE-6	. Perform Long & Cross Sectioning	37

0732B&CE-7	. Perform Tacheometry	40

0732B&CE-8	. Perform Contouring	42

0732B&CE-9	. Perform Hydrographic Survey	44

0732B&CE-10	. Compute Volume by Contouring	46

10.2.	Civil Engineering. Drawing	48

0732B&CE-11	. Produce Templates, Title Block/ Strip & Draw Lines	48

0732B&CE-12	. Perform Lettering and Printing On Sheet.	51

0732B&CE-13	. Construct Scales for Drawing	53

0732B&CE-14	. Construct Geometrical Figures	55

0732B&CE-15	. Produce Orthographic Projections	58

0732B&CE-16	. Perform Sectioning	60

0732B&CE-17	. Perform Dimensioning On Drawings	62

0732B&CE-18	. Produce Pictorial Drawings	65

0732B&CE-19	. Develop Symbols of Engineering Drawings	68

0732B&CE-20	.    Produce Cross Section of Wall.	71

0732B&CE-21	.  Develop Building Drawings	73

0732B&CE-22	.  Perform House Planning	77

0732B&CE-23	. Produce Layout Plans for Building Services.	79

0732B&CE-24	.  Produce Drawings of Sanitary Structures	82

10.3.	Building Materials	84

0732B&CE-25	. Identify bricks	84

0732B&CE-26	. Dress stone for masonry	87

0732B&CE-27	. Prepare slaked lime	89

0732B&CE-28	. Identify and store cement for construction	91

0732B&CE-29	. Select sand for construction	93

0732B&CE-30	. Identify ferrous and non-ferrous metals	95

0732B&CE-31	. Identify advanced construction materials	97

0732B&CE-32	. Work Safely in Construction Industry	99

0732B&CE-33	. Interpret Simple Building Plans	101

0732B&CE-34	. Read and Interpret plans and Specifications	103

0732B&CE-35	. Apply power and hand tools in construction.	105

10.4.	Workshop Practice-I (wood working)	107

0732B&CE-36	. Carry out OH & S requirement in workshop	107

0732B&CE-37	. Determine properties and types of locally manufactured timber	109

0732B&CE-38	. Sharpen carpentry tools	111

0732B&CE-39	. Apply hand tools	113

0732B&CE-40	. Perform wooden joinery work	115

0732B&CE-41	. Apply fastenings	117

0732B&CE-42	. Apply traditional spirits polishing techniques	118

0732B&CE-43	. Apply portable power tools	120

0732B&CE-44	. Make simple calculations of timber	122

0732B&CE-45	. Construct a basic timber product	123

10.5.	Workshop Practice-II (Electrical wiring)	125

0732B&CE-46	. Carry out OH & S requirement in workshop	125

0732B&CE-47	. Maintain safety	127

0732B&CE-48	. Perform electrical wiring	129

0732B&CE-49	. Install electrical system	132

0732B&CE-50	. Construct circuits	135

0732B&CE-51	. Measure properties of electricity	138

0732B&CE-52	. Perform electrical wiring of a single room	140

11. B-	143

List of Tools, Equipment and Consumable	145

# 1. Introduction

Civil engineering is considered as the first discipline of the various branches of engineering after military engineering, and includes the designing, planning, construction, and maintenance of the infrastructure. The works include roads, bridges, buildings, dams, canals, water supply and numerous other facilities that affect the life of human beings. Civil engineering is intimately associated with the private and public sectors, including the individual homeowners and international enterprises. It is one of the oldest engineering professions, and ancient engineering achievements due to civil engineering include the pyramids of Egypt and road systems developed by the Romans.

Civil engineering has a significant role in the life of every human being, though one may not truly sense its importance in our daily routine. The function of civil engineering commences with the start of the day when we take a shower, since the water is delivered through a water supply system including a well-designed network of pipes, water treatment plant and other numerous associated services. The network of roads on which we drive while proceeding to school or work, the huge structural bridges we come across and the tall buildings where we work, all have been designed and constructed by civil engineers. Even the benefits of electricity we use are available to us through the contribution of civil engineers who constructed the towers for the transmission lines. In fact, no sphere of life may be identified that does not include the contribution of civil engineering. Thus, the importance of civil engineering may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Civil Technology under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# 2. Purpose of the Qualification

The purpose of this qualifications is to set high professional standards for civil technology sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for civil technology industry

- Enable the existing workforce to capacitate themselves in new technologies and methods

# 3.  Core competencies of the Qualification

The proposed curriculum is composed of 306 competencies that will be covered in 3600 Learning hours. Duration of the course is proposed to be Three years. The total weightage for technical competencies is 3600 hours. A total of 1200 hours have been reserved for allied subjects i.e. Islamic studies, English, Mathematics, Physics and Chemistry.

The details of technical modules are given as under:

Level 1 & 2 = 6 months. (Single Semester)

Level - 3 = 6 months. (Single Semester)

Level - 4 = 1 Year. (Two Semesters)

Level - 5 = 1 Year. (Two Semesters)

The overall distribution of contact hours and Credit Hours is given below:

Total.		3600 hours. & 360 Credits

Theory.	976 hours (27.11 %)

Practical.	2624 hours (72.89 %)

The distribution of contact hours and credit hours in each level is given below:

Level 1 & 2 = 6 months (16 weeks)

Total.		592 hours. & 59.2 Credits

Theory.	128 hours (22 %)

Practical.	464 hours (78 %)

Level - 3 = 6 months (16 weeks)

Total.		608 hours. & 60.8 Credits

Theory.	128 hours (21 %)

Practical.	480 hours (79 %)

Level - 4 = 1 Year (16 weeks + 16 Weeks)

Total.		1200 hours. & 120 Credits

Theory.	240 hours (20 %)

Practical.	960 hours (80 %)

Level - 5 = 1 Year (16 weeks + 16 Weeks)

Total.		1200 hours. & 120 Credits

Theory.	480 hours (40 %)

Practical.	720 hours (60 %)

National Vocational Certificate Level 1 & 2 Site assistant in Civil Technology

| S# | Competency Standard | Category | Level | Theory | Practical | Total | Credit Hour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11.01 Basic Engineering Surveying |  |  | 32 | 160 | 192 | 19.2 |
| 0732B&CE001 | Perform Chain Survey | Core | 2 | 4 | 9 | 13 | 1.3 |
| 0732B&CE002 | Perform Compass Traversing | Core | 2 | 4 | 15 | 19 | 1.9 |
| 0732B&CE003 | Carry out Plane Table Survey | Core | 2 | 4 | 12 | 16 | 1.6 |
| 0732B&CE004 | Perform Levelling | Core | 2 | 4 | 27 | 31 | 3.1 |
| 0732B&CE005 | Perform Reciprocal Levelling | Core | 2 | 2 | 21 | 23 | 2.3 |
| 0732B&CE006 | Perform Long & Cross Sectioning | Core | 2 | 2 | 22 | 24 | 2.4 |
| 0732B&CE007 | Perform Tacheometry | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE008 | Perform Contouring | Core | 2 | 2 | 18 | 20 | 2 |
| 0732B&CE009 | Perform Hydrographic Survey | Core | 2 | 6 | 15 | 21 | 2.1 |
| 0732B&CE010 | Compute Volume by Contouring | Core | 2 | 2 | 12 | 14 | 1.4 |
|  | 11.02 Civil Engineering Drawing |  |  | 32 | 160 | 192 | 19.2 |
| 0732B&CE011 | Produce Templates, Title Block/ Strip & Draw Lines | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE012 | Perform Lettering and Printing on Sheet. | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE013 | Construct Scales for Drawing | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE014 | Construct Geometrical Figures | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE015 | Produce Orthographic Projections | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE016 | Perform Sectioning | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE017 | Perform Dimensioning On Drawings | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE018 | Produce Pictorial Drawings | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE019 | Develop Symbols of Engineering Drawings | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE020 | Produce Cross Section of Wall. | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE021 | Develop Building Drawings | Core | 2 | 6 | 34 | 40 | 4 |
| 0732B&CE022 | Perform House Planning | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE023 | Produce Layout Plans for Building Services. | Core | 2 | 2 | 18 | 20 | 2 |
| 0732B&CE024 | Produce Drawings of Sanitary Structures | Core | 2 | 2 | 9 | 11 | 1.1 |
|  | 11.03 Building Materials |  |  | 32 | 48 | 80 | 8 |
| 0732B&CE025 | Identify bricks | Core | 2 | 4 | 6 | 10 | 1 |
| 0732B&CE026 | Dress stone for masonry | Core | 2 | 4 | 6 | 10 | 1 |
| 0732B&CE027 | Prepare slaked lime | Core | 2 | 2 | 2 | 4 | 0.5 |
| 0732B&CE028 | Identify and store cement for construction | Core | 2 | 4 | 6 | 10 | 1 |
| 0732B&CE029 | Select sand for construction | Core | 2 | 2 | 3 | 5 | 0.5 |
| 0732B&CE030 | Identify ferrous and non-ferrous metals | Core | 2 | 2 | 6 | 8 | 1 |
| 0732B&CE031 | Identify advanced construction materials | Core | 2 | 4 | 9 | 13 | 1 |
| 0732B&CE032 | Work Safely in Construction Industry | Generic | 2 | 4 | 1 | 5 | 0.5 |
| 0732B&CE033 | Interpret Simple Building Plans | Core | 2 | 2 | 3 | 5 | 0.5 |
| 0732B&CE034 | Read and Interpret plans and Specifications | Core | 2 | 2 | 3 | 5 | 0.5 |
| 0732B&CE035 | Apply power and hand tools in construction. | Core | 2 | 2 | 3 | 5 | 0.5 |
|  | 11.04 Workshop Practice (Wood Work) |  |  | 16 | 48 | 64 | 6.4 |
| 0732B&CE036 | Carry out OH & S requirement in workshop | Generic | 2 | 2 | 3 | 5 | 0.5 |
| 0732B&CE037 | Determine properties and types of locally manufactured timber | Generic | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE038 | Sharpen carpentry tools | Core | 2 | 1 | 6 | 7 | 0.7 |
| 0732B&CE039 | Apply hand tools | Core | 2 | 1 | 6 | 7 | 0.7 |
| 0732B&CE040 | Perform wooden joinery work | Core | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE041 | Apply fastenings | Core | 2 | 1 | 3 | 4 | 0.4 |
| 0732B&CE042 | Apply traditional spirits polishing techniques | Core | 2 | 2 | 3 | 5 | 0.5 |
| 0732B&CE043 | Apply portable power tools | Core | 2 | 1 | 3 | 4 | 0.4 |
| 0732B&CE044 | Make simple calculations of timber | Core | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE045 | Construct a basic timber product | Core | 2 | 2 | 6 | 8 | 0.8 |
|  | 11.05 Workshop Practice (Electrical Wiring) |  |  | 16 | 48 | 64 | 6.4 |
| 0732B&CE046 | Carry out OH & S requirement in workshop | Generic | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE047 | Maintain safety | Generic | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE048 | Perform electrical wiring | Core | 2 | 3 | 9 | 12 | 1.2 |
| 0732B&CE049 | Install electrical system | Core | 2 | 3 | 9 | 12 | 1.2 |
| 0732B&CE050 | Construct Circuits | Core | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE051 | Measure properties of electricity | Core | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE052 | Perform electrical wiring of a single room | Core | 2 | 2 | 6 | 8 | 0.8 |

# 4. Date of Validation

The level 5 of National DAE qualification on Civil Technology has been validated by the Qualifications Development Committee (QDC) members on 27-29 May 2019 and will remain in currency until May, 2022

# 5. Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Civil Technology level-5 |  |
| --- | --- |
| Code | Description |
| 000000000 | National Vocational Certificate of level1& 2, Site assistant in “Civil Technology” |
| 000000000 | National Vocational Certificate of level-3,Assistant surveyor in “Civil Technology” |
| 000000000 | National Vocational Certificate of level-4 Part-I, Supervisor in “Civil Technology” |
| 000000000 | National Vocational Certificate of level-4 Part-II, Supervisor in “Civil Technology” |
| 000000000 | National Diploma in Cicil Technology Level-5 Part-I |
| 000000000 | National Diploma in Civil Technology Level-5 Part-II |

# 6. Members of Qualifications Development Committee

The following members participated in the qualification’s development and validation of these qualifications:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Azhar Iqbal Shad | Chief Instructor (Civil) | GCT Raiwnd Road, LHR |
| 2 | Engr. Mazhar Abbas Naqvi | Principal | GSTC, Faisalabad |
| 3 | Engr. Naveed Ahmad Ajmal | Sr. Instructor (Civil) | GCT Bahalpur |
| 4 | Imtiaz Ahmad Awan | Sr. Instructor (Civil) | GCT Rasul (M.B.Din) |
| 5 | Muhammad Amjad Rafique | Sr. Instructor (Civil) | GCT Rasul (M.B.Din) |
| 6 | Engr.Zuneera Ashfaq | Asstt Director | PITAC Lahore |
| 7 | Engr Rehan Younis | Asstt Director | FWO- Rawalpindi |
| 8 | Engr. Mohsin Jahanzeb | Instructor (Civil) | GSTC, Faisalabad |
| 9 | Muhammad Asad Mahmood | Contractor | G.Five Construction Co. Faisal Abad |
| 10 | Engr. Norheen Amina | Consultant | Allied Engg. Services Lahore |
| 11 | Hira Ishtiaq | Coordinator Engr. | Allied Engg. Services Lahore |
| 12 | Engr. Hafsah Khalid | Project Manager | ANS Associates Lahore |
| 13 | Engr. Habiba Mohsin | Project Manager | Jers Engg. Co. Lahore |
| 14 | Mian Muhammad Naeem | Consultant Engr. | Mian Associates Lahore |
| 15 | Engr. Rana Haroon Mujahid | SDO, Bridges & Roads | C&W Faisalabad |
| 16 | Bushra Asghar | Instructor | GCT (w) Lahore |
| 17 | Engr. Inayet ur Rehman | DACUM, Facilitator | Peshawar |
| 18 | Muhammad Ishaq | QDC Coordinator | NAVTTC, Islamabad |

# 7. Members of Qualifications Validation Committee

The following members participated in the qualification development and validation of these qualifications:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Azhar Iqbal Shad | Chief Instructor (Civil) | GCT Raiwnd Road, LHR |
| 2 | Engr. Mazhar Abbas Naqvi | Principal | GSTC, Faisalabad |
| 3 | Mr. Imtiaz Ahmad Awan | Sr. Instructor (Civil) | GCT Rasul (M.B.Din) |
| 4 | Mr. Tariq Ali | A / Professor (Civil) | GPI Batkhela KP-TEVTA |
| 5 | Mr. Fida Hussain Mirjat | Principal | GMI Matli Hyderabad |
| 5 | Mr. Muhammad Khurram Murad | DCE PBTE | Lahore |
| 6 | Engr. Inayat ur Rehmen | DACUM Facilitator | Ex. KP-TEVTA |
| 7 | Mr. Sikander Masood | Director SS & C. | NAVTTC Islamabad |

# 8. Entry Requirements

| National Vocational Certificate Level 1 & 2, Site assistant in Civil Technology | Middle |
| --- | --- |
| National Vocational Certificate Level 3, Assistant Surveyor in Civil Technology | Level 1 & 2 |
| National Vocational Certificate Level 4, Site technology in Civil Technology | Level 3 |
| National Diploma in Civil Technology Level 5, | Level 4 |

# 9. Proposed Scheme of Studies

(000000000) National Vocational Certificate Level 1 & 2 (Site assistant in Civil Technology)

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 000000000 | Basic Engineering Surveying | Technical | 32 | 160 | 192 | 19.2 | 2 | 10 | 12 |
| 000000000 | Civil Engineering Drawing | Technical | 32 | 160 | 192 | 19.2 | 2 | 10 | 12 |
| 000000000 | Building Materials | Technical | 32 | 48 | 80 | 8 | 2 | 3 | 05 |
| 000000000 | Workshop Practice-I (Wood Working) | Technical | 16 | 48 | 64 | 6.4 | 1 | 3 | 4 |
| 000000000 | Workshop Practice-II (Electrical Wiring) | Technical | 16 | 48 | 64 | 6.4 | 1 | 3 | 4 |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| Total |  |  | 128 | 464 | 592 | 59.2 | 8 | 29 | 37 |

# 9. A- Occupation Packaging of Civil Technology

| S.# | Name of Occupation | Level | Digital Skills | Soft Skills | Technical Skills |
| --- | --- | --- | --- | --- | --- |
|  | Civil Surveyor | 3 | 0732B&CE- 109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-1 to 21, 53 to 66 |
|  | Draftsman | 3 | 0732B&CE-109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-11 to 24, 67 to 78 |
|  | Quantity Surveyor | 5 | 0732B&CE- 109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-6, 8, 10, 11 to 25, 67 to 78, 151 to 169, 206 to 218, 219 to 234, 241 to 246, 256 to 260, 277 to 286 |
|  | Quantity Surveyor after Draftsman | 5 | ----- | ----- | 0732B&CE-6, 8, 10, 25 to 35, 151 to 169, 206 to 218, 219 to 234, 241 to 246, 256 to 260, 277 to 286 |
|  | Materials Technician | 5 | 0732B&CE- 109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-25, 173, 174, 206 to 211, 249 to 255, 261 to 265, 279, 280. |
|  | Construction Quality control Supervisor | 5 | 0732B&CE-109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-1 to 21, 25 to 35, 53 to 66, 173, 174, 206 to 211, 249 to 255, 261 to 265, 267 to 274, 279, 280 |
|  | Mason | 2 | ------ | ------ | 0732B&CE-25 to 35, 79 to 100, 131 to 150, 189 to 194, 199, 203, 204. |
|  | Plumber | 2 | ----- | --- | 0732B&CE-101 to 108, 164, 228, 230, 231 |
|  | Domestic Electrician | 2 | ------ | ------ | 0732B&CE-46 to 52 |

# B- Qualification Packaging of Civil Technology

| S.# | Name of Qualification | Level | Digital Skills | Soft Skills | Technical Skills |
| --- | --- | --- | --- | --- | --- |
| 1 | Site supervisor | 4 | 0732B&CE- 109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-1 to 24, 25 to 35, 36 to 45, 46 to 52, 53 to 66, 67 to 78, 79 to 100, 101 to 108, 131 to 150, 164 to 169, 189 to 205, 206 to 209, 226 to 234, 241 to 246, 256 to 260, 277 to 286 |

# Detail of Qualifications and its Competency Standards Level Wise

- A. National Qualification Certificate in Civil Technology Level 2

  - Basic Civil Surveying

- . Perform Chain Survey

Overview: This competency standard covers the skills and knowledge required to perform chainage offset and perform chain traversing survey by using the Engineer, meter chain and other different type of chains.

| Competency Units | Performance Criteria |
| --- | --- |
| - Measure a Line | - Identify appropriate chain to be used for chainage - Perform ranging of line by using ranging rods - Unfold chain to record chainage by identifying talley - Fold chain properly according to standard procedure |
| - Perform Chain survey | - Select appropriate chain and ranging rods for chain survey - Mark station points which would cover area to be surveyed - Perform ranging of every line to be chainage - Record chainage of every line of traverse. - Identify cross staff to take normal offsets - Record offsets of required points - Prepare map of area |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1.  Explain Chain Surrey, their specifications and its measurement.

K2.  Explain cross staff with its types.

K3.  Explain Compass, its parts, measure bearing

K4.  Prepare Traverse sheet and calculate areas.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare layout by chain survey

- Taking offsets by cross staff

- Calculate area by chain survey

- Fold and unfold the chain.

- Run a traverse and calculate its area

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Calculator,

- Measuring scale,

- Drawing sheet,

- Measuring Tape,

- Compass,

- Chain (Engineering, Gunter & Metric)

- Cross Staff

- Tripod

- Ranging Rods

- Pegs

- Arrows

- White Chalk/powder

- Scotch Tape

- . Perform Compass Traversing

Overview: This competency standard covers the skills and knowledge required to perform chainage offset and perform compass traversing survey by using the magnetic compass and different type of chains.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Read Bearing by Compass | - Identify compass and tripod stand to read bearing of line - Adjust compass on initial point of line by fixing and levelling of compass on tripod - Record bearing of line by sighting final point of line - Repeat procedure for back bearing by shifting compass at final point |
| CU-2 Perform Traversing by Compass | - Identify compass and tripod stand to read bearing of line - Mark station points which would cover area to be surveyed - Record bearing of each line of traverse in terms of fore bearing and back bearing - Measure distance of each line by chain or tape - Compile data of traverse in tabular form - Check for local attraction in readings of bearings - Apply correction for any error in readings of bearings - Plot traverse on drawing sheet |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1. Explain Chain Surrey, their specifications and its measurement.

K2.  Explain Compass, its parts, measure bearing

K3. Explain traversing and types of traverse

K4. Prepare Traverse sheet and calculate areas.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Measure distances between stations with Chain

- Prepare traverse by compass

- Focus the prism of compass

- Take and measure the bearings

- Plot the traverse and calculate area

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Calculator,

- Measuring scale,

- Drawing sheet,

- Measuring Tape,

- Compass,

- Chain (Engineering, Gunter & Metric)

- Cross Staff

- Tripod

- Ranging Rods

- Pegs

- Arrows

- White Chalk

- Scotch Tape

- . Carry out Plane Table Survey

Overview: This competency standard covers the skills and knowledge required to perform plane Table Survey by Orientation, Radiation and intersection Method by using the plane table u-frame, plumb bob, Alidade, telescopic Alidade and Trough compass

| Competency Units | Performance Criteria |
| --- | --- |
| - Carry out Plane Table Survey | - Identify the instruments used in plane table surveying - Fix the table on tripod at given station point by using instrument - Check leveling & centering of the table with the help of plumb bob - Fix Drawing Sheet & mark North by trough compass - Sight the objects by sight vane and measure distance - Show the conventional symbols of objects on map |
| - Carry out Plane Table Survey (By Radiation) | - Identify the instruments used in plane table surveying - Adjust the table on tripod at suitable point from where all objects could be covered - Fix Drawing Sheet & Mark North by trough compass - Sight the objects by sight vane and measure distance - Draw traverse by scale with Radiation - Show conventional symbols of objects on map |
| - Carry out Plane Table Survey (By Intersection) | - Identify the instruments used in plane table surveying - Mark base line which can be used for sighting the points from initial and final point of line - Adjust the table on initial point of base line - Fix Drawing Sheet & Mark North by trough compass - Sight the objects by sight vane and measure distance - Adjust the table on final point of base line - Sight the objects by sight vane and measure distance - Draw traverse by scale with Intersection - Show conventional symbols of objects on map |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard.

This includes the knowledge of:

K1.  Explain Plane Table Survey with its different methods.

K2.  Explain base and check line in the Traverse.

K3.  Explain trough Compass, alidade, u-frame and plumb bob functions.

K4.  Explain Traverse Method and calculate areas.

K5.  Explain different conventional symbols with its importance.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Plan the work station

- Take centering  by u-frame

- Draw the rays with Alidade

- Check base line.

- Fold and unfold the tripod and u-frame.

- Prepare traverse by radiation, orientation and intersection method.

- Fix the plumb bob with tripod.

- Sighting by sight vane.

- Plot the traverse and calculate area

TOOLS AND EQUIPMENT’S

- Calculator,

- Measuring scale,

- Drawing sheet,

- Measuring Tape,

- Trough Compass,

- Tripod

- Ranging Rods

- Pegs

- Arrows

- Pencil

- Eraser

- White chalk

- Plane Table with clamp

- Plubing Fork (U-Frame)

- Scotch Tape

- Telescopic Alidade

- Laser level

- . Perform Levelling

Overview: This competency standard covers the skills and knowledge required to perform Levelling, by using the different type of leveling instruments, Staff and chains.

| Competency Units | Performance Criteria |
| --- | --- |
| - Determine height of instrument in Levelling | - Identify leveling instrument and leveling staffs - Select appropriate level and staff for leveling - Identify temporary Bench Mark - Take B.S on Bench mark - Determine height of instrument |
| - Perform Simple Levelling | - Select appropriate level and staff - Perform temporary adjustment of level - Observe staff reading on given Bench Mark - Record BS & FS in field book - Calculate Reduced Level |
| - Perform Differntial Levelling | - Select appropriate level and staff - Perform temporary Adjustment of level - Take readings on given Bench Mark with staff - Record BS, IS & FS in Field Book - Calculate Reduced Levels - Apply arithmetic check |
| - Carry out Fly Leveling | - Select appropriate level and staff - Perform temporary Adjustment of level - Take readings on given Bench Mark with staff - Record BS,& FS in Field Book - Calculate Reduced Levels - Apply arithmetic check |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1	Define leveling and describe the purpose of leveling.

K2	Define technical terms, level line, level surface, datum, datum line, horizontal plane, vertical plane, Horizontal line, vertical line, level line, line of collimation, Axis of telescope, bubble tube axes, back sight, foresight, Intermediate sight, change point, station point.

K3	Describe bench mark and its types Identify the parts and function of various types of tilting levels and Auto set level

K4	Explain with sketches leveling staves and their uses.

K5	List the steps involved in performing temporary adjustment of a level.

K6	Compute the reduced levels by rise & fall method and height of instrument method and recording the same on level book.

K7	Solve numerical problem on reciprocal leveling

K8	Describe the procedures for taking readings to reciprocal leveling precise leveling etc.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Set out Leveling points Level rom existing control stations.

- Carry out Arithmetic checks by formula

- Undertake setting out in the field using level,

- Taking offsets by cross staff

- Fold and unfold the tripod

- Focus the level

- Took and measure the readings BS, IS & FS

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Field Book

- Sharpner,

- Calculator,

- Measuring scale,

- Measuring steelTape,

- Level (Auto Set, Dumpy or Tilting)

- Tripod

- Ranging Rods

- Pegs

- Levelling staff (Metric or FPS)

- Thermometer

- Ball pein hammer

- . Perform Reciprocal Levelling

Overview: This competency standard covers the skills and knowledge required to determining the elevations, leveling between two widely separated points in which observations are made in both directions to eliminate the effects of atmospheric refraction and the curvature of the earth.

| Competency Units | Performance Criteria |
| --- | --- |
| - Carry out Reciprocal leveling | - Select appropriate level and staff - Mark two reciprocal points - Perform temporary Adjustment near X point. - Observe staff readings on both points X and Y - Perform temporary Adjustment near Y points - Calculate difference of both pair of staves readings. - Compare RL’s of both points - Compute elevations |
| - Carry out Reciprocal Ranging | P1. Mark any two reciprocal points P2. Range the points on line P3. Mark intermediate point on survey line P4. Observe ranging |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define fly leveling,

- Explain Profile Leveling

- Determine the missing data of a level book page.

- Define fly leveling.

- State precautions in leveling operation.

- Describe the procedures for taking readings to plot L-section x-section

- Plot Long section

- Describe parts and functions of Laser Level

- Explain the procedure of leveling by use of Laser Level. With their specifications and its measurement.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Find out BM & RLs.

- Focus the eye piece

- Took and measure the staff readings

- Make temporary adjustment of leveling instrument

- Compute different levels at different R.Ds

- Calculate height of Instrument

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Calculator,

- Measuring scale,

- Measuring Steel Tape,

- Tripod

- Pegs

- Arrows

- Level

- Levelling Staff

- White Chalk

- Thermometer

- Ball pein hammer

- . Perform Long & Cross Sectioning

Overview: This competency standard covers the skills and knowledge required to draw Cross sections, lines of levels or short profiles made perpendicular to the center line of the project. (For example, taking a cross section profile of a stream bed while doing a profile survey of the stream.) Cross sections are usually taken at regular intervals and at sudden changes in the center-line profile. long sectioning and cross sectioning are required to calculate earthwork for roads, Railway track and earthen embankment.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform Cross Sectioning | - Adjust Level & Locate temporary bench mark. - Take readings every 25 feet interval of existing track (Transversely) - Observe first reading on BM and the center line - Observe staff reading on specific interval starting from center line to right & left sides and record in the field book - Continue staff readings on every RD of 25 ft with same pattern (Left & Right) of given line - Calculate Reduced Levels & Re-check by formula |
| - Draw Cross Sections | - Compile the data obtained from cross sectioning to plot the cross section - Fix drawing sheet on drawing table - Draw cross section |
| - Perform Long Sectioning | - Locate temporary bench mark. - Establish stations every 30m /100 ft. interval of existing track (Longitudinally) - Observe first reading on temporary bench mark. - Observe reading on specific interval (30m /100 ft) and record in the field book - Calculate Reduced Levels at the centre line readings to plot the Longitudinal section |
| - Draw Long Sections | - Compile the data obtained from long sectioning to plot the long section - Fix drawing sheet on drawing table - Draw long section |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain cross hair and line of collimation for the level.

- Define cross-sectioning & Long Sectioning

- State precautions in leveling operation.

- Describe the procedures for taking readings to plot x-section

- Plot X-sections and Long Sections

- Describe Road chain age, cross fall, and road edge details

- Compute and tabulate of setting out data for the road alignment and Curves.

- Explain the purpose of the specialized surveying technique

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare layout survey for Road & Railway

- Take offsets by Theodolite

- Apply survey techniques

- Compute road alignment data

- Carry out appropriate checks

- Cross sections

- Pin and unpin the Leveling Instrument from the box.

- Prepare Cross section diagram.

- Focus the Eyepiece of the instrument.

- Take readings and measure the angles.

- Apply formula to calculate earthwork for site.

- Differentiate between long section and cross sections

- Draw long and cross sections

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Calculator,

- Measuring scale,

- Levelling Instrument

- Field Book,

- Steel Tape,

- Tripod

- Ranging Rods

- Pegs

- Arrows

- Staff

- White Chalk

- Thermometer

- Ball pein hammer / Mallet

- Umbrella with stand

- . Perform Tacheometry

Overview: This competency standard covers the skills and knowledge required to perform Lvelling for the land survey i-e making angles, workout vertical and horizontal distances by stadia cross hair using stadia constant.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1 Find out Horizontal distance | - Adjust the level on station point from where distance is to be measured - Place staff on the point to where distance is to be measured - Record upper stadia lower stadia and cross hair readings. - Calculate horizontal distance by stadia tachometry |
| CU-2 Find out Vertical distance | - Adjust the level on station point from where distance is to be measured - Place staff on the point to where distance is to be measured - Record upper stadia lower stadia and cross hair readings. - Calculate elevation by stadia tachometry |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1.  Explain Tachometry with its methods.

K2.  Explain Stadia lines.

K3.  Explain stadia constant.

K4.  Describe rule to find out distances.

K5.  Describe the method to take the readings by theodolite and levels.

K6.  Explain to apply the formula.

K7.  Describe to locate the Bench Mark.

K8.  Calculate vertical and horizontal distance

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare level orTheodolite on its tripod

- Take readings BS, IS, FS.

- Apply survey techniques

- Compute BM & RLs.

- Carry out appropriate checks

- Calculate Distances by using Theodolites

- Indicate Level, Staff and Tripod.

- Calculate difference between cross hair

- Locate stadia line

- TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Calculator,

- Tee scale,

- Set square

- Drawing Sheet

- Level (with stadia hairs), tripod  and staff

- Theodolite with tripod  and staff

- Ranging Rods

- Pegs

- Chain / Measuring tape

- Theodolite (Digital) with tripod

- White Chalk

- Thermometer

- Ball pein hammer

- Mallet

- Umbrella with stand

- . Perform Contouring

Overview: This competency standard covers the skills and knowledge required to perform Contouring for the land survey i-e an imaginary line passing through points of equal reduced levels by using the level and Theodolite with its components. A contour line may also be defined “as the intersection of a level surface with the surface of the earth”. Thus, contour lines on a plan illustrates the topography of the area

| Competency Units | Performance Criteria |
| --- | --- |

| - Carry out Contouring (Direct Mthod) | P1. Identify instruments P2. Perform temporary adjustment P3. Locate points P4. Draw radial Lines P5. Set contour interval P6. Draw contour lines on drawing sheet as per drawn on earth and put their levels according to given scale |
| --- | --- |
| - Carry out Contouring (In-Direct Mthod) | P1. Identify instruments P2. Perform temporary adjustment P3. Locate points P4. Draw squares on the ground P5. Set contour interval P6. Draw contour lines on drawing sheet as per drawn on earth and put their levels according to given or assumed scale P7. Recheck the calculations |
| - Draw Contour plan | P1. Take Terrain levels or arbitrary level P2. Take Contours intervals (as required) P3. Check Datum level on site P4. Building footprint and floor levels P5. Draw Utility services (drainage, gas, power line, sewer line, Telephone line, water etc.) |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1.  Explain Countering and its methods.

K2.  Explain Radial line and base line.

K3.  Explain contour interval.

K4.  Describe rule to draw the contour lines.

K5.  Explain contour for ponds and filling areas

K6.  Explain square in contours.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare contour map on drawing sheet

- Take contour for hill and pond.

- Apply survey techniques

- Compute contour for sewer alignment.

- Carry out appropriate checks

- Draw contour for hill and valley

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Calculator,

- Tee scale,

- Set square

- Drawing Sheet

- Plane table board with tripod

- Compass

- Ranging Rods

- Pegs

- Chain / Measuring tape

- Theodolite,(Digital) with tripod

- Staff

- White Chalk

- Thermometer

- Ball pein hammer / Mallet

- . Perform Hydrographic Survey

Overview: This competency standard covers the skills and knowledge required to perform in water with its components. It also covers the skills and knowledge required to perform Hydrographic survey for measurement and description of features which affect maritime navigation, marine construction, dredging, offshore oil exploration / offshore oil drilling and related activities.

| Competency Units | Performance Criteria |
| --- | --- |
| - Measure depth of a channel (by sounding rod) | - Perform Reconnaissance survey - Identify equipment for Hydrographic Survey - Locate Horizontal & vertical Control - Use stationery boat - Use sounding rod - Observe bed level of channel by sounding rod |
| - Measure depth of a channel (by lead line) | - Perform Reconnaissance survey - Identify equipment for Hydrographic Survey - Locate Horizontal & vertical Control - Use stationery boat - Use sounding chain - Observe chain reading - Determine bed level of channel |
| - Determine velocity of flowing water by current meter | - Identify current meter to determine velocity of water - Assemble current meter - Use stationery boat - Place current meter at two specified depths - Observe reading |
| - Determine velocity of flowing water by float | - Select area where turbulence is minimum - Mark section at least 3 times of x-section - Place float and observe time crossed by float in marked section - Determine velocity by float |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1.  Describe rule for hydrographic survey.

K2.  Describe Hydrographic Survey.

K3.  Explain Hydrographic Survey Instruments.

K4.  Describe the method to find out depth of channel.

K5.  Explain sounding Rods

K6. Explain sounding boats

K7.  Explain reconnaissance survey

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Drive sounding boat

- Compute depth for bed at water level.

- Carry out appropriate checks

- Calculate depth of channel

- Indicate sounding rods.

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Calculator,

- Tee scale,

- Set square

- Drawing Sheet

- Compass

- Sounding Rods

- Sounding Boat

- White Chalk

- Velocity Rod

- Umbrella with stand

- Sounding chain

- Sounding meter

- . Compute Volume by Contouring

Overview: This competency standard covers the skills and knowledge required to calculate Volume by a contour map, but the volume calculated by this method is approximate. It cannot be compared with the volume calculated by the cross-section method.

| Competency Units | Performance Criteria |
| --- | --- |
| - Compute volume by contouring (By Average End Area Method) | - Interpolate required contours for calculation - Find out the area of each contour with relevant instrument. - Apply Average End Area (trapezoidal) Method formula to find out volume between two contours - Sum up the volume in tabular forms for all contours |
| - Compute volume by contouring (By Prismoidal Method) | - Interpolate required contours for calculation - Find out the area of each contour with relevant instrument. - Apply prismoidal formula to find out volume between two contours - Sum up the volume in tabular forms for all contours |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain the uses of contour map.

- Explain the methods of performing contour survey.

- Interpolate contours on a plan.

- Explain the procedure to lay down alignment of road, railway and channel on contour map and describe procedure for measuring gradient.

- Compute the capacity of reservoirs and volume of earth from the contour map.

- Explain Countering and its methods.

- Explain Radial line and base line.

- Explain contour interval.

- Describe rule to draw the contour lines.

- Explain contour for ponds and filling areas

- Explain average area method

- Explain prismoidal formula

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare contour map on drawing sheet

- Take contour for hill and pond.

- Apply survey techniques

- Compute contour interval for different alignment.

- Carry out appropriate checks

- Apply prismoidal formula

- Apply average area formula

- Calculate volume for ground / earth work

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Calculator,

- Chain / Measuring tape

- Level (Digital) with tripod

- Staff

- White Chalk

- Thermometer

- Umbrella

  - Civil Engineering. Drawing

- . Produce Templates, Title Block/ Strip & Draw Lines

Overview: This competency standard covers the skills and knowledge required to prepare report interpreting health & safety considerations as per national standards, Select, Identify, uses of drawing instruments, draw title stip, draw title block on sheets, and draw different lines used in drawings.

| Competency Units | Performance Criteria |
| --- | --- |
| - Investigate and prepare a short report on the health and safety considerations in construction of drawings. | - Observe Considerations: light (natural, artificial), - CAD (computer screens, electrical protection devices) - Observe neatness for drawing place. - Check cleanliness of drawing instruments. |
| - Select Instruments for drawings | - Identify instruments (adjustable drawing board, tee square, set square, scale rule, compasses, drawing pens/pencils, flexible curves, French curve and templates) - Select instruments in correct orientation for drawings - Select Instruments for horizontal, vertical & inclined lines. |
| - Set out a drawing sheet | - Identify drawing sheets (British, American and ISO standards). - Draw margins as per local standard. - Draw title strip as per standards on imperial & half imperial size sheet. - Draw title block as per standards on imperial & half imperial size sheet. |
| - Draw different types of lines. | - Draw lines used in construction drawings. (basic construction line, main object outline, broken line, chain line, section line, grid line, cutting plane line, short break line, long break line.) - Use correct grades of pencils. - Draw lines of correct weight. - Draw lines with standard measurements. - Observe principles of drawing lines, cleanliness. |
| - Store prepared drawings. | - Fold A0, A1, A2, A3, A4, drawing sheets as per requirements. - Fold Double elephant, Imperial- full, half, quarter, antiquarian, drawing sheets as per requirements. - Store the drawing sheet as per requirements |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- State the importance of civil drafting as an engineering communication medium.

- Understand necessity of civil drafting in different engineering fields.

- Indicate the link between drafting and other subjects of study in diploma course.

- Describe the health and safety considerations involved in the production of construction drawings. Considerations: light (natural, artificial), CAD (computer screens, electrical protection devices), cleanliness.

- Identify the various types of drawing medium available and describe their use. Drawing medium: cartridge paper, tracing paper.

- Identify standard drawing sheet sizes. Sizes: A0, A1, A2, A3, A4, Double elephant, Imperial- full, half, quarter, antiquarian, A, B, C, D, E.

- Identify and state the use of the various types of drawing equipment along with sizes. Drawing Instruments: adjustable drawing board, tee, square, set square, scale rule, compasses, drawing pens/pencils, flexible curves, French curve, templates.

- Explain the different types of lines and arrowheads used in drawings. Line types: basic construction line, main object outline, broken line, chain line, section line, grid line

- Explain the weight of lines, grades of pencils.

- State the components of Title Block and Title Strip.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Report on health & safety considerations as per national standards.

- Drawing sheet of different types of of lines.

- Drawing of Title Block, Title Strip

- Describe half imperial and Quarter imperial sheets.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Graph sheets |
| 6 | Handkerchief | 6 | Thumb pins |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- . Perform Lettering and Printing On Sheet.

Overview: This competency standard covers the skills and knowledge required to draw letters on grapgh sheet, in 4:5 & 4:7 ratio, double stroke, single stroke, upper case and lower case lettering and printing on drawing sheet.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw Single Stroke letters in ratio 4:5 and 4:7 taking 1.5” letter height on graph sheet. | - Observes standards for basics of drawing. - Distribute space on sheet - Draw single stroke letters as per ratio. - Draw tile strip on drawing sheet. - Draw tile block on drawing sheet. |
| - Distribute space of half imperial drawing sheet for lettering. | - Observes standards for basics of drawing. - Use of correct grades of pencils. - Provide horizontal & vertical spaces - Distribution Sheet as per required standards. |
| - Prepare graph for drawing 1.5” letters in ratio 4:5 and 4:7 on half imperial sheet. | - Observes standards for basics of drawing. - Use of parallel lines principle for division of line into required number of segments vertically. - Use of requisite set square for horizontal division. - Draw graph lines. |
| - Draw Single Stroke letters in ratio 4:5 and 4:7 taking 1.5” letter height on half imperial drawing sheet. | - Observes standards for basics of drawing. - Distribute space with given ratio on drawing sheet. - Draw single stroke letters as per ratio. - Draw tile strip OR title block for drawing sheet. |
| - Draw Double Stroke letters in ratio 4:5 and 4:7 taking 1.5” letter height on half imperial drawing sheet. | - Observes standards for basics of drawing. - Distribute space and draw both graphs on drawing sheet. - Draw double stroke letters as per ratio. - Draw tile strip OR title block for drawing sheet. |
| - Draw Single Stroke upper case letters 1 cm high and ¼” high in ratio 4:5 and 4:7. | - Observes standards for basics of drawing. - Distribute space and draw both graphs on drawing sheet. - Draw single stroke upper case letters as per ratio. - Draw single stroke upper case free hand italic letters. |
| - Draw Free hand Single Stroke lower case letters 1 cm high and ¼” high printing in ratio 4:5 and 4:7. | - Observes standards for basics of drawing. - Distribute space and draw both graphs on drawing sheet. - Draw single stroke lower case free hand lettering & printing as per ratio. - Draw single stroke upper case free hand italic letters. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain the procedure of division of lines, space distribution of sheet.

- Explain principle of parallel lines and preparation of graph.

- State importance of lettering.

- State the need and different types of lettering.

- Describe different uses of lettering stencils for a given application.

- State the principles of lettering.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of letters on grapgh sheet in 4:5 & 4:7 ratio, double stroke, single stroke, upper case.

- Drawing of letters on half imperial sheet in 4:5 ratio, double stroke, single stroke, upper case and lower case lettering and printing.

- Drawing of letters on half imperial sheet in 4:7 ratio, double stroke, single stroke, upper case and lower case lettering and printing.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Graph sheets |
| 6 | Handkerchief | 6 | Thumb pins |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- . Construct Scales for Drawing

Overview: This competency standard covers the skills and knowledge required to draw reducing scales in MKS and FPS Units.

| Competency Units | Performance Criteria |
| --- | --- |
| - Construct a plain scale of RF 1: 5& 1:20 showing metres & decimetres on half imperial drawing sheet. | - Observes local standards for basics of drawing. - Distribute space in required part of drawing sheet - Draw tile strip OR title block for drawing sheet. - Draw scale of RF = 1:5 & 1:20 using principle of parallel lines. |
| - Construct a plain scale of RF 1:6 & 1: 48 showing feet & inches on half imperial drawing sheet. | - Observes local standards for basics of drawing. - Distribute space in required parts. - Draw tile strip for drawing sheet. - Draw scale of RF = 1:6 & 1:48 using principle of parallel lines. |
| - Construct a Diagonal scale of RF 1: 50 showing metres, decimetres & centimetres, of RF1:500 showing km, hectometres & decametres on A3 drawing sheet. | - Observes standards for basics of drawing. - Distribute space in relevant part of sheet. - Draw tile strip OR title block for drawing sheet. - Draw scale of RF = 1:50 using principle of parallel lines and diagonals. - Draw scale of RF = 1:500 using principle of parallel lines and diagonals. |
| - Construct a Diagonal scale of RF 1:36 showing yards, feet & inches, &of RF 1: 96 showing mile & furlong & 1/8 furlong on A3 drawing sheet. | - Observes standards for basics of drawing. - Distribute space in required parts. - Draw tile strip OR title block for drawing sheet. - Draw scale of RF = 1:36 using principle of parallel lines and diagonals. - Draw scale of RF = 1:96 using principle of parallel lines and diagonals. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define scale, Scale Factor and R.F.

- State the types of scales- Reducing scales, enlarging scales, full scale.

- Describe plane scales and diagonal scales.

- Describe the open-ended scale and full divided scales

- Explain the procedure of constructing plane scales.

- Explain the procedure of constructing diagonal scales.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of letters on grapgh sheet in 4:5 & 4:7 ratio, double stroke, single stroke, upper case.

- Drawing of letters on half imperial sheet in 4:5 ratio, double stroke, single stroke, upper case and lower case lettering and printing.

- Drawing of letters on half imperial sheet in 4:7 ratio, double stroke, single stroke, upper case and lower case lettering and printing.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- . Construct Geometrical Figures

Overview: This competency standard covers the skills and knowledge required to draw triangles, quadrilaterals, polygons- inscribed & circumscribed, draw prisms, draw pyramids, conic sections, circle, ellipse, parabola hyperbola and solid figures.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw sketches of different types of triangles according to side length and angles free hand and to scale. | - Observes standards for basics of drawing. - Distribute space on drawing sheet - Draw tile strip OR title block for drawing sheet - Draw equilateral triangle of side length 2”. - Draw isosceles triangle with base 5 cm and height= 6cm - Draw scalene triangle of lengths 2”, 1.5”, 1”. - Draw acute angled triangle. - Draw right angles triangle. - Draw obtuse angled triangle. |
| - Draw sketches of polygons, circle, semi-circle, segment of circle, sector of circle & zone of circle free hand and to a scale. | - Observe local standards for basics of drawing. - Distribute space on drawing sheet - Draw tile strip OR title block for drawing sheet - Draw a regular pentagon of side length = 4 cm. - Draw hexagon inscribes in a circle and circumscribed about a circle of 3 inches diameter. - Draw circle, semi-circle, segment of circle, sector of circle & zone of circle of diameter 1.5 inch and 3.5 inch. |
| - Draw sketches of all quadrilaterals | - Observes standards for basics of drawing. - Distribute space in required parts. - Draw title strip for drawing sheet - Draw square of side length 2”and 4.5 “ - Draw rectangle with length 5 cm and width= 3cm - Draw rhombus of side length=2 cm & internal angle= 45o - Draw parallelogram of length6 cm, width= 3 cm having internal angle= 45o - Draw trapezoid of parallel sides’ length4 cm & 6cm spaced at a distance of 3 cm. - Draw trapezium of side lengths 5 cm, 4 cm, 6 cm & 3 cm. |
| - Draw conic sections with cutting plane at different angles | - Draw conic section resulting in a circle. - Draw conic section resulting in an ellipse. - Draw conic section resulting in a parabola. - Draw conic section resulting in a hyperbola. - Draw title strip on sheet |
| - Draw full scale ellipses major axis = 10 cm and minor axis = 8 cm with four methods. | - Draw ellipse by four centers method. - Draw ellipse by basic method. - Draw ellipse by parallelogram method. - Draw ellipse by off-set method in concentric circles. - Draw title block on sheet |
| - Draw full scale parabola major axis = 10 cm and minor axis = 8 cm with four methods. | - Draw parabola by basic method. - Draw parabola by tangent method. - Draw parabola by rectangle method - Draw title block on sheet |
| - Draw sketched of prisms, cylinders, pyramids, cones in isometric views free hand and to a scale. | - Draw right & oblique rectangular prism. - Draw right and oblique cylinders. - Draw right & oblique hexagonal pyramids. - Draw right & oblique cones. - Draw title block on sheet |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- State plane and solid figures.

- Freeh and sketching of plane figures and solid figures.

- State the difference between plane and sold figure.

- Draw free hand sketches of different plane and solid figures.

- Describe the procedure for the construction of angles.

- Describe different types of triangles, quadrilaterals and polygons.

- State difference between inscribed and circumscribed figures.

- State the terms used in a circle.

- Sketch and label different lines and arcs in a circle.

- Define ellipse and parabola

- State cone, conical sections, (circle, parabola, ellipse and hyperbola).

- Explain conical sections in civil engineering drawings.

- Explain the procedure of drawing plane and solid geometrical figures.

- Explain the procedure of drawing ellipses & parabolas by different methods.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Construction of triangles, quadrilaterals, polygons- inscribed & circumscribed

- Drawings for prism Cylinders, pyramids, Cones, & different conical sections

- Drawings for circle, ellipse, parabola hyperbola and solid figures.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- . Produce Orthographic Projections

Overview: This competency standard covers the skills and knowledge required to draw orthographic projections, First angle projections, third angle projections of vee block, hollow concrete block, wooden cut blocks and draw missing view.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw orthographic projections of Vee Block in first angle projection system. | - Apply basic operations on A3 size sheet. - Distribute space on sheet by calculations. - Draw title strip on sheet - Draw front, top and side views by use of projector and drawing instruments. |
| - Draw orthographic projections of hollow concrete Block in first angle projection system. | - Apply basic operations on A3 size sheet - Distribute space by calculations. - Draw title strip on sheet - Draw front, top and side views by use of projector and drawing instruments. |
| - Draw orthographic projections of wooden cut block in first angle projection system. | - Apply basic operations on A3 size sheet. - Distribute space by calculations. - Draw title strip on sheet - Draw front, top and side views by use of projector and drawing instruments. |
| - Complete missing views when two views are given. | - Apply basic operations on A3 size sheet. - Distribute space by calculations. - Draw title strip on block - Draw front, top and side views by use of projector and drawing instruments. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define plane, principal plane.

- Explain the principle of orthographic projection with simple sketches.

- State the definition of projector and projection lines and their use.

- State and differentiate between dihedral and trihedral angles.

- State the types of orthographic projection.

- Sketch the orthographic views of a simple engineering part of given pictorial drawing.

- Identify the object from a number of orthographic views of the object.

- Select the minimum number of views needed to fully represent a given object

- Differentiate between First & third angle projections for different 3-D objects

- Define 3-D objects

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of First angle projections of V-Block

- Drawing of third angle projections of hollow concrete block.

- Drawing of first angle & third angle projections of wooden cut blocks.

- Drawing of third angle projections of missing view.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- . Perform Sectioning

Overview: This competency standard covers the skills and knowledge required to draw full sectional views & offset sectional views of hollow concrete block, hollow hexagonal prism nad rectangular pyramid.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw full sectional front view and outside top view of the hollow concrete block. | - Apply basic operations on A3 size sheet. - Draw title strip on sheet - Distribute space by calculations. - Draw front, top and side views of hollow concrete block. - Draw materials symbols - Hatch the section portion in views. |
| - Drawing of full sectional front view, side view and top view of the prisms, pyramids of different types. | - Apply basic operations on A3 size sheet. - Draw title strip on sheet - Distribute space by calculations. - Draw front, top and side views of hollow hexagonal prism. - Draw front, top and side views of rectangular pyramid. - Hatch the section portion in views. |
| - Draw offset sectional front view and outside top view of the hollow concrete block. | - Apply basic operations on A3 size sheet. - Draw title strip on sheet - Distribute space by calculations. - Draw front, top and side views of hollow concrete block. - Hatch the section portion in views. |
| - Drawing of offset sectional front view, side view and top view of the prisms, pyramids of different types. | - Apply basic operations on A3 size sheet. - Draw title strip on sheet - Distribute space by calculations. - Draw front, top and side views of hollow hexagonal prism. - Draw front, top and side views of rectangular pyramid. - Hatch the section portion in views. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- State the definition of section and sectioning.

- Explain purpose of sectional views.

- State cutting plane and cutting plane line.

- State the purpose of cutting plane line.

- State conventional representation of engineering materials.

- Know rule of putting arrowhead on cutting plane line.

- State types of sectional views.

- Select the position of cutting plane line to give maximum details of object.

- Define Hatching

- Explain the principles of hatching.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of full sectional views & offset sectional views of hollow concrete block

- Drawing of full sectional views & offset sectional views of hollow hexagonal prism nad rectangular pyramid.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- . Perform Dimensioning On Drawings

Overview: This competency standard covers the skills and knowledge required to dimension a drawing by adopting Uni-directional and bi-directional dimensioning, Base line dimensioning system, continuous dimensioning system, location dimensioning.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare a Dimensioned drawing by adopting Uni-directional and bi-directional dimensioning. | - Apply basic operations on A3 size sheet. - Draw title strip on sheet - Distribute space by calculations. - Draw front, top and side views of Vee block. - Dimension the drawing Uni-directionally. - Dimension the drawing bi-directionally. |
| - Prepare a Dimensioned drawing by adopting Base line dimensioning system, | - Apply basic operations on A3 size sheet. - Draw title strip on sheet - Distribute space by calculations. - Draw front, top and side views of Vee block with holes. - Dimension the drawing as per base line dimensioning system. - Draw system of dimensioning - Draw dimensioning of holes - Draw dimensioning of arc - Draw dimensioning circles - Draw dimensioning of angles |
| - Prepare a Dimensioned drawing by adopting continuous dimensioning system | - Apply basic operations on A3 size sheet. - Draw title strip on sheet - Distribute space by calculations. - Draw front, top and side views of Vee block with holes. - Dimension the drawing as per continuous dimensioning system. - Draw system of dimensioning - Draw dimensioning of holes - Draw dimensioning of arc - Draw dimensioning circles - Draw dimensioning of angles |
| - Prepare a Dimensioned drawing by adopting location dimensioning system | - Apply basic operations on A3 size sheet. - Draw title strip on sheet - Distribute space by calculations. - Draw front, top and side views of Vee block with holes. - Dimension the drawing location of center of holes - Draw system of dimensioning - Draw dimensioning of holes - Draw dimensioning of arc - Draw dimensioning circles - Draw dimensioning of angles |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Dimensioning.

- State the need of dimensioning drawings according to accepted standards

- State the dimension and extension line, arrow head, dimension, leader.

- State the types of arrow head, length of arrowhead.

- Identify the system of placement of dimensions of a given dimensioned drawing.

- Describe Uni-directional and bi-directional dimensioning.

- Define Base line dimensioning system, continuous dimensioning system, location dimensioning.

- Draw dimension for location and size of holes, arcs, circles.

- Dimension a given drawing using standard notations and desired system of dimensioning.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Dimensioned drawing by adopting Uni-directional and bi-directional dimensioning,

- Dimensioned drawing by Base line dimensioning system, continuous dimensioning system, location dimensioning.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- . Produce Pictorial Drawings

Overview: This competency standard covers the skills and knowledge required to draw isometric views, oblique views, and perspective views of objects. Draw isometric, oblique and perspective views from principal views.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw isometric views of a cube having circular hole in its focus and R.C.C. stairs (First three steps). | - Apply basic operations on different size sheet. - Draw title strip on sheet - Divide space on drawing sheet. - Select base line on sheet - Draw isometric view observing OHSE requirements. - Draw holes (ellipses in three isometric planes.) |
| - Draw isometric views of R.C.C. stairs-First three steps. | - Select base line on sheet - Draw title strip on sheet - Draw isometric view observing OHSE requirements. - Perform lettering & Dimensioning. |
| - Create an oblique drawing of different prisms and pyramids from its given principal views. | - Select base line. - Draw title strip on sheet - Draw oblique view with the help of given principal views. - Perform lettering & Dimensioning. |
| - Perspective drawing of slotted block and different wooden blocks from there given principal views. | - Select base line. - Draw the principal views. - Select the vanishing points, mirror line - Draw Perspective view. - Perform lettering & Dimensioning. - Draw title strip on sheet |
| - Draw Isometric view with the help of orthographic projections of hollow concrete block | - Select base line. - Draw title strip on sheet - Draw isometric view with the help of given principal views. - Perform lettering & Dimensioning. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define pictorial drawing.

- State the types of pictorial drawings and their general uses.

- Sketch isometric axis, angles, scales, arcs and circles.

- Differentiate between the isometric and non-isometric lines.

- Sketch isometric drawing and isometric projection.

- Sketch the isometric projection from the given orthographic drawings.

- Explain the angle of receding axis.

- State the oblique drawing and its uses.

- Sketch and letter the oblique cavalier and cabinet views.

- Define perspective drawing.

- Explain the purpose of perspective drawing.

- State the vanishing point.

- State the principles of making perspective views.

- State the parallel and angular (diametric and trimetric) perspective

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of isometric views of a cube having circular hole in its focus and R.C.C. stairs (First three steps).

- Drawing of an oblique view of different prisms and pyramids from its given principal views.

- Perspective drawing of slotted block and different wooden blocks from there given principal views.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- . Develop Symbols of Engineering Drawings

Overview: This competency standard covers the skills and knowledge required to prepare report on the health and safety considerations while preparing drawings, draw different symbols of Engineering materials, components of building in plan, elevation & section, draw symbols of electrical installations, water supply installation, gas installations and sanitary installations.

| Competency Units | Performance Criteria |
| --- | --- |
| - Investigate and prepare a short report on the health and safety considerations in preparing drawings. | - Observe Considerations: light (natural, artificial), - CAD (computer screens, electrical protection devices) - Observe neatness & cleanliness of drawing |
| - Draw symbols of engineering materials. | - Select the suitable instruments to draw symbols. - Draw title strip on sheet - Distribute space of drawing sheet. - Draw symbols duly hatched as per local standards of- Sand, ballasts, metals, timbers, soil-natural, cutting, filling, fabrics, rock, glass, ceramics, plastics, asbestos. |
| - Draw symbols of building components in plan, section, & section. | - Select the suitable instruments to draw symbols. - Draw title strip on sheet - Distribute space of drawing sheet. - Draw symbols in plan, section, & section duly hatched as per local standards of- brick work, stone work, block work, doors, windows, ventilators, RCC work, R.B. work, PCC, DPC, Lintels, stairs |
| - Draw symbols of electrical installations for ceiling and walls. | - Select the suitable instruments to draw symbols. - Draw title strip on sheet - Distribute space of drawing sheet. - Draw symbols duly hatched as per local standards of- energy meter, main switches, sub-main switches, circuit breakers, kit kat, panel box, DFB, tube lights, holders, fans, bulbs, switches, socket, boards, circuit diagram |
| - Draw symbols of water supply and gas installations. | - Select the suitable instruments to draw symbols. - Draw title strip on sheet - Distribute space of drawing sheet. - Draw symbols duly hatched as per local standards of- water pipe lines, mixers, valves, cocks, taps, showers, pump, meter, cooler. Gas pipe line, Gas meter, gas heater, gas light, gas geezer, gas burner. |
| - Draw symbols of sanitary installations. | - Select the suitable instruments to draw symbols. - Draw title strip on sheet - Distribute space of drawing sheet. - Draw symbols duly hatched as per local standards of- sewer lines, wash hand basins, water closets, bath tubs, urinals, sinks, dish washers, looking mirror, toilet paper holder, soap dish, shelf, towel rail, vent pipe, manhole, intercepting chambers, traps, and grating. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define conventional symbols and give its importance.

- Enlist and sketch symbols of Engineering materials- Sand, ballasts, metals, timbers, soil-natural, cutting, filling, fabrics, rock, glass, ceramics, plastics, asbestos.

- Enlist and sketch components of building in plan, elevation & section- brick work, stone work, block work, doors, windows, ventilators, RCC work, R.B. work, PCC, DPC, Lintels, stairs.

- Enlist and sketch symbols of electrical installations- energy meter, main switches, sub-main switches, circuit breakers, kit kat, panel box, DFB, tube lights, holders, fans, bulbs, switches, socket, boards, circuit diagram.

- Enlist and sketch water supply installation, gas installations- water pipe lines, mixers, valves, cocks, taps, showers, pump, meter, cooler. Gas pipe line, Gas meter, gas heater, gas light, gas geezer, gas burner.

- Enlist and sketch sanitary installations- sewer lines, wash hand basins, water closets, bath tubs, urinals, sinks, dish washers, looking mirror, toilet paper holder, soap dish, shelf, towel rail, vent pipe, manhole, intercepting chambers, traps, grating.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing symbols of Engineering materials.

- Drawing symbols of components of building in plan, elevation & section

- Drawing symbols of symbols of electrical installations

- Drawing symbols of water supply installation

- Drawing symbols of gas installations

- Drawing symbols of sanitary installations.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- .    Produce Cross Section of Wall.

Overview: This competency standard covers the skills and knowledge required to draw 2 dimensional & 3 dimensional cross section of wall along with flooring, roofing & finishing details.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw Cross section of 9” thick wall, 13.5” Thick wall. | - Observes standards for basics of drawing. - Draw title block on sheet - Distribute space and draw both graphs on graph sheet. - Draw parts of wall- foundation concrete, brick work, GL. FL., DPC, Super structure, plastering, lintel, sunshade, doors, windows, ventilator, almirah. - Label drawing parts. |
| - Draw Cross section of 20 cm thick wall, 30 cm Thick wall. | - Observes standards for basics of drawing. - Draw title block on sheet - Distribute space of sheet. - Draw parts of wall- foundation concrete, brick work, GL. FL., DPC, Super structure, plastering, lintel, sunshade, doors, windows, ventilator and almirah. - Label drawing parts. |
| - Draw parts of flooring | - Observes standards for basics of drawing. - Draw title strip on sheet - Draw parts of flooring- earth filling, sand filling, base concrete, topping floor. - Label drawing parts. |
| - Draw parts of flooring. | - Observes standards for basics of drawing. - Draw title strip on sheet - Parts of roofing- roof slab, slab projection, parapet, drip course, cornice, bitumen coating, polythene sheet, earth filling over plaster, tile flooring. - Label drawing parts. |
| - Draw cross section of 13.5” thick wall in isometric view. | - Draw substructure of 13.5” thick walls. - Draw title strip on sheet - Draw super structure components of wall. - Draw roofing components along with parapet- roof slab, slab projection, parapet, drip course, cornice, bitumen coating, polythene sheet, earth filling over plaster, tile flooring. - Draw flooring components- earth filling, sand, base concrete and topping floor. - Draw window, lintel and sunshade. - Label drawing parts. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Enlist & sketch parts of wall- foundation concrete, brick work, GL. FL., DPC, Super structure, plastering, lintel, sunshade, doors, windows, ventilator, almirah.

- Enlist & sketch parts of flooring- earth filling, sand filling, base concrete, topping floor.

- Enlist & sketch parts of roofing- roof slab, slab projection, parapet, drip course, cornice, bitumen coating, polythene sheet, earth filling over plaster, tile flooring.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of cross section of wall along with flooring, roofing & finishing detail.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- .  Develop Building Drawings

Overview: This competency standard covers the skills and knowledge required to read, draw plans, elevations, sections, schedules of single room, two roomed quarter, c-type building and submission drawing.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare report on various features after reading the drawing of an existing building. | - Observe feature like rooms and its sizes. - Investigate inter-relation of rooms. - Check openings in building. - Observe building services. |
| - Prepare drawings for single room 14’ x 12’ having wall thickness 9”. | - Observes local standards for basics of drawing. - Draw title block for drawing sheet. - Distribute space in required parts on drawing sheet - Draw line plan of room as per given data. - Draw detailed plan of room. - Draw Elevation of single room. - Draw Section of room passing through opening. - Perform dimensioning and lettering - Indicate North direction. |
| - Prepare drawings for two roomed quarter along with bath, kitchen, verandah, courtyard. | - Observes local standards for basics of drawing. - Draw title strip for drawing sheet. - Distribute space for drawing sheet. - Draw line plan of two roomed quarter as per given data. - Draw detailed plan of two roomed quarter. - Draw Elevation of two roomed quarter. - Draw Section of two roomed quarter passing through opening. - Perform dimensioning, lettering - Indicate North direction. |
| - Prepare drawings for C-type residence. | - Observes local standards for basics of drawing. - Draw title strip on drawing sheet. - Distribute space of drawing sheet. - Draw line plan of C-type residence as per given data. - Draw detailed plan of C-type residence. - Draw Elevation of C-type residence. - Draw Section of C-type residence passing through opening. - Perform dimensioning, lettering - Indicate North direction. |
| - Prepare Submission drawing for a double storey residence of plot size 50‘x78’. | - Observes local standards for basics of drawing. - Draw title block on drawing sheet. - Distribute space for drawing sheet. - Draw detailed plans of double storey residence as per given data. - Draw front, side & rear elevations of double storey residence. - Draw two sections of double storey residence passing through opening. - Draw site plan, landscape plan of double storey residence. - Perform dimensioning, lettering - Prepare schedule of area, openings, iron fittings. - Indicate North direction. |
| - Prepare foundation plan for a double storey residence of plot size 50‘x78’. | - Observes local standards for basics of drawing. - Draw title block on drawing sheet. - Distribute space for drawing sheet. - Draw foundation plan of double storey residence. - Draw front, side & rear elevations of double storey residence. - Draw two sections of double storey residence passing through opening. - Draw site plan, landscape plan of double storey residence. - Perform dimensioning, lettering - Prepare schedule of area, openings, iron fittings. - Indicate North direction. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe the elements of report on residential building.

- State the different types of building drawings.

- Describe the different rooms in residential and educational building of different classes.

- Describe the different rooms in hospital and industrial building of different classes.

- Describe the different rooms in public & commercial building of different classes.

- State the sizes of rooms for different classes of residences and other buildings.

- Follow measurements from a given plan.

- Describe the plot area and covered area of public residential buildings

- Define site plan, line plan, detailed plan, layout plan, index plan, foundation plan, elevations & sections.

- Sketch plans, elevations, and sections of buildings from given line diagrams.

- Label the parts of given plan.

- Explain the procedure for preparing plans, elevations and sections for single storey and double storey buildings.

- Prepare schedule of openings in building.

- Prepare schedule of area, and iron fittings for doors, windows, almirah etc.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Report on various features after reading the drawing of an existing building.

- Drawings for  single room 14’ x 12’ having wall thickness 9”.

- Drawings for  two roomed quarter along with bath, kitchen, verandah, courtyard.

- Drawings for  C-type residence.

- Drawings Submission drawing for a double storey residence of plot size 50‘x78’.

- Defice “C” type building

- State Importance for north dierction on drawing sheet

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- .  Perform House Planning

Overview: This competency standard covers the skills and knowledge required to prepare report on orientation & ventilation of an existing building and plan & prepare drawing of c-type residence as per local regulations.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare report on orientation & ventilation of an existing building. | - Observe orientation of rooms. - Compare opening sizes as per local regulations. - Indicate ventilation for a building - Draft the report on ventilation. |
| - Plan & prepare drawing of c-type residence as per local regulations for ventilation | - Observes local standards for basics of drawing. - Prepare title block on sheet - Distribute space in required parts. - Calculate area of floors, volume of rooms as per ventilation aspects. - Draw detailed plan of residence as per regulations. - Draw Elevation of residence. - Draw Section of room passing through opening. - Prepare schedule of area, openings. - Perform dimensioning, lettering of drawings. - Indicate North direction. |
| - Plan & prepare drawing of c-type residence as per local regulations for seismic auspects. | - Observes local standards for basics of drawing. - Distribute space in required parts. - Prepare title block on sheet - Calculate area of floors, volume of rooms as per seismic aspects. - Draw detailed plan of residence as per regulations. - Draw Elevation of residence. - Draw Section of room passing through opening. - Prepare schedule of area, openings. - Perform dimensioning, lettering for drawings. - Indicate North direction. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define House planning

- State the necessity of house planning

- State the factors, which govern the selection of site for building

- Define orientation and ventilation.

- State the factors affecting the planning of a house

- State the inter-relationship of different rooms

- State the minimum area of the building openings for ventilation.

- Describe the positions of openings in building

- Describe materials for building structures

- State principles of providing building services

- Draw sketches of different sizes of plots along with location of commercial area

- State building bye laws of different national agencies i.e., CDA, LDA

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Report on orientation & ventilation of an existing building.

- Drawing of c-type residence planned as per local regulations.

- State Importance and process of Ventilation

- Importance of Seismic aspects

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- . Produce Layout Plans for Building Services.

Overview: This competency standard covers the skills and knowledge required to produce detailed plan for water supply, gas, sanitary, electricity installations in residence & circuit diagram.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare detailed plan of water supply installations for “A” class bath on A4 sheet. | - Observe local standards for basics of drawing. - Distribute space in required parts. - Draw title block for drawing sheet. - Draw detailed plan of A class bath. - Draw water supply installations. - Draw sanitary installations. - Perform dimensioning, lettering for drawings. |
| - Prepare detailed plan of sanitary installations for “A” class bath on A4 sheet. | - Observe local standards for basics of drawing. - Distribute space in required parts. - Draw title block for drawing sheet. - Draw detailed plan of A class bath. - Draw water supply installations. - Draw sanitary installations. - Perform dimensioning, lettering |
| - Prepare detailed plan of water supply, gas & sanitary installations for A class kitchen on A4 sheet. | - Observe local standards for basics of drawing. - Distribute space in required parts. - Draw title block for drawing sheet. - Draw detailed plan of A class kitchen. - Draw water supply installations. - Draw gas installations. - Draw sanitary installations. - Perform dimensioning and lettering |
| - Prepare detailed plan of water supply gas & sanitary installations for C type residence on A3 sheet. | - Observe local standards for basics of drawing. - Distribute space in required parts. - Draw title block for drawing sheet. - Draw detailed plan of C type residence. - Draw water supply installations. - Draw gas installations. - Draw sanitary installations. - Prepare schedule of installations. - Perform dimensioning and lettering - Indicate North direction on sheet. |
| - Prepare detailed plan of electrical installations & Circuit diagram for C type residence on A3 sheet. | - Observe local standards for basics of drawing. - Draw title strip for drawing sheet. - Distribute space in required parts. - Draw detailed plan of C type residence. - Draw electrical installations. - Draw Circuit diagram - Prepare schedule of installations. - Perform dimensioning and lettering - Indicate North direction on sheet. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define building services.

- Enlist water supply, gas, sanitary & electricity installations for residential building.

- Prepare schedule of different fixtures required for bath, kitchen, dining and courtyards.

- Sketch the different fixtures in kitchen and bathrooms at their proper places.

- Sketch the building services in C class residence at their proper places.

- Importance for dimensioning and lettering

- Explain schedule for installation

- Sketch for different sanitary fixtures.

- Free hand sketch for bath room fittings

- Free hand sketching for kitchen accessories

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of detailed plan for water supply, & sanitary installations of A class bath.

- Drawing of detailed plan for water supply, gas, & sanitary installations of A class kitchen.

- Drawing of detailed plan for water supply, gas &sanitary installations of C class residence.

- Drawing of detailed plan for electricity installations & circuit diagram in C class residence.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

- .  Produce Drawings of Sanitary Structures

Overview: This competency standard covers the skills and knowledge required to produce drawings of soakage pit, septic tank, manhole and sewer line.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare drawing of Soakage Pit for 25 users on A3 sheet. | - Observe local standards for basics of drawing. - Draw title strip for drawing sheet. - Distribute space in required parts. - Draw plan & section of soakage pit. - Perform dimensioning and lettering |
| - Prepare drawing of rectangular Septic Tank for 25 users on A3 sheet. | - Observe local standards for basics of drawing. - Draw title strip for drawing sheet. - Distribute space in required parts. - Draw plan & sections of Septic Tank. - Perform dimensioning and lettering |
| - Prepare drawing of rectangular Manhole on A3 sheet. | - Observe local standards for basics of drawing. - Draw title strip for drawing sheet - Distribute space in required parts. - Draw plan, long section & cross section of manhole. - Perform dimensioning and lettering. |
| - Prepare drawing of Sewer line on A4 sheet. | - Observe local standards for basics of drawing. - Distribute space in required parts. - Draw title strip for drawing sheet. - Draw cross section of sewer line. - Perform dimensioning and lettering |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define sanitary structures.

- Enlist sanitary structures- soakage pit, soak pit, septic tank, manhole and sewer line.

- Sketch & label sanitary structures- soakage pit, soak pit, septic tank, manhole & sewer line.

- State the purpose of sanitary structures.

- Differentiate between manhole and drop manhole

- Importance of sanitary structure

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of Soakage pit & Soakage pit for 25 users.

- Drawing of rectangular Septic Tank for 25 users

- Drawing of rectangular and circular manhole.

- Drawing of cross section of sewer line.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Description (Consumable) |
| --- | --- | --- | --- |
| 1 | Drawing board | 1 | Drawing sheet |
| 2 | T square | 2 | Eraser |
| 3 | Pair of Set squares | 3 | HB, H, 2H, 3H Pencils |
| 4 | Drawing Box (Compasses, divider, protractor, rule) | 4 | Scotch tape |
| 5 | Sharpener/ Sand paper | 5 | Thumb Pins |
| 6 | Handkerchief | 6 | Duster |
| 7 | French Curves |  |  |
| 8 | Set of Card board scales |  |  |

  - Building Materials

- . Identify bricks

Overview: This competency standard covers the skills and knowledge required to interpret brick is a man-made building material used to make walls and make places to walk. It is a single unit of a kneaded clay-bearing soil, sand and lime, or concrete material, fire-hardened or air-dried, used in masonry construction. Bricks are made mostly of clay. Bricks used outdoors on the ground are called "pavers".

| Competency Unit | Performance Criteria |
| --- | --- |
| CU-1. Inspect Brick Kiln. | P1. Identify constituents’ materials P2. Ensure mixing proportions P3. Inspect brick making method P4. Prepare Report. |
| CU-2. Inspect Pre-Cast block unit factory. | P1. Identify manufacturing items P2. Identify constituents’ materials P3. Ensure mixing proportions P4. Record making process |
| CU-3. Inspect a quarry / crusher. | P1. Inspect raw material. P2. Separate aggregate types P3. Record the crushing procedure. P4. Prepare Report |
| CU-4. Interpret brick manufacturing process. | P1. Identify raw materials P2. Mix raw material P3. Check ingredients. P4. Clean & fill mould P5. Remove bricks from mould P6. Dry bricks P7. Burn bricks in kiln P8. Use PPEs |
| CU-5. Test bricks. | P1. Select bricks P2. Find compressive strength of selected bricks P3. Find water absorption of selected bricks Check ingredients. P4. Check dimensional tolerance of selected bricks P5. Find efflorescence of selected bricks P6. Perform hardness test on selected bricks P7. Perform abrasion test on selected bricks P8. Check structure of selected bricks |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard.

This includes the knowledge of:

K1.  Explain Uses of different building materials.

K2.  Describe and explain the Method to prepare brick kiln

K3.  Describe essential need for aggregate

K4.  Explain the quarry and its working

K5. Explain demand of water for different building materials

K6. Explain variation in size of bricks

K7. Describe optimizing quality of crush

K8. Explain network for different building materials

K9. Explain safety precautions for laying, preparing brick & aggregate

K10. Explain types of quarry.

K11. Explain blasting method

K12. Explain manufacturing of bricks

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare layout of brick kiln

- Preparation plan to laying of brick at construction site as per standards

- Apply method to prepare a kiln

- Identify Method to carry out quarry process

- Check construction specification for bricks and aggregates.

- Apply variation for brick manufacturing

- Apply brick manufacturing process

- Apply safety precautions

- Apply sieving process at quarry

- Be able to

  - Find compressive strength of selected bricks

  - Find water absorption of selected bricks Check ingredients.

  - Check dimensional tolerance of selected bricks

  - Find efflorescence of selected bricks

  - Perform hardness test on selected bricks

  - Perform abrasion test on selected bricks

  - Check structure of selected bricks

TOOLS AND EQUIPMENT’S

- Brick Making Machine

- Brick Molds

- Hand roller

- Thermometer

- Umbrella with stand

- Spirit level

- PPEs

- . Dress stone for masonry

Overview: This competency standard covers the skills and knowledge required to perform roughly dressed stones are laid in a mortar the result is a stone rubble masonry and Ashlar Masonry. Stone masonry using dressed (cut) stones is known as ashlar masonry. Stone Veneer. Stone veneer is used as a protective and decorative covering for interior or exterior walls and surfaces.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Inspect dressing of natural stone | P1. Identify materials P2. Observe dressing process P3. Record the process |
| CU-2. Interpret Stone masonry (Rubble) | P1. Identify Materials P2. Pick Rough stones P3. Prepare cement sand mortar P4. Perform alignment P5. Place stones in layers P6. Carry out Joints filling with mortar P7. Perform safety precautions |
| CU-3. Interpret Stone masonry (Ashlar) | P1. Identify Materials P2. Sharpen edge of Rough stones P3. Prepare cement sand mortar P4. Perform alignment P5. Place stones with proper bond P6. Perform safety precautions |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1.Comprehend the purpose and application of stone masonry

K2 Knowledge for method to apply rough stone masonry

K3. Explain method to apply ashlar stone masonry

K4- Describe working of stone masonry precautions.

K5.  Explain different type of stones used in masonry

K6. Describe different jointing methods of stones

K7. Describe safety procedure for joining of stones

K8.  Explain hole filling method in rough stone masonry

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Define Joints in stone masonry.

- Draw sketch of different stone masonry work.

- Interpret different stone masonry procedures

- Calculate quantity for stone masonry

- Plan safety procedure during construction of stone masonry

- Plan the steps to joining of stones in layers as Rubble

TOOLS AND EQUIPMENT’S

- Calculator

- Thermometer

- Stone masonry instruments

- Scissor

- trowel

- Umbrella with stand

- Spirit level

- PPEs

- Hammer

- Chisels

- . Prepare slaked lime

Overview:  This competency standard covers the skills and knowledge required apply white alkaline substance consisting of calcium hydroxide, made by adding water to quicklime and used in traditional building methods to make plaster, mortar, and lime wash it Quicklime reacts with water to form slaked lime (calcium hydroxide). Slaked lime is used to reduce the acidity of lakes and soils. Lakes and soils can become acidic because of acid rain.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Prepare slacked lime | P1. Identify requisite material P2. Spread lime at ponds P3. Add water P4. Observe change in lime P5. Check strength |
| CU-2. Store quick lime | P1. Classify lime as per use P2. Identify process for storing lime P3. Prepare lime putty to store lime |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard.

This includes the knowledge of:

K1-Describe lime with its properties

K2- Explain Method to convert quick lime into slaked lime.

K3- Explain procedure to get lime from quarry.

K4. Describe design procedure for maintaining a quarry

K5. Explain safety precaution to get lime from quarry

K6. Explain PPEs

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions at quarry

- Maintain safety precautions to convert lime property

- Handling the quarry process

- Design the lime properties

- Execute and plan to change lime property

TOOLS AND EQUIPMENT’S

- Spirit level

- PPEs

- Kassie

- Thermometer

- Lime beds

- Umbrella with stand

- . Identify and store cement for construction

Overview: This competency standard covers the skills and knowledge required for basics as construction material powdery substance made by calcining lime and clay, mixed with water to form mortar or mixed with sand, gravel, and water to make concrete. Store cement in a building which is dry, leak proof and as moisture proof as possible. There should be minimum number of windows in the storage building. Stack the cement bags off the floor on wooden planks in such a way, so that it is about 150 mm to 200 mm above the floor.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Develop Flow diagram for manufacturing of cement | P1. Identify drawing instruments P2. Fix Drawing sheet on board P3. Select scale P4. Use safety precautions for drawing P5. Draw flow diagram |
| CU-2. Manage storage process for cement | P1. Identify material P2. Locate moisture proof place P3. Stacked cement bags P4. Cover with plastic sheet P5. Use PPEs |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard.

This includes the knowledge of:

K1-Describe cement storage process

K2- Explain Method to cover the cement in moisture proof place.

K3- Explain procedure to get cement from moisture proof room.

K4. Describe design procedure for maintaining a moisture proof room

K5. Explain safety precaution to get and use of cement from moisture proof room

K6. Explain PPEs

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions for cement storage

- Handling the storage process

- Design cement storage process at different sites

- Execute and plan to change cement into different categories

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Measuring scale,

- Drawing sheet,

- PPEs

- Thermometer

- Umbrella with stand

- Plastic sheets

- . Select sand for construction

Overview: This competency standard covers the skills and knowledge required for basics as construction granular material composed of finely divided rock and mineral particles. It is defined by size, being finer than gravel and coarser than silt. Desert sand, although plentiful, is not suitable for concrete, and 50 billion tons of beach sand and fossil sand is needed each year for construction.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Test sand for use | P1. Select sand from heap P2. Find salts in sand P3. Determine bulking of sand P4. Find clay content in selected sand P5. Find size of sand grains. |
| CU-2. Manage storage process of sand at site | P1. Locate safe place for storage of sand P2. Dump sand at located place P3. Cover sand with plastic sheets P4. Use PPEs |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard.

This includes the knowledge of:

K1-Describe sand storage process

K2- Explain Method to cover the sand and water sprinkling on sand.

K3- Explain procedure to get sand from quarry

K4. Describe design procedure for maintaining quarry for sand

K5. Explain safety precaution to get and use of sand at construction site

K6. Explain PPEs

K7. Describe type of sand ingredients.

K8. Explain sieving of sand

K9. Explain working of sand in concrete

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions at sand quarrying site

- Design the sand ingredients size

- Execute and plan to change concrete ratio with sand

- Manage and Plan different type of sand at site

- Handling storage of sand at different sites

TOOLS AND EQUIPMENT’S

- Weighing Balance

- Measuring Cylinders

- Plastic Sheets

- Sieve Analysis

- PPE’s

- . Identify ferrous and non-ferrous metals

Overview: This competency standard covers the skills and knowledge required for Non-Ferrous Metals do not contain Iron, are not magnetic and are usually more resistant to corrosion than ferrous metals. Some examples of Non-Ferrous Metals we deal which are Aluminum, Aluminum Alloys and Copper.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Identify ferrous metals | P1. Identify materials P2. Select ferrous metals P3. Check iron particles P4. Perform magnetic operation P5. Use PPEs P6. Perform results |
| CU-2. Identify Non-ferrous metals | P1. Identify materials P2. Select nonferrous metals P3. Check iron particles P4. Perform magnetic operation P5. Use PPEs P6. Perform results |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1-Describe ferrous and nonferrous metals

K2- Explain Method to perform strength for metals.

K3- Explain different alloys

K4. Explain PPEs

K5. Explain types of nonferrous metal

K6. Explain types of ferrous metal

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions for ferrous metal

- Maintain safety precautions for non-ferrous metal

- Handling of ferrous and non-ferrous metals

TOOLS AND EQUIPMENT’S

  - Ferrous Metal

  - Non-ferrous Metal

  - Magnets

  - PPEs

  - Thermometer

- . Identify advanced construction materials

Overview: This competency standard covers the skills and knowledge required Traditional construction materials, such as timber, steel, asphalt and Portland cement concrete are often used in many construction projects. Modern materials, such as. Polymers and composites are making headway into the construction industry.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Identify Timber | P1. Identify materials P2. Select color for wood P3. Count annual rings P4. Check pith & rays P5. Select bark & sap wood P6. Perform results |
| CU-2. Identify steel | P1. Identify materials - P2. Perform Tensile Strength. - P3. Perform Ductility. - P4. Check Malleability. - P5. Monitor Durability. - P6. Identify Conductivity. - P7. Check Luster P8. Use PPEs |
| CU-3. Identify asphalt | P1. Identify materials - P2. Perform Brittleness - P3. Perform Ductility - P4. Check Hardening - P5. Monitor Viscosity - P6. Identify Conductivity - P7. Check Luster P8. Use PPEs |
| CU-4. Identify cement | - P1. Indicate material - P2. Check Fineness - P3. Administer Soundness. - P4. Monitor Consistency. - P5. Check Strength. - P6. Supervise Setting time. - P7. Check Heat of hydration. - P8. Study Loss of ignition. - P9. Perform Bulk density. P10.Use PPES |
| CU-5. Identify glass | P1. Identify material P2. Check transparency, P3. Perform heat resistance, P4. Check pressure P5. Indicate breakage resistance P6. Check chemical resistance. P7. Use PPEs |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1-Describe different construction material

K2- Explain Method to perform strength for different materials

K3- Explain different material properties

K4. Describe design procedure for manufacturing different materials

K5. Explain safety precaution to manufacturing and finishing of materials

K6. Explain PPEs

K7. Explain type of nonferrous metals

K9. Explain safe working process to cut the metals and timber

K10. Explain temperature zone for cutting different materials

K11.  Explain Annealing process for metals

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Maintain safety precautions for cutting of metals

- Maintain safety precautions while alloys process

- Handling the alloy process at plant

- Design the alloy for better performance

- Execute and plan to change the heat while alloying

- Manage and Plan different type alloy processing at plant

- Manage to cut the timber

- Plan Designing in timber process

- . Work Safely in Construction Industry

Overview: This competency standard covers the skills and knowledge required to identify OHS legislative requirements, identify construction hazards and control measures, identify OHS communication and reporting processes and identify OHS incident response procedures.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Identify OHS legislative requirements. | P1. Identify and explain applicable OHS legislative requirements relevant to own work, role and responsibilities. P2. Identify duty of care requirements. P3. Identify and explain own responsibilities to comply with safe work practices |
| - Identify construction hazards and control measures. | P1. Identify basic principles of risk management. P2. Identify & discuss Common construction hazards. P3. Identify measures for controlling hazards and risks. |
| - Identify OHS communication and reporting processes. | P1. Identify & discuss OHS communication processes, information and documentation. P2. Identify & explain role of designated OHS personnel. P3. Identify & explain Safety signs and symbols. P4. Identify & discuss Procedures and relevant authorities for reporting hazards, incidents and injuries are identified. |
| - Identify OHS incident response procedures. | P1. Identify & explain General procedures for responding to incidents and emergencies. P2. Identify procedures for accessing first aid. P3. Identify & demonstrate requirements for the selection and use of relevant personal protective equipment. P4. Identify & discuss Fire safety equipment. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe applicable Commonwealth, State or Territory OHS legislation, regulations, standards, codes of practice and industry standards/guidance notes relevant to own work, role and responsibilities

- Describe basic principles of risk management and assessment for construction work.

- Describe common construction hazards, common construction safety signage and its meanings

- Describe general construction emergency response and evacuation procedures.

- Describe general construction work activities that require licenses, tickets or certificates of competency.

- Describe general first aid response requirements and general procedures for raising OHS issues.

- Explain general procedures for reporting OHS hazards, accidents, incidents, emergencies, injuries, near misses and dangerous occurrences.

- Describe general procedures for responding to hazards, incidents and injuries.

- Describe general workers' compensation and injury management requirements.

- Describe OHS hierarchy of controls, OHS responsibilities and rights of duty holders.

- Describe persons in control of construction work/projects- employers and self-employed persons, supervisors, employees, designers, inspectors and manufacturers and suppliers

- Describe own responsibilities to comply with safe work practices relating to: housekeeping, identification of hazards, preventing bullying or harassment, smoking, use of amenities, use of drugs and alcohol

- Describe role of OHS committees and representatives

- Describe types of common personal protective equipment and fire safety equipment

- Explain types of OHS information and documentation.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Applicable OHS legislative and safety requirements for construction work including duty of care

- The range of common construction hazards and procedures for the assessment of risk and application of the hierarchy of control

- OHS communication processes, information and documentation including the role of OHS committees and representatives, the meaning of common safety signs and symbols, and procedures for reporting hazards, incidents and injuries

- General procedures for responding to incidents and emergencies including evacuation, first aid, fire safety equipment and PPE.

TOOLS AND EQUIPMENT’S

Personal protective equipment for construction industry.

- . Interpret Simple Building Plans

Overview: This competency standard covers the skills and knowledge required to locate, interpret and apply relevant information standards of simple building plans,

| Competency Unit | Performance Criteria |
| --- | --- |
| - Identify types of drawings and their functions. | - Identify the Main types of plans and drawings used in the construction sector of the industry. - Identify Key features and functions of each type of drawing. - Recognize the Quality requirements of company operations. - Identify Environmental requirements and controls from job plans, specifications and environmental plan. |
| - Recognize amendments. | - Check the Title panel of project documentation to verify latest amendments to drawing. - Check the amendments to specifications to ensure currency of information and convey to others where appropriate. |
| - Recognize commonly used symbols and abbreviations. | - Recognize the Construction symbols and abbreviations. - Locate legend on project drawings, and interpret correctly symbols & abbreviations |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Basic calculations of heights, areas, volumes and grades

- Knowledge of Commonly used construction symbols and abbreviations

- Define different Construction terminology

- Knowledge of Drawing conventions

- Knowledge for features of plans and elevations, including direction, scale, key, contours, symbols and abbreviations

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Locate, interpret and apply relevant information, standards and specifications

- Comply with site safety plan, OHS regulations and state and territory legislation applicable to workplace operations

- Communicate and work effectively and safely with others

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Calculator |
| 2 | Ruler |
| 3 | Pencil |

- . Read and Interpret plans and Specifications

Overview: This competency standard covers the skills and knowledge required to read, interpret plans and specifications.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Identify types of drawings and their functions. | - Identify main types of plans and drawings used in the construction sector - Identify key features and functions of each type of drawing - Recognize quality requirements of company operations - Identify environmental requirements and controls from job plans, specifications and environmental plan. |
| - Recognize amendments. | - Check title panel of project documentation - Check to ensure amendments to specifications. |
| - Recognize commonly used symbols and abbreviations. | - Recognize construction symbols and abbreviations. - Interpret legend located on project drawings, symbols and abbreviations |
| - Locate and identify key features on a site plan. | - Position plan according to site. - Identify key features of the site. - Access to site and identify services, main features, contours and datum |
| - Identify project requirements. | - Identify dimensions of project on locations. - Identify construction types and dimensions for nominated locations. - Check Environmental control. - Identify location, dimensions and tolerances for ancillary works. |
| - Read and interpret job specifications. | - Identify job specifications from drawings, notes and descriptions. - Identify standards of work, finishes and tolerances from project specifications. - Identify material attributes from specifications. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Basic calculations of heights, areas, volumes and grades

- Knowledge of commonly used construction symbols and abbreviations

- Describe different Construction terminology

- Knowledge of drawing conventions

- Knowledge for features of plans and elevations, including direction, scale, key, contours, symbols and abbreviations

- Develop job safety analysis (JSA) and safe work method statements

- Knowledge for key features of formal job specifications

- Describe processes for application of scales in plan preparation and interpretation

- Define project quality requirements

- Explain site and equipment safety (OHS) requirements

- Describe techniques for orienting/confirming the orientation of a plan.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Locate, interpret and apply relevant information, standards and specifications

- Comply with site safety plan, OHS regulations and state and territory legislation applicable to workplace operations

- Communicate and work effectively and safely with others for a minimum of two different projects, read and interpret the project plans, including:

- Confirmation of amendment status and drawings confirmed 'for construction'

- Orientation of plans to the ground

- Six key features on both the plan and the site

- Confirmation of six items of information from the title block of the project plans

- Six construction dimensions, levels and locations from the project plans

- Six ancillary works dimensions, levels and locations from the project plans for a minimum of two formal specifications, identify the dimensions, material requirements and processes to be followed.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Calculator |
| 2 | Ruler |
| 3 | Pencil |

- . Apply power and hand tools in construction.

Overview: This competency standard covers the skills and knowledge required to identify and select hand, power and pneumatic tools for given tasks and safely use and maintain a minimum of rule, tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock and crow bar and pinch bar for given tasks.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. |
| - Identify and select hand, power and pneumatic tools | - Identify and select hand tools, power and pneumatic tools. - Apply safety requirements when using hand, power and pneumatic tools. - Observe pre-operational checks, including lubricants, hydraulic fluid and water. |
| - Use tools safely and effectively | - Use hand tools appropriate to the task - Use power and pneumatic tools are safely and effectively in accordance with manufacturer recommendations. - Sharpen and maintain tools. |
| - Identify, select and use plant and equipment | - Select and use plant and equipment consistent with OHS requirements respective to job. - Observe pre-operational checks, including lubricants, hydraulic fluid and water. |
| - Clean up work place. | - Clean work area and dispose of waste in accordance with work specifications. - Clean, maintain and store tools plant and equipment in accordance with workplace practices. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain need for store in a project

- State the classification of stores- stock, tools and plants, Road metal and materials charged direct to the work.

- Knowledge of Construction terminology, particularly names of tools and equipment

- Describe Manual handling and correct posture for tool and equipment use

- Define use of Personal protective equipment

- Explain Safety techniques for tool and equipment use

- Describe safety requirements for workplace and equipment

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Identify and select hand, power and pneumatic tools for given tasks.

- Safely use and maintain a minimum of rule, tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock and crow bar and pinch bar for given tasks.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Hand tools |
| 2 | Power tools |
| 3 | Pneumatic tools |
| 4 | PPE’s |

  - Workshop Practice-I (wood working)

- . Carry out OH & S requirement in workshop

Overview: This competency standard covers the skills and knowledge required to assess risks, plan and prepare for safe work practices, maintain safety of self and others, apply emergency response and cleanup work site area.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Assess risks | P1. Identify hazards in the work area P2. Identify OHS issues and risks in the work area P3. Prepare reports on OHS, hazard, accident or incident |
| CU2. Plan and prepare for safe work practices | P1. Select personal protective equipment (PPE) P2. Select tools and equipment consistent with safe work practice requirements P3. Determine and erect Required barricades, hoardings and signage P4. Identify and apply Material Safety Data Sheets (MSDS) |
| CU3. Maintain safety of self and others | P1. Identify safety signs in terms of color and shape, symbols and alarms P2. Identify, handle and store hazardous chemicals and materials maintaining the safety to self, others and the environment P3. Identify common causes of accidents in the industry P4. Implement prevention measures for accidents |
| CU4. Apply emergency response | P1. Identify area requiring emergencies P2. Provide emergency response in accordance to requirements P3. Prepare report detail of actions taken |
| CU5. Clean up work site area | P1. Clear workplace by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to assess risks, plan and prepare for safe work practices, maintain safety of self and others, apply emergency response and cleanup work site area required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define work shop orientation

- Enlist safety precautions

- Enlist PPE’s

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Explain uses of PPE’s

- Knowledge of workshop cleanliness

- Knowledge of proper storage of tools and equipment

- Observe causes of accidents in industries

Tools and Equipment

- Ear Muff, Ear Plug

- Safety shoes

- Gloves

- Safety Goggles

- Safety Helmet

- Respiratory protective equipment i.e. Masks

- Life Jacket

- High visibility jacket

- Safety harness

- Face shield

- Rubber shoes/ long shoes

- . Determine properties and types of locally manufactured timber

Overview: This competency standard covers the skills and knowledge required to identify type of timber, determine specific gravity of timber, find out moisture content of timber and determine compressive strength of timber

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify type of timber | P1. Recognize color and odor of locally produced timber P2. Prepare a list of locally produced timber in accordance with color and odor P3. Identify type of given timber sample |
| CU2. Determine specific gravity of timber | P1. Identify tools and equipment P2. Measure dimensions of sample P3. Weigh the sample P4. Calculate specific gravity of timber by computing density |
| CU3. Find out moisture content of timber | P1. Identify tools and equipment P2. Weigh sample before oven drying P3. Weigh sample after drying sample P4. Calculate moisture content |
| CU4. Determine compressive strength of timber | P1. Identify tools and equipment P2. Assemble sample in equipment by placing sample perpendicular to grain P3. Find out compressive strength according to specification P4. Repeat process by placing sample parallel to grain P5. Interpret compressive strength |

Knowledge & Understanding

The candidate must be able to develop skills and understanding identify type of timber, determine specific gravity of timber, find out moisture content of timber and determine compressive strength of timber required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define wood

- Explain Constructional wood (timber) with sketch

- Explain structure and growth of timber.

- Explain wood types and uses of various wooden products

- Perform felling and conversion of timber.

- Enlist the defects and decay of timber.

- Explain characteristics of timber

- Draw section of timber

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare report on specific gravity of timber

- Prepare report on moisture content of timber

- Prepare report on compressive strength of timber

Tools and Equipment

- Compression testing machine

- Digital weighing balance

- Laboratory oven

- . Sharpen carpentry tools

Overview: This competency standard covers the skills and knowledge required to identify tools used in carpentry and perform sharpening of carpentry tools

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify tools used in carpentry | P1. Recognize tools used in carpentry P2. Identify tools needed to be sharpen |
| CU2. Perform sharpening of carpentry tools | P1. Identify tools and equipment P2. Perform plane iron sharpening P3. Perform chisel sharpening P4. Perform scraper sharpening P5. Perform router bit sharpening P6. Perform craving tool sharpening |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to identify tools used in carpentry and perform sharpening of carpentry tools required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Enlist wood tools - Enlist wood measuring tools - Enlist wood marking tools - Enlist wood holding tool - Enlist cutting tool - Explain the working of different tools

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Explain sharpening of tools

- Explain OHS requirement for tools sharpening

Tools and Equipment

- Files

- Diamond stones

- Water stones

- Oil stones

- Ceramic stones

- . Apply hand tools

Overview: This competency standard covers the skills and knowledge required to identify and select hand tools, Identify the tools and cleanup work site area.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify and select hand tools | - Identify and select hand , power and pneumatic tools , their functions, operations and limitations - Recognize OHS requirements for using hand tools |
| CU2. Identify the tools | - Use appropriate hand tools to the task and materials in accordance with OHS requirements - Sharpen and maintain tools |
| CU3. Clean up the work site area | - Clear work area and dispose materials (reused or recycled) in accordance with legislation, regulations and codes of practice and job specification. - Clean, check, maintain and store tools according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to assess risks, plan and prepare for safe work practices, maintain safety of self and others, apply emergency response and cleanup work site area required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Enlist wood tools

- Enlist wood measuring tools

- Explain wood marking tools

- Enlist wood holding tool

- Enlist cutting tool

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Explain use of tools

- Check compliance to OHS requirement

Tools and Equipment

- Wood tool

- Measuring tool

- Marking tools

- Holding tool

- Cutting tool

- Planning tool

- . Perform wooden joinery work

Overview: This competency standard covers the skills and knowledge required to make butt, biscuit, miter, edge, dovetail, mortise and tenon and dado joint.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Make butt joints | - Identify butt joint - Cut timber according to requirement of butt joint - Join cut pieces according to requirement |
| CU2. Perform Biscuit joinery | - Identify biscuit joint - Cut timber according to requirement of biscuit joint - Join cut pieces according to requirement |
| CU3. Make Miter joints | - Identify Miter joints - Cut timber according to requirement of Miter joints - Join cut pieces according to requirement |
| CU4. Make Edge joint | - Identify Edge joint - Cut timber according to requirement of Edge joint - Join cut pieces according to requirement |
| CU5. Make Dovetail joint | - Identify Dovetail joint - Cut timber according to requirement of Dovetail joint - Join cut pieces according to requirement |
| CU6. Make Mortise and tenon joint | - Identify Mortise and tenon joint - Cut timber according to requirement of Mortise and tenon joint - Join cut pieces according to requirement |
| CU7. Make Dado joint | - Identify Dado joint - Cut timber according to requirement of Dado joint - Join cut pieces according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding make butt, biscuit, miter, edge, dovetail, mortise and tenon and dado joint required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain types of wood joints

- Describe the uses of wood joints

- Define Butt joint

- Define Biscuit joint

- Define Miter joint

- Define Edge joint

- Define Dovetail joint

- Define Mortise and tenon joint

- Define Dado joint

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify joint types

- Knowledge of cutting of wood

- Explain procedure for joining of wood

Tools and Equipment

- Wood tool

- Measuring tool

- Marking tools

- Holding tool

- Cutting tool

- Planning tool

- . Apply fastenings

Overview: This competency standard covers the skills and knowledge required to prepare hole, identify and apply fastenings.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare hole | - Identify and select power and pneumatic tools for making hole - Perform boring to make hole in accordance with OHS requirement |
| CU2. Identify fastenings | - Identify and select appropriate fastening - Identify and select tools for fastenings |
| CU3. Apply fastenings | - Insert fastening in hole prepared before - Use tools for fastenings |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to prepare hole, Identify fastenings and apply fastenings required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define types of fastenings

- Enlist fastening tools

- Explain the standard procedures for fastening

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Explain use of tools for fastening

- Selection of appropriate fastening

- Compliance to OHS requirement

Tools and Equipment

- Fastening tools

- Boring machine

- Fastenings

- . Apply traditional spirits polishing techniques

Overview: This competency standard covers the skills and knowledge required to plan and prepare for work, prepare for application of polish, perform skin in job surface, perform body up on skinned in surface, apply spirit on surface and clean up.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Plan and prepare for work | - Identify work requirements correctly from drawings/specifications/instructions - Recognize quality assurance requirements - Observe OHS requirements, including personal protection needs throughout the work - Select tools and equipment consistent with job requirements and work instructions - Prepare skinning in, bodying up and spiriting out/stiffening up rubbers consistent with the requirements of the work |
| CU2. Prepare for application of polish | - Check surfaces for contamination and correct preparation according to workplace procedures and standards - Identify products with surface or other faults - Identify techniques for hand application of polish using traditional methods - Prepare materials and equipment suitable for skinning in, bodying up and spiriting out/stiffening up operations are - Stain, fill in and seal surfaces with polish using a mop brush and blender. |
| CU3. Perform skin in job surface | - Perform sand surface lightly - Perform skinning in rubber and apply polish for job color and clarity - Perform skin in surface covering small areas at a time, using correct amount of pressure and suitable rubber movements - Apply pumice powder as necessary in accordance with workplace procedure. - Inspect skinned in surface |
| CU4. Perform body up on skinned in surface | - Use polish, lubricating oil and pumice powder is used to body up the skinned in surface, using correct amount of pressure and suitable rubber movements - Monitor and evaluate rubber and job surface continuously - Recharge body up rubber and lubricating oil - Apply pumice as necessary to ensure surface faults and blemishes do not develop |
| CU5. Apply spirit on surface | - Apply spirit on surface using correct amount of pressure and suitable rubber movements - Observe blemishes or rubber marks |
| CU6. Clean up | - Clear workplace of debris and unused materials - Store rubbers separately in airtight glass or plastic containers in accordance with workplace procedures - Clean, maintain and store tools and equipment |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to Plan and prepare for work, prepare for application of polish, Skin in job surface, Body up a skinned in surface, Spirit off/stiffen up surface and clean up required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain uses of glue

- Enlist types of fastener

- Explain procedure for sprit polish

- Explain use of polish, lubricating oil and pumice powder

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Explain uses of tools

- Procedure to apply polish on object

- Compliance to OHS requirement

Tools and Equipment

- Lac

- Polishes

- Paraffin and linseed oils

- Methylated spirits

- Rubbers

- Sprayer

- Glass

- Mop brush

- Blender

- . Apply portable power tools

Overview: This competency standard covers the skills and knowledge required to identify and select power and pneumatic tools, use of tools and cleanup work site area.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify and select power and pneumatic tools | - Identify and select power and pneumatic tools, their functions, operations and limitations - Recognize OHS requirements for using power and pneumatic tools - Check lubricants, hydraulic fluid and water according to manufacturer recommendations |
| CU2. Use of tools | - Use hand tools appropriate to the task and materials in accordance with OHS requirements - Use of power and pneumatic tools safely and effectively in accordance with manufacturer recommendations and state or territory OHS requirements - Sharpen and maintain tools |
| CU3. Clean up work site area. | - Clear work area and dispose materials (reused or recycled) in accordance with legislation, regulations and codes of practice and job specification. - Clean, check, maintain and store plant, tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to assess risks, plan and prepare for safe work practices, maintain safety of self and others, apply emergency response and cleanup work site area required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Introduction of tools

- Define Circular SAW

- Explain Wood Work Planer

- Explain Wood Turning lathe, Spindle, Boring, Sander

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Procedure for use of tools

- Explain use of power and pneumatic tools

- Compliance to OHS requirement

Tools and Equipment

- Circular SAW

- Wood Planer

- Wood Turning lathe

- Spindle

- Boring machine

- Sander

- . Make simple calculations of timber

Overview: This competency standard covers the skills and knowledge required to prepare a drawing of product and estimate timber quantity.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare drawing of given specimen | - Identify drawing tools - Perform basics of drawing - Draw plan, cross section and elevation of specimen |
| CU2. Estimate timber quantity | - Identify components of specimen - Recognize timber requirement for specific components of timber - Calculate quantities of timber |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to prepare a drawing of product and estimate timber quantity required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain measurement of wood square and board

- Draw plan, cross section and elevation of specimen

- Explain measurement of Log

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of tools

- Calculate quantities of timber

- Compliance to OHS requirement

Tools and Equipment

- Drafting table

- Drafting tools

- Drawing sheet

- Pencils

- Eraser

- Drawing board

- Sharpener

- . Construct a basic timber product

Overview: This competency standard covers the skills and knowledge required to estimate timber quantity, prepare timber for joinery work, apply fastenings and apply polish.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Estimate timber quantity | - Prepare a drawing of product - Identify components of product - Recognize timber requirement for specific components of product - Calculate quantities of timber |
| CU2. Prepare timber for joinery work | - Cut timber according to drawing/specifications - Perform necessary work for joints |
| CU3. Apply fastenings | - Prepare holes on required product - Apply fastenings |
| CU4. Apply polish | - Prepare surface for polishing - Apply polish on job surface - Apply Spirit on surface |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to construct a basic timber product required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain planning for product construction

- Describe procedure of selection of timber

- Describe procedure for cutting of timber

- Define joinery work

- Explain fastenings

- Explain procedure for polishing

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Explain use of tools

- Calculate quantities of timber

- Identify components of given product

- Compliance to OHS requirement

Tools and Equipment

- Drafting table

- Drafting tools

- Drawing sheet

- Pencils

- Eraser

- Drawing board

  - Workshop Practice-II (Electrical wiring)

- . Carry out OH & S requirement in workshop

Overview: This competency standard covers the skills and knowledge required to assess risks, plan and prepare for safe work practices, maintain safety of self and others, apply emergency response and cleanup work site area.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Assess risks | P1. Identify hazards in the work area P2. Identify OHS issues and risks in the work area P3. Complete reports on OHS, hazard, accident or incident |
| CU2. Plan and prepare for safe work practices | P1. Select personal protective equipment (PPE) P2. Select tools and equipment consistent with safe work practice requirements P3. Determine and erect Required barricades, hoardings and signage P4. Identify and apply Material safety data sheets (MSDS) |
| CU3. Maintain safety of self and others | P1. Identify safety signs in terms of colour and shape, symbols and alarms P2. Identify, handle and store hazardous chemicals and materials maintaining the safety to self, others and the environment P3. Identify common causes of accidents in the industry P4. Implement prevention measures for accidents |
| CU4. Apply emergency response | P1. Identify emergencies P2. Provide emergency response in accordance to requirements P3. Report detail of actions taken |
| CU5. Clean up work site area | P1. Clear work are by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to assess risks, plan and prepare for safe work practices, maintain safety of self and others, apply emergency response and cleanup work site area required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1. Define work shop orientation

K2. Enlist safety precautions

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of PPE

- Workshop cleanliness

- Proper storage of tools and equipment

Tools and Equipment

- Ear Muff, Ear Plug

- Safety shoes

- Gloves

- Safety Goggles

- Safety Helmet

- Respiratory protective equipment i.e. Masks

- Life Jacket

- High visibility jacket

- Safety harness

- Face shield

- Rubber shoes/ long shoes

- . Maintain safety

Overview: This competency standard covers the skills and knowledge required to use electricity safely and treat against electric shock

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Use electricity safely | P1. Determine safest supply and route for electrical supply P2. Support and place leads in accordance with regulations. P3. Conduct check on power board visually P4. Check leads and equipment for tags and visual damage P5. identify and report electrical hazards |
| CU2. Treat against electric shock | P1. Turn off the source of electricity, if possible. If not, move the source away from you and the person, using a dry, non-conducting object made of cardboard, plastic or wood P2. Begin CPR if the person shows no signs of circulation, such as breathing, coughing or movement P3. Try to prevent the injured person from becoming chilled P4. Apply a bandage. Cover any burned areas with a sterile gauze bandage, if available, or a clean cloth. Don't use a blanket or towel, because loose fibers can stick to the burns. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to use electricity safely and treat against electric shock required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Safety precautions for electricity

- Treatment against electric shock

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of PPE

- Workshop cleanliness

- Proper storage of tools and equipment

Tools and Equipment

- Ear Muff, Ear Plug

- Safety shoes

- Gloves

- Safety Goggles

- Safety Helmet

- Respiratory protective equipment i.e. Masks

- Life Jacket

- High visibility jacket

- Safety harness

- Face shield

- Rubber shoes/ long shoes

- . Perform electrical wiring

Overview: This competency standard covers the skills and knowledge required to Interpret Drawing, Arrange Resources for electrical wiring, Perform Wiring (Domestic & Industrial), termination and tagging, Perform Wire Dressing, and Perform Checking & Testing of electrical wiring.

| Competency Units | Performance Criteria |
| --- | --- |
| - Interpret Drawing | P1. Differentiate between control and power wiring as per job requirement P2. Perform preliminary marking before termination of cables P3. Follow drawing to perform wiring and for reporting/record purpose |
| - Arrange Resources for electrical wiring | P1. Enlist and arrange tools and material as per job P2. Arrange work permit for the wiring task P3. Arrange backup resources for lighting, power and safety purposes as per job requirement |
| - Perform Wiring (Domestic & Industrial), termination and tagging | P1. Perform Wiring using following methods: • Concealed • Conduit • Busway • Open • Duct P2. Follow procedure to remove insulation and to attach thimbles P3. Perform different types of terminations including • High tension (HT) • Low tension (LT) • Control shrouding • Sleeves P4. Perform different types of joints • High tension • Low tension • Water proof/underground • Open P5. Attach tags or numbers to wires for identification P6. Use personal protective equipment as per job requirement |
| - Perform Wire Dressing | P1. Maintain distance between power, communication and control wires according to job requirements P2. perform alignment of cables as per job requirement |
| - Perform Checking & Testing of electrical wiring | P1. Check continuity using line of terminal P2. Perform test to identify open/close circuits P3. Perform live test for verification of wiring P4. Verify proper grounding / Panel Earthing (PE) on designated locations |

Knowledge & Understanding

The candidate must be able to develop skills and understanding o Interpret Drawing, Arrange Resources for electrical wiring, Perform Wiring (Domestic & Industrial), termination and tagging, Perform Wire Dressing, Perform Checking & Testing of electrical wiring required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain electrical symbols

- Describe different types of drawings (e.g. power, control, single line etc.)

- Describe how to prepare drawing if not available

- Describe types of breakers, contactors, relays etc.

- Describe standard tools used in wiring

- Describe usage of back up resources including Emergency lights, ladder, safety belts etc

- Explain different types of wiring techniques including: • Concealed • Conduit • Busway• Open • Duct

- Describe purpose of color coding and tagging during wiring K4. Describe types and size of thimbles and use of thimble puncher

- Explain Termination techniques for HT, LT, Control shrouding, sleeves K6.describe Jointing techniques for HT, LT

- Describe purpose of distancing

- Describe purpose of cable alignment

- Describe Importance of terminals

- Describe open, close circuits and testing procedures

- Describe Importance of controlled live test

- Describe Importance of grounding

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of PPE

- Use of tools

- Proper storage of tools and equipment

Tools and Equipment

- Electrical drawing

- Computer

- Emergency light

- Ladder

- drill machine tool kit

- Thimble puncher

- Shroud

- PVC tape

- HT tape

- Thimbles

- cable knife

- wire stripper

- drill machine

- PPE

- Cable tie

- Clamps

- Plier

- Cutter

- spiral/ flexible pipes

- tie base

- ravel bolt

- ravel plug

- hilty bolt

- wooden screw

- AVO meter

- test lamp

- earth tester

- line tester

- megger

- . Install electrical system

Overview: This competency standard covers the skills and knowledge required to Interpret Drawing, Arrange Resources for electrical wiring, Perform Wiring (Domestic & Industrial), termination and tagging, Perform Wire Dressing, and Perform Checking & Testing of electrical wiring.

| Competency Units | Performance Criteria |
| --- | --- |
| - Interpret Electrical layout | P1. Interpret layout of the job for installations P2. Read/interpret electrical drawing for electrical wirings P3. Follow drawing to connect components of equipment |
| - Arrange Resources (e.g. Power Supply, Tools & Equipment) for electrical equipment installation/dismantling | P1. Enlist and arrange tools and material as per job P2. Arrange work permit for the wiring task P3. Arrange backup resources for lighting, power and safety purposes as per job requirement |
| - Perform Electrical Panel Installation | P1. Extract information from layout regarding panel installations P2. Place the panel according to layout P3. Connect input and output of cables in relevant panels as per electrical layouts |
| - Perform Cable Installation | P1. Interpret layouts for cable route P2. Handle cable for laying with the help of stacker P3. Tag both ends of cables for cable tracing P4. Lay cables in cable trays according to route plan P5. Perform testing of cables e.g. insulation, continuity etc |
| - Perform Cable Dismantling | P1. Interpret layouts for cable route P2. Disconnect electric supply from the job P3. Handle cable for laying with the help of stacker P4. Tag both ends of cables for cable tracing P5. Remove cables in cable trays according to route plan |
| - Perform Electrical Appliances Installation | P1. Interpret vendor’s user manual for electrical installations P2. Coordinate with concerned departments before installations for feedback P3. Connect cables with machines as per operation manual P4.Verify the connections with respect to color coding/tagging/numbering |
| - Perform Electrical equipment Dismantling | P1. Extract information from layouts regarding electrical equipment dismantling P2. Disconnect input/output cables of relevant equipment P3. Remove equipment from location as per SOPs P4. Handle removed equipment as per manufacturers’ instructions P5. Place tags on dismantled items as per SOPs P6. Store the dismantled equipment at designated place |
| - Perform Earthing | P1. Perform earthing of cables according to calculated load P2. Perform earthing of electrical appliances according to calculated load P3. Perform earth resistance test as per standards |
| - Provide Power Supply to machine | P1. Interpret electrical drawings for power supply distribution P2. Perform power distribution according to drawings P3. Perform verification of electrical supply as per operations manual P4. Perform communication with all concerned persons for electrical appliance dry-run |

Knowledge & Understanding

The candidate must be able to develop skills and understanding o Interpret Drawing, Arrange Resources for electrical wiring, Perform Wiring (Domestic & Industrial), termination and tagging, Perform Wire Dressing, Perform Checking & Testing of electrical wiring required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define electrical symbols to be used in drawings

- Describe types of drawings (civil, electrical and communication) for the job

- Explain connection scheme of the job

- Explain all types of drawings (civil, electrical and communication)

- Explain the hierarchy of the organization to rectify the issues (if occurred)

- Describe the quality standards of original products

- Describe the quantity of products against purchase requisition

- Define electrical symbols to be used in drawings

- Describe types of drawings (civil, electrical) for machine

- Explain handling techniques for placement

- Describe all the types of drawings (civil and electrical)

- Describe the safe handling techniques of cable routing

- Explain the different methods of cable testing

- Describe different the types of drawings e.g. civil and electrical etc.

- Describe the safe handling techniques of cable routing

- Describe methods of installing the electrical appliances

- Describe basic knowledge of electrical appliances (A/C, water dispensers, microwave oven etc)

- Describe the different types of coding procedures (e.g. colour coding/tagging/numbering)

- Describe different the types of drawings e.g. civil and electrical etc.

- Enlist appropriate positions of tags for visual controls

- Explain earthing and testing procedures

- Describe working principle of earth tester

- Explain Describe different the types of drawings e.g. civil and electrical etc.

- Explain verification parameters according to standards

- Explain the importance of communication with relevant persons

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of PPE

- Use of tools

- Proper storage of tools and equipment

Tools and Equipment

Drawings, operations manuals

Layouts, ICT, measuring tools (calipers, cable chart, hand hacksaw, measuring tape, laser gun for length), quality standards, vendor certificate, grinding cutter, drill machines, hammer

Layouts, lifters, chain-pulley, jacks, sprit level

Layouts, measuring tools, cutting tools like wire cutter, side cutter, cable cutter, thimble presser, megger, stacker, tags, glands, shrouds, conduits, cable tray etc

Layouts, cutting tools e.g. wire cutter, side cutter, cable cutter, stacker, tags etc.

Drawing, layout, manual, fork lifter, drill machine, PPE

Layouts, lifters, chain-pulley, jacks, tags

Soldering, earth tester, thimble presser,

Drawings, multi meters,

- . Construct circuits

Overview: This competency standard covers the skills and knowledge required to construct parallel circuit, series circuit and tunnel circuit.

| Competency Units | Performance Criteria |
| --- | --- |
| C1. Construct parallel circuit for two lamps | - Observe OHS requirements - Identify tools and equipment - Draw diagram of parallel circuits for two lamps. - Perform preliminary marking before termination of cables. - Follow drawing to perform wiring and for reporting/ record purpose. |
| C2. Construct series circuit for two lamps. | - Observe OHS requirements - Identify tools and equipment - Draw diagram of series circuits for two lamps. - Perform preliminary marking before termination of cables. - Follow drawing to perform wiring and for reporting/ record purpose. |
| C3. Construct tunnel circuit for multi storey building stair case. | - Observe OHS requirements - Identify tools and equipment - Draw diagram of tunnel circuit for multi storey staircase. - Perform preliminary marking before termination of cables. - Follow drawing to perform wiring and for reporting/ record purpose. |

Knowledge & Understanding

The candidate must be able to develop skills and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain electrical symbols

- Describe different types of drawings parallel circuits, series circuit and tunnel circuit.

- Describe how to prepare drawing if not available

- Describe types of breakers, contactors, relays etc.

- Describe standard tools used in wiring

- Explain different types of wiring techniques including: • Concealed • Conduit • Busway• Open • Duct

- Describe purpose of color coding and tagging during wiring

- Describe purpose of cable alignment

- Describe open, close circuits and testing procedures

- Describe Importance of controlled live test

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of PPE

- Use of tools

- Proper storage of tools and equipment

- Wiring of

Tools and Equipment

- Electrical drawing

- Computer

- Emergency light

- Ladder

- drill machine tool kit

- Thimble puncher

- Shroud

- PVC tape

- cable knife

- wire stripper

- drill machine

- PPE

- Cable tie

- Clamps

- Plier

- Cutter

- spiral/ flexible pipes

- tie base

- ravel bolt

- ravel plug

- hilty bolt

- wooden screw

- AVO meter

- test lamp

- earth tester

- line tester

- megger

- . Measure properties of electricity

Overview: This competency standard covers the skills and knowledge required to measure voltage, current, find out resistance and power.

| Competency Units | Performance Criteria |
| --- | --- |
| - Measure voltage | - Identify tools and equipment - Measure value of voltage according to specification - Observe OHS requirement |
| - Measure current | - Identify tools and equipment - Measure value of current according to specification - Observe OHS requirement |
| - Find out resistance | - Record values of voltage and current - Calculate resistance by ohm’s law |
| - Find out power | - Record values of voltage and current - Calculate power |

Knowledge & Understanding

The candidate must be able to develop skills and understanding to measure voltage, current, find out resistance and power required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe Conductor, Insulator, Semi-Conductor current, Ampere voltage, Resistance and ohm

- Define ohms law specific resistance laws up Resistance make calculation using

- Define these laws resistance in series and parallels solve series and parallel circuits

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of PPE

- Use of tools

- Proper storage of tools and equipment

Tools and Equipment

- Voltmeter

- Ammeter

- Multi meter

- Ohmmeter

- . Perform electrical wiring of a single room

Overview: This competency standard covers the skills and knowledge required to Interpret Drawing, Arrange Resources for electrical wiring, Perform Wiring (Domestic & Industrial), termination and tagging, Perform Wire Dressing, and Perform Checking & Testing of electrical wiring.

| Competency Units | Performance Criteria |
| --- | --- |
| - Interpret Drawing | P1. Differentiate between control and power wiring as per job requirement P2. Perform preliminary marking before termination of cables P3. Follow drawing to perform wiring and for reporting/record purpose |
| - Arrange Resources for electrical wiring | P1. Enlist and arrange tools and material as per job P2. Arrange work permit for the wiring task P3. Arrange backup resources for lighting, power and safety purposes as per job requirement |
| - Perform Wiring (Domestic & Industrial), termination and tagging | P1. Perform Wiring using following methods: • Concealed • Conduit • Busway • Open • Duct P2. Follow procedure to remove insulation and to attach thimbles P3. Perform different types of terminations including • High tension (HT) • Low tension (LT) • Control shrouding • Sleeves P4. Perform different types of joints • High tension • Low tension • Water proof/underground • Open P5. Attach tags or numbers to wires for identification P6. Use personal protective equipment as per job requirement |
| - Perform Wire Dressing | P1. Maintain distance between power, communication and control wires according to job requirements P2.perform alignment of cables as per job requirement |
| - Perform Checking & Testing of electrical wiring | P1.Check continuity using line of terminal P2. Perform test to identify open/close circuits P3. Perform live test for verification of wiring P4. Verify proper grounding / Panel Earthing (PE) on designated locations |

Knowledge & Understanding

The candidate must be able to develop skills and understanding o Interpret Drawing, Arrange Resources for electrical wiring, Perform Wiring (Domestic & Industrial), termination and tagging, Perform Wire Dressing, Perform Checking & Testing of electrical wiring required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain electrical symbols

- Describe different types of drawings (e.g. power, control, single line etc.)

- Describe how to prepare drawing if not available

- Describe types of breakers, contactors, relays etc.

- Describe standard tools used in wiring

- Describe usage of back up resources including Emergency lights, ladder, safety belts etc

- Explain different types of wiring techniques including: • Concealed • Conduit • Busway• Open • Duct

- Describe purpose of color coding and tagging during wiring K4. Describe types and size of thimbles and use of thimble puncher

- Explain Termination techniques for HT, LT, Control shrouding, sleeves K6.describe Jointing techniques for HT, LT

- Describe purpose of distancing

- Describe purpose of cable alignment

- Describe Importance of terminals

- Describe open, close circuits and testing procedures

- Describe Importance of controlled live test

- Describe Importance of grounding

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of PPE

- Use of tools

- Proper storage of tools and equipment

Tools and Equipment

- Electrical drawing

- Computer

- Emergency light

- Ladder

- drill machine tool kit

- Thimble puncher

- Shroud

- PVC tape

- HT tape

- Thimbles

- cable knife

- wire stripper

- drill machine

- PPE

- Cable tie

- Clamps

- Plier

- Cutter

- spiral/ flexible pipes

- tie base

- ravel bolt

- ravel plug

- hilty bolt

- wooden screw

- AVO meter

- test lamp

- earth tester

- line tester

- megger

# List of Tools, Equipment and Consumable

01: Basic Engineering Surveying

| S# | Tools and Equipment | S# | Consumable Material |
| --- | --- | --- | --- |
| 1 | Ranging rods | 1. | Lime |
| 2 | Steel Tape | 2. | Drawing Sheets |
| 3 | Steel Band | 3. | Lead Pencils |
| 4 | Invar tape | 4. | Eraser |
| 5 | Metallic Tape |  |  |
| 6 | Arrows |  |  |
| 7 | Pegs |  |  |
| 8 | Mallet |  |  |
| 9 | Umbrella with stand |  |  |
| 10 | Engineer’s chain |  |  |
| 11 | Gunter’s chain |  |  |
| 12 | Revenue Chain |  |  |
| 13 | Metric chain |  |  |
| 14 | Cross staff, |  |  |
| 15 | line ranger, |  |  |
| 16 | Optical square, |  |  |
| 17 | Prismatic compass with stand. |  |  |
| 18 | Telescopic Survey compass |  |  |
| 19 | Telescopic Alidad |  |  |
| 20 | Surveyor’s compass |  |  |
| 21 | Plane table with all accessories |  |  |
| 22 | Tilting Level with parallel plate micrometer (Precise level) |  |  |
| 23 | Auto Level |  |  |
| 24 | Laser Level |  |  |
| 25 | Abney’s Level |  |  |
| 26 | Levelling Staff |  |  |
| 27 | Digital planimeter |  |  |
| 28 | Pentagraph |  |  |
| 29 | Barometer |  |  |
| 30 | Prism with Prism Pole |  |  |
| 31 | Ball Pen Hammer |  |  |
| 32 | Velocity Rods (Floats). |  |  |
| 33 | Current Meter |  |  |
| 34 | Sounding Rods |  |  |
| 35 | Drawing Lab Instruments |  |  |
| 36 | Target Staff |  |  |

02: Civil Engineering Drawing

| S# | Tools and Equipment | S# | Consumable Material |
| --- | --- | --- | --- |
| 1. | Drafting Table- Portable (Taper Type), Table size 32" x 24" | 1. | Lead Pencils |
| 2. | Instruments Box-Special Compass Set | 2. | Eraser |
| 3. | Tee Square (600 mm) Transparent | 3. | Tracing Sheets |
| 4. | Set Square- Transparent medium size (300 mm) | 4. | Tracing Cloth |
| 5. | Templates- Circle Square, Hexagon, Triangle (03 each) | 5. | Drawing Sheets |
| 6. | French Curves - Good Quality, medium size | 6. | Graph paper |
| 7. | Sharpener Machine | 7. | Scotch Tape |
| 8. | Drafting Machine Elbow type- Table size A o complete with all accessories | 8. | Handkerchief |

|  | 03: Building Materials |  |  |
| --- | --- | --- | --- |
| S# | Tools and Equipment | S# | Consumable Material |
| 1 | Brick Making Machine | 1 | Sand |
| 2 | Brick Molds | 2 | Sand Dust |
| 3 | Hand roller | 3 | cement |
| 4 | Thermometer | 4 | Lime |
| 5 | Umbrella with stand | 5 | Brick |
| 6 | PPEs | 6 | Stones |
| 7 | Calculator | 7 | Ferrous metal samples |
| 8 | Stone masonry instruments | 8 | Non-Ferrous metal samples |
| 9 | Scissor | 9 | Pencil, |
| 10 | Mason Trowel | 10 | Eraser, |
| 11 | Hammer | 11 | Sharpener, |
| 12 | Hammer Forged steel rectangle head | 12 | Drawing sheet, |
| 13 | Lump hammers |  |  |
| 14 | Power Tools |  |  |
| 15 | Hand Tools |  |  |

|  | 04: Workshop Practice-I (Wood Work) |  |  |
| --- | --- | --- | --- |
| S# | Tools and Equipment | S# | Consumable Material |
| 1. | Ear Muff, Ear Plug | 1. | Timber Deodar |
| 2. | Safety shoes | 2. | Timber Partal |
| 3. | Gloves | 3. | Nails |
| 4. | Safety Goggles | 4. | Screws |
| 5. | Safety Helmet | 5. | Gums |
| 6. | Respiratory protective equipment i.e. Masks | 6. | Polishes |
| 7. | Life Jacket | 7. | Lacquer |
| 8. | High visibility jacket | 8. | lac |
| 9. | Safety harness | 9. | paraffin and linseed oils |
| 10. | Face shield | 10. | methylated spirits |
| 11. | Rubber shoes/ long shoes | 11. | rubbers |
| 12. | Compression testing machine | 12. | Deodar Wood for Wooden Joints |
| 13. | Digital weighing balance | 13. | Fire Fighting Safety Equipment (Fire Extinguishers and Safety Care, etc.) |
| 14. | Laboratory oven | 14. | Glue 5 Kg |
| 15. | Files | 15. | Kail Wood for Door & Frame |
| 16. | Diamond stones | 16. | Kerosene Oil |
| 17. | Water stones | 17. | Lubricating Oil / Grease |
| 18. | Oil stones | 18. | Nails |
| 19. | Ceramic stones | 19. | Overall / Apron for Trainees |
| 20. | Wood tool | 20. | Safety Shoes for Trainees |
| 21. | Measuring tool | 21. | Screw ¾“, 1”, 1½ “, 2” (2 Packets each) |
| 22. | Marking tools | 22. | Sheshum Wood |
| 23. | Holding tool | 23. | Veneer Board (4" x 8’ x ¾”) |
| 24. | Cutting tool |  |  |
| 25. | Planning tool |  |  |
| 26. | Fastening tools |  |  |
| 27. | Boring machine |  |  |
| 28. | Fastenings |  |  |
| 29. | Sprayer |  |  |
| 30. | Circular SAW |  |  |
| 31. | W.W, Planer |  |  |
| 32. | Wood Turning lathe |  |  |
| 33. | Spindle |  |  |
| 34. | Boring machine |  |  |
| 35. | Sander |  |  |

|  | 05: Workshop Practice-II (Electrical Wiring) |  |  |
| --- | --- | --- | --- |
| S# | Tools and Equipment | S# | Consumable Material |
| 1. | Ear Muff, Ear Plug | 1. | Cable 3/0.029, 7/0.029, 7/0.036, flexible, |
| 2. | Safety shoes | 2. | single core, double core each |
| 3. | Gloves | 3. | Fire Fighting Safety Equipment (Fire Extinguishers and Safety Care, etc.) |
| 4. | Safety Goggles | 4. | Wiring accessories (switch, |
| 5. | Safety Helmet | 5. | sockets, |
| 6. | Respiratory protective equipment i.e. Masks | 6. | boards of sizes, |
| 7. | Life Jacket | 7. | bulb holders, |
| 8. | High visibility jacket | 8. | dimmer, |
| 9. | Safety harness | 9. | Ceiling rose, |
| 10. | Face shield | 10. | indicators, |
| 11. | Rubber shoes/ long shoes | 11. | breakers, |
| 12. | Electrical drawing | 12. | main switches, |
| 13. | Computer | 13. | two way switch, |
| 14. | Emergency light | 14. | change overs, |
| 15. | Ladder | 15. | regulators, |
| 16. | drill machine tool kit | 16. | cleats, |
| 17. | Thimble puncher | 17. | clips, |
| 18. | Shroud | 18. | insulation tape, |
| 19. | PVC tape | 19. | screws of sizes, |
| 20. | HT tape | 20. | steel nails etc |
| 21. | Thimbles | 21. | pipes, |
| 22. | cable knife | 22. | casing and capping, |
| 23. | wire stripper | 23. | batten each |
| 24. | drill machine | 24. | bulbs, |
| 25. | PPE | 25. | tube lights, |
| 26. | Cable tie | 26. | energy savers, |
| 27. | Clamps | 27. | Circuit Breakers |
| 28. | Plier | 28. | Electric Ballasts (40 W) |
| 29. | Cutter | 29. | Panel Board |
| 30. | spiral/ flexible pipes | 30. | Earthing system materials |
| 31. | tie base |  |  |
| 32. | ravel bolt |  |  |
| 33. | ravel plug |  |  |
| 34. | hilty bolt |  |  |
| 35. | wooden screw |  |  |
| 36. | AVO meter |  |  |
| 37. | test lamp |  |  |
| 38. | earth tester |  |  |
| 39. | line tester |  |  |
| 40. | Megger |  |  |
| 41. | Voltmeter |  |  |
| 42. | Ammeter |  |  |
| 43. | Multi meter |  |  |
| 44. | Ohmmeter |  |  |
| 45. | Drawings, operations manuals |  |  |
| 46. | Layouts, ICT, measuring tools (calipers, cable chart, hand hacksaw, measuring tape, laser gun for length), quality standards, vendor certificate, grinding cutter, drill machines, hammer |  |  |
| 47. | Layouts, lifters, chain-pulley, jacks, sprit level |  |  |
| 48. | Layouts, measuring tools, cutting tools like wire cutter, side cutter, cable cutter, thimble presser, megger, stacker, tags, glands, shrouds, conduits, cable tray etc |  |  |
| 49. | Layouts, cutting tools e.g. wire cutter, side cutter, cable cutter, stacker, tags etc. |  |  |
| 50. | Drawing, layout, manual, fork lifter, drill machine, |  |  |
| 51. | Layouts, lifters, chain-pulley, jacks, tags |  |  |
| 52. | Soldering, earth tester, thimble presser, |  |  |
| 53. | Drawings, multi meters, |  |  |
| 54. | Wiring Boards 3’ x 5’ Wall Mounted |  |  |
| 55. | Wood Chisel 1” |  |  |
| 56. | Work Benches 4’ x 6’ x 2.5’ |  |  |
| 57. | Single-Phase Electric Motors |  |  |
| 58. | Single–Phase Energy Meter |  |  |
| 59. | Air Conditioners |  |  |
| 60. | 3-Phase Electric Motors |  |  |
| 61. | 3-Phase Energy Meter |  |  |
| 62. | AVO Meters |  |  |
| 63. | Clamp on Meter |  |  |
| 64. | Cold Chisels 10” Long |  |  |
| 65. | Combination Pliers 8” |  |  |
| 66. | Electric Bells |  |  |
| 67. | Fluorescent Tubes (with electric choke) |  |  |
| 68. | Hack Saws 15” |  |  |
| 69. | Hand-Drill Machine Electric Two Speeds 3/4” Chuck Heavy Duty |  |  |
| 70 | Kerosene Burners |  |  |