National Competency Standards Level-3 for “Civil Technology”

Competency Standards

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

ACKNOWLEDGEMENTS

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to many representatives of business, industry, academia, government agencies, Provincial TEVTAs, Sector Skill Councils and trade associations who speared their time and expertise to the development and validation of these National Vocational Qualifications (Competency Standards, Curricula, Assessments Packs and related material). This work would not have been possible without the financial and technical support of the TVET Sector Support Programme co-funded by European Union, Norwegian and German Governments implemented by GIZ Pakistan. NAVTTC is especially indebted to Dr. Muqeem ul Islam, who lead the project from the front. The core team was comprised on:

- Dr. Muqeem ul Islam, Director General (Skills, Standards and Curricula) NAVTTC

- Mr. Muhammad Naeem Akhtar, Senior Technical Advisor TSSP-GIZ

- Mr. Muhammad Yasir, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Ishaq, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Fayaz Soomro, Deputy Director (SS&C Wing) NAVTTC

NAVTTC team under the leadership of Dr. Muqeem ul Islam initiated development of CBT & A based qualifications of diploma level-5 as a reform project of TVET sector in November 2018 and completed 27 NVQF diplomas of Level-5 in September, 2019. It seems worth highlighting that during this endeavor apart from developing competency standards/curricula in conventional trades new dimensions containing high-tech trades in TVET sector in the context of generation IR 4.0 trades have also been developed which inter alia includes Robotics, Mechatronics, artificial intelligence, industrial automation, instrumentation and process control. Moreover, trades like entrepreneurship, green/environmental skills and variety of soft/digital skill have also been developed to equip the Pakistani youth with skills set as per requirement of the global trends. These skills have been made integral part of all the 27 diplomas.

Nobody has been more important in the pursuit of this project than Dr. Nasir Khan, Executive Director, NAVTTC, whose patronage and support remain there throughout the development process and lastly to thanks specially to Syed Javed Hassan, Chairman NAVTTC and Raja Saad Khan, Deputy Team Lead TSSP-GIZ who made it happened in this challenging time.

Contents

1. Introduction	5

2. Purpose of the Qualification	5

3.  Core competencies of the Qualification	5

4. Date of Validation	22

5. Codes of Qualifications	22

6. Members of Qualifications Development Committee	23

7. Members of Qualifications Validation Committee	23

8. Entry Requirements	24

9. Proposed Scheme of Studies	24

9. A- Occupation Packaging of Civil Technology	26

9.	B- Qualification Packaging of Civil Technology	27

10.	Detail of Qualifications and its Competency Standards Level Wise	27

11. B- Certificate in Civil Technology Level 3	28

10.1.	Advanced Surveying	28

0732B&CE-1	. Measure Angles by Theodolite	28

0732B&CE-2	Perform Trigonometric Levelling by Theodolite	31

0732B&CE-3	Perform Traversing by Theodolite	33

0732B&CE-4	Set Out Horizontal Curves by Theodolite	35

0732B&CE-5	Set out Vertical Curves by Theodolite	38

0732B&CE-6	Carry out Triangulation by Theodolite	40

0732B&CE-7	Perform Temporary Adjustment of Total Station	42

0732B&CE-8	Measure Distance by Total Station	44

0732B&CE-9	Measure Angles by Total Station	46

0732B&CE-10	Measure Area by Total Station	48

0732B&CE-11	Set out Curves by Total Station	50

0732B&CE-12	Perform Integration of GPS	52

0732B&CE-13	Measure Co-ordinates by GPS	54

0732B&CE-14	Set out Civil Engineering Structures by GPS	55

10.2.	Auto CAD	58

0732B&CE-15	Perform Basic Computer Operations	58

0732B&CE-16	Perform Drafting Settings and Formatting	61

0732B&CE-17	Perform Dimensioning, Lettering and Plotting	63

0732B&CE-18	Develop 2d Drawings of Structures	64

0732B&CE-19	Produce Drawing of Framed Structure Building	67

0732B&CE-20	Develop Sectional Drawing for foundations	69

0732B&CE-21	Develop Drawings for Doors & Windows	71

0732B&CE-22	Produce Drawings of Stairs and Stair Cases	73

0732B&CE-23	Produce Drawings of Water Tanks	74

0732B&CE-24	Produce Drawings of Cross Drainage Works	76

0732B&CE-25	Develop 3d Drawings	79

0732B&CE-26	Produce Civil Technology Drawings	81

10.3.	Construction I	84

0732B&CE-79	Perform Simple Levelling	84

0732B&CE-80	Lay out Building plans	86

0732B&CE-81	Carryout excavation and install supports	89

0732B&CE-82	Place Concrete in Foundations	92

0732B&CE-83	Prepare for construction Process (Brick/ block laying)	94

0732B&CE-84	Mix Cementitious materials (Mortar & Concrete)	97

0732B&CE-85	Install flashing and damp proof course	100

0732B&CE-86	Carry out brick work	103

0732B&CE-87	Construct Cavity Wall	107

0732B&CE-88	Construct Curved Wall	112

0732B&CE-89	Construct Masonry arches (semi-circular and segmental)	114

0732B&CE-90	Construct Scaffolding	119

0732B&CE-91	Install doors and window frames	122

0732B&CE-92	Prepare Surface and fix tiles	126

0732B&CE-93	Prepare Surface for Painting	132

0732B&CE-94	Paint by spray, brush and roller	135

0732B&CE-95	Apply stains and clear timber finishes	138

0732B&CE-96	Prepare site layout of civil engineering projects	143

0732B&CE-97	Construct masonry steps and stairs	146

0732B&CE-98	Install stairs	149

0732B&CE-99	Prepare for solid dry wall plastering	152

0732B&CE-100	Apply plaster	155

10.4.	Plumbing	158

0732B&CE-101	Identify plumbing tools & equipment	158

0732B&CE-102	Perform cutting, threading & reaming	160

0732B&CE-103	Join pipes & fittings	161

0732B&CE-104	Layout for domestic water supply	165

0732B&CE-105	Make water connection	167

0732B&CE-106	Install plumbing fixtures	169

0732B&CE-107	Install & repair water pumps	173

0732B&CE-108	Install turbine pumps	175

10.5.	Digital Skills	177

0732B&CE-109	Install computer operating systems and hardware	177

0732B&CE-110	Operate digital media technology	180

0732B&CE-111	Perform computer operations	182

0732B&CE-112	Use computer applications	184

0732B&CE-113	Create user documentation	187

0732B&CE-114	Create technical documentation	188

0732B&CE-115	Create basic databases	190

0732B&CE-116	Use social media tools for collaboration and engagement	192

0732B&CE-117	E-Commerce- SEO (Search Engine Optimization)	194

0732B&CE-118	E-Commerce- SCM (Supply Chain Management)	195

0732B&CE-119	E-Commerce- Social Media Marketing	196

0732B&CE-120	Use digital devices	197

0732B&CE-121	Operate word-processing applications	199

0732B&CE-122	Operate spreadsheet applications	201

0732B&CE-123	Operate presentation packages	203

0732B&CE-124	Perform writing and editing tasks	205

0732B&CE-125	Write and Edit Copy	207

0732B&CE-126	Build Java applets	209

0732B&CE-127	Manipulate Images (Illustrator)	211

0732B&CE-128	Manipulate Images (Photoshop)	213

0732B&CE-129	Design simple web page layouts	215

0732B&CE-130	Apply introductory programming techniques	217

List of Tools, Equipment and Consumable	222

# 1. Introduction

Civil engineering is considered as the first discipline of the various branches of engineering after military engineering, and includes the designing, planning, construction, and maintenance of the infrastructure. The works include roads, bridges, buildings, dams, canals, water supply and numerous other facilities that affect the life of human beings. Civil engineering is intimately associated with the private and public sectors, including the individual homeowners and international enterprises. It is one of the oldest engineering professions, and ancient engineering achievements due to civil engineering include the pyramids of Egypt and road systems developed by the Romans.

Civil engineering has a significant role in the life of every human being, though one may not truly sense its importance in our daily routine. The function of civil engineering commences with the start of the day when we take a shower, since the water is delivered through a water supply system including a well-designed network of pipes, water treatment plant and other numerous associated services. The network of roads on which we drive while proceeding to school or work, the huge structural bridges we come across and the tall buildings where we work, all have been designed and constructed by civil engineers. Even the benefits of electricity we use are available to us through the contribution of civil engineers who constructed the towers for the transmission lines. In fact, no sphere of life may be identified that does not include the contribution of civil engineering. Thus, the importance of civil engineering may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Civil Technology under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# 2. Purpose of the Qualification

The purpose of this qualifications is to set high professional standards for civil technology sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for civil technology industry

- Enable the existing workforce to capacitate themselves in new technologies and methods

# 3.  Core competencies of the Qualification

The proposed curriculum is composed of 306 competencies that will be covered in 3600 Learning hours. Duration of the course is proposed to be Three years. The total weightage for technical competencies is 3600 hours. A total of 1200 hours have been reserved for allied subjects i.e. Islamic studies, English, Mathematics, Physics and Chemistry.

The details of technical modules are given as under:

Level 1 & 2 = 6 months. (Single Semester)

Level - 3 = 6 months. (Single Semester)

Level - 4 = 1 Year. (Two Semesters)

Level - 5 = 1 Year. (Two Semesters)

The overall distribution of contact hours and Credit Hours is given below:

Total.		3600 hours. & 360 Credits

Theory.	976 hours (27.11 %)

Practical.	2624 hours (72.89 %)

The distribution of contact hours and credit hours in each level is given below:

Level 1 & 2 = 6 months (16 weeks)

Total.		592 hours. & 59.2 Credits

Theory.	128 hours (22 %)

Practical.	464 hours (78 %)

Level - 3 = 6 months (16 weeks)

Total.		608 hours. & 60.8 Credits

Theory.	128 hours (21 %)

Practical.	480 hours (79 %)

Level - 4 = 1 Year (16 weeks + 16 Weeks)

Total.		1200 hours. & 120 Credits

Theory.	240 hours (20 %)

Practical.	960 hours (80 %)

Level - 5 = 1 Year (16 weeks + 16 Weeks)

Total.		1200 hours. & 120 Credits

Theory.	480 hours (40 %)

Practical.	720 hours (60 %)

National Vocational Certificate Level 1 & 2 Site assistant in Civil Technology

| S# | Competency Standard | Category | Level | Theory | Practical | Total | Credit Hour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11.01 Basic Engineering Surveying |  |  | 32 | 160 | 192 | 19.2 |
| 0732B&CE001 | Perform Chain Survey | Core | 2 | 4 | 9 | 13 | 1.3 |
| 0732B&CE002 | Perform Compass Traversing | Core | 2 | 4 | 15 | 19 | 1.9 |
| 0732B&CE003 | Carry out Plane Table Survey | Core | 2 | 4 | 12 | 16 | 1.6 |
| 0732B&CE004 | Perform Levelling | Core | 2 | 4 | 27 | 31 | 3.1 |
| 0732B&CE005 | Perform Reciprocal Levelling | Core | 2 | 2 | 21 | 23 | 2.3 |
| 0732B&CE006 | Perform Long & Cross Sectioning | Core | 2 | 2 | 22 | 24 | 2.4 |
| 0732B&CE007 | Perform Tacheometry | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE008 | Perform Contouring | Core | 2 | 2 | 18 | 20 | 2 |
| 0732B&CE009 | Perform Hydrographic Survey | Core | 2 | 6 | 15 | 21 | 2.1 |
| 0732B&CE010 | Compute Volume by Contouring | Core | 2 | 2 | 12 | 14 | 1.4 |
|  | 11.02 Civil Engineering Drawing |  |  | 32 | 160 | 192 | 19.2 |
| 0732B&CE011 | Produce Templates, Title Block/ Strip & Draw Lines | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE012 | Perform Lettering and Printing on Sheet. | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE013 | Construct Scales for Drawing | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE014 | Construct Geometrical Figures | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE015 | Produce Orthographic Projections | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE016 | Perform Sectioning | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE017 | Perform Dimensioning On Drawings | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE018 | Produce Pictorial Drawings | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE019 | Develop Symbols of Engineering Drawings | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE020 | Produce Cross Section of Wall. | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE021 | Develop Building Drawings | Core | 2 | 6 | 34 | 40 | 4 |
| 0732B&CE022 | Perform House Planning | Core | 2 | 2 | 9 | 11 | 1.1 |
| 0732B&CE023 | Produce Layout Plans for Building Services. | Core | 2 | 2 | 18 | 20 | 2 |
| 0732B&CE024 | Produce Drawings of Sanitary Structures | Core | 2 | 2 | 9 | 11 | 1.1 |
|  | 11.03 Building Materials |  |  | 32 | 48 | 80 | 8 |
| 0732B&CE025 | Identify bricks | Core | 2 | 4 | 6 | 10 | 1 |
| 0732B&CE026 | Dress stone for masonry | Core | 2 | 4 | 6 | 10 | 1 |
| 0732B&CE027 | Prepare slaked lime | Core | 2 | 2 | 2 | 4 | 0.5 |
| 0732B&CE028 | Identify and store cement for construction | Core | 2 | 4 | 6 | 10 | 1 |
| 0732B&CE029 | Select sand for construction | Core | 2 | 2 | 3 | 5 | 0.5 |
| 0732B&CE030 | Identify ferrous and non-ferrous metals | Core | 2 | 2 | 6 | 8 | 1 |
| 0732B&CE031 | Identify advanced construction materials | Core | 2 | 4 | 9 | 13 | 1 |
| 0732B&CE032 | Work Safely in Construction Industry | Generic | 2 | 4 | 1 | 5 | 0.5 |
| 0732B&CE033 | Interpret Simple Building Plans | Core | 2 | 2 | 3 | 5 | 0.5 |
| 0732B&CE034 | Read and Interpret plans and Specifications | Core | 2 | 2 | 3 | 5 | 0.5 |
| 0732B&CE035 | Apply power and hand tools in construction. | Core | 2 | 2 | 3 | 5 | 0.5 |
|  | 11.04 Workshop Practice (Wood Work) |  |  | 16 | 48 | 64 | 6.4 |
| 0732B&CE036 | Carry out OH & S requirement in workshop | Generic | 2 | 2 | 3 | 5 | 0.5 |
| 0732B&CE037 | Determine properties and types of locally manufactured timber | Generic | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE038 | Sharpen carpentry tools | Core | 2 | 1 | 6 | 7 | 0.7 |
| 0732B&CE039 | Apply hand tools | Core | 2 | 1 | 6 | 7 | 0.7 |
| 0732B&CE040 | Perform wooden joinery work | Core | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE041 | Apply fastenings | Core | 2 | 1 | 3 | 4 | 0.4 |
| 0732B&CE042 | Apply traditional spirits polishing techniques | Core | 2 | 2 | 3 | 5 | 0.5 |
| 0732B&CE043 | Apply portable power tools | Core | 2 | 1 | 3 | 4 | 0.4 |
| 0732B&CE044 | Make simple calculations of timber | Core | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE045 | Construct a basic timber product | Core | 2 | 2 | 6 | 8 | 0.8 |
|  | 11.05 Workshop Practice (Electrical Wiring) |  |  | 16 | 48 | 64 | 6.4 |
| 0732B&CE046 | Carry out OH & S requirement in workshop | Generic | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE047 | Maintain safety | Generic | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE048 | Perform electrical wiring | Core | 2 | 3 | 9 | 12 | 1.2 |
| 0732B&CE049 | Install electrical system | Core | 2 | 3 | 9 | 12 | 1.2 |
| 0732B&CE050 | Construct Circuits | Core | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE051 | Measure properties of electricity | Core | 2 | 2 | 6 | 8 | 0.8 |
| 0732B&CE052 | Perform electrical wiring of a single room | Core | 2 | 2 | 6 | 8 | 0.8 |

National Vocational Certificate Level 3 Assistant surveyor in Civil Technology

| S# | Competency Standard | Category | Level | Theory | Practical | Total | Credit Hour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11.06 Advanced Surveying |  |  | 48 | 192 | 240 | 24 |
| 0732B&CE053 | Measure Angles by Theodolite | Core | 3 | 4 | 18 | 22 | 2.2 |
| 0732B&CE054 | Perform Trigonometric Levelling by Theodolite | Core | 3 | 3 | 9 | 12 | 1.2 |
| 0732B&CE055 | Perform Traversing by Theodolite | Core | 3 | 4 | 15 | 19 | 1.9 |
| 0732B&CE056 | Set Out Horizontal Curves by Theodolite | Core | 3 | 4 | 18 | 22 | 2.2 |
| 0732B&CE057 | Set out Vertical Curves by Theodolite | Core | 3 | 3 | 9 | 12 | 1.2 |
| 0732B&CE058 | Carry out Triangulation by Theodolite | Core | 3 | 3 | 21 | 24 | 2.4 |
| 0732B&CE059 | Perform Temporary Adjustment of Total Station | Core | 3 | 4 | 9 | 13 | 1.3 |
| 0732B&CE060 | Measure Distance by Total Station | Core | 3 | 4 | 12 | 16 | 1.6 |
| 0732B&CE061 | Measure Angles by Total Station | Core | 3 | 4 | 18 | 22 | 2.2 |
| 0732B&CE062 | Measure Area by Total Station | Core | 3 | 3 | 18 | 21 | 2.1 |
| 0732B&CE063 | Set out Curves by Total Station | Core | 3 | 4 | 9 | 13 | 1.3 |
| 0732B&CE064 | Perform Integration of GPS | Core | 3 | 2 | 6 | 8 | 0.8 |
| 0732B&CE065 | Measure Co-ordinates by GPS | Core | 3 | 2 | 6 | 8 | 0.8 |
| 0732B&CE066 | Set out Civil Engineering Structures by GPS | Core | 3 | 4 | 24 | 28 | 2.8 |
|  | 11.07 AutoCAD |  |  | 32 | 128 | 160 | 16 |
| 0732B&CE067 | Perform Basic Computer Operations | Core | 3 | 3 | 6 | 9 | 0.9 |
| 0732B&CE068 | Perform Drafting Settings and Formatting | Core | 3 | 2 | 6 | 8 | 0.8 |
| 0732B&CE069 | Perform Dimensioning, Lettering & Plotting | Core | 3 | 2 | 6 | 8 | 0.8 |
| 0732B&CE070 | Develop 2d Drawings of Structures | Core | 3 | 3 | 15 | 18 | 1.8 |
| 0732B&CE071 | Produce Drawing of Framed Structure Building | Core | 3 | 3 | 18 | 21 | 2.1 |
| 0732B&CE072 | Develop Sectioning Drawing for foundations | Core | 3 | 3 | 18 | 21 | 2.1 |
| 0732B&CE073 | Develop Drawings for Doors & Windows | Core | 3 | 2 | 9 | 11 | 1.1 |
| 0732B&CE074 | Produce Drawings of Stairs and Stair Cases | Core | 3 | 2 | 9 | 11 | 1.1 |
| 0732B&CE075 | Produce Drawings of Water Tanks | Core | 3 | 3 | 18 | 21 | 2.1 |
| 0732B&CE076 | Produce Drawings Of Cross Drainage Works | Core | 3 | 2 | 6 | 8 | 0.8 |
| 0732B&CE077 | Develop 3d Drawings | Core | 3 | 4 | 6 | 10 | 1 |
| 0732B&CE078 | Produce Civil Technology Drawings | Core | 3 | 3 | 11 | 14 | 1.4 |
|  | 11.08 Construction I |  |  | 16 | 112 | 128 | 12.8 |
| 0732B&CE079 | Perform Simple Levelling | Core | 2 | 0.5 | 3 | 3.5 | 0.35 |
| 0732B&CE080 | Lay out Building plans | Core | 2 | 0.5 | 5 | 5.5 | 0.55 |
| 0732B&CE081 | Carryout excavation and install supports | Core | 2 | 1 | 5 | 6 | 0.6 |
| 0732B&CE082 | Place Concrete in Foundations | Core | 2 | 0.5 | 3 | 3.5 | 0.35 |
| 0732B&CE083 | Prepare for construction Process (Brick/ block laying) | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE084 | Mix Cementitious materials (Mortar & Concrete) | Core | 3 | 1 | 3 | 4 | 0.4 |
| 0732B&CE085 | Install flashing and damp proof course | Core | 3 | 1 | 3 | 4 | 0.4 |
| 0732B&CE086 | Carry out brick work | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE087 | Construct Cavity Wall | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE088 | Construct Curved Wall | Core | 3 | 1 | 6 | 7 | 0.7 |
| 0732B&CE089 | Construct Masonry arches (semi-circular and segmental) | Core | 3 | 1 | 6 | 7 | 0.7 |
| 0732B&CE090 | Construct Scaffolding | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE091 | Install doors and window frames | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE092 | Prepare Surface and fix tiles | Core | 3 | 1 | 3 | 4 | 0.4 |
| 0732B&CE093 | Prepare Surface for Painting | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE094 | Paint by spray, brush and roller | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE095 | Apply stains and clear timber finishes | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE096 | Prepare site layout of civil engineering projects | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE097 | Construct masonry steps and stairs | Core | 3 | 1 | 6 | 7 | 0.7 |
| 0732B&CE098 | Install stairs | Core | 3 | 1 | 9 | 10 | 1 |
| 0732B&CE099 | Prepare for solid dry wall plastering | Core | 3 | 1 | 3 | 4 | 0.4 |
| 0732B&CE100 | Apply plaster | Core | 3 | 1 | 3 | 4 | 0.4 |
|  | 11.09 Plumbing |  |  | 16 | 32 | 48 | 4.8 |
| 0732B&CE101 | Identify plumbing tools & equipment | Core | 3 | 4 | 2 | 6 | 0.6 |
| 0732B&CE102 | Perform cutting, threading & reaming | Core | 3 | 2 | 4 | 6 | 0.6 |
| 0732B&CE103 | Make water connection | Core | 3 | 1 | 4 | 5 | 0.5 |
| 0732B&CE104 | Layout for domestic water supply | Core | 3 | 1 | 4 | 5 | 0.5 |
| 0732B&CE105 | Join pipes & fittings | Core | 3 | 3 | 5 | 8 | 0.8 |
| 0732B&CE106 | Install plumbing fixtures | Core | 3 | 2 | 4 | 6 | 0.6 |
| 0732B&CE107 | Install & repair water pumps | Core | 3 | 1 | 4 | 5 | 0.5 |
| 0732B&CE108 | Install turbine pumps | Core | 3 | 2 | 5 | 7 | 0.7 |
|  | 11.10 Digital Skills | 32 |  | 16 | 16 | 32 | 3.2 |
| 0732B&CE109 | Install computer operating systems and hardware | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE110 | Operate digital media technology | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE111 | Perform computer operations | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE112 | Use computer applications | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE113 | Create user documentation | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE114 | Create technical documentation | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE115 | Create basic databases | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE116 | Use social media tools for collaboration and engagement | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE117 | E-Commerce- SEO (Search Engine Optimization) | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE118 | E-Commerce- SCM (Supply Chain Management) | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE119 | E-Commerce- Social Media Marketing | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE120 | Use digital devices | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE121 | Operate word-processing applications | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE122 | Operate spreadsheet applications | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE123 | Operate presentation packages | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE124 | Perform writing and editing tasks | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE125 | Write and Edit Copy | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE126 | Build Java applets | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE127 | Manipulate Images (Illustrator) | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE128 | Manipulate Images (Photoshop) | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE129 | Design simple web page layouts | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE130 | Apply introductory programming techniques | Generic | 3 | 1 | 1 | 2 | 0.2 |

National Vocational Certificate Level 4 Part-1 Site Supervisor in Civil Technology

| S# | Competency Standard | Category | Level | Theory | Practical | Total | Credit Hour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11.11 Construction Techniques II |  |  | 32 | 128 | 160 | 16 |
| 0732B&CE131 | Prepare subgrade, base, and bedding course for paving | Core | 4 | 3 | 10 | 13 | 1.3 |
| 0732B&CE132 | Lay paving | Core | 4 | 1 | 5 | 6 | 0.6 |
| 0732B&CE133 | Perform concrete, tiles and mosaic flooring | Core | 4 | 2 | 10 | 12 | 1.2 |
| 0732B&CE134 | Perform marble/granite flooring | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE135 | Apply water proofing remedial processes | Core | 4 | 2 | 5 | 7 | 0.7 |
| 0732B&CE136 | Apply and install sealants | Core | 4 | 1 | 5 | 6 | 0.6 |
| 0732B&CE137 | Perform water proofing works | Core | 4 | 2 | 5 | 7 | 0.7 |
| 0732B&CE138 | Apply water proofing tools and equipment | Core | 4 | 3 | 5 | 8 | 0.8 |
| 0732B&CE139 | Perform water proofing of basements | Core | 4 | 2 | 5 | 7 | 0.7 |
| 0732B&CE140 | Apply water proofing to internal wet areas | Core | 4 | 2 | 10 | 12 | 1.2 |
| 0732B&CE141 | Install Physical termite barriers | Core | 4 | 2 | 10 | 12 | 1.2 |
| 0732B&CE142 | Erect and dismantle slip formwork | Core | 4 | 2 | 10 | 12 | 1.2 |
| 0732B&CE143 | Install trench support | Core | 4 | 1 | 7 | 8 | 0.8 |
| 0732B&CE144 | Construct underpinning | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE145 | Carry out reinforced concrete work | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE146 | Work safely on roofs | Generic | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE147 | Install roof components | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE148 | Install roof drainage | Core | 4 | 2 | 7 | 9 | 0.9 |
| 0732B&CE149 | Lay roof tiles | Core | 4 | 1 | 7 | 8 | 0.8 |
| 0732B&CE150 | Erect roof trusses | Core | 4 | 1 | 7 | 8 | 0.8 |
|  | 11.12 Basic Quantity Surveying |  |  | 16 | 176 | 192 | 19.2 |
| 0732B&CE151 | Produce rough cost estimates of buildings | Core | 4 | 1 | 15 | 16 | 1.6 |
| 0732B&CE152 | Evaluate items of work for straight wall, T, L, H, U shaped and circular walls. | Core | 4 | 1 | 12 | 13 | 1.3 |
| 0732B&CE153 | Produce detailed estimate of a single storey building. | Core | 4 | 1 | 15 | 16 | 1.6 |
| 0732B&CE154 | Produce Annual/special repair estimates-Building and Road. | Core | 4 | 1 | 12 | 13 | 1.3 |
| 0732B&CE155 | Evaluate Earth work for Civil Engineering structures | Core | 4 | 1 | 18 | 19 | 1.9 |
| 0732B&CE156 | Produce detailed estimate of Arterial roads | Core | 4 | 1 | 12 | 13 | 1.3 |
| 0732B&CE157 | Produce material statements | Core | 4 | 1 | 12 | 13 | 1.3 |
| 0732B&CE158 | Analyze Rates of different items of work for building | Core | 4 | 2 | 16 | 18 | 1.8 |
| 0732B&CE159 | Produce of rough cost estimate of water supply scheme. | Core | 4 | 2 | 12 | 14 | 1.4 |
| 0732B&CE160 | Produce of rough cost estimate of Sewerage scheme. | Core | 4 | 2 | 15 | 17 | 1.7 |
| 0732B&CE161 | Evaluate Value & Standard Rent of Buildings | Core | 4 | 1 | 12 | 13 | 1.3 |
| 0732B&CE162 | Produce Detailed Estimate of Culverts | Core | 4 | 1 | 13 | 14 | 1.4 |
| 0732B&CE163 | Produce Detailed Estimate of Bridges. | Core | 4 | 1 | 12 | 13 | 1.3 |
|  | 11.13 Water Supply |  |  | 16 | 64 | 80 | 8 |
| 0732B&CE164 | Estimate demand for Water Supply | Core | 4 | 2 | 12 | 14 | 1.4 |
| 0732B&CE165 | Prepare sketches of aqueducts & water pipes | Core | 4 | 2 | 12 | 14 | 1.4 |
| 0732B&CE166 | Draw sketches of intakes | Core | 4 | 2 | 7 | 9 | 0.9 |
| 0732B&CE167 | Plan water treatment (Clarification) | Core | 4 | 4 | 12 | 16 | 1.6 |
| 0732B&CE168 | Plan water treatment (Filtration) | Core | 4 | 4 | 9 | 13 | 1.3 |
| 0732B&CE169 | Plan water distribution system | Core | 4 | 2 | 12 | 14 | 1.4 |
|  | 11.14 Strength of Materials |  |  | 16 | 64 | 80 | 8 |
| 0732B&CE170 | Locate centroid of different sections | Core | 4 | 3 | 12 | 15 | 1.5 |
| 0732B&CE171 | Find out moment of inertia of different sections | Core | 4 | 3 | 12 | 15 | 1.5 |
| 0732B&CE172 | Produce Shear Force and Bending Moment Diagrams | Core | 4 | 4 | 16 | 20 | 2 |
| 0732B&CE173 | Find hardness of metals | Core | 4 | 3 | 12 | 15 | 1.5 |
| 0732B&CE174 | Conduct Tensile Strength Test | Core | 4 | 3 | 12 | 15 | 1.5 |
|  | 11.15. Soft Skills |  |  | 32 | 64 | 96 | 9.6 |
| 0732B&CE175 | Develop workplace policy and procedures for sustainability | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE176 | Manage meetings | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE177 | Manage recruitment selection and induction processes | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE178 | Manage personal work priorities and professional development | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE179 | Manage workforce planning | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE180 | Undertake project work | Generic | 4 | 3 | 6 | 9 | 0.9 |
| 0732B&CE181 | Identify and communicate trends in career development | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE182 | Apply specialist interpersonal and counselling interview skills | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE183 | Work safely in an office environment | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE184 | Develop workplace documents | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE185 | Prepare and implement negotiation | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE186 | Maintain professionalism in the workplace | Generic | 4 | 2 | 4 | 6 | 0.6 |
| 0732B&CE187 | Maintain professional development and career professionalism | Generic | 4 | 4 | 8 | 12 | 1.2 |
| 0732B&CE188 | Organize schedules | Generic | 4 | 3 | 6 | 9 | 0.9 |

National Vocational Certificate Level 4 Part-2 Site Supervisor in Civil Technology

| S# | Competency Standard | Category | Level | Theory | Practical | Total | Credit Hour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11.16 Construction Techniques III |  |  | 16 | 64 | 80 | 8 |
| 0732B&CE189 | Erect and dismantle formwork to suspended slabs, columns, beams and walls | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE190 | Operate elevated platforms | Core | 4 | 1 | 7 | 8 | 0.8 |
| 0732B&CE191 | Construct, erect and dismantle formwork for stairs and ramps | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE192 | Evaluate materials for multi-storey buildings | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE193 | Apply structural principles to the construction of large, high rise and complex buildings | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE194 | Install fire and smoke containment systems | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE195 | Inform clients about thermal performance of residential buildings | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE196 | Design air conditioning and ventilation system | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE197 | Design acoustic of an auditorium | Core | 4 | 0.5 | 4 | 4.5 | 0.45 |
| 0732B&CE198 | Insulate sound insulation products in building | Core | 4 | 1 | 4 | 5 | 0.5 |
| 0732B&CE199 | Install ceiling insulation | Core | 4 | 1 | 3 | 4 | 0.4 |
| 0732B&CE200 | Design an earthquake resistant building | Core | 4 | 1 | 3 | 4 | 0.4 |
| 0732B&CE201 | Coordinate maintenance and repair of buildings | Core | 4 | 1 | 3 | 4 | 0.4 |
| 0732B&CE202 | Assess construction faults in buildings | Core | 4 | 1 | 3 | 4 | 0.4 |
| 0732B&CE203 | Repair and rectify concrete | Core | 4 | 1 | 3 | 4 | 0.4 |
| 0732B&CE204 | Repair wall and floor tiles | Core | 4 | 0.5 | 3 | 3.5 | 0.35 |
| 0732B&CE205 | Apply building codes and standards to building projects | Core | 4 | 1 | 3 | 4 | 0.4 |
|  | 11.17 Concrete Technology |  |  | 32 | 64 | 96 | 9.6 |
| 0732B&CE206 | Test the Properties of Cement | Core | 4 | 6 | 12 | 18 | 1.8 |
| 0732B&CE207 | Find Out Strength of Cement | Core | 4 | 4 | 9 | 13 | 1.3 |
| 0732B&CE208 | Test the Properties of Aggregate | Core | 4 | 6 | 12 | 18 | 1.8 |
| 0732B&CE209 | Test the Properties of Fresh Concrete | Core | 4 | 6 | 10 | 16 | 1.6 |
| 0732B&CE210 | Destructive Tests on Hardened Concrete | Core | 4 | 6 | 9 | 15 | 1.5 |
| 0732B&CE211 | Perform Non-Destructive Tests on Hardened Concrete | Core | 4 | 4 | 12 | 16 | 1.6 |
|  | 11.18 Advanced Quantity Surveying |  |  | 16 | 144 | 160 | 16 |
| 0732B&CE212 | Produce Detailed Estimate of Water Tanks. | Core | 4 | 2 | 18 | 20 | 2 |
| 0732B&CE213 | Produce Detailed Esti mate of Wells. | Core | 4 | 2 | 18 | 20 | 2 |
| 0732B&CE214 | Produce Detailed Estimate of Steel Structures | Core | 4 | 2 | 18 | 20 | 2 |
| 0732B&CE215 | Produce Detailed Estimate of RCC Framed Structure | Core | 4 | 2 | 21 | 23 | 2.3 |
| 0732B&CE216 | Produce Detailed Estimate of Building Services. | Core | 4 | 2 | 12 | 14 | 1.4 |
| 0732B&CE217 | Produce Detailed Estimates Using Computer Software -Excel. | Core | 4 | 3 | 27 | 30 | 3 |
| 0732B&CE218 | Produce Project of Road in AutoCAD Civil 3D software | Core | 4 | 3 | 30 | 33 | 3.3 |
|  | 11.19 Contract Management |  |  | 32 | 64 | 96 | 9.6 |
| 0732B&CE219 | Produce Prequalification documents and collect EOI | Core | 4 | 6 | 8 | 14 | 1.4 |
| 0732B&CE220 | Set Evaluation Criteria for Assessment | Core | 4 | 3 | 3 | 6 | 0.6 |
| 0732B&CE221 | Produce Tender Documents & Collect Proposals | Core | 4 | 6 | 18 | 24 | 2.4 |
| 0732B&CE222 | Perform Tender Evaluation & Award of Contract | Core | 4 | 5 | 17 | 22 | 2.2 |
| 0732B&CE223 | Perform Post Award Legal/ Contract Requirements and Finance | Core | 4 | 5 | 5 | 10 | 1 |
| 0732B&CE224 | Maintain record of Stores. | Core | 4 | 3 | 4 | 7 | 0.7 |
| 0732B&CE225 | Perform Variation orders, claims and Dispute Adjudication Board | Core | 4 | 4 | 9 | 13 | 1.3 |
|  | 11.20 Sanitary Engineering |  |  | 16 | 64 | 80 | 8 |
| 0732B&CE226 | Design sanitary sewers | Core | 4 | 2 | 9 | 11 | 1.1 |
| 0732B&CE227 | Design storm water sewers | Core | 4 | 1.5 | 7 | 8.5 | 0.85 |
| 0732B&CE228 | Layout of sewers | Core | 4 | 2 | 6 | 8 | 0.8 |
| 0732B&CE229 | Prepare sketches of sewer appurtenance | Core | 4 | 2 | 6 | 8 | 0.8 |
| 0732B&CE230 | Construct Sewers | Core | 4 | 1.5 | 9 | 10.5 | 1.05 |
| 0732B&CE231 | Maintain Sewers | Core | 4 | 1.5 | 6 | 7.5 | 0.75 |
| 0732B&CE232 | Install Sewage Pumps | Core | 4 | 2 | 6 | 8 | 0.8 |
| 0732B&CE233 | Produce sketches of sewage treatment plant | Core | 4 | 1.5 | 9 | 10.5 | 1.05 |
| 0732B&CE234 | Dispose sewage | Core | 4 | 2 | 6 | 8 | 0.8 |
|  | 11.21 Engineering Mechanics |  |  | 16 | 64 | 80 | 8 |
| 0732B&CE235 | Design Homogeneous Beam | Core | 4 | 3 | 12 | 15 | 1.5 |
| 0732B&CE236 | Determine Deflection of Beams | Core | 4 | 2 | 9 | 11 | 1.1 |
| 0732B&CE237 | Determine load carrying capacity of columns | Core | 4 | 3 | 12 | 15 | 1.5 |
| 0732B&CE238 | Evaluate Efficiency of Joints-riveted and welded | Core | 4 | 2 | 9 | 11 | 1.1 |
| 0732B&CE239 | Analyse forces in truss members | Core | 4 | 3 | 12 | 15 | 1.5 |
| 0732B&CE240 | Determine stability of retaining wall | Core | 4 | 3 | 10 | 13 | 1.3 |

National Diploma in Civil Technology Level 5 Part-1

| S# | Competency Standard | Category | Level | Th | Practical | Total | Credit Hour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11.22 Transportation Engineering I |  |  | 64 | 48 | 112 | 11.2 |
| 0732B&CE241 | Develop construction plan of Tunnels | Core | 5 | 8 | 3 | 11 | 1.1 |
| 0732B&CE242 | Produce sketches of an airport | Core | 5 | 8 | 3 | 11 | 1.1 |
| 0732B&CE243 | Produce sketches of permanent Way | Core | 5 | 12 | 12 | 24 | 2.4 |
| 0732B&CE244 | Produce sketches of points and crossings | Core | 5 | 12 | 9 | 21 | 2.1 |
| 0732B&CE245 | Produce sketches of station yards and signals | Core | 5 | 12 | 9 | 21 | 2.1 |
| 0732B&CE246 | Produce sketches of harbour | Core | 5 | 12 | 12 | 24 | 2.4 |
|  | 11.23 Hydraulics Engineering |  |  | 32 | 48 | 80 | 8 |
| 0732B&CE247 | Determine Hydrostatic pressure | Core | 5 | 16 | 21 | 37 | 3.7 |
| 0732B&CE248 | Apply Hydro kinematic Principles | Core | 5 | 16 | 27 | 43 | 4.3 |
|  | 11.24 Soil Mechanics |  |  | 32 | 48 | 80 | 8 |
| 0732B&CE249 | Determine moisture content and specific gravity of soil | Core | 5 | 4 | 6 | 10 | 1 |
| 0732B&CE250 | Determine Atterberg’s Limits | Core | 5 | 4 | 6 | 10 | 1 |
| 0732B&CE251 | Identify type of soil | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE252 | Conduct soil compaction tests | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE253 | Determine permeability of soil | Core | 5 | 4 | 6 | 10 | 1 |
| 0732B&CE254 | Investigate shear strength of soil | Core | 5 | 4 | 6 | 10 | 1 |
| 0732B&CE255 | Find out bearing capacity of soil | Core | 5 | 4 | 6 | 10 | 1 |
|  | 11.25 Bridge Engineering |  |  | 32 | 48 | 80 | 8 |
| 0732B&CE256 | Produce sketches of bridges w.r.t. functions | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE257 | Produce sketches of bridges w.r.t. material of construction | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE258 | Produce sketches of bridges w.r.t. type of super structure | Core | 5 | 8 | 12 | 20 | 2 |
| 0732B&CE259 | Produce sketches of bridges w.r.t. floor type | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE260 | Produce sketches of culverts, low level and high level causeway | Core | 5 | 6 | 9 | 15 | 1.5 |
|  | 11.26 Environmental Technology |  |  | 32 | 48 | 80 | 8 |
| 0732B&CE261 | Determine Water Pollution | Core | 5 | 6 | 12 | 18 | 1.8 |
| 0732B&CE262 | Evaluate Air Pollution | Core | 5 | 6 | 12 | 18 | 1.8 |
| 0732B&CE263 | Determine Noise Pollution | Core | 5 | 3 | 5 | 8 | 0.8 |
| 0732B&CE264 | Evaluate Solid Waste Pollution | Core | 5 | 4 | 3 | 7 | 0.7 |
| 0732B&CE265 | Adopt occupational health and safety | Generic | 5 | 9 | 9 | 18 | 1.8 |
| 0732B&CE266 | Respond to Natural Disaster | Core | 5 | 4 | 7 | 11 | 1.1 |
|  | 11.27. Construction Project Planning & Management |  |  | 64 | 96 | 160 | 16 |
| 0732B&CE267 | Develop Project Management Life Cycle | Core | 5 | 4 | 6 | 10 | 1 |
| 0732B&CE268 | Develop Preliminary Project Plan | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE269 | Develop Construction Project Plan | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE270 | Conduct on-site supervision of construction projects | Core | 5 | 5 | 12 | 17 | 1.7 |
| 0732B&CE271 | Perform Basic operation in Primavera P6 | Core | 5 | 8 | 12 | 20 | 2 |
| 0732B&CE272 | Perform road Project Activities Scheduling in Primavera P6 | Core | 5 | 8 | 12 | 20 | 2 |
| 0732B&CE273 | Perform road Project Resources Costing &Planning in Primavera P6 | Core | 5 | 8 | 12 | 20 | 2 |
| 0732B&CE274 | Manage construction Project in Primavera P6 | Core | 5 | 8 | 12 | 20 | 2 |
| 0732B&CE275 | Identify Project Risk and Consequences on Project life cycle | Core | 5 | 5 | 6 | 11 | 1.1 |
| 0732B&CE276 | Perform Evaluation to assess the severity of Risk and formulate the strategy to manage the risk | Core | 5 | 6 | 6 | 12 | 1.2 |

National Diploma in Civil Technology Level 5 Part-2

| S# | Competency Standard | Category | Level | Th | Practical | Total | Credit Hour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11.28 Transportation Engineering I |  |  | 42 | 48 | 90 | 9.0 |
| 0732B&CE277 | Plan Route Alignment of Highway | Core | 5 | 4 | 6 | 10 | 1.0 |
| 0732B&CE278 | Design Highway Geometrics | Core | 5 | 4 | 6 | 10 | 1.0 |
| 0732B&CE279 | Perform tests on stone metal for pavement construction | Core | 5 | 4 | 6 | 10 | 1.0 |
| 0732B&CE280 | Perform tests on bitumen | Core | 5 | 12 | 21 | 33 | 3.3 |
| 0732B&CE281 | Prepare drawings of flexible/rigid pavement | Core | 5 | 6 | 3 | 9 | 0.9 |
| 0732B&CE282 | Plan highway drainage | Core | 5 | 6 | 3 | 9 | 0.9 |
| 0732B&CE283 | Develop maintenance plan for roads | Core | 5 | 6 | 3 | 9 | 0.9 |
|  | 11.29. Irrigation Engineering |  |  | 64 | 48 | 112 | 11 |
| 0732B&CE284 | Produce sketches of canals and head works | Core | 5 | 21 | 18 | 39 | 3.9 |
| 0732B&CE285 | Design Canals | Core | 5 | 22 | 15 | 37 | 3.7 |
| 0732B&CE286 | Produce sketches of Hydraulic structures | Core | 5 | 21 | 15 | 36 | 3.6 |
|  | 11.30. RCC Design |  |  | 80 | 63 | 144 | 14 |
| 0732B&CE287 | Design and Draw Reinforcement for Beams | Core | 5 | 14 | 12 | 26 | 2.6 |
| 0732B&CE288 | Design and Draw Reinforcement for Slabs | Core | 5 | 24 | 15 | 39 | 3.9 |
| 0732B&CE289 | Design and Draw Reinforcement for Columns | Core | 5 | 19 | 11 | 30 | 3 |
| 0732B&CE290 | Design and Draw Reinforcement for Footings | Core | 5 | 8 | 10 | 18 | 1.8 |
| 0732B&CE291 | Design and Draw Reinforcement for Stairs | Core | 5 | 15 | 15 | 30 | 3 |
|  | 11.31. Civil Engineering Project (Any one project to a group of 8 students |  |  | 0 | 138 | 138 | 13.8 |
| 0732B&CE292 | Develop Project for construction of RCC framed Structure Class Room. | Core | 5 | 0 | 138 | 138 | 13.8 |
| 0732B&CE293 | Develop Project plan for construction of RCC OHR of 10000 gal (UK) capacity. | Core | 5 | 0 | 138 | 138 | 13.8 |
| 0732B&CE294 | Develop Project plan for construction of Water Supply Scheme. | Core | 5 | 0 | 138 | 138 | 13.8 |
| 0732B&CE295 | Develop Project plan for construction of sewerage scheme | Core | 5 | 0 | 138 | 138 | 13.8 |
| 0732B&CE296 | Develop Project plan for construction of Highway | Core | 5 | 0 | 138 | 138 | 13.8 |
| 0732B&CE297 | Develop Project plan for construction of Irrigation Canal | Core | 5 | 0 | 138 | 138 | 13.8 |
|  | 11.32. Entrepreneurial Skills |  |  | 32 | 63 | 95 | 9.5 |
| 0732B&CE298 | Investigate micro business opportunities | Generic | 5 | 3 | 6 | 9 | 0.9 |
| 0732B&CE299 | Develop a micro business proposal | Generic | 5 | 4 | 8 | 12 | 1.2 |
| 0732B&CE300 | Develop a marketing plan and business plan. | Generic | 5 | 7 | 15 | 22 | 2.2 |
| 0732B&CE301 | Organize finances for the micro business | Generic | 5 | 4 | 8 | 12 | 1.2 |
| 0732B&CE302 | Manage human resources | Generic | 5 | 3 | 6 | 9 | 0.9 |
| 0732B&CE303 | Market products and services | Generic | 5 | 4 | 8 | 12 | 1.2 |
| 0732B&CE304 | Monitor and review business performance | Generic | 5 | 4 | 8 | 12 | 1.2 |
| 0732B&CE305 | Negotiate for resolving business issues | Generic | 5 | 3 | 4 | 7 | 0.7 |
|  | 11.33. Green Skills |  |  | 6 | 24 | 30 | 3 |
| 0732B&CE306 | Perform green skills in construction | Generic | 5 | 6 | 24 | 30 | 3 |

# 4. Date of Validation

The level 5 of National DAE qualification on Civil Technology has been validated by the Qualifications Development Committee (QDC) members on 27-29 May 2019 and will remain in currency until May, 2022

# 5. Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Civil Technology level-5 |  |
| --- | --- |
| Code | Description |
| 000000000 | National Vocational Certificate of level1& 2, Site assistant in “Civil Technology” |
| 000000000 | National Vocational Certificate of level-3,Assistant surveyor in “Civil Technology” |
| 000000000 | National Vocational Certificate of level-4 Part-I, Supervisor in “Civil Technology” |
| 000000000 | National Vocational Certificate of level-4 Part-II, Supervisor in “Civil Technology” |
| 000000000 | National Diploma in Cicil Technology Level-5 Part-I |
| 000000000 | National Diploma in Civil Technology Level-5 Part-II |

# 6. Members of Qualifications Development Committee

The following members participated in the qualification’s development and validation of these qualifications:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Azhar Iqbal Shad | Chief Instructor (Civil) | GCT Raiwnd Road, LHR |
| 2 | Engr. Mazhar Abbas Naqvi | Principal | GSTC, Faisalabad |
| 3 | Engr. Naveed Ahmad Ajmal | Sr. Instructor (Civil) | GCT Bahalpur |
| 4 | Imtiaz Ahmad Awan | Sr. Instructor (Civil) | GCT Rasul (M.B.Din) |
| 5 | Muhammad Amjad Rafique | Sr. Instructor (Civil) | GCT Rasul (M.B.Din) |
| 6 | Engr.Zuneera Ashfaq | Asstt Director | PITAC Lahore |
| 7 | Engr Rehan Younis | Asstt Director | FWO- Rawalpindi |
| 8 | Engr. Mohsin Jahanzeb | Instructor (Civil) | GSTC, Faisalabad |
| 9 | Muhammad Asad Mahmood | Contractor | G.Five Construction Co. Faisal Abad |
| 10 | Engr. Norheen Amina | Consultant | Allied Engg. Services Lahore |
| 11 | Hira Ishtiaq | Coordinator Engr. | Allied Engg. Services Lahore |
| 12 | Engr. Hafsah Khalid | Project Manager | ANS Associates Lahore |
| 13 | Engr. Habiba Mohsin | Project Manager | Jers Engg. Co. Lahore |
| 14 | Mian Muhammad Naeem | Consultant Engr. | Mian Associates Lahore |
| 15 | Engr. Rana Haroon Mujahid | SDO, Bridges & Roads | C&W Faisalabad |
| 16 | Bushra Asghar | Instructor | GCT (w) Lahore |
| 17 | Engr. Inayet ur Rehman | DACUM, Facilitator | Peshawar |
| 18 | Muhammad Ishaq | QDC Coordinator | NAVTTC, Islamabad |

# 7. Members of Qualifications Validation Committee

The following members participated in the qualification development and validation of these qualifications:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Azhar Iqbal Shad | Chief Instructor (Civil) | GCT Raiwnd Road, LHR |
| 2 | Engr. Mazhar Abbas Naqvi | Principal | GSTC, Faisalabad |
| 3 | Mr. Imtiaz Ahmad Awan | Sr. Instructor (Civil) | GCT Rasul (M.B.Din) |
| 4 | Mr. Tariq Ali | A / Professor (Civil) | GPI Batkhela KP-TEVTA |
| 5 | Mr. Fida Hussain Mirjat | Principal | GMI Matli Hyderabad |
| 5 | Mr. Muhammad Khurram Murad | DCE PBTE | Lahore |
| 6 | Engr. Inayat ur Rehmen | DACUM Facilitator | Ex. KP-TEVTA |
| 7 | Mr. Sikander Masood | Director SS & C. | NAVTTC Islamabad |

# 8. Entry Requirements

| National Vocational Certificate Level 1 & 2, Site assistant in Civil Technology | Middle |
| --- | --- |
| National Vocational Certificate Level 3, Assistant Surveyor in Civil Technology | Level 1 & 2 |
| National Vocational Certificate Level 4, Site technology in Civil Technology | Level 3 |
| National Diploma in Civil Technology Level 5, | Level 4 |

# 9. Proposed Scheme of Studies

(000000000) National Vocational Certificate Level 1 & 2 (Site assistant in Civil Technology)

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 000000000 | Basic Engineering Surveying | Technical | 32 | 160 | 192 | 19.2 | 2 | 10 | 12 |
| 000000000 | Civil Engineering Drawing | Technical | 32 | 160 | 192 | 19.2 | 2 | 10 | 12 |
| 000000000 | Building Materials | Technical | 32 | 48 | 80 | 8 | 2 | 3 | 05 |
| 000000000 | Workshop Practice-I (Wood Working) | Technical | 16 | 48 | 64 | 6.4 | 1 | 3 | 4 |
| 000000000 | Workshop Practice-II (Electrical Wiring) | Technical | 16 | 48 | 64 | 6.4 | 1 | 3 | 4 |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| Total |  |  | 128 | 464 | 592 | 59.2 | 8 | 29 | 37 |

(00000000) National Vocational Certificate Level 3 (Assistant Surveyor in Civil Technology)

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 000000000 | Advanced Surveying | Technical | 48 | 192 | 240 | 24 | 3 | 12 | 15 |
| 000000000 | AutoCAD | Technical | 32 | 128 | 160 | 16 | 2 | 8 | 10 |
| 000000000 | Construction Techniques I | Technical | 16 | 112 | 128 | 12.8 | 1 | 7 | 8 |
| 000000000 | Plumbing | Technical | 16 | 32 | 48 | 4.8 | 1 | 2 | 3 |
| 000000000 | Digital Skills | Technical | 16 | 16 | 32 | 3.2 | 1 | 1 | 2 |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| Total |  |  | 128 | 480 | 608 | 60.8 | 8 | 30 | 38 |

(000000) National Vocational Certificate Level 4 (Site supervisor in Civil Technology) Part 1

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 000000000 | Construction Techniques II | Technical | 32 | 128 | 160 | 16 | 2 | 8 | 10 |
| 000000000 | Basic Quantity Surveying | Technical | 16 | 176 | 192 | 19.2 | 1 | 11 | 12 |
| 000000000 | Water Supply | Technical | 16 | 64 | 80 | 8 | 1 | 4 | 5 |
| 000000000 | Strength of Materials | Technical | 16 | 64 | 80 | 8 | 1 | 4 | 5 |
| 000000000 | Soft Skills | Technical | 32 | 64 | 96 | 9.6 | 2 | 4 | 6 |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| Total |  |  | 112 | 496 | 608 | 60.8 | 7 | 31 | 38 |

(000000) National Vocational Certificate Level 4 (Site supervisor in Civil Technology) Part-2

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 000000000 | Construction Techniques III | Technical | 16 | 64 | 80 | 8 | 1 | 4 | 5 |
| 000000000 | Concrete Technology | Technical | 32 | 64 | 96 | 9.6 | 2 | 4 | 6 |
| 000000000 | Advanced Quantity Surveying | Technical | 16 | 144 | 160 | 16 | 1 | 9 | 10 |
| 000000000 | Contract Management | Technical | 32 | 64 | 96 | 9.6 | 2 | 4 | 6 |
| 000000000 | Sanitary Engineering | Technical | 16 | 64 | 80 | 8 | 1 | 4 | 5 |
| 000000000 | Engineering Mechanics | Technical | 16 | 64 | 80 | 8 | 1 | 4 | 5 |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| Total |  |  | 128 | 464 | 592 | 59.2 | 8 | 29 | 37 |

(000000) National Diploma in Civil Technology Level 5 (Part 1)

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 000000000 | Transportation Engineering-I | Technical | 64 | 48 | 112 | 11.2 | 4 | 3 | 7 |
| 000000000 | Hydraulics Engineering | Technical | 32 | 48 | 80 | 8 | 2 | 3 | 5 |
| 000000000 | Soil Mechanics | Technical | 32 | 48 | 80 | 8 | 2 | 3 | 5 |
| 000000000 | Bridge Engineering | Technical | 32 | 48 | 80 | 8 | 2 | 3 | 5 |
| 000000000 | Environmental Technology | Technical | 32 | 48 | 80 | 8 | 2 | 3 | 5 |
| 000000000 | Construction Project Planning and Management | Technical | 64 | 96 | 160 | 16 | 4 | 6 | 10 |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| Total |  |  | 256 | 336 | 592 | 59.2 | 16 | 21 | 37 |

(000000) National Diploma in Civil Technology Level 5 (Part-2)

| Code | Name of Subjects | Category | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 000000000 | Transportation Engineering-II | Technical | 42 | 48 | 90 | 9.0 | 3 | 3 | 6 |
| 000000000 | Irrigation Engineering | Technical | 64 | 48 | 112 | 11.2 | 4 | 3 | 7 |
| 000000000 | R.C.C. Design | Technical | 80 | 63 | 143 | 14.3 | 5 | 4 | 9 |
| 000000000 | Civil Engineering Project | Technical | 0 | 138 | 138 | 13.8 | 0 | 10 | 10 |
| 000000000 | Entrepreneurial Skill | Generic | 32 | 63 | 95 | 9.5 | 2 | 4 | 6 |
| 00000000 | Green Skills in construction | Generic | 6 | 24 | 30 | 3 | 1 | 3 | 4 |
| 00000000 |  | Allied |  |  |  |  |  |  |  |
| 000000000 |  | Allied |  |  |  |  |  |  |  |
| Total |  |  | 224 | 384 | 608 | 60.8 | 15 | 27 | 42 |

# 9. A- Occupation Packaging of Civil Technology

| S.# | Name of Occupation | Level | Digital Skills | Soft Skills | Technical Skills |
| --- | --- | --- | --- | --- | --- |
|  | Civil Surveyor | 3 | 0732B&CE- 109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-1 to 21, 53 to 66 |
|  | Draftsman | 3 | 0732B&CE-109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-11 to 24, 67 to 78 |
|  | Quantity Surveyor | 5 | 0732B&CE- 109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-6, 8, 10, 11 to 25, 67 to 78, 151 to 169, 206 to 218, 219 to 234, 241 to 246, 256 to 260, 277 to 286 |
|  | Quantity Surveyor after Draftsman | 5 | ----- | ----- | 0732B&CE-6, 8, 10, 25 to 35, 151 to 169, 206 to 218, 219 to 234, 241 to 246, 256 to 260, 277 to 286 |
|  | Materials Technician | 5 | 0732B&CE- 109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-25, 173, 174, 206 to 211, 249 to 255, 261 to 265, 279, 280. |
|  | Construction Quality control Supervisor | 5 | 0732B&CE-109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-1 to 21, 25 to 35, 53 to 66, 173, 174, 206 to 211, 249 to 255, 261 to 265, 267 to 274, 279, 280 |
|  | Mason | 2 | ------ | ------ | 0732B&CE-25 to 35, 79 to 100, 131 to 150, 189 to 194, 199, 203, 204. |
|  | Plumber | 2 | ----- | --- | 0732B&CE-101 to 108, 164, 228, 230, 231 |
|  | Domestic Electrician | 2 | ------ | ------ | 0732B&CE-46 to 52 |

# B- Qualification Packaging of Civil Technology

| S.# | Name of Qualification | Level | Digital Skills | Soft Skills | Technical Skills |
| --- | --- | --- | --- | --- | --- |
| 1 | Site supervisor | 4 | 0732B&CE- 109 to 130 | 0732B&CE-175 to 188 | 0732B&CE-1 to 24, 25 to 35, 36 to 45, 46 to 52, 53 to 66, 67 to 78, 79 to 100, 101 to 108, 131 to 150, 164 to 169, 189 to 205, 206 to 209, 226 to 234, 241 to 246, 256 to 260, 277 to 286 |

# Detail of Qualifications and its Competency Standards Level Wise

# 11. B- Certificate in Civil Technology Level 3

  - Advanced Surveying

- . Measure Angles by Theodolite

Overview: This competency standard covers the skills and knowledge required to perform angle measurement of different angles by Theodolite with its basic components. It also covers the knowledge by using Theodololite Surveying.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Perform Temporary adjustment of theodolite. | - Identify the Equipment - Set Theodolite on tripod at station point - Perform centering and levelling on station point |
| CU-2. Measure Horizontal Angle by Repitition Method | - Adjust theodolite on station point O - Set angle to 0⁰-0’-0”at given point A - Record reading of angle at point B as angle AOB - Repeat angle measurement three times for each face - Calculate mean of the readings |
| CU-3. Measure Horizontal Angle by Reiteration Method | - Adjust theodolite on station point O - Set angle to 0⁰-0’-0”at given point A - Bisect the second point B, C and D to record angle readings between AOB, BOC and COD - Close the horizon by bisecting the D to A to check if angle is 3600 - Adjust error according to standard procedure - Repeat the procedure by changing face of the theodolite. |
| CU-4. Measure Vertical Angle measurement | - Adjust theodolite on station - Set theodolite for vertical angle measurement - Record vertical angle by sighting the object - Take 2 more readings for mean |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

    - Define theodolite, sketch & label theodolite and list the types of theodolites

    - Define the terms; centering, transiting, face left, Face right, swinging the telescope,  axis of level tube, horizontal & vertical axis

    - Explain the procedure of temporary adjustment of a theodolites

    - Explain the procedure of measuring horizontal & vertical angles.

    - Explain the procedure of setting out an angle and prolonging a line

    - Explain the procedure of fixing inter-mediate points between two given points

    - Explain various methods of traversing and enlist steps involved in traversing

    - State the standards of accuracy in linear and angular measurement.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare layout by theodolite taking angles

- Calculate area by theodolite survey

- Fold and unfold the tripod

- Take temporary adjustment for Theodolite

- Focus eye piece for Theodolite

- Take and measure the angles

- calculate vertical & horizontal angles

- Carry out appropriate checks

- Undertake setting out in the field using Theodolites, tapes,

- Know Horizontal control and Vertical control

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Theodolite with tripod

- Measuring scale,

- Measuring Tape,

- Ranging Rods

- Pegs

- White Chalk

- Mallet

- Umbrella with stand

- Arrows

- Thread (Dori)

- Scientific Calculator

- Rough Copy

- Perform Trigonometric Levelling by Theodolite

Overview: This competency standard covers the skills and knowledge required to perform process of determining the differences of elevations of stations from observed vertical angles and known distances. The vertical angles are measured by means of theodolite. Measurements are to be carried out in two or sometimes three stages. The said leveling is to be done by Theodolite.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1.Carry out Trigonometric Leveling (base of point is accessible) | - Adjust theodolite on a station point from where altitude point is accessible - Set theodolite for vertical angle measurement - Take reading on bench mark - Record vertical angle of point to whom elevation is to be determined - Measure horizontal distance from the station to the point - Calculate elevation of point by trigonometry. |
| CU-2. Carry out Trigonometric Levelling (base of point is inaccessible) | - Mark two station points in a line - Adjust theodolite on a station point closer to point of observation - Set theodolite for vertical angle measurement - Record vertical angle from closer station point - Adjust theodolite on second station point which is farther from point but in line of first station point - Record vertical angle from second station point - Measure distance between station points - Calculate elevation of point by trigonometry. |
| CU-3. Carryout Trigonometric Leveling (base of point is inaccessible and no plane surface is available nearby) | - Adjust theodolite on a station point in depression and record height of trunion axis. - Hold the levelling staff at higher level and read vertical angle. - Record stadia hairs readings and line of collimation reading. - Calculate the horizontal and inclined distances. - Calculate elevation of point by trigonometry. |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe tacheometry and trigonometry

- Explain the principles of tachometry and enlist the method of tacheometry

- Describe the instruments used in stadia survey and state tacheometric constants

- Lists the steps involved in taking stadia observations in field to find elevations and distances of stations and compute the elevation and horizontal distances.

- Solve examples for finding horizontal and vertical distances by tachometry Checks on open and closed traverse

- Explain the method of plotting traverse and adjustment of closing error.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Plan the work station

- Check base line.

- Fold and unfold the tripod

- Prepare plan for triangulation with both method.

- Fix the tripod with plumb bob.

- Understand sighting by sight vane.

- Plot the traverse and calculate area

- Measure horizontal distance

TOOLS AND EQUIPMENT’S

- Calculator,

- Measuring scale,

- Drawing sheet,

- Measuring Tape,

- Tripod

- Ranging Rods

- Pegs

- Pencil

- Eraser

- Field Book

- Thread ( Dori)

- Umbrella with stand

- White chalk

- Vernier / Digital Theodolite

- Perform Traversing by Theodolite

Overview: This competency standard covers the skills and knowledge required to perform Traverse in the field of surveying to establish control networks. It is also used in geodesy. Traverse networks involve placing survey stations along a line or path of travel, and then using the previously surveyed points as a base for observing the next point.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Cary out area of traversing (by digital theodolite) | - Mark station points which would cover area - Record bearing of first line by prismatic compass - Adjust theodolite on first station point - Record horizontal internal or external angle between lines originating from first station point - Shift theodolite to the next stations - Record horizontal internal or external angles between lines - Measure the distances between the station points - Compile the data |
| CU-2. Carry out calculations in Gale’s Traverse Table | P1. Compile the data in Gale’s Table P2. Apply necessary corrections P3. Calculate co-ordinates P4. Convert the dependent co-ordinates into independent co-ordinates P5. Calculate the area by coordinates method |
| CU-3. Plot traverse of the area | P1. Fix the sheet on drawing board with relevant orientation. P2. Use drawing instruments to plot the control points by use of coordinate with appropriate scale. P3. Draw the survey lines. P4. Perform relevant lettering. |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain traverse & its types, methods of traversing

- Explain the objects and standard of accuracy of traversing

- Check of open & closed traverse

- Know plotting & graphical adjustment of closing error

- Calculate angles from given bearings and vice versa

- Explain coordinates its types and technical terms.

- Compute co-ordinates and computation of missing data associated with theodolites traversing

- Understand the balancing a  traverse by different methods

- Explain a range of specialized surveying techniques.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Set the temporary adjustment of theodolite.

- Set out coordinated points from existing control stations.

- Calculate distance from coordinates

- Carry out appropriate checks by formula

- Undertake setting out in the field using theodolite, tapes and Ranging Rods.

- Fold and unfold the tripod

- Prepare traverse by theodolite

- Focus the eyepiece of the theodolite

- Take and measure readings

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Field Book

- Sharpner,

- Calculator,

- Measuring scale,

- Measuring steelTape,

- Digital / vernier Theodolite

- Tripod

- Ranging Rods

- Pegs

- Arrows

- Thermometer

- Ball pein hammer

- Umbrella with stand

- Set Out Horizontal Curves by Theodolite

Overview: This competency standard covers the skills and knowledge required to set Horizontal Curves of the two important transition elements in geometric design for highways. A horizontal curve provides a transition between two tangent strips of roadway, allowing a vehicle to negotiate a turn at a gradual rate rather than a sharp cut.

| Competency Units | Performance Criteria |
| --- | --- |
| - Set out Horizontal Curve by Taking offsets from Long Chord | - Compute and compile data for setting out curve by taking offsets from long chord. - Mark Tangent points T1 & T2 and Intersection point for setting out curve with the help of theodolite - Mark tangent lines and long chord with string and lime. - Locate points of curve by taking normal offsets from long chord at specified points. - Lay curve by joining points with lime. |
| - Set out Horizontal Curve by Taking offsets from Tangent | - Compute and compile data for setting out curve by Taking offsets (radial and square) from Tangent - Mark Tangent points T1 & T2 and Intersection point for setting out curve with help of theodolite - Mark tangent lines and long chord with string and lime. - Locate points of curve by taking normal offsets from tangent - Locate points of curve by taking radial offsets from tangent - Lay curve by joining points with lime |
| - Set out Horizontal Curve by deflection angle method (Rankine’s method) | - Compute and compile data for setting out curve by deflection angle method - Mark Tangent points T1 & T2 and Intersection point for setting out curve with the help of theodolite - Mark tangent lines and long chord with string and lime. - Locate points of curve by relevant deflection angle and sub-chord with the help of theodolite and tape. - Lay curve by joining points with lime. |
| - Set out transition curve in the field | - Compute and compile data for setting out transition curve - Set two points T1, T2 at shift of curve - Erect pole in direction of intersection point from T1 and T2. - Mark the shift points for both tangents. - Start setting out from first tangent point T1 - Start setting out from second tangent point T2 - Set out circular curve - Lay curve by lime |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define a curve and differentiate its types. Describe necessity of providing curves.

- Define designation of curves

- Describe elements and notation of simple circular curves & their relationship

- Calculate the data & apply the methods of setting out simple circular curves

- Setting out simple curve beyond obstacles

- Define transition curves and explain its necessity. State Characteristics, elements & notation of transition curve and their inter relationship

- Explain  super elevation and methods of calculation the length of transition curve with numerical problems

- Calculate the data and apply the methods of setting out of transition curve

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Calculate deflection angles for curves

- Set out curves by theodolite

- Fold and unfold the tripod

- Find out Tangent length

- Focus the eyepiece

- Take and measure the chord length

- Adjust temporarily of the theodolite

- Compute Reduced Levels

- Find out radial / perpendicular off sets

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Scientific Calculator,

- Measuring scale,

- Measuring Steel Tape,

- Tripod

- Ranging Rods

- Pegs

- Arrows

- Umbrella with stand

- Theodolite

- Staff

- White Chalk

- Thermometer

- Ball pein hammer

- Field Book

- Set out Vertical Curves by Theodolite

Overview: This competency standard covers the skills and knowledge required to perform Vertical Curves of the two important transition elements in geometric design for highways. A vertical curve provides a transition between two sloped roadways, allowing a vehicle to negotiate the elevation rate change at a gradual rate rather than a sharp cut.

| Competency Units | Performance Criteria |
| --- | --- |
| - Interpret to set out vertical curve | - Compute and compile data for setting out curve - Identify gradients and rate of change of gradient - Locate point of intersection - Calculate length of vertical curve - Calculate tangent corrections - Find out levels of curve - Draw vertical curve |
| - Set out Vertical Curve in the field | - Set the theodolite on first tangent point T1 - Locate second tangent point T2 - Fix the poles at intermediate points - Perform leveling to find NSL - Mark the point on pole by using the difference in NSL |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain vertical curves

- Explain the types of vertical curves, elements, terms and their inter relationship.

- Calculate the data and apply Setting out vertical curves

- Apply various steps in setting out of combined curves

K6.       Explain Theodolite with its parts.

- Compute, tabulate and set out data for the road alignment and Curves.

- Explain the purpose of the specialized surveying technique

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Take RL’s by leveling instrument

- Apply survey techniques

- Compute road alignment data

- Carry out appropriate check

- Pin and unpin survey instrument from the box.

- Prepare long section diagram.

- Apply formula to calculate earthwork for site.

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Scientific Calculator,

- Measuring scale,

- Field Book,

- Steel Tape,

- Level (Digital)

- Tripod

- Ranging Rods

- Pegs

- Arrows

- Staff

- White Chalk

- Thermometer

- Ball pein hammer / Mallet

- Umbrella with stand

- Carry out Triangulation by Theodolite

Overview: This competency standard covers the skills and knowledge required to perform series or network of triangles in order to determine the distances and relative positions of points spread over an area, especially by measuring the length of one side of each triangle and deducing its angles and the length of the other two sides by observation from this baseline.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Mark station points by dividing the area by triangles | P1. Carry out reconnaissance of area P2.Mark stations covering the area P2. Divide the area in triangles P3. Fix a base line |
| CU-2. Measure the base line | P1.Identify equipment P2.Measure the base line P3.Apply corrections |
| CU-3. Measure the angles for triangulation | P1. Identify equipment P2.Fix theodolite on a station point P3.Measure the angles P4.Shift the instrument on next station point P5.Measure all the required angles |
| CU-4. Calculate the area | P1. Compile data obtained from survey P2. Calculate missing data P3. Apply formula P4. Calculate the area |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain triangulation and describe the types of triangulation.

- Explain the methods to solve Triangles

- Understand the  Sine rule and its application in triangulation

- Define well condition and ill condition in Triangulation

- Understand the selection of station points

- Know measurement of base line

- Apply correction of base line measurement in triangulation

- Explain the purpose of the specialized surveying technique

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare layout of survey for calculation of area

- Take base line

- Apply survey techniques

- Compute data for entire area

- Carry out appropriate checks

- Pin and unpin the theodolite from the box.

- Calculate angles.

- Focus the eyepiece of Theodolite.

- Take readings and measure the angles by Theodolite.

TOOLS AND EQUIPMENT’S

- Pencil,

- Eraser,

- Sharpner,

- Scientific Calculator,

- Measuring scale,

- Field Book,

- Steel Tape,

- Theodolite,(Digital)

- Tripod

- Ranging Rods

- Pegs

- Arrows

- Staff

- White Chalk

- Thermometer

- Ball pein hammer / Mallet

- Umbrella with stand

- Perform Temporary Adjustment of Total Station

Overview: This competency standard covers the skills and knowledge required to Temporary adjustment of total station on a station point. It is set of operations which are performed on a total station to make it ready for taking observations. These include its initial setting up on a tripod or other stand, centering, leveling up and focusing of eyepiece.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Adjust temporarily total station on a given point | P1. Identify the equipment P2. Unfold tripod P3. Perform centering of total station at station point P4. Perform leveling of total station P5. Adjust foot screws & center the bubble P6. Focus the telescope |
| CU-2. Set the Total station Electronically | P1. Set instrument on station point P2. Fix battery P3. Turn on instrument P4. Show opening screen as “MEAS” screen. P5. Adjust foot screw P6. Rotate instrument 90 degree P7. Repeat method and check |
| CU-3. Set co-ordinates of occupied station | P1. Set instrument on station point P2. Turn on instrument P3. Select co-ordinate menu P4. Input co-ordinates. P5. Check Result |

Knowledge & Understanding

The candidate must be able to develop skills in carrying out setting of total station and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Identify the total station, its parts, functions and modes

- Identify and describe the accessories of total station and their operation

- Describe the parameters, setting of total station and its operation.

- Explain the methods of co-ordinates entry in the total station.

- Explain to connect and disconnect its battery

- Explain to change of “Face”

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Draw sketch of total station with labels

- Explain detailed procedure of setting up total station

- Explain detailed parts of total station

Prepare  calculation sheet for traversing

- Observe the required traverse data to an acceptable level of accuracy

- Calculate the co-ordinates (3D) of the traverse stations

- Plot a scale drawing of the traverse plan

- Change the “face” of total station

Tools and Equipment

- Ranging rods

- Prism with Prism Pole

- Total station

- Measuring tapes

- Pegs

- Mallet

- Battery Charger

- Tripod

- Scientific Calculator

- Field Book

- Measure Distance by Total Station

Overview: This competency standard covers the skills and knowledge required to identify the   horizontal, vertical and inclined measurements taken by Total Station with its components.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1 Measure distance by Total Station (Horizontal & Vertical) | P1. Set up total station on station point & install battery P2. Show opening screen at measuring mode P3. Input the co-ordinates of the station point P4. Set the prism on the required point P5. Record readings P6. Recheck distance by steel tape |
| CU-2 Measure distance by Total Station (Inclined Distance) | P1. Set up total station on station point & install battery P2. Show opening screen at inclined measurement P3. Set the prism on the required point P4. Adjust Total station toward Prism P5. Record readings on screen P5. Recheck distance by invar tape |
| CU-3 Stack out different point at various distances on a line | P1. Set up total station on station point P2. Select stack out menu P3. Select the method to stake out the point P4. Select "XY" to stake out by coordinates P5. Use the keypad to enter the coordinates or distances |

Knowledge & Understanding

The candidate must be able to develop skills in carrying out survey by Total station and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Identify the Total station, its Parts and describe its types

- Describe the functions and modes of Total station

K3.  Explain the methods to set the Total Station at station point

K4.  Describe the parameters, setting of total station and its operation.

K5.  Explain the methods of measurement of distances, angles, bearings and co-ordinates by total station.

K6.   Explain the procedure to measure the height of an object with total station by  using

REM

K7.  Setting and calculation co-ordinate value of occupied station, setting height of

instrument & height of reflector

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Sketch of Total Station and label its parts

- Understand detailed procedure of seting up instrument for measurments

- Apply detailed proedure of measuring techniques

- Explain detail procedure to set the prism

- Explain detail procedure of  use of hand held distometer

Tools and Equipment

- Total Station (single frequency and hand held)

- Thermometer

- Ranging rods

- Prism with Prism Pole

- Total station

- Measuring tapes

- Pegs

- Mallet

- Umbrella with stand

- Lime Powder

- Thread (Dori)

- Arrows

- Battery Charger

- Measure Angles by Total Station

Overview: This competency standard covers the skills and knowledge required to identify the Total station with its components and function. It also cover the Kinematics of techniques for using Total Station in the field along with its safety precautions.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Measure horizontal angles by Total Station | P1. Identify station points P2. Set total station on intermediate station point P3. Fix ranging rod on the first point P4. Fix ranging rod on the other point P5. Sight first point P6. Set 0˚-0’-0” on total station for horizontal angle P7. Sight the other point P8. Record the measurement P9. Repeat by changing face |
| CU-2. Measure vertival angles by Total Station | P1. Select the station point P2. Set total station on station point P3. Sight the first point P4. Set 0˚-0’-0” on total station for vertical angle P5. Sight the prism P6. Record measurement for vertical angle P7. Set percentage for Angle |
| CU-3 Measure the height of an object with total station by using REM | P1. Set up total station on station point P2. Place the mirror target on the ground at the corner of the building P3. Record the mirror target height from the range pole scale P4. Set target height by setting target height P5. Lock position with horizontal and vertical clamps P6. Take observation using REM menu P7. Unlock the vertical clamp and sight in the top of the building corner P8. Re-lock vertical clamp P9. Record height by using REM |
| CU-4. Measure the distance between different station points with the help of occupied point (MLM) by using measure data or by using coordinate data | P1. Set up total station on station point P2. Place the mirror on reference point P3. Measure the reference point P4. Aim at the first target point P5. Measure the target point P6. Aim at the second target point P7. Measure the second target point P8. Record results |

Knowledge & Understanding

The candidate must be able to develop skills in carrying out and evaluating angles by Total station (vertical, Horizontal and Inclined). It also give the knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1.  Identify the setting of Total Station

K2.  Describe the functions and modes of Total station

K3.  Explain the different parameters of Total Station.

K3. Explain the methods of observations for Total Station.

K4.  Explain the methods for Safety Precaution while using Total Station along with

its components.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Know detailed procedure of seting up instrument for measurments of angles

- Undertand detailed proedure of measuring techniques

- Know fixing the scale of Total Staion at  00.00

- Understand fixing of Prism

- Calculate angles

- Apply formula on Total Station

Tools and Equipment

- Ranging rods

- Prism with Prism Pole

- Total station

- Measuring tapes

- Pegs

- Mallet

- Umbrella with stand

- Measure Area by Total Station

Overview: This competency standard covers the skills and knowledge required to adjust total station on a station point temporarily and to find out the area of plots and transfer data into computer by networking method.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Measure the area of a traverse by direct field observations | P1. Set up total station on station point P2. Create a file name using file management P3. Enter instrument height and prism code after setting zero against point number or name P4. Measure first point and save it P5. Turn the total station to second point P6. Repeat the steps to the rest of the stations P7. Calculate area by using functions of total station |
| CU-2. Measure area for rectangular Plot | P1. Set up the total station P2. Turn on total station P3. Position the horizontal circle. P4. Select boundary points P5. Apply area command P6. Show Area P7. Recheck Mathematically |

Knowledge & Understanding

The candidate must be able to develop skills in carrying out and evaluating angles by Total station (vertical, Horizontal and Inclined) It also give the knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1.  Identify the setting of Total Station

K2.  Describe the functions and modes of Total station

K3.  Explain the different parameters of Total Station.

K3. Explain the methods of observations for Total Station.

K4.  Explain the methods for Safety Precaution while using Total Station along with

its components.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Explain detailed procedure of seting up instrument for measurments of angles

- explain detailed proedure of measuring techniques

- Fix the scale of Total Staion at  00.00

- Understand  the fixing of Prism

- Measure area for plots

Tools and Equipment

- Ranging rods

- Prism with Prism Pole

- Total station

- Measuring tapes

- Pegs

- Mallet

- Umbrella with stand

- Scientific Calculator

- Set out Curves by Total Station

Overview: This competency standard covers the skills and knowledge required to adjust total station on a station point temporarily and to lay the curves with its different methods in Railway and Roads. It also helps to transfer data into PC.

| Competency Units | Performance Criteria |
| --- | --- |
| - Set horizontal curve by Total Station (by Coordinates) | - Compute and compile data for setting out curve - Locate station points by total station - Adjust total station on a station point - Adjust total station to the S-O Arc function. - Select the type of curve. - Enter coordinates of Total Station, Point of Curvature, Point of Tangency, direction of arc, Radius of arc etc. - Give the distance of first point on arc and mark the point on earth. - Give the distances of other points on arc and mark the point on earth. - Lay out the curve by joining point with lime. |
| - Set Vertical curve by Total Station | - Compute and compile data for setting out curve - Locate station points by total station - Adjust total station on a station point - Adjust total station to the S-O Arc function. - Select the type of curve. - Enter coordinates of Total Station, Point of Curvature, Point of Tangency, sag or hog of arc, gradient of arc etc. - Give the distance of first point on arc and mark the point on earth with long pegs/ poles etc. - Give the distances of other points on arc and mark the point on earth with long pegs/ poles etc. |

Knowledge & Understanding:

The candidate must be able to develop skills in carrying out and evaluating curves by Total station (vertical and horizontal curves). It also gives the knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1.  Explain curves with its types

K2.  Draw the curves by Total station

K3.  Explain the different parameters of Total Station performing curves.

K4. Explain the methods of observations for Total Station at curves setting

K5.  Explain the methods for Safety Precaution while using Total Station along with

its components.

K6.  Explain Chords and Tangents

K7.  Explain the method to transfer data into PC

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Apply  detailed procedure of seting up instrument for laying of curves in field

- Apply  detailed procedure for laying different type of curves with its techniques

- Fix the scale of Total Staion at  000.000

- Unertake fixing of Prism at relevant places

- Adjust chord for Laying curves

- Perfom Tangent offset for curves

- Understand the importance of curves and set Total Station on Tangent and Chord Mode

- Perform data transfer into PC

Tools and Equipment

- Ranging rods

- Prism with Prism Pole

- Total station

- Measuring tapes

- Pegs

- Mallet

- Umbrella with stand

- Personal Computer

- Battery charger

- Lime Powder

- Dori (Thread)

- Perform Integration of GPS

Overview: This competency standard covers the skills and knowledge required to Identify the parts of GPS, Identify modes/functions of GPS, traverse the area by static techniques, traverse the area by Stop & Go techniques and traverse the area by Real Time Kinematics techniques. Its also covers a unique position of a specific feature. With this information, one can navigate back to it. However, one cannot relate this “feature position” to any other “feature position” unless one is standing at the site and other features are visible.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Identify parts of GPS | P1. Identify GPS P2. Identify type of GPS P3. Identify parts of GPS P4. Install battery to GPS |
| CU-2. Identify modes / functions of GPS | P1. Identify modes of GPS P2. Indicate functions of GPS P3. Identify parameters of GPS P4. Identify sources of Errors in GPS |

Knowledge & Understanding

The candidate must be able to develop skills in carrying out the working of GPS to evaluate and draw curves, calculate areas, traversing by Global Positioning System through internet. It also gives the knowledge and understanding to carry out tasks and concept is based on time and the known position of GPS specialized satellites. The satellites carry very stable atomic clocks that are synchronized with one another and with the ground clocks. In the same manner, the satellite locations are known with great precision. GPS receivers have clocks as well, but they are less stable and less precise.

Each GPS satellite continuously transmits a radio signal containing the current time and data about its position. The time delay between when the satellite transmits a signal and the receiver receives it is proportional to the distance from the satellite to the receiver. A GPS receiver monitors multiple satellites and solves equations to determine the precise position of the receiver and its deviation from true time. At a minimum, four satellites must be in view of the receiver for it to compute four unknown quantities. This includes the knowledge of:

K1.  Explain GPS with its types

K2.  Draw the Sketches of GPS

K3.  Explain the different parameters of GPS Performing surveying and leveling.

K4.  Explain the methods of observations for GPS at Different task setting

K5.  Explain the methods for Safety Precaution while using GPS along with

its components.

K5.  Explain Working process to GPS

K6.  Explain different mode for GPS

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Undertake detailed procedure of seting up GPS

- Undertake detailed proedure for Performing different task on GPS

- Fixing of GPS at work station

- Undertake Fixing of Prism at relevant places

- Adjust GPS with PC

- Perfom Data transfer in to PC

- Explain the importance of GPS in field

- Install & Transfer data for GPS into PC

- Use frequently GPS for different functions

- Convert mode easily

- Compile task properly

- Installed battery

Tools and Equipment

- G.P.S

- Data cables

- Wi fi

- Battery charger

- Measure Co-ordinates by GPS

Overview: This competency standard covers the skills and knowledge required to draw co. Ordinates by GPS on given station point. It also helps to transfer data into PC.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Set out co-ordinates by GPS | P1. Set out GPS P2. Turn on GPS P3. Open google map P4. Finding your co-ordinates P5. Tap cross hair icon P6. Hold the blue dot P7. Note co-ordinate location |
| CU-2. Set coordinates within REFLEXW by GPS | P1. Set out GPS P2. Turn ON GPS P3. Use REFLEXW mode P4. view profile by google map P5. Use Processing options P6. Take 3-D, topography P7. Generate Data P8. Check Result |

Knowledge & Understanding

The candidate must be able to develop skills in carrying out and evaluating GPS along with its type and working. It also give the knowledge and understanding required to carry out tasks and recovered the coordinates by your choice OR already prescribed. This includes the knowledge of:

K1.  Explain GPS with its types

K2.  Explain the working of GPS

K3.  Explain the basis of GPS is "co-ordinates" from satellites.

K4.  Explain "co-ordinates," a GPS receiver measures distance using the travel time of radio

signals.

K5.   Explain to measure travel time,

K6.   Describe GPS needs very accurate timing which it achieves with some tricks.

K7.   Explain the different parameters of GPS for single frequency type.

K8.   Explain the different parameters of GPS for Double frequency type.

K9.    Explain the methods to observe co-ordinates and its setting.

K10.  Explain the methods for transfer the data in PC / android.

K11.  Describe Safety Precaution while using GPS along with its components.

K12.  Explain wi-fi signals with its working

K13.  Explain to generate data

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Undertake detailed procedure of Working of GPS seting up

- Carry out  detailed proedure to  draw co.ordinates with its techniques

- Fix co-ordinates of a station point

- Know the method of turn on and turn off the GPS

- Understand to the use of differen parameters of GPS

- Explain the importance of working at GPS

- Ensure wi-fi signals

- Transfer data into PC

- Work efficiently on system

- Save data into requisite place into PC

- Generate and transfer data

- Transfer data from one PC to another PC

- Process e-mail

Tools and Equipment:

- G.P.S

- Data cables

- Wi fi

- Umbrella with stand

- Lime Powder

- Battery with charger

- Set out Civil Engineering Structures by GPS

Overview: This competency standard covers the skills and knowledge required to use the Global Positioning System (GPS) as a tool for Civil Engineering on monitoring the vibrations of large road structures, notably the bridges. To be characterized as a structural tool it was developed and tested a method which is based on the interferometry principle. The method uses the L1 carrier phase that needs to be collected from only two satellites.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1. Set out Civil Engineering Structures by GPS (by using the classic RTK technique) | P1. Set GPS at station point P2. Install battery P3. Select “RTK” Network Approach P4. Apply (R.T) Real Time Positioning by GPS P5. Use GPS Derived “COORDINATES” P6. Use Laser Pointing System P7. Evaluate “RTK” Accuracy Results |
| CU-2. Use 3-D data interpretation By GPS | P1. Set GPS at station point P2. Install battery P3. Import data P4. Process Data P5. Check Results. |

Knowledge & Understanding

The candidate must be able to Surveying work needed on construction sites is usually dependent on the determination of accurate plane coordinates and heights. With current developments in the Global Navigation Satellite Systems (GNSS), real-time 3D positioning can be generally achieved at the cm level of accuracy. Such accuracy helps to expand the use of Real Time Kinematic (RTK) GPS positioning techniques for construction surveying work, particularly for large sites when rapid survey is needed.

Surveying work needed on construction sites is usually dependent on the determination of accurate plane coordinates and heights. With current developments in the Global Navigation Satellite Systems (GNSS), real-time 3D positioning can be generally achieved at the cm level of accuracy. Such accuracy helps to expand the use of Real Time Kinematic (RTK) GPS positioning techniques for construction surveying work, particularly for large sites when rapid survey is needed.

K1.  Explain GPS with its working

K2.  Draw the sketch for GPS and its parts.

K3.  Explain the different parameters of GPS performing different task.

K3. Explain the methods of observations for GPS.

K4.  Explain the methods for Safety Precaution while using GPS along with its components.

K5.  Explain Data transfer method into PC

K6.  Explain Global Navigation Satellite Systems

K7.  Explain Real Time Kinematic (RTK) GPS positioning techniques

K8.  Explain Universal Traverse Mercator Coordinate System (UTM) Conversion

K9.  Describe space segment

K10. Explain satellite geometry

K11. Explain carrier phase tracking system

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Detailed procedure of seting up instrument for construction survey work

- Detailed proedure for survey techniques while using GPS

- Fix GPS at  00.00

- Fix Prism at relevant places

- Adjust its modulator

- Perfom Transfer data

- Import data from google

- Perform UTM Conversion

- Use Satellite Geometry

- Control Space Segment

- Work on carrier phase tracking system

- Work on different modulation system

- Indicate & locate wi-fi signals

Tools and Equipment

- Double Frequency GPS

- Measuring tapes

- Peg

- Umbrella with stand

- Lime Powder

- Wi-Fi signals

- GPS Battery with Charger

  - Auto CAD

- Perform Basic Computer Operations

Overview: This competency standard covers the skills and knowledge required to prepare report on the health and safety considerations while working on computer, recognize basic computer hardware, software, applications and perform general troubleshooting, demonstrate your skills of MS Word, MS PowerPoint, MS Excel as well as installation and troubleshooting of Operating System and software.

| Competency Units | Performance Criteria |
| --- | --- |
| - Investigate and prepare a short report on the health and safety considerations while working on IT equipment. | - Observe Considerations: light (natural, artificial), - Identify any hazard risk - Perform CAD (computer screens, electrical protection devices) - Repetitive work, working postures, Incorrect screen settings, working without break |
| - Configure Computer System | - Connect computer components and peripherals as per requirements - Install System software and applications software according to the Instruction Manual - Troubleshoot Applications to trace and fix faults (if any) to bring it in a running condition - Follow health, safety and security procedures to ensure safe working environment |
| - Create a document Using MS Word | - Compose a document as per requirements - Assign name and location to save a file in word file format - Format Word Document according to given requirements - Generate hard copy according to job requirements |
| - Prepare a Worksheet Using MS Excel | P1. Develop a worksheet as per given data P2. Format the worksheet according to given job requirements P3. Apply Formulas according to given criteria P4. Generate Charts/Graphs according to the given data |
| - Prepare presentation in power point software. | P1. Insert Slides with different Layouts according to requirements of presentation P2. Insert text, tables, images, etc. according to the requirements P3. Apply a set of effects to animate the slide according to requirements P4. Apply Slide Transitions on Slides according to requirement P5. Apply Sound Effects on Objects/text/images according to job requirements |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Understand Operating systems- Describe procedure of Installation of software applications.

- Describe how to Locate, Open, close files and manage files.

- Describe Hardware and Software- Access a CD/DVD-ROM, Data traveler, Troubleshoot hardware and software problems.

- Explain use of Internet and E-mailing- send an e-mail message, Receive an e-mail message, Attach a file to an e-mail, Basic search in search engine, download and view file from webpage, upload files to cloud location and print web page.

- Describe basic formatting in MS Office- bold, italic & center.

- Save a file and print a file

- Describe Hyperlinking and referencing

- Describe how to apply Formulas

- Describe Short Keys use in MS Office

- Explain how to assign Animations and transitions to different slides

- Communication skills

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Report on Health & Safety considerations.

- Install MS Office Application correctly

- Prepare a formatted document using MS Word

- Enter  data  into  the  respective  columns  and  rows  as  per  given instructions

- Set page layouts and margins

- Apply any slide transition on entire presentation

Instruments & Consumables

| Sr. No. | Description (Instruments) | Sr. No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, PowerPoint and Excel) Drawing sheet |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | 3D Studio Max |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | Printer Paper |
| 8 | Internet Connection | 8 | NVQF/Policy Documentation |

- Perform Drafting Settings and Formatting

Overview: This competency standard covers the skills and knowledge required to create two dimensional drawings by using various tools and commands, create & modify objects and drawings in AutoCAD to meet specific targets according to civil technology requirements.

| Competency Units | Performance Criteria |
| --- | --- |
| - Set up user interface for require specifications | - Load the AutoCAD software. - Set the units, Limits, coordinate system, Workspace, - Set Object Snap Settings. - Set up the grid, snap, ortho, tracking settings. - Save AutoCAD drawing files in different file formats (DWG, DWT). |
| - Perform formation of entities- line, line type, text, layers, points, dimensioning. | - Setup user interface settings for required specifications - Load centre line, dashed line, border, dot. - Create layer with lines, colours, weight. - Format text styles & multiline styles. - Format dimension styles. - Format point styles. |
| - Create templates of sheets sizes A2, A3 & A4 having geometrical figures in 2D. | - Setup user interface settings for required specifications - Format the line, line type, text, layers, points. - Draw rectangle with line command by adopting coordinates- rectangular (absolute & relative). - Draw, polygons, ellipse & circle. - Create title block layout for sheets sizes A2, A3 & A4. - Use Design centre for inserting styles in new file. - Save the files in DWT format. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe and use of Basic Drawing Settings- Unit setting, Limits setting, User coordinate system, Workspace setting, and Object Snap Settings

- Use Direct distance entry system for drawing lines

- Set U.C.S for different situations

- Basic Commands and Concepts- Angles and lines in AutoCAD

  - Differentiate between absolute, relative and polar system

  - Explain DIMSTYLE and MTEXT commands

- Describe the procedure for Navigation in AutoCAD for AutoCAD classic, 2D

- Drafting and Annotation, 3D Modelling and Initial Setup Workspace.

- Describe the options in File, Edit, View, Insert, Format, Tools, Draw, modify, Parametric, Windows, help, and Express in AutoCAD classic.

- Describe the use of ribbons of “2D drafting and annotation” i.e. Home, Insert, Annotate, Parametric, View, Manage, output and express tools.

- Describe the use of various categories in ribbons of Home, Insert, Annotate, Parametric, View, Manage, output and express tools in   2D drafting and annotation menus.

- Understand the use of osnap, inquiry, standard, formatting & other Tool Bars

- Understand the use of different ribbons-home, solid, surface, mesh, render, parametric, insert, annotate, view, manage, output, plug-in, Express Tools.

- Describe the use of tool bars of AutoCAD classic i.e. Draw, Modify, Standards, properties, Layers, Osnap, Dimension and their practical application.

- Explain the procedure for creation of user tool bars.

- Describe Design center feature- Scale and paper sizes

- Modify dimension style and text size according to paper size

- Apply drafting settings (Units, Limits, Snap, Auto On & Off) and their application

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Template of sheets sizes A2, A3 & A4.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, PowerPoint and Excel) Drawing sheet |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | 3D Studio Max |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | Printer Paper |
| 8 | Internet Connection | 8 | NVQF/Policy Documentation |

- Perform Dimensioning, Lettering and Plotting

Overview: This competency standard covers the skills and knowledge required to create 3- Dimensional Models by using various tools and commands in AutoCAD software, demonstrate skills to modify 3D objects and models to ensure civil technology requirements and present a rendered 3D Model final outcome.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform dimensioning of drawing. | - Select dimension styles as per scale. - Applying dimension styles as per scale. - Set Dimension location of object. - Check Dimension drawing - Aligned dimension, continuous dimension, base line dimension, leader. |
| - Perform lettering (Text) of styles. | - Select & Insert multi-line text style lettering for drawing type. - Select & Insert single-line text style lettering for notes on drawing. - Select & Insert single-line text style lettering for schedule on drawing. - Select & Insert single-line text style lettering for schedule on drawing. |
| - Create Layouts for printing for Printing. | - Perform setting in layouts as per required scale - Perform setting in layouts as per required orientation. - Plot drawing on scale according to required size. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe Design center feature- Scale and paper sizes

- Modify dimension style and text size according to paper size

- Apply drafting settings (Unites, Limits, Snap, Auto On & Off) and their application

- describe layer options, dimensioning & text and their application

- Explain layout settings & printing of CAD drawings and their application

- Describe the purpose and use of the use of blocks and symbols.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of Render 3D prototype Model including materials, lights, scene and different camera views

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, PowerPoint and Excel) Drawing sheet |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | 3D Studio Max |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | Printer Paper |
| 8 | Internet Connection | 8 | NVQF/Policy Documentation |

- Develop 2d Drawings of Structures

Overview: This competency standard covers the skills and knowledge required to create two dimensional drawings by using various tools and commands, create & modify objects and drawings in AutoCAD to meet specific targets according to civil technology requirements.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform basic settings for AutoCAD. | - Setup drawing interface for required specifications - Create formation of entities- line, line type, layers. - Create styles- text, points, - Create lines and multi-lines |
| - Execute draw commands objects. | - Draw construction lines. - Draw lines with precision as per standards. - Draw curves with precision as per standards. - Hide the construction lines layer. |
| - Perform application of Modify tools / commands to delete whole or part of surplus entities. | - Apply FILLET & TRIM, ERASE, BREAK commands to delete whole or part of surplus entities in given scenario. - Apply CHAMFER, EXTEND & FILLET to modify corners. - Apply SCALE, LENGTHEN, STRETCH for multiple purposes. |
| - Change properties of objects to meet set standards. | - Apply ROTATE to rotate & MOVE to move objects. - Use OFFSET, MIRROR, ARRAY & copy to create multiple objects. - Modify properties by SCALE, EXPLODE, PEDIT, JOIN for multiple purposes. |
| - Assign symbols to specified materials | - Set the scale factors for hatching patterns. - Select specified area/ object on drawing. - Apply the hatch to the area. - Calculate hatched area by Auto CAD command |
| - Modify drawing objects using properties option. | - Perform inquiry commands to get area & distance of object. - observe the objects properties- location, linear & angular measurements, and other drafting formations. - Modify the objects properties- location, - Apply linear measurements with other drafting formations - Apply angular measurements with other drafting formations |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Apply draw commands and tools.

- Describe and use modify commands and tools.

- Describe HATCHING concepts in AutoCAD.

- Describe zooming options

- Describe features of Tools palettes window and their application

- Describe Design center feature- Scale and paper sizes

- Apply drafting settings (Unites, Limits, Snap, Auto On & Off) and their application

- Explain layout settings & printing of CAD drawings and their application

- Explain integration of total station with AutoCAD & MS Excel and their application

- Explain how to extend, trim, stretch, break and scale drawn items.

- Explain how to edit properties of objects.

- Describe the purpose and use of the use of blocks and symbols.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Create 2D Drawings of C-type residence including Plan,

- Draw working plan, section, elevation, legend, schedule, areas & measurements according to civil technology.

- Create X-section for 2D Drawings of C-type residence.

Instruments & Consumables

| Sr. No. | Description (Instruments) | S No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, PowerPoint and Excel) Drawing sheet |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | 3D Studio Max |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | Printer Paper |
| 8 | Internet Connection | 8 | NVQF/Policy Documentation |

5-

- Produce Drawing of Framed Structure Building

Overview: This competency standard covers the skills and knowledge required to draw architectural & structural drawing for a double storey “RCC” framed structure building (key plan, layout plan, detailed plan, elevations, sections, structural drawing of building components), detailed drawing of steel trussed roof shed (60’ x 40’) and sketches of arches, floors & roofs.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare architectural drawings of a Small two storey RCC framed Structure Building. | - Select the suitable instruments to draw frame. - Draw title strip on sheet. - Distribute space of drawing sheet. - Draw key plan, layout plan, detailed plan of building, roof drainage plan & Floor plan of building. - Draw Elevation, and sections of building. - Perform lettering and dimensioning. - Prepare schedule of area, openings, fittings. - Indicate North direction - Perform printing for drawing |
| - Prepare structural drawings of a Small two storey RCC framed Structure Building. | - Draw structural detail of roof, floor, lintels on half imperial sheet. - Draw title strip on sheet. - Distribute space of drawing sheet. - Draw Reinforcement details of RCC column with base, beams, slabs, and stairs. - Perform lettering and dimensioning. - Indicate North direction - Perform printing for drawing |
| - Prepare detailed drawing of steel trussed roof shed (60’ x 40’) | - Select the suitable instruments to draw sheet. Draw title strip on sheet OR block. - Distribute space of drawing sheet. - Draw plan of roof shed. - Draw frame of truss with roofing detail- sheets, purlin, gutter, ridging and wind ties. - Draw detail of joints and fixing at support. - Perform lettering and dimensioning. - Indicate North direction - Perform printing for drawing |
| - Draw sketches of arches | - Select the suitable instruments to draw sketches. - Draw title strip on sheet OR block. - Distribute space of drawing sheet. - Sketch semi-circular, circular, segmental, two cusp, three cusp, elliptical, & flat arches. - Perform lettering and dimensioning. - Indicate North direction - Perform printing for drawing |
| - Draw sketches of roofs | - Select the suitable instruments to draw sketches. - Draw title strip on sheet OR block. - Distribute space of drawing sheet. - Sketch first class mud roof, 2nd Class mud roof, R.C.C, R.B slabs, Pre-cast, roof. Jack arch and sloping roofs. - Perform lettering and dimensioning. - Indicate North direction - Perform printing for drawing |
| - Draw sketches of floors | - Select the suitable instruments to draw sketches. - Draw title strip on sheet OR block. - Distribute space of drawing sheet. - Sketch conglomerate, mosaic, terrazzo, glazed tile, brick tile, wooden, vinyl, rubber, cork, & tuff tile floors. - Perform lettering and dimensioning. - Indicate North direction - Perform printing for drawing |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Sketch out the x-section of lintels and arches.

- State the various parts of lintels and arches.

- Define different types of floors.

- Draw the sketches of different parts of floors.

- Explain the standard proportions for the different layers of floors.

- State different types of roofs i.e. first-class mud roofing, 2nd Class mud roofing, R.C.C and R.B roof and Pre-cast roof slabs.

- State different types of sloping roof.

- Draw the sketches of steel trusses up to 40' span from the given data.

- Label the sketches of different parts of trusses.

- Distribute the space for different views evenly on drawing sheet.

- Define columns.

- Sketch & show steel reinforcement at appropriate place in the column sections of different shapes.

- State the position of over laps and its length.

- Sketch different types of hooks and bends with their standard dimensions.

- Define beam & types of beam

- Sketch the x-section and L-section of a singly reinforced beam & show steel reinforcement (simply supported, cantilever, over hanging, continuums beam)

- Differentiate between the Primary and Secondary beam.

- Sketch and label the details of Reinforcement of T-and L-Beam.

- Sketch and label the details of Reinforcement of two-way continuous slab over Tee-Beam.

- State the purpose of stirrups and bent up bars

- Types of mild steel bars (plain, deformed, cold twisted, presented)

Critical Evidence(S) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of a small two storey RCC framed Structure building.

- Drawing of steel trussed roof shed (60’ x 40’)

- Sketches of Arches, Roofs & Floors.

Instruments & Consumables Sr. No.; Description (Instruments); S No.; Software& Consumable. 1; PCs/Laptops; 1; Microsoft Office (Word, PowerPoint and Excel) Drawing sheet. 2; Multimedia Projector; 2; AutoCAD. 3; Scanner; 3; 3D Studio Max. 4; Printer/ Plotter; 4; Windows OS. 5; USB Sticks; 5; White Board & Markers. 6; Tracing Sheet; 6; Note Books. 7; Scissor/ Paper Cutter; 7; Printer Paper. 8; Internet Connection; 8; NVQF/Policy Documentation.

- Develop Sectional Drawing for foundations

Overview: This competency standard covers the skills and knowledge required to draw structural drawing of raft foundation, grillage foundation, Pile Foundation, well foundation, spread footing, column base & pillar.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare structural detail of Raft foundation. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw structural steel drawings of raft foundation. - Perform printing for drawing |
| - Prepare structural detail of Grillage foundation. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw structural drawings of grillage foundation. - Perform printing for drawing |
| - Prepare structural detail of Pile foundation. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw structural steel drawings of Pile foundation. - Perform printing for drawing |
| - Prepare structural detail of Well foundation. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw structural drawings of well foundation. - Perform printing for drawing |
| - Prepare structural detail of spread footing, pillar, foundations & column base. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw structural drawings of spread footing, pillar, & column footing. - Perform printing for drawing |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define and sketch the spread footing.

- Define and sketch the raft foundations.

- Define and sketch the grillage foundation.

- Define and sketch the well and pile foundation.

- Define and sketch the caisson foundations.

- Label different parts of spread footing i.e. base concrete, sub grade, steps offsets, and plinth.

- Calculate the depth and breadth required for spread footings.

- Sketch and label the raft foundations with steel reinforcement.

- Sketch and label the pile foundations with steel reinforcement.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of structural detail of Raft foundation.

- Drawing of structural detail of Grillage foundation.

- Drawing of structural detail of Pile foundation.

- Drawing of structural detail of Well foundation.

- Drawing of structural detail of spread footing, pillar, foundations & column base.

Instruments & Consumables

| Sr. No. | Description (Instruments) | S No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, PowerPoint and Excel) Drawing sheet |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | 3D Studio Max |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | Printer Paper |
| 8 | Internet Connection | 8 | NVQF/Policy Documentation |

- Develop Drawings for Doors & Windows

Overview: This competency standard covers the skills and knowledge required to draw sketches of ledged, ledged & batten, louvers, collapsible, sliding, rolling, revolving doors, sketches glazed, louvers windows and detailed drawings of panelled & glazed, flush and wire gauzed doors & windows.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare sketches of doors | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Sketch the ledged, ledged & batten, louvers, collapsible, sliding, rolling, revolving doors. - Sketch glazed, louvers windows. - Perform lettering & dimensioning. - Perform printing for drawing |
| - Prepare structural drawing of Panelled & Glazed Doors & Windows | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan, sections & elevation panelled & glazed door & window. - Perform lettering & dimensioning. - Perform printing for drawing |
| - Prepare structural drawing of Flush and Wire Gauzed Doors & Windows | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan, sections & elevation flush door& window. - Draw detailed plan, sections & elevation wire gauzed door& window. - Perform lettering & dimensioning. - Perform printing for drawing |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- State need of doors and windows.

- Define clerestory windows and ventilators.

- State the different types of doors and windows.

- Explain the various parts of doors and windows.

- State the different materials used for doors, windows and ventilators

- Sketch the elevations, sectional plans and vertical sections of doors, windows and ventilators.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing sketches of ledged, ledged & batten, louvers, collapsible, sliding, rolling, revolving doors, sketches glazed, louvers windows

- Structural drawings of panelled & glazed, flush and wire gauzed doors & windows.

Instruments & Consumables

| Sr. No. | Description (Instruments) | S No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, PowerPoint and Excel) Drawing sheet |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | 3D Studio Max |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | Printer Paper |
| 8 | Internet Connection | 8 | NVQF/Policy Documentation |

- Produce Drawings of Stairs and Stair Cases

Overview: This competency standard covers the skills and knowledge required to draw sketches of open newel, straight, bifurcated, geometrical, circular, spiral, & dog-legged stair & Structural drawings of Dog-legged and Spiral Stair cases.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare sketches of stairs | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Sketch open newel, straight, bifurcated, geometrical, circular, spiral, & dog-legged stair. - Perform lettering & dimensioning. - Perform printing for drawing |
| - Prepare structural drawing of Dog-legged Stair. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of dog legged stair. - Draw detailed plan & sections spiral stair case - Perform lettering & dimensioning. - Perform printing for drawing |
| - Prepare structural drawing of Spiral Stair Case. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of dog legged stair. - Draw detailed plan & sections spiral stair case - Perform lettering & dimensioning. - Perform printing for drawing |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define stair and stair case

- Define the terms and parts used in different types of stairs.

- Explain the stairs according to their layout.

- State the suitability of each type of stair.

- Sketch the plans and sections of different types of stairs according to their layout.

- Define different type of Stairs

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing sketches of open newel, straight, bifurcated, geometrical, circular, spiral, & dog-legged stair.

- Structural drawings of Dog-legged and Spiral Stair cases.

- Instruments & Consumables

| S No. | Description (Instruments) | S No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, Power Point and Excel) |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | Printer Paper |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | NVQF/Policy Documentation |
| 8 | Internet Connection |  |  |

- Produce Drawings of Water Tanks

Overview: This competency standard covers the skills and knowledge required to draw masonary underground masonry water tank, RCC Circular Surface Water tank and Square RCC over head water Tank.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare structural drawing of RCC surface circular / square water tank. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of masonry underground water tank. - Draw reinforcement detailed drawing. - Perform lettering & dimensioning. - Perform printing for drawing |
| - Prepare structural drawing of RCC square overhead water tank. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan of RCC surface water tank. - Draw detailed cross sections of RCC surface water tank. - Draw reinforcement detailed drawing. - Perform lettering and dimensioning. - Perform printing for drawing |
| - Prepare structural drawing of RCC Circular overhead water tank. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of RCC overhead water tank. - Draw reinforcement detailed drawing. - Perform lettering and dimensioning. - Perform printing for drawing |
| - Prepare structural drawing of RCC Rectangular overhead water tank. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of RCC overhead water tank. - Draw reinforcement detailed drawing. - Perform lettering and dimensioning. - Perform printing for drawing |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define water tank.

- Define the terms and parts used in different types of water tanks.

- Explain the tanks according to their material, & level.

- State the suitability of each type of water tank.

- Sketch the plans and sections of different types of water tanks.

- Differentiate between surface and overhead water tank

- Describe reinforcement for overhead water tank

- Explain reinforcement overlapping, binding and cutting

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of masonary underground rectangular water tank,

- Drawing of RCC Circular Surface Water tank

- Drawing of Square RCC over head water Tank.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, Power Point and Excel) |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | Printer Paper |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | NVQF/Policy Documentation |
| 8 | Internet Connection |  |  |

- Produce Drawings of Cross Drainage Works

Overview: This competency standard covers the skills and knowledge required to draw detailed drawing of RCC Deck high level 2 spans bridge, prepare detailed drawing of masonry segmental arched single span bridge and Prepare detailed drawing of steel plate girder single span bridge.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare detailed drawing of RCC Deck high level 2 spans bridge | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan, elevation & sections of RCC deck 2 span high level bridge. - Draw reinforcement detailed drawing. - Perform lettering & dimensioning. - Perform printing for drawing |
| - Prepare detailed drawing of masonry segmental arched single span bridge. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of Segmental Arch single span bridge. - Perform lettering & dimensioning. - Perform printing for drawing |
| - Prepare detailed drawing of masonry segmental arched triple span bridge. | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of Segmental Arch triple span bridge. - Perform lettering & dimensioning. - Perform printing for drawing |
| - Prepare detailed drawing of steel plate girder bridge for single span | - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of steel plate girder bridge for single span. - Draw reinforcement detailed drawing. - Perform lettering & dimensioning. - Perform printing for drawing |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Sketch the X-section of concrete Road structure.

- State various parts of culverts i.e. abutment, wing wall, toe wall parapet,	base plate.

- Sketch the Plan, Foundation Plan, Long Section and X-Section of Culvert.

- Explain the various terms used in Bridge.

- State the difference between culvert and Bridge.

- Explain the various types of bridges.

- Sketch the Plan, Foundation Plan, Long section and X-Section of two Span Bridge.

- Sketch the detailed drawing of 25’ span plate girder steel bridge.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Prepare detailed drawing of RCC Deck high level 2 spans bridge

- Prepare detailed drawing of masonry segmental arched single span bridge.

- Prepare detailed drawing of steel plate Girder Bridge 1 span

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, Power Point and Excel) |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | Printer Paper |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | NVQF/Policy Documentation |
| 8 | Internet Connection |  |  |

- Develop 3d Drawings

Overview: This competency standard covers the skills and knowledge required to create 3- Dimensional Models by using various tools and commands in AutoCAD software, demonstrate skills to modify 3D objects and models to ensure civil technology requirements and present a rendered 3D Model final outcome.

| Competency Units | Performance Criteria |
| --- | --- |
| - Develop 3D Objects | - Setup & save 3D Drawing Interface for required specifications - Setup 3D User Interface settings for required specifications - Create 3D Objects with given data - Perform printing for drawing |
| - Manipulate 3D Objects Using 3D Editing Tools | - Modify 3D Objects in line with the requirements - Make customized 3D Models according to the requirements of the given job - Convert 3D Face Objects into Single Mesh Objects - Perform printing for drawing |
| - Render 3D Model | - P1. Apply material to required 3D Model as per given specifications - Apply lights to get the requisite scene of required 3D model - Assign cameras to execute different views of required3D Model - Render and print the 3D model according to required size and orientation - Perform printing for rendered model |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe 3D Modeling in AutoCAD- 3D solids, Surfaces, Meshes, Wireframe Objects

- Differentiate between Surface Modeling and Solid Modeling

- Explain 3D Face and Edges

- Explain Boolean Operation Concepts- Subtraction, Intersection, Union

- Describe 3D Navigate Control- functions of different camera settings, importance of scene creation, PRESET views such as isometric, top, bottom, front, left, etc.

- Describe Perspective projection and parallel projection

- Describe walk and Constrained Orbit.

- Describe how to assign Material and Light Control- planner mapping, texture map, opacity control, render context and render sampling.

- Explain rendering requirements

- Explain modelling procedure

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Drawing of Render 3D prototype Model including materials, lights, scene and different camera views

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, PowerPoint and Excel) Drawing sheet |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | 3D Studio Max |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | Printer Paper |
| 8 | Internet Connection | 8 | NVQF/Policy Documentation |

- Produce Civil Technology Drawings

Overview: This competency standard covers the skills and knowledge required to Printout on A4 paper drawings of c-type residential building, Printout on A2 paper submission drawings of a small two storey R.C.C. framed structure building, Printout on A4 paper x-sections of canals, drains and roads and Printout on A4 paper drawings of masonry 10 ft segmental arched culvert.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare & printout on A4 paper drawings of c-type residential building. | - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Draw detailed plan of C type building. - Draw front & side elevation of C type building. - Draw x-section of C type building. - Adjust layouts in viewports for printing C type building. - Perform lettering & dimensioning - Print the drawings |
| - Prepare & printout on A2 paper submission drawings of a small two storey R.C.C. framed structure building | - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Draw detailed plans of framed structure building. - Draw front & side elevation of framed structure building. - Draw site plan, layout plan of framed structure. - Draw x-sections of framed structure building. - Draw reinforcement detail of RCC structures. - Adjust layouts in viewports for printing frame structure building. - Perform lettering & dimensioning - Print the drawings |
| - Prepare & printout on A4 paper x-sections of canals and drains | - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Draw cross section of canal fully in cutting, fully in filling and partly in cutting & partly in filling. - Draw x-sections of type-I. type-II & type 8 drains - Adjust layouts in viewports for printing cross section of canals and drains. - Perform lettering & dimensioning - Print the drawings |
| - Prepare & printout on A4 paper x-sections of roads. | - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Draw cross section of canal fully in cutting, fully in filling and partly in cutting & partly in filling. - Draw cross section of national highway dual carriage in rural and built up area. - Draw cross section of district highway double lane in rural and built up area. - Draw x-sections of type-I. type-II & type 8 drains - Adjust layouts in viewports for printing cross section of road. - Perform lettering & dimensioning - Print the drawings |
| - Prepare & printout on A4 paper drawings of masonry 10 ft segmental arched culvert. | - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Draw detailed plan, & layout plan of culvert. - Draw long section & cross section of culvert. - Draw elevation of culvert. - Adjust layouts in viewports for printing segmental arch culvert. - Perform lettering & dimensioning - Print the drawings |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Modify dimension style and text size according to paper size

- Apply drafting settings (Unites, Limits, Snap, Auto On & Off) and their application

- describe layer options, dimensioning & text and their application

- Explain features of dimensioning & text options and their application

- Explain layout settings & printing of CAD drawings and their application

- Explain integration of total station with AutoCAD & MS Excel and their application

- Explain how to extend, trim, stretch, break and scale drawn items.

- Explain how to edit properties of objects.

- Describe the purpose and use of the use of blocks and symbols.

- Describe adjustment of layouts along with viewports for printing.

- Describe the procedure of printing drawings.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Printout on A4 paper drawings of c-type residential building.

- Printout on A2 paper submission drawings of a small two storey R.C.C. framed structure building

- Printout on A4 paper x-sections of canals, drains and roads.

- Printout on A4 paper drawings of masonry 10 ft segmental arched culvert.

Instruments & Consumables

| S No. | Description (Instruments) | S No. | Software& Consumable |
| --- | --- | --- | --- |
| 1 | PCs/Laptops | 1 | Microsoft Office (Word, Power Point and Excel) |
| 2 | Multimedia Projector | 2 | AutoCAD |
| 3 | Scanner | 3 | Printer Paper |
| 4 | Printer/ Plotter | 4 | Windows OS |
| 5 | USB Sticks | 5 | White Board & Markers |
| 6 | Tracing Sheet | 6 | Note Books |
| 7 | Scissor/ Paper Cutter | 7 | NVQF/Policy Documentation |
| 8 | Internet Connection |  |  |

  - Construction I

    - Perform Simple Levelling

Overview: This competency standard covers the skills and knowledge required to Prepare for work, Perform manual leveling and Cleanup work area.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Prepare for work. | - Determine requirements of the job and obtain various levels. - Observe work health and safety (WHS) requirements associated with levelling activities. - Identify quality assurance requirements according to workplace requirements. - Select and check serviceability of tools and equipment, including personal protective equipment. |
| - Perform manual leveling. | - Identify height to be transferred drawings, plans or instructions. - Carryout hand levelling and operate according to manufacturer instructions. - Record levels and mark according to job requirements. |
| - Clean up work area. | - Clean work area, and waste disposed of reused or recycled in accordance with work specifications. - Clean, maintain and store tool, plant and equipment in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe different types of levelling equipment, their applications and their method of operation

- Explain process for how to access relevant information, including codes and technical standards

- Develop job safety analysis (JSA) and safe work method statements (SWMS)

- Describe process of establishing, and checking levels and alignment

- Explain procedure for the calculations to carry out leveling

- Describe workplace and equipment safety requirements

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Locating, interpreting and applying relevant information, standards and specifications for the establishment, and checking of levels

- Applying safety requirements throughout the work sequence, including electrical safety requirements and the use of personal protective clothing and equipment

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Spirit level |
| 2 | Staff rod |
| 3 | Auto level |

    - Lay out Building plans

Overview: This competency standard covers the skills and knowledge required to Plan and prepare, Identify and indicate site boundaries, Set out first line for building alignment, Set out right angled corners, Install other building lines, Check building lines for square, Clean-up work place

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare | - Obtain work instructions, including plans, specifications, quality requirements and operational details, and apply relevant information for planning and preparation. - Follow safety (OHS) requirements in accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Select tools and equipment to carry out tasks consistent with job requirements, and check serviceability, and any faults. - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Identify, obtain, prepare and handle safely materials appropriate to the work application. - Identify applied environmental requirements for project in accordance with environmental plans and statutory and regulatory authority obligations. |
| - Identify and indicate site boundaries. | - Identify and locate survey pegs at corners of site in accordance with job drawings, specifications and site topography. - Set string lines accurately into position to identify site boundary markings in accordance with site plan and survey pegs. |
| - Set out first line for building alignment. | - Determine measurements of building line from boundary or existing building from site drawings for setting out. - Determine approximate position and length of line, plus building clearance measurement at each end for hurdle location in accordance with site plan and survey pegs. - Install pegs and hurdles/profiles approximately level across and between one another with adequate provision to mark footing width on hurdle/profile in accordance with job drawings and specifications. - Mark location for line accurately with nails on hurdles/ profiles and line set taut into position to true alignment with boundary in accordance with job drawings and specifications without error. |
| - Set out right angled corners. | - Determine corner of building on set building line to true measurement from adjacent boundary and mark with peg in accordance with job drawings and specifications. - Set up right angle to line from corner peg using triangulation principles. - Install hurdles/ profiles to approximate level of other hurdles and set taut line to right angled alignment. |
| - Install other building lines. | - Install hurdles for remaining building lines to appropriate locations, approximately level with established hurdles in accordance with job drawings and specifications. - Mark measurements for remaining building lines accurately and nail on hurdles to dimensions from site drawings. - Set taut string lines into position to nailed locations on hurdles in accordance with job drawings and specifications. |
| - Check building lines for square. | - Inspect diagonal measurements for square and adjust lines to provide square relationship within 5mm tolerance over minimum diagonal length of 15m. - Check measurements for accuracy. |
| - Clean up work place | - Clean work area and dispose of waste in accordance with work specifications. - Clean, maintain and store tools plant and equipment in accordance with workplace practices. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Knowledge of application and requirements for line, level and plumb in construction projects

- Explain basic construction processes

- Describe construction plan, symbols and construction terminology

- Define different construction terminology

- Enlist job safety analysis (JSA) and safe work method statements

- Describe processes for interpreting engineering drawings and sketches

- Explain processes for setting out

- Describe project quality requirements

- Explain site and equipment safety (OHS) requirements

- Describe site isolation and traffic control responsibilities and authorities

- Define Types, characteristics, technical capabilities and limitations of setting out devices.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Locate, interpret and apply relevant information, standards and specifications

- Comply with site safety plan, OHS regulations and state and territory legislation applicable to workplace operations

- Comply with organizational policies and procedures, including quality requirements

- Safely and effectively use tools and equipment

- Communicate and work effectively and safely with others

- Set out a full size L shape building on a relatively level site to specifications.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Signage & Barricade equipment’s |
| 2 | Surveying pegs |
| 3 | Measuring Tape |
| 4 | Nails |
| 5 | PPE’s |

    - Carryout excavation and install supports

Overview: This competency standard covers the skills and knowledge required to excavate and install trench and excavation support on a new or existing site, in order to make provisions for footings/slabs or to provide/repair/divert services.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare | - Confirm work instructions with supervisor. - Calculate material quantity requirements in accordance with specifications. - Obtain materials needed and check & prepare for compliance. |
| - Locate excavation site and erect safety equipment | - Locate excavation site and establish line & depth from site plans and instructions. - Set and mark service points and the excavation limits. - Establish temporary drainage system to divert surface and sub-surface water. - Identify service markers and avoid interference with underground services. |
| - Dig Excavations | - Dig safely excavations with hand tools to ensure correct route, line and depth. - Assist machine operator to ensure correct route, line and depth of excavation. - Install trench or excavation support if required. |
| - Plan and prepare for installing trench support | - Access, interpret and apply trench support documentation, and ensure the work activity is compliant - Obtain, read, interpret, clarify and confirm work requirements - Identify and address potential risks, hazards and environmental issues, and implement control measures - Select and wear personal protective equipment appropriate for work activities - Identify, obtain and implement traffic signage requirements. - Select, and check for faults, equipment and/or attachments for work activities - Obtain and interpret emergency procedures, and be prepared for fire/accident/emergency |
| - Install trench shoring | - Communicate with plant operator and ensure the excavation of trenches complies with site plan, line and depth - Determine and prepare shoring method - Set out positioning of shoring - Position/erect shoring within the trench - Secure shoring in position and ensure structural conformity with regulations - Clean out excavation - Locate ladders for safe access and egress |
| - Remove trench shoring | - Release jacking mechanisms and remove ladders - Check shoring and prepare it for lifting from the trench - Remove shoring from trench and store it |
| - Conduct housekeeping activities | - Clear work area and dispose of or recycle materials - Clean and maintain condition of equipment, ensure suitability for use, and address/report issues - Manage/report hazards and maintain a safe working environment - Process written records |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Apply common skills and knowledge specified in the introduction to these RMCS Commonly used in-ground services

- Describe excavation materials

- Describe excavation techniques

- Interpret Plans, specifications and drawings

- Define Shoring

- Describe types & components of shoring

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Effectively use tools, plant and equipment

- Mark out and excavate site as to without damaging services.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Shovels |
| 2 | Battens, props, planks for shoring |
| 3 | Jacks |
| 4 | Ladder |

    - Place Concrete in Foundations

.

Overview: This competency standard covers the skills and knowledge required to identify and select hand, power and pneumatic tools for given tasks and safely use and maintain a minimum of rule, tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock and crow bar and pinch bar for given tasks.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare | - Confirm work instructions with supervisor. - Select Tools and equipment as required, check serviceability and report any faults. - Calculate material quantities in accordance with specifications. - Obtain materials needed, checked for compliance and prepare. |
| - Define and prepare work area. | - Receive concrete, check for debris and discharge into wheelbarrow, kibble, pump or hopper. - Determine location of concrete placement from plans and specifications and check for free of debris and waste. |
| - Place, screed and level concrete | - Place concrete in horizontal layers into location to marked out levels. - Minimize height of vertical drop of concrete to avoid segregation of concrete materials. - Consolidate poured concrete during process using compaction or vibration method. - Check finished levels using appropriate levelling device. - Perform screed on concrete to correct levels and grades using appropriate straight edged tool/ formwork-mounted screed. |
| - Clean up | - Clean work area and dispose of, reuse or recycle waste in accordance with work specifications. - Clean tools plant and equipment, check faults and maintain and store in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain concrete ingredients ratio

- Describe batching and finishing techniques

- Enlist concrete materials

- Describe procedure for placement of concrete

- Explain curing times

- Explain levelling techniques

- Explain types, characteristics, uses and limitations of plant, tools and equipment

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Use a hand trowel and power trowel to finish a minimum of 10 square meters of concrete in foundation.

- Consolidation methods

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Chutes - Line pumps - Measuring tapes and rules, mechanized dumpers - Rakes - Screed boards - Shovels - Stipple devices - Trowels troweling machines - Vibrators - Wheelbarrows |

    - Prepare for construction Process (Brick/ block laying)

Overview: This competency standard covers the skills and knowledge required to Plan work, Set out brickwork/ blockwork, and Lay bricks/blocks forming steps and wing walls and Clean up

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Confirm work instructions with supervisor. - Select tools and equipment as required, check for serviceability and report faults. - Calculate material quantity requirements in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Set out brickwork/ blockwork | - Erect work platform in accordance with workplace requirements. - Check location and relative level of footing from job drawings and specifications. - Calculate rise and going of flight and individual steps to codes and workplace requirements. - Set our flight and individual steps from calculations and job drawings. |
| - Lay bricks/blocks forming steps and wing walls | - Prepare mortar mix to required conformity and set out bricks/blocks laid to according to specifications. - Construct base brickwork to specifications. - Lay & form steps square, level, plumb, true and specified bond. - Construct profile of steps to bond, design, aligned and plumb to specifications. - Form parallel wing walls to step alignment in accordance with specifications. - Carry out jointing to work specifications. - Lay brickwork/blockwork to line, set out with gauge and completed to work specifications. - Clean brick/block faces free of mortar. |
| - Clean up | - Clean work area, and waste disposed of, reused or recycled in accordance with work specifications. - Clean, maintain and store tool, plant and equipment in accordance with workplace practices. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain brick and block expansion and growth, control and articulation joints

- Describe brick bond patterns

- Describe characteristics and applications of materials for masonry steps and stairs construction

- Draw plans, specifications and drawings

- Enlist techniques of masonry steps and stairs construction

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Construct a flight of stairs with a minimum of three pre-cast treads, to specifications; and

- Construct a flight of three solid masonry treads with a parallel wing wall on one side to specifications.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Bolsters, - Brick buggies and brick grabs, - Brooms - Buckets - Builders’ lines - Concrete mixers - Dumpy levels - Elevators - Forklifts - Hammers (bricks, club and scotch) - Hoses - Jig saws - Jointing tools - Line blocks and line pins - Margin or raking tools - masonry saws - Masonry squares - Materials hoists - Measuring tapes and rulers - mortar boards - Pallet trolleys, plumb rules, profiles - Scaffolds - Shovels - Small petrol or diesel engines and compressors - Spirit levels - Straight edges - String lines - Trowels - Wheelbarrows |

    - Mix Cementitious materials (Mortar & Concrete)

Overview: This competency standard covers the skills and knowledge required to Plan work, Set out brickwork/ blockwork, and Lay bricks/blocks forming steps, wing walls and Clean up

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. |
| - Set out brickwork/ blockwork | - Calculate materials quantity in accordance with ratio/ specifications. - Obtain materials needed, check for compliance and prepare. - Mix the materials uniformly. - Pour the water and mix thoroughly for uniform color. |
| - Lay bricks/blocks forming steps and wing walls | - Calculate materials quantity in accordance with ratio/ specifications. - Obtain materials needed, check for compliance and prepare. - Mix the materials uniformly. - Pour the water and mix thoroughly for uniform colour. |
| - Clean up | - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Clean plant tools and equipment, check for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain brick and block expansion and growth, control and articulation joints

- Describe brick bond patterns

- Describe characteristics and applications of materials for masonry steps and stairs construction

- Draw plans, specifications and drawings

- Explain techniques of masonry steps and stairs construction

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Construct a flight of stairs with a minimum of three pre-cast treads, to specifications; and

- Construct a flight of three solid masonry treads with a parallel wing wall on one side to specifications.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Bolsters - Brick buggies and brick grabs - Brooms - Buckets - Builders’ lines - Concrete mixers - Dumpy levels - Elevators - Forklifts - Hammers (bricks, club and scotch) - Hoses - Jig saws - Jointing tools - Line blocks and line pins - Margin or raking tools - Masonry saws - Masonry squares - Materials hoists - Measuring tapes and rulers - Mortar boards - Pallet trolleys - plumb rules - profiles - Scaffolds - Shovels - Small petrol or diesel engines - Compressors - Spirit levels - Straight edges - String lines - Trowels - Wheelbarrows |

    - Install flashing and damp proof course

Overview: This competency standard covers the skills and knowledge required install flashings and damp proofing products to different types and styles of buildings.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. - Calculate material quantity in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Prepare for work | - Identify type of flashing and damp proofing material in accordance with and state of structure. - Identify area of structure for damp proofing from or site inspection and inspected for defects. - Correct the defects and carryout surface preparation of structure requiring damp proofing to manufacturer specifications |
| - Install damp proof course | - Install damp proof in accordance with work specifications. - Lay flashing or damp proof course and lap in accordance with specifications. - Apply damp proof material with a consistent mortar bed on top and bottom. - Fold damp proof course to follow shape of surrounding structures. |
| - Install moisture proof barrier or flashings | - Prepare flashing materials for application to surrounding structures in accordance with requirements. - Lay, lap, and join flashing material to follow shape of surrounding structure. - Form flashings or moisture barrier material and seal around openings. - Install flashing and damp proof course to project outside of mortar joint on external surface, and outside of brickwork in accordance with specifications. |
| - Clean up | - Clean work area, dispose waste and reuse or recycle material in accordance with work specifications. - Clean plant tools and equipment, checked for faults and maintained and stored in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain characteristics and applications of materials to install flashings and DPC

- Describe techniques for installing flashings and DPC

- Explain waterproofing methods

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Install according to specifications, flashings of suitable materials for cavity brickwork including internal and external corners, window and door heads and vertical flashings; and stepped and tray flashings to gables and/or parapets.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Bolsters - Brick buggies and brick grabs - Brooms - Buckets - Builders’ lines - Concrete mixers - Dumpy levels - Elevators - Forklifts - Hammers (bricks, club and scotch) - Hoses - Jig saws - Jointing tools - Line blocks and line pins - Margin or raking tools - Masonry saws - Masonry squares - Materials hoists - Measuring tapes and rulers - Mortar boards - Pallet trolleys - Plumb rules - Profiles - Scaffolds - Shovels - Small petrol or diesel engines and compressors - Spirit levels - Straight edges - String lines - Trowels - Wheelbarrows |

    - Carry out brick work

Overview: This competency standard covers the skills and knowledge required to lay and construct brick and masonry buildings.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. - Calculate material quantity in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Set out brickwork/ blockwork | - Identify, select and check bricks/blocks for conformity with specifications. - Develope work platform in accordance with workplace requirements. - Identify location and structural details of brickwork/ blockwork from plans. - Set out base brickwork construction, below and above floor to location, dimensions and specifications. |
| - Install damp proof course | - Prepare and check Mortar mix for conformity and bricks/blocks laid to set out to specification. - Determine brickwork/ blockwork gauge and prepare set out rod. - Construct base brickwork/blockwork for veneer construction to requirements of regulations and specifications. - Install sub-floor ventilation in accordance with specifications. |
| - Construct Veneer Walls | - Check structural frame to ensure it is ready for brick or block veneer construction maintaining minimum cavity. - Install damp proof courses to specifications. - Build ventilation for veneer construction to specifications. - Position and correctly fix wall ties to framework as per specifications. - Construct openings and install flashings. - Keep cavities clear of mortar droppings and bridging. - Install lintels as per requirement. - Construct top brickwork/blockwork to eaves/gable/ ceiling level in accordance with standards. - Construct veneer gable as required by plans and specifications. - Build walls to gauge straight and true in plumb, line and level within standards tolerance. - Form control joints in accordance with locations on job drawings and specifications. - Locate and build weep holes, brick/block reinforcing and wall flashing. - Cut sill bricks where required and lay to line in accordance with work specifications. |
| - Apply rake/ rule joints and clean face. | - Apply rake or rule joints of laid brickwork/blockwork to correct depth and profile in accordance with work specifications. - Brush down brickwork/blockwork prior to drying to remove unwanted mortar and face is cleaned. - Remove excess mortar from brick/blockwork surfaces and clean cavities free of mortar and debris. |
| - Clean up | - Clean work area, and waste disposed of, reused or recycled in accordance with work specifications. - Clean, maintain and store tool, plant and equipment in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain brick and block expansion and growth, control and articulation joints

- Describe brick bond patterns

- Explain the characteristics and applications of materials for masonry veneer construction

- Explain techniques of masonry veneer construction

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Construct a section of a brick veneer building, including set out and gauge brickwork to a base; damp proof course (DPC) if applicable, weep holes, air vents, control joints and veneer ties; lintels and flashings; brick gables; and brick sills and flashings, completing all to specifications.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Bolsters - Brick buggies and brick grabs - Brooms - Buckets - Builders’ lines - Concrete mixers - Dumpy levels - Elevators - Forklifts - Hammers (bricks, club and scotch) - Hoses - Jig saws - Jointing tools - Line blocks and line pins - Margin or raking tools - Masonry saws - Masonry squares - Materials hoists - Measuring tapes and rulers - Mortar boards - Pallet trolleys - Plumb rules - Profiles - Scaffolds - Shovels - Small petrol or diesel engines and compressors - Spirit levels - Straight edges - String lines - Trowels - Wheelbarrows |

    - Construct Cavity Wall

Overview: This competency standard covers the skills and knowledge required to prepare for work, set out brickwork/ blockwork, construct base of brickwork/ blockwork, position door and window frames, construct brick walls, apply rake or rule joints, and clean up.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Prepare for work | - Obtain work instructions, including plans, specifications, quality requirements and operational details from relevant information, confirm and apply to determine bricklaying and block laying tasks. - Follow safety (OHS) requirements in accordance with safety plans and policies. - Identify signage and barricade requirements& implemented. - Select plant, tools and equipment to carry out tasks in consistent with job requirements, check for serviceability, and rectify or report any faults prior to commencement. - Calculate material quantity in accordance with plans, specifications and quality requirements. - Identify, obtain, prepare, and safely handle materials appropriate to the work application, including required fire resistance rating, ready for use. - Identify environmental requirements for project in accordance with environmental plans and statutory and regulatory authority obligations. |
| - Set out brickwork/ blockwork | - Identify bricks and/or blocks, select and check for conformity with specifications and compliance with standards. - Erect work platform in accordance with regulatory and workplace requirements. - Identify location and structural details of brickwork/ blockwork from job drawings and job specifications. - Set out base brickwork below floor construction to location, dimensions and specifications in compliance with standards. - Set out load bearing brickwork, including engaged piers, dwarf walls, isolated piers and corbelling to job drawings and specifications. - Set out cavity brick wall to requirements of job drawings. |
| - Construct Base of brickwork/ blockwork. | - Prepare mortar mix and check for conformity and lay bricks/ blocks to set out according to specifications and standards. - Determine brickwork/blockwork gauge and prepare set out rod. - Construct base brickwork/blockwork for cavity construction to requirements of regulations and specifications. |
| - Position door and window frames. | - Locate window frames and build in to cavity walls to specification and protect frames from mortar droppings during construction. - Locate door jambs, built in and fix to cavity walls and single leaf walls according to job drawings and specifications. |
| - Construct brick walls. | - Construct brick or block cavity construction walls to job specifications and standards. - Lay/ build damp proof courses and flashings to job specifications. - Construct ventilation for solid brick construction to requirements of job specification and regulations. - Check straightness, plumb, line and level of walls within standard tolerances. - Position wall ties to comply with standards. - Construct openings and install flashing to job specifications. - Install lintels to job specifications. - Form control joints in accordance with locations on job drawings and standards. - Locate weep holes, brick reinforcing, vermin proofing and wall flashings and build in to job specifications. - Construct gables and parapets by plans and specifications. - Cut and lay sill bricks to line in accordance with job specifications. - Install tie down and lateral support systems for ceiling/roof structures to walls in accordance with plans, specifications and standards. |
| - Apply rake or rule joints. | - Apply rake or rule joints of laid brickwork/blockwork to correct depth and profile in accordance with work specifications. - Brush down brickwork/blockwork prior to drying to remove unwanted mortar and face is cleaned. - Remove excess mortar from brick/blockwork surfaces and clean cavities free of mortar and debris. |
| - Clean up | - Clean work area, and waste disposed of, reused or recycled in accordance with work specifications. - Clean, maintain and store tool, plant and equipment in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain brick and block expansion and growth, control and articulation joints

- Explain brick bond patterns

- Explain characteristics and applications of materials for cavity brick construction, including fire control and separation required by the Building Code of Australia (BCA) and other legislation

- Describe construction terminology

- Describe job safety analysis (JSA) and safe work method statements

- Explain material safety data sheets (MSDS)

- Explain materials storage and environmentally friendly waste management

- Draw plans, specifications and drawings

- Explain processes for the calculation of material requirements, quality requirements

- Explain techniques of cavity brick construction, including:- anti-termite measures, closing of cavities and capping systems, damp proofing, floor, wall and roof members, gable and eaves construction, lintels and load bearing components, stepped and level flashing for parapets and gables, sub-floor construction, tying components, ventilation, vermin control

- Explain types, characteristics, uses and limitations of plant, tools and equipment

- Describe workplace and equipment safety requirements.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.locate, interpret and apply relevant information, standards and specifications

- Comply with site safety plan and OHS legislation, regulations and codes of practice applicable to workplace operations

- Comply with organizational policies and procedures, including quality requirements

- Safely and effectively use tools, plant and equipment

- Communicate and work effectively and safely with others

- Construct a section of a cavity brick building, including sub-floor walling; damp proof course (DPC) and ventilation; attached and isolated piers; openings (window and door) and lintels; brick cavity ties and roof tie-downs; and parapets (stepped flashing)

- Correct identification of requirement and finishing of the task

- Correct selection and use of appropriate processes, tools and equipment

- Completing all work to specification ensuring correct selection and use of fire-rated materials and methods of construction

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Bolsters - Brick buggies and brick grabs - Brooms - Buckets - Builders’ lines - Concrete mixers - Dumpy levels - Elevators - Forklifts - Hammers (bricks, club and scotch) - Hoses - Jig saws - Jointing tools - Line blocks and line pins - Margin or raking tools - Masonry saws - Masonry squares - Materials hoists - Measuring tapes and rulers - Mortar boards - Pallet trolleys - Plumb rules - Profiles - Scaffolds - Shovels - Small petrol or diesel engines and compressors - Spirit levels - Straight edges - String lines - Trowels - Wheelbarrows |

    - Construct Curved Wall

Overview: This competency standard covers the skills and knowledge required to plan work, set out curve for construction of curved masonry wall, lay curved wall, finish joints and clean up

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. - Calculate material quantity in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Set out curve for construction of curved masonry wall | - Plot planned curve points from job drawings and establish all trammel centers on footing slab. - Plan curve of wall to specified location from trammel or plotted points and mark on the footing slab. |
| - Lay curved wall. | - Mix mortar to specifications. - Lay bricks/blocks for first course to planned set out for line and specify bond according to specifications. - Maintain gauge within standard tolerance at every course level. - Maintain vertical face alignment. - Lay bricks/blocks level over the length of the wall to the established plan profile. - Lay bricks/blocks to specified bond with perpendicular joints (perpends) maintained in a vertical line - Complete construction to requirements of job drawings and specifications. |
| - Finish Joints | - Remove excess mortar from brick/blockwork surfaces in accordance with work specifications. - Rake or rule joints of laid brickwork/blockwork are to correct profile and depth to. - Brush down brickwork/blockwork prior to drying. |
| - Clean up | - Clean work area and dispose of, reuse or recycle waste in accordance with work specifications. - Clean, check tools plant and equipment are for faults and maintain & store in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain brick and block expansion and growth, control and articulation joints

- Explain brick bond patterns, types of joints and finishing

- Explain characteristics and applications of materials for masonry arch construction

- Explain construction and arch terminology

- Perform geometric calculations and drawing

- Draw plans, specifications and drawings

- Explain types of masonry arches and techniques of construction

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Construct a curved wall using an arc center and trammel

- Construct a serpentine curved wall using the plotted points and template technique

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Bolsters - Brick buggies and brick grabs - Brooms - Buckets - Builders’ lines - Concrete mixers - Dumpy levels - Elevators - Forklifts - Hammers (bricks, club and scotch) - Hoses - Jig saws - Jointing tools - Line blocks and line pins - Margin or raking tools - Masonry saws - Masonry squares - Materials hoists - Measuring tapes and rulers - Mortar boards - Pallet trolleys - Plumb rules - Profiles - Scaffolds - Shovels - Small petrol or diesel engines and compressors - Spirit levels - Straight edges - String lines - Trowels - Wheelbarrows |

    - Construct Masonry arches (semi-circular and segmental)

Overview: This competency standard covers the skills and knowledge required to Plan work, Set out first course for masonry arch, Prepare for arch construction, Construct arch center, Set up arch center, Cut and lay brick/ block to arch, Finish Joints and Clean up

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan work | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. - Calculate material quantity in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Set out first course for masonry arch | - Set out location and line of brickwork/blockwork wall on concrete footing/slab to job drawings. - Determine pan of arch from prepared allotted arch center plus 4mm. - Set out arch spans to location for first course. |
| - Prepare for arch construction. | - Prepare mortar mix and lay bricks/blocks to form wall to set out. - Maintain gauge of abutting walls within standard tolerance at each course level. - Maintain plumb and alignment of vertical wall face. - Cut bricks and lay in level and to line over length of wall. - Lay abutment jambs/piers vertical up to springing line. - Lay bricks/blocks in stretcher bond to springing line of arch with perpendicular joints maintained in vertical line. |
| - Construct arch center. | - Set out arch center and draw curve in accordance with specifications and plan. - Transfer plan to material and cut to shape. |
| - Set up arch center. | - Determine height to springing line and confirm height to crown of arch to be within standard tolerance. - Determine height of toms and wedges or adjustable metal props to set up and support timber arch center. - Adjust supports to ensure arch center is level at right angles to wall face and level across springing line. - Locate props, toms, packers and wedges are located for easy removal. - Establish position of central key brick/block for gauged arch and tape used to mark gauge. |
| - Cut and lay brick/ block to arch. | - Cut and lay bricks and blocks on center to form arch to specifications. - Maintain joints to equal size and parallel on the extrados of an arch. - Maintain same size wedge shape on face. - Maintain centerline of key brick/block wedge through vertical center line of arch. - Maintain even joint thickness around extrados for cut brickwork and blockwork. - Cut all bricks and lay to maintain even joints. - Apply all joints evenly to depth and shape to specifications. |
| - Finish Joints. | - Remove excess mortar from brick/blockwork surfaces in accordance with work specifications. - Apply rake and rule joints of laid brickwork/blockwork to correct profile and depth. - Brush down brickwork/blockwork prior to drying. |
| - Clean up | - Clean work area, and waste disposed of, reused or recycled in accordance with work specifications. - Clean, maintain and store tool, plant and equipment in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain brick and block expansion and growth, control and articulation joints

- Explain brick bond patterns, types of joints and finishing

- Explain characteristics and applications of materials for masonry arch construction

- Explain construction and arch terminology

- Perform geometric calculations and drawing

- Draw plans, specifications and drawings

- Explain types of masonry arches and techniques of construction

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Construct five arches - three are in brick and two in block, including bull's eye, segmental and Gothic with the construction of the arch center for one of the arches

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Bolsters - Brick buggies and brick grabs - Brooms - Buckets - Builders’ lines - Concrete mixers - Dumpy levels - Elevators - Forklifts - Hammers (bricks, club and scotch) - Hoses - Jig saws - Jointing tools - Line blocks and line pins - Margin or raking tools - Masonry saws - Masonry squares - Materials hoists - Measuring tapes and rulers - Mortar boards - Pallet trolleys - Plumb rules - Profiles - Scaffolds - Shovels - Small petrol or diesel engines and compressors - Spirit levels - Straight edges - String lines - Trowels - Wheelbarrows |

    - Construct Scaffolding

Overview: This competency standard covers the skills and knowledge required to erect and dismantle a range of modular scaffolding systems to provide work platforms for construction purposes.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment throughout the work. - Calculate material quantity in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Whip, tie, splice and inspect ropes | - Inspect ropes and cords for damage and wear. - Whip and splice designated rope ends in accordance with regulations and project specifications. - Apply and inspect bends and hitches in accordance with project specifications. |
| - Erect scaffolding | - Confirm purpose for scaffolding and identify associated work tasks. - Determine expected loading on scaffold and supporting structure using load tables and manufacturer specifications. - Select and inspect scaffolding and components and label & reject damaged components. - Select sole board/base plate in accordance with regulations, legislation, codes of practice and manufacturer specifications. - Set out and erect scaffolding in accordance with regulatory and manufacturer requirements. - Erect and install static lines where specified in accordance with regulatory requirements. - Assemble and erect lifting device where specified. |
| - Inspect, repair and alter erected scaffolding | - Inspect erected modular scaffolding for damage, corrosion, wear and compatibility. - Check current use of scaffolding against original design and in accordance with regulations and specifications - Inspect and confirm scaffolding stability. - Carry out alteration or repair where specified. - Complete and date inspection log and handover ready for signing by a certified scaffolder. |
| - Dismantle scaffolding | - Isolate scaffolding and appropriately sign and barricade to ensure safe dismantling. - Dismantle scaffolding using reverse procedure as for erection. |
| - Clean up | - Clean work area and dispose of, reuse or recycle waste in accordance with work specifications. - Clean, check, and tools plant and equipment for faults and maintain and store in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Enlist scaffolding equipment

- Explain scaffolding techniques

- Explain procedure for scaffolding

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Complete erect and dismantle a modular scaffolding system, in accordance with specifications, including a minimum of:

- Three bays with an internal and external return

- Three lifts, including ties

- Ladder and stair access

- Fall and edge protection

- Complete whipping, splicing, tying and inspecting of five fiber ropes in accordance with regulations.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Adjustable base plates - Bends and hitches, box spanners, braces, bracket scaffolds (tank and formwork) - Cantilevered hoists (materials only with maximum capacity of 500kg) - Couplers and accessories - Fiber ropes - Gin wheels, guard rails - Hammers - Ledgers - Mesh guards - Mid rails - Modular and prefabricated scaffolds - Hammers, prefabricated components - Safety nets, scaffold belts, scaffolding planks shovels, spanners, spirit levels - Stairs or ladders, standards, static lines, steel and aluminum tubes - Tape measures, torpedo levels, transoms - Wire nips, wrenches. - Pallet trolleys, plumb rules, profiles - Scaffolds, shovels, small petrol or diesel engines and compressors, spirit levels, straight edges, string lines - Trowels - Wheelbarrows |

    - Install doors and window frames

Overview: This competency standard covers the skills and knowledge required set out and install timber and metal window and door units of different types, with appropriate weather proofing, smooth access and good security.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment. - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. - Calculate material quantity in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Install window units to frame | - Check window opening size to be greater than overall window frame. - Join reveals and fix securely to frames where specified. - Locate window unit to suit brickwork and eaves finish for veneer construction, whichever is applicable. - Position window unit in place so that head/sill is level and stiles are plumb and in wind, ensuring reveals or frame are finished flush with face of inside wall lining. - Pack and fix window to wall frame through/to studs, in accordance with specified fixing and fastening methods. |
| - Replace window units/door frames | - Remove architraves and nosing. - Remove sill bricks or cladding where specified. - Cut fasteners, remove packing and detach flashing from frame. - Remove window unit/ doorframe. - Install window unit/door frame and window components to plumb, level and wind. |
| - Architraves and moldings are replaced | - Prepare door opening, and construct and fix jamb. - Check doorframe opening size to be greater than the overall door jamb width and height, allowing for plumbing of stiles, thickness of floor covering, levelling of door head and level of floor. - Mark jamb stiles and cut to length allowing for clearances according to specifications. - Accommodate trench head to jamb stiles allowing for clearance according to specification. - Assemble jamb frame, square and brace with rebates flush. - Clean and finish joints and rebates. |
| - Install door and door unit | - Position door unit in place so that the head/sill is level and stiles are plumb and in wind, ensuring reveals or frame finished flush with face of inside wall lining. - Fit door to jamb allowing for clearances according to specifications with lock stile door backed off to facilitate correct operation. - Mark out hinges on door and jamb. - Fit hinges to door and jamb. - Make final adjustments of door. - Fit door furniture components and fix to manufacturer specifications |
| - Perform clean up | - Clean work area, and dispose of, reuse or recycle waste in accordance with work specifications. - Clean tools plant and equipment, check for faults and maintain & store in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain flashing requirements and installation techniques

- Explain processes for setting out

- Explain window and door installation and replacement techniques

- Enlist window and door materials

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Construct and fit one standard external rebated door jamb/frame to specifications;

- Fit and hang one standard door, including the door furniture and a pair of doors with door jambs to specifications;

- Install one standard window or glazed sliding door unit to specifications; and

- Replace one standard window or glazed sliding door unit to specifications.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Air compressors and hoses - Bevels - Chisels - Drills - Hammers, hand/power saws - Marking equipment, measuring tapes and rules - Nail guns - Protractors - Spirit levels, squares (combination/tri), steel squares and fences, string lines Materials may include: - Barrel bolts - Cabin hooks closers - Dead bolts - Flash bolts, flashings - Handles hinges (butt and parliament) - Latches, locks - Metal - Night latches - Passage sets - Timber Windows may include: - All size windows - Casement - Double hung - Glazed sliding doors, curved and bay - Hopper - Straight and sliding |

    - Prepare Surface and fix tiles

Overview: This competency standard covers the skills and knowledge required set out and install timber and metal window and door units of different types, with appropriate weather proofing, smooth access and good security, and fix wall tiles

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. - Calculate material quantity in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Prepare materials for tiling application | - Check floor and wall tiling materials to ensure they are suitable and meet specifications. - Carry out material preparation to satisfy the requirements of the specified application process. |
| - Prepare underlay and sheeting substrate | - Provide assistance with underlay preparation under supervision. - Finish substrate surface to specification, with joints flush and sealed. |
| - Prepare render substrate surface | - Detach safely surface-mounted construction units and attachments. - Clean surfaces of substrate structure to remove all loose material. - Mix proportion of materials for splash coat as per instructions ready for application to wet surface. - Prepare horizontal and vertical surrounds for tiling process in accordance with type of tile and specified finish. - Mix proportion of materials for render coat as per instructions ready for application. - Scratch, render, cure and dry Surface in accordance with specifications for tile application. |
| - Cut tiles as required. | - Cut tiles without jagged or damage to tile surfaces or finish, in accordance with workplace procedures and manufacturer recommendations. - Cut by hand or machine recess hole or curve to shape and size and to specified tolerance. - Form a miter by edging tile jolly so that biscuit is not exposed at the joint in accordance with workplace procedures and manufacturer recommendations. |
| - Fix wall tiles | - Prepare and apply mortar and/or adhesive to tile surface in accordance with manufacturer recommendations. - Prepare and fix tiles with pad tiles set to level alignment. - Check horizontal joint for straightness, and check tile edges and surface alignment for conformity. - Fix tiles to alignment maintaining designed pattern to specification. - Cut bottom course and fix to create a rake or square corner in accordance with drawings and specification. - Fix splayed, manufactured, formed coves in accordance with drawings and specifications. - Finish vertical tiles plumb and true to square corners. - Maintain Joints straight and uniform in width with due allowance for tolerance of tile sizes. - Build in control joints, in accordance with specifications and manufacturer recommendations. - Make miter joints, maintaining glazing on miter without damage to tile surfaces or finish and maintaining uniformity of miter in accordance with company procedures and manufacturer recommendations |
| - Apply tiles on external corners. | - Check setting out for plumb, level and check square be within specified tolerance. - Check external corners to ensure surface intersections are straight. - Fix curved bead angle trim or tiles so that external return and bead are square and measurements are accurate to junction with tiles and set out, where applicable. - Fix tiles with minimum voids in tile bed while maintaining fully bedded alignment to specifications. - Keep corner square within specified tolerance and finish to specifications. |
| - Apply tiles on internal corners | - Check internal corner to ensure surfaces are flat and intersection is straight. - Cut tiles where required and fix one wall to maintain alignment in accordance with set out and specifications. - Cut tiles where required and fixed abutting adjacent wall tiles to line, set out and specifications. - Make joints for abutting tiles are made in accordance with designed margin for grouting or for expansion joint, where applicable, to specifications. - Install coved tile or trim to coved internal wall or wall and floor junctions. - Install coved tile or trim so that line is straight and, where applicable, aligned with set out. - Fix tiles to cove tile or trim and finish to alignment and specifications. |
| - Perform grout wall tile face | - Clean and prepare joints to receive grout according to manufacturer specifications. - Mix and apply grout in accordance with workplace and manufacturer specifications and to meet job requirements. - Clean and polish tiles with dry cloth to specifications, removing all dust from surface and joints. |
| - Perform clean up | - Clean work area, and dispose of, reuse or recycle waste in accordance with work specifications. - Clean tools plant and equipment, check for faults and maintain & store in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain surface preparation materials and techniques.

- Enlist control joints

- Explain techniques in fixing tiles to walls and corners, including preparation of substrates

- Describe tiling materials, including tiles, adhesives, mortar, grouting and substrates, their qualities, characteristics, preparation, techniques, applications, limitations and finishing.

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Prepare a concrete bathroom floor and pointed fiber cement sheet wall for tiling

- Prepare a timber bathroom floor and pointed fiber cement sheet wall for tiling

- Tile the front and return of a standard framed or bricked bath, including a straight hob between the bath and wall, using adhesive or mortar to fix the tiles; tiles are not to be proud of the bath and coving to floor is to allow for control joints as required.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Brooms - Brushes - Buckets - Caulking guns - Cement sheet cutters - Concrete mixers - Electrical leads - Hammers - Hose - Water spray - Ladders - Levelling equipment - Lump hammers - Measuring tapes and rules - Mortar boards - Nippers - Pointed grouters - Power drills - Power screwdrivers - Power grinders - Power sanders and sanders - Rags, rubber mallets - Sanding blocks - Saws - Scrapers - Shovels - Spacers and wedges - Spatulas - Sponges - Squares - Squeegees - Straight edges - String lines - Trowels - Wet and dry diamond saws - Wheelbarrows - Wire brushes - Wooden floats - Work platforms Materials may include: - Acoustic underlay material - Caulking compound - Cement mortar (with and without additives) - Clouts - Cornice adhesive - Crack suppression membrane - Fixings and fasteners - Patching materials - Plasterboard nails - Pre-mixed and Mixed fillers - Sand and Cement - Self-tapping screws - Soft sheet nails - Wall board adhesive |

    - Prepare Surface for Painting

Overview: This competency standard covers the skills and knowledge required to prepare new or uncoated surfaces for painting or clear finish, prepare previously coated surfaces for painting or clear finish, remove wallpaper and prepare surface for painting and perform clean up.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. - Calculate material quantity in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Prepare new or uncoated surfaces for painting or clear finish | - Determine suitability of surface for painting or clear finish application in accordance with manufacturer recommendations. - Select surface preparation method in accordance with the environment, finish and substrate requirements. - Prepare surface according to manufacturer specifications in compliance with substrate requirements. - Stop, fill and sand the surface to a smooth finish, ready for painting. |
| - Prepare previously coated surfaces for painting or clear finish | - Determine and test condition and nature of existing substrate and surface material. - Select surface preparation in accordance with the environment, finish and substrate requirements. - Prepare surfaces by removing unwanted coatings and loose debris. - Repair, fill and sand surface defects to smooth finish ready for painting in accordance with specifications. |
| - Remove wallpaper and prepare surface for painting | - Determine type, condition and nature of existing type of wallpaper prior to removal. - Select surface preparation method in accordance with the environment, finish and substrate requirements. - Remove wallpaper using the most appropriate method. - Prepare surfaces for paint application by removing loose debris. - Repair surface defects and stop imperfections to smooth finish ready for painting in accordance with specifications. |
| - Perform clean up | - Clean work area, and dispose of, reuse or recycle waste in accordance with work specifications. - Clean tools plant and equipment, check for faults and maintain & store in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain paint application testing procedures

- Explain prevention and/or rectification procedures for surface coating defects

- Procedures, products and techniques associated with preparation of surfaces and wallpaper removal

- Explain properties and surface preparation requirements of new substrates

- Explain required protection for application of clear or stained finishes

- Explain the term surface coating technology

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Remove wallpaper from a room, cubicle or equivalent and prepare the surface for painting

- Identify repair of surface defects

- Determine type and condition of wallpaper

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Drop sheets - Duster brushes - Filling knives and blades - Hammers - Hand sanders - Heat removal equipment - Mechanical sanders - Nail punches - Putty knives - Scrapers - Water blasters - Wire brushes |

    - Paint by spray, brush and roller

Overview: This competency standard covers the skills and knowledge required to apply brushed, rolled or sprayed paint coatings to different materials to form a protective and decorative finish.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. - Calculate material quantity in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Prepare work area | - Prepare materials and substrate surfaces in accordance with manufacturer recommendations. - Protect surfaces not to be painted by drop sheets, masking or removal of objects. - Provide adequate ventilation to maintain a safe environment. |
| - Mix paint color | - Identify base color by sample analysis or specification, and correct tint base, paint type and determine sheen level. - Select colorants suitable for the color match, mixed with the base, allowed to dry and match against the sample/specification for accuracy. - Mix materials for specified paint finish in accordance with manufacturer recommendations. - Prepare correct amounts of paint material to specified ratio and note drying time in accordance with manufacturer specification. |
| - Apply paint with brush or roller | - Select brush and/or roller to suit specified surface profile, size of area, type of paint and finish specified. - Apply sealant, undercoat, intermediate coat and finish coat by brush/roller to achieve the required level of opacity, finish, texture and sheen, in accordance with specifications. |
| - Apply paint by spray | - Select appropriate spray equipment, accessories and lines and set up for operation in accordance with the manufacturer’s recommendations. - Mix paint and adjust viscosity to allow for spray painting. - Apply paint to achieve correct finish specifications. |
| - Finish the application | - Cure and test finished paint surface using curing method in accordance with manufacturer recommendations. - Reinstall doors, windows and furniture removed for painting application correctly and without damage to finished |
| - Perform clean up | - Clean work area, and dispose of, reuse or recycle waste in accordance with work specifications. - Clean tools plant and equipment, check for faults and maintain & store in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe compatibility factors of preparatory materials and paint systems

- Explain hazards associated with solvents, chemicals and dust

- Explain surface coating technology, including specification of paint systems for interior and exterior painting projects to maximize durability, protection and aesthetic considerations

- Explain testing techniques and support materials

- Enlist theoretical principles relating to adhesion and cohesion of paint

- Explain theoretical principles relating to pigmentation and coloring agents, drying and curing processes and the role of solvents

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Explain minimum application of six square meters brushed or rolled

- Characteristics of timber panel door

- Explain timber window, including architrave and frame with moving parts

- Enlist four different surface type.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | - Brush ware - Compressor - Conventional spray unit - Diaphragm - Drop sheets - Filling knives and blades filters - Hand sanders - Heat and flame paint removal equipment - High-volume low-pressure (hvlp) spray - Hose - Masking equipment - Mechanical sanders - Nail punches - Paint pots and buckets - Paint stirrers - Piston airless spray unit (electrical, pneumatic and petrol) - Putty knives - Regulator - Roller frames - Scrapers - Spray equipment - Spray guns |

    - Apply stains and clear timber finishes

Overview: This competency standard covers the skills and knowledge required to apply stains and clear timber finishes to different material surfaces, to form a protective and decorative finish.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Plan and prepare. | - Confirm work instructions from supervisor. - Check serviceability of tools and equipment - Observe safety requirements, and warnings, including use of personal protective equipment, throughout the work. - Calculate material quantity in accordance with specifications. - Obtain materials needed, check for compliance and prepare. |
| - Prepare materials and application area | - Protect adjoining surfaces by masking off or covering. - Provide adequate ventilation to maintain a safe environment. - Take measures to ensure application area remains free of dust and foreign matter. - Apply strips on existing stained or finished surfaces using appropriate techniques. |
| - Apply stain to bare timber surface. | - Select stain for type of timber in accordance with manufacturer recommendations. - Prepare stain to proportions and consistency in accordance with manufacturer recommendations. - Select application method for the specified surface, area size and type of finish. - Apply stain to bare timber surface according to specifications. - Select wood filler and putty, mixed, color-matched and applied to timber in accordance with specifications. |
| - Apply finish surface | - Apply coats of selected clear finish to achieve required level of opacity, finish and polish in accordance with specifications. - Allow drying time between coats and cure clear finish surfaces in accordance with manufacturer recommendations. |
| - Perform clean up | - Clean work area, and dispose of, reuse or recycle waste in accordance with work specifications. - Clean tools plant and equipment, check for faults and maintain & store in accordance with workplace practices |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain hazards associated with solvents, chemicals and dust

- Explain surface preparation techniques for clear wood finishing

- Explain testing techniques and support materials

- Explain theoretical principles relating to pigmentation and coloring agents, drying and curing processes and the role of solvents

- Explain types, properties, uses and limitations of clear timber finishes and timber stains

- Explain types, uses and limitations of commonly used brushes, rollers and other applicators

Critical Evidence(s) Required

The candidate needs to produce following critical evidences in order to competent in this competency standard.

- Prepare and or strip surfaces; and

- Select and apply stains and finishes to timber in accordance with specifications and to suit the timber type, condition and finish required.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Construction Lab Tools |
|  | - Brush ware and accessories, buckets - Drop sheets, duster brushes - Filling knives and blades - Hammers - Heat guns - Nail punches - Paint pots and buckets - Paint stirrers - putty knives - Roller frames and accessories - Sanders – hand and mechanical - Scrapers - Spray equipment |

Tools and Equipment

| Sr. # | Description |
| --- | --- |
| 1 | Calculator |
| 2 | Ruler |
| 3 | Pencil |
| 4 | Construction Lab Tools |
|  | - Adjustable metal props - Bolsters - Brick buggies and brick grabs - Brooms - Buckets - Builders’ lines - Concrete mixers - Dumpy levels - Elevators - Forklifts - Hammers (bricks, club and scutch), hoses - Jig saws - Jointing tools - Line blocks and line pins - Margin or raking tools - Masonry saws - Masonry squares - Materials hoists - Measuring tapes and rules - Mortar boards - Pallet trolleys - Plumb rules - Profiles - Scaffolds - Shovels - Small petrol or diesel engines and compressors - Spirit levels - Straight edges - String lines - Trowels - Wheelbarrows - Brush ware and accessories - Buckets - Drop sheets - Duster brushes - Filling knives and blades - Hammers, heat guns - Nail punches - Paint pots and bucket - Paint stirrers - Putty knives - Roller frames and accessories - Sanders – hand and mechanical - Scrapers - Spray equipment |

    - Prepare site layout of civil engineering projects

. Overview: This competency standard covers the skills and knowledge required to prepare for work, prepare site for temporary fencing and structures, and prepare the erection of site security fencing/hoarding, interpret construction of temporary site structures and organize storage of tools and machine accessories

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare for work | - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details. - Follow Safety (OHS) requirements in Accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Select plant, tools and equipment to carry out tasks consistent with job requirements, check for serviceability, and any faults - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Identify, obtain, prepare, and safely handle materials appropriate to the work application |
| CU2. Prepare site for temporary fencing and structures | P1. Identify services, location and work areas are accurately P2. Place adequate warning signs and safeguards to support and protect services and substructures identified in work area. P3. Set up and secure temporary signs, control systems and protective barriers in accordance with job and safety instructions. |
| CU3. Prepare the erection of site security fencing/hoarding | P1, Select appropriate type fencing materials in adequate quantities. P2. Confirm dimensions of installed fence components to job specifications and/or work instructions. P3. Check erected fence as meeting security and enclosure requirements. |
| CU4. Interpret construction of temporary site structures | P1. Interpret instruction/specification for temporary site structures accurately to the relevant persons. P2. Lay out temporary site structures to facilitate ease of access and to prevent obstruction of other site activities. P3. Check erected temporary site structures as conforming to job specification/ instruction given. |
| CU5. Organize storage of tools and machine accessories | P1. Lay out storage facilities correctly and built to specifications. P2. Correctly demonstrate working knowledge of the type of items to be stored P3. Organize store for proper delivery, checking and control of materials, tools and equipment. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain types and purpose of job information/instructions

- Explain OH&S requirements for preparing construction sites

- Demonstrate physical conditions of site

- Enlist use of tools, machines and equipment

- Describe site fencing and hoarding

- Explain relevant statutory requirements relating to site boundaries

- Demonstrate traffic access and routes

- Explain area for storage and control of tools and materials

- Procedure for convenient, visible and safe storage

- Explain quality standards and specifications

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Compliance with occupational health and safety regulations applicable to workplace operations

- Application of organizational management policies and procedures including quality assurance requirements where applicable

- Demonstrate ability to read and interpret job related documentations

- Demonstrate ability to co-ordinate the clearing of construction sites

- Demonstrate ability to co-ordinate the preparation of access route, and the construction of temporary fencing/hoarding and structures

- Demonstrate ability to organize the storage of tools, equipment and materials on site

- Relate to people from varied social, cultural and ethnic background

- Evidence is best gathered through a holistic assessment activity, which integrates the elements of competency

Tools and Equipment

| Sr. # | Description |
| --- | --- |
| 1 | Calculator |
| 2 | Ruler |
| 3 | Pencil |
| 4 | Fencing |
| 5 | Hoarding |
| 6 | Barriers |

If workplace based, the resources should relate specifically to organization policies, guidelines, requirements, resources and equipment.

Resources may include:

• A real or simulated environment

• Project resources

• Organization policies and guidelines

• Access to relevant resource personnel

• Relevant legal and statutory documentation

    - Construct masonry steps and stairs

. Overview: This competency standard covers the skills and knowledge required to construct masonry steps, stairs and wing walls for different types and styles of buildings. It includes planning, types and styles of stairs and set out and installation of the masonry.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare for work. | P1. Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details, from relevant information to determine bricklaying and block laying tasks. P2. Follow safety (OHS) requirements in accordance with safety plans and policies. P3. Identify and implement signage and barricade requirements. P4. Select plant, tools and equipment to carry out tasks consistent with job requirements, check for serviceability, and any faults P5. Calculate material quantity requirements in accordance with plans, specifications and quality requirements. P6. Identify, obtain prepare, and safely handle Materials appropriate to the work application are and located ready for use. |
| CU2.Set out brickwork/ block work. | P1. Check work platform in accordance with regulatory and workplace requirements and brick and block stair construction requirements. P2. Check location and relative level of prepared footing from job drawings and specifications. P3. Calculate rise and fall of flight and individual steps as per codes and workplace requirements. |
| CU3.Lay bricks/blocks forming steps and wing walls. | - Prepare mortar mix to required conformity and specifications. - Construct base brickwork to specifications and requirements of standards and codes. - Construct square steps, level, and plumb and lay to specified bond. - Develop design and profile of steps and aligned to bond and plumb to specifications. - Carry out joints according to work specifications. - Set out brickwork/block work laid to line, with gauge and complete to work specifications. - Clean brick/block faces free of mortar |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain brick bond patterns characteristics and applications of materials for masonry steps and stairs construction

- Explain brick and block expansion and growth, control and articulation joints

- Define construction terminology

- Explain materials storage and environmentally friendly waste management

- Draw plans, specifications and drawings

- Explain processes for the calculation of material requirements

- Demonstrate quality requirements

- Explain techniques of masonry steps and stairs construction

- Explain types, characteristics, uses and limitations of plant, tools and equipment

- Demonstrate workplace and equipment safety requirements

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to:

- Locate, interpret and apply relevant information, standards and specifications

- Safely and effectively use tools, plant and equipment

- Communicate and work effectively and safely with others

- As a minimum, construct a flight of stairs with a minimum of three pre-cast treads, and

- Construct a flight of three solid masonry treads with a parallel wing wall on one side, ensuring:

  - correct identification of requirement and finishing of the tasks

  - correct selection and use of appropriate processes, tools and equipment

  - Completing all work to specification.

Tools and equipment:

- Bolsters

- Brick grabs

- Brooms

- Buckets

- Builders' lines

- Concrete mixers

- Dumpy levels

- Elevators

- Hammers (bricks, club and scotch)

- Hoses

- Jig saws

- Jointing tools

- Line blocks

- Line pins

- Mason's squares

- Masonry saws

- Measuring tapes and rules

- Mortar boards

- Plumb rules

- Margin or raking tools

- Profiles

- Shovels

- Spirit levels

- Straight edges

- String lines

- Trowels

- Wheelbarrows

    - Install stairs

. Overview: This competency standard covers the skills and knowledge required to construct masonry steps, stairs and wing walls for different types and styles of buildings. It includes planning, preparation, set out and installation of the stairs.

| Competency Units | Performance Criteria |
| --- | --- |
| - Plan and prepare materials | - Confirm work instructions in accordance supervisor. - Select required tools and equipment, serviceability and any faults. - Calculate materials quantity requirements accordance with specifications. - Obtain materials needed for compliance, prepared quality requirements. |
| - Select and prepare materials and components | P1. Identify methods of assembling erected for stairs P2. Check components for appropriate locations in stair structure. P3. Determine method of assembling and fixing in accordance with stair design and location |
| CU3. Assemble strings and newels | P1. Identify Specific position for stairs P2. Check measurements and adjustments if applicable. P3. Assemble Strings and newels to design and specification. P4. Fix strings to walls temporarily supported or directly fixed in position /specification. |
| CU4. Install treads and risers | - Assemble strings and newels temporarily braced in vertical position. - Fix treads and risers about newels to assembly - Check flight is true and square. - Fit intermediate treads and risers. |
| CU5. Assemble and install landings | P1. Fix bearers, joists and level according to specification. P2. Fix nosing and floor landing according to finish specification |
| CU6. Install handrail and balustrade | - Fix balusters/intermediate railing and handrails to form stair balustrade according to specification, - Check and ensure plumb fit of Newels prior to final fixing - Fix hand railings fitted to wall in accordance with specifications |
| CU7.Install spiral stair and curved strings | P1. Mark location of stair and first step on floor and erect central post is into true position, fixed at floor and temporarily braced at top. P2. Support initial string section temporarily supported in place for assembly, and fit treads and risers into position to specification. P3. Develop stair progressively with the extending, supporting and fixing of curved string, and complete with head secured to floor/landing, balustrade installed and central post fixed to specifications. |
| CU8. Secure stair to structure and line spandrel area | P1. Carry out securing of stair to building during/on completion of assembly. P2. Frame, line and fix spandrel, where applicable, to specified finish. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Enlist adhesives, fixings and fasteners related to stair construction

- Explain assembling procedures for stairs

- Explain leveling techniques

- Explain procedure for marking of components

- Explain materials and their characteristics, relevant to stair construction

- Explain procedure for measuring and setting out related to assembling and installing stairs

- Explain stair construction and joining methods

- Enlist types of stairs

- Explain selection criteria and use of safe and efficient procedures in installing treads, risers, flooring and nosing

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Accurately set out stair location and check levels for adjustments on newels;

- Adopt and use appropriate techniques to fit and fix balustrades;

- Complete installation with stair true to plumb and level, and fixed securely to structure with

- Surfaces finished free of marks;

- Demonstrate safe and effective procedures in assembling strings and newels and installing

- Landing bearers and joists;

- Select and use appropriate processes, tools and equipment to assemble stair components;

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: - Mason hammer - Pointing towels - Measuring tape - Mason thread - Plumb bob - Leveling tools (water level, spirit level) - Bucket - Mortar pan - Wooden float - Steel float - Shovel - Spade - Wire brush - Screeds (wooden/steel). |

    - Prepare for solid dry wall plastering

.

Overview: This competency standard covers the skills and knowledge required to effectively carry out the preparation process of dry wall plastering, and applies to effectively prepare the process for carrying out solid plastering work, and applies to individuals working in masonry in the construction industry.

| Competency Units | Performance Criteria |
| --- | --- |
| CUI. Plan for construction process | P1. Assure quality requirements of company’s construction operations P2. Identify preparation and planning requirements from drawings/work location and/or supervisor’s instructions. P3. Observe OH&S requirements and accordance with application tasks and workplace environment. P4. Identify safety hazards and correct procedures in order to minimize risk to self and others. P5. Select materials to supervisor’s instructions, P6.Select appropriate personal protective equipment, correctly. P6.Select tools and equipment consistent with the job requirements. P7. Select fixtures/fasteners consistent with job requirements and check for specifications, quality requirements and operational details, P8. Calculate materials quantity requirements in accordance with plans, specifications and quality requirements. P9. Identify materials appropriate to the work application are, obtained, prepared, safely handled and located ready for use. |
| CU2. Prepare work area suitable forconstruction process | - Identify activities to be carried out in work area from surfaces to be lined and height to be accessed. - Prepare work area for construction process to supervisor’s instructions |
| CU3. Use tools, plant and equipmentappropriate for construction process | P1. Use regular hand and power tools suitable for application process identified to job requirements. P2. Carry out hand and power tools used safely and effectively processes |
| CU4. Use tools, plant and equipmentappropriate for constructionprocess | P1. Identified Regular hand and power tools suitable for application process to job requirements. P2. Use Hand and power tools safely and effectively to carry out processes. |
| CU5. Prepare background of brick,concrete or blockwork for solid plastering | - Identify Structure and surface prepared. Depressions patched with suitable material to supervisor’s instructions. - Apply adhesive Concrete surface where appropriate rough. - Mix materials for scratch coat to instructions ready for application to wet surface. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain workplace and equipment safety requirements

- Explain drawings and specifications portable power tools and tools and equipment materials relative to dry wall plastering materials handling

  - Explain measurement relative to dry wall plastering

  - Explain fixing and fasteners consistent with dry wall plastering requirements

  - Demonstrate workplace communication sand and cement

  - Define plaster

  - Define cornice adhesive

  - Enlist caulking compounds

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate compliance with occupational health and safety regulations applicable to work operations at all times.

- Mix materials as per the job requirement following standard procedures.

- Plaster to required alignments and thickness following standard practices.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: - Mason hammer - Pointing towels - Measuring tape - Mason thread - Plumb bob - Leveling tools (water level, spirit level) - Bucket - Mortar pan - Wooden float - Steel float - Shovel - Spade - Wire brush - Screeds (wooden/steel). |

    - Apply plaster

Overview: This competency standard covers the skills and knowledge required to prepare for work, mortar mix, and scaffolding and perform plastering work.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare for work | - Work according to instructions, plans, specifications, quality requirements and operational details, from relevant information to determine bricklaying and block laying tasks. - Safety (OHS) requirements are followed in Accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Select tools and equipment to carry out tasks are consistent with job requirements, check for serviceability and any faults are rectified or reported prior to commencement. - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Identify materials appropriate to the work application. |
| CU2.Prepare mortar mix | P1. Select PPE, tools and equipment in accordance with the job requirements. P2. Estimate materials required as per job specifications. P3. Prepare the surface/mixing machines for mixing of mortar as per the job requirement. P4. Mix materials as per the job requirement following standard procedures. P5. Transport mortar mix as per the job requirement following standard procedures. |
| CU3. Prepare scaffolding | P1. Select and use PPE according to the job requirements. P2. Select and use tools and equipment as per the job requirement following standard procedures. P3. Prepare estimate of materials required based on the job requirements. P4. Prepare scaffolds components as per the job requirement following standard procedures P5. Erect and assemble scaffolds components as per the job requirements following standard procedures. P6. Dismantle scaffolding as per the job requirement following standard procedures. P7. Store or stockpile components according to standard procedures. |
| CU4. Perform plastering work | P1. Prepare the surface to be plastered as per the job requirement following standard. P2. Prepare and apply cement slurry as per the job requirement following standard practices. P3. Apply mortar mix as per the job requirement to the required specifications following standard procedures. P4. Cure the plastered surfaces as per the job requirement following standard practices. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Explain occupational Health and Safety (OHS) Regulations

- Explain composition, properties and specifications of plastering materials

- Describe economic use of materials

- Explain procedure for estimation of materials

- Describe purpose of plastering/pointing

- Enlist types of plaster/pointing

- Explain methods of plastering/pointing

- Explain defects in plastering/pointing

- Define curing

- Demonstrate repair and maintenance work

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate compliance with occupational health and safety regulations applicable to work operations at all times.

- Mix materials as per the job requirement following standard procedures.

- Plaster to required alignments and thickness following standard practices.

Instruments & Consumables

| S No. | Description (Instruments) |
| --- | --- |
| 1 | Personal protective equipment for construction industry. |
| 2 | Construction Lab Equipment: - Mason hammer - Pointing towels - Measuring tape - Mason thread - Plumb bob - Leveling tools (water level, spirit level) - Bucket - Mortar pan - Wooden float - Steel float - Shovel - Spade - Wire brush - Screeds (wooden/steel). |

  - Plumbing

    - Identify plumbing tools & equipment

Overview: This competency standard covers the skills and knowledge required to interpret layout drawing for Plumbing Lab its specifications, its Requirement for tools and equipment.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw Layout drawing for Plumbing Lab | P1. Mark North Point. P2. Measure the Lab dimension. P3. Develop layout drawing P4. Analyse Signs, Symbols and data P5. Identify location for Doors and windows. P6. Identify instruments places. P7. Identify sitting plan. |
| - Identify Plumbing Tools & Equipment | P1. Identify Tools & equipment P2. Check for safety hazards P3. Conduct operational testing P4. Maintain professional proficiency |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1. Explain safety requirements, for the required tools for task.

K2.  Explain lay out drawings with its type

K3.  Explain different specification for tools and equipment

K4.  Explain use of different plumbing tools

K5.   Describe different symbols for plumbing

K6.   Describe Plumbing History and Plumbing importance

K7.   Explain Safety measures while performing Plumbing tasks

K8.  Explain different drawing instruments

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare layout drawing of lab

- Create sketches and drawing

- Preparation is done according to area and standards

- Prepare space for entire work

- Plan for relevant job

- Adopt safety measures

TOOLS AND EQUIPMENT’S

- Pencil

- Eraser

- Sharpner

- Calculator

- Measuring scale

- Drawing sheet

- Drawing board

- Tee Square

- Set Square

- Measuring Tape

- Plumbing lab tools.

    - Perform cutting, threading & reaming

Overview: This competency standard covers the skills and knowledge required to perform pipe cutting, its threading and reaming.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform Pipe Cutting. | P1. Identify tools for cutting. P2. Secure work piece firmly P3. Measure required Length P3. Use cutter OR Use Hacksaw P4. Loose the cutting Piece P6. Check length for cutting piece P7. Observe OHSA |
| - Perform Pipe Threading. | P1. Identify tools for threading. P2. Adopt safety measures P3. Fix work piece firmly P4. Clamp Threading die P5. Prepare threading P6. Unclamp the threading die P7. Observe OHSA |
| - Perform Pipe Reaming. | P1. Identify tools for Reaming. P2. Adopt safety measures P3. Fix work piece firmly P4. Clamp Threading pipe piece P5. Use reamer P6. Unclamp pipe piece P7. Observe OHSA |

Knowledge & Understanding.

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1.    Comprehend the purpose and application of plumbing tools

K2.     Knowledge of plumbing tools (pipe wrench, spanner, screw drivers, pipe cutters etc.)

K3.     Knowledge of Plumbing equipment (Threading die, Hack Saw, Bench Vice, Leg Vice and reamer)

K4-    Describe calibration of tools.

K5.    Explain safety precaution for using tools

K6.    Explain different type of Dies

K7.    Explain the procedure to use of threading die

K8.   Explain the procedure to use of Hack saw

K9.   Explain the procedure to use of Reamer

K10. Enlist clear work place procedure

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Define tools with it usage

- Define safety measures

- Fix and Change blade for the die

- Preparation is done according to area and standards

- Prepare space for entire work

- Plan for relevant job

TOOLS AND EQUIPMENT’S

- Pipe Cutting Tools,

- Plumbing pliers,

- Plumbing wrenches,

- Plumbing spanners,

- PPEs

- Threading Tools

- Reaming Tools

    - Join pipes & fittings

Overview: This competency standard covers the skills and knowledge required to prepare for different type of pipe joints and it’s fitting on residential and commercial plumbing works.

| Competency Units | Performance Criteria |
| --- | --- |
| - Undertake pipe fitting and Installation | P1. Observe basic principles for pipe fittings P2. Identify and select materials for task P3. Identify tools and equipment for task P4. Attach Pipe with fittings P5. Locate fixtures properly P6. Undertake pipe leakage test P7. Use PPEs |
| - Install Hot and Cold Water Supply Pipes | P1. Observe basic principles for pipe fitting P2. Identify and select materials for task P3. Identify tools and equipment for task P4. Attach cold Pipe with fittings P5. Attach Hot Pipe with Geyser and fittings P6. Locate cross over properly P7. Locate drainage pipes P8. Undertake pipe leakage test P9. Use PPEs |
| - Join Copper Pipes | P1. Observe basic principles for Joining pipes P2. Select materials for task P3. Identify tools and equipment for task P4. Melt Lead P5. Fill lead in collar of pipe with proper tool P6. Undertake pipe leakage test P7. Use PPEs |
| - Join G.I Pipes | P1. Observe basic principles for G.I. pipes P2. Select materials for task P3. Identify tools and equipment for task P4. Perform threading & cutting on pipes P5. Tight pipe with socket P6. Undertake pipe leakage test P9. Use PPEs |
| - Join PPRC Pipes | P1. Observe basic principles for PPRC pipes P2. Adopt safety measures P3. Identify and select materials for task P4. Select tools and equipment for task P5. Switch on heater P6. Join Heated pipe to required point P7. Undertake pipe leakage test P8. Use PPEs |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1- Knowledge of Safety requirements for installing cold and hot water pipes in residential buildings.

K2.  Knowledge of Fittings attach with hot water

K3.  Explain signs and symbols for Water supply system

K4.  Explain specifications for cold and hot water connections

K5- Knowledge of faults in hot water pipes

K6- Explain use of safety valves in geysers

K7.   Explain the equipment for installing and fittings of pipes

K8. Explain drain Pipes

K9.  Explain method for different type of pipe joints

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Explain safety requirements at work place

- Prepare plan for installation/replacement

- Performing installation/replacement

- Perform fittings for installation of geyser

- Perform different pipe fittings methods.

- Perform safety measures

- Perform different pipe jointing techniques

TOOLS AND EQUIPMENT’S

- Pipe Benders,

- Pipe Cutting Tools,

- Deburrers,

- Plumbing pliers,

- Plumbing wrenches,

- Plumbing spanners,

- Pressure testing kits,

- PPEs

- Hot water fittings

- Hot water accessories

- G.I. & Cost Iron Pipes according to specifications

- Pipe Vice

- Hack Saw with blade

- Different Fittings

- Geyser

- Safety Valve

- Cost Iron Pipe

- Lead

- PPRC Pipe with fittings

- Cocking Tools

    - Layout for domestic water supply

Overview: This competency standard covers the skills and knowledge required for layout domestic water supply in residence and residential schemes.

| Competency Units | Performance Criteria |
| --- | --- |
| CS-1. Prepare for works | P1. Interpret work instructions P2. Plan according to job requirements P3. Organise duties P4. Select appropriate tools & equipment P5. Select piping materials and plumbing fixtures P6. Select PPEs |
| CS-2. Lay out for water supply scheme | P1. Mark North Point on drawing sheet P2. Measure the Scheme area. P3. Interpret layout plan by scale P4. Identify sources of water supply P5. Indicate Water Conveyance System. P6. Show storage Reservoirs P7. Locate Main, sub main and branches |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1- Define water Layout system.

K2- Define fundamentals of water reservoirs

K3- Explain Types of water sources

K4- Describe applications of water connections with main to branches

K5- Knowledge to prepare work OR Task.

K6- Knowledge of safety measures.

K7. Explain gravity and pressure supply of water

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Study of different water supply system

- Installation of water supply scheme layout

- Indicate water reservoir

- Indicate gravity and pumping system

- Clarity to organise duties to perform task

TOOLS AND EQUIPMENT’S

- Pencil

- Eraser

- Sharpner

- Calculator

- Measuring scale

- Drawing sheet

- Drawing board

- Tee Square

- Set Square

- Measuring Tape

- Computer

- AutoCAD

- PPEs

    - Make water connection

Overview: This competency standard covers the skills and knowledge required to make the new connection for house from the main water.

| Competency Units | Performance Criteria |
| --- | --- |
| - Make water connection | P1- Identify work place P2. Select tools and equipment P3- Measure pipe P4- Prepare pipe as per length P5- Join Clamp on main pipe P6. Fix the valve P7. Adjust branch pipe with fitting P8. Perform final testing of pipes |
| - Clean up Work site area | P1. Clear work area by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1-Describe different tools and equipment for making new connection for water supply

K2- Explain taking connection procedure

K3- Explain cutting and threading procedures

K4.  Explain PPEs

K5.  Explain use of different plumbing tools

K6. Explain clamps with its type

K7.  Explain use of valve while taking connection

K8.  Explain branch and main pipe

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Mount pipe connection perfectly

- Use of cutting, threading and connections without leakage

- Use Safety measure at place

- Use effective tools for taking connection

- Fixing of clamp on main pipe

- Preparation is done according to area and standards

- Prepare work area for performance

- Plan for relevant job

TOOLS AND EQUIPMENT’S

- Pipe Cutting Tools,

- Deburrers OR Reamers

- Plumbing pliers,

- Plumbing wrenches,

- Plumbing spanners,

- Plungers,

- Pressure testing kits,

- PPEs

- Clamps for pipe

- Pipe (G.I OR PPRC)

    - Install plumbing fixtures

Overview: This competency standard covers the skills and knowledge required to install plumbing fixtures at work place i-e installation of water closet, flushing cistern, Wash hand Basin, English WC and Urinal

| Competency Units | Performance Criteria |
| --- | --- |
| - Install Wash Hand Basin | P1. Identify safety requirements for installation P2. Identify tools for work P3. Identify Fixtures P4. Perform measurements P5. Mark out Installation point P6. Fix Pedestal by Drilling P7. Install Wash hand Basin P8. Fix Pillar cock, Rubber connection, waste pipes with coupling P9. Connect to drain pipe |
| - Install English water closet | P1. Mark out Installation point P2. Install the closet flange P3. Secure it to the subfloor P4. Connect outlet pipes with water supply P5. Perform final quality inspection P6. Check Level |
| - Install English water closet | P1. Mark out Installation point P2. Fix floor trap P3. Set WC at Level P4. Connect outlet pipes with water supply P5. Perform final quality inspection P6. Check Level P7. Use PPEs |
| - Install Flushing Cistern | P1. Mark out Installation point P2. Measure height for flushing cistern P3. Fix flushing pipe P4. Connect inlet pipe with water supply P5. Perform final quality inspection P6. Check Level P7. Use PPEs |
| - Install Urinal | P1. Mark out Installation point P2. Measure Height for flushing cistern P3. Fix Urinal with flushing pipe P4. Connect inlet pipe with water supply P5. Perform final quality inspection P6. Check Level P7. Use PPEs |
| - Install Sink | P1. Mark out Installation point P2. Measure height for sink P3. Fix sink fat trap with drain pipe P4. Connect inlet pipe with water supply P5. Perform final quality inspection P6. Check Level P7. Conduct Operational Testing P8. Use PPEs |
| - Install Bath Tub | P1. Mark out Installation point P2. Measure Height for bath tub P3. Fix Bath Tub with drain pipe P4. Connect inlet pipe with water supply P5. Perform leakage test P6. Perform Safety Precaution P7. Use PPEs |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1- Knowledge of water closet, Flushing Cistern with its types

K2- Use of tools and equipment’s

K3- Knowledge of joining techniques and with fixtures

K4- Knowledge of levelling and fixing WC

K5. Define WC with its types

K6.  Describe various trap fittings

K7.  Explain wash hand basin Pedestal type

K8.  Explain fitting procedure to install wash hand basin

Critical Evidence(s)

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Fixing and use of water closet and flushing cistern

- Demonstrate the installation of water closet and flushing cistern

- Install wash hand basin and shower

- Install geyser with its fitting and valve

- Fix trap and measure the size

- Installation of urinal

TOOLS AND EQUIPMENT’S

- Pipe Benders,

- Pipe Cutting Tools,

- Pipe vice with stand,

- Deburrers,

- Plumbing pliers,

- Plumbing wrenches,

- Plumbing spanners,

- Plungers,

- Pressure testing kits,

- PPEs

- P-Trap

- W.C (Indian & English)

- Wash hand basin

- Shower

- Geyser

- Bolt Kit

- Screws

- Clamps

- Nails

- Hammer

- Bath Tub with accessories

- Mallet

- Urinal with fitting

- Urinal Cistern

- Tee cock

- Pillar cock

- Sink with mixer

    - Install & repair water pumps

Overview: This competency standard covers the skills and knowledge required to install drinking water pumps and its repair work.

| Competency Units | Performance Criteria |
| --- | --- |
| - Install Air ejector Pump | P1. Mark out Installation point P2. Perform Safety Precaution P3. Measure Pipes with fittings P4. Fix pump with suction & delivery pump P5. Connect with electricity P6. Perform leakage test P7. Use PPEs |
| - Install Centrfugal Pump | P1. Mark out Installation point P2. Perform Safety Precaution P3. Measure Pipes with fittings P4. Fix flanges with suction & delivery pipes P5. Connect with electricity P6. Perform leakage test P7. Use PPEs |
| - Install Rotary Pump | P1. Mark out Installation point P2. Perform Safety Precaution P3. Measure Pipes with fittings P4. Fix flanges with suction & delivery pipes P5. Connect with electricity P6. Perform leakage test P7. Use PPEs |
| - Carry out repair of Pump | P1. Perform Safety Precaution P2. Disconnect electricity and pump P3. Use PPEs P4. Check for Fault P5. Repair fault using proper tools P6. Refit with suction & delivery pipes P7. Connect with electricity P8. Perform leakage test |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1. Use of tools and equipment’s

K2. Knowledge of joining techniques and pipe fittings

K3- Knowledge of leakage in pipes and pumps

K4.  Explain working of different pumps

K5.  Explain repairing of different pumps.

K6.  Explain safety measures while performing repair of works

K7.  Explain leakage checking with its methods

Critical Evidence(s)

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Fit and refit the pump

- Install different pumps with its fittings

- Install flanges with pumps and pipes

- Check for leakage

TOOLS AND EQUIPMENT’S

- Pipe Benders,

- Pipe Cutting Tools,

- Chain Pipe Wrench

- Deburrers,

- Plumbing pliers,

- Plumbing wrenches,

- Plumbing spanners,

- Plungers,

- Pressure testing kits,

- Radiator tools

- PPEs

- Different Pumps with fittings

- Electricity

    - Install turbine pumps

Overview: This competency standard covers the skills and knowledge required to Install turbine

| Competency Units | Performance Criteria |
| --- | --- |
| - Install turbine | P1. Mark out Installation point P2. Use PPEs P3. Check depth of bore P4. Adopt Safety Measures P5. Measure length of turbine Pipes with fittings P6. Fix Turbine with flange &Pipes P7. Insert turbine into bore with tripod & chain pulley P8. Connect Motor with shaft fittings P9. Connect electricity and start P10. Remove tripod P11. Check discharge |
| - Clean-up work site area | P1. Clear work area by disposing of materials (reused or recycled) in accordance to legislation and regulations P2. Clean, check, maintain and store tools and equipment according to requirement |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1- Knowledge of turbine with its working

K2- Explain Use of tools and equipment’s for turbine fitting

K3- Explain different joining techniques and pipe fittings

K4- Knowledge of leakage testing

K5. Explain turbine impellers with its fittings

K6.  Explain installation method into bore.

K7.  Explain tripod and chain pulley function

Critical Evidence(s)

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use of turbine

- Installation of Turbine

- Discharge finding

- Install tripod with chain pulley

- Function of Chain pulley

TOOLS AND EQUIPMENT’S

- Pipe Cutting Tools,

- Chain Pipe Wrench

- Deburrers,

- Plumbing pliers,

- Plumbing wrenches,

- Plumbing spanners,

- Pressure testing kits,

- Radiator tools

- PPEs

- Turbine with impellers & Motor

- Electricity motor

- Turbine pipe with fittings

- Pipe wooden / steel clamps

  - Digital Skills

    - Install computer operating systems and hardware

Overview: This unit describes the performance outcomes, skills and knowledge required to select, configure and use computer operating systems and basic computer hardware.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Identify operating system and hardware components | 1.1 Determine ICT organizational requirements and specifications 1.2 Identify and select operating system 1.3 Identify appropriate external hardware components 1.4 Identify internal hardware components |
| 2. Install and configure operating system and application software with hardware components | 2.1 Install and configure operating system to meet organizational requirements 2.2 Identify the functions associated with the operating system and associated boot process 2.3 Configure power-management settings to minimize power consumption as an environmentally sustainable measure 2.4 Use both the graphical user interface and the command line interface to perform basic tasks 2.5 Install or upgrade application software onto the operating system and hardware configuration 2.6 Determine the relationship between an application program, the operating system and hardware 2.7 Identify general differences between the different computer platforms and their respective operating systems |
| 3. Optimize operating system and hardware components | 3.1 Optimize operating system using included tools or third-party utilities 3.2 Customize the graphical user interface 3.3 Use techniques unique to the command line interface 3.4 Set up and configure external hardware components and check functionality 3.5 Install drivers as appropriate and check functionality |

Knowledge & Understanding

K1: Basic knowledge of current industry-accepted operating system, hardware and software products

K2: Compatibility of an operating system, in respect to other versions

K3: Function of single-user and multi-user operating systems

K4: Interoperability between operating systems

K5: OHS principles and responsibilities, including ergonomic principles to avoid injury associated with using computer systems.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to select, configure and use computer operating systems and basic computer hardware. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Use an operating system in a variety of scenarios and across functions, including:

  - scheduling, loading, initiating, and supervising the execution of programs

  - allocating storage

  - initiating and controlling input and output operations

  - handling errors

- Identify and install suitable hardware components

- Install and upgrade application software.

    - Operate digital media technology

Overview: This unit describes the performance outcomes, skills and knowledge required to identify, select and use a digital media package and supporting technologies.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Use appropriate OHS office work practices | 1.1 Use safe work practices to ensure ergonomic, work organization, energy and resource conservation requirements are addressed 1.2 Use wrist rests and document holders where appropriate 1.3 Use monitor anti-glare and radiation reduction screens where appropriate |
| 2. Identify and select appropriate digital media package | 2.1 Identify the basic requirements of a design brief, including user environment 2.2 Research and review suitable available digital media packages 2.3 Select an appropriate digital media package to meet design brief requirements |
| 3. Use digital media package | 3.1 Procure or create suitable data to meet requirements of the brief 3.2 Manipulate data using digital media package tools 3.3 Ensure naming and storing of documents in appropriate file format in directories or folders |
| 4. Review digital media design | 4.1 Evaluate design for creative, dramatic and technical quality, file size, and suitability to meet the brief 4.2 Test and run any incorporated graphics, video or sound as part of a digital media presentation and present designs in the appropriate format 4.3 Review final product against design brief |

Knowledge & Understanding

K1: Basic principles of visual design

K2: Functions and features of digital media packages and technologies

K3: Graphic design and stylistic language conventions

K4: OHS principles and responsibilities for ergonomics, such as work periods and breaks

K5: Principles of digital imaging and file formats, video and sound file formats, file management and transfer systems

K6: Vendor product directions in digital media hardware and software

K7: Visualization and interpreting creative information, scripts (text) and images

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to identify, select and use a digital media package and supporting technologies. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorised work practices, safety requirements and environmental constraints. Demonstrated evidence of the ability to:

- Identify basic requirements of a design brief

- Use digital media package to meet organizational requirements

- Use OHS principles and responsibilities for ergonomics, such as work periods and breaks

- Use help manuals and online help when appropriate

- Use digital media technologies to support design brief requirements.

    - Perform computer operations

Overview: This unit covers the knowledge, skills and attitudes and values needed to perform computer operations which include inputting, accessing, producing and transferring data using the appropriate hardware and software.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Plan and prepare for task to be undertaken | 1.1 Requirements of task are determined as per standard operating procedures 1.2 Appropriate hardware and software is selected according to task assigned and required outcome 1.3 Task is planned to ensure |
| 2. Input data into computer | 2.1 Data are entered into the computer using appropriate program/application in accordance with company procedures 2.2 Accuracy of information is checked and information is saved in accordance with standard operating procedures 2.3 Inputted data are stored in storage media according to requirements 2.4 Work is performed within ergonomic guidelines |
| 3. Access information using computer | 3.1 Correct program/application is selected based on job requirements 3.2 Program/application containing the information required is accessed according to company procedures 3.3 Desktop icons are correctly selected, opened and closed for navigation purposes 3.4 Keyboard techniques are carried out in line with OH & S requirements for safe use of keyboards |
| 4. Produce/output data using computer system | 4.1 Entered data are processed using appropriate software commands 4.2 Data are printed out as required using computer hardware/peripheral devices in accordance with standard operating procedures 4.3 Files and data are transferred between compatible systems using computer software, hardware/ peripheral devices in accordance with standard operating procedures |
| 5. Maintain computer equipment and systems | 5.1 Systems for cleaning, minor maintenance and replacement of consumables are implemented 5.2 Procedures for ensuring security of data, including regular back-ups and virus checks are implemented in accordance with standard operating procedures 5.3 Basic file maintenance procedures are implemented in line with the standard operating procedures |

Knowledge & Understanding

K1: Basic ergonomics of keyboard and computer use

K2: Main types of computers and basic features of different operating systems

K3: Main parts of a computer

K4: Storage devices and basic categories of memory

K5: Relevant types of software

K6: General security

K7: Viruses

K8: OH & S principles and responsibilities

K9: Calculating computer capacity

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Selected and used hardware components correctly and according to the task requirement

- Identified and explain the functions of both hardware and software used, their general features and capabilities

- Produced accurate and complete data in accordance with the requirements

- Used appropriate devices and procedures to transfer files/data accurately

- Maintained computer system

    - Use computer applications

Overview: This unit describes the performance outcomes, skills and knowledge required to identify, select and operate three commercial software packages, including a word-processing, a spreadsheet and presentation application package.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Use appropriate OHS office work practices | 1.1 Use safe work practices to ensure ergonomic, work organization, energy and resource conservation requirements are addressed 1.2 Use wrist rests and document holders where appropriate 1.3 Use monitor anti-glare and radiation reduction screens where appropriate |
| 2. Install and remove software | 2.1 Select software to be installed 2.2 Follow installation instructions 2.3 Delete unrequired software |
| 3. Use appropriate word-processing software | 3.1 Select word-processing software appropriate to perform activity 3.2 Identify document purpose, audience and presentation requirements, and clarify with personnel as required 3.3 Identify organizational requirements for text-based business documents and design document structure and layout to ensure consistency of style and image 3.4 Match document requirements with software functions to provide efficient production of documents 3.5 Use technical functions, other data and formatting to finalize documents 3.6 Ensure the naming and storing of documents in appropriate directories or folders and the printing of documents to the required specifications |
| 4. Use appropriate spreadsheet software | 4.1 Select spreadsheet software appropriate to perform activity 4.2 Identify document purpose, audience and presentation requirements, and clarify with personnel as required 4.3 Enter simple formulas and functions using cell referencing where required 4.4 Customize spreadsheet settings and format documents to meet requirements 4.5 Ensure the naming and storing of documents in appropriate directories or folders and the printing of documents to the required specifications |
| 5. Use appropriate presentation software | 5.1 Select software application package appropriate to perform activity 5.2 Identify purpose, audience and presentation requirements, and clarify with personnel as required 5.3 Use technical functions, other data and formatting to finalize documents 5.4 Ensure documents are named and stored in appropriate directories or folders and printed to required specifications 5.5 Make a presentation |

Knowledge & Understanding

K1: Application software packages used by the organization

K2: Basic technical terminology related to reading help files and responding to system help prompts

K3: Basic knowledge of system usage

K4: Current business practices related to using software to prepare reports

K5: Features and functions of commercial computing packages

K6: Import and export software functions

K7: Linking documents

K8: OHS principles and responsibilities for ergonomics, such as work periods and breaks

K9: Purpose, use and functions of applications

K10: Use of input and output devices

K11: Functions and uses of word processing, spreadsheet and presentation software

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to identify, select and operate three commercial software packages, including a word-processing and a spreadsheet application package. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the ability to:

- Produce workplace documents using a minimum of three different software application packages

- Open, amend and save files and documents according to organizational requirements

- Use OHS principles and responsibilities for ergonomics, such as work periods and breaks

- Use help manuals and online help.

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Use an operating system in a variety of scenarios and across functions, including:

  - scheduling, loading, initiating, and supervising the execution of programs

  - allocating storage

  - initiating and controlling input and output operations

  - handling errors

- Identify and install suitable hardware components

- Install and upgrade application software.

    - Create user documentation

Overview: This unit describes the performance outcomes, skills and knowledge required to create user documentation that is clear to the target audience and easy to navigate.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Determine documentation standards and requirements | 1.1 Determine documentation requirements 1.2 Investigate documentation and industry standards for requirements and determine appropriate application to user documentation 1.3 Design documentation templates using appropriate software and obtain approval from appropriate person |
| 2. Produce user documentation | 2.1 Conduct a review of the subject system, program, network or application in order to understand its functionality 2.2 Gather existing technical, design or user specifications and supporting documentation 2.3 Create user documentation based on template to record the operation of the subject system, program, network or application |
| 3. Review and obtain sign-off | 3.1 Submit user documentation to target audience for review 3.2 Gather and analyze feedback 3.3 Make changes to user documentation 3.4 Submit user documentation to appropriate person for approval |

Knowledge & Understanding

K1: Content features, including clarity and readability

K2: Document design, web design and usability

K3: Functions and features of templates and style guides

K4: Instructional design principles

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create user documentation that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to create user documentation that:

- Meets business requirements

- Caters for a diverse readership

- Is clear to the target audience

- Is easy to navigate.

    - Create technical documentation

Overview: This unit describes the performance outcomes, skills and knowledge required to create technical documentation that is clear to the target audience and easy to navigate.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Identify and analyze documentation requirements and client needs | 1.1 Consult with client to identify documentation requirements 1.2 Interpret and evaluate documentation requirements and confirm details with client 1.3 Investigate industry and documentation standards for requirements 1.4 Define and document the scope of work to be produced 1.5 Consult with client to validate and confirm the scope of work |
| 2. Design documentation | 2.1 Identify information requirements with reference to layout and document structure 2.2 Create document templates and style guides consistent with information requirements 2.3 Conduct a review of the system in order to understand its functionality 2.4 Extract content that meets information requirements according to copyright restrictions 2.5 Develop the structure of the technical documentation giving focus to the flow of information, style, tone and content format 2.6 Validate the technical documentation structure with the client |
| 3. Develop documentation | 3.1 Write technical documentation based on the template and scope of work using the information gathered 3.2 Translate technical terminology into plain English where appropriate 3.3 Apply content format and style according to documentation standards and templates |
| 4. Evaluate and edit documentation | 4.1 Submit technical documentation to appropriate person for review 4.2 Gather and analyze feedback 4.3 Incorporate alterations into the technical documentation 4.4 Edit the technical documentation for technical and grammatical accuracy |
| 5. Prepare documentation for publication | 5.1 Check that the completed technical documentation meets client requirements and scope of work 5.2 Submit the technical documentation to appropriate person for approval 5.3 Prepare the technical documentation for publication and distribution using appropriate channels |

Knowledge & Understanding

K1: Content features, such as clarity and readability

K2: Document design, web design and usability

K3: Functions and features of templates and style guides

K4: Instructional design principles

K5: Organizational policies, procedures and standards that cover document design.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create technical documentation that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Establish customer needs

- Design and develop technical documentation, such as system, procedures, training material and user guides, incorporating appropriate standards

- Update document with client feedback

- Prepare documentation for publication.

    - Create basic databases

Overview: This unit describes the skills and knowledge required to design, develop and test a database in order to meet a specification. It applies to individuals who may be either database, or web designers, required to create a simple database to store information for an online application, using a simple entity relational database.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Analyze the requirements for the database | 1.1 Determine the information that the database is required to hold 1.2 Develop a written requirement report for the functionality of the database 1.3 Complete the documentation, and submit it to the appropriate person for approval |
| 2. Use data modeling to design the database to suit requirements | 2.1 Design an entity-relationship (ER) diagram to model the relationships between the entities and the attributes that the database will hold 2.2 Develop primary and foreign keys to link the entities 2.3 Develop a data dictionary 2.4 Complete the documentation, and submit it to the appropriate person for approval |
| 3. Create a database on a web or database server | 3.1 Use the appropriate language on a web or database server to create one or more databases 3.2 Use the appropriate language on a web or database server to create tables 3.3 Populate the database fields |
| 4. Test the database and debug | 4.1 Test the database on the web or database server 4.2 Ensure that the information represented matches the requirements |

Knowledge & Understanding

K1: outline the principles of open platforms, including browsers and databases

K2: list the processes associated with the creation of entities, attributes, and I populating fields, using both software solutions and script- based input

K3: describe data-modeling techniques to design a database

K4: outline the steps in database design, modeling and implementation

K5: describe the internet operation related to web servers and clients

K6: identify the naming conventions appropriate to database design

K7: identify security restrictions on servers, incorporating some theoretical concepts

K8: describe best practice communication, and accessibility, for audiences with special needs.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create technical documentation that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- research client requirements for a database solution

- design a database that meets client requirements

- create a database on a web hosting service or server to meet client requirements by a due date

- Test and debug the database.

    - Use social media tools for collaboration and engagement

Overview: This unit describes the performance outcomes, skills and knowledge required to establish a social networking presence using social media tools and applications. The unit specifically identifies the requirement to review, compare and use different types of social networking tools and applications.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Describe different types of social media tools and applications | 1.1 Explain characteristics of the term social media 1.2 Identify different types of social-media tools and applications 1.3 Illustrate some of the issues associated with the use of social media tools and applications |
| 2. Compare different types of social media tools and applications | 2.1 Select one social media type for review 2.2 Review most popular tools and applications within that social media type 2.3 Itemize benefits across a range of the most popular tools and applications 2.4 Select most appropriate social media tool or application |
| 3. Set up and use popular social media tools and applications | 3.1 Identify social media tools and applications for possible implementation 3.2 Initiate preferred social media tools and applications for use 3.3 Establish social media interface using text and file content 3.4 Initiate social networking interaction 3.5 Test and evaluate tools and applications for ease of use 3.6 Present findings |

Knowledge & Understanding

K1: Basic technical terminology in relation to social networking and social media applications and tools

K2: Basic knowledge of uploading images, text files, pdf files, audio files, video files and link associated files

K3: Features and functions of social media applications

K4: Import and export software functions

K5: Linking documents

K6: OHS principles and responsibilities for ergonomics, including work periods and breaks

K7: Tagging to facilitate collaborative folksonomy

K8: Social media applications and procedures for connecting to social networking sites

K9: Use of input and output devices

K10: Use of RSS feeds to connect a social network.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create technical documentation that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Establish customer needs

- Design and develop technical documentation, such as system, procedures, training material and user guides, incorporating appropriate standards

- Update document with client feedback

- Prepare documentation for publication.

    - E-Commerce- SEO (Search Engine Optimization)

Overview: The aim of this module is to develop efficient E-Marketing strategies in accordance with the Vision and Mission statement of the organization driven by Electronic means.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Identify SOE Techniques | - Apply SEO techniques - Employ SEO key words |
| - Indicate SEO (Search Engine Optimization) | - Demonstrate SEO techniques to priorities their site or web application using automated tools - Develop efficient E-marketing strategies |

Knowledge & Understanding

K1: Explain different SEO Methods including but not limited to Getting Indexed, Preventing Crawling, and Increasing Prominence.

K2: Elaborate White-hat, Black-hat SEO techniques for web application

K3: Knowledge of SEO key words for web pages translation.

K4: Application of SEO tools usage

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to write and edit copy that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

    - E-Commerce- SCM (Supply Chain Management)

Overview: The aim of this module is to develop efficient E-Marketing strategies in accordance with the Vision and Mission statement of the organization driven by Electronic means.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Identify SCM (Supply Chain Management) | - Identity potential Suppliers - Select the appropriate supplier - Place order as per requirement/inventory - Inspect received order |
| - Prepare updated inventory detail | - Maintain Inventory as per Inventory Control / store keeping techniques - Identity different available transportation mode - Identify steps of reverse SCM i-e from consumer to organization |

Knowledge & Understanding

K1: Elaborate knowledge of procurement Cycle (Launch of RFP/RFQ, Tender, Bidding, Comparative Statement, Award of Contract, Maintenance)

K2: Explain different techniques to manage

K3: Explain product delivery and their traceability

K4: Knowledge of Incorporation of Outsourcing in logistics.

K5: Information about electronic Data Interchange methodologies and format

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to write and edit copy that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

    - E-Commerce- Social Media Marketing

Overview: The aim of this module is to develop efficient E-Marketing strategies in accordance with the Vision and Mission statement of the organization driven by Electronic means.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Social Media Marketing | - Identify different Social media marketing techniques - Apply suitable Classified Advertisement techniques on social media |
| - Develop Marketing Techniques | - Perform Electronic Mail Marketing - Creation of Blogs |

Knowledge & Understanding

K1:  Describe Knowledge of different social media sites that is Facebook, Twitter, LinkedIn, Google+ etc., Comparative Statement, Award of Contract, Maintenance)

K2 Explain Brand pages creation on social media sites.

K3: Evaluate familiarity of banner ads integration on different web sites like newspaper site in any demographic region.

K4: Mention skills to regularly update brand/product/service blogs.K5: Information about electronic Data Interchange methodologies and format

K5: Elaborate direct marketing techniques e.g. Email, SMS (Mobile- Commerce) for the projection of company newsletters

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to implement e-marketing strategies that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

    - Use digital devices

Overview: This unit describes the skills and knowledge required to use a range of digital devices, such as a digital camera, video camera, or personal digital assistant (PDA) device. It applies to individuals who require entry level information and communications technology (ICT) knowledge and literacy skills to support their work in a home office or small office environment.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Prepare to use the digital device | - Review the instruction manual and ensure identified components are available - Identify the physical components of the digital device - . Turn on and follow access procedures to activate the digital device - Alter the digital device settings to best suit intended use - Configure power management settings where appropriate to minimize power consumption, as an environmentally sustainable measure |
| - Set up and use the digital device | - Identify and set the basic operating, security and menu settings - Navigate and manipulate the screen environment - Customize screen icons and access to applications where applicable - Use the digital device, and save and edit output where applicable - Identify more advanced features available and use as required |
| - Access and use basic connectivity devices | - Connect to external digital devices, such as computer devices or storage devices, to retrieve, copy, move and save information - Check physical connectivity of computer devices or storage devices to ensure operation and performance - Connect to a printer either through a computer device or directly, and use printer settings and print data - Access audio-visual devices to view and play a multimedia file |
| - Shut down digital device | - Save current work and back up important data - Close open programs on the digital device and any computer device or storage device - Shut down digital devices, according to manufacturer instructions |

Knowledge & Understanding

K1: outline the capabilities and connectivity requirements of relevant:

- audio-visual devices

- peripheral devices

- storage devices

K2: list basic security functions

K3: explain basic software operation and associated applications

K4: explain digital device functions

K5: explain digital device settings.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to use digital devices that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- turn on a digital device

- use the menu features and navigate a graphical user interface

- use device features to perform tasks

- Save the results of work.

    - Operate word-processing applications

Overview: This unit describes the skills and knowledge required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Apply workplace health and safety (WHS) practices | 1.1 Use workplace ergonomic work practices and strategies 1.2 Organize work area to ensure an ergonomic work environment |
| 2. Create documents | 2.1 Open word-processing application, create document and add data according to information requirements 2.2 Use document templates as required 2.3 Use simple formatting tools when creating the document 2.4 Save document to directory |
| 3. Customize basic settings to meet page layout conventions | 3.1 Adjust page layout to meet information requirements 3.2 Open and view different toolbars 3.3 Change font format to suit document purpose 3.4 Change alignment and line spacing according to document information requirements 3.5 Modify margins to suit the document purpose 3.6 Open and switch between several documents |
| 4. Format documents | 4.1 Use formatting features and styles as required 4.2 Highlight and copy text from another area in the document or from another active document 4.3 Insert headers and footers to incorporate necessary data 4.4 Save document in another file format 4.5 Save and close document to a storage device |
| 5. Create tables | 5.1 Insert standard table into document 5.2 Change cells to meet information requirements 5.3 Insert and delete columns and rows as necessary 5.4 Use formatting tools according to style requirements |
| 6. Add images | 6.1 Insert appropriate images into document and customize as necessary 6.2 Position and resize images to meet document formatting needs |
| 7. Print documents | 7.1 Preview document in print preview mode 7.2 Select basic print settings 7.3 Print document or part of document from printer |

Knowledge & Understanding

K1: describe formatting styles and their effect on formatting, readability and appearance of documents

K2: identify organizational requirements for ergonomics, including work periods and breaks

K3: select organizational style guide to use

K4: outline purpose, use and function of word-processing software.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create word documents that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- follow organizational ergonomic work health and safety (WHS) requirements and practices

- create, open and retrieve documents using customized basic settings

- format documents by creating tables and adding text, objects and images

- Save and prints documents.

    - Operate spreadsheet applications

Overview: This unit describes the skills and knowledge required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Create presentations | 1.1 Open presentation package and create a simple design for a presentation according to organizational requirements 1.2 Open blank presentation and add text and graphics 1.3 Apply existing styles within a presentation 1.4 Use presentation template and slides to create a presentation 1.5 Use various tools to improve the look of the presentation 1.6 Save presentation to the appropriate storage device and folder |
| 2. Customize basic settings | 2.1 Adjust display to meet user requirements 2.2 Open and view different toolbars to view options 2.3 Ensure font settings are appropriate for the presentation purpose 2.4 View multiple slides at once |
| 3. Format presentations | 3.1 Use and incorporate organizational charts and bulleted lists, and modify as required 3.2 Add objects and manipulate to meet presentation purposes 3.3 Import objects and modify for presentation purposes 3.4 Modify slide layout, including text and colors, to meet presentation requirements 3.5 Use formatting tools as required within the presentation 3.6 Duplicate slides within and across a presentation 3.7 Reorder sequence of slides and delete slides for presentation purposes 3.8 Save presentation in another format 3.9 Save to storage device and close presentation |
| 4. Add slide show effects | 4.1 Incorporate pre-set animation and multimedia effects into presentation as required to enhance the presentation 4.2 Add slide transition effects to presentation to ensure smooth progression through the presentation 4.3 Test presentation for overall effect 4.4 Use onscreen navigation tools to start and stop slide show or move between different slides as required |
| 5. Print presentation and notes | 5.1 Select appropriate print format for presentation 5.2 Select preferred slide orientation 5.3 Add notes and slide numbers 5.4 Preview slides and run spell check before presentation 5.5 Print selected slides and submit presentation to appropriate person for feedback |

Knowledge & Understanding

K1: list basic technical terminology to read help files and prompts

K2: outline the different types of:

- formal and informal presentations

- audience

K3: explain the effect of design and formatting on the readability and usability of presentations

K4: outline presentation pitfalls

K5: identify suitable presentation effects for different audiences.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to presentations that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- create, format and prepare presentations for distribution and display

- customize basic settings

- Add slide show effects.

    - Operate presentation packages

Overview: This unit describes the skills and knowledge required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Create spreadsheets | 1.1 Open the spreadsheet application, create spreadsheet files and enter numbers, text and symbols into cells according to information requirements 1.2 Enter simple formulas and functions using cell referencing when required 1.3 Correct formulas when error messages occur 1.4 Use a range of common tools during spreadsheet development 1.5 Edit columns and rows within the spreadsheet 1.6 Use the auto-fill function to increment data where required 1.7 Save the spreadsheet to a folder on a storage device |
| 2. Customize basic settings | 2.1 Adjust page layout to meet user requirements or special needs 2.2 Open and view different toolbars 2.3 Change font settings so they are appropriate for the document purpose 2.4 Change alignment options and line spacing according to spreadsheet formatting features 2.5 Format cell to display different styles as required 2.6 Modify margin sizes to suit the purpose of the spreadsheets 2.7 View multiple spreadsheets concurrently |
| 3. Format spreadsheet | 3.1 Use formatting features as required 3.2 Copy selected formatting features from another cell in the spreadsheet or from another active spreadsheet 3.3 Use formatting tools as required within the spreadsheet 3.4 Align information in a selected cell as required 3.5 Insert headers and footers using formatting features 3.6 Save spreadsheet as another file type 3.7 Save to storage device and close spreadsheet |
| 4. Incorporate object and chart in spreadsheet | 4.1 Import an object into an active spreadsheet 4.2 Manipulate imported object by using formatting features 4.3 Create a chart using selected data in the spreadsheet 4.4 Display selected data in a different chart 4.5 Modify chart using formatting features |
| 5. Print spreadsheet | 5.1 Preview spreadsheet in print preview mode 5.2 Select basic printer options 5.3 Print spreadsheet or selected part of spreadsheet 5.4 Submit the spreadsheet to appropriate person for approval or feedback |

Knowledge & Understanding

K1: list basic technical terminology related to reading help files and prompts

K2: explain the effect of formatting and appearance on the readability and usability of spreadsheets

K3: outline log-in procedures relating to accessing a personal computer (PC)

K4: describe the purpose, use and function of spreadsheet applications

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create technical documentation that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- create spreadsheets

- customize basic settings

- format spreadsheets

- create basic formulas

- insert objects and charts in spreadsheets

- Save and print spreadsheets.

    - Perform writing and editing tasks

Overview: This unit describes the skills and knowledge required to apply the conventions of plain English to writing and editing tasks of different forms. It also includes editing and proofreading techniques. It applies to individuals in various writing contexts who write and edit texts using appropriate language, style, grammar, spelling, and standard conventions for editing and proofreading.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Apply clear and appropriate language and style to writing and editing tasks | 1.1 Use safe work practices including addressing ergonomic requirements when undertaking writing tasks 1.2 Use clear, concise and plain English in writing and editing tasks 1.3 Apply appropriate paragraph structure to written material to ensure clarity of meaning and ease of reading 1.4 Make clear and logical connections between sentences, paragraphs and sections 1.5 Determine and incorporate the language and style of the audience |
| 2. Apply the appropriate voice, tone and tense | 2.1 Determine appropriate voice, tone and tense of the written materials according to audience requirements 2.2 Maintain consistent voice, tone and tense throughout written material |
| 3. Apply appropriate grammar, spelling and punctuation | 3.1 Apply appropriate grammar conventions to a range of written contexts including use of numbers, quotations, and tables 3.2 Apply appropriate spelling and punctuation conventions in writing and editing tasks. |
| 4. Perform editing and proofreading tasks to meet requirements | 4.1 Edit written material to ensure clear meaning through language and paragraphs, consistent voice, tone and tense 4.2 Copyedit written material by checking grammar, spelling and punctuation using standard editing conventions 4.3 Proofreading using style guides and by monitoring written material for errors |

Knowledge & Understanding

K1: main features of clear, concise and plain English language for written material

K2: grammar, punctuation and spelling conventions that meet the task requirements

K3: editing conventions used in substantive editing and copyediting of written material

K4: basic software used to write and collect feedback

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to perform writing and editing tasks that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- write and edit at least one written material (2500-4000 words) and edit another material written by another author (2500-4000 words), of different forms (e.g. blog, journal, book) that demonstrate use of:

  - clear and concise language

  - clear and logical paragraph structures

  - appropriate voice, tone, tense and language

  - plain English grammar, spelling and punctuation

  - accepted grammar conventions for a range of written contexts including use of numbers, quotation and tables

  - standard editing conventions

- complete editing and proofreading tasks using handwritten and digital methods

- Accurately follow a style guide where required.

- follow relevant health and safety practices for writing tasks

    - Write and Edit Copy

Overview: This unit is about writing and editing copy (i.e. final version of text) for a range of formats in different media

| Unit of Competency | Performance Criteria |
| --- | --- |
| CU1. Undertaking copy-writing for different media | P1. Present the facts and information in a manner that is coherent, comprehensible and appropriate for the target audience P2. Write and edit items P3. Craft an engaging narrative, conceptualize and clarify ideas P4. Display strong command of the language including correct grammar, spelling, sentence construction, diction and pronunciation skills P5. Ensure that finished scripts meet legal and regulatory norms, and do not pose any risks to the individual and/ or organization |
| CU2. Undertaking script-writing for various media and types of stories | P1. Edit a story or script based on the brief and prescribed word/ time limits P2. Write and edit items such as: headlines, captions, intros, outros, cues and other types of links P3. Craft an engaging narrative, conceptualize and clarify ideas and develop stories that meet the broader creative/editorial objectives of the organization, if appropriate to the role P4. Display strong command of the language including correct grammar, spelling, sentence construction, diction and pronunciation skills P5. Ensure that finished scripts meet legal and regulatory norms |

Knowledge & Understanding

K1: State editorial standards followed by the organization/ individual that is commissioning the content (including applicable local, national or international laws)

K2: Elaborate the resource limitations including any constraints on budget, resources and time availability (i.e. deadlines)

K3: Explain how to structure one’s thoughts and ideas and write clearly and coherently

K4: Explain how to use a wide range of vocabulary and writing techniques to minimize repetition and create interesting pieces of work

K5: Explain the difference between facts and opinion/ point of view – and write in a manner that makes this distinction evident to the reader/ viewer

K6: Describe the applicable legal and regulatory framework for different types of stories and scripts (e.g. disclosure, consent, intellectual property/ ownership and possible litigation) and writing/ editing in a manner that minimizes these risks

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to perform writing and editing tasks that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Edit stories and scripts as per the brief, or own editorial vision if appropriate

- Undertake background research, gather references and collect information

- Read and analyze various sources of news including wire services and other publications/ channels/ websites/ media

- Plan own work and work for additional staff (e.g. copy desk, art editors, video editors) according to deadlines, as per role

    - Build Java applets

Overview: This unit describes the skills and knowledge required to compile and run an applet that executes in Java-enabled browsers, and interacts with users. It applies to individuals involved in software, web or games development and who are required to build applets, using Java that interacts with users via a browser.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Create Java source file | 1.1 Create a file with a text editor 1.2 Create a sub-class of the class Java. Applet. 1.3 Ensure that applet sub-class implements at least one of the following methods: init and paint 1.4 Define classes that contain instance variables, methods and local variables 1.5 Ensure that the init method initializes instance variables, and constructs any graphical interface used in the applet sub-class 1.6 Implement the paint and update methods to manage output that is drawn in the applet window 1.7 Incorporate event handling methods 1.8 Implement the code that allows a user to enter values, and assigns these values to variables 1.9 Use available graphical user interface (GUI) components to allow user interaction with the applet 1.10 Specify and load images and sounds 1.11 Include comments to describe the behavior of the applet |
| 2. Compile a source file | 2.1 Use a Java compiler to compile the file 2.2 Correct errors detected by the compiler 2.3 Confirm basic correctness of the file, to ensure that all variables have been initialized 2.4 Ensure the compiler creates a class file, and that the class is interpreted correctly |
| 3. Run applet | 3.1 Create and name a hypertext markup language (HTML) file, and add APPLET tag 3.2 Write the applet class file to match the HTML document that contains an APPLET tag 3.3 Confirm that the applet loads and executes correctly 3.4 Identify and correct run-time errors 3.5 Identify and correct logic errors 3.6 Ensure that the appearance of the applet window renders it accessible and intuitive for the user, and that its design complies with organizational standards 3.7 Ensure that user interaction is implemented efficiently and effectively |

Knowledge & Understanding

K1: explain object-oriented programming concepts

K2: outline the theoretical concepts of Java programming

K3: recognize, and describe, the Australian Computer Society Code of Ethics

K4: identify the sources of browser security restrictions.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to build Java applets that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- review the requirements to provide an applet that executes in Java-enabled browsers, and allows users to customize the applet's operation

- write, compile and run an applet that accepts user input, and generates a response based on the input

    - Manipulate Images (Illustrator)

Overview: This unit describes the skills and knowledge required to compile and manipulate images. It applies to individuals involved in software, image editing and who are required to manipulate images, using Illustrator.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Familiarize with illustrator interface | 1.1 Able to describe differences between raster and vector imaging and uses for both 1.2 Understand the capabilities of Illustrator and alternative programs 1.3 able to comfortably navigate Illustrator |
| 2. create and edit basic vector paths | 2.1 Able to use path creation and editing tools 2.2 Use the terminology associated with vector paths 2.3 Able to save an illustrator file 2.4 Export a drawing to various other file types |
| 3. Apply and edit colors in their drawings | 3.1 Use variety of basic color tools and settings 3.2 Create a cohesive color theme 3.3 create and edit gradients |
| 4. Use specific tools to modify the appearance of paths | 4.1 Apply advanced stroke and width settings to a path 4.2 Understand the basic function and process of using brushes |
| 5. Create, edit, and combine shapes in a variety of ways | 5.1 Use the shape tools effectively 5.2 Rotate, reflect, scale, skew, and free transform a shape 5.3 Utilize various ways to copy and paste a shape 5.4 Use the knife and scissors tools to split paths and shapes 5.5 Create a complex shape by combining simple ones 5.6 Create and edit clipping paths |
| 6. Apply and edit the appearance of images | 6.1 Able to victories and paint a raster image 6.2 Use text effectively 6.3 Able to effectively live trace an image 6.4 Able to add, edit, and warp text 6.5 Apply and edit object effects |

.

Knowledge & Understanding

K1: Explain the usage of pen tool

K2: Describe the benefits of clipping masks

K3: Explain the ways to create the perfect shape using path finder panel

K4: Elaborate the ways to make brushes

K5: Explain the usage of layering

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to use illustrator that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Learning the Interface

- Creating shapes

- Ways to create complex shapes with the Shape Builder tool.

- Stokes and fills

- Practical use of effects and the appearance panel.

- Creating cool 3D effects for texts and logos.

    - Manipulate Images (Photoshop)

Overview: This unit describes the skills and knowledge required to compile and manipulate images. It applies to individuals involved in software, image editing and who are required to manipulate images, using Illustrator.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Performs basic image edits using Photoshop | 1.1 Able to describe differences between raster and vector imaging and uses for both 1.2 Edit Photos non-destructively 1.3 Able to comfortably navigate Photoshop 1.4 Crop picture effectively 1.5 Straighten the picture 1.6 Use the layering process in Photoshop |
| 2. Use masks and combine images | 2.1 Make and edit masks 2.2 Make selection 2.3 Make adjustment layers with masks 2.4 Use masks and layers to combine photos |
| 3. Merge and heal photos | 3.1 Use Photo merge 3.2 Fill selections with content aware fill 3.3 Use healing tools correctly 3.4 Apply Filters to photos |
| 4. Use non-photo elements in Photoshop designs | 4.1 Use text tool 4.2 Use brushes 4.3 Create shapes 4.4 Apply colors and gradients 4.5 Apply Layer Effects 4.6 Combine elements in pleasing ways |

Knowledge & Understanding

K1: State the names of the basic tools

K2: Explain the organization of layer method

K3: Elaborate the effects of masking, brush, cropping, and adjusting layers

K4: Describe the short keys for tools

K5: Explain the process of opening and saving image

K6: Describe the ways to add effects on an image

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to use Photoshop that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- get familiarize with Photoshop interface

- master all the tools in depth

- blend Images seamlessly

- decorate an Image

- create reflection effect

- create awesome text effect

- create rain effect , cloud effect

- create pixel perfect designs

- create blur backgrounds , polygon backgrounds

- Create whirlpool Effect, Bokeh Effect

    - Design simple web page layouts

Overview: This unit is about writing and editing copy (i.e. final version of text) for a range of formats in different media

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Identify main layout sections from design specification | 1.1 Identify the required sections of the web page 1.2 Create the web page structure |
| 2. Layout the web page to match design specification | 2.1 Position document elements 2.2 Style web page elements to match the design specifications |
| 3. Test and validate the layout | 3.1 Test the website in various browsers 3.2 Validate the web pages against industry standards |

Knowledge & Understanding

K1: Summaries standard web and web scripting design principles

K2: define and explain the application of the following for website development:

- cascading style sheets (CSS)

- the hypertext transfer protocol (HTTP)

- a markup language, such as hypertext markup language (HTML), or extensible hypertext markup language (XHTML)

- World Wide Web Consortium (W3C) standards.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to Design simple web page layouts that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- develop a website according to design specifications

- lay out page elements

- test the web pages in a variety of browsers

- Validate the web pages, against industry standards.

    - Apply introductory programming techniques

Overview: This unit describes the performance outcomes, skills and knowledge required to undertake introductory programming tasks in the development of a game or application. This unit addresses the knowledge, processes and techniques necessary to develop skills to create simple applications or games.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Apply language syntax and layout | 1.1 Apply basic language syntax rules 1.2 Use language data types, operators and expressions to create clear and concise code 1.3 Apply variables and variable scope 1.4 Use library functions in a program 1.5 Use commenting to create clear meaning to code |
| 2. Apply control structures | 2.1 Apply language syntax for sequence, selection and iteration constructs 2.2 Use logical operators to create expressions for use in selection and iteration constructs |
| 3. Code using standard algorithms | 3.1 Develop algorithms that use the sequence, selection and iteration constructs 3.2 Create and use arrays 3.3 Code standard sequential access algorithms for reading and writing text files, including end-of-file detection loops 3.4 Apply string manipulation |
| 4. Test code | 4.1 Use debugging techniques to trace code execution and examine variable contents to detect and correct errors 4.2 Create and conduct simple tests to confirm code meets design specification 4.3 Document the tests performed and results achieved |
| 5. Create an application or game | 5.1 Design an algorithm in response to basic program specifications 5.2 Develop the application or game to meet the program specification 5.3 Test and confirm that application or game meets the initial specifications |

Knowledge & Understanding

K1: Programming language syntax and command structure

K3: Small-sized application or game development.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to undertake introductory programming tasks in the development of a game or application. This unit addresses the knowledge, processes and techniques necessary to develop skills to create simple applications or games. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Apply programming language syntax, sequence, selection and iteration control structures to the development of an application or game

- Produce an application or game that is designed and built from a provided program specification

- Confirm that the created application or game meets the original program specifications and obtain user sign-off for completed program.

# List of Tools, Equipment and Consumable

06: Advanced Surveying

| S# | Tools and Equipment | S# | Consumable Material |
| --- | --- | --- | --- |
| 1 | Digital Theodolite | 1. | Lime |
| 2 | Total Station complete set along with accessories and software. | 2. | Drawing Sheets |
| 3 | Dual Frequency GNNS GPS complete set with all accessories and software i/c Battery with Charger. | 3. | Lead Pencils |
| 4 | Hand Held GPS | 4. | Eraser |
| 5 | Tribrach |  |  |
| 6 | Optical plummets |  |  |
| 7 | Target Staff |  |  |
| 8 | Single Prism Targets with stands |  |  |
| 9 | Tripple Prism Targets with stands |  |  |
| 10 | Ball Pen Hammer |  |  |
| 11 | Spring balances |  |  |
| 12 | Ranging rods |  |  |
| 13 | Steel Tape |  |  |
| 14 | Steel Band |  |  |
| 15 | Invar tape |  |  |
| 16 | Metallic Tape |  |  |
| 17 | Arrows |  |  |
| 18 | Pegs |  |  |
| 19 | Mallet |  |  |
| 20 | Umbrella with stand |  |  |
| 21 | Engineer’s chain |  |  |
| 22 | Thermometers |  |  |
| 23 | Drawing Lab Instruments |  |  |
| 24 | Computer Lab equipment |  |  |

07: AutoCAD

| S# | Tools and Equipment | S# | Consumable Material |
| --- | --- | --- | --- |
| 1 | Computer latest model with Server |  | Printer papers |
| 2 | Computer latest model work stations |  | Printer Toner |
| 3 | Laptop computer |  | USB |
| 4 | Net working |  | Printing Paper Roles |
| 5 | UPS 2000 Watt with batteries |  | Plotter Toner |
| 6 | Multimedia Projector |  |  |
| 7 | Scanner for A-3 paper |  |  |
| 8 | Printer LaserJet |  |  |
| 9 | USB Sticks |  |  |
| 10 | Computer Chair (will be added in furniture portion) without arm rest – 5 legs base |  |  |
| 11 | Computer Table (will be added in furniture portion) 2½ x 2 x 2½ |  |  |
| 12 | License software for education Latest Versions: Auto CAD M.S Office Antivirus Microsoft windows |  |  |
| 13 | Plotter |  |  |

|  | 08: Construction I |  |  |
| --- | --- | --- | --- |
| S# | Tools and Equipment | S# | Consumable Material |
| 1. | Brick Making Machine | 1. | Sand |
| 2. | Brick Molds | 2. | Sand Dust |
| 3. | Hand roller | 3. | cement |
| 4. | Thermometer | 4. | self-tapping screws |
| 5. | Umbrella with stand | 5. | soft sheet nails |
| 6. | PPEs | 6. | Wall board adhesive |
| 7. | Calculator | 7. | Distemper |
| 8. | Stone masonry instruments | 8. | Knotting & Lashing Rope |
| 9. | Scissor | 9. | Lime |
| 10. | Mason Trowel | 10. | Paint |
| 11. | Pointing Trowel | 11. | Bitumen |
| 12. | Troweling Blades | 12. | Polythene Sheet |
| 13. | Hammer | 13. | Polythene Role for DPC Bricks |
| 14. | Hammer Forged steel rectangle head | 14. |  |
| 15. | Lump hammers | 15. | Chalk |
| 16. | Hammers (bricks, club and scutch) | 16. | Luminous Slip |
| 17. | Sledge hammers | 17. | Stone Crush |
| 18. | Air compressors and hoses | 18. | Lime beds |
| 19. | Bevels | 19. | Pencil, |
| 20. | hand/power saws | 20. | Eraser, |
| 21. | Core cutting saw | 21. | Sharpener, |
| 22. | Jig saws | 22. | Drawing sheet, |
| 23. | Masonry saws | 23. | Acoustic underlay material, adhesives |
| 24. | Wet and dry diamond saws | 24. | Caulking compound, cement mortar (with and without additives) |
| 25. | Marking equipment | 25. | Clouts, cornice adhesive, crack suppression membrane |
| 26. | Measuring tapes and rules, | 26. | Fixings and fasteners |
| 27. | Metallic Tape 100 feet (30m) | 27. | Patching materials, plasterboard nails, pre-mixed and mixed fillers |
| 28. | Steel Tape- Slide Lock, Automatic Returns, Metallic / plastic case, 5-meter, Graduation on both side in feet & meter | 28. | Lime Color (Different) |
| 29. | Measuring scale, | 29. | Tile Bond |
| 30. | Nail guns | 30. | Tile Spacer (Different Sizes) |
| 31. | Protractors | 31. | Brick Ballast |
| 32. | squares (combination/tri) | 32. | Road Metals |
| 33. | steel squares and fences | 33. | Thermophore for Joints 2” & 1” thick |
| 34. | string lines | 34. | Water Stopper for Concrete (Plastic) |
| 35. | Line blocks and line pins | 35. | Barrel bolts, Cabin hooks closers, Dead bolts, Flash bolts, flashings, handles hinges (butt and parliament), Latches, locks, Metal, Night latches, Passage sets, Timber. |
| 36. | builders’ lines | 36. | Windows may include: All size windows, Casement, Double hung, Glazed sliding doors, curved and bay, Hopper, Straight and sliding |
| 37. | static lines |  |  |
| 38. | Compass |  |  |
| 39. | Corner Float and float Blades |  |  |
| 40. | Steel float Set (Small, medium, large) Wooden Handle |  |  |
| 41. | Wooden Float (Small, medium, large) Wooden Handle |  |  |
| 42. | Groove Cutter |  |  |
| 43. | Cement sheet cutters |  |  |
| 44. | Tile cutters |  |  |
| 45. | Edger |  |  |
| 46. | Brooms |  |  |
| 47. | Polishers |  |  |
| 48. | Hand Grinders |  |  |
| 49. | Power grinders |  |  |
| 50. | Vibrator |  |  |
| 51. | Vibrating Rod |  |  |
| 52. | Form work |  |  |
| 53. | Molds |  |  |
| 54. | Workability test apparatus |  |  |
| 55. | Core cutting machine |  |  |
| 56. | Hand Drill Machines with drills |  |  |
| 57. | Power drill machines with drills |  |  |
| 58. | Sheet plate |  |  |
| 59. | Plate bar |  |  |
| 60. | Plate rod |  |  |
| 61. | Adjustable base plates |  |  |
| 62. | Channels |  |  |
| 63. | Forgings area |  |  |
| 64. | Casting platform |  |  |
| 65. | Furnace |  |  |
| 66. | Wheel Barrow: |  |  |
| 67. | Bucket |  |  |
| 68. | Paint Scraper Set |  |  |
| 69. | Potien Filler Knife Set |  |  |
| 70. | Chisel Set (9"-12"-18") |  |  |
| 71. | Plumb Bob |  |  |
| 72. | plumb rules |  |  |
| 73. | Straight Edge |  |  |
| 74. | Shovels with handle |  |  |
| 75. | Kassi (Plies) with handle |  |  |
| 76. | Spirit Level (Aluminum 2 ft. length) |  |  |
| 77. | torpedo levels |  |  |
| 78. | Dumpy Level |  |  |
| 79. | Pick Axe |  |  |
| 80. | Tray (Galvanized Iron, 14 SWG) SET |  |  |
| 81. | Pointing Tray |  |  |
| 82. | Striker for vertical joint (pointing) |  |  |
| 83. | Striker for horizontal joint (pointing) |  |  |
| 84. | Bamboo (10 feet) |  |  |
| 85. | Bamboo (15 feet) |  |  |
| 86. | Battens (5'x3½"x2½") |  |  |
| 87. | Shesham Wood Planks (7'x7"x1½") |  |  |
| 88. | Mortar Pan -GI Sheet (20 SWG) Size, 45 cm dia |  |  |
| 89. | mortar boards |  |  |
| 90. | Adjustable Wrench Set- * Hardened Steel, tempered and chrome plated, * Set include 8" (20cm), 12" (30cm), 18" (45cm) |  |  |
| 91. | Mixing Spoon Set- heavy gauge stainless steel, 150mm, 200mm, and 250mm long, round bottom |  |  |
| 92. | Concrete mixers |  |  |
| 93. | Elevators |  |  |
| 94. | Forklifts |  |  |
| 95. | jointing tools |  |  |
| 96. | Margin or raking tools |  |  |
| 97. | materials hoists |  |  |
| 98. | Cantilevered hoists (materials only with maximum capacity of 500kg) |  |  |
| 99. | Wall Scaffolds |  |  |
| 100. | scaffold belts |  |  |
| 101. | scaffolding planks |  |  |
| 102. | bracket scaffolds (tank and formwork) |  |  |
| 103. | prefabricated scaffolds |  |  |
| 104. | braces |  |  |
| 105. | small petrol or diesel engines and compressors, |  |  |
| 106. | Bolsters |  |  |
| 107. | brick buggies |  |  |
| 108. | brick grabs |  |  |
| 109. | Bends and hitches |  |  |
| 110. | box spanners |  |  |
| 111. | Couplers and accessories |  |  |
| 112. | Fiber ropes |  |  |
| 113. | Gin wheels |  |  |
| 114. | guard rails |  |  |
| 115. | Ledgers |  |  |
| 116. | Mesh guards |  |  |
| 117. | mid rails |  |  |
| 118. | modular and prefabricated components |  |  |
| 119. | Safety nets |  |  |
| 120. | Stairs or ladders |  |  |
| 121. | Standards |  |  |
| 122. | steel and aluminum tubes (pipes) |  |  |
| 123. | transoms |  |  |
| 124. | Wire nips |  |  |
| 125. | Pallet trolleys |  |  |
| 126. | profiles |  |  |
| 127. | Brushes |  |  |
| 128. | Caulking guns |  |  |
| 129. | Electrical leads |  |  |
| 130. | Hose and water sprays |  |  |
| 131. | Nippers |  |  |
| 132. | Pointed grouters |  |  |
| 133. | power screwdrivers |  |  |
| 134. | sanders |  |  |
| 135. | Rags |  |  |
| 136. | rubber mallets |  |  |
| 137. | Sanding blocks |  |  |
| 138. | spacers and wedges |  |  |
| 139. | Spatulas |  |  |
| 140. | sponges, squeegees |  |  |
| 141. | wire brushes, |  |  |
| 142. | Work platforms |  |  |
| 143. | scribes |  |  |

|  | 09: Plumbing |  |  |
| --- | --- | --- | --- |
| S# | Tools and Equipment | S# | Consumable Material |
| 1. | Adjustable Wrench Set | 1. | Complete Set Of Bath Room Fitting |
| 2. | Pipe Wrench Chain Type | 2. | Complete Set of Bath Sanitary Fixture |
| 3. | Adjustable Pipe Wrench | 3. | Complete set of GI pipe fittings |
| 4. | Pipe Reamer | 4. | Complete set of CI pipe fittings |
| 5. | Pipe Vice (Stand Type) | 5. | Complete set of PPRC pipe fittings |
| 6. | Pipe Vice (Bench Type) | 6. | Cement |
| 7. | Pipe Thread Cutting Kit | 7. | Sand |
| 8. | Tap Set | 8. | Plastic rings |
| 9. | Pipe Cutter | 9. | Spun yarn |
| 10. | Pliers Adjustable | 10. | Gasket |
| 11. | Cold Chisel Set | 11. | Sealant |
| 12. | Steel Tape | 12. | Nut bolts, Clamps, Jute, Rubber Rings |
| 13. | Mason’s Spirit Level | 13. | Lime |
| 14. | Steel Rule | 14. | Plumbing Fixtures each type |
| 15. | Electric Drill Machine | 15. | GI, CI, PPRC, Specials each type and size |
| 16. | Screw Driver Set | 16. | G.I. Pipe ½ dia |
| 17. | Trowel (200-280 Mm Long Steel) | 17. | ½-3/4 reducing socket, elbow, bush |
| 18. | Mortar Pan | 18. | Gate valve ½ |
| 19. | Shovel With Handle | 19. | Gate valve ¾ |
| 20. | Hack Saw | 20. | Check valve ¾ |
| 21. | Bench Vice | 21. | Ball valve ¾ |
| 22. | Ball Pein Hammer Set | 22. | Water meter |
| 23. | Claw Hammer | 23. | Ferule valve |
| 24. | Files Set | 24. | Hand wash basin |
| 25. | Scissors | 25. | Basin brackets |
| 26. | Centre Punch | 26. | Pillar cock (swan type) |
| 27. | Melting Pot | 27. | Rubber connection 18 |
| 28. | Ladle | 28. | Copper connection pipe 18 |
| 29. | Pipe Yarning and Caulking Tools | 29. | Stop cock |
| 30. | Oil Cane | 30. | Bib cock |
| 31. | Drill/Bit Set | 31. | Flushing cistern (low level) |
| 32. | Complete Set Of Bath Room Fitting | 32. | Flushing cistern (high level) (cast iron) 02 no. |
| 33. | Complete Set Of Bath Sanitary Fixture | 33. | PVC hockey pipe |
| 34. | Personal Protective equipment, dresses | 34. | PVC waste pipe |
| 35. | Plungers, | 35. | Waster coupling |
| 36. | Turbine with impellers & Motor Electricity | 36. | Water closet (Indian type) |
| 37. | Turbine pipe with fittings | 37. | Water closet (English type) |
| 38. | Pipe wooden / steel clamps | 38. | Urinal |
| 39. | Wheel barrow | 39. | P trap |
| 40. | Umbrella | 40. | Cast iron pipe 6 ft size |
|  |  | 41. | C.I cowl |
|  |  | 42. | C.I Tee 4 |
|  |  | 43. | Lead |
|  |  | 44. | Lead wool |
|  |  | 45. | Threading taps |
|  |  | 46. | PVC pipe and polyethylene pipe ½ size |
|  |  | 47. | Connection clamp with ¾ socket 4 size 06 no |
|  |  | 48. | Steel foot rule |
|  |  | 49. | Hand measuring tape (3 meter/ 15 ft) 25 no |
|  |  | 50. | Ball Peen hammer ½ kg |
|  |  | 51. | Center punch |
|  |  | 52. | Hacksaw Blade |
|  |  | 53. | Try Square |
|  |  | 54. | Oil cane |
|  |  | 55. | Mobil Oil |
|  | 10: Digital Skills |  |  |
| S# | Tools and Equipment | S# | Consumable Material |
| 1 | Computer latest model with Server |  | Printer papers |
| 2 | Computer latest model work stations |  | Printer Toner |
| 3 | Laptop computer |  | USB |
| 4 | Net working |  |  |
| 5 | UPS 2000 Watt with batteries |  |  |
| 6 | Multimedia Projector |  |  |
| 7 | Scanner for A-3 paper |  |  |
| 8 | Printer LaserJet |  |  |
| 9 | USB Sticks |  |  |
| 10 | Computer Chair (will be added in furniture portion) without arm rest – 5 legs base |  |  |
| 11 | Computer Table (will be added in furniture portion) 2½ x 2 x 2½ |  |  |
| 12 | License software for education Latest Versions: M.S Office Antivirus Microsoft windows |  |  |