National Vocational Certificate Level 3 Assistant Surveyor in Civil Technology

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Contents

1.	Introduction	14

2.	Purpose of the Qualification:	14

3.	Overall objectives of training program	15

4.	Entry level of trainees	15

5.	Minimum qualification for teachers	16

6.	Duration of the course:	16

7.	Description and structure of the course	18

8.	Overview of the Curriculum.	36

9.	Detail of Modules Level wise	42

a.	National Vocational  CertificateLevel 1 & 2 Site Assistant in Civil Technology	42

9.1. Basic Engineering Surveying	42

0732C&BE-1: Perform Chain Survey	42

0732B&CE-2: Perform Compass Traversing	44

0732B&CE-3: Carryout Plane Table Survey	47

0732B&CE-4: Perform Levelling	51

0732B&CE-5: Perform Reciprocal Levelling	55

0732B&CE-6: Perform Long and Cross Sectioning	57

0732B&CE-7: Perform Tachometry	61

0732B&CE-8: Perform Contouring	63

0732B&CE-9: Perform hydrographic Survey	66

0732B&CE-10: Compute Volume by Contouring	69

9.2. Civil Engineering Drawing	71

0732B&CE-011. Produce Templates, Title Block / Strip & Draw Lines	71

0732B&CE-012. Perform Lettering and Printing on Sheet	75

0732B&CE-013.Construct Scale for Drawings	80

0732B&CE-014. Construct Geometrical Figures	83

0732B&CE-015. Produce Orthographic Projection	88

0732B&CE-016. Perform Sectioning	92

0732B&CE-017. Perform Dimensioning on Drawings	95

0732B&CE-018. Produce Pictorial Drawings	97

0732B&CE-019. Develop Symbols of Engineering Drawings	101

0732B&CE-020. Produce Cross Section of Wall	106

0732B&CE-021. Develop Building Drawings	111

0732B&CE-22. Perform House Planning	115

0732B&CE-023. Produce Layout Plans for Building Services.	118

0732B&CE-024. Produce Drawings of Sanitary Structures	122

9.3.  Building Materials	126

0732B&CE-025. Identify bricks	126

0732B&CE-026. Dress stone for masonry	129

0732B&CE-027. Prepare slaked lime	131

0732B&CE-028. Identify and store cement for construction	132

0732B&CE-029. Select sand for construction	134

0732B&CE-030. Identify ferrous and non-ferrous metals	136

0732B&CE-031. Identify advanced construction materials	138

0732B&CE-032. Work Safely in Construction Industry	142

0732B&CE-033. Interpret Simple Building Plans	145

0732B&CE-034. Read and Interpret plans and Specifications	147

0732B&CE-035. Apply power and hand tools in construction	150

9.4.	Workshop Practice - I (wood working)	153

0732B&CE-036. Carry out OH & S requirement in workshop	153

0732B&CE-037. Determine properties and types of locally manufactured timber	157

0732B&CE-038. Sharpen carpentry tools	159

0732B&CE-039. Apply hand tools	161

0732B&CE-040. Perform wooden joinery work	163

0732B&CE-041. Apply fastenings	167

0732B&CE-042. Apply traditional spirits polishing techniques	169

0732B&CE-043. Apply portable power tools	173

0732B&CE-044. Make simple calculations of timber	175

0732B&CE-045. Construct a basic timber product	176

9.5. Workshop Practice - II (Electrical Wiring)	178

0732B&CE-046. Carry Out Oh & S Requirement in Workshop	178

0732B&CE-047. Maintain Safety	181

0732B&CE-048. Perform Electrical Wiring	183

0732B&CE-049. Install electrical system	186

0732B&CE-050. Construct Circuits	191

0732B&CE-051. Measure Properties of Electricity	193

0732B&CE-052. Perform Electrical Wiring of a Single Room	195

b.	National Vocational Certificate Level 3 Assistant Surveyor in Civil Technology	198

9.6. Advanced Surveying	198

0732B&CE-053: Measure Angles by Theodolite	198

0732B&CE-054: Perform Trigonometric Levelling by Theodolite	201

0732B&CE-055: Perform Traversing by Theodolite	203

0732B&CE-056: Set Out Horizontal Curves by Theodolite	205

0732B&CE-057: Set Out Vertical Curves by Theodolite	208

0732B&CE-058: Carry Out Triangulation by Theodolite	210

0732B&CE-059: Perform Temporary Adjustment of Total Station	213

0732B&CE-060: Measure Distance by Total Station	215

0732B&CE-061: Measure Angles by Total Station	217

0732B&CE-062:  Measure Area by Total Station	220

0732B&CE-063: Set Out Curves by Total Station	222

0732B&CE-064: Perform Integration of GPS	224

0732B&CE-065: Measure Co-ordinates by GPS	225

0732B&CE-066: Set Out Civil Engineering Structures by GPS	227

9.7. AutoCAD	229

0732B&CE-67. Perform Basic Computer Operations.	229

0732B&CE-68. Perform Drafting Settings and Formatting	233

0732B&CE-69. Perform Dimensioning and Printing, Lettering and Plotting	236

0732B&CE-70. Develop 2D Drawings of Structures	239

0732B&CE-71. Produce Drawing of Framed Structure Building	244

0732B&CE-72. Develop Sectionals Drawing for Foundations	250

0732B&CE-73. Develop Drawings for Doors & Windows	255

0732B&CE-74. Produce Drawings of Stairs and Stair Case.	259

0732B&CE-75. Produce Drawings of Water Tanks	263

0732B&CE-76. Produce Drawings of Cross Drainage Structures.	268

0732B&CE-77. Develop 3D Drawings	271

0732B&CE-78. Produce Civil Technology Drawings	275

9.8. Construction I	280

0732B&CE-79. Perform Simple Levelling	280

0732B&CE-80. Lay out Building plans	282

0732B&CE-81. Carryout excavation and install supports	287

0732B&CE-082. Place Concrete in Foundations	292

0732B&CE-083. Prepare for construction Process (Brick/ block laying)	295

0732B&CE-084. Mix Cementitious materials (Mortar & Concrete)	298

0732B&CE-085. Install flashing and damp-proof course	302

0732B&CE-086. Carry out brick work	307

0732B&CE-087. Construct Cavity Wall	313

0732B&CE-088. Construct Curved Wall	322

0732B&CE-089. Construct Masonry arches (semi-circular and segmental)	326

0732B&CE-090. Construct Scaffolding	334

0732B&CE-091. Install doors and window frames	339

0732B&CE-092. Prepare Surface and fix tiles	344

0732B&CE-093. Prepare Surface for Painting	355

0732B&CE-094. Paint by spray, brush and roller	359

0732B&CE-095. Apply stains and clear timber finishes	364

0732B&CE-096. Prepare site layout of civil engineering projects	368

0732B&CE-097. Construct masonry steps and stairs	373

0732B&CE-098. Install stairs	376

0732B&CE-099. Prepare for solid dry wall plastering	382

0732B&CE-100. Apply plaster	386

9.9.  Plumbing	391

0732B&CE-101. Identify plumbing tools & equipment	391

0732B&CE-102. Perform cutting, threading & reaming	393

0732B&CE-103. Make water connection	395

0732B&CE-104: Layout for domestic water supply	397

0732B&CE-105. Join pipes & fittings	399

0732B&CE-106. Install plumbing fixtures	404

0732B&CE-107. Install & repair water pumps	409

0732B&CE-108. Install turbine pumps	411

9.10.  Digital Skills	413

0732B&CE109-	Install computer operating systems and hardware	413

0732B&CE110-	Operate digital media technology	417

0732B&CE111-	Perform computer operations	421

0732B&CE112-	Use computer applications	426

0732B&CE113-	Create user documentation	431

0732B&CE114-	Create technical documentation	434

0732B&CE115-	Create basic databases	439

0732B&CE116-	Create basic databases	443

0732B&CE117-	E-Commerce- SEO (Search Engine Optimization)	446

0732B&CE118-	E-Commerce- SCM (Supply Chain Management)	448

0732B&CE119-	E-Commerce- Social Media Marketing	450

0732B&CE120-	Use digital devices	452

0732B&CE121-	Operate word-processing applications	455

0732B&CE122-	Operate spreadsheet applications	460

0732B&CE123-	Operate presentation packages	465

0732B&CE124-	Perform writing and editing tasks	470

0732B&CE125-	Write and Edit Copy	473

0732B&CE126-	Build Java applets	475

0732B&CE127-	Manipulate Images (Illustrator)	478

0732B&CE128-	Manipulate Images (Photoshop)	481

0732B&CE129-	Design simple web page layouts	483

0732B&CE130-	Apply introductory programming techniques	485

c.	National Vocational Certificate Level 4 Site Supervisor in Civil Technology	488

9.11 Construction II	488

0732B&CE-131. Prepare subgrade, base, and bedding course for paving	488

0732B&CE-132. Lay paving	492

0732B&CE-133. Perform concrete, tiles and mosaic flooring	496

0732B&CE-134. Perform marble/granite flooring	500

0732B&CE-135. Apply water proofing remedial processes	503

0732B&CE-136. Apply and install sealants	509

0732B&CE-137. Perform water proofing works	512

0732B&CE-138. Apply water proofing tools and equipment	520

0732B&CE-139. Perform water proofing of basements	529

0732B&CE-140. Apply water proofing to internal wet areas	536

0732B&CE-141. Install Physical termite barriers	544

0732B&CE-142. Erect and dismantle slip formwork	551

0732B&CE-143. Install trench support	557

0732B&CE-144. Construct underpinning	560

0732B&CE-145. Carry out reinforced concrete work	565

0732B&CE-146. Work safely on roofs	570

0732B&CE-147. Install roof components	573

0732B&CE-148. Install roof drainage	578

0732B&CE-149. Lay roof tiles	583

0732B&CE-150. Erect roof trusses	587

9.12. Quantity Surveying	590

0732B&CE-151. Produce rough cost estimates of buildings	590

0732B&CE-152.  Evaluate items of work for straight wall, T, L, H, U shaped and circular walls.	593

0732B&CE-153. Produce detailed estimate of a single storey building.	595

0732B&CE-154. Produce Annual/Special repair estimates-Building and Road.	597

0732B&CE-155. Evaluate Earth work for Civil Engineering structures.	600

0732B&CE-156. Produce detailed estimate of Arterial roads	604

0732B&CE-157. Produce material statements.	608

0732B&CE-158. Analyse Rates of different items of work for building.	612

0732B&CE-159. Produce of rough cost estimate of water supply scheme.	620

0732B&CE-160. Produce rough cost estimate of Sewerage scheme.	623

0732B&CE-161. Evaluate value & standard rent of an existing building	628

0732B&CE-162. Produce Detailed Estimate of Culverts.	631

0732B&CE-163. Produce Detailed Estimate of Bridge.	634

9.13 Water Supply	637

0732B&CE-164. Estimate demand for Water Supply	637

0732B&CE-165. Prepare sketches of aqueducts & water pipes	639

0732B&CE-166. Draw sketches of intakes	642

0732B&CE-167. Plan water treatment (Clarification)	643

0732B&CE-168. Plan water treatment (Filtration)	646

0732B&CE-169. Plan water distribution system	648

9.14. Strength of Materials	651

0732B&CE-170. Locate centroid of different sections	651

0732B&CE-171. Find out Moment of Inertia of Different Sections	658

0732B&CE-172. Produce Shear Force and Bending Moment Diagrams	665

0732B&CE-173. Find Hardness of Metals	672

0732B&CE-174. Conduct Tensile Strength Test	675

9.17 Soft Skills	677

0732B&CE175-	Develop workplace policy and procedures for sustainability	677

0732B&CE176-	Manage meetings	680

0732B&CE177-	Manage recruitment selection and induction processes.	683

0732B&CE178-	Manage personal work priorities and professional development	687

0732B&CE179-	Manage workforce planning	690

0732B&CE180-	Undertake project work	694

0732B&CE181-	Identify and communicate trends in career development	698

0732B&CE182-	Apply specialist interpersonal and counseling interview skills.	702

0732B&CE183-	Work safely in an office environment	708

0732B&CE184-	Develop workplace documents	710

0732B&CE185-	Prepare and implement negotiation	714

0732B&CE186-	Maintain professionalism in the workplace	720

0732B&CE187-	Maintain professional development and career professionalism	724

0732B&CE188-	Organize schedules	727

9.16 Construction III	729

0732B&CE-189. Erect and dismantle formwork to suspended slabs, columns, beams and walls	729

0732B&CE-190. Operate elevated platforms	733

0732B&CE-191. Construct, erect and dismantle formwork for stairs and ramps	738

0732B&CE-192. Evaluate materials for multi-storey buildings	746

0732B&CE-193. Apply structural principles to the construction of large, high rise and complex buildings	750

0732B&CE-194. Install fire and smoke containment systems	755

0732B&CE-195. Inform clients about thermal performance of residential buildings	764

0732B&CE-196. Design air conditioning and ventilation system	769

0732B&CE-197. Design acoustic of an auditorium	774

0732B&CE-198. Insulate sound insulation products in building	777

0732B&CE-199. Install ceiling insulation	781

0732B&CE-200. Design an earthquake resistant building	788

0732B&CE-201. Coordinate maintenance and repair of buildings	790

0732B&CE-202. Assess construction faults in buildings	793

0732B&CE-203. Repair and rectify concrete	797

0732B&CE-204. Repair wall and floor tiles	801

0732B&CE-205. Apply building codes and standards to building projects	805

9.17 Concrete Technology	808

0732B&CE-206. Test the Properties of Cement	808

0732B&CE-207. Find Out Strength of Cement	812

0732B&CE-208. Test the Properties of Aggregate	815

0732B&CE-209. Test the Properties of Fresh Concrete	819

0732B&CE-210. Destructive Tests on Hardened Concrete	822

0732B&CE-211. Perform Non-Destructive Tests on Hardened Concrete	827

9.18. Advanced Quantity Surveying	831

0732B&CE-212. Produce Detailed Estimate of Water Tanks.	831

0732B&CE-213. Produce Detailed Estimate of Wells.	837

0732B&CE-214. Produce Detailed Estimate of Steel Structures	842

0732B&CE-215. Produce Detailed Estimate of RCC Framed Structure	846

0732B&CE-216. Produce Detailed Estimate of Building Services.	849

0732B&CE-217. Produce Detailed Estimates Using Computer Software -Excel.	853

0732B&CE-218. Produce Project of Road in AutoCAD Civil 3D software	862

9.19. Construction Management	866

0732B&CE-219. Produce Pre-Qualification Documents & collect expression of interest	866

0732B&CE-220. Set Evaluation Criteria for the assessment	870

0732B&CE-221. Produce Tender Documents & Collect Proposals	872

0732B&CE-222. Perform Tender Evaluation & Award of Contract	876

0732B&CE-223. Perform Post-Award Legal/Contract Requirements and Finance.	878

0732B&CE-224. Maintain record of Stores.	881

0732B&CE-225. Perform Variation Orders, Claims and Dispute Adjudication Board	884

9.20 Sanitary Engineering	888

0732B&CE-226. Design sanitary sewers	888

0732B&CE-227. Design storm water sewers	891

0732B&CE-228. Layout of sewers	893

0732B&CE-229. Prepare sketches of sewer appurtenance	895

0732B&CE-230. Construct Sewers	900

0732B&CE-231. Maintain Sewers	903

0732B&CE-232. Install Sewage Pumps	905

0732B&CE-233. Produce sketches of sewage treatment plant	907

0732B&CE-234. Dispose sewage	909

9.21. Engineering Mechanics	911

0732B&CE-235. Design Homogeneous Beams	911

0732B&CE-236. Determine deflection of beams	917

0732B&CE-237. Determine Load Carrying Capacity of Columns	921

0732B&CE-238. Evaluate Efficiency of Riveted and Welded Joints	925

0732B&CE-239. Analyze Forces in Truss Members	932

0732B&CE-240. Determine Stability of Retaining Wall	936

d.	National Diploma  in Civil Technology  Level 5	945

9.22. Transportation Engineering I	945

0732B&CE-241. Develop construction plan of Tunnels	945

0732B&CE-242. Produce sketches of an airport	947

0732B&CE-243. Produce sketches of permanent Way	951

0732B&CE-244. Produce sketches of points and crossings	954

0732B&CE-245. Produce sketches of station yards and signals	957

0732B&CE-246. Produce sketches of harbor	960

9.23. Hydraulics Engineering	965

0732B&CE-247. Determine Hydrostatic pressure	965

0732B&CE-248. Apply Hydro kinematic Principles	968

9.24. Soil Mechanics	971

0732B&CE-249. Determine moisture content and specific gravity of soil	971

0732B&CE-250. Determine Atterberg’s Limits	973

0732B&CE-251. Identify Type of Soil	975

0732B&CE-252. Conduct soil compaction tests	978

0732B&CE-253. Determine permeability of soil	981

0732B&CE-254. Investigate shear strength of soil	983

0732B&CE-255. Find out bearing capacity of soil	985

9.25. Bridge Engineering	987

0732B&CE-256. Produce sketches of bridges w. r. t. functions	987

0732B&CE-257. Produce sketches of bridges w. r. t. material of construction	989

0732B&CE-258. Produce sketches of bridges w. r. t. type of super structure	991

0732B&CE-259. Produce sketches of bridges w. r. t. floor type	994

0732B&CE-260. Produce sketches of culverts, low level and high-level causeway	995

9.26. Environmental Technology	998

0732B&CE-261. Determine Water Pollution	998

0732B&CE-262. Evaluate Air Pollution	1002

0732B&CE-263. Determine Noise Pollution	1004

0732B&CE-264. Evaluate Solid Waste Pollution	1005

0732B&CE-265. Adopt occupational health and safety	1007

0732B&CE-266. Respond to Natural Disaster	1012

9.27. Construction Project planning and Management	1014

0732B&CE267-	Develop Project Management Life Cycle	1014

0732B&CE268-	Develop Preliminary Project Plan	1018

0732B&CE269-	Develop Construction Project Plan	1020

0732B&CE270-	Conduct on-site supervision of construction projects	1024

0732B&CE271-	Perform Basic operation in Primavera P6	1027

0732B&CE272-	Perform road Project Activities Scheduling in Primavera P6	1029

0732B&CE273-	Perform road Project Resources Costing &Planning in Primavera P6	1033

0732B&CE274-	Manage construction Project in Primavera P6	1038

0732B&CE275-	Identify Project Risk and Consequences on Project life cycle	1040

0732B&CE276-	Perform Evaluation to assess the severity of Risk and formulate the strategy to manage the risk	1042

9.28. Transportation Engineering II	1044

0732B&CE277-	Plan Route Alignment of Highway	1044

0732B&CE278-	Design Highway Geometrics	1046

0732B&CE279-	Perform tests on stone metal for pavement construction	1049

0732B&CE280-	Perform tests on bitumen	1051

0732B&CE281-	Prepare drawings of flexible/rigid pavement	1056

0732B&CE282-	Plan highway drainage	1059

0732B&CE283-	Develop maintenance plan for roads	1061

9.29. Irrigation Engineering	1063

0732B&CE284-	Produce sketches of canals and head works	1063

0732B&CE285-	Design Canals	1068

0732B&CE286-	Produce sketches of Hydraulic structures	1071

9.30. R.C.C. Design	1074

0732B&CE287-	Design and Draw Reinforcement for Beams	1074

0732B&CE288-	Design and Draw Reinforcement for Slabs	1077

0732B&CE289-	Design and Draw Reinforcement for Columns	1078

0732B&CE290-	Design and Draw Reinforcement for Footings	1081

0732B&CE291-	Design and Draw Reinforcement for Stairs	1083

9.31. Civil Engineering Project	1084

0732B&CE292-	Develop Project for construction of RCC framed Structure Class Room.	1084

Learning Unit	1084

0732B&CE293-	Develop Project plan for construction of RCC OHR of 10000-gal (UK) capacity	1087

0732B&CE294-	Develop Project plan for construction of Water Supply Scheme	1090

0732B&CE295-	Develop Project plan for construction of sewerage scheme	1093

0732B&CE296-	Develop Project plan for construction of Highway	1096

0732B&CE297-	Develop Project plan for construction of Irrigation Canal	1099

9.32. Entrepreneurial Skills.	1102

0732B&CE298-	Investigate micro business opportunities	1102

0732B&CE299-	Develop a micro business proposal	1108

0732B&CE300-	Develop a marketing plan and business plan.	1112

0732B&CE301-	Organize finances for the micro business.	1120

0732B&CE302-	Manage human resources.	1124

0732B&CE303-	Market products and services.	1129

0732B&CE304-	Monitor and review business performance.	1133

0732B&CE305-	Negotiate for resolving business issues	1137

9.33 Green Skills.

0732B&CE306-	Perform Green Skills in Construction

10.	List of Tools, Equipment and Consumable	1140

11.	Members of Curriculum Development Committee	1178

# Introduction

Civil engineering is considered as the first discipline of the various branches of engineering after military engineering, and includes the designing, planning, construction, and maintenance of the infrastructure. The works include roads, bridges, buildings, dams, canals, water supply and numerous other facilities that affect the life of human beings. Civil engineering is intimately associated with the private and public sectors, including the individual homeowners and international enterprises. It is one of the oldest engineering professions, and ancient engineering achievements due to civil engineering include the pyramids of Egypt and road systems developed by the Romans.

Civil engineering has a significant role in the life of every human being, though one may not truly sense its importance in our daily routine. The function of civil engineering commences with the start of the day when we take a shower, since the water is delivered through a water supply system including a well-designed network of pipes, water treatment plant and other numerous associated services. The network of roads on which we drive while proceeding to school or work, the huge structural bridges we come across and the tall buildings where we work, all have been designed and constructed by civil engineers. Even the benefits of electricity we use are available to us through the contribution of civil engineers who constructed the towers for the transmission lines. In fact, no sphere of life may be identified that does not include the contribution of civil engineering. Thus, the importance of civil engineering may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Civil Technology under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# Purpose of the Qualification:

The purpose of this qualifications is to set high professional standards for civil technology sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for civil technology industry

- Enable the existing workforce to capacitate themselves in new technologies and methods

# Overall objectives of training program

The Civil qualification of level 5 consists of both the theoretical and practical details and having the following Occupations

- Civil Surveyor

- Draftsman

- Quantity Surveyor

- Quantity Surveyor after Draftsman

- Materials Technician

- Construction Quality control Supervisor

- Mason

- Plumber

- Domestic Electrician.

- Site supervisor

# Entry level of trainees

| National Vocational Certificate Level 1 & 2 (Site Assistant in Civil Technology) | Middle |
| --- | --- |
| National Vocational Certificate Level 3 ( Assistant Surveyor in Civil Technology) | Level 1 & 2 |
| National Vocational Certificate Level 4 ( Site Supervisor in Civil Technology) | Level 3 |
| National Diploma in Civil Technology Level 5 | Level 4 |

# Minimum qualification for teachers

- B. Tech in Civil Technology 4 Years

- B.Sc. / B.E. in Civil Engineering 4 Years

- D. A. E. in Civil Technology with 3 Years teaching experience

- Must be able to communicate effectively

Medium of instruction

English, and Urdu.

# Duration of the course:

The proposed curriculum is composed of 306 modules that will be covered in 3600 Learning hours. Duration of the course is proposed to be Three years. The total weightage for technical modules is 3600 hours. A total of 1200 hours have been reserved for allied subjects i.e. Islamic studies, English, Mathematics, Physics and Chemistry.

The details of technical modules are given as under:

Level 1- 2 = 6 months. (Single Semester)

Level - 3 = 6 months. (Single Semester)

Level - 4 = 1 Year. (Two Semesters)

Level - 5 = 1 Year. (Two Semesters)

The overall distribution of contact hours and Credit Hours is given below:

Total.		3600 hours. & 360 Credits

Theory.	976 hours (27.11 %)

Practical.	2624 hours (72.89 %)

The distribution of contact hours and credit hours in each level is given below:

Level 1- 2 = 6 months (16 weeks)

Total.		592 hours. & 59.2 Credits

Theory.	128 hours (22 %)

Practical.	464 hours (78 %)

Level - 3 = 6 months (16 weeks)

Total.		608 hours. & 60.8 Credits

Theory.	128 hours (21 %)

Practical.	480 hours (79 %)

Level - 4 = 1 Year (16 weeks + 16 Weeks)

Total.		1200 hours. & 120 Credits

Theory.	240 hours (20 %)

Practical.	960 hours (80 %)

Level - 5 = 1 Year (16 weeks + 16 Weeks)

Total.		1200 hours. & 120 Credits

Theory.	480 hours (40 %)

Practical.	720 hours (60 %)

# Description and structure of the course

Following is the structure of the course:

(National Vocational Certificate Level 3 Assistant Surveyor in Civil Technology)

| Module Code | Competency Standard | Category | Level | Theory | Practical | Total | Credit Hour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9.06 Advanced Surveying |  |  | 48 | 192 | 240 | 24 |
| 0732B&CE053 | Measure Angles by Theodolite | Core | 3 | 4 | 18 | 22 | 2.2 |
| 0732B&CE054 | Perform Trigonometric Levelling by Theodolite | Core | 3 | 3 | 9 | 12 | 1.2 |
| 0732B&CE055 | Perform Traversing by Theodolite | Core | 3 | 4 | 15 | 19 | 1.9 |
| 0732B&CE056 | Set Out Horizontal Curves by Theodolite | Core | 3 | 4 | 18 | 22 | 2.2 |
| 0732B&CE057 | Set out Vertical Curves by Theodolite | Core | 3 | 3 | 9 | 12 | 1.2 |
| 0732B&CE058 | Carry out Triangulation by Theodolite | Core | 3 | 3 | 21 | 24 | 2.4 |
| 0732B&CE059 | Perform Temporary Adjustment of Total Station | Core | 3 | 4 | 9 | 13 | 1.3 |
| 0732B&CE060 | Measure Distance by Total Station | Core | 3 | 4 | 12 | 16 | 1.6 |
| 0732B&CE061 | Measure Angles by Total Station | Core | 3 | 4 | 18 | 22 | 2.2 |
| 0732B&CE062 | Measure Area by Total Station | Core | 3 | 3 | 18 | 21 | 2.1 |
| 0732B&CE063 | Set out Curves by Total Station | Core | 3 | 4 | 9 | 13 | 1.3 |
| 0732B&CE064 | Perform Integration of GPS | Core | 3 | 2 | 6 | 8 | 0.8 |
| 0732B&CE065 | Measure Co-ordinates by GPS | Core | 3 | 2 | 6 | 8 | 0.8 |
| 0732B&CE066 | Set out Civil Engineering Structures by GPS | Core | 3 | 4 | 24 | 28 | 2.8 |
|  | 9.07 AutoCAD |  |  | 32 | 128 | 160 | 16 |
| 0732B&CE067 | Perform Basic Computer Operations | Core | 3 | 3 | 6 | 9 | 0.9 |
| 0732B&CE068 | Perform Drafting Settings and Formatting | Core | 3 | 2 | 6 | 8 | 0.8 |
| 0732B&CE069 | Perform Dimensioning, Lettering & Plotting | Core | 3 | 2 | 6 | 8 | 0.8 |
| 0732B&CE070 | Develop 2d Drawings of Structures | Core | 3 | 3 | 15 | 18 | 1.8 |
| 0732B&CE071 | Produce Drawing of Framed Structure Building | Core | 3 | 3 | 18 | 21 | 2.1 |
| 0732B&CE072 | Develop Sectioning Drawing for foundations | Core | 3 | 3 | 18 | 21 | 2.1 |
| 0732B&CE073 | Develop Drawings for Doors & Windows | Core | 3 | 2 | 9 | 11 | 1.1 |
| 0732B&CE074 | Produce Drawings of Stairs and Stair Cases | Core | 3 | 2 | 9 | 11 | 1.1 |
| 0732B&CE075 | Produce Drawings of Water Tanks | Core | 3 | 3 | 18 | 21 | 2.1 |
| 0732B&CE076 | Produce Drawings Of Cross Drainage Works | Core | 3 | 2 | 6 | 8 | 0.8 |
| 0732B&CE077 | Develop 3d Drawings | Core | 3 | 4 | 6 | 10 | 1 |
| 0732B&CE078 | Produce Civil Technology Drawings | Core | 3 | 3 | 11 | 14 | 1.4 |
|  | 9.08 Construction I |  |  | 16 | 112 | 128 | 12.8 |
| 0732B&CE079 | Perform Simple Levelling | Core | 2 | 0.5 | 3 | 3.5 | 0.35 |
| 0732B&CE080 | Lay out Building plans | Core | 2 | 0.5 | 5 | 5.5 | 0.55 |
| 0732B&CE081 | Carryout excavation and install supports | Core | 2 | 1 | 5 | 6 | 0.6 |
| 0732B&CE082 | Place Concrete in Foundations | Core | 2 | 0.5 | 3 | 3.5 | 0.35 |
| 0732B&CE083 | Prepare for construction Process (Brick/ block laying) | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE084 | Mix Cementitious materials (Mortar & Concrete) | Core | 3 | 1 | 3 | 4 | 0.4 |
| 0732B&CE085 | Install flashing and damp-proof course | Core | 3 | 1 | 3 | 4 | 0.4 |
| 0732B&CE086 | Carry out brick work | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE087 | Construct Cavity Wall | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE088 | Construct Curved Wall | Core | 3 | 1 | 6 | 7 | 0.7 |
| 0732B&CE089 | Construct Masonry arches (semi-circular and segmental) | Core | 3 | 1 | 6 | 7 | 0.7 |
| 0732B&CE090 | Construct Scaffolding | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE091 | Install doors and window frames | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE092 | Prepare Surface and fix tiles | Core | 3 | 1 | 3 | 4 | 0.4 |
| 0732B&CE093 | Prepare Surface for Painting | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE094 | Paint by spray, brush and roller | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE095 | Apply stains and clear timber finishes | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE096 | Prepare site layout of civil engineering projects | Core | 3 | 0.5 | 6 | 6.5 | 0.65 |
| 0732B&CE097 | Construct masonry steps and stairs | Core | 3 | 1 | 6 | 7 | 0.7 |
| 0732B&CE098 | Install stairs | Core | 3 | 1 | 9 | 10 | 1 |
| 0732B&CE099 | Prepare for solid dry wall plastering | Core | 3 | 1 | 3 | 4 | 0.4 |
| 0732B&CE100 | Apply plaster | Core | 3 | 1 | 3 | 4 | 0.4 |
|  | 9.09 Plumbing |  |  | 16 | 32 | 48 | 4.8 |
| 0732B&CE101 | Identify plumbing tools & equipment | Core | 3 | 4 | 2 | 6 | 0.6 |
| 0732B&CE102 | Perform cutting, threading & reaming | Core | 3 | 2 | 4 | 6 | 0.6 |
| 0732B&CE103 | Make water connection | Core | 3 | 1 | 4 | 5 | 0.5 |
| 0732B&CE104 | Layout for domestic water supply | Core | 3 | 1 | 4 | 5 | 0.5 |
| 0732B&CE105 | Join pipes & fittings | Core | 3 | 3 | 5 | 8 | 0.8 |
| 0732B&CE106 | Install plumbing fixtures | Core | 3 | 2 | 4 | 6 | 0.6 |
| 0732B&CE107 | Install & repair water pumps | Core | 3 | 1 | 4 | 5 | 0.5 |
| 0732B&CE108 | Install turbine pumps | Core | 3 | 2 | 5 | 7 | 0.7 |
|  | 9.10 Digital Skills | 32 |  | 16 | 16 | 32 | 3.2 |
| 0732B&CE109 | Install computer operating systems and hardware | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE110 | Operate digital media technology | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE111 | Perform computer operations | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE112 | Use computer applications | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE113 | Create user documentation | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE114 | Create technical documentation | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE115 | Create basic databases | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE116 | Use social media tools for collaboration and engagement | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE117 | E-Commerce- SEO (Search Engine Optimization) | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE118 | E-Commerce- SCM (Supply Chain Management) | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE119 | E-Commerce- Social Media Marketing | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE120 | Use digital devices | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE121 | Operate word-processing applications | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE122 | Operate spreadsheet applications | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE123 | Operate presentation packages | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE124 | Perform writing and editing tasks | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE125 | Write and Edit Copy | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE126 | Build Java applets | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE127 | Manipulate Images (Illustrator) | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE128 | Manipulate Images (Photoshop) | Generic | 3 | 0.5 | 0.5 | 1 | 0.1 |
| 0732B&CE129 | Design simple web page layouts | Generic | 3 | 1 | 1 | 2 | 0.2 |
| 0732B&CE130 | Apply introductory programming techniques | Generic | 3 | 1 | 1 | 2 | 0.2 |

# Scheme of Studies.

(National Vocational Certificate Level 3 Assistant Surveyor in Civil Technology)

| Code | Name of Subjects | Category | Numbers of Module | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 000000000 | Advanced Surveying | Technical | 14 | 48 | 192 | 240 | 24 | 3 | 12 | 15 |
| 000000000 | AutoCAD | Technical | 18 | 32 | 128 | 160 | 16 | 2 | 8 | 10 |
| 000000000 | Construction Techniques I | Technical | 22 | 16 | 112 | 128 | 12.8 | 1 | 7 | 8 |
| 000000000 | Plumbing | Technical | 08 | 16 | 32 | 48 | 4.8 | 1 | 2 | 3 |
| 000000000 | Digital Skills | Technical | 22 | 16 | 16 | 32 | 3.2 | 1 | 1 | 2 |
| 00000000 |  | Allied |  |  |  |  |  |  |  |  |
| 000000000 |  | Allied |  |  |  |  |  |  |  |  |
| Total |  |  | 78 | 128 | 480 | 608 | 60.8 | 8 | 30 | 38 |

- Detail of Modules Level wise

  - National Vocational Certificate Level 3 Assistant Surveyor in Civil Technology

## 9.6. Advanced Surveying

### 0732B&CE-053: Measure Angles by Theodolite

Objective: This module covers the knowledge and  skills required to perform angle measurement of different angles by Theodolite with its basic components. It also covers the knowledge by using Theodolite Surveying.

Duration: 22 Hours						Theory: 04 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Adjust theodolite temporarily on a station point | Trainee will be able to: - Identify the Equipment - Set Theodolite on tripod at station point - Perform centering and levelling on station point | - Theodolite - Types of theodolites - Terms; centering, transiting, face left, Face right, swinging the telescope, axis of level tube, horizontal & vertical axis - Procedure of temporary adjustment of a theodolite Practical Activity - Adjust theodolite temporarily on a station point | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpener - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer - Thread | Class Room and Survey Field |
| - Measure Horizontal Angle by Repetition Method | Trainee will be able to: - Adjust theodolite on station point O - Set angle to 0⁰-0’-0”at given point A - Record reading of angle at point B as angle AOB - Repeat angle measurement three times for each face - Calculate mean of the readings | - Procedure of measuring horizontal angles by repetition method. Practical Activity - Measure Horizontal Angle by Repetition Method | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer - Thread | Class Room and Survey Field |
| - Measure Horizontal Angle by Reiteration Method | Trainee will be able to: - Adjust theodolite on station point O - Set angle to 0⁰-0’-0”at given point A - Bisect the second point B, C and D to record angle readings between AOB, BOC and COD - Close the horizon by bisecting the D to A to check if angle is 360 - Adjust error according to standard - Repeat the procedure by changing face | - Procedure of measuring horizontal angles by reiteration method. Practical Activity - Measure Horizontal Angle by Repetition Method | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | - Pencil - Eraser - Field Book - Sharpener - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer - Thread | Class Room and Survey Field |
| - Measure Vertical Angle | Trainee will be able to - Adjust theodolite on station - Set theodolite for vertical angle measurement - Record vertical angle by sighting the object - Take 2 more readings for mean | - Procedure of measuring vertical angles. Practical Activity - Measure Vetical Angle | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer | Class Room and Survey Field |

### 0732B&CE-054: Perform Trigonometric Levelling by Theodolite

Objective: This module covers the knowledge and  skills required to perform process of determining the differences of elevations of stations from observed vertical angles and known distances. The vertical angles are measured by means of theodolite. Measurements are to be carried out in two or sometimes three stages. The said levelling is to be done by Theodolite.

Duration: 12 Hours							Theory: 03 Hours				Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration |  | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- | --- |
| - Carryout Trigonometric Levelling (base is accessible of point) | Trainee will be able to: - Adjust theodolite on a station point from where altitude point is accessible - Set theodolite for vertical angle measurement - Take reading on bench mark - Record vertical angle of point to whom elevation is to be determined - Measure horizontal distance from the station to the point - Calculate elevation of point by trignometry | - Trigonometric levelling - Method of determining elevation of accessible point Practical Activity - Find out height of a building or tower | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer |  | Class Room and Survey Field |
| - Carryout Trigonometric Levelling (base is inaccessible of point) | Trainee will be able to: - Mark two station points in a line - Adjust theodolite on a station point closer to point of observation - Set theodolite for vertical angle measurement - Record vertical angle from closer station point - Adjust theodolite on second station point which is farther from point but in line of first station point - Record vertical angle from second station point - Measure distance between station points - Calculate elevation of point by trigonometry | - Method of determining elevation of inaccessible point Practical Activity - Find out height of a inaccessible point | Theory-02 Hrs Practice-06 Hrs Total- 08 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer |  | Class Room and Survey Field |

### 0732B&CE-055: Perform Traversing by Theodolite

Objective: This module covers the knowledge and  skills required to perform Traverse in the field of surveying to establish control networks. It is also used in geodesy. Traverse networks involve placing survey stations along a line or path of travel, and then using the previously surveyed points as a base for observing the next point.

Duration: 19 Hours							Theory: 04 Hours				Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration |  | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- | --- |
| - Perform traversing of an area (by Digital theodolite) | Trainee will be able to: - Mark station points which would cover area - Record bearing of first line by prismatic compass - Adjust theodolite on first station point - Record horizontal internal or external angle between lines originating from first station point - Shift theodolite to the next stations - Record horizontal internal or external angle between lines - Measure the distances between the station points - Compile the data | - Traverse & its types - Methods of traversing - Standards of accuracy of traversing - Check of open & closed traverse - Calculation of angles from given bearings and vice versa Practical Activity - Perform traversing of an area | Theory-02 Hrs Practice-09 Hrs Total-11 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Compass - Hammer - Thread |  | Class Room and Survey Field |
| - Plot traverse of the area (by Gale’s traverse table) | Trainee will be able to: - Compile the data in Gale’s Table - Apply necessary corrections - Calculate co-ordinates - Convert the dependent co-ordinates into independent co-ordinates - Calculate the area - Plot the traverse | - Introduction to coordinates, types and technical terms. - Computation of co-ordinates and computation of missing data associated with theodolites traversing - Balancing the traverse by different methods - Plotting & graphical adjustment of closing error Practical Activity - Plot traverse of area (by Gale’s traverse table) | Theory-02 Hrs Practice-06 Hrs Total- 08 Hrs | - Pencil - Eraser - Drawing sheet - Sharpner - Ruler - T- Scale - Proteractor - Compass - Drafting Triangles T&E - Drawing Table |  | Class Room and Drawing Hall |

### 0732B&CE-056: Set Out Horizontal Curves by Theodolite

Objective: This module covers the knowledge and  skills required to set Horizontal Curves of the two important transition elements in geometric design for highways. A horizontal curve provides a transition between two tangent strips of roadway, allowing a vehicle to negotiate a turn at a gradual rate rather than a sharp cut.

Duration: 22 Hours						Theory: 04 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set out Horizontal Curve by Taking offsets from Long Chord | Trainee will be able to: - Compute and compile data for setting out curve - Mark station points for setting out curve with help of theodolite - Mark tangent lines and long chord - Locate points of curve by taking normal offsets from long chord - Lay curve by lime | - Definition, types and necessity of curves - Designation of curves - Elements and notation of simple circular curves & their relationship - Calculation of data & method of setting out simple circular curve by Taking offsets from Long Chord Practical Activity - Set out simple circular curve by Taking offsets from Long Chord | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs - Lime T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer | Class Room and Survey Field |
| - Set out Horizontal Curve by Taking offsets from Tangent | Trainee will be able to: - Compute and compile data for setting out curve - Mark station points for setting out curve with help of theodolite - Mark tangent lines and long chord - Locate points of curve by taking normal offsets from tangent - Locate points of curve by taking radial offsets from tangent - Lay curve by lime | - Setting out simple curve beyond obstacles - Calculation of data & method of setting out simple circular curve by Taking offsets from Tangent Practical Activity - Set out simple circular curve by taking offsets from tangent | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs - Lime T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer | Class Room and Survey Field |
| - Set out Horizontal Curve by deflection agle method | Trainee will be able to: - Compute and compile data for setting out curve - Adjust the theodolite on first tangent point T1 - Locate second tangent point T2 - Set the half deflection angle at point T1 and mark the intersection point - Set angle 0º 0’ 0’’ by bisecting the intersection point - Set the angle for 1st chord - Fix peg on the point - Set all the angles - Set chord of curve - Lay curve by lime | - Calculation of data & method of setting out simple circular curve by deflection angle Practical Activity - Set out simple circular curve by deflection angle method | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs - Lime T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod Hammer | Class Room and Survey Field |
| - Set out transition curve in the field | Trainee will be able to: - Compute and compile data for setting out curve - Set two points T1, T2 at shift of curve - Erect pole in direction of B from T1 and T2 - Start setting out from T1 - Start setting out from T2 - Set out circular arc - Lay curve by lime | - Description, types and necessity of transition curves. Characteristics, elements & notation of transition curve and their inter relationship - Introduction to super elevation and methods of calculation length of transition curve with numerical problems - Calculation of data and methods of setting out of transition curve Practical Activity - Set out transit curve | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs - Lime T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer | Class Room and Survey Field |

### 0732B&CE-057: Set Out Vertical Curves by Theodolite

Objective: This module covers the knowledge and  skills required to perform Vertical Curves of the two important transition elements in geometric design for highways. A vertical curve provides a transition between two sloped roadways, allowing a vehicle to negotiate the elevation rate change at a gradual rate rather than a sharp cut.

Duration: 12 Hours								Theory: 03 Hours				Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Interpret to set out vertical curve | Trainee will be able to: - Compute and compile data for setting out curve - Identify gradients and rate of change of gradient - Locate point of intersection - Calculate length of vertical curve - Calculate tangent corrections - Find out levels of curve - Draw vertical curve | - Explain vertical curves - Introduction to vertical curves, - Types of vertical curves, elements, terms and their inter relationship. - Calculation of data and Setting out vertical curves Practical Activity - Prepare data of vertical curve and draw it. | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Drawing sheet - Sharpner - Ruler - T- Scale - Proteractor - Compass - Drafting Triangles T&E - Drawing Table | Class Room and Drawing Hall |
| - Set out Vertical Curve in the field | Trainee will be able to: - Set the theodolite on first tangent point T1 - Locate second tangent point T2 - Fix the poles at intermediate points - Perform leveling to find NSL - Mark the point on pole by using the difference in NSL | - Various steps in setting out vertical curves Practical Activity - Prepare data of vertical curve and draw it. | Theory-02 Hrs Practice-06 Hrs Total- 08 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs - Scoth tape - Pemranent Marker T&E - Calculator - Measuring steel Tape - Digital Theodolite - Tripod - Prism - Hammer - Thread - Hammer | Class Room and Survey Field |

### 0732B&CE-058: Carry Out Triangulation by Theodolite

Objective: This module covers the knowledge and  skills required to perform series or network of triangles in order to determine the distances and relative positions of points spread over an area, especially by measuring the length of one side of each triangle and deducing its angles and the length of the other two sides by observation from this baseline.

Duration: 24 Hours								Theory: 03 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Mark station points by dividing the area by triangles | Trainee will be able to: - Reconnaissance area - Mark stations covering the area - Divide the area in triangles - Fix a base line acceding to standards | - Introduction & types of triangulation. - Well condition and ill condition in Triangulation - Selection of station points Practical Activity - Mark station points by dividing the area by triangles | Theory-0.5 Hrs Practice-03 Hrs Total- 3.5 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer | Class Room and Survey Field |
| - Measure the base line | Trainee will be able to: - Identify equipment - Measure the base line - Apply corrections | - Measurement of base line - Correction of base line measurement Practical Activity - Measure base line | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Tripod - Thermometer - Spring Balance - Hammer | Class Room and Survey Field |
| - Measure the angles for triangulation | Trainee will be able to: - Identify equipment - Fix theodolite on a station point - Measure the angles - Shift the instrument on next station point - Measure all the required angles | - Measurement procedure of horizontal angles Practical Activity - Measure angles for triangulation | Theory-01 Hrs Practice-09 Hrs Total- 10 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Digital Theodolite - Tripod - Hammer | Class Room and Survey Field |
| - Calculate the area | Trainee will be able to - Compile data obtained from survey - Calculate missing data - Apply formula - Calculate the area | - Methods to solve Triangles - Sine rule and its application in triangulation Practical Activity - Calculate area of triangulation | Theory-0.5 Hrs Practice-03 Hrs Total- 3.5 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler | Class Room |

### 0732B&CE-059: Perform Temporary Adjustment of Total Station

Objective: This module covers the knowledge and  skills required to Temporary adjustment of total station on a station point. It is set of operations which are performed on a total station to make it ready for taking observations. These include its initial setting up on a tripod or other stand, centering, levelling up and focusing of eyepiece.

Duration: 13 Hours							Theory: 04 Hours				Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Adjust temporarily total station on a given point | Trainee will be able to: - Identify the equipment - Unfold tripod - Perform centering of total station at station point - Perform leveling of total station - Adjust foot screws & center the bubble - Focus the telescope | - Introduction, types, main parts and accessories of Total Stations - Procedure of temporary adjustment of total station Practical Activity - Perform temporary adjustment of total station on a given point | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Ranging rods - Total Station - Tripod - Hammer | Class Room and Survey Field |
| - Set the Total station Electronically | Trainee will be able to: - Set instrument on station point - Fix battery - Turn on instrument - Show opening screen as “MEAS” screen. - Adjust foot screw - Rotate instrument 90 degree - Repeat method and check | - Functions and modes of a Total Station - Setting of parameters, tilt correction and preparation for observations/ operations Practical Activity - Perform temporary adjustment of total station on a given point | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Total Station - Tripod - Prism - Hammer | Class Room and Survey Field |
| - Set co-ordinates of occupied station | Trainee will be able to: - Set instrument on station point - Turn on instrument - Select co-ordinate menu - Input co-ordinates. - Check Result | - Co-ordinates - Setting and calculation co-ordinate value of occupied station, setting height of instrument & height of reflector Practical Activity - Set co-ordinates of occupied station | Theory-02 Hrs Practice-03 Hrs Total- 05 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Total Station - Tripod - Prism - Hammer | Class Room and Survey Field |

### 0732B&CE-060: Measure Distance by Total Station

Objective: This module covers the knowledge and  skills required to identify the taken measurement horizontal, vertical and inclined by Total Station with its components.

Duration: 16 Hours						Theory: 04 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure distance by Total Station (Horizontal & Vertical) | Trainee will be able to: - Set up total station on station point & install battery - Show opening screen at measuring mode - Input the co-ordinates of the station point - Set the prism on the required point - Record readings - Recheck distance by steel tape | - Methods of measurement of horizontal and vertical distances Practical Activity - Practice to measure distance by Total Station (Horizontal & Vertical) | Theory-02 Hrs Practice-03 Hrs Total- 05 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Total Station - Tripod - Prism - Hammer | Class Room and Survey Field |
| - Measure distance by Total Station (Inclined Distance) | Trainee will be able to: - Set up total station on station point & install battery - Show opening screen at inclined measurement - Set the prism on the required point - Adjust Total station toward Prism - Record readings on screen - Recheck distance by invar tape | - Methods of measurement of inclined distance Practical Activity - Practice to measure distance by Total Station (Inclined Distance) | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Total Station - Tripod - Prism - Hammer | Class Room and Survey Field |
| - Stack out different point at various distances on a line | Trainee will be able to: - Set up total station on station point - Select stack out menu - Select the method to stake out the point - Select "XY" to stake out by coordinates - Use the keypad to enter the coordinates or distances | - Methods to stack out different point at various distances on a line Practical Activity - Practice to stack out different point at various distances on a line | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Total Station - Tripod - Prism - Hammer | Class Room and Survey Field |

### 0732B&CE-061: Measure Angles by Total Station

Objective: This module covers the knowledge and  skills required to identify the Total station with its components and function. It also cover the Kinematics techniques for using Total Station in the field along with its safety precautions.

Duration: 22 Hours							Theory: 04 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure horizontal angles by Total Station | Trainee will be able to: - Identify station points - Set total station on intermediate station point - Fix ranging rod on the first point - Fix ranging rod on the other point - Sight first point - Set 0˚-0’-0” on total station for horizontal angle - Sight the other point - Record the measurement - Repeat by changing face | - Methods of measuring horizontal angles, bearings and co-ordinates by total station. Practical Activity - Practice to measure horizontal angle by repetition method with total station | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Digital Theodolite - Tripod - Prism - Hammer - Thread | Class Room and Survey Field |
| - Measure vertical angles by Total Station | Trainee will be able to: - Select the station point - Set total station on station point - Sight the first point - Set 0˚-0’-0” on total station for vertical angle - Sight the prism - Record measurement for vertical angle - Set percentage for Angle | - Methods of measuring vertical angles Practical Activity - Practice to measure vertical angle with total station | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Digital Theodolite - Tripod - Prism - Hammer | Class Room and Survey Field |
| - Measure the height of an object with total station by using REM | Trainee will be able to: - Set up total station on station point - Place the mirror target on the ground at the corner of the building - Record the mirror target height from the range pole scale - Set target height by setting target height - Lock position with horizontal and vertical clamps - Take observation using REM menu - Unlock the vertical clamp and sight in the top of the building corner - Re-lock vertical clamp - Record height BY using REM | - Procedure to measure the height of an object with total station by using REM Practical Activity - Practice to measure the height of an object with total station by using REM | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Digital Theodolite - Tripod - Prism - Hammer | Class Room and Survey Field |
| - Measure the distance between different station points with the help of occupied point (MLM) by using measure data or by using coordinate data | Trainee will be able to - Set up total station on station point - Place the mirror on reference point - Measure the reference point - Aim at the first target point - Measure the target point - Aim at the second target point - Measure the second target point - Record results | - Method of measuring the distance between different station points with the help of occupied point (MLM) by using measure data or by using coordinate data Practical Activity - Practice to measure the height of an object with total station by using REM | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Digital Theodolite - Tripod - Prism - Hammer | Class Room and Survey Field |

### 0732B&CE-062:  Measure Area by Total Station

Objective: This module covers the knowledge and  skills required to adjust total station on a station point temporarily and to find out the area of plots and transfer data into computer by networking method.

Duration: 21 Hours							Theory: 03 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure the area of a traverse by direct field observations | Trainee will be able to: - Set up total station on station point - Create a file name using file management - Enter instrument height and prism code after setting zero against point number or name - Measure first point and save it - Turn the total station to second point - Repeat the steps to the rest of the stations - Calculate area by using functions of total station | - Method to measure the area of a traverse by direct field observations. Practical Activity - Practice to measure the area of a traverse by direct field observations. | Theory-1.5 Hrs Practice-09 Hrs Total- 10.5 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Digital Theodolite - Tripod - Prism - Hammer | Class Room and Survey Field |
| - Measure area for rectangular Plot | Trainee will be able to: - Set up the total station - Turn on total station - Position the horizontal circle. - Select boundary points - Apply area command - Show Area - Recheck Mathematically | - Method to measure the area of a rectangular plot by direct field observations. Practical Activity - Practice to measure the area of a traverse by direct field observations. | Theory-1.5 Hrs Practice-09 Hrs Total- 10.5 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs T&E - Calculator - Measuring steel Tape - Digital Theodolite - Tripod - Prism - Hammer | Class Room and Survey Field |

### 0732B&CE-063: Set Out Curves by Total Station

Objective: This module covers the knowledge and  skills required to adjust total station on a station point temporarily and to lay the curves with its different methods in Railway and Roads. It also helps to transfer data into PC.

Duration: 13 Hours							Theory: 04 Hours				Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set horizontal curve by Total Station (by Coordinates) | Trainee will be able to: - Compute and compile data for setting out curve - Locate station points by total station - Adjust total station on a station point - Locate points of curve - Lay curve by lime | - Co-ordinates - Setting and calculation co-ordinate value of occupied station, setting height of instrument & height of reflector - Stack out method of total station Practical Activity - Set out horizontal curve by total station (by coordinates) | Theory-02 Hrs Practice-06 Hrs Total- 08 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs - Lime T&E - Calculator - Measuring steel Tape - Total Station - Tripod - Prism - Hammer - Thread | Class Room and Survey Field |
| - Set Vertical curve by Total Station | Trainee will be able to: - Compute and compile data for setting out curve - Locate station points by total station - Adjust total station on a station point - Locate points of curve - Mark points of curve on poles | - Co-ordinates - Setting and calculation co-ordinate value of occupied station, setting height of instrument & height of reflector Practical Activity - Set out vertical curve by total station | Theory-02 Hrs Practice-03 Hrs Total- 05 Hrs | - Pencil - Eraser - Field Book - Sharpner - Ruler - Pegs - Scoth tape - Pemranent Marker T&E - Calculator - Measuring steel Tape - Digital Theodolite - Tripod - Prism - Hammer - Thread | Class Room and Survey Field |

### 0732B&CE-064: Perform Integration of GPS

Objective: This module covers the knowledge and  skills required to Identify the parts of GPS, Identify modes/functions of GPS, traverse the area by static techniques, traverse the area by Stop & Go techniques and traverse the area by Real Time Kinematics techniques. Its also cover a unique position of a specific feature. With this information, one can navigate back to it. However, one cannot relate this “feature position” to any other “feature position” unless one is standing at the site and other features are visible.

Duration: 08 Hours								Theory: 02 Hours				Practice: 06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify parts of GPS | Trainee will be able to: - Identify GPS - Identify type of GPS Identify parts of GPS - Install battery to GPS | - Introduction to GPS & its types. - Working of GPS - Process of Data in GPS Practical Activity - Identify parts of GPS | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - T&E - Handheld GPS | Class Room and Survey Lab |
| - Identify modes / functions of GPS | Trainee will be able to: - Identify modes of GPS - Indicate functions of GPS - Identify parameters of GPS - Identify sources of Errors in GPS | - Functions and modes of GPS. - Setting parameters Practical Activity - Prepare GPS for operation | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner T&E - Handheld GPS | Class Room and Survey Lab |

### 0732B&CE-065: Measure Co-ordinates by GPS

Objective: This module covers the knowledge and  skills required to draw Co-ordinates by GPS on given station point. It also helps to transfer data into PC.

Duration: 08 Hours								Theory: 02 Hours				Practice: 06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set out co-ordinates by GPS | Trainee will be able to: - Set out GPS - Turn ON GPS - Find your co-ordinates - Tap cross hair icon - Hold the blue dot - Note co-ordinate location | - Coordinates - Location of Pakistan in coordinates - Method to input coordinates by GPS Practical Activity - Set out coordinates of a station point by GPS | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Pegs T&E - Handheld GPS - Hammers | Class Room and Survey Field |
| - Set coordinates within REFLEXW by GPS | Trainee will be able to: - Set out GPS - Turn ON GPS - Use REFLEXW mode - View profile by google map - Use Processing options - Take 3-D, topography - Generate Data - Check Result | - REFLEXW by GPS - Method to set coordinates within REFLEXW by GPS Practical Activity - Set coordinates within REFLEXW by GPS | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Field Book - Sharpner - Pegs T&E - Handheld GPS - Hammer | Class Room and Survey Field |

### 0732B&CE-066: Set Out Civil Engineering Structures by GPS

Objective: This module covers the knowledge and  skills required to use the Global Positioning System (GPS) as a tool for Civil Engineering on monitoring the vibrations of large road structures, notably the bridges. To be characterized as a structural tool it was developed and tested a method which is based on the interferometry principle. The method uses the L1 carrier phase that needs to be collected from only two satellites.

Duration: 28 Hours								Theory: 04 Hours				Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - Set out Civil Engineering Structures by GPS (BY USING THE CLASSIC RTK TECHNIQUE) | Trainee will be able to: - Set GPS at station point - Install battery - Select “RTK” Network Approach - Apply (R.T) Real Time Positioning by GPS - Use GPS Derived “COORDINATES” - Use Laser Pointing System - Evaluate “RTK” Accuracy Results | - RTK Technique - Method to set out structures by GPS (BY USING THE CLASSIC RTK TECHNIQUE) Practical Activity - Set out Civil Engineering Structures by GPS (BY USING THE CLASSIC RTK TECHNIQUE) | Theory-02 Hrs Practice-12 Hrs Total- 14 Hrs | - Pencil - Eraser - Field Book - Sharpner - Pegs - Lime T&E - Handheld GPS - Hammer |  | Class Room and Survey Field |
| - Use 3-D data interpretation By GPS | Trainee will be able to: - Import data from GPS - Process Data - Check Results. | - Introduction to GIS - REFLEXW - 3D-datainterpretation of REFLEXW - Generating a 3D-file without interpolation - Generating a 3D-file from Reflexw formatted parallel 2D-lines - Generating a 3D-file with interpolation (done within the 3Ddatainterpretation) - Resulting REFLEXW 3D-datafile - combine different 3D-datafiles - Processing a 3D-data file (done within the 2D-dataanalysis) Practical Activity - Practice to use 3-D data interpretation By GPS | Theory-02 Hrs Practice-12 Hrs Total- 14 Hrs | - Pencil - Eraser - Field Book - Sharpner T&E - Handheld GPS - Data Cable - C.P.U - Mouse - Keyboard - Multimedia - Printer - USB |  | Class Room and Computer Lab |

## 9.7. AutoCAD

### 0732B&CE-67. Perform Basic Computer Operations.

Objective: This module covers the knowledge and  skills required to prepare report on the health and safety considerations while working on C.P.U, recognize basic C.P.U hardware, software, applications and perform general troubleshooting, demonstrate your skills of MS Word,  MS  PowerPoint,  MS  Excel  as  well  as  installation  and  troubleshooting  of Operating System and software.

Duration:  9.0 Hours							Theory: 3.0 Hours				Practice: 6.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Investigate and prepare a short report on the health and safety considerations while working on IT equipment. | Trainee will be able to: - Observe Considerations: light (natural, artificial), - Identify any hazard risk - Perform CAD (Computer screens, electrical protection devices) - Repetitive work, working postures, Incorrect screen settings, working without break | - Introducing HSE - Preparing working environment - Connecting PC to electricity - Connecting multimedia - Setting out work - Installing CAD Software Practical Activity prepare a short report on the health and safety considerations while working for IT Equipment | Theory0.5Hrs Practice-1 Hrs Total- 1.5 Hrs | Drawing Sheet Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse Data cables Printer USB Multimedia | Computer Lab |
| Configure C.P.U System | Trainee will be able to: - Connect C.P.U components and peripherals as per requirements - Install System software and applications software according to the Instruction Manual - Troubleshoot Applications to trace and fix faults (if any) to bring it in a running condition - Follow health, safety and security procedures to ensure safe working environment | - Understanding Operating systems - Procedure of Installation the software application. - Connecting system to electric supply - Introducing and Connecting data cables - Troubleshooting the application - Troubleshooting the software - Ensuring HSE - Margin setting - Page layout Practical Activity Configure C.P.U System before preparing file. | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Create a document Using MS Word | Trainee will be able to: - Compose a document as per requirements - Assign name and location to save a file in word file format - Format Word Document according to given requirements - Generate hard copy according to job requirements | - Understand Operating systems. - Describing procedure of Installation of software applications. (MS Word) - Locating Open, close files and manage files. - Describing Hardware and Software. - Accessing a CD / DVD / ROM and Data traveller. - Troubleshooting hardware and software problems. - Assigning the file name - Formatting the document - Generating Hard Copy - Margin setting - Page layout Practical Activity Create a document Using MS Word | Theory0.5Hrs Practice-1 Hrs Total- 1.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Prepare a Worksheet Using MS Excel | Trainee will be able to - Develop a worksheet as per given data - Format the worksheet according to given job requirements - Apply Formulas according to given criteria - Generate Charts/Graphs according to the given data | - Basic formatting bold, italic & centre. (M.S. Excel) - Save a file - Print a file - Hyperlinking and referencing - Apply Formulas - Short Keys use in MS Office Practical Activity Create Worksheet Using MS Excel | Theory0.5Hrs Practice-1 Hrs Total- 1.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB | Computer Lab |
| Prepare presentation in power point software. | Trainee will be able to - Insert Slides with different Layouts according to requirements of presentation - Insert text, tables, images, etc. according to the requirements - Apply a set of effects to animate the slide according to requirements - Apply Slide Transitions on Slides according to requirement - Apply Sound Effects on Objects / text / images according to job requirements. | - Basic formatting bold, italic & centre. (POWER POINT) - Create slides - Change font & color on slides - Editing on slides - Save a file - Print a file - Hyperlinking and referencing - Insert and deletes slides - Images insert in slides - Set animations - Transit slides - Sound effects in slides Practical Activity Create presentation in power point software. | Theory1.0Hrs Practice-1 Hrs Total- 2.0 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |

### 0732B&CE-68. Perform Drafting Settings and Formatting

Objective: This module covers the knowledge and skills required to create two dimensional drawings by using various tools and commands, create & modify objects and drawings in AutoCAD to meet specific targets according to civil technology requirements.

Duration:  8.0 Hours						Theory: 2.0 Hours					Practice: 6.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Set up user interface for require specifications | Trainee will be able to: - Load the AutoCAD software. - Set the units, Limits, coordinate system, Workspace, - Set Object Snap Settings. - Set up the grid, snap, ortho, tracking settings. - Save AutoCAD drawing files in different file formats (DWG, DWT). | - Basic Drawing Settings - Software Installing - Unit Setting - Co. Ordinates setting - Setting OSNAP - ortho, tracking settings. - Saving a file of CAD - Printing a file of CAD Practical Activity Set up user interface for require specifications | Theory 1.0Hrs Practice-2 Hrs Total- 3.0 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse Data cables Printer USB Multimedia | Computer Lab |
| Perform formation of entities- line, line type, text, layers, points, dimensioning. | Trainee will be able to: - Setup user interface settings for required specifications - Load centre line, dashed line, border, dot. - Create layer with lines, colours, weight. - Format text styles & multiline styles. - Format dimension styles. - Format point styles. | - Use Direct distance entry system for drawing lines - Set U.C.S for different situations - Understanding Basic Commands - Angles and lines setting in AutoCAD - Creating layer and its setting - Loading of centre line and borders - Formatting text style - Formatting Dimension style - Formatting point style Practical Activity Perform formation of entities- line, line type, text, layers, layer, points, dimensioning in Auto CAD Software | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Create templates of sheets sizes A2, A3 & A4 having geometrical figures in 2D. | Trainee will be able to: - Setup user interface settings for required specifications - Format the line, line type, text, layers, points. - Draw rectangle with line command by adopting coordinates- rectangular (absolute & relative). - Draw, polygons, ellipse & circle. - Create title block layout for sheets sizes A2, A3 & A4. - Use Design centre for inserting styles in new file. - Save the files in DWT format. | - Defining templates - Different geometrical figures - Setting up user interface to given specification - Formatting different lines, text, layers etc - Prepare ellipse or polygons - Title block creating - Inserting style and manage different styles - Saving & Printing the aid file Practical Activity Create templates of sheets sizes A2, A3 & A4 having geometrical figures in 2D. | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |

### 0732B&CE-69. Perform Dimensioning and Printing, Lettering and Plotting

Objective: This module covers the knowledge and skills required to create 3- Dimensional Models by using various tools and commands in AutoCAD software, demonstrate skills to modify 3D objects and models to ensure civil technology requirements and present a rendered 3D Model final outcome.

Duration: 8.0 Hours							Theory: 2.0 Hours				Practice: 6.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Perform dimensioning of drawing. | Trainee will be able to: - Select dimension styles as per scale. - Dimension the drawing by applying dimension styles as per scale. - Dimension location of object. - Dimension drawing aligned dimension, continuous dimension, base line dimension and leader. | - Drafting settings (Unites, Limits, Snap - Auto (On & Off) Snap setting with their application - Dimension location for given objects. - Layer options with their application. - Text and their application - Multi line text application - Arrows and leaders Practical Activity Perform dimensioning of given drawing with different patterns in Auto CAD Software. | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Perform lettering (Text) of styles. | Trainee will be able to: - Select & Insert multi-line text style lettering for drawing type. - Select & Insert single-line text style lettering for notes on drawing. - Select & Insert single-line text style lettering for schedule on drawing. - Select & Insert single-line text style lettering for schedule on drawing. | - Defining lettering - Lettering types - Importance of lettering in Auto CAD - Text styles (Single and double line) - Lettering on new and old drawing - Inclined lettering - Default lettering Practical Activity Perform lettering (Text) with different styles on new and old drawing in Auto CAD. Software. | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Create Layouts for printing for Printing. | Trainee will be able to: - Perform setting in layouts as per required scale - Perform setting in layouts as per required orientation. - Plot drawing on scale according to required size. | - Printing importance - Printing option - Printing command - Attach printer with CPU by data cable. - Select scale for printing - Plot drawing for printing Practical Activity Create Layouts for printing a drawing on Auto CAD Software. | Theory1.0 Hrs Practice-2 Hrs Total- 3.0 Hrs | Printing Papers Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse Data cables Printer USB Multimedia | Computer Lab |

### 0732B&CE-70. Develop 2D Drawings of Structures

Objective: This module covers the knowledge and skills required to create two dimensional drawings by using various tools and commands, create & modify objects and drawings in AutoCAD to meet specific targets according to civil technology requirements.

Duration: 18.0 Hours							Theory: 3.0 Hours				Practice: 15.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Perform basic settings for AutoCAD. | Trainee will be able to: - Setup drawing interface for required specifications - Create formation of entities- line, line type, layers. - Create styles- text, points, multi-lines - Create lines and multi-lines | - Setting up different commands and tools. - Modify commands and tools. - HATCHING concepts in AutoCAD. - Zooming options - Features of Tools palettes window and their application - Design centre feature- Scale and paper sizes - Multi line formation Practical Activity Perform basic settings for an object by “AutoCAD” software. | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Execute draw commands objects. | Trainee will be able to: - Draw construction lines. - Draw lines with precision as per standards. - Draw curves with precision as per standards. - Hide the construction lines layer. | - Commands and tools. - Describe and use modify commands and tools. - Lines precision - Curve with Precision - Layer with its hide command - Construction of line - Multi Lines Practical Activity Execute draw commands objects with its dimension and lettering. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Perform application of Modify tools / commands to delete whole or part of surplus entities. | Trainee will be able to: - Apply FILLET & TRIM, ERASE, BREAK commands to delete whole or part of surplus entities in given scenario. - Apply CHAMFER, EXTEND & FILLET to modify corners. - Apply SCALE, LENGTHEN, STRETCH for multiple purposes. | - Apply draw commands and tools. - Use modify commands and tools. - HATCHING concepts in AutoCAD. - Use zooming options - Features of Tools palettes window and their application - Design centre feature Scale and paper sizes - Fillet & extend command - Chamfer command Practical Activity Perform application of Modify tools / commands to delete whole or part of surplus entities. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Change properties of objects to meet set standards. | Trainee will be able to - Apply ROTATE to rotate & MOVE to move objects. - Use OFFSET, MIRROR, ARRAY & copy to create multiple objects. - Modify properties by SCALE, EXPLODE, PEDIT, JOIN for multiple purposes. | - Rotate command - Move and Shift Command - Mirror & Array command - Modify object with its properties - Scale command - Edit & P. edit command - Join & Explode command Practical Activity Change properties of objects to meet set standards. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Assign symbols to specified materials | Trainee will be able to - Set the scale factors for hatching patterns. - Select specified area / object on drawing. - Apply the hatch to the area. - Calculate hatched area by Auto CAD command | - Defining symbols - Hatching of Symbols - Hatching of specified area - Osnap on / off - Edit properties of objects. - Lettering command - Line & Multi line command - Circle and arc command Practical Activity Assign symbols to specified materials to a given objects. | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Modify drawing objects using properties option. | Trainee will be able to - Perform inquiry commands to get area & distance of object. - observe the objects properties- location, linear & angular measurements, and other drafting formations. - Modify the objects properties- location, linear & angular measurements, and other drafting formations - Apply linear measurements with other drafting formations - Apply angular measurements with other drafting formations | - Extend command - Trim Command - Stretch command - Break and scale drawn items. - Joining of lines - Changing layer colour Practical Activity Modify drawing objects using properties option on Auto CAD Software | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheet Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse & USB Data cables Printer Multimedia | Computer Lab |

### 0732B&CE-71. Produce Drawing of Framed Structure Building

Objective: This module covers the knowledge and skills required to draw architectural & structural drawing for a double storey RCC framed structure building (key plan, layout plan,.0 detailed plan, elevations, sections, structural drawing of building components), detailed drawing of steel trussed roof shed (60’ x 40’) and sketches of arches, floors & roofs.

Duration: 21 Hours								Theory: 3.0 Hours			Practice: 18.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare architectural drawings of a Small two storey RCC framed Structure Building. | Trainee will be able to: - Select the suitable instruments to draw frame. - Distribute space of drawing sheet. - Draw key plan, layout plan, detailed plan of building, roof drainage plan & Floor plan of building. - Draw Elevation, and sections of building. - Perform lettering, printing & dimensioning. - Prepare schedule of area, openings and fittings. - Indicate North direction - Perform printing for drawing | - architectural Drawings. - Differentiate between key plan and lay out plan - drainage - elevation - lettering Practical Activity Draw Architectural drawings of a Small two storey RCC framed Structure Building. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales Card Compass Protractor Stencils | Computer Lab |
| Prepare structural drawings of a Small two storey RCC framed Structure Building. | Trainee will be able to: - Draw structural detail of roof, floor, and lintels. - Draw title strip on sheet. - Draw Reinforcement details of RCC column with base, beams, slabs, and stairs. - Perform lettering, printing & dimensioning. - Indicate North direction - Perform printing for drawing | - Sketch out the x-section of lintels and arches. - The various parts of lintels and arches. - Different types of floors. - Draw the sketches of different parts of floors. - The standard proportions for the different layers of floors. - Different types of roofs i.e. R.C.C and R.B roof and Pre-cast roof slabs. Practical Activity Prepare structural drawings of a Small two storey RCC framed Structure Building. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales (Card, Plane & Diagonal) Compass Protractor Stencils | Computer Lab |
| Prepare detailed drawing of steel trussed roof shed (60’ x 40’) | Trainee will be able to: - Select the suitable instruments to draw shed. - Distribute space of drawing sheet. - Draw plan of shed. - Draw frame of truss with roofing detail- sheets, purlin, gutter, ridging and wind ties. - Draw detail of joints and fixing at support. - Perform lettering, printing & dimensioning. - Indicate North direction - Perform printing for drawing | - Label the sketches of different parts of trusses. - Distribute the space for different views evenly on drawing sheet. - Different types of sloping roof. - Draw the sketches of steel trusses up to 40' span from the given data. - joint for truss - Gutter - wind tie Practical Activity Prepare detailed drawing of steel trussed roof shed (60’ x 40’) | Theory-0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales (Card, Plane & Diagonal) Compass Protractor Stencils | Computer Lab |
| Draw sketches of arches | Trainee will be able to - Select the suitable instruments to draw sketches. - Draw title strip on sheet OR block. - Distribute space of drawing sheet. - Sketch semi-circular, circular, segmental, two cusp, three cusp, elliptical, & flat arches. - Perform lettering and dimensioning. - Indicate North direction - Perform printing for drawing | - Arches - Differentiate between lintel and Arches - segmental arches - bulls’ eye arch - flat arch - reinforcement for arch Practical Activity Draw sketches of different arches | Theory 0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Graph Sheets Duster Pencils Eraser Sharpener Sand paper Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales Card Compass Protractor Stencils | Computer Lab |
| Draw sketches of roofs | Trainee will be able to: - Select the suitable instruments to draw sketches. - Draw title strip on sheet OR block. - Distribute space of drawing sheet. - Sketch first class mud roof, 2nd Class mud roof, R.C.C, R.B slabs, Pre-cast, roof. Jack arch and sloping roofs. - Perform lettering and dimensioning. - Indicate North direction - Perform printing for drawing | - different types of roofs i.e. first class mud roofing, 2nd Class mud roofing, R.C.C and R.B roof and Pre-cast roof slabs. - Different types of sloping roof. - Sloping angle - Pitch Practical Activity Draw sketches of different floors | Theory 0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales Cards Compass Protractor Stencils | Computer Lab |
| Draw sketches of floors | Trainee will be able to: - Select the suitable instruments to draw sketches. - Draw title strip on sheet OR block. - Distribute space of drawing sheet. - Sketch conglomerate, mosaic, terrazzo, glazed tile, brick tile, wooden, vinyl, rubber, cork, & tuff tile floors. - Perform lettering and dimensioning. - Indicate North direction - Perform printing for drawing | - Different types of floors. - Draw the sketches of different parts of floors. - The standard proportions for the different layers of floors. - terrazzo floor - glazed floor - tuff tile with its type Practical Activity Draw sketches of different floors | Theory 0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Graph Sheets Duster Pencils Eraser Sharpener Sand paper Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils | Computer Lab |

### 0732B&CE-72. Develop Sectionals Drawing for Foundations

Objective: This module covers the knowledge and skills required to draw structural drawing of raft foundation, grillage foundation, Pile Foundation, well foundation, spread footing, column base & pillar.

Duration: 21.0 Hours							Theory: 3.0 Hours				Practice: 18.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare structural detail of Raft foundation. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw structural steel drawings of raft foundation. - Perform printing for drawing | - Sketch the raft foundations - Type of foundation - structural steel for foundation - Calculate the depth and breadth required for spread footings. Practical Activity Prepare structural detail of Raft foundation. | Theory0.5Hrs Practice-4 Hrs Total- 4.5 Hrs | Drawing Graph Sheets Duster Pencils Eraser Sharpener Sand paper Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils | Computer Lab |
| Prepare structural detail of Grillage foundation. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw structural drawings of grillage foundation. - Perform printing for drawing | - Sketch the spread footing. - Sketch the grillage foundation. - need for grillage foundation - Describe type of foundations - Describe structural design for grillage foundation Practical Activity Prepare structural detail of Grillage foundation. | Theory0.5Hrs Practice-4 Hrs Total- 4.5 Hrs | Drawing Graph Sheets Duster Pencils Eraser Sharpener Sand paper Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils | Computer Lab |
| Prepare structural detail of Pile foundation. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw structural steel drawings of Pile foundation. - Perform printing for drawing | - Sketch the well and pile foundation. - Sketch the caisson foundations. - importance of pile foundation - Differentiate between shallow and deep foundation Practical Activity Prepare structural detail of Pile foundation. | Theory0.5Hrs Practice-4 Hrs Total- 4.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales Cards, Compass Protractor Stencils | Computer Lab |
| Prepare structural detail of Well foundation. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw structural drawings of well foundation. - Perform printing for drawing | - Sketch and label the raft & pile foundations with steel reinforcement. - method to distribute space on drawing sheet - steel reinforcement for well foundation Practical Activity Prepare structural detail of Well foundation. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales Card Compass Protractor Stencils | Computer Lab |
| Prepare structural detail of spread footing, pillar, foundations & column base. | Trainee will be able to - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw structural drawings of spread footing, pillar, & column footing. - Perform printing for drawing | - Calculate the depth and breadth required for spread footings. - footing - column base - footing - footing reinforcement Practical Activity Prepare structural detail of spread footing, pillar, foundations & column base. | Theory 1.0 Hrs Practice-3 Hrs Total- 4.0 Hrs | Drawing and Graph Sheets Duster Pencils Eraser Sharpener Sand paper Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils | Computer Lab |

### 0732B&CE-73. Develop Drawings for Doors & Windows

Objective: This module covers the knowledge and skills required to draw sketches of ledged, ledged & batten, louvers, collapsible, sliding, rolling, revolving doors, sketches glazed, louvers windows and detailed drawings of panelled & glazed, flush and wire gauzed doors & windows.

Duration:  11.0 Hours							Theory: 2.0 Hours				Practice: 9.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare sketches of doors | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Sketch the ledged, ledged & batten, louvers, collapsible, sliding, rolling, revolving doors. - Sketch glazed, louvers windows. - Perform lettering & dimensioning. - Perform printing for drawing | - Need of doors. - Clerestory ventilators. - The different types of doors. - The various parts of doors. - glazed door - louvers door - Ventilator. Practical Activity Draw structural drawing of Panelled & Glazed, Flush and Wire Gauzed Doors on drawing sheet by scale with long and cross section. | Theory-1 Hrs Practice-3 Hrs Total- 4 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils Templates | Computer Lab |
| Prepare structural drawing of Panelled & Glazed Doors & Windows | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan, sections & elevation panelled & glazed door & window. - Perform lettering & dimensioning. - Perform printing for drawing | - The various parts of windows. - importance of windows - the different materials used for windows and ventilators - Sketch the elevations, sectional plans and vertical sections of windows and ventilators. - Parts of doors Practical Activity Draw structural drawing of Paneled & Glazed, Flush Doors & Windows on drawing sheet by scale with long and cross section. | Theory-0.5 Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils Templates | Computer Lab |
| Prepare structural drawing of Flush and Wire Gauzed Doors & Windows | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan, sections & elevation flush door& window. - Draw detailed plan, sections & elevation wire gauzed door& window. - Perform lettering & dimensioning. - Perform printing for drawing | - The various parts of windows. - importance of windows - the different materials used for windows and ventilators - Sketch the elevations, sectional plans and vertical sections of windows and ventilators. - Parts of doors and CW Practical Activity Draw structural drawing of Wire Gauzed Windows & Doors on drawing sheet by scale with long and cross section. | Theory-0.5 Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils Templates | Computer Lab |

### 0732B&CE-74. Produce Drawings of Stairs and Stair Case.

Objective: This module covers the knowledge and skills required to draw sketches of open newel, straight, bifurcated, geometrical, circular, spiral, & dog-legged stair & Structural drawings of Dog-legged and Spiral Stair cases.

Duration:  11.0 Hours						Theory: 2.0 Hours				Practice: 9.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare sketches of stairs | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Sketch open newel, straight, bifurcated, geometrical, circular, spiral, & dog-legged stair. - Perform lettering & dimensioning. - Perform printing for drawing | - stair and stair case - The terms and parts used in different types of stairs. - The stairs according to their layout. - Suitability for different type of stairs. - Flight of stair. - pitch - Differentiate between tread and riser. - landing - Winder. Practical Activity Draw structural and detail drawings of Different type of stairs on drawing sheet. | Theory-1 Hrs Practice-3 Hrs Total- 4 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales (Card, Plane & Diagonal) Compass Protractor Stencils Templates | Computer Lab |
| Prepare structural drawing of Dog-legged Stair | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of dog legged stair. - Draw detailed plan & sections spiral stair case - Perform lettering & dimensioning. - Perform printing for drawing | - The stairs according to their materials. - Design procedure for straight stair. - Sketch the plans and sections of different types of stairs according to their layout. - Spiral Stair. - Dog legged stair. - Well in stair - quarter landing Practical Activity Draw structural and detail drawings of Dog-legged Stair Case by scale on drawing sheet. | Theory-0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales (Card, Plane & Diagonal) Compass Protractor Stencils Templates | Computer Lab |
| Prepare structural drawing of Spiral Stair Case. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of dog legged stair. - Draw detailed plan & sections spiral stair case - Perform lettering & dimensioning. - Perform printing for drawing | - The stairs according to their materials. - Design procedure for straight stair. - Sketch the plans and sections of different types of stairs according to their layout. - Spiral Stair. - Dog legged stair. - Well in stair - quarter landing Practical Activity - Draw structural and detail drawings of SPIRAL Stair Case by scale on drawing sheet. | Theory-0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales (Card, Plane & Diagonal) Compass Protractor Stencils Templates | Computer Lab |

### 0732B&CE-75. Produce Drawings of Water Tanks

Objective: This module covers the knowledge and skills required to draw masonry underground masonry water tank, RCC Circular Surface Water tank and Square RCC overhead water Tank.

Duration: 21.0 Hours							Theory: 3.0 Hours				Practice: 18.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare structural drawing of RCC surface circular water tank. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of masonry underground water tank. - Draw reinforcement detailed drawing. - Perform lettering & dimensioning. - Perform printing for drawing | - Water tank. - The terms and parts used in different types of water tanks. - Inlet & outlet for the tanks. - The suitability of each type of water tank. - design aspects circular water tank. Practical Activity Prepare structural drawing of RCC surface circular water tank. | Theory-1 Hrs Practice-4 Hrs Total- 5 Hrs | Drawing Graph Sheets Duster Pencils Eraser Sharpener Sand paper Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils Templates | Computer Lab |
| Prepare structural drawing of RCC square overhead water tank. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan of RCC surface water tank. - Draw detailed cross sections of RCC surface water tank. - Draw reinforcement detailed drawing. - Perform lettering and dimensioning. - Perform printing for drawing | - Water tank. - The terms and parts used in different types of water tanks. - The tanks according to their material, & level. - The suitability of overhead water tank. - reinforcement for water tank - Overflow pipe for tank. Practical Activity Prepare structural drawing of RCC square overhead water tank. | Theory-1 Hrs Practice-5 Hrs Total- 6 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils Templates | Computer Lab |
| Prepare structural drawing of RCC Circular overhead water tank. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of RCC overhead water tank. - Draw reinforcement detailed drawing. - Perform lettering and dimensioning. - Perform printing for drawing | - Sketch the plans and sections of different types of water tanks. - Reinforcement for circular water tank. - Formula for finding area when diameter is given. - Foundation for overhead tank. - Printing process for the drawing. Practical Activity Prepare structural drawing of RCC circular overhead water tank. | Theory-0.5Hrs Practice-5 Hrs Total- 5.5 Hrs | Drawing Papers Duster Pencils Eraser Sharpener Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils Templates Protector | Computer Lab |
| Prepare structural drawing of RCC Rectangular overhead water tank. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of RCC overhead water tank. - Draw reinforcement detailed drawing. - Perform lettering and dimensioning. - Perform printing for drawing | - Sketch the plans and sections of different types of water tanks. - Reinforcement for rectangular water tank. - Formula for finding area when diameter is given. - Foundation for overhead tank. - Printing process for the drawing. Practical Activity - Prepare structural drawing of RCC rectangular overhead water tank. | Theory-0.5Hrs Practice-4 Hrs Total- 4.5 Hrs | Paperss Duster Pencils Eraser Sharpener Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass Protractor Stencils Templates Protector | Computer Lab |

### 0732B&CE-76. Produce Drawings of Cross Drainage Structures.

Objective: This module covers the knowledge and skills required to draw detailed drawing of RCC Deck high level 2 spans bridge, Prepare detailed drawing of masonry segmental arched single span bridge and Prepare detailed drawing of steel plate girder single span bridge.

Duration: 8.0 Hours							Theory: 2.0 Hours				Practice: 6.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare detailed drawing of RCC Deck high level 2 spans bridge | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan, elevation & sections of RCC deck 2 span high level bridge. - Draw reinforcement detailed drawing. - Perform lettering & dimensioning. - Perform printing for drawing | - Bridge - Deck - cause way - Reinforcement for bridge. - lettering on sheet - causeway - span of bridge - Abutments - type of wing walls - Type of foundation for bridge. - Uses of reinforcement for bridges Practical Activity Prepare detailed drawing of RCC Deck high level 2 spans bridge | Theory0.5 Hrs Practice 1.5Hrs Total- 2.0 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales (Card, Plane & Diagonal) Compass Protractor Stencils Templates | Computer Lab |
| Prepare detailed drawing of masonry segmental arched single span bridge. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of Segmental Arch single span bridge. - Perform lettering & dimensioning. - Perform printing for drawing | - Sketch the X-section of concrete Road structure. - Various parts of culverts i.e. abutment, wing wall, toe wall parapet, base plate. - Sketch the Plan, Foundation Plan, Long Section and X-Section of Culvert. - The various terms used in Bridge. - Differentiate between culvert and Bridge. - The various types of bridges. Practical Activity Prepare detailed drawing of masonry segmental arched single span bridge on drawing sheet. | Theory0.5 Hrs Practice 2.5Hrs Total-3.0 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Scales (Card, Plane & Diagonal) Compass Protractor Stencils Templates | Computer Lab |
| Prepare detailed drawing of masonry segmental arched triple span bridge. | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of Segmental Arch triple span bridge. - Perform lettering & dimensioning. - Perform printing for drawing | - Plate girder bridges. - Foundation Plan, Long section and X-Section of two Span Bridge. - Sketch the detailed drawing of 25’ span masonry arch bridge. - deck for segmental arch bridge. Practical Activity Prepare detailed drawing masonry segmental arched triple span bridge on drawing sheet | Theory0.5 Hrs Practice 1.0Hrs Total- 1.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass | Computer Lab |
| Prepare detailed drawing of steel plate girder bridge for single span | Trainee will be able to: - Select the suitable instruments to draw foundation. - Distribute space of drawing sheet. - Draw detailed plan & sections of steel plate girder bridge for single span. - Draw reinforcement detailed drawing. - Perform lettering & dimensioning. - Perform printing for drawing | - Steel Plate girder bridges. - Foundation Plan, Long section and X-Section of single Span girder Bridge. - Sketch the detailed drawing of 25’ span plate girder steel bridge. - deck for plate girder bridge. Practical Activity Prepare detailed drawing steel plate girder bridge for single span | Theory0.5 Hrs Practice 1.0Hrs Total- 1.5 Hrs | Drawing Sheets Duster Pencils Eraser Sharpener Sand paper Graph Papers Stencil Paper Tools Drawing Board Tee Square Set Square French Curves Card Scales Compass | Computer Lab |

### 0732B&CE-77. Develop 3D Drawings

Objective: This module covers the knowledge and skills required to create 3- Dimensional Models by using various tools and commands in AutoCAD software, demonstrate skills to modify 3D objects and models to ensure civil technology requirements and present a rendered 3D Model final outcome.

Duration:  10.0 Hours							Theory: 4.0 Hours				Practice: 6.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Develop 3D Objects | Trainee will be able to: - Setup & save 3D Drawing Interface for required specifications - Setup 3D User Interface settings for required specifications - Create 3D Objects with given measurements - Perform printing for drawing | - Defining 3D Drawings - Importance of 3D Drawings - Types of 3D Drawings - Set up and interface for 3D Drawings - Creating 3D Drawings - Modifying 3D Drawings Practical Activity Develop 3D Objects as per given data. | Theory-1.5Hrs Practice-2 Hrs Total- 3.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Manipulate 3D Objects Using 3D Editing Tools | Trainee will be able to: - Modify 3D Objects in line with the requirements - Make customized 3D Models according to the requirements of the given job - Convert 3D Face Objects into Single Mesh Objects - Perform printing for drawing | - 3D Navigate Control- functions - Camera setting - Importance of scene creation. - PRESET views such as isometric, top, bottom, front, left, etc. - Modifying 3D Drawing - 3D Editing with editing tools - Describe Perspective projection and parallel projection - Describe walk and Constrained Orbit. Practical Activity Manipulate 3D Objects Using 3D Editing Tools | Theory-1.5Hrs Practice-2 Hrs Total- 3.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Render 3D Model | Trainee will be able to: - Apply material to required 3D Model as per given specifications - Apply lights to get the requisite scene of required 3D model - Assign cameras to execute different views of required3D Model - Render and print the 3D model according to required size and orientation - Perform printing for rendered model | - Defining Rendering - Rendering importance - Rendering command with their application - 3D model material specification - 3D model light requisition scene - Assigning camera with commands - 3D Model orientation Practical Activity Render 3D Model as given data. | Theory-1.0Hrs Practice-2 Hrs Total- 3.0 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse Data cables Printer USB Multimedia | Computer Lab |

### 0732B&CE-78. Produce Civil Technology Drawings

Objective: This module covers the knowledge and  skills required to Printout on A4 paper drawings of c-type residential building, Printout on A2 paper submission drawings of a small two storey R.C.C. framed structure building, Printout on A4 paper x-sections of canals, drains and roads and Printout on A4 paper drawings of masonry 10 ft segmental arched culvert.

Duration: 14.0 Hours							Theory: 3.0 Hours				Practice: 11.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Prepare & printout on A4 paper drawings of c-type residential building. | Trainee will be able to: - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Draw detailed plan of C type building. - Draw front & side elevation of C type building. - Draw x-section of C type building. - Adjust layouts in viewports for printing C type building. - Perform lettering & dimensioning | - Definition & importance for printing - Printing Paper size - Set up for printing - Interface required for printing the Auto CAD Drawing - User interface setting for printing - Defining view point and its adjustment - Printing command Practical Activity Prepare & printout on A4 paper drawings of c-type residential building. | Theory-1.0Hrs Practice-2 Hrs Total- 3.0 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tracing Paper Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Prepare & printout on A2 paper submission drawings of a small two storey R.C.C. framed structure building | Trainee will be able to: - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Draw detailed plans of framed structure building. - Draw front & side elevation of framed structure building. - Draw site plan, layout plan of framed structure. - Draw x-sections of framed structure building. - Draw reinforcement detail of RCC structures. - Adjust layouts in viewports for printing frame structure building. - Perform lettering & dimensioning - Print the drawings | - Drawing submission introduction - Importance of drawing submission - Setup interface setting - Importance for interface setting - Interface specifications - Plotting and printing the drawing on drawing sheet - Printout site layout plan - Print different x-section for building drawings. - Adjusting view point layout. Practical Activity Prepare & printout on paper submission drawings of a small two storey R.C.C. framed structure building along with their reinforcement. | Theory-0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tracing paper Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Prepare & printout on A4 paper x-sections of canals and drains | Trainee will be able to: - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Draw cross section of canal fully in cutting, fully in filling and partly in cutting & partly in filling. - Draw cross section of national highway dual carriage in rural and built up area. - Draw cross section of district highway double lane in rural and built up area. - Draw x-sections of type-I. type-II & type 8 drains - Adjust layouts in viewports for printing frame structure building. - Perform lettering & dimensioning - Print the drawings | - Setup interface setting - Importance for interface setting - Interface specifications - Print cutting and filling cross sections for canals and drains - Highway cross sections printing - Drains producing, importance and its printing - Adjust layout in viewport - Print frame structure drawings Practical Activity Prepare & printout on different size of paper x-sections of canals, drains and roads. | Theory-0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Prepare & printout on A4 paper x-sections of roads. | Trainee will be able to - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Draw cross section of canal fully in cutting, fully in filling and partly in cutting & partly in filling. - Draw cross section of national highway dual carriage in rural and built up area. - Draw cross section of district highway double lane in rural and built up area. - Draw x-sections of type-I. type-II & type 8 drains - Adjust layouts in viewports for printing cross section of road. - Perform lettering & dimensioning | - procedure of printing drawings. - Importance of drawings printing - Interface required specification - Roads x-sections, Plans and its printing Practical Activity Prepare & printout on different size paper, drawings of roads. | Theory-0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Prepare & printout on A4 paper drawings of masonry 10 ft segmental arched culvert. | Trainee will be able to - Setup drawing interface for required specifications - Setup user interface settings for required specifications - Draw detailed plan, & layout plan of culvert. - Draw long section & cross section of culvert. - Draw elevation of culvert. - Adjust layouts in viewports for printing segmental arch culvert. - Perform lettering & dimensioning - Print the drawings | - procedure of printing drawings. - Importance of drawings printing - Interface required specification - culverts x-sections, Plans and its printing Practical Activity - Prepare & printout on different size paper, drawings of culverts. | Theory-0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |

## 9.8. Construction I

### 0732B&CE-79. Perform Simple Levelling

Objective: This module covers the knowledge and  skills required to  Prepare for work, Perform manual leveling and Cleanup work area.

Duration: 3.5 Hours									Theory: 0.5 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work. | Trainee will be able to: - Requirements of the job are determined and various levels are obtained. - Work health and safety (WHS) and environmental requirements associated with levelling activities are adhered to throughout the work. - Quality assurance requirements are identified and adhered to according to workplace requirements. - Tools and equipment, including personal protective equipment, are selected and checked for serviceability. | - Workplace and equipment safety requirements - Steps to access relevant information, including codes and technical standards - SI system of measurements - Simple calculations relating to carrying out levelling - Practical activity: - Plan simple leveling of floor area in lab. | Theory- 0.16 Hrs Practical-1 Hrs Total- 1.16 Hrs | Pencil Ruler Calculator | Class Room and Construction lab |
| - Perform manual leveling. | Trainee must be able to - Identify height to be transferred from drawings, plans or instructions. - Carryout hand levelling and operate according to manufacturer instructions. - Levels are taken and marked according to job requirements. | - Different types of levelling equipment, their applications and their method of operation - Process of establishing, and checking levels and alignment - Job safety analysis (JSA) and safe work method statements (SWMS) Practical activity: - Perform manual leveling of lab floor. | Theory- 0.18 Hrs Practical-1.5 Hrs Total- 1.68 Hrs | Spirit level | Class room and Construction Lab |
| - Clean up work area | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices | - Explanation of need for store in a project - Classification of stores- stock - Practical activity: - Clean the area and tools and store it in proper storage area in lab. | Theory-0.16 Hrs Practical 0.5 Hrs Total- 0.66 Hrs | Broom Duster | Construction Lab |

### 0732B&CE-80. Lay out Building plans

Objective: This module covers the knowledge and  skills required to identify and select hand, power and pneumatic tools for given tasks and safely use and maintain a minimum of rule, tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock and crow bar and pinch bar for given tasks.

Duration: 5.5 Hours								Theory: 0.5 Hours				Practice: 5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plan and prepare | Trainee will be able to: - Work instructions, including plans, specifications, quality requirements and operational details, are obtained, confirmed and applied from relevant information for planning and preparation. - Safety (OHS) requirements are followed in accordance with safety plans and policies. - Signage and barricade requirements are identified and implemented. - Tools and equipment selected to carry out tasks are consistent with job requirements, checked for serviceability, and any faults are rectified or reported prior to commencement. - Material quantity requirements are calculated in accordance with plans, specifications and quality requirements. - Materials appropriate to the work application are identified, obtained, prepared, safely handled and located ready for use. - Environmental requirements are identified for the project in accordance with environmental plans and statutory and regulatory authority obligations, and are applied. | - Site and equipment safety (OHS) requirements - Job safety analysis (JSA) and safe work method statements - Construction plan, symbols and construction terminology - Processes for interpreting engineering drawings and sketches - Project quality requirements Practical activity: Interpret lay out of a 5 marla house and prepare materials and equipment. | Theory-0.07 Hrs Practical-0.71 Hrs Total- 0.78 Hrs | Pencil Eraser Notebook | Class Room |
| - Identify and indicate site boundaries. | Trainee will be able to: - Survey pegs at corners of site are located and identified in accordance with job drawings, specifications and site topography. - String lines are set accurately into position to identify site boundary markings in accordance with site plan and survey pegs. | - Application and requirements for line, level and plumb in construction projects - Basic construction processes Practical activity: Mark boundaries of given lay out in college ground. | Theory-0.07 Hrs Practical- 0.71 Hrs Total- 0.78 Hrs | Survey lab tools | Class Room and college ground |
| - Set out first line for building alignment. | Trainee must be able to: - Measurements of building line from boundary or existing building are determined from site drawings for setting out. - Approximate position and length of line, plus building clearance measurement at each end, are determined for hurdle location in accordance with site plan and survey pegs. - Pegs and hurdles/profiles are installed approximately level across and between one another with adequate provision to mark footing width on hurdle/profile in accordance with job drawings and specifications. - Location for line is accurately marked with nails on hurdles/profiles and line is set taut into position to true alignment with boundary in accordance with job drawings and specifications without error | - Steps of processes for setting out - Types, characteristics, technical capabilities and limitations of setting out devices. Practical activity: Set out lines for alignment in college ground. | Theory-0.07 Hrs Practical-0.71 Hrs Total- 0.78 Hrs | Survey lab tools | Class Room and college ground |
| - Set out right angled corners. | Trainee must be able to - Corner of building is determined on set building line to true measurement from adjacent boundary and marked with peg in accordance with job drawings and specifications. - Right angle is set up to line from corner peg using triangulation principles. - Hurdles/profiles are installed to approximate level of other hurdles and line is set taut to right angled alignment. | - Setting out techniques Practical activity: Set out right angled corners from corner peg using triangulation principles in college ground. | Theory-0.07 Hrs Practical-0.81 Hrs Total- 0.88 Hrs | Survey lab tools | Class Room and college ground |
| - Install other building lines. | Trainee must be able to - Hurdles for remaining building lines are installed to appropriate locations, approximately level with established hurdles in accordance with job drawings and specifications. - Measurements for remaining building lines are accurately marked and nailed on hurdles to dimensions from site drawings. - String lines are set taut into position to nailed locations on hurdles in accordance with job Class Room and college ground drawings and specifications. | - Site isolation and traffic control responsibilities and authorities Practical activity: Set string lines in position for remaining building lines in college ground. | Theory-0.07 Hrs Practical-0.61 Hrs Total- 0.68 Hrs | Survey lab tools | Class Room and college ground |
| - Building lines are checked for square | Trainee must be able to - Diagonal measurements are checked for square and lines are adjusted to provide square relationship within 5mm tolerance over minimum diagonal length of 15m. - Measurements are checked for accuracy. | - Basic mathematical techniques associated with setting out Practical activity: Check all building lines to provide a square relationship. | Theory-0.07 Hrs Practical-0.71 Hrs Total- 0.78 Hrs | Survey lab tools | Class Room and college ground |
| - Clean up | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices | - Explanation of need for store in a project - Classification of stores- stock Practical activity: Clean the area and tools and store it in proper storage area in lab. | Theory-0.07 Hrs Practical-0.71 Hrs Total- 0.78 Hrs |  |  |

### 0732B&CE-81. Carryout excavation and install supports

Objective: This module covers the knowledge and  skills required to excavate and install trench and excavation support on a new or existing site, in order to make provisions for footings/slabs or to provide/repair/divert services.

Duration: 6 Hours								Theory: 1 Hours				Practice: 5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plan and prepare | Trainee will be able to: - Work instructions are confirmed with supervisor. - Material quantity requirements are calculated in accordance with specifications. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared | - Steps to Interpret Plans, specifications and drawings - common skills and knowledge specified in the introduction to these RMCS Commonly used in-ground services - material quality requirement - calculation of material quantity Practical activity: Interpret excavation lay out of a 5 Marla house calculate quantities required. | Theory-0.125 Hrs Practical-0.625 Hrs Total- 0.75 Hrs | Pencil Eraser Notebook | Class Room |
| - Locate excavation site and erect safety equipment | Trainee will be able to: - Excavation site is located, and line and depth are established from site plans and instructions. - Service points and the excavation limits are set and marked. - Temporary drainage system is established to divert surface and sub-surface water. - Service markers are identified and interference with underground services avoided. | - Setting out techniques - Use of levelling equipment Practical activity: Mark boundaries of given lay out of 5 Marla house in college ground. | Theory-0.125 Hrs Practical-0.625 Hrs Total- 0.75 Hrs | Hoe Trowel Rake Mattock Spade shovel PPEs | Class Room and college ground |
| - Dig Excavations | Trainee must be able to: - Excavations are safely dug with hand tools to ensure correct route, line and depth. - Machine operator is assisted with excavation to ensure correct route, line and depth. - Trench or excavation support is installed if required. | - Description of excavation materials - Description of excavation techniques - Definition of excavation. - Types of excavation supports. Practical activity: Dig excavation on marked layout of 5 Marla house in college ground. | Theory-0.125 Hrs Practical-0.75 Hrs Total- 0.875 Hrs | Hoe Trowel Rake Mattock Spade shovel PPEs | Class Room and college ground |
| - Perform Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Purpose of maintaining and cleaning equipment and tools Practical activity: Clean the area and tools and store it in proper storage area in lab. | Theory-0.125 Hrs Practical-0.5 Hrs Total- 0.625Hrs | Cleaning tools | Class Room and college ground |
| - Plan and prepare for installing trench support | Trainee must be able to - Access, interpret and apply trench support documentation, and ensure the work activity is compliant - Obtain, read, interpret, clarify and confirm work requirements - Identify and address potential risks, hazards and environmental issues, and implement control measures - Select and wear personal protective equipment appropriate for work activities - Identify, obtain and implement traffic signage requirements - Select, and check for faults, equipment and/or attachments for work activities - Obtain and interpret emergency procedures, and be prepared for fire/accident/emergency | - Definition of shoring. - Description types and components of shoring. - Safety measures during excavation - Types of signage and barricades used during excavation activity Practical activity: Set out safety measures and use PPEs for installing trench support. | Theory-0.125 Hrs Practical-0.625 Hrs Total- 0.75 Hrs | Pencil Eraser Notebook | Class Room and college ground |
| - Install trench shoring | Trainee must be able to - Communicate with plant operator and ensure the excavation of trenches complies with site plan, line and depth - Determine and prepare shoring method - Set out positioning of shoring - Position/erect shoring within the trench - Secure shoring in position and ensure structural conformity with regulations - Clean out excavation - Locate ladders for safe access and egress | - Procedure to install shoring. - Shoring elements. - Practical activity: Install shoring elements in designed excavation in lab/college ground. | Theory-0.125 Hrs Practical-0.625 Hrs Total- 0.75 Hrs | Battens Props planks sole plate | Class Room and college ground |
| - Remove trench shoring | Trainee must be able to - Release jacking mechanisms and remove ladders - Check shoring and prepare it for lifting from the trench - Remove shoring from trench and store it | - Explanation of need of shoring - Explanation jacking. - Procedure to remove jacking Practical activity: Remove the shoring elements from trench in lab / college ground. | Theory-0.125 Hrs Practical-0.625 Hrs Total- 0.75 Hrs | Battens Props planks sole plate | Class Room and college ground |
| - Conduct housekeeping activities | Trainee must be able to - Clear work area and dispose of or recycle materials - Clean and maintain condition of equipment, ensure suitability for use, and address/report issues - Manage/report hazards, and maintain a safe working environment - Process written records | - Understanding of equipment’s usage and maintenance Practical activity: Clean the area and tools and store it in proper storage area in lab. | Theory-0.125 Hrs Practical-0.625 Hrs Total- 0.75 Hrs | Cleaning tools | College ground |

### 0732B&CE-082. Place Concrete in Foundations

Objective: This module covers the knowledge and  skills required to identify and select hand, power and pneumatic tools for given tasks and safely use and maintain a minimum of rule, tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock and crow bar and pinch bar for given tasks.

Duration: 3.5 Hours									Theory: 0.5 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plan and prepare | Trainee will be able to: - Work instructions are confirmed with supervisor. - Material quantity requirements are calculated in accordance with specifications. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared | - Concrete ingredients ratio - Concrete materials - Batching techniques Practical activity: Calculate the tools and materials required in concrete placing and plan the activity. | Theory-0.125 Hrs Practical-1 Hrs Total- 1.125 Hrs | Pencil Eraser Notebook | Class Room |
| - Define and prepare work area | Trainee will be able to: - Concrete is received, checked for debris and discharged into wheelbarrow, kibble, pump or hopper. - Location of concrete placement is determined from plans and specifications and is checked to be free of debris and waste. | - Requirements of area where concrete is place - Concrete setting time Practical activity: Identify any water tight surface in lab | Theory-0.125 Hrs Practical-0.5 Hrs Total- 0.65 Hrs | Water tight platform / sheet | Class Room and Construction lab |
| - Place, screed and level concrete | Trainee must be able to: - Concrete is placed in horizontal layers into location to mark out levels. - Height of vertical drop of concrete is minimized to avoid - Segregation of concrete materials. - Poured concrete is consolidated during process using compaction or vibration method. - Finished levels are checked using appropriate levelling device. - Concrete is screened to correct levels and grades using appropriate straight edged tool/formwork-mounted screed. | - Finishing techniques of concrete and tools used - What is Concrete segregation - Vibrators types - Compaction requirement in concrete - Types, characteristics, uses and limitations of plant, tools and equipment Practical activity: pPace concrete in specified area of lab and apply levelling techniques | Theory-0.125 Hrs Practical-1 Hrs Total- 1.125 Hrs | Chutes Line pumps Measuring tapes and rules, mechanized dumpers Rakes Screed boards shovels, stipple devices Trowels /troweling machines Vibrators Wheelbarrows | Class Room and Construction lab |
| - Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - What is the purpose of maintaining and cleaning equipment and tools? Practical activity: Set out right angled corners from corner peg using triangulation principles in college ground. | Theory-0.125 Hrs Practical-0.5 Hrs Total- 0.65 Hrs | Cleaning tools | Class Room and Construction lab |

### 0732B&CE-083. Prepare for construction Process (Brick/ block laying)

Objective: This module covers the knowledge and  skills required to to Plan work, Set out brickwork/ blockwork, Lay bricks/blocks forming steps and wing walls and Clean up

Duration: 6.5 Hours								Theory: 0.5 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plan work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Material quantity requirements are calculated in accordance with specifications. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared | - Definition of masonry work. - Types of brick masonry. - Understanding of Plans, specifications and drawings Practical activity: Calculate the materials and tools required in brick /block masonry work. | Theory-0.125 Hrs Practical-1.5 Hrs Total- 1.625 Hrs | Pencil Eraser Notebook | Class Room |
| - Set out brickwork/ blockwork | Trainee will be able to: - Work platform is erected in accordance with workplace requirements. - Location and relative level of prepared footing are checked from job drawings and specifications. - Rise and going of flight and individual steps are calculated to codes and workplace requirements. - Flight and individual steps are set out from calculations and job drawings. | - Characteristics and applications of materials for masonry steps and stairs construction - Description of the types Stairs. - Explanation of elements of stairs. Practical activity: Set out brickwork/ blockwork in construction lab. | Theory-0.125 Hrs Practical- 2 Hrs Total- 2.125 Hrs |  | Class Room and Construction lab |
| - Mix concrete | Trainee must be able to: - Mortar mix is prepared to required conformity and bricks/blocks are laid to set out according to specifications. - Base brickwork is constructed to specifications. - Steps are formed square, level, plumb and true and laid to specified bond. - Profile of steps is constructed to bond and design, aligned and plumb to specifications. - Parallel wing walls are formed to step alignment in accordance with specifications. - Jointing is carried out to work specifications. - Brickwork/block work is laid to line, set out with gauge and completed to work specifications. - Brick/block faces are cleaned free of mortar. | - Techniques of masonry steps and stairs construction - Procedure of Horizontal and vertical Leveling - Definition of wing wall Practical activity: Mix concrete in construction lab. | Theory-0.125 Hrs Practical-1.5 Hrs Total- 1.625 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressor spirit levels, straight edges string lines, Trowels, Wheel barrows | Class Room and Construction lab |
| - Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Purpose of maintaining and cleaning equipment and tools. Practical activity: Clean the area and maintain equipment’s | Theory-0.125 Hrs Practical-1 Hrs Total- 1.125 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-084. Mix Cementitious materials (Mortar & Concrete)

Objective: This module covers the knowledge and  skills required to Plan work, mix mortar, mix concrete and Clean up

Duration: 4 Hours									Theory: 1 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plan work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Material quantity requirements are calculated in accordance with specifications. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared | - Definition of mortar. - Types of mortar - Ingredients in mortar - Definition of Concrete - Types of concrete. - Ingredients in concrete Practical activity: Calculate the materials and tools required in mortar and concrete | Theory- 0.25 Hrs Practical-0.95 Hrs Total- 1.2 Hrs | Pencil Eraser Notebook | Class Room |
| - Mix mortar | Trainee will be able to: - Work platform is erected in accordance with workplace requirements. - Location and relative level of prepared. - Mix dry ingredients according to standards - Add water to desired consistency in 3 steps and mix well - Apply the mortar. | - Steps to mix mortar. - W/c requirement - Explanation of different characteristics of mortar. Practical activity: Mix mortar. | Theory- 0.25 Hrs Practical-0.55 Hrs Total- 0.8 Hrs | Water tight platform / sheet brooms, buckets, Concrete mixers, Dumpy levels, hoses, shovels, small petrol or diesel engines spirit levels, straight edges string lines, Trowels, Wheelbarrows | Class Room and construction lab |
| - Mix concrete. | Trainee must be able to: - Work platform is erected in accordance with workplace requirements. - Location and relative level of prepared. - Mix dry ingredients according to standards - Add water to desired consistency in 3 steps and mix well - Apply the concrete. | - Techniques of mixing concrete - Definition of concrete. Practical activity: Mix concrete in construction lab | Theory- 0.25 Hrs Practical-0.75 Hrs Total- 1 Hrs | brooms, buckets, Concrete mixers, Dumpy levels, hoses, shovels, small petrol or diesel engines spirit levels, straight edges string lines, Trowels, Wheelbarrows | Class Room and construction lab |
| - Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Explanation of purpose of maintaining and cleaning equipment and tools. Practical activity: Clean up. | Theory- 0.25 Hrs Practical-0.75 Hrs Total- 1 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-085. Install flashing and damp-proof course

Objective: This module covers the knowledge and  skills required to required install flashings and damp proofing products to different types and styles of buildings.

Duration: 4 Hours								Theory: 1 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plan work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Material quantity requirements are calculated in accordance with specifications. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared | - Definition of waterproofing - Description of waterproofing materials. - Definition of DPC. Practical activity: Calculate the materials and tools required in installing damp proof course | Theory-0.2 Hrs Practical-0.6 Hrs Total- 0.8 Hrs | Pencil Eraser Notebook | Class Room |
| - Prepare for work. | Trainee will be able to: - Type of flashing and damp proofing material are identified in accordance with and state of structure. - Area of structure for damp proofing is identified from or site inspection and inspected for defects. - Defects are corrected and surface preparation of structure requiring damp proofing is carried out to manufacturer specifications | - Characteristics and applications of materials to install flashings and DPC - Types Stairs - Explanation of elements of stairs - Explanation of surface defects. - Surface preparation for DPC. Practical activity: Mark boundaries of drawn lay out in college ground. | Theory-0.2 Hrs Practical-0.6 Hrs Total- 0.8 Hrs | Water tight platform / sheet | Class Room and construction lab |
| - Install damp proof course | Trainee must be able to: - Damp proof is installed in accordance with work specifications. - Flashing or damp-proof course is laid and lapped in accordance with specifications. - Damp proof material is applied with a consistent mortar bed on top and bottom. - Damp proof course is folded to follow shape of surrounding structures | - Procedure of Laying of DPC. Practical activity: Set out lines for alignment in college ground. | Theory-0.2 Hrs Practical-0.6 Hrs Total- 0.8 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressors spirit levels, straight edges string lines, Trowels Wheel barrows | Class Room and construction lab |
| - Install moisture proof barrier or flashings | Trainee must be able to: - Flashing materials are prepared for application to surrounding structures in accordance with requirements. - Flashing material is laid, lapped and joined to follow shape of surrounding structure. - Flashings or moisture barrier material is formed and sealed around openings. - Flashing and damp-proof course are installed to project outside of mortar joint on external surface, and outside of brickwork in accordance with specifications. | - Definition and purpose of flashing Practical activity: Set out right angled corners from corner peg using triangulation principles in college ground. | Theory-0.2 Hrs Practical-0.6 Hrs Total- 0.8 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressor spirit levels, straight edges string lines, Trowels, Wheelbarrows | Class room and construction lab |
| - Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Purpose of maintaining and cleaning equipment and tools. Practical activity: Set out right angled corners from corner peg using triangulation principles in college ground. | Theory-0.2 Hrs Practical-0.6 Hrs Total- 0.8 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-086. Carry out brick work

Objective: This module covers the knowledge and  skills required to lay and construct brick and masonry buildings.

Duration: 6.5 Hours									Theory: 0.5 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plan work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Material quantity requirements are calculated in accordance with specifications. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared | - Definition of veneer construction - Explanation of materials used in veneer construction. Practical activity: Calculate the materials and tools required in veneer construction. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs | Pencil Eraser Notebook | Class Room |
| - Set out brickwork/ block work. | Trainee will be able to: - Bricks/blocks are identified, selected and checked for conformity with specifications. - Work platform is erected in accordance with workplace requirements. - Location and structural details of brickwork/block work are identified from plans. - Base brickwork construction, below and above floor is set out to location, dimensions and specifications. | - Characteristics and applications of materials for veneer construction. - Explanation of types brick bond pattern mostly used. Practical activity: Lay brickwork/ block work for a straight wall in lab. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs | Water tight platform / sheet | Class Room and construction lab |
| - Install damp proof course | Trainee must be able to: - Mortar mix is prepared and checked for conformity and bricks/blocks laid to set out to specification. - Brickwork/blockwork gauge is determined and set out rod is prepared. - Base brickwork/blockwork is constructed for veneer construction to requirements of regulations and specifications. - Sub-floor ventilation is installed in accordance with specifications. | - Description of Surface requirement for placing DPC. - Explanation of Materials used as DPC. Practical activity: Install DPC above brick wall in lab. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressor spirit levels, straight edges string lines, Trowels, Wheelbarrows | Class Room and construction lab |
| - Construct Veneer Walls | Trainee will be able to: - Structural frame is checked to ensure it is ready for brick or block veneer construction maintaining minimum cavity. - Damp proof courses are installed to specifications. - Ventilation for veneer construction is built to specifications. - Wall ties are positioned and correctly fixed to framework to specifications. - Openings are constructed and flashings installed. - Cavities are kept clear of mortar droppings and bridging. - Lintels are installed. - Top brickwork/ block work is constructed to eaves/gable level in accordance with standards. - Veneer gable is constructed as required by plans and specifications. - Walls are built to gauge straight and true in plumb, line and level within standards tolerance. - Control joints are formed in accordance with locations on job drawings and specifications. - Weep holes, brick/block reinforcing and wall flashing are located and built in to. - Sill bricks are cut where required and laid to line in accordance with work specifications. | - Explanation of elements of veneer wall - Description of wall ties - Description of purpose of lintel - Definition of weep holes - Control joint purpose and design. - Definition of dry wall - Definition of sheathing Practical activity: Construct a veneer wall, also provide an opening in it in lab area. | Theory-0.085 Hrs Practical-1.5 Hrs Total- 1.585 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressors spirit levels, straight edges string lines, Trowels, Wheelbarrows | Class Room and construction lab |
| - Rake/ rule joints and clean face. | Trainee will be able to: - Joints of laid brickwork/block work are raked or ruled to correct depth and profile in accordance with work specifications. - Brickwork/block work is brushed down prior to drying to remove unwanted mortar and face is cleaned. - Excess mortar is removed from brick/block work surfaces and cavities are cleaned free of mortar and debris. | - Explanation of types of brick joints - Definition of raking of joints. - Types of raking. Practical activity: Rake the joints in brick wall in lab. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs |  | Class Room and construction lab |
| - Clean up | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Explanation of purpose of maintaining and cleaning equipment and tools. Practical activity: Set out right angled corners from corner peg using triangulation principles in college ground. | Theory-0.083 Hrs Practical-0.5 Hrs Total- 0.583 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-087. Construct Cavity Wall

Objective: This module covers the knowledge and  skills required to prepare for work, set out brickwork/ blockwork, construct base of brickwork/ block work, position door and window frames, construct brick walls, rake or rule joints, and clean up.

Duration: 6.5 Hours								Theory: 0.5 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work | Trainee will be able to: - Work instructions, including plans, specifications, quality requirements and operational details are obtained from relevant information, confirmed and applied to determine bricklaying and block laying tasks. - Safety (OHS) requirements are followed in accordance with safety plans and policies. - Signage and barricade requirements are identified and implemented. - Plant, tools and equipment selected to carry out tasks are consistent with job requirements, checked for serviceability, and any faults are rectified or reported prior to commencement. - Material quantity requirements are calculated in accordance with plans, specifications and quality requirements. - Materials appropriate to the work application, including required fire resistance rating, are identified, obtained, prepared, safely handled and located ready for use. - Environmental requirements are identified for the project in accordance with environmental plans and statutory and regulatory authority obligations, and are applied. | - Definition of cavity wall - Explanation of elements of cavity wall construction. - Steps of processes for the calculation of material requirements, quality requirements Practical activity: Calculate the materials and tools required in cavity construction. | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Pencil Eraser Notebook | Class Room |
| - Set out brickwork/ block work. | Trainee will be able to: - Bricks/blocks are identified, selected and checked for conformity with specifications - Work platform is erected in accordance with workplace requirements. - Location and structural details of brickwork/block work are identified from plans. - Base brickwork construction, below and above floor is set out to location, dimensions and specifications. - Load bearing brickwork, including engaged piers, dwarf walls, isolated piers and corbelling are set out to job drawings and specifications. - Cavity brick wall is set out to requirements of job drawings. | - Characteristics and techniques of cavity wall construction. Practical activity: Lay brickwork/ block work for cavity wall of in straight line in lab. | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressor spirit levels, straight edges string lines, Trowels Wheel barrows | Class Room and construction lab |
| - Construct Base of brickwork/ block work. | Trainee must be able to: - Mortar mix is prepared and checked for conformity and bricks/blocks laid to set out to specification. - Brickwork/block work gauge is determined and set out rod is prepared. - Base brickwork/block work is constructed for cavity construction to requirements of regulations and specifications. | - What is Surface requirement for placing DPC. - What are the Materials used as DPC. - Techniques of cavity brick construction, including: - anti-termite measures, closing of cavities and capping systems, damp proofing, floor, wall and roof members, gable and eaves construction, lintels and load bearing components, stepped and level flashing for parapets and gables, sub-floor construction, tying components, ventilation, vermin control Practical activity: Construct a base of brick work for cavity wall in lab. | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet Trolleys, Plumb Rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressor spirit levels, straight edges string lines, Trowel Wheel barrows | Class Room and construction lab |
| - Position door and window frames. | Trainee will be able to: - Window frames are located and built in to cavity walls to specification, and are protected from mortar droppings during construction. - Door jambs are located, built in and fixed to cavity walls and single leaf walls according to job drawings and specifications | - Explanation of types of door - Explanation of elements of door and door frame. - Procedure to install door and window in cavity wall - Description of purpose of cavity wall Practical activity: Install door in cavity wall construction in lab. | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs |  | Class Room and construction lab |
| - Construct brick walls. | Trainee will be able to: - Brick or block cavity construction walls are constructed to job specifications and standards. - Damp proof courses and flashings are laid/built in to job specifications. - Ventilation for solid brick construction is built to requirements of job specification and regulations. - Walls are to be straight and true in plumb, line and level within standard tolerances. - Wall ties are positioned to comply with standards. - Openings are constructed and flashing is installed to job specifications. - Lintels are installed to job specifications. - Control joints are formed in accordance with locations on job drawings and standards. - Weep holes, brick reinforcing, vermin proofing and wall flashings are located and built in to job specifications. - Gables and parapets are constructed by plans and specifications. - Sill bricks are cut and laid to line in accordance with job specifications. - Tie down and lateral support systems for ceiling/roof structures are installed to walls in accordance with plans, specifications and standards. | - Gable and parapet. - Lateral support system for ceiling. - Procedure for hz and vt leveling of cavity wall - Mortar and c concrete requirements. Practical activity: Provide wall, ties DPC, lintel, control joints, weep holes and levelling of cavity wall in lab. | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, Masonry squares Hoists, Measuring tapes and rules, Mortar Boards Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressor spirit levels, straight edges string lines, Trowels Wheelbarrows | Class Room and construction lab |
| - Rake or rule joints | Trainee will be able to: - Joints of laid brickwork/block work are raked or ruled to correct depth and profile in accordance with job specifications. - Brickwork/block work is brushed down prior to drying to remove unwanted mortar. - Excess mortar is removed from brick/block work surfaces and cavities are cleaned free of mortar and debris in accordance with manufacturer recommendations, job specifications and standards. | - Procedure for raking the joints. Practical activity Rake the joints in brick cavity wall in lab. | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Raking tools |  |
| - Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Description of purpose of maintaining and cleaning equipment and tools. Practical activity: Clean the area and check tools and equipment | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-088. Construct Curved Wall

Objective: This module covers the knowledge and  skills required to Plan work, Set out curve for construction of curved masonry wall, Lay curved wall, Finish Joints and Clean up

Duration: 7 Hours									Theory: 1 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. | - Cavity wall - Elements of cavity wall construction. - Processes for the calculation of material requirements, quality requirements Practical activity: Calculate the materials and tools required in curved wall construction. | Theory-0.2 Hrs Practical-1.2 Hrs Total- 1.4 Hrs | Pencil Eraser Notebook | Class Room |
| - Set out curve for construction of curved masonry wall | Trainee will be able to: - Planned curve points are plotted from job drawings and all trammel centers are established on footing slab. - Curve of wall is planned to specified location from trammel or plotted points and marked on the footing slab. | - Geometric calculation Practical activity: Plot curved line on floor in construction lab. | Theory-0.2 Hrs Practical-1.2 Hrs Total- 1.4 Hrs | Measuring tape | Class Room and construction lab |
| - Lay curved wall. | Trainee must be able to: - Mortar is mixed to specifications. - Bricks/blocks are laid for first course to planned set out for line and specified bond according to specifications. - Gauge is maintained within standard tolerance at every course level. - Vertical face alignment is maintained. - Bricks/blocks are laid level over the length of the wall to the established plan profile. - Bricks/blocks are laid to specified bond with perpendicular joints (perpends) maintained in a vertical line - Construction is completed to requirements of job drawings and specifications | - Procedure of laying curved wall - Types of bricks (w.r.t cutting) Practical activity: Construct a curved wall on a planned curved line in construction lab. | Theory-0.2 Hrs Practical-1.9 Hrs Total- 2.1 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressor spirit levels, straight edges string lines, Trowels Wheelbarrows | Class Room and construction lab |
| - Finish Joints. | Trainee will be able to: - Excess mortar is removed from brick/ blockwork surfaces in accordance with work specifications. - Joints of laid brickwork/blockwork are raked or ruled to correct profile and depth to. - Brickwork/blockwork is brushed down prior to drying. | - Finishing techniques of brick wall Practical activity: Rake / rule the brick wall. | Theory-0.2 Hrs Practical-1.2 Hrs Total- 1.4 Hrs | Wire brush Raking tools Masonry square Trowels Duster Broom | Class Room and construction lab |
| - Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Description of purpose of maintaining and cleaning equipment and tools. Practical activity: Clean the area and check tools and equipment | Theory-0.2 Hrs Practical-0.5 Hrs Total- 0.7 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-089. Construct Masonry arches (semi-circular and segmental)

Objective: This module covers the knowledge and  skills required to Plan work, Set out first course for masonry arch, Prepare for arch construction, Construct arch center, Set up arch center, Cut and lay brick/ block to arch, Finish Joints and Clean up

Duration: 7 Hours								Theory: 1 Hours				Practice: 6Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Select Tools and equipment as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. | - Definition of Arches - Explanation of arch types - Processes for the calculation of material requirements, quality requirements Practical activity: Calculate the materials and tools required in arch construction | Theory-0.125 Hrs Practical-0.75 Hrs Total-0.875 Hrs | Pencil Eraser Notebook | Class Room |
| - Set out first course for masonry arch | Trainee will be able to: - Location and line of brickwork/blockwork wall are set out on concrete footing/slab to job drawings. - Pan of arch is determined from prepared allotted arch centre plus 4mm. - Arch spans are set out to location for first course. | - Understanding of of Geometric calculation drawing - Explanation of Elements of arch Practical activity: Set out first course for masonry arch in construction lab. | Theory-0.125 Hrs Practical-0.75 Hrs Total-0.875 Hrs | Measuring tape | Class Room and construction lab |
| - Prepare for arch construction. | Trainee must be able to: - Mortar mix is prepared and bricks/blocks are laid to form wall to set out. - Gauge of abutting walls is maintained within standard tolerance at each course level. - Plumb and alignment of vertical wall face are maintained. - Bricks are cut and laid level and to line over length of wall. - Abutment jambs/piers are laid vertical up to springing line. - Bricks/blocks are laid in stretcher bond to springing line of arch with perpendicular joints maintained in vertical line. | - Procedure of laying curved wall - Types of bricks (w.r.t cutting) - Alignment techniques Practical activity: Construct a curved wall on a planned curved line in construction lab. | Theory-0.125 Hrs Practical-0.75 Hrs Total-0.875 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressor Wheelbarrows | Class Room and construction lab |
| - Construct arch center. | Trainee must be able to: - Arch center is set out and curve is drawn up in accordance with specifications and plan. - Plan is transferred to material and cut to shape. | - Procedure to set center of arch Practical activity Construct arch center in lab | Theory-0.125 Hrs Practical-0.75 Hrs Total-0.875 Hrs |  | Class room and construction lab |
| - Set up arch center | Trainee must be able to: - Height to springing line is determined and height to crown of arch is confirmed to be within standard tolerance. - Height of toms and wedges or adjustable metal props are determined to set up and support timber arch center. - Supports are adjusted to ensure arch center is level at right angles to wall face and level across springing line. - Props, toms, packers and wedges are located for easy removal. - Position of central key brick/block is established for gauged arch and tape used to mark gauge | - Procedure to set formwork of arch - Procedure to Cutting of brick /blocks to designed shape - PPEs to work at height - Description of elements of formwork - Supporting elements of arch Practical activity Set up arch center | Theory-0.125 Hrs Practical-0.75 Hrs Total-0.875 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets, Concrete mixers, Dumpy levels, Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressor Wheelbarrows Required formwork elements | Class room and construction lab |
| - Cut and lay brick/ block to arch. | Trainee must be able to: - Bricks and blocks are cut and laid on center to form arch to specifications. - Joints are maintained to equal size and parallel on the extrados of an arch. - Same size wedge shape is maintained on face. - Centre line of key brick/block wedge is maintained through vertical center line of arch. - Even joint thickness is maintained around extrados for cut brickwork and blockwork. - All bricks are cut and laid to maintain even joints. - All joints are struck evenly to depth and shape to specifications. | - Understanding of drawing specification and interpretation - Procedure to check hz and vt levels Practical activity Cut and lay brick/ block to arch. | Theory-0.125 Hrs Practical-0.75 Hrs Total-0.875 Hrs | Bolsters, brick buggies and brick grabs, brooms, buckets Elevators, Forklifts, Hammers (bricks, club and scotch), hoses, Jig saws, jointing tools, Line blocks and line pins masonry saws, masonry squares, hoists, measuring tapes and rules, mortar boards, Pallet trolleys, Plumb rules, profiles Scaffolds, shovels, small petrol or diesel engines and compressor Wheelbarrows |  |
| - Finish Joints. | Trainee must be able to: - Excess mortar is removed from brick/block work surfaces in accordance with work specifications. - Joints of laid brickwork/block work are raked or ruled to correct profile and depth to. - Brickwork/block work is brushed down prior to drying. | - Finishing techniques of brick masonry work - Procedure of raking brickwork face of arch Practical activity: Rake / rule the brick arch wall in lab | Theory-0.125 Hrs Practical-0.75 Hrs Total-0.875 Hrs | straight edges string lines, Trowels Broom Raking tools Margin or raking tools | Class Room and construction lab |
| - Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Description of purpose of maintaining and cleaning equipment and tools. Practical activity: Clean the area and check tools and equipment | Theory-0.125 Hrs Practical-0.75 Hrs Total-0.875 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-090. Construct Scaffolding

Objective: This module covers the knowledge and  skills required to erect and dismantle a range of modular scaffolding systems to provide work platforms for construction

Duration: 6.5 Hours							Theory: 0.5 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. | - Definition of scaffolding. - Explanation of scaffolding types - Processes for the calculation of material requirements, quality requirements Practical activity: Calculate the materials and tools required in arch construction | Theory-0.083 Hrs Practical-1.5 Hrs Total- 1.583 Hrs | Pencil Eraser Notebook | Class Room |
| - Whip, tie, splice and inspect ropes | Trainee will be able to: - Ropes and cords are inspected for damage and wear. - Designated rope ends are whipped and spliced in accordance with regulations and project specifications. - Bends and hitches are applied and inspected in accordance with project specifications. | - Explanation of Elements of scaffolding - Understanding of specification of scaffolding elements Practical activity: Whip, tie, splice and inspect ropes in lab. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs |  | Class Room and construction lab |
| - Erect scaffolding | Trainee must be able to: - Purpose for scaffolding is confirmed and associated work tasks are identified. - Expected loading on scaffold and supporting structure is determined using load tables and manufacturer specifications. - Scaffolding and components are selected and inspected, and damaged components are labelled and rejected. - Sole board/base plate is selected in accordance with regulations, legislation, codes of practice and manufacturer specifications. - Scaffolding is set out and erected in accordance with regulatory and manufacturer requirements. - Static lines are erected and installed where specified in accordance with regulatory requirements. - Lifting device is assembled and erected where specified. | - Procedure of setting scaffolding - Alignment techniques - Definition of lifting devices. - Procedure of using lifting devices - Safety precaution to Erect scaffolding - PPEs Practical activity: Erect scaffolding in lab. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs | Adjustable base plates, Bends and hitches, box spanners, braces, bracket scaffolds (tank and formwork) Cantilevered hoists (materials only with maximum capacity of 500kg) Couplers and accessories Fiber ropes Gin wheels, guard rails Hammers Ledgers Mesh guards, mid rails, modular and prefabricated scaffolds Podgers hammers, prefabricated components Safety nets, scaffold belts, scaffolding planks shovels, spanners, spirit levels Stairs or ladders, standards, static lines, steel and aluminum tubes Tape measures, torpedo levels, transoms, Wire nips, wrenches. | Class Room and construction lab |
| - Inspect, repair and alter erected scaffolding | Trainee must be able to: - Erected modular scaffolding is inspected for damage, corrosion, wear and compatibility. - Current use of scaffolding is checked against original design and is in accordance with regulations and specifications - Scaffolding stability is inspected and confirmed. - Alteration or repair is carried out where specified. - Inspection log and handover is completed and dated, ready for signing by a certified scaffolder. | - Procedure to inspect erected scaffolding - Role of scaffolder - Repair procedure - Defects in scaffolding elements - Defects in scaffolding platform Practical activity Inspect, repair and alter erected scaffolding | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs |  | Class room and construction lab |
| - Dismantle scaffolding | Trainee must be able to: - Scaffolding is isolated and appropriately signed and barricaded to ensure safe dismantling. - Scaffolding is dismantled using reverse procedure as for erection. | - Procedure to dismantle scaffolding - Procedure to set signs and barricade - PPEs to work at height - Safety precaution for Dismantle scaffolding Practical activity Dismantle scaffolding in lab. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs | Dismantling tools | Class room and construction lab |
| - Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Description of purpose of maintaining and cleaning equipment and tools. Practical activity: Clean the area and check tools and equipment | Theory-0.083 Hrs Practical-0.5 Hrs Total- 0.583 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-091. Install doors and window frames

Objective: This module covers the knowledge and  skills required to set out and install timber and metal window and door units of different types, with appropriate weather proofing, smooth access and good security.

Duration: 6.5 Hours							Theory: 0.5 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. | - Definition of door and window - Explanation of types of doors and windows. - Processes for the calculation of material requirements, quality requirements Practical activity: Calculate the materials and tools required for window/door installation in lab. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs | Pencil Eraser Notebook | Class Room |
| - Install window units to frame | Trainee will be able to: - Window opening size is checked to be greater than overall window frame. - Reveals are joined and fixed securely to frames where specified. - Window unit is located to suit brickwork and eaves finish for veneer construction, whichever is applicable. - Window unit is positioned in place so that head/sill is level and stiles are plumb and in wind, ensuring reveals or frame are finished flush with face of inside wall lining. - Window is packed and fixed to wall frame through/to studs, in accordance with specified fixing and fastening methods. | - Explanation of Elements of door and windows - Understanding of specification of door and window elements - Process of installation of door/ window - Process of setting out Practical activity: Install window units to frame in lab. | Theory-0.083 Hrs Practical-1.5 Hrs Total- 1.583 Hrs | Bevels, Chisels Drills, Hammers, hand/power saws, marking equipment, measuring tapes and rules, Nail guns, Protractors Spirit levels, squares (combination/tri), steel squares and fences, string lines Barrel bolts Cabin hooks closers Dead bolts Flash bolts, flashings Handles hinges (butt and parliament) Latches, locks | Class Room and construction lab |
| - Replace window units/door frames | Trainee must be able to: - Architraves and nosing are removed. - Sill bricks or cladding are removed where specified. - Fasteners are cut, packing removed and flashing detached from frame. - Window unit/ doorframe is removed. - Window unit/door frame and window components are installed to plumb, level and wind | - Procedure of Window and door replacement techniques - Safety precaution to work at site - PPEs Practical activity: Replace window units/door frames in lab. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs | Bevels Chisels Drills Hammers, hand/power saws Marking equipment, measuring tapes and rules Nail guns Protractors Spirit levels, squares (combination/tri), steel squares and fences, string lines Barrel bolts Cabin hooks closers Dead bolts Flash bolts, flashings Handles hinges (butt and parliament) Latches, locks | Class Room and construction lab |
| - Architraves and moldings are replaced | Trainee must be able to: - Prepare door opening, and construct and fix jamb. - Door frame opening size is checked to be greater than the overall door jamb width and height, allowing for plumbing of stiles, thickness of floor covering, levelling of door head and level of floor. - Jamb stiles are marked and cut to length allowing for clearances according to specifications. - Head is trenched to accommodate jamb stiles allowing for clearance according to specification. - Jamb frame is assembled, squared and braced with rebates flush. - Joints and rebates are cleaned and finished. | - Procedure to inspect door and window frame - Defects in alignments Practical activity Architraves and moldings are replaced in lab. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs | Bevels Chisels Drills Hammers, hand/power saws Marking equipment, measuring tapes and rules Nail guns Protractors Spirit levels, squares (combination/tri), steel squares and fences, string lines Barrel bolts Cabin hooks closers Dead bolts Flash bolts, flashings Handles hinges (butt and parliament) Latches, locks | Class room and construction lab |
| - Install door and door unit | Trainee must be able to: - Door unit is positioned in place so that the head/sill is level and stiles are plumb and in wind, ensuring reveals or frame finished flush with face of inside wall lining. - Door is fitted to jamb allowing for clearances according to specifications with lock stile door backed off facilitating correct operation. - Hinges are marked out on door and jamb. - Hinges are fitted to door and jamb. - Final adjustments of door are made. - Door furniture components are fitted and fixed to manufacturer specifications | - Procedure to dismantle scaffolding - Procedure to install door - Types of fasteners - Fastening methods Practical activity Install door and door unit in lab. | Theory-0.083 Hrs Practical-1 Hrs Total- 1.083 Hrs | Bevels Chisels Drills Hammers, hand/power saws Marking equipment, measuring tapes and rules Nail guns Protractors Spirit levels, squares (combination/tri), steel squares and fences, string lines Barrel bolts Cabin hooks closers Dead bolts Flash bolts, flashings Handles hinges (butt and parliament) Latches, locks | Class room and construction lab |
| - Perform Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Description of procedure of maintaining and cleaning equipment and tools. Practical activity: Clean the area and check tools and equipment | Theory-0.083 Hrs Practical-0.5 Hrs Total- 0.583 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-092. Prepare Surface and fix tiles

Objective: This module covers the knowledge and  skills required to prepare surface and fix tiles.

Duration: 4 Hours							Theory: 1 Hours			Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. | - Description of purpose of surface preparation - Explanation of types of surface preparation materials. - Description of finishing materials. - Description of Types of tiles - Processes for the calculation of material requirements, quality requirements Practical activity: Calculate the materials and tools required for surface preparation and tile fixing in lab. | Theory-0.1 Hrs Practical-0.3 Hrs Total- 0.4 Hrs | Pencil Eraser Notebook | Class Room |
| - Prepare materials for tiling application | Trainee will be able to: - Floor and wall tiling materials are checked to ensure they are suitable and meet specifications. - Material preparation is carried out to satisfy the requirements of the specified application process. | - Explanation of types of materials for fixing tiles. - Definition of adhesives. - Properties of adhesives. - Process of preparation of fixing materials. Practical activity: Prepare materials for tiling application in lab. | Theory-0.1 Hrs Practical-0.3 Hrs Total- 0.4 Hrs | Shovels Spatulas Sponges Bucket | Class Room and construction lab |
| - Prepare underlay and sheeting substrate | Trainee must be able to: - Assistance with underlay preparation is provided under supervision. - Substrate surface is finished to specification, with joints flush and sealed. | - Definition of substrate. - Safety precaution to work at site - PPEs Practical activity: Replace window units/door frames in lab. | Theory-0.1 Hrs Practical-0.3 Hrs Total- 0.4 Hrs |  | Class Room and construction lab |
| - Prepare render substrate surface | Trainee must be able to: - Surface-mounted construction units and attachments are safely detached. - Surfaces of substrate structure are cleaned to remove all loose material. - Materials for splash coat are proportioned and mixed to instructions ready for application to wet surface. - Horizontal and vertical surrounds are prepared for tiling process in accordance with type of tile and specified finish. - Materials for render coat are proportioned and mixed to instructions ready for application. - Surface is scratched, rendered, cured and dried in accordance with specifications for tile application | - Procedure to prepare substrate surface. - Definition of rendering. - Definition of curing. - Definition of splash coat. Practical activity Prepare render substrate surface in lab. | Theory-0.1 Hrs Practical-0.3 Hrs Total- 0.4 Hrs |  | Class room and construction lab |
| - Cut tiles as required. | Trainee must be able to: - Tiles are cut without jagged, flaid edges or damage to tile surfaces or finish, in accordance with workplace procedures and manufacturer recommendations. - Recess hole or curve is cut by hand or machine to shape and size and to specified tolerance. - Tile jolly is edged to form a mitre so that biscuit is not exposed at the joint in accordance with workplace procedures and manufacturer recommendations. | - Procedure to cut the tiles. - Methods of tile cutting - Description of edges requirements - Recess holes Practical activity Cut tiles as required. in lab. | Theory-0.1 Hrs Practical-0.3 Hrs Total- 0.4 Hrs | Tile cutters Measuring tape | Class room and construction lab |
| - Fix wall tiles | Trainee must be able to: - Mortar and/or adhesive is prepared and applied to tile surface in accordance with manufacturer recommendations. - Tiles are prepared and fixed, with pad tiles set to level alignment. - Horizontal joint is checked for straightness, and tile edges and surface alignment are checked for conformity. - Tiles are fixed to alignment maintaining designed pattern to specification. - Even margins are shown around openings, frames and fittings to specification. - Bottom course is cut and fixed to create a rake or square corner in accordance with drawings and specifications. - Splayed, manufactured, formed coves are fixed in accordance with drawings and specifications. - Vertical tiles are finished plumb and true to square corners. - Joints are maintained straight and uniform in width with due allowance for tolerance of tile sizes. - Control joints are built in, in accordance with specifications and manufacturer recommendations. - Miter joints are made, maintaining glazing on miter without damage to tile surfaces or finish and maintaining uniformity of miter in accordance with company procedures and manufacturer recommendations | - Application of adhesives. - Procedure to fix tiles. - Procedure to check alignment Practical activity Fix wall tiles | Theory-0.1 Hrs Practical-0.5 Hrs Total- 0.6 Hrs | Brooms, Brushes, Buckets Caulking guns, Cement sheet cutters, Concrete mixers Electrical leads Hammers, Hose and water sprays Ladders, Levelling equipment, Lump hammers Mortar boards Nippers, Power drills, Power screwdrivers, Power grinders, power sanders Power grinders and sanders Rags, rubber mallets Sanding blocks, saws, scrapers, shovels, spacers and wedges Spatulas, sponges, squares, squeegees, straight edges, string lines Trowels Wet and dry diamond saws, wheelbarrows, wire brushes, wooden floats Work platforms Tile cutters and scribes trowels | Class Room and construction lab |
| - Tile external corners. | Trainee must be able to: - Setting out for plumb, level and square is checked to be within specified tolerance. - External corners are checked to ensure surface intersections are straight. - Curved bead angle trim or tiles are fixed so that external return and bead are square and measurements are accurate to junction with tiles and set out, where applicable. - Tiles are fixed with minimum voids in tile bed while maintaining fully bedded alignment to specifications. - Corner is kept square within specified tolerance and finish to specifications. | - Setting out techniques Practical activity Tile external corners in lab | Theory-0.1 Hrs Practical-0.3 Hrs Total- 0.4 Hrs | Brooms, brushes, Buckets Caulking guns, Cement sheet cutters, Concrete mixers Electrical leads Hammers, Hose and water sprays Ladders, Levelling equipment, Lump hammers Measuring tapes and rules, mortar boards, Nippers Pointed grouters, Power drills, Power screwdrivers, Power grinders, power sanders Power grinders and sanders Rags, rubber mallets Sanding blocks, saws, scrapers, shovels, spacers and wedges Spatulas, sponges, squares, squeegees, straight edges, string lines Trowels Wet and dry diamond saws, wheelbarrows, wire brushes, wooden floats Work platforms Tile cutters and scribes, trowels | Class Room and construction lab |
| - Tile internal corners | Trainee must be able to: - Internal corner is checked to ensure surfaces are flat and intersection is straight. - Tiles are cut where required and fixed to one wall to maintain alignment in accordance with set out and specifications. - Tiles are cut where required and fixed abutting adjacent wall tiles to line, set out and specifications. - Joints for abutting tiles are made in accordance with designed margin for grouting or for expansion joint, where applicable, to specifications. - Coved tile or trim is installed to coved internal wall or wall and floor junctions. - Coved tile or trim is installed so that line is straight and, where applicable, aligned with set out. - Tiles are fixed to cove tile or trim and finish to alignment and specifications. | - Setting out techniques Practical activity Tile external corners in lab | Theory-0.1 Hrs Practical-0.3 Hrs Total- 0.4 Hrs | Brooms, Brushes, Buckets Caulking guns, Cement sheet cutters, Concrete mixers Electrical leads Hammers, Hose and water sprays Ladders, Levelling equipment, Lump hammers Measuring tapes and rules, mortar boards Nippers Pointed grouters, Power drills, Power screwdrivers, power grinders, power sanders Power grinders and sanders Rags, rubber mallets Sanding blocks, saws, scrapers, shovels, spacers and wedges Spatulas, sponges, squares, squeegees, straight edges, string lines Trowels Wet and dry diamond saws, wheelbarrows, wire brushes, wooden floats Work platforms Tile cutters and scribes trowels | Class Room and construction lab |
| - Grout wall tile face | Trainee must be able to: - Joints are cleaned and prepared to receive grout according to manufacturer specifications. - Grout is mixed and applied in accordance with workplace and manufacturer specifications and to meet job requirements. - Tiles are cleaned and polished with dry cloth to specifications, removing all dust from surface and joints. | - Definition of grouting - Description of grouting materials and its characteristics and examples Practical activity: Clean the tiles | Theory-0.1 Hrs Practical-0.3 Hrs Total- 0.4 Hrs | Broom Brush Sponge Trowel Spatula Dry and wet cloth Pointed grouter | Class Room and construction lab |
| - Perform Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Description of procedure of maintaining and cleaning equipment and tools. Practical activity: Clean the area and check tools and equipment | Theory-0.1 Hrs Practical-0.1 Hrs Total- 0.2 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-093. Prepare Surface for Painting

Objective: This module covers the knowledge and  skills required to prepare new or uncoated surfaces for painting or clear finish, prepare previously coated surfaces for painting or clear finish, remove wallpaper and prepare surface for painting and perform clean up

Duration: 6.5 Hours									Theory: 0.5 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. | - Definition of paint and its constituents. - Explanation of types of paints - Description of surface coating terminologies - Processes for the calculation of material requirements. Practical activity: Calculate the materials and tools required for surface preparation for painting lab. | Theory-0.1 Hrs Practical-1.2 Hrs Total- 1.3 Hrs | Pencil Eraser Notebook | Class Room |
| - Prepare new or uncoated surfaces for painting or clear finish | Trainee will be able to: - Suitability of surface for painting or clear finish application is determined in accordance with manufacturer recommendations. - Surface preparation method is correctly selected in accordance with the environment, finish and substrate requirements. - Surface is prepared to manufacturer specifications in compliance with substrate requirements. - Surface imperfections are stopped, filled and sanded to a smooth finish ready for painting. | - Explanation of Properties and surface preparation requirements of new substrates - Methods to prepare surface - Process of preparation of surface before painting. Practical activity: Prepare new or uncoated surfaces for painting or clear finish in lab. | Theory-0.1 Hrs Practical-1.4 Hrs Total- 1.6 Hrs | Drop sheets, duster brushes Filling knives blades Hammers, hand sanders, heat removal equipment Mechanical sanders Nail punches Putty knives Scrapers Water blasters, wire brushes | Class Room and construction lab |
| - Prepare previously coated surfaces for painting or clear finish | Trainee must be able to: - Condition and nature of existing substrate and surface material are determined and tested. - Surface preparation method is correctly selected in accordance with the environment, finish and substrate requirements. - Surfaces are prepared by removing unwanted coatings and loose debris. - Surface defects are repaired, filled and sanded to smooth finish ready for painting in accordance with specifications. | - Definition of substrate. - Description of Prevention and/or rectification procedures for surface coating defects - Safety precaution to work at site - PPEs - Explanation of types of surface defects. Practical activity: Prepare previously coated surfaces for painting or clear finish in lab. | Theory-0.1 Hrs Practical-1.4 Hrs Total- 1.5 Hrs | Drop sheets, duster brushes Filling knives blades Hammers, hand sanders, heat removal equipment Mechanical sanders Nail punches Putty knives Scrapers Water blasters, wire brushes | Class Room and construction lab |
| - Remove wallpaper and prepare surface for painting | Trainee must be able to: - Type, condition and nature of existing type of wallpaper are determined prior to removal. - Surface preparation method is correctly selected in accordance with the environment, finish and substrate requirements. - Wallpaper is removed using the most appropriate method. - Surfaces are prepared for paint application by removing loose debris. - Surface defects are repaired and imperfections stopped, to smooth finish ready for painting in accordance with specifications. | - Procedure to remove wall paper. - Description of rendering. - Procedure to repair surface Practical activity Prepare render substrate surface in lab. | Theory-0.1 Hrs Practical-1.2 Hrs Total- 1.3 Hrs | Drop sheets, duster brushes Filling knives blades Hammers, hand sanders, heat removal equipment Mechanical sanders Nail punches Putty knives Scrapers Water blasters, wire brushes | Class room and construction lab |
| - Perform Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Description of procedure of maintaining and cleaning equipment and tools. Practical activity: Clean the area and check tools and equipment | Theory-0.1 Hrs Practical-1 Hrs Total- 1.1 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-094. Paint by spray, brush and roller

Objective: This module covers the knowledge and  skills required to apply brushed, rolled or sprayed paint coatings to different materials to form a protective and decorative finish.

Duration: 6.5 Hours									Theory: 0.5 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work | Trainee will be able to: - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. | - Definition of paint and its constituents. - Explanation of types of paints - Description of surface coating terminologies - Processes for the calculation of material requirements. Practical activity: Calculate the materials and tools required for surface preparation for painting lab. | Theory-0.07 Hrs Practical-0.8 Hrs Total- 0.87 Hrs | Pencil Eraser Notebook | Class Room |
| - Prepare work area | Trainee will be able to: - Materials and substrate surfaces are prepared in accordance with manufacturer recommendations. - Surfaces not to be painted are protected by drop sheets, masking or removal of objects. - Adequate ventilation is provided to maintain a safe environment. | - Compatibility of preparatory materials and paint systems - Methods to prepare surface - Process of preparation of surface before painting. - Definition of ventilation and purpose - Explanation of Types of ventilation Practical activity: - Prepare work area in lab. | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Drop sheets, Duster Brushes Filling knives and blades filters Hand Sanders, Removal equipment Hose Masking equipment, mechanical sanders Putty knives Scrapers, wire brushes | Class Room and construction lab |
| - Mix paint color | Trainee must be able to: - Base color is identified by sample analysis or specification, and correct tint base, paint type and sheen level determined. - Colorants suitable for the color match are selected, mixed with the base, allowed to dry and matched against the sample/ specification for accuracy. - Materials for specified paint finish are mixed in accordance with manufacturer recommendations. - Correct amounts of paint material are prepared to specified ratio and drying time in accordance with manufacturer specification. | - Description of Hazards associated with solvents, chemicals and dust - Description of mix proportions of constituents - Examples of base, solvents. - Safety precaution to work at site - PPEs - Explanation of effect of colorants Practical activity: Mix paint color in lab. | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Brush ware, Brush ware accessories, Drop sheets, Heat and Flame paint Hose Masking equipment, mechanical sanders Nail punches Paint pots and buckets, Paint stirrers Putty knives Regulator, roller frames Scrapers, | Class Room and construction lab |
| - Apply paint with brush or roller | Trainee must be able to: - Brush and/or roller is selected to suit specified surface profile, size of area, type of paint and finish specified. - Sealant, undercoat, intermediate coat and finish coat are applied by brush/roller to achieve the required level of opacity, finish, texture and sheen, in accordance with specifications. | - Explanation of Surface coating technology, including specification of paint systems for interior and exterior painting. - Description Theoretical principles relating to adhesion and cohesion of paint - Theoretical principles relating to pigmentation and coloring agents, drying and curing processes and the role of solvents Practical activity Apply paint with brush or roller in lab. | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Brush ware, Brush ware accessories Compressor, Conventional spray unit Diaphragm, Hand Sanders, Heat and Flame paint Removal equipment, Masking equipment, Mechanical sanders Nail punches Paint pots and buckets, Paint stirrers Putty knives Regulator, roller frames | Class room and construction lab |
| - Apply paint by spray | Trainee must be able to - Appropriate spray equipment, accessories and lines are selected and set up for operation in accordance with the manufacturer’s recommendations. - Paint is mixed and the viscosity adjusted to allow for spray painting. - Paint is applied to achieve correct finish specifications. | - Description of process of applying spray paints. Practical activity Apply paint by spray in lab. | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Compressor, Conventional spray unit High-volume low-pressure (hvlp) spray application, Masking equipment, Mechanical sanders Nail punches Paint pots and buckets, Piston airless Spray unit (electrical, pneumatic and petrol), Spray equipment, Spray guns, | Class Room and construction lab |
| - Finish the application | Trainee must be able to - Finished paint surface is cured and tested using curing method in accordance with manufacturer recommendations. - Doors, windows and furniture removed for painting application are re-installed correctly and without damage to finished | - Procedure to Installation of door and windows Practical activity Finish the application in lab | Theory-0.07 Hrs Practical-0.85 Hrs Total- 0.92 Hrs | Sanders | Class Room and construction lab |
| - Perform Clean up. | Trainee must be able to - Work area is cleaned and waste disposed of, reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with workplace practices. | - Description of procedure of maintaining and cleaning equipment and tools. Practical activity: Clean the area and check tools and equipment | Theory-0.07 Hrs Practical-0.55 Hrs Total- 0.62 Hrs | Cleaning tools | Class Room and construction lab |

### 0732B&CE-095. Apply stains and clear timber finishes

Objective: This module covers the knowledge and  skills required to apply stains and clear timber finishes to different material surfaces, to form a protective and decorative finish.

Duration: 6.5 Hours									Theory: 0.5 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plan and prepare. | Trainee will be able to: - Work instructions are confirmed with supervisor. - Tools and equipment are selected as required, checked for serviceability and any faults are fixed or reported. - Material quantity requirements are calculated in accordance with specifications. - Materials needed are obtained, checked for compliance and prepared. | - Information of faults and fixing Practical Activity Plan and prepare. | Theory-0.1Hrs Practical-1.2 Hrs Total- 1.3 Hrs | Eraser Pencil Sharpener Scale Notepad Charts Calculator Construction lab tools Pressure | Class Room and Construction Lab |
| - Prepare materials and application area | Trainee will be able to: - Adjoining surfaces are protected by masking off or covering. - Adequate ventilation is provided to maintain a safe environment. - Measures are taken to ensure application area remains free of dust and foreign matter. - Existing stained or finished surfaces for application are stripped using appropriate techniques. | - Information of Testing techniques and support materials Practical Activity Prepare materials and application area | Theory-0.1Hrs Practical-1.2 Hrs Total- 1.3 Hrs | Personal Protective equipment | Class Room and Construction Lab |
| - Stain bare timber surface. | Trainee must be able to: - Stain is selected or type of timber in accordance with manufacturer recommendations. - Stain is prepared to proportions and consistency in accordance with manufacturer recommendations. - Application method for the specified surface, area size and type of finish is selected. - Stain is applied to bare timber surface to specifications. - Wood filler and putty are selected, mixed, color-matched and applied to timber in accordance with specifications. | - Hazards associated with solvents, chemicals and dust - Surface preparation techniques for clear wood finishing Practical Activity Stain bare timber surface | Theory-0.1Hrs Practical-1.3 Hrs Total- 1.4 Hrs | Elevators Forklifts Hammers (bricks, club and scutch), hoses Jigsaws, jointing tools Line blocks and line pins Margin or raking tools, masonry saws, masonry squares, materials hoists, measuring tapes and rules, mortarboards Pallet trolleys, plumb rules, profiles | Class Room and Construction Lab |
| - Apply finish surface | Trainee must be able to: - Coats of selected clear finish are applied to achieve required level of opacity, finish accordance with specifications. - Drying time is allowed between coat sand clear finish surfaces are cured in accordance with manufacturer recommendations. | - The principles relating to pigmentation and coloring agents, drying and curing processes and the role of solvents Practical Activity Apply finish surface | Theory-0.1Hrs Practical-1.2 Hrs Total- 1.3 Hrs |  | Class Room and Construction Lab |
| - Perform clean up | Trainee must be able to: - Work area is cleaned and waste disposed of reused or recycled in accordance with work specifications. - Tools plant and equipment are cleaned, checked for faults and maintained and stored in accordance with work place practices | - Information of faults and maintenance according to the workplace Practical Activity Perform clean up | Theory-0.1Hrs Practical-1.2 Hrs Total- 1.3 Hrs |  | Class Room and Construction Lab |

### 0732B&CE-096. Prepare site layout of civil engineering projects

Objective: This module covers the knowledge and  skills required to Prepare for work, Prepare site for temporary fencing and structures, Co-ordinate the erection of site security fencing/hoarding, Co-ordinate construction of temporary site structures and Co-ordinate storage of tools and machine accessories

Duration: 6 Hours									Theory: 0.5 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work | Trainee will be able to: - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details. - Follow Safety (OHS) requirements in Accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Select plant, tools and equipment to carry out tasks consistent with job requirements, check for serviceability, and any faults - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Identify, obtain, prepare, and safely handle materials appropriate to the work application | - Types and purpose of job information/instructions - OH&S requirements for preparing construction sites Practical Activity Prepare for work | Theory-0.1Hrs Practical-1.2 Hrs Total- 1.3 Hrs | Eraser Pencil Sharpener Scale Notepad Charts A real or simulated environment Project resources Organization policies and guidelines Access to relevant resource personnel Relevant legal and statutory documentation | Class Room and Construction lab |
| - Prepare site for temporary fencing and structures | Trainee will be able to: - Identify services, location and work areas are accurately - Place adequate warning signs and safeguards to support and protect services and substructures identified in work area. - Set up and secure temporary signs, control systems and protective barriers in accordance with job and safety instructions. | - Physical conditions of site - Tools, machines and equipment - Site fencing and hoarding Practical Activity Prepare site for temporary fencing and structures | Theory-0.1Hrs Practical-1.2 Hrs Total- 1.3 Hrs | Eraser Pencil Sharpener Scale Notepad Charts A real or simulated environment Project resources Organization policies and guidelines Access to relevant resource personnel Relevant legal and statutory documentation | Class Room and Construction lab |
| - Co-ordinate the erection of site security fencing/hoarding | Trainee will be able to: - Select appropriate type fencing materials in adequate quantities. - Confirm dimensions of installed fence components to job specifications and/or work instructions. - Check erected fence as meeting security and enclosure requirements | - Relevant statutory requirements relating to site boundaries - Access and traffic routes - Storage and control of tools and materials Practical Activity Co-ordinate the erection of site security fencing/hoarding | Theory-0.1Hrs Practical-1.2 Hrs Total- 1.3 Hrs | Eraser Pencil Sharpener Scale Notepad Charts A real or simulated environment Project resources Organization policies and guidelines Access to relevant resource personnel Relevant legal and statutory documentation | Class Room and Construction lab |
| - Co-ordinate construction of temporary site structures | Trainee will be able to: - Interpret instruction/specification for temporary site structures accurately to the relevant persons. - Lay out temporary site structures to facilitate ease of access and to prevent obstruction of other site activities. - Check erected temporary site structures as conforming to job specification/ instruction given. | - Convenient, visible and safe storage - Storage space and zone - Quality standards and specifications Practical Activity Co-ordinate construction of temporary site structures | Theory-0.1Hrs Practical-1.2 Hrs Total- 1.3 Hrs | Eraser Pencil Sharpener Scale Notepad Charts A real or simulated environment Project resources Organization policies and guidelines Access to relevant resource personnel Relevant legal and statutory documentation | Class Room and Construction lab |
| - Co-ordinate storage of tools and machine accessories | Trainee will be able to: - Lay out Storage facilities correctly and built to specifications. - Correctly demonstrate working knowledge of the type of items to be stored - Organize store for proper delivery, checking and control of materials, tools and equipment. | - Working knowledge of the type of items to be stored - Storage of materials, tools and equipment Practical Activity Co-ordinate storage of tools and machine accessories | Theory-0.1Hrs Practical-1.2 Hrs Total- 1.3 Hrs | Eraser Pencil Sharpener Scale Notepad Charts A real or simulated environment Project resources Organization policies and guidelines Access to relevant resource personnel | Class Room and Construction lab |

### 0732B&CE-097. Construct masonry steps and stairs

Objective: This module covers the knowledge and  skills required to construct masonry steps, stairs and wing walls for different types and styles of buildings. It includes planning, types and styles of buildings. Set out and installation of the masonry.
Duration: 7 Hours									Theory: 1 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work. | Trainee will be able to: - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details, from relevant information to determine brick laying and block laying tasks. - Follow safety (OHS) requirements in accordance with safety plans and policies. - Identify and implement signage and barricade requirements. - Select plant, tools and equipment to carry out tasks consistent with job requirements, check for serviceability, and any faults - Calculate material quantity requirements in accordance with plans, specifications and quality requirements. - Identify, obtain prepare, and safely handle Materials appropriate to the work application are and located ready for use. | - Brick bond patterns characteristics and applications of materials for masonry steps and stairs construction - Brick and block expansion and growth, control and articulation joints Practical Activity Prepare for work. | Theory-0.33 Hr Practical-01 Hrs Total- 1.33 Hrs | Pencil Eraser Scale Sharpener Notepad Personal protective equipment for construction industry. | Class Room and Construction lab |
| - Set out brickwork/ block work. | Trainee will be able to: - Erect work platform in accordance with regulatory and workplace requirements and brick and block stair construction requirements. - Check location and relative level of prepared footing from job drawings and specifications. - Calculate rise and going of flight and individual steps as per codes and workplace requirements. | - Construction terminology - Materials storage and environmentally friendly waste management - Plans, specifications and drawings Practical Activity Set out brickwork/ block work. | Theory-0.33 Hr Practical-2.5 Hrs Total- 2.83 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Wooden float Steel float Shovel Spade Wire brush Screeds (wooden/steel) | Class Room and Construction lab |
| - Lay bricks forming steps and wing walls | Trainee will be able to: - Prepare mortar mix to required conformity and specifications. - Construct base brickwork to specifications and requirements of standards and codes. - Construct square steps, level, and plumb and lay to specified bond. - Construct design and profile of steps and aligned to bond and plumb to specifications. - Carry out Joints to work specifications. - Set out Brickwork/block work laid to line, with gauge and complete to work specifications. - Clean brick/block faces free of mortar | - Processes for the calculation of material requirements - Quality requirements - Techniques of masonry steps and stairs construction - Types, characteristics, uses and limitations of plant, tools and equipment - Workplace and equipment safety requirements Practical Activity Lay bricks forming steps and wing walls | Theory-0.33 Hr Practical-2.5 Hrs Total- 2.83 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Wooden float Steel float Shovel Spade Wire brush Screeds | Class Room and Concrete lab |

### 0732B&CE-098. Install stairs

Objective: This module covers the knowledge and  skills required to construct masonry steps, stairs and wing walls for different types and styles of buildings. It includes planning, types and styles of buildings. It includes planning, preparation, set out and installation of the masonry.
Duration: 10 Hours									Theory: 1 Hours				Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plan and prepare | Trainee will be able to: - Confirm Work instructions in accordance supervisor. - Select required tools and equipment, serviceability and any faults. - Calculate materials quantity requirements accordance with specifications. - Obtain materials needed for compliance, prepared quality requirements. | - Calculation of quantity of materials with respect to specifications Practical Activity Plan and prepare | Theory-0.125 Hrs Practical-1.125 Hrs Total- 1.25 Hrs | Eraser Pencil Sharpener Scale Notepad Drawing sheet Drawing Board Calculator Drawing instruments Personal protective equipment for construction industry. | Class Room and Construction lab |
| - Select and prepare materials and components | Trainee will be able to: - Identify methods of assembling erected for stairs - Check components for appropriate locations in stair structure. - Determine method of assembling and fixing in accordance with stair design and location | - Components of stairs - Design of stairs Practical Activity Select and prepare materials and components | Theory-0.125 Hrs Practical-1.125 Hrs Total- 1.25 Hrs | Eraser Pencil Sharpener Scale Notepad Calculator | Class Room and Construction lab |
| - Assemble strings newels | Trainee must be able to: - Identify Specific position for stairs - Check measurements and adjustments if applicable. - Assemble Strings and newels to design and specification. - Fix strings to walls temporarily supported or directly fixed in position /specification. | - Adhesives, fixings and fasteners related to stair construction - Assembling procedures for stairs - Leveling techniques - marking of components Practical Activity Assemble strings newels | Theory-0.125 Hrs Practical-1.125 Hrs Total- 1.25 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Wooden float Steel float Shovel Spade Wire brush Screeds (wooden/steel) | Class Room and Construction lab |
| - Install treads and risers | Trainee must be able to - Assemble strings and newels temporarily braced in vertical position. - Fix treads and risers about newels to assembly - Check flight is true and square - Fit intermediate treads and risers. | - Measuring and setting out related to assembling and installing stairs Practical Activity Install treads and risers | Theory-0.125 Hrs Practical-1.125 Hrs Total- 1.25 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Wooden float Steel float Shovel Spade Screeds (wooden/steel) | Class Room and Construction lab |
| - Assemble and install landings | Trainee must be able to - Fix bearers, joists and level according to specification. - Fix nosing and floor landing according to finish specification basin | - Stair construction and joining methods Practical Activity Assemble and install landings | Theory-0.125 Hrs Practical-1.125 Hrs Total- 1.25 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Wooden float Steel float Shovel Spade Wire brush Screeds (wooden/steel) | Class Room and Construction lab |
| - Install handrail and balustrade | Trainee must be able to - Fix balusters/intermediate railing and handrails to form stair balustrade according to specification, - Check and ensure plumb fit of Newels prior to final fixing - Fix hand railings fitted to wall in accordance with specifications | - Fixing of hand rails and balustrade with respect to specifications Practical Activity Install handrail and balustrade | Theory-0.125 Hrs Practical-1.125 Hrs Total- 1.25 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Wooden float Steel float Shovel Spade Wire brush Screeds (wooden/steel) | Class Room and Construction lab |
| - Install spiral stair and curved strings | Trainee must be able to - Mark location of stair and first step on floor and erect central post is into true position, fixed at floor and temporarily braced at top. - Support initial string section temporarily supported in place for assembly, and fit treads and risers into position to specification. - Develop stair progressively with the extending, supporting and fixing of curved string, and complete with head secured to floor/landing, balustrade installed and central post fixed to specifications. | - Location of stair case - Development of stairs according to specifications Practical Activity Install spiral stair and curved strings | Theory-0.125 Hrs Practical-1.125 Hrs Total- 1.25 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Wooden float Steel float Shovel Spade Wire brush Screeds (wooden/steel) | Class Room and Construction lab |
| - Secure stair to structure and line spandrel area | Trainee must be able to - Carry out securing of stair to building during/on completion of assembly. - Frame, line and fix spandrel, where applicable, to specified finish. | - Types of stairs and elements of stairs - Select and use safe and efficient procedures in installing treads, risers, flooring and nosing Practical Activity Secure stair to structure and line spandrel area | Theory-0.125 Hrs Practical-1.125 Hrs Total- 1.25 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Wooden float Steel float Shovel Spade Wire brush Screeds (wooden/steel) | Class Room and Construction lab |

### 0732B&CE-099. Prepare for solid dry wall plastering

Objective: This module covers the knowledge and  skills required effectively carry out the preparation process of dry wall plastering, and applies to effectively prepare the process for carrying out solid plastering work, and applies to individuals working in masonry in the construction industry.

Duration: 4 Hours									Theory: 1 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Plan for construction process | Trainee will be able to: - Assure Quality requirements of company’s construction operations - Identify Preparation and planning requirements from drawings/work location and/or supervisor’s instructions. - Observe OH&S requirements and accordance with application tasks and workplace environment. - Identify Safety hazards and correct procedures in order to minimize risk to self and others. - Select Materials to supervisor’s instructions, - Select appropriate personal protective equipment, correctly. - Select Tools and equipment consistent with the job requirements. - Select Fixtures/fasteners consistent with job requirements and check for specifications, quality requirements and operational details, - Calculate Materials quantity requirements in accordance with plans, specifications and quality requirements. - Identify Materials appropriate to the work application are, obtained, prepared, safely handled and located ready for use. | - Identification of Preparation and planning requirements from drawings - Identification of safety hazards and correct procedures - Selection of materials - Selection of Tools and equipment - Calculation of materials quantity - Safe handling of materials for the work Practical Activity Plan for construction process | Theory-0.2 Hr Practical- 0.6 Hrs Total- 0.8 Hr | Compass Drawing sheet Drawing instruments Drawing Boards Compass Eraser Pencil Sharpener Ruler Notepad Calculator | Class Room and Construction Lab |
| - Prepare work area suitable for construction process | Trainee will be able to: - Identify activities to be carried out in work area from surfaces to be lined and height to be accessed. - Prepare work area for construction process to supervisor’s instructions | - Knowledge of workplace and equipment safety requirements Practical Activity Prepare work area suitable for construction process | Theory-0.2 Hr Practical- 0.6 Hrs Total- 0.8 Hr | Compass | Class Room and Construction Lab |
| - Use tools, plant and equipment appropriate for construction process | Trainee must be able to: - Use regular hand and power tools suitable for application process identified to job requirements. - Carry out hand and power tools used safely and effectively processes | - Drawings and specifications portable power tools and tools and equipment materials relative to dry wall plastering materials handling Practical Activity Use tools, plant and equipment appropriate for construction process | Theory-0.2 Hr Practical- 0.6 Hrs Total- 0.8 Hr | Construction tools | Class Room and Construction Lab |
| - Use tools, plant and equipment appropriate for construction process | - Trainee must be able to: - Identified Regular hand and power tools suitable for application process to job requirements. - Use Hand and power tools safely and effectively to carry out processes. | - Measurement relative to dry wall plastering - Fixing and fasteners consistent with drywall plastering requirements - Workplace communications and cement Practical Activity Use tools, plant and equipment appropriate for construction process | Theory-0.2 Hr Practical- 0.6 Hrs Total- 0.8 Hr | Construction tools | Class room and Construction Lab |
| - Prepare background of brick, concrete or block work for solid plastering | Trainee must be able to: - Identify Structure and surface prepared. - Depressions patched with suitable material to supervisor’s instructions. - Apply adhesive Concrete surface where appropriate rough. - Mix materials for scratch coat to instructions ready for application to wet surface. | - Identification of surfaced prepared - Application of Cornice adhesive - Mixing of materials according to instructions Practical Activity Prepare background of brick, concrete or block work for solid plastering | Theory-0.2 Hr Practical- 0.6 Hrs Total- 0.8 Hr | Construction tools | Class room and Construction Lab |

### 0732B&CE-100. Apply plaster

Objective: This module covers the knowledge and  skills required to construct masonry steps, stairs and wing walls for different types and styles of buildings. It includes planning, types and styles of buildings. It includes planning, preparation, set out and installation of the masonry.
Duration: 4 Hours									Theory: 1 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for work | Trainee will be able to: - Work instructions, including plans, specifications, quality requirements and operational details, are obtained, confirmed and applied from relevant information to determine bricklaying and block laying tasks. - Safety (OHS) requirements are followed in Accordance with safety plans and policies. - Signage and barricade requirements are identified and implemented. - Plant, tools and equipment selected to carry out tasks are consistent with job requirements, checked for serviceability, and any faults are rectified or reported prior to commencement. - Material quantity requirements are calculated in accordance with plans, specifications and quality requirements. - Materials appropriate to the work application are identified, obtained, prepared, safely handled and located ready for use. | - Occupational Health and Safety (OHS) Regulations - Composition, properties and specifications of plastering materials - Economic use of materials - Estimation of materials Practical Activity Prepare for work | Theory-0.25 Hrs Practical-0.35 Hrs Total- 0.6 Hrs | Eraser Pencil Sharpener Ruler Notepad Calculator | Class Room and Construction lab |
| - Prepare mortar mix | Trainee will be able to: - Select PPE, tools and equipment in accordance with the job requirements. - Estimate materials required as per job specifications. - Prepare the surface/mixing machines for mixing of mortar as per the job requirement. - Mix materials as per the job requirement following standard procedures. - Transport mortar mix as per the job requirement following standard procedures. | - Selection of tools and equipment - Estimation of materials - Preparing of surface - Mixing of materials as per the standards Practical Activity Prepare mortar mix | Theory-0.25 Hrs Practical-0.95 Hrs Total- 1.2 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Wooden float Steel float Shovel Spade Wire brush Screeds (wooden/steel) | Class Room and Construction Lab |
| - Prepare scaffolding | Trainee will be able to: - Select and use PPE according to the job requirements. - Select and use tools and equipment as per the job requirement following standard procedures. - Prepare estimate of materials required based on the job requirements. - Prepare scaffolds components as per the job requirement following standard procedures - Erect and assemble scaffolds components as per the job requirements following standard procedures. - Dismantle scaffolding as per the job requirement following standard procedures. - Store or stockpile components following standard procedures. | - Use of PPEs - Selection and use of tools and equipment - Preparation of estimation of materials - Preparation of scaffolding - Assembling of scaffolding - Dismantling of scaffolding - Storing of components as per specification Practical Activity Prepare scaffolding | Theory-0.25 Hrs Practical-0.95 Hrs Total- 1.2 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Wooden float Steel float Shovel Spade Wire brush Screeds (wooden/steel) | Class Room and Construction Lab |
| - Perform plastering work | Trainee will be able to: - Prepare the surface to be plastered as per the job requirement following standard. - Prepare and apply cement slurry as per the job requirement following standard practices. - Apply mortar mix as per the job requirement to the required specifications following standard procedures. - Cure the plastered surfaces as per the job requirement following standard practices. | - Preparation of plaster - Preparation and application of cement slurry as per the job requirement following standard practices. - Application of mortar mix as per the job requirement to the required specifications following standard procedures. - Curing of the plastered surfaces as per the job requirement following standard practices. - Practical Activity - Perform plastering work | Theory-0.25 Hrs Practical-0.75 Hrs Total- 1 Hrs | Mason hammer Pointing towels Measuring tape Mason thread Plumb bob Leveling tools (water level, spirit level) Bucket Mortar pan Shovel | Class Room and Construction Lab |

## 9.9.  Plumbing

### 0732B&CE-101. Identify plumbing tools & equipment

Objective: This module covers the knowledge and skills required to interpret layout drawing for Plumbing Lab its specifications, its Requirement for tools and equipment.

Duration: 6.0 	Hours							Theory: 04 Hours				Practice: 02 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Draw Layout drawing for Plumbing Lab | Trainee will be able to: - Mark North Point. - Measure the Lab dimension. - Develop layout drawing - Analyze Signs, Symbols and data - Identify location for Doors and windows. - Identify instruments places. - Identify sitting plan. | - Explain lay out drawings with its type. - Explain different drawing instruments - Describe different symbols for plumbing Practical Activity Draw layout of plumbing lab | Theory-01 Hrs Practical-1.5Hrs Total- 2.5 Hrs | Pencil Eraser Sharpener Calculator Measuring scale Drawing sheet Drawing board Tee Square Set Square Measuring Tape Computer AutoCAD | Class Room and workshop |
| - Identify Plumbing Tools & Equipment | Trainee will be able to: - Identify Tools & equipment - Check for safety hazards - Conduct operational testing - Maintain professional proficiency | - Explain safety requirements, for the required tools for task - Explain different specification for tools and equipment - Explain procedures for using different tools - Describe Plumbing History and Plumbing importance - Explain Safety measures while performing Plumbing tasks Practical Activity Identify Plumbing Tools & Equipment | Theory-03 Hrs Practical-0.5 Hrs Total- 3.5 Hrs | T & E Plumbing tools and equipment | Class Room and workshop |

### 0732B&CE-102. Perform cutting, threading & reaming

Objective: This module covers the knowledge and skills required to perform pipe cutting, its threading and reaming.

Duration:  06 Hours									Theory: 02 Hours			Practice: 04 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform Pipe Cutting. | Trainee will be able to: - Identify tools for cutting. - Secure work piece firmly - Measure required Length - Use cutter OR Hacksaw - Loose the cutting Piece - Check length for cutting piece - Observe OHSA | - Comprehend the purpose and application of plumbing tools - Explain safety precaution for using cutter and hacksaw - Explain the procedure to use of Hack saw Practical Activity - Perform cutting of pipe | Theory-0.5 Hrs Practical-01 Hrs Total- 1.5 Hrs | Pipe Cutting Tools Plumbing pliers, Plumbing wrenches Plumbing spanners, PPEs | Class Room and workshop |
| - Perform Pipe Threading. | Trainee will be able to: - Identify tools for threading. - Adopt safety measures - Fix work piece firmly - Clamp Threading die - Prepare threading - Unclamp the threading die - Observe OHSA | - Describe calibration of threading tools - Explain safety precaution for using threading tools - Explain different type of Dies - Explain the procedure to use of threading die Practical Activity - Perform threading of pipe | Theory-1.0 Hrs Practical-02 Hrs Total- 3.0 Hrs | Plumbing pliers, Plumbing wrenches Plumbing spanners, PPEs Threading Tools | Class Room and workshop |
| - Perform Pipe Reaming. | Trainee must be able to: - Identify tools for Reaming. - Adopt safety measures - Fix work piece firmly - Clamp Threading pipe piece - Use reamer - Unclamp pipe piece - Observe OHSA | - Explain safety precaution for using reamer - Explain the procedure to use of Reamer Practical Activity - Perform reaming of pipe | Theory-0.5 Hrs Practical-01 Hrs Total- 1.5 Hrs | Plumbing pliers, Plumbing wrenches Plumbing spanners PPEs Reaming tools | Class Room and workshop |

### 0732B&CE-103. Make water connection

Objective: This module covers the knowledge and skills required to make the new connection for house from the main water.

Duration:  05 Hours								Theory: 01 Hours				Practice: 04 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Make water connection | Trainee will be able to: - Identify work place - Dig a pit on connection site to main water supply pipe. - Mount the clamp over the main water supply pipe. - Set the rubber seal in the clamp - Set the ferrule at the rubber seal under the clamp - Tighten the screws of the clamp with screw wrench - Mark hole in the main pipe with centre punch and hammer - Connect the socket on the ferrule - Connect the house pipe in the socket - Fit the sluice/gate and non-return valve in the house water supply line | - Describe different tools and equipment for making new connection for water supply - Explain taking connection procedure - Explain cutting and threading procedures - Explain PPEs - Explain uses of different tools - Explain clamps with its type - Explain use of valve while taking connection - Explain branch and main pipe Practical Activity Make water connection | Theory-0.75 Hrs Practical-3.5 Hrs Total- 4.25 Hrs | Pipe Cutting Tools, Deburrers OR Reamers Plumbing pliers, Plumbing wrenches Plumbing spanners Plungers, Pressure testing kits, PPEs Clamp for pipe Pipe (G.I OR PPRC) | Class Room and workshop |
| LU2. Clean-up work site area | Trainee will be able to: - Clear work area by disposing of materials (reused or recycled) in accordance to legislation and regulations - Clean, check, maintain and store tools and equipment according to requirement | - Information of faults and maintenance according to the workplace Practical Activity - Perform clean up | Theory-0.25Hrs Practical-0.5 Hrs Total- 0.75 Hrs |  | Class Room and workshop |

### 0732B&CE-104: Layout for domestic water supply

Objective: This module covers the knowledge and skills required for layout domestic water supply in residence and residential schemes.

Duration:  	05 Hours			Theory: 01 Hours				Practice: 04 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare for works | Trainee will be able to: - Interpret work instructions - Plan according to job requirements - Organise duties - Select appropriate tools & equipment |  | Theory-0 Hrs Practical-01 Hrs Total- 01 Hrs |  | workshop |
| - Lay out for water supply scheme | Trainee will be able to: - Mark North Point on drawing sheet - Measure the Scheme area. - Interpret layout plan by scale - Identify sources of water supply - Indicate Water Conveyance System. - Show storage Reservoirs - Locate Main, sub main and branches | - Define water Layout system. - Define fundamentals of water reservoirs - Explain Types of water sources - Describe applications of water connections with main to branches - Explain gravity and pressure supply of water | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs | - Pencil - Eraser - Sharpener - Calculator - Measuring scale - Drawing sheet - Drawing board - Tee Square - Set Square - Measuring Tape - Computer - AutoCAD - PPEs | Class Room and workshop |

### 0732B&CE-105. Join pipes & fittings

Objective: This module covers the knowledge and skills required to prepare for different type of pipe joints and it’s fitting on residential and commercial plumbing works.

Duration: 08 Hours							Theory: 03 Hours				Practice: 05 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Undertake pipe fitting and Installation | Trainee will be able to: - Observe basic principles for pipe fittings - Identify and select materials for task - Identify tools and equipment for task - Attach Pipe with fittings - Locate fixtures properly - Undertake pipe leakage test - Use PPEs | - Explain signs and symbols for water supply system - Explain method for different type of pipe joints - Explain different types of joints in pipe - Which test are performed for pipe leakage Practical Activity Installation of pipe fittings | Theory-0.42Hrs Practical-0.5 Hrs Total- 0.92 Hrs | Pipe Benders Pipe Cutting Tools, Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Pressure testing kits PPEs Hack Saw with blade Different Fittings Safety Valve PPRC pipe and fittings | Class Room and workshop |
| - Install Hot and Cold-Water Supply Pipes | - Trainee will be able to: - Observe basic principles for pipe fitting - Identify and select materials for task - Identify tools and equipment for task - Attach cold Pipe with fittings - Attach Hot Pipe with Geyser and fittings - Locate cross over properly - Locate drainage pipes - Undertake pipe leakage test - Use PPEs | - Knowledge of Safety requirements for installing cold and hot water pipes in residential buildings. - Knowledge of Fittings attach with hot water - Explain specifications for cold and hot water connections - Knowledge of faults in hot water pipes - Explain safety values use in geysers Practical Activity Installation of hot and cold water pipes | Theory-0.42Hrs Practical-01 Hrs Total- 1.42 Hrs | Pipe Bender Pipe Cutting Tools Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Pressure testing kits PPEs Hot water fittings Hot water accessories G.I. & Cost Iron Pipes according to specifications Pipe Vice Hack Saw with blade Different Fittings Geyser Safety Valve | Class Room and workshop |
| - Joining Cast iron pipes | - Trainee must be able to: - Place the spigot end of a pipe inside the bell end of another pipe making sure that both are clean and dry. - Inert the gasket into the hub of joint using yarning iron and then pack to the proper depth by using the packing iron. - Pour the molten lead into the joint up to the top of the hub. - Perform caulking after the lead has solidified and cooled - Align the pipe - Undertake pipe leakage test - Use PPEs | - Define cast iron pipes - Explain the equipment for installing and fittings of cast iron pipes - Usage of cast iron pipes - Name different types of cast iron pipes - Practical Activity - Joining of cast iron pipe | Theory-0.42Hrs Practical-01 Hrs Total- 1.42 Hrs | Pipe Benders Pipe Cutting Tools, Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Pressure testing kits PPEs Hack Saw with blade Different Fittings Safety Valve Cost Iron Pipe Lead Cocking Tools | Class Room and workshop |
| - Join G.I Pipes | - Trainee must be able to - Observe basic principles for G.I. pipes - Select materials for task - Identify tools and equipment for task - Threading & Cutting the Pipe - Tight pipe with socket - Undertake pipe leakage test - Use PPEs | - Define G.I pipes - Explain the equipment for installing and fittings of G.I pipes. - Define types of G.I pipes - Practical Activity - Joining of G.I pipes | Theory-0.42Hrs Practical-01 Hrs Total- 1.42 Hrs | Pipe Benders Pipe Cutting Tools, Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Pressure testing kits PPEs Hack Saw with blade Different Fittings Safety Valve G.I Pipes | Class Room and workshop |
| - Join PPRC Pipes | - Trainee must be able to - Observe basic principles for PPRC pipes - Adopt safety measures - Prepare welding machine with required dies of diameter. - Cut the pipes at right angles to the pipe axis using suitable cutter. - Remove any burrs or chips by cleaning the cutting area. - Heat the machine up to 260°C - Mark the welding depth on pipe - Insert the end of pipe into the heating sleeve up to the marked welding depth and mount the fitting also. - Leave the pipe and fitting on the heating tool until the heating time is elapsed (5 to 7 sec) - Remove the pipe and fitting from heating tool and push them immediately into each other up to marked welding depth without rotating. - Allow the joint to cool down | - Define PPRC pipes - Explain the equipment for installing and fittings of PPRC pipes. - Usage of PPRC pipes - Practical Activity - Joining of PPRC pipes | Theory-0.42Hrs Practical-0.5 Hrs Total- 0.92 Hrs | Pipe Benders Pipe Cutting Tools, Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Pressure testing kits PPEs Hack Saw with blade Different Fittings Safety Valve PPRC Pipe with fittings Cocking Tools | Class Room and workshop |
| - Joining PVC Pipes | - Clean the ends of pipes to be joined. - Make one end of pipe should be cup/bell/ socket type if they are not in required shape. - Apply jointing solution in the bell/socket and over the spigot end - Insert the spigot end of a pipe into bell/socket of another pipe | - Define PVC pipes and its joints - Explain the equipment for installing and fittings of pipes - Practical Activity - Joining of PVC pipes | Theory-0.42Hrs Practical-0.5 Hrs Total- 0.92 Hrs | Pipe Benders Pipe Cutting Tools, Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Pressure testing kits PPEs Hack Saw with blade Different Fittings PVC pipes Safety Valve | Class Room and workshop |
| - Joining Concrete Pipes | - Correctly position and bed the first pipe. - Prepare the bedding for the second pipe. - Apply the seal on the spigot end. - Ensure the joint ring is not twisted and correctly located on spigot. - Centre the socket/bell and fill the gap with cement mortar | - Define concrete pipes and its joints - Explain the equipment for installing and fittings of pipes - Explain drain Pipes - Practical Activity - Joining of concrete pipes | Theory-0.42Hrs Practical-0.5 Hrs Total- 0.92 Hrs | Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Pressure testing kits PPEs Different Fittings Safety Valve Concrete pipes | Class Room and workshop |

### 0732B&CE-106. Install plumbing fixtures

Objective: This module covers the knowledge and skills required to install plumbing fixtures at work place i-e installation of water closet, flushing cistern, Wash hand Basin, English WC and Urinal.

Duration: 06 Hours							Theory: 02 Hours				Practice: 04Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required |  | Learning Place |
| --- | --- | --- | --- | --- | --- | --- |
| - Install Wash Hand Basin | Trainee will be able to: - Identify safety requirements for installation - Identify tools for work - Identify Fixtures - Perform measurements - Mark out Installation point - Turn the water supply off of the concerned section. - Mark and drill pilot holes for the floor fixing screws. - Mark, drill and plug the wall holes for basin fixing screw - Install tee stop cock at the end of water supply pipe (hot and cold) - Connect the flexible rubber connection pipes with pillar cocks or mixture - Fit the coupling in the hole of basin. Fit floor trap under the waste pipe and adjust waste pipe with it. - Re-position the pedestal and screw it into proper place. - Position the basin and check its level then screw the basin to wall. - Inject silicon sealant for sealing. - Tighten the pedestal screws. - Turn on the water supply and check for leaks. | - Define hand basin and its types. - Use of tools and equipment’s for installing wash hand basin. - Knowledge of joining techniques for hand basin. - Explain wash hand basin Pedestal type - Explain fitting procedure to install wash hand basin Practical Activity - Installing wash hand basin | Theory-0.5 Hrs Practical-0.5 Hrs Total- 1.0 Hrs |  | Pipe Benders Pipe Cutting Tools Pipe vice with stand Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Plungers PPEs Wash hand basin Bolt Kit Screws Clamps Hammer Tee cock Pillar cock | Class Room and workshop |
| - Install English water closet | Trainee will be able to: - Mark out Installation point - Compact and Level the floor at the place of installation of water closet. - Place P-trap at compacted and levelled surface. - Place the W.C pot on the trap in levelled surface and provide support to it. - Fill the P-trap with cement and cloth bandage soaked in cement solution. - Connect soil pipe to P-trap - Connect tee with soil pipe outside the toilet wall. - Joint the vent pipe with small end of tee. - Connect larger end of soil pipe to septic tank. - Attach flush pipe in to the pot hole. - Drill holes at the required place and height for flushing cistern. - Install flushing with screws and joint it with flush pipe. - Fill the earth to required level. - Finish conglomerate floor or tile flooring | - Define water closet and its types - Use of tools and equipment’s for installing water closet. - Knowledge of joining techniques for water closet - Knowledge of levelling and fixing WC Practical Activity - Installing water closet | Theory-0.5 Hrs Practical-0.5 Hrs Total- 1.0 Hrs |  | Pipe Benders Pipe Cutting Tools Pipe vice with stand Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Plungers PPEs Bolt Kit Screws Clamps Hammer P-Trap W.C (Indian & English) Tee cock Pillar cock | Class Room and workshop |
| - Install Flushing Cistern | Trainee must be able to: - Mark out Installation point - Measure height for flushing cistern - Drill holes at the required place and height for flushing cistern. - Install flushing with screws and joint it with flush pipe - Fix flushing pipe - Connect inlet pipe with water supply - Perform final quality inspection - Check Level - Use PPEs | - Define flushing cistern with its types. - Use of tools and equipment’s for installing flushing cistern - Knowledge of installing techniques for flushing cistern Practical Activity - Install flushing cistern | Theory-0.25 Hrs Practical-0.75 Hrs Total- 1.0 Hrs |  | Pipe Benders Pipe Cutting Tools Pipe vice with stand Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Plungers PPEs Bolt Kit Flushing cistern Screws Clamps Tee cock Pillar cock | Class Room and workshop |
| - Install Urinal | Trainee must be able to - Mark out Installation point - Turn the water supply off of the concerned section. - Install adequate support for wall hangers and install wall hangers if needed. - Drill the holes at two feet height and drill the holes. - Determine the position for waste pipe nipple - Apply thread sealant tape. - Install female collar to the waste pipe. - Place the gasket on the female collar with the bevelled surface facing away from the collar. - Position the urinal on the wall hangers and screwed them. - Secure the urinal bottle trap and waste pipe to female collar using the screws. - Install the flush valve according to manufacturer instructions. - Connect the water supply and complete waste pipe connection. - Turn on water supply. - Flush and check the entire installation for leakage | - Define urinal and its types - Use of tools and equipment’s for installing urinal - Knowledge of installing techniques for urinal Practical Activity - Installing urinal | Theory-0.25 Hrs Practical-0.75 Hrs Total- 1.0 Hrs |  | Pipe Benders Pipe Cutting Tools Pipe vice with stand Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Plungers PPEs Bolt Kit Urinal Urinal with fitting Urinal Cistern | Class Room and workshop |
| - Install Sink | Trainee must be able to - Mark out Installation point - Measure height for sink - Install adequate support for wall hangers and install wall hangers if needed. - Fix sink fat measured height with drain pipe - Connect inlet pipe with water supply - Perform final quality inspection - Check Level - Conduct Operational Testing - Use PPEs | - Define sink and its types. - Use of tools and equipment’s for installing sink. - Knowledge of installing techniques for sink. Practical Activity - Installing sink | Theory-0.25 Hrs Practical-0.75 Hrs Total- 1.0 Hrs |  | Pipe Benders Pipe Cutting Tools Pipe vice with stand Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Plungers PPEs Bolt Kit Sink with mixer Tee cock Pillar cock | Class Room and workshop |
| - Install Bath Tub | - Mark out Installation point - Measure Height for bath tub - Set and level Bath Tub at designated place - Install a ledger board for supporting tub - Secure the tub to studs according to manufacturer instructions. - Attach drain to tub - Connect the overflow drain - Connect the drain pipe with P-trap - Connect inlet pipe with water supply - Perform leakage test - Perform Safety Precaution - Use PPEs | - Define bath tub and its types. - Use of tools and equipment’s for installing bath tub. - Knowledge of installing techniques for bath tub. - Practical Activity - Installing bath tub | Theory-0.25 Hrs Practical-0.75 Hrs Total- 1.0 Hrs |  | Pipe Benders Pipe Cutting Tools Pipe vice with stand Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Plungers PPEs Bolt Kit Shower Geyser Bath Tub with accessories Mallet Tee cock Pillar cock | Class room and workshop |

### 0732B&CE-107. Install & repair water pumps

Objective: This module covers the knowledge and skills required to install drinking water pumps and its repair work.

Duration:  05 Hours								Theory: 01 Hours				Practice: 04 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Install Air ejector Pump | Trainee will be able to: - Mark out Installation point - Perform Safety Precaution - Measure Pipes with fittings - Fix pump with suction & delivery pump - Connect with electricity - Perform leakage test - Use PPEs | - Knowledge of joining techniques and pipe fittings of air ejector pump - Knowledge of leakage in pipes and pumps - Explain working of air ejector pump Practical Activity - Installation of air ejector pump | Theory-0.25 Hrs Practical-1 Hrs Total- 1.25 Hrs | Pipe Bender Pipe Cutting Tools Chain Pipe Wrench Deburrers Plumbing pliers, Plumbing wrenches Plumbing spanners Plungers Pressure testing kits Radiator tools PPEs Air ejector Pump with fitting | Class Room and workshop |
| - Install Centrifugal Pump | Trainee will be able to: - Mark out Installation point - Perform Safety Precaution - Measure Pipes with fittings - Fix flanges with suction & delivery pipes - Connect with electricity - Perform leakage test - Use PPEs | - Knowledge of joining techniques and pipe fittings of centrifugal pump. - Knowledge of leakage in pipes and in centrifugal pumps - Explain working of centrifugal pump. Practical Activity - Installing centrifugal pump | Theory-0.25 Hrs Practical-1 Hrs Total- 1.25 Hrs | Pipe Bender Pipe Cutting Tools Chain Pipe Wrench Deburrers Plumbing pliers, Plumbing wrenches Plumbing spanners Plungers Pressure testing kits Radiator tools PPEs Centrifugal Pump with fitting. | Class Room and workshop |
| - Install Rotary Pump | Trainee must be able to: - Mark out Installation point - Perform Safety Precaution - Measure Pipes with fittings - Fix flanges with suction & delivery pipes - Connect with electricity - Perform leakage test - Use PPEs | - Knowledge of joining techniques and pipe fittings of rotary pump. - Knowledge of leakage in pipes and in rotary pumps - Explain working of rotary pump. Practical Activity - Installing rotary pump | Theory-0.25 Hrs Practical-1 Hrs Total- 1.25 Hrs | Pipe Bender Pipe Cutting Tools Chain Pipe Wrench Deburrers Plumbing pliers, Plumbing wrenches Plumbing spanners Plungers Pressure testing kits Radiator tools PPEs Rotary Pump with fitting. Electricity | Class Room and workshop |
| - Carry out repair of Pump | Trainee must be able to - Perform Safety Precaution - Disconnect electricity and pump - Use PPEs - Check for Fault - Repair it - Refit with suction & delivery pipes - Connect with electricity - Perform leakage test | - Explain repair for different pumps. - Explain safety measures while performing repair of works - Explain leakage checking with its methods - Knowledge of leakage in pipes and pumps Practical Activity - Repairing of different pumps | Theory-0.25 Hrs Practical-01 Hrs Total- 1.25 Hrs | Pipe Bender Pipe Cutting Tools Chain Pipe Wrench Deburrers Plumbing pliers, Plumbing wrenches Plumbing spanners Plungers Pressure testing kits Radiator tools PPEs Different Pumps with fittings Electricity | Class Room and workshop |

### 0732B&CE-108. Install turbine pumps

Objective: This module covers the knowledge and skills required to install turbine

Duration: 07 Hours								Theory: 02 Hours				Practice: 05 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Install turbine | Trainee will be able to: - Mark out Installation point - Use PPEs - Check depth of bore - Adopt Safety Measures - Measure turbine Pipes with fittings - Fix Turbine with flange & Pipes - Insert turbine into bore with tripod & chain pulley - Connect Motor with shaft fittings - Connect electricity and start - Remove tripod - Check discharge | - Define turbines and its types. - Explain different joining techniques and pipe fittings - Knowledge of leakage testing - Explain turbine impellers with its fittings - Explain installation method into bore. - Explain tripod and chain pulley function | Theory-1.5 Hrs Practical-04 Hrs Total- 5.5 Hrs | Pipe Cutting Tools Chain Pipe Wrench Deburrers Plumbing pliers Plumbing wrenches Plumbing spanners Plungers Pressure testing kits Radiator tools PPEs Turbine with impellers & Motor Electricity Turbine pipe with fittings Pipe wooden / steel clamps | Class Room and workshop |
| LU2. Clean-up work site area | - Clear work area by disposing of materials (reused or recycled) in accordance to legislation and regulations - Clean, check, maintain and store tools and equipment according to requirement | - Information of faults and maintenance according to the workplace Practical Activity - Perform clean up | Theory-0.5 Hrs Practical-01 Hrs Total- 1.5 Hrs |  | Class Room and workshop |

## 9.10.  Digital Skills

.

- Install computer operating systems and hardware

Objective: This module covers the performance outcomes, skills and knowledge required to select, configure and use computer operating systems and basic computer hardware

Duration:  2.0 Hours							Theory: 1.0 Hours				Practice: 1.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Identify operating system and hardware components | Trainee will be able to: - Determine ICT organizational requirements and specifications - Identify and select operating system - Identify appropriate external hardware components - 1.4 Identify internal hardware components | - Introducing HSE - Preparing working environment - Connecting PC to electricity - Connecting multimedia - Setting out work - Installing CAD Software Practical Activity prepare a short report on the hardware operating system along with its components. | Theory0.5Hrs Practice-0.5 Hrs Total- 1.0 Hrs | Drawing Sheet Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse Data cables Printer USB Multimedia | Computer Lab |
| Install and configure operating system and application software with hardware components | Trainee will be able to: - Install and configure operating system to meet organizational requirements - Identify the functions associated with the operating system and associated boot process - Configure power-management settings to minimize power consumption as an environmentally sustainable measure - Use both the graphical user interface and the command line interface to perform basic tasks - Install or upgrade application software onto the operating system and hardware configuration - Determine the relationship between an application program, the operating system and hardware - 2.7 Identify general differences between the different computer platforms and their respective operating systems | - Understanding Operating systems - Procedure of Installation the software application. - Connecting system to electric supply - Introducing and Connecting data cables - Troubleshooting the application - Troubleshooting the software - Ensuring HSE Practical Activity Configure C.P.U System before installing software. | Theory0.25Hrs Practice-.25 Hrs Total- 0.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Optimize operating system and hardware components | - Optimize operating system using included tools or third-party utilities - Customize the graphical user interface - Use techniques unique to the command line interface - Set up and configure external hardware components and check functionality - Install drivers as appropriate and check functionality | - Understand Operating systems. - Describing procedure of Installation of software applications. (MS Word) - Describing Hardware and Software. - Accessing a Data traveler. - Troubleshooting hardware and software problems. Practical Activity Conduct Optimize operating system | Theory0.25Hrs Practice-0.25Hrs Total- 0.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |

- Operate digital media technology

Objective: This module covers the performance outcomes, skills and knowledge required to identify, select and use a digital media package and supporting technologies.

Duration: 1.0 Hours						Theory: 0.5 Hours					Practice: 0.5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Use appropriate OHS office work practices | Trainee will be able to: - Use safe work practices to ensure ergonomic, work organization, energy and resource conservation requirements are addressed - Use wrist rests and document holders where appropriate - Use monitor anti-glare and radiation reduction screens where appropriate | - Basic for OHS - Apply OHS - OHS principles - Responsibilities for ergonomics, - Ordinates setting Practical Activity Practice for OHS in lab and office | Theory 1.0Hrs Practice-2 Hrs Total- 3.0 Hrs | Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse Data cables Printer USB Multimedia | Computer Lab |
| Identify and select appropriate digital media package | Trainee will be able to: - Identify the basic requirements of a design brief, - Check user environment - Research and review suitable available digital media packages - Select an appropriate digital media package to meet design brief requirements | - Set U.C.S for different situations - Understanding Basic Commands - support design brief requirements. Practical Activity Perform formation of digital media package | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Use digital media package | Trainee will be able to: - Procure or create suitable data to meet requirements of the brief - Manipulate data using digital media package tools - Ensure naming and storing of documents in appropriate file format in directories or | - Defining media package - Configure media package - Setting up digital media tools. - Principles of digital imaging - tools - video and sound file formats, file management and transfer systems Practical Activity Create data using digital media tools. | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Review digital media design | - Evaluate design for creative, dramatic and technical quality, file size, and suitability to meet the brief - Test and run any incorporated graphics, video or sound as part of a digital media presentation and present designs in the appropriate format - Review final product against design brief | - Defining media design package - Configure media design package - Setting up design media tools. - Visual Design - file management and transfer systems for digital media Practical Activity - Create data using digital media design tools. | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse Data cables Printer USB Multimedia |  |

- Perform computer operations

Objective: This module covers the knowledge, skills and attitudes and values needed to perform computer operations which include inputting, accessing, producing and transferring data using the appropriate hardware and software.

Duration: 2.0 Hours							Theory: 1.0 Hours				Practice: 1.0 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Plan and prepare for task to be undertaken | Trainee will be able to: - Requirements of task are determined as per standard - operating procedures - Appropriate hardware and software is selected according to task assigned and required outcome - 1.3 Task is planned to ensure | - Drafting settings (Unites, Limits, Snap - Auto (On & Off) Snap setting with their application - Layer options with their application. Practical Activity Perform different task on operating system. | Theory0.5Hrs Practice-.2 Hrs Total- 0.5 Hrs | Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Input data into computer | Trainee will be able to: - Data are entered into the computer using appropriate program/application in accordance with company procedures - Accuracy of information is checked and information is saved in accordance with standard operating procedures - Inputted data are stored in storage media according to requirements - Work is performed within ergonomic guidelines | - Importance of input data into PC - Text Input Data - Style / type of input data Practical Activity Perform input data in to PC | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Access information using computer | Trainee will be able to: - Correct program/application is selected based on job requirements - Program/application containing the information required is accessed according to company procedures - Desktop icons are correctly selected, opened and - closed for navigation purposes - Keyboard techniques are carried out in line with OH & S requirements for safe use of keyboards | - Define access information - Attach data cables with CPU - Importance of access information Practical Activity Create Access information using PC. | Theory1.0 Hrs Practice-2 Hrs Total- 3.0 Hrs | Printing Papers Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse Data cables Printer USB Multimedia | Computer Lab |
| Produce/output data using computer system | Trainee will be able to: - Entered data are processed using appropriate software commands - Data are printed out as required using computer hardware/peripheral devices in accordance with standard operating procedures - Files and data are transferred between compatible - systems using computer software, hardware/ peripheral - devices in accordance with standard operating procedures | - Define output data information - Create information data file - Importance of output data Practical Activity - Create output data by using input | Theory1.0 Hrs Practice-2 Hrs Total- 3.0 Hrs | Printing Papers Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse Data cables Printer USB Multimedia | Computer Lab |
| Maintain computer equipment and systems | Trainee will be able to: - Systems for cleaning, minor maintenance and replacement of consumables are implemented - Procedures for ensuring security of data, including regular back-ups and virus checks are implemented in accordance with standard operating procedures - Basic file maintenance procedures are implemented in line with the standard operating procedures | - Define PC equipment system - Maintain PC with its different equipment - Importance of Maintaining PC equipment’s Practical Activity - Maintain computer system | Theory1.0 Hrs Practice-2 Hrs Total- 3.0 Hrs | Printing Papers Duster A4-Papers Soft wares Tools C.P.U & LCD Key board Mouse Data cables Printer USB Multimedia | Computer Lab |

- Use computer applications

Objectives: This module covers the performance outcomes, skills and knowledge required to identify, select and operate three commercial software packages, including a word-processing, a spreadsheet and presentation application package.

Duration: 1.0 Hours							Theory: 0.5 Hours				Practice: 0.5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Use appropriate OHS office work practices | Trainee will be able to: - Use safe work practices to ensure ergonomic, work organization, energy and resource conservation requirements are addressed - Use wrist rests and document holders where appropriate - Use monitor anti-glare and radiation reduction screens where appropriate | - Basic for OHS - Apply OHS - OHS principles - Responsibilities for ergonomics, - Ordinates setting Practical Activity Perform basic work practice adopting OHS. | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Install and remove software | Trainee will be able to: - Select software to be installed - Follow installation instructions - Delete unrequired software | - Define installation. - Describe importance of installation. - Software installation command - Removal of installation - Differentiate between un-installation and removal Practical Activity Practice to install and un-install the software. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Use appropriate word-processing software | Trainee will be able to: - Select word-processing software appropriate to perform activity - Identify document purpose, audience and presentation requirements, and clarify with personnel as required - Identify organizational requirements for text-based business documents and design document structure and layout to ensure consistency of style and image - Match document requirements with software functions to provide efficient production of documents - Use technical functions, other data and formatting to finalize documents - Ensure the naming and storing of documents in appropriate directories or folders and the printing of documents to the required specifications | - Apply word processing. - Use modify commands and tools. - Install Word software in PC. - Use COMMANDS to un-install the software - Features of installation - Installation steps for Word Practical Activity Install and un-install MS word SOFTWARE into PC. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Use appropriate spreadsheet software | Trainee will be able to - Select spreadsheet software appropriate to perform activity - Identify document purpose, audience and presentation requirements, and clarify with personnel as required - Enter simple formulas and functions using cell referencing where required - Customize spreadsheet settings and format documents to meet requirements - Ensure the naming and storing of documents in appropriate directories or folders and the printing of documents to the required specifications | - Apply MS. Excel. - Use modify commands and tools. - Install MS. Excel software in PC. - Use COMMANDS to un-install the software - Features of installation - Installation steps for MS. Excel Practical Activity Install and un-install MS Excel software into PC. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Use appropriate presentation software | Trainee will be able to - Select software application package appropriate to perform activity - Identify purpose, audience and presentation requirements, and clarify with personnel as required - Use technical functions, other data and formatting to finalize documents - Ensure documents are named and stored in appropriate directories or folders and printed to required specifications - Make a presentation | - Defining presentation - Presentation installation software - Define power point - Install power point - Use COMMANDS to un-install the software - Features of installation Practical Activity Install and un-install MS power point software into PC. | Theory0.5Hrs Practice-2 Hrs Total- 2.5 Hrs | Drawing Sheets Duster A4-Papers Soft wares Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |

- Create user documentation

Objective: This module covers the performance outcomes, skills and knowledge required to create user documentation that is clear to the target audience and easy to navigate.

Duration: 1 Hours								Theory: 0.50 Hours			Practice: 0.50 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Determine documentation standards and requirements | Trainee will be able to: - Determine documentation requirements - Investigate documentation and industry standards for requirements and determine appropriate application to user documentation - Design documentation templates using appropriate software and obtain approval from appropriate person | - Define standards. - Differentiate between documents and documentation - Documentation requirements - Documents design Practical Activity Prepare documentation standards along with its requirements. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Duster Pencils Eraser Papers Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Produce user documentation | Trainee will be able to: - Conduct a review of the subject system, program, network or application in order to understand its functionality - Gather existing technical, design or user specifications and supporting documentation - Create user documentation based on template to record the operation of the subject system, program, network or application | - Produce standards. - Differentiate between documents and documentation - Documentation design Practical Activity Prepare user documentation standards along with its requirements. | Theory0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Review and obtain sign-off | Trainee will be able to: - Submit user documentation to target audience for review - Gather and analyze feedback - Make changes to user documentation - Submit user documentation to appropriate person for approval | - Differentiate between documents and documentation - Documentation design - Audience to review - Define feedbacks Practical Activity Practice for Review and obtain sign-off. | Theory-0.5Hrs Practice-3 Hrs Total- 3.5 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |

- Create technical documentation

Objective: This module covers the performance outcomes, skills and knowledge required to create technical documentation that is clear to the target audience and easy to navigate.

Duration: 1.0 Hours							Theory: 0.50 Hours				Practice: 0.50 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Identify and analyze documentation requirements and client needs. | Trainee will be able to: - Consult with client to identify documentation requirements - Interpret and evaluate documentation requirements and confirm details with client - Investigate industry and documentation standards for requirements - Define and document the scope of work to be produced - Consult with client to validate and confirm the scope of work | - Define documentation - Evaluate Documentation requirements - Industry documentation standards - Office documentation standards - Client needs for documentation Practical Activity Practice to analyze client needs document requirements. | Theory0.5Hrs Practice-.5 Hrs Total- 1 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Design documentation | Trainee will be able to: - Identify information requirements with reference to layout and document structure - Create document templates and style guides consistent with information requirements - Conduct a review of the system in order to understand its functionality - Extract content that meets information requirements according to copyright restrictions - Develop the structure of the technical documentation giving focus to the flow of information, style, tone and content format - Validate the technical documentation structure with the client | - Differentiate between documents and document structure - Documentation design templates - Copy rights for documentation - Define feedbacks - Content that meets information requirements - Technical documentation Practical Activity Practice for Prepare design documents. | Theory0.5Hrs Practice-.25Hrs Total- 0.5 Hrs | Duster Pencils Eraser Papers Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Develop documentation | Trainee will be able to: - Write technical documentation based on the template and scope of work using the information gathered - Translate technical terminology into plain English where appropriate - Apply content format and style according to documentation standards and templates | - Style for design documentation - Documentation design templates - Copy rights for documentation - Content for documentation - Technical documentation Practical Activity Practice for Develop design documents. | Theory0.5Hrs Practice-.5 Hrs Total- 1.0 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Evaluate and edit documentation | Trainee will be able to: - Submit technical documentation to appropriate person for review - Gather and analyze feedback - Incorporate alterations into the technical documentation - Edit the technical documentation for technical and grammatical accuracy | - Style for evaluation documentation - Documentation evaluation design templates - Edit documentation - Analyze documentation - grammatical accuracy Practical Activity Practice for evaluate the documents. | Theory0.5Hrs Practice-0.5 Hrs Total- 1 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Prepare documentation for publication | Trainee will be able to - Check that the completed technical documentation meets client requirements and scope of work - Submit the technical documentation to appropriate person for approval - Prepare the technical documentation for publication and distribution using appropriate channels | - evaluation for documents - Documentation evaluation design - Edit public documentation - Analyze public documentation - grammatical and technical accuracy - client feedback Practical Activity Perform documentation for publication. Prepare structural detail of spread footing, pillar, foundations & column base. | Theory 0.5 Hrs Practice-0.5 Hrs Total- 1 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |

- Create basic databases

Objective: This module covers the skills and knowledge required to design, develop and test a database in order to meet a specification. It applies to individuals who may be either database, or web designers, required to create a simple database to store information for an online application, using a simple entity relational database.

Duration:  1 Hours							Theory: 0.5 Hours				Practice: 0.5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Analyze the requirements for the database | Trainee will be able to: - Determine the information that the database is required to hold - Develop a written requirement report for the functionality of the database - Complete the documentation, and submit it to the appropriate person for approval | - Knowledge of principles of open platforms - Knowledge of creation of entities, attributes, and populating - Knowledge of creation of entities, attributes, and populating Practical Activity Perform database | Theory-0.125 Hrs Practice-0.125 Hrs Total- 0.25 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Use data modeling to design the database to suit requirements | Trainee will be able to: - Design an entity-relationship (ER) diagram to model the relationships between the entities and the attributes that the database will hold - Develop primary and foreign keys to link the entities - Develop a data dictionary - Complete the documentation, and submit it to the appropriate person for approval | - Knowledge of data-modeling techniques - Knowledge of database design, modeling and implementation Practical Activity Perform data modeling to design the database to suit requirements | Theory-0.125 Hrs Practice-0.125 Hrs Total- 0.25 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Create a database on a web or database server | Trainee will be able to: - Use the appropriate language on a web or database server to create one or more databases - 3.2 Use the appropriate language on a web or database server to create tables - 3.3 Populate the database fields | - Knowledge of internet operation related to web servers and clients - Knowledge of naming conventions appropriate to database design Practical Activity Perform database on a web or database server | Theory-0.125 Hrs Practice-0.125 Hrs Total- 0.25 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| Test the database and debug | Trainee will be able to: - Test the database on the web or database server - Ensure that the information represented matches the requirements | - Knowledge of security restrictions on servers - Knowledge of practice communication, and accessibility, for audiences with special needs. Practical Activity Perform database and debug | Theory-0.125 Hrs Practice-0.125 Hrs Total- 0.25 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |

- Create basic databases

Objective: This unit describes the performance outcomes, skills and knowledge required to establish a social networking presence using social media tools and applications. The unit specifically identifies the requirement to review, compare and use different types of social networking tools and applications.

Duration:  2 Hours							Theory: 1 Hours				Practice: 1 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Describe different types of social media tools and applications | Trainee will be able to: - Explain characteristics of the term social media - 1.2 Identify different types of social-media tools and applications - Illustrate some of the issues associated with the use of social media tools and applications | - Knowledge basic technical terminology in relation to social networking and social media application - Knowledge of uploading images, text files, pdf files, audio files, video files - Knowledge of features and functions of social media applications - Knowledge of import and export software functions Practical Activity Perform different types of social media tools and applications | Theory-0.33 Hrs Practice-0.33Hrs Total- 0.66 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| - Compare different types of social media tools and applications | Trainee will be able to: - Select one social media type for review - Review most popular tools and applications within that social media type - Itemize benefits across a range of the most popular tools and applications - Select most appropriate social media tool or application | - Knowledge of Linking documents - Knowledge of OHS principles and responsibilities for ergonomics, including work periods and breaks - Knowledge of tagging to facilitate collaborative folksonomy Practical Activity Perform different types of social media tools and applications | Theory-0.33 Hrs Practice-0.33Hrs Total- 0.66 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |
| - Set up and use popular social media tools and applications | Trainee will be able to: - Identify social media tools and applications for possible implementation - Initiate preferred social media tools and applications for use - Establish social media interface using text and file content - Initiate social networking interaction - Test and evaluate tools and applications for ease of use - Present findings | - Knowledge of social media applications and procedures - Knowledge of use of input and output devices - Knowledge of use of RSS feeds to connect a social network. Practical Activity Perform popular social media tools and applications | Theory-0.33 Hrs Practice-0.33Hrs Total- 0.66 Hrs | Duster Pencils Eraser Sharpener Tools C.P.U Key board Mouse Data cables LCD Printer USB Multimedia | Computer Lab |

- E-Commerce- SEO (Search Engine Optimization)

Objective: This module covers the knowledge and  skills required to develop efficient E-Marketing strategies in accordance with the Vision and Mission statement of the organization driven by Electronic means

Duration:  01 Hours			Theory: 0.5 Hours				Practice: 0.5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify SEO Techniques | Trainee will be able to: - Apply SEO techniques - Employ SEO key words | - SEO Methods - SEO key words for web pages translation Practical Activity - Apply SEO techniques | Theory-0.25 Hrs Practical-0 .25Hrs Total- 0.5 Hrs | - Computer | Class Room and workshop |
| - Indicate SEO (Search Engine Optimization) | Trainee will be able to: - Demonstrate SEO techniques to priorities their site or web application using automated tools - Develop efficient E-marketing strategies | - White-hat, Black-hat SEO techniques for web application - SEO tools usage Practical Activity Identify SEO on web application | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs | - Computer | Class Room and workshop |

- E-Commerce- SCM (Supply Chain Management)

Objective: This module covers the knowledge and  skills required to develop efficient E-Marketing strategies in accordance with the Vision and Mission statement of the organization driven by Electronic means

Duration: 01 Hours			Theory: 0.5 Hours				Practice:  0.5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify SCM (Supply Chain Management) | Trainee will be able to: - Identity potential Suppliers - Select the appropriate supplier - Place order as per requirement/inventory - Inspect received order | - Procurement Cycle (Launch of RFP/RFQ, Tender, Bidding, Comparative Statement, Award of Contract, Maintenance) - Product delivery and their traceability Practical Activity Identify supply chain management | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs | Computer | Class Room and workshop |
| - Prepare updated inventory details | Trainee will be able to: - Maintain Inventory as per Inventory Control / store keeping techniques - Identity different available transportation mode - Identify steps of reverse SCM i-e from consumer to organization | - Techniques to manage - Incorporation of Outsourcing in logistics - Electronic Data Interchange methodologies and format Practical Activity Prepare inventory | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs | Computer | Class Room and workshop |

- E-Commerce- Social Media Marketing

Objective: This module covers the knowledge and  skills required to  develop efficient E-Marketing strategies in accordance with the Vision and Mission statement of the organization driven by Electronic means

Duration: 01 Hours			Theory: 0.5 Hours				Practice: 0.5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Media Marketing Techniques | Trainee will be able to: - Identify different Social media marketing techniques - Apply suitable Classified Advertisement techniques on social media | - Knowledge of different social media sites that is Facebook, Twitter, LinkedIn, Google+ etc., Comparative Statement, Award of Contract, Maintenance) - Skills to regularly update brand/product/service blogs Practical Activity Perform media marketing techniques | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs | - Computer - Internet | Class Room and workshop |
| - Develop marketing techniques | Trainee will be able to: - Perform Electronic Mail Marketing - Creation of Blogs | - Brand pages creation on social media sites - Direct marketing techniques e.g. Email, SMS (Mobile- Commerce) for the projection of company newsletters Practical Activity - Create blogs and e-marketing | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0 .5Hrs | - Computer - Internet | Class Room and workshop |

- Use digital devices

Objective: This module covers the knowledge and  skills required to describes the skills and knowledge required to use a range of digital devices, such as a digital camera, video camera, or personal digital assistant (PDA) device. It applies to individuals who require entry level information and communications technology (ICT) knowledge and literacy skills to support their work in a home office or small office environment.

Duration:  02 Hours			Theory: 01 Hours				Practice: 01 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare to use the digital device | Trainee will be able to: - Review the instruction manual and ensure identified components are available - Identify the physical components of the digital device - Turn on and follow access procedures to activate the digital device - Alter the digital device settings to best suit intended use - . Configure power management settings where appropriate to minimize power consumption, as an environmentally sustainable measure | - audio-visual devices - peripheral devices - storage devices Practical Activity Use digital device | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs | - Computer - Digital devices | Class Room and workshop |
| - Set up and use the digital device | Trainee will be able to: - Identify and set the basic operating, security and menu settings - Navigate and manipulate the screen environment - Customize screen icons and access to applications where applicable - Use the digital device, and save and edit output where applicable - Identify more advanced features available and use as required | - basic security functions - basic software operation and associated applications Practical Activity Set up digital device | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs | - Computer - Digital devices | Class Room and workshop |
| - Access and use basic connectivity devices | Trainee must be able to: - Connect to external digital devices, such as computer devices or storage devices, to retrieve, copy, move and save information - Check physical connectivity of computer devices or storage devices to ensure operation and performance - Connect to a printer either through a computer device or directly, and use printer settings and print data - . Access audio-visual devices to view and play a multimedia file | - digital device functions - Digital device settings. Practical Activity Use basic connectivity devices | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs | - Computer - Digital devices | Class Room and workshop |
| - Shut down digital device | Trainee must be able to - Save current work and back up important data - Close open programs on the digital device and any computer device or storage device - Shut down digital devices, according to manufacturer instructions | - Device shutdown - Safety precautions Practical Activity Shut off digital device. | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs | - Computer - Digital devices | Class Room and workshop |

- Operate word-processing applications

Objective: This module covers the knowledge and  skills required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility

Duration:  02 Hours			Theory: 01 Hours				Practice: 01 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Apply workplace health and safety (WHS) practices | Trainee will be able to: - Use workplace ergonomic work practices and strategies - Organize work area to ensure an ergonomic work environment | - Workplace health safety practices - Strategies for ergonomic work Practical Activity Apply WHS to workplace | Theory-0.14 Hrs Practical-0.14 Hrs Total- 0.28 Hrs |  | Class Room and workshop |
| - Create documents | Trainee will be able to: - Open word-processing application, create document and add data according to information requirements - Use document templates as required - Use simple formatting tools when creating the document - Save document to directory | - formatting styles - readability and appearance of documents Practical Activity Develop document using MS word | Theory-0.14 Hrs Practical-0.14 Hrs Total- 0.28 Hrs | - Computer - MS Word | Class Room and workshop |
| - Customize basic settings to meet page layout conventions | Trainee must be able to: - Adjust page layout to meet information requirements - Open and view different toolbars - Change font format to suit document purpose - Change alignment and line spacing according to document information requirements - Modify margins to suit the document purpose - Open and switch between several documents | - Settings for page layout - Customization of page layout Practical Activity Customize page according to requirement | Theory-0.14 Hrs Practical-0.14 Hrs Total- 0.28 Hrs | - Computer - MS Word | Class Room and workshop |
| - Format documents | Trainee must be able to - Use formatting features and styles as required - Highlight and copy text from another area in the document or from another active document - Insert headers and footers to incorporate necessary data - Save document in another file format - Save and close document to a storage device | - formatting features - Highlight and copy text - headers and footers - Save and close document Practical Activity Prepare a document by inserting header, footer, highlighting text and saving file. | Theory-0.14 Hrs Practical-0.14 Hrs Total- 0.28 Hrs | - Computer - MS Word | Class Room and workshop |
| - Create tables | Trainee must be able to - Insert standard table into document - Change cells to meet information requirements - Insert and delete columns and rows as necessary - Use formatting tools according to style requirements | - Insert standard table - Insert and delete columns and rows Practical Activity Prepare a time table of your activity using MS word | Theory-0.14 Hrs Practical-0.14 Hrs Total- 0.28 Hrs | - Computer - MS Word | Class Room |
| - Add images | - Insert appropriate images into document and customize as necessary - Position and resize images to meet document formatting needs | - Insert images - Position and resize images Practical Activity Add images to a MS word file | Theory-0.14 Hrs Practical-0.14 Hrs Total- 0.28 Hrs | - Computer - MS Word |  |
| - Print documents | - Preview document in print preview mode - Select basic print settings - Print document or part of document from printer | - Print settings - Preview document - Print document Practical Activity Print a file | Theory-0.14 Hrs Practical-0.14 Hrs Total- 0.28 Hrs | - Computer - MS Word - Print |  |

- Operate spreadsheet applications

Objective: This module covers the knowledge and  skills required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility

Duration:  02 Hours			Theory: 01 Hours				Practice: 01 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Create presentations | Trainee will be able to: - Open presentation package and create a simple design for a presentation according to organizational requirements - Open blank presentation and add text and graphics - Apply existing styles within a presentation - Use presentation template and slides to create a presentation - Use various tools to improve the look of the presentation - Save presentation to the appropriate storage device and folder | - Formal and informal presentations - Outline presentation pitfalls Practical Activity Prepare a presentation | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | - Computer - MS office | Class Room and workshop |
| - Customize basic settings | Trainee will be able to: - Adjust display to meet user requirements - Open and view different toolbars to view options - Ensure font settings are appropriate for the presentation purpose - View multiple slides at once | - Technical terminology to read help files and prompts - Font settings Practical Activity Customize file using basic settings | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | - Computer - MS office | Class Room and workshop |
| - Format presentations | Trainee must be able to: - Use and incorporate organizational charts and bulleted lists, and modify as required - Add objects and manipulate to meet presentation purposes - Import objects and modify for presentation purposes - Modify slide layout, including text and colors, to meet presentation requirements - Use formatting tools as required within the presentation - Duplicate slides within and across a presentation - Reorder sequence of slides and delete slides for presentation purposes - Save presentation in another format - Save to storage device and close presentation | - Design and formatting on the readability and usability of presentations - Save presentation Practical Activity Format a prepared presentation | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | - Computer - MS office | Class Room and workshop |
| - Add slide show effects | Trainee must be able to - Incorporate pre-set animation and multimedia effects into presentation as required to enhance the presentation - Add slide transition effects to presentation to ensure smooth progression through the presentation - Test presentation for overall effect - Use onscreen navigation tools to start and stop slide show or move between different slides as required | - Identify suitable presentation effects for different audiences. - Use onscreen navigation tools Practical Activity Present prepared work using slide show | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | - Computer - MS office | Class Room and workshop |
| - Print presentation and notes | Trainee must be able to - Select appropriate print format for presentation - Select preferred slide orientation - Add notes and slide numbers - Preview slides and run spell check before presentation - Print selected slides and submit presentation to appropriate person for feedback | - Print settings - Preview document - Print document Practical Activity - Print a file | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | - Computer - MS office - Printer | Class Room And workshop |

- Operate presentation packages

Objective: This module covers the knowledge and  skills required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility.

Duration:  	02 Hours			Theory: 01 Hours				Practice: 01 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Create spreadsheets | Trainee will be able to: - Open the spreadsheet application, create spreadsheet files and enter numbers, text and symbols into cells according to information requirements - Enter simple formulas and functions using cell referencing when required - Correct formulas when error messages occur - Use a range of common tools during spreadsheet development - Edit columns and rows within the spreadsheet - Use the auto-fill function to increment data where required - Save the spreadsheet to a folder on a storage device | - Spreadsheet application - Create spreadsheet files - Formulas and functions - Function of spreadsheet applications Practical Activity Prepare a spreadsheet document | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | - Computer - MS Office | Class Room and workshop |
| - Customize basic settings | Trainee will be able to: - Adjust page layout to meet user requirements or special needs - Open and view different toolbars - Change font settings so they are appropriate for the document purpose - Change alignment options and line spacing according to spreadsheet formatting features - Format cell to display different styles as required - Modify margin sizes to suit the purpose of the spreadsheets - View multiple spreadsheets concurrently | - Terminology related to reading help files and prompts - Font settings - Format cells Practical Activity Prepare a customize file using basic settings | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | - Computer - MS Office | Class Room and workshop |
| - Format spreadsheet | Trainee must be able to: - Use formatting features as required - Copy selected formatting features from another cell in the spreadsheet or from another active spreadsheet - Use formatting tools as required within the spreadsheet - Align information in a selected cell as required - Insert headers and footers using formatting features - Save spreadsheet as another file type - Save to storage device and close spreadsheet | - Effect of formatting and appearance on the readability and usability of spreadsheets - Formatting tools - Saving file Practical Activity Format a prepared spreadsheet | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | - Computer - MS Office | Class Room and workshop |
| - Incorporate object and chart in spreadsheet | Trainee must be able to - Import an object into an active spreadsheet - Manipulate imported object by using formatting features - Create a chart using selected data in the spreadsheet - Display selected data in a different chart - Modify chart using formatting features | - Import objects - Create charts Practical Activity Add objects and charts in spreadsheet | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | - Computer - MS Office | Class Room and workshop |
| - Print spreadsheet | Trainee must be able to - Preview spreadsheet in print preview mode - Select basic printer options - Print spreadsheet or selected part of spreadsheet - Submit the spreadsheet to appropriate person for approval or feedback | - Print settings - Preview document - Print document Practical Activity - Print a file | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | - Computer - MS Office - Printer | Class Room |

- Perform writing and editing tasks

Objective: This module covers the knowledge and  skills required to apply the conventions of plain English to writing and editing tasks of different forms. It also includes editing and proofreading techniques. It applies to individuals in various writing contexts who write and edit texts using appropriate language, style, grammar, spelling, and standard conventions for editing and proofreading.

Duration:  02 Hours			Theory: 01 Hours				Practice: 01 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Apply clear and appropriate language and style to writing and editing tasks | Trainee will be able to: - Use safe work practices including addressing ergonomic requirements when undertaking writing tasks - Use clear, concise and plain English in writing and editing tasks - Apply appropriate paragraph structure to written material to ensure clarity of meaning and ease of reading - Make clear and logical connections between sentences, paragraphs and sections - Determine and incorporate the language and style of the audience | - Features of clear, concise and plain English language Practical Activity Use appropriate language and style to writing and editing | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs |  | Class Room and workshop |
| - Apply the appropriate voice, tone and tense | Trainee will be able to: - Determine appropriate voice, tone and tense of the written materials according to audience requirements - Maintain consistent voice, tone and tense throughout written material | - Voice, tone and tense for audience. Practical Activity Use appropriate voice, tone and tense | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs |  | Class Room and workshop |
| - Apply appropriate grammar, spelling and punctuation | Trainee must be able to: - Apply appropriate grammar conventions to a range of written contexts including use of numbers, quotations, and tables - Apply appropriate spelling and punctuation conventions in writing and editing tasks. | - Punctuation and spelling conventions Practical Activity Use appropriate grammar, spelling and punctuation | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs |  | Class Room and workshop |
| - Perform editing and proofreading tasks to meet requirements | Trainee must be able to - Edit written material to ensure clear meaning through language and paragraphs, consistent voice, tone and tense - Copyedit written material by checking grammar, spelling and punctuation using standard editing conventions - Proofreading using style guides and by monitoring written material for errors | - Editing conventions - copyediting of written material - Proofreading Practical Activity Perform editing and proofreading | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs |  | Class Room and workshop |

- Write and Edit Copy

Objective: This unit is about writing and editing copy (i.e. final version of text) for a range of formats in different media

Duration:  1 Hours							Theory: 0.5 Hours				Practice: 0.5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Undertaking copy-writing for different media | Trainee will be able to: - Present the facts and information in a manner that is coherent, comprehensible and appropriate for the target audience - Write and edit items - Craft an engaging narrative, conceptualize and clarify ideas - Display strong command of the language including correct grammar, spelling, sentence construction, diction and pronunciation skills - Ensure that finished scripts meet legal and regulatory norms, and do not pose any risks to the individual and/ or organization | - Knowledge of editorial standards - Knowledge of resource limitations - Knowledge of structure one’s thoughts and ideas Practical Activity Perform copy writing for different media | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs | Computer | Class Room and workshop |
| - Undertaking script-writing for various media and types of stories | Trainee will be able to: - Edit a story or script based on the brief and prescribed word/ time limits - Write and edit items such as: headlines, captions, intros, outros, cues and other types of links - Craft an engaging narrative, conceptualize and clarify ideas and develop stories that meet the broader creative/editorial objectives of the organization, if appropriate to the role - Display strong command of the language including correct grammar, spelling, sentence construction, diction and pronunciation skills | - Knowledge of use of wide range of vocabulary and writing techniques - Knowledge of difference between facts and opinion/ point of view - Knowledge of applicable legal and regulatory framework Practical Activity Perform script-writing for various media and types of stories | Theory-0.25 Hrs Practical-0.25 Hrs Total- 0.5 Hrs | Computer | Class Room and workshop |

- Build Java applets

Objective: This unit describes the skills and knowledge required to compile and run an applet that executes in Java-enabled browsers, and interacts with users. It applies to individuals involved in software, web or games development and who are required to build applets, using Java that interacts with users via a browser.

Duration:  1 Hours							Theory: 0.5 Hours				Practice: 0.5 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Create Java source file | Trainee will be able to: - Create a file with a text editor - Create a sub-class of the class Java. Applet. - Ensure that applet sub-class implements at least one of the following methods: init and paint - Define classes that contain instance variables, methods and local variables - Ensure that the init method initializes instance variables, and constructs any graphical interface used in the applet sub-class - Implement the paint and update methods to manage output that is drawn in the applet window - Incorporate event handling methods - Implement the code that allows a user to enter values, and assigns these values to variables - Use available graphical user interface (GUI) components to allow user interaction with the applet - Specify and load images and sounds - Include comments to describe the behavior of the applet | - Knowledge of object-oriented programming concepts Practical Activity Creation java resource file | Theory-0.166 Hrs Practical-0.166 Hrs Total- 0.332 Hrs | Computer | Class Room and workshop |
| - Compile a source file | Trainee will be able to: - Use a Java compiler to compile the file - Correct errors detected by the compiler - Confirm basic correctness of the file, to ensure that all variables have been initialized - Ensure the compiler creates a class file, and that the class is interpreted correctly | - Knowledge of theoretical concepts of Java programming Practical Activity Compilation of a source file | Theory-0.166 Hrs Practical-0.166 Hrs Total- 0.332 Hrs | Computer | Class Room and workshop |
| - Run applet | Trainee will be able to: - Create and name a hypertext markup language (HTML) file, and add APPLET tag - Write the applet class file to match the HTML document that contains an APPLET tag - Confirm that the applet loads and executes correctly - Identify and correct run-time errors - Identify and correct logic errors - Ensure that the appearance of the applet window renders it accessible and intuitive for the user, and that its design complies with organizational standards Ensure that user interaction is implemented efficiently and effectively | - Knowledge of Australian Computer Society Code of Ethics - Knowledge of browser security restrictions. Practical Activity Run applet | Theory-0.166 Hrs Practical-0.166 Hrs Total- 0.332 Hrs | Computer | Class Room and workshop |

- Manipulate Images (Illustrator)

Objective: This unit describes the skills and knowledge required to compile and manipulate images. It applies to individuals involved in software, image editing and who are required to manipulate images, using Illustrator.

Duration:  1 Hours							Theory: 0.5 Hours				Practice: 0.5 Hours

| Familiarize with illustrator interface | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Familiarize with illustrator interface | Trainee will be able to: - Able to describe differences between raster and vector imaging and uses for both - Understand the capabilities of Illustrator and alternative programs - Able to comfortably navigate Illustrator | - Knowledge of usage of pen tool Practical Activity Perform illustrator interface | Theory-0.083 Hrs Practical-0.083 Hrs Total- 0.166 Hrs | Computer | Class Room and workshop |
| - Create and edit basic vector paths | Trainee will be able to: - Able to use path creation and editing tools - Use the terminology associated with vector paths - Able to save an illustrator file - Export a drawing to various other file types | - Knowledge of benefits of clipping masks Practical Activity Create and edit basic vector paths | Theory-0.083 Hrs Practical-0.083 Hrs Total- 0.166 Hrs | Computer | Class Room and workshop |
| - Apply and edit colors in their drawings | Trainee will be able to: - Use variety of basic color tools and settings - Create a cohesive color theme - Create and edit gradients | - Knowledge of ways to create the perfect shape using path finder panel Practical Activity Apply and edit colors in their drawings | Theory-0.083 Hrs Practical-0.083 Hrs Total- 0.166 Hrs | Computer | Class Room and workshop |
| - Use specific tools to modify the appearance of paths | Trainee will be able to: - Apply advanced stroke and width settings to a path - Understand the basic function and process of using brushes | - Knowledge of ways to make brushes Practical Activity Use specific tools to modify the appearance of paths |  |  |  |
| - Create, edit, and combine shapes in a variety of ways | Trainee will be able to: - Use the shape tools effectively - Rotate, reflect, scale, skew, and free transform a shape - Utilize various ways to copy and paste a shape - Use the knife and scissors tools to split paths and shapes - Create a complex shape by combining simple ones Create and edit clipping paths | - Knowledge of usage of layering Practical Activity Create, edit, and combine shapes in a variety of ways | Theory-0.083 Hrs Practical-0.083 Hrs Total- 0.166 Hrs | Computer | Class Room and workshop |
| - Apply and edit the appearance of images | Trainee will be able to: - Able to victories and paint a raster image - Use text effectively - Able to effectively live trace an image - Able to add, edit, and warp text - Apply and edit object effects | - Knowledge of add, edit and wrap text - Knowledge of apply and edit objects Practical Activity Apply and edit the appearance of images | Theory-0.083 Hrs Practical-0.083 Hrs Total- 0.166 Hrs | Computer | Class Room and workshop |

- Manipulate Images (Photoshop)

Objective: This unit describes the skills and knowledge required to compile and manipulate images. It applies to individuals involved in software, image editing and who are required to manipulate images, using Illustrator.

Duration:  1 Hours							Theory: 0.5 Hours				Practice: 0.5 Hours

| Familiarize with illustrator interface | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Performs basic image edits using Photoshop | Trainee will be able to: - Able to describe differences between raster and vector imaging and uses for both - Edit Photos non-destructively - Able to comfortably navigate Photoshop - Crop picture effectively - Straighten the picture - Use the layering process in Photoshop | - Knowledge of basic tools Practical Activity Performs basic image edits using Photoshop | Theory-0.125 Hrs Practical-0.125 Hrs Total- 0.25 Hrs | Computer | Class Room and workshop |
| - Use masks and combine images | Trainee will be able to: - Make and edit masks - Make selection - Make adjustment layers with masks - Use masks and layers to combine photos | - Knowledge of organization of layer method Practical Activity Practice of masks and combine images | Theory-0.125 Hrs Practical-0.125 Hrs Total- 0.25 Hrs | Computer | Class Room and workshop |
| - Merge and heal photos | Trainee will be able to: - Use Photo merge - Fill selections with content aware fill - Use healing tools correctly - Apply Filters to photos | - Knowledge of effects of masking, brush, cropping, and adjusting layers - Knowledge of short keys for tools Practical Activity Perform merging and healing of photos | Theory-0.125 Hrs Practical-0.125 Hrs Total- 0.25 Hrs | Computer | Class Room and workshop |
| - Use non-photo elements in Photoshop designs | Trainee will be able to: - Use text tool - Use brushes - Create shapes - Apply colors and gradients - Apply Layer Effects - Combine elements in pleasing ways | - Knowledge of processing of opening and saving image - Knowledge of ways to add effects on an image Practical Activity Perform non-photo elements in Photoshop designs | Theory-0.125 Hrs Practical-0.125 Hrs Total- 0.25 Hrs | Computer | Class Room and workshop |

- Design simple web page layouts

Objective: This unit is about writing and editing copy (i.e. final version of text) for a range of formats in different media.

Duration:  2 Hours							Theory: 01 Hours				Practice: 1 Hours

| Familiarize with illustrator interface | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify main layout sections from design specification | Trainee will be able to: - Identify the required sections of the web page - Create the web page structure | - Knowledge of standard web and web scripting design principles Practical Activity Perform identification of main layout sections from design specification | Theory-0.33 Hrs Practical-0.33 Hrs Total- 0.55 Hrs | Computer | Class Room and workshop |
| - Layout the web page to match design specification | Trainee will be able to: - Position document elements - Style web page elements to match the design specifications | - Knowledge of positioning of document - Knowledge of cascading style sheets (CSS) - Knowledge of Hypertext transfer protocol (HTTP) Practical Activity Perform layout the web page to match design specification | Theory-0.33 Hrs Practical-0.33 Hrs Total- 0.55 Hrs | Computer | Class Room and workshop |
| - Test and validate the layout | Trainee will be able to: - Test the website in various browsers - Validate the web pages against industry standards | - Knowledge of web pages against industry standards - Knowledge of hypertext markup language (HTML), or extensible hypertext markup language (XHTML) - Knowledge of World Wide Web consortium (WWWC) standards. Practical Activity Perform test and validate the layout | Theory-0.33 Hrs Practical-0.33 Hrs Total- 0.55 Hrs | Computer | Class Room and workshop |

- Apply introductory programming techniques

Objective: This unit describes the performance outcomes, skills and knowledge required to undertake introductory programming tasks in the development of a game or application. This unit addresses the knowledge, processes and techniques necessary to develop skills to create simple applications or games.

Duration:  2 Hours							Theory: 1 Hours				Practice: 1 Hours

| Familiarize with illustrator interface | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Apply language syntax and layout | Trainee will be able to: - Apply basic language syntax rules - Use language data types, operators and expressions to create clear and concise code - Apply variables and variable scope - Use library functions in a program - Use commenting to create clear meaning to code | - Knowledge of language syntax rules - Knowledge of data types - Knowledge of library functions Practical Activity Perform language syntax and layout | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | Computer | Class Room and workshop |
| - Apply control structures | Trainee will be able to: - Apply language syntax for sequence, selection and iteration constructs - Use logical operators to create expressions for use in selection and iteration constructs | - Knowledge of iteration constructs - Knowledge of logical operators Practical Activity Perform control structures | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | Computer | Class Room and workshop |
| - Code using standard algorithms | Trainee will be able to: - Develop algorithms that use the sequence, selection and iteration constructs - Create and use arrays - Code standard sequential access algorithms for reading and writing text files, including end-of-file detection loops - Apply string manipulation | - Knowledge of algorithms - Knowledge of arrays - Knowledge of standard sequential access algorithms for reading and writing text files - Knowledge of application of string manipulation Practical Activity Perform code using standard algorithms |  |  |  |
| - Test code | Trainee will be able to: - Use debugging techniques to trace code execution and examine variable contents to detect and correct errors - Create and conduct simple tests to confirm code meets design specification - Document the tests performed and results achieved | - Knowledge of debugging techniques to trace code execution - Knowledge of create and conduct simple tests Practical Activity Perform test code | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | Computer | Class Room and workshop |
| - Create an application or game | Trainee will be able to: - Design an algorithm in response to basic program specifications - Develop the application or game to meet the program specification - Test and confirm that application or game meets the initial specifications | - Knowledge of design an algorithm - Knowledge of developing the application or game - Knowledge of application or game meets the initial specifications Practical Activity Perform operation on an application or game | Theory-0.2 Hrs Practical-0.2 Hrs Total- 0.4 Hrs | Computer | Class Room and workshop |

- List of Tools, Equipment and Consumable

06: Advanced Surveying

| S# | Tools and Equipment | S# | Consumable Material |
| --- | --- | --- | --- |
| 1 | Digital Theodolite | 1. | Lime |
| 2 | Total Station complete set along with accessories and software. | 2. | Drawing Sheets |
| 3 | Dual Frequency GNNS GPS complete set with all accessories and software i/c Battery with Charger. | 3. | Lead Pencils |
| 4 | Hand Held GPS | 4. | Eraser |
| 5 | Tribrach |  |  |
| 6 | Optical plummets |  |  |
| 7 | Target Staff |  |  |
| 8 | Single Prism Targets with stands |  |  |
| 9 | Tripple Prism Targets with stands |  |  |
| 10 | Ball Pen Hammer |  |  |
| 11 | Spring balances |  |  |
| 12 | Ranging rods |  |  |
| 13 | Steel Tape |  |  |
| 14 | Steel Band |  |  |
| 15 | Invar tape |  |  |
| 16 | Metallic Tape |  |  |
| 17 | Arrows |  |  |
| 18 | Pegs |  |  |
| 19 | Mallet |  |  |
| 20 | Umbrella with stand |  |  |
| 21 | Engineer’s chain |  |  |
| 22 | Thermometers |  |  |
| 23 | Drawing Lab Instruments |  |  |
| 24 | Computer Lab equipment |  |  |

07: AutoCAD

| S# | Tools and Equipment | S# | Consumable Material |
| --- | --- | --- | --- |
| 1 | Computer latest model with Server |  | Printer papers |
| 2 | Computer latest model work stations |  | Printer Toner |
| 3 | Laptop computer |  | USB |
| 4 | Net working |  | Printing Paper Roles |
| 5 | UPS 2000 Watt with batteries |  | Plotter Toner |
| 6 | Multimedia Projector |  |  |
| 7 | Scanner for A-3 paper |  |  |
| 8 | Printer LaserJet |  |  |
| 9 | USB Sticks |  |  |
| 10 | Computer Chair (will be added in furniture portion) without arm rest – 5 legs base |  |  |
| 11 | Computer Table (will be added in furniture portion) 2½ x 2 x 2½ |  |  |
| 12 | License software for education Latest Versions: Auto CAD M.S Office Antivirus Microsoft windows |  |  |
| 13 | Plotter |  |  |

|  | 08: Construction I |  |  |
| --- | --- | --- | --- |
| S# | Tools and Equipment | S# | Consumable Material |
| 1. | Brick Making Machine | 1. | Sand |
| 2. | Brick Molds | 2. | Sand Dust |
| 3. | Hand roller | 3. | cement |
| 4. | Thermometer | 4. | self-tapping screws |
| 5. | Umbrella with stand | 5. | soft sheet nails |
| 6. | PPEs | 6. | Wall board adhesive |
| 7. | Calculator | 7. | Distemper |
| 8. | Stone masonry instruments | 8. | Knotting & Lashing Rope |
| 9. | Scissor | 9. | Lime |
| 10. | Mason Trowel | 10. | Paint |
| 11. | Pointing Trowel | 11. | Bitumen |
| 12. | Troweling Blades | 12. | Polythene Sheet |
| 13. | Hammer | 13. | Polythene Role for DPC Bricks |
| 14. | Hammer Forged steel rectangle head | 14. |  |
| 15. | Lump hammers | 15. | Chalk |
| 16. | Hammers (bricks, club and scutch) | 16. | Luminous Slip |
| 17. | Sledge hammers | 17. | Stone Crush |
| 18. | Air compressors and hoses | 18. | Lime beds |
| 19. | Bevels | 19. | Pencil, |
| 20. | hand/power saws | 20. | Eraser, |
| 21. | Core cutting saw | 21. | Sharpener, |
| 22. | Jig saws | 22. | Drawing sheet, |
| 23. | Masonry saws | 23. | Acoustic underlay material, adhesives |
| 24. | Wet and dry diamond saws | 24. | Caulking compound, cement mortar (with and without additives) |
| 25. | Marking equipment | 25. | Clouts, cornice adhesive, crack suppression membrane |
| 26. | Measuring tapes and rules, | 26. | Fixings and fasteners |
| 27. | Metallic Tape 100 feet (30m) | 27. | Patching materials, plasterboard nails, pre-mixed and mixed fillers |
| 28. | Steel Tape- Slide Lock, Automatic Returns, Metallic / plastic case, 5-meter, Graduation on both side in feet & meter | 28. | Lime Color (Different) |
| 29. | Measuring scale, | 29. | Tile Bond |
| 30. | Nail guns | 30. | Tile Spacer (Different Sizes) |
| 31. | Protractors | 31. | Brick Ballast |
| 32. | squares (combination/tri) | 32. | Road Metals |
| 33. | steel squares and fences | 33. | Thermophore for Joints 2” & 1” thick |
| 34. | string lines | 34. | Water Stopper for Concrete (Plastic) |
| 35. | Line blocks and line pins | 35. | Barrel bolts, Cabin hooks closers, Dead bolts, Flash bolts, flashings, handles hinges (butt and parliament), Latches, locks, Metal, Night latches, Passage sets, Timber. |
| 36. | builders’ lines | 36. | Windows may include: All size windows, Casement, Double hung, Glazed sliding doors, curved and bay, Hopper, Straight and sliding |
| 37. | static lines |  |  |
| 38. | Compass |  |  |
| 39. | Corner Float and float Blades |  |  |
| 40. | Steel float Set (Small, medium, large) Wooden Handle |  |  |
| 41. | Wooden Float (Small, medium, large) Wooden Handle |  |  |
| 42. | Groove Cutter |  |  |
| 43. | Cement sheet cutters |  |  |
| 44. | Tile cutters |  |  |
| 45. | Edger |  |  |
| 46. | Brooms |  |  |
| 47. | Polishers |  |  |
| 48. | Hand Grinders |  |  |
| 49. | Power grinders |  |  |
| 50. | Vibrator |  |  |
| 51. | Vibrating Rod |  |  |
| 52. | Form work |  |  |
| 53. | Molds |  |  |
| 54. | Workability test apparatus |  |  |
| 55. | Core cutting machine |  |  |
| 56. | Hand Drill Machines with drills |  |  |
| 57. | Power drill machines with drills |  |  |
| 58. | Sheet plate |  |  |
| 59. | Plate bar |  |  |
| 60. | Plate rod |  |  |
| 61. | Adjustable base plates |  |  |
| 62. | Channels |  |  |
| 63. | Forgings area |  |  |
| 64. | Casting platform |  |  |
| 65. | Furnace |  |  |
| 66. | Wheel Barrow: |  |  |
| 67. | Bucket |  |  |
| 68. | Paint Scraper Set |  |  |
| 69. | Potien Filler Knife Set |  |  |
| 70. | Chisel Set (9"-12"-18") |  |  |
| 71. | Plumb Bob |  |  |
| 72. | plumb rules |  |  |
| 73. | Straight Edge |  |  |
| 74. | Shovels with handle |  |  |
| 75. | Kassi (Plies) with handle |  |  |
| 76. | Spirit Level (Aluminum 2 ft. length) |  |  |
| 77. | torpedo levels |  |  |
| 78. | Dumpy Level |  |  |
| 79. | Pick Axe |  |  |
| 80. | Tray (Galvanized Iron, 14 SWG) SET |  |  |
| 81. | Pointing Tray |  |  |
| 82. | Striker for vertical joint (pointing) |  |  |
| 83. | Striker for horizontal joint (pointing) |  |  |
| 84. | Bamboo (10 feet) |  |  |
| 85. | Bamboo (15 feet) |  |  |
| 86. | Battens (5'x3½"x2½") |  |  |
| 87. | Shesham Wood Planks (7'x7"x1½") |  |  |
| 88. | Mortar Pan -GI Sheet (20 SWG) Size, 45 cm dia |  |  |
| 89. | mortar boards |  |  |
| 90. | Adjustable Wrench Set- * Hardened Steel, tempered and chrome plated, * Set include 8" (20cm), 12" (30cm), 18" (45cm) |  |  |
| 91. | Mixing Spoon Set- heavy gauge stainless steel, 150mm, 200mm, and 250mm long, round bottom |  |  |
| 92. | Concrete mixers |  |  |
| 93. | Elevators |  |  |
| 94. | Forklifts |  |  |
| 95. | jointing tools |  |  |
| 96. | Margin or raking tools |  |  |
| 97. | materials hoists |  |  |
| 98. | Cantilevered hoists (materials only with maximum capacity of 500kg) |  |  |
| 99. | Wall Scaffolds |  |  |
| 100. | scaffold belts |  |  |
| 101. | scaffolding planks |  |  |
| 102. | bracket scaffolds (tank and formwork) |  |  |
| 103. | prefabricated scaffolds |  |  |
| 104. | braces |  |  |
| 105. | small petrol or diesel engines and compressors, |  |  |
| 106. | Bolsters |  |  |
| 107. | brick buggies |  |  |
| 108. | brick grabs |  |  |
| 109. | Bends and hitches |  |  |
| 110. | box spanners |  |  |
| 111. | Couplers and accessories |  |  |
| 112. | Fiber ropes |  |  |
| 113. | Gin wheels |  |  |
| 114. | guard rails |  |  |
| 115. | Ledgers |  |  |
| 116. | Mesh guards |  |  |
| 117. | mid rails |  |  |
| 118. | modular and prefabricated components |  |  |
| 119. | Safety nets |  |  |
| 120. | Stairs or ladders |  |  |
| 121. | Standards |  |  |
| 122. | steel and aluminum tubes (pipes) |  |  |
| 123. | transoms |  |  |
| 124. | Wire nips |  |  |
| 125. | Pallet trolleys |  |  |
| 126. | profiles |  |  |
| 127. | Brushes |  |  |
| 128. | Caulking guns |  |  |
| 129. | Electrical leads |  |  |
| 130. | Hose and water sprays |  |  |
| 131. | Nippers |  |  |
| 132. | Pointed grouters |  |  |
| 133. | power screwdrivers |  |  |
| 134. | sanders |  |  |
| 135. | Rags |  |  |
| 136. | rubber mallets |  |  |
| 137. | Sanding blocks |  |  |
| 138. | spacers and wedges |  |  |
| 139. | Spatulas |  |  |
| 140. | sponges, squeegees |  |  |
| 141. | wire brushes, |  |  |
| 142. | Work platforms |  |  |
| 143. | scribes |  |  |

|  | 09: Plumbing |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S# | Tools and Equipment |  | S# |  | Consumable Material |
| 1. | Adjustable Wrench Set |  | 1. |  | Complete Set Of Bath Room Fitting |
| 2. | Pipe Wrench Chain Type |  | 2. |  | Complete Set of Bath Sanitary Fixture |
| 3. | Adjustable Pipe Wrench |  | 3. |  | Complete set of GI pipe fittings |
| 4. | Pipe Reamer |  | 4. |  | Complete set of CI pipe fittings |
| 5. | Pipe Vice (Stand Type) |  | 5. |  | Complete set of PPRC pipe fittings |
| 6. | Pipe Vice (Bench Type) |  | 6. |  | Cement |
| 7. | Pipe Thread Cutting Kit |  | 7. |  | Sand |
| 8. | Tap Set |  | 8. |  | Plastic rings |
| 9. | Pipe Cutter |  | 9. |  | Spun yarn |
| 10. | Pliers Adjustable |  | 10. |  | Gasket |
| 11. | Cold Chisel Set |  | 11. |  | Sealant |
| 12. | Steel Tape |  | 12. |  | Nut bolts, Clamps, Jute, Rubber Rings |
| 13. | Mason’s Spirit Level |  | 13. |  | Lime |
| 14. | Steel Rule |  | 14. |  | Plumbing Fixtures each type |
| 15. | Electric Drill Machine |  | 15. |  | GI, CI, PPRC, Specials each type and size |
| 16. | Screw Driver Set |  | 16. |  | G.I. Pipe ½ dia |
| 17. | Trowel (200-280 Mm Long Steel) |  | 17. |  | ½-3/4 reducing socket, elbow, bush |
| 18. | Mortar Pan |  | 18. |  | Gate valve ½ |
| 19. | Shovel With Handle |  | 19. |  | Gate valve ¾ |
| 20. | Hack Saw |  | 20. |  | Check valve ¾ |
| 21. | Bench Vice |  | 21. |  | Ball valve ¾ |
| 22. | Ball Pein Hammer Set |  | 22. |  | Water meter |
| 23. | Claw Hammer |  | 23. |  | Ferule valve |
| 24. | Files Set |  | 24. |  | Hand wash basin |
| 25. | Scissors |  | 25. |  | Basin brackets |
| 26. | Centre Punch |  | 26. |  | Pillar cock (swan type) |
| 27. | Melting Pot |  | 27. |  | Rubber connection 18 |
| 28. | Ladle |  | 28. |  | Copper connection pipe 18 |
| 29. | Pipe Yarning and Caulking Tools |  | 29. |  | Stop cock |
| 30. | Oil Cane |  | 30. |  | Bib cock |
| 31. | Drill/Bit Set |  | 31. |  | Flushing cistern (low level) |
| 32. | Complete Set Of Bath Room Fitting |  | 32. |  | Flushing cistern (high level) (cast iron) 02 no. |
| 33. | Complete Set Of Bath Sanitary Fixture |  | 33. |  | PVC hockey pipe |
| 34. | Personal Protective equipment, dresses |  | 34. |  | PVC waste pipe |
| 35. | Plungers, |  | 35. |  | Waster coupling |
| 36. | Turbine with impellers & Motor Electricity |  | 36. |  | Water closet (Indian type) |
| 37. | Turbine pipe with fittings |  | 37. |  | Water closet (English type) |
| 38. | Pipe wooden / steel clamps |  | 38. |  | Urinal |
| 39. | Wheel barrow |  | 39. |  | P trap |
| 40. | Umbrella |  | 40. |  | Cast iron pipe 6 ft size |
|  |  |  | 41. |  | C.I cowl |
|  |  |  | 42. |  | C.I Tee 4 |
|  |  |  | 43. |  | Lead |
|  |  |  | 44. |  | Lead wool |
|  |  |  | 45. |  | Threading taps |
|  |  |  | 46. |  | PVC pipe and polyethylene pipe ½ size |
|  |  |  | 47. |  | Connection clamp with ¾ socket 4 size 06 no |
|  |  |  | 48. |  | Steel foot rule |
|  |  |  | 49. |  | Hand measuring tape (3 meter/ 15 ft) 25 no |
|  |  |  | 50. |  | Ball Peen hammer ½ kg |
|  |  |  | 51. |  | Center punch |
|  |  |  | 52. |  | Hacksaw Blade |
|  |  |  | 53. |  | Try Square |
|  |  |  | 54. |  | Oil cane |
|  |  |  | 55. |  | Mobil Oil |
|  | 10: Digital Skills |  |  |  |  |
| S# | Tools and Equipment | S# |  | Consumable Material |  |
| 1 | Computer latest model with Server |  |  | Printer papers |  |
| 2 | Computer latest model work stations |  |  | Printer Toner |  |
| 3 | Laptop computer |  |  | USB |  |
| 4 | Net working |  |  |  |  |
| 5 | UPS 2000 Watt with batteries |  |  |  |  |
| 6 | Multimedia Projector |  |  |  |  |
| 7 | Scanner for A-3 paper |  |  |  |  |
| 8 | Printer LaserJet |  |  |  |  |
| 9 | USB Sticks |  |  |  |  |
| 10 | Computer Chair (will be added in furniture portion) without arm rest – 5 legs base |  |  |  |  |
| 11 | Computer Table (will be added in furniture portion) 2½ x 2 x 2½ |  |  |  |  |
| 12 | License software for education Latest Versions: M.S Office Antivirus Microsoft windows |  |  |  |  |

- Members of Curriculum Development Committee

The following members participated in the Curriculum development Committee:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Azhar Iqbal Shad | Chief Instructor (Civil) | GCT Raiwnd Road, LHR |
| 2 | Mr. Imtiaz Ahmed Awan | Sr. Instructor (Civil) | GCT Rasul (Mandi Bahauddin) |
| 3 | Mr. Muhammad Amjad Rafique | Sr. Instructor (Civil) | GCT Rasul (Mandi Bahauddin) |
| 4 | Engr. Zuneera Ashfaq | Assistant Director | PITAC Lahore |
| 5 | Engr. Mohsin Jahanzeb | Instructor (Civil) | GSTC, Faisalabad |
| 6 | Engr. Norheen Amina | Consultant | Allied Engineering Services Lahore |
| 7 | Engr. Hira Ishtiaq | Consultant. | Allied Engineering Services Lahore |
| 8 | Engr. Habiba Mohsin | Design Engineer | JERS Engineering Consultants Lahore |
| 9 | Engr. Rana Haroon Mujahid | Deputy Director | C&W Department Govt. of Punjab |
| 10 | Engr. Arsalan Hameed Khan | Assistant Manager (Project) | Planning & Project department Lahore Waste Management Company. |
| 11 | Mr. Syed Fayyaz Mustafa Naqvi | Junior Instructor | GSTC Faisalabad |
| 12 | Engr. Inayat-ur-Rehman | DACUM, Facilitator | Peshawar |
| 13 | Mr. Muhammad Ishaq | Director HR | NAVTTC, Islamabad |