National Diploma Level - 5 in Civil Technology

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Contents

1.	Introduction	6

2.	Purpose of the Qualification:	6

3.	Overall objectives of training program	7

4.	Entry level of trainees	7

5.	Minimum qualification for teachers	8

6.	Duration of the course:	8

7.	Description and structure of the course	10

8.	Detail of Module Level-5	16

National Diploma in Civil Technology Level-5 (Part-I)	16

9.22. Transportation Engineering- I	16

0732B&CE-239. Develop construction plan of Tunnels	16

0732B&CE-240. Produce sketches of an airport	18

0732B&CE-241. Produce sketches of permanent Way	21

0732B&CE-242. Produce sketches of points and crossings	24

0732B&CE-243. Produce sketches of station yards and signals	27

0732B&CE-244. Produce sketches of harbor	30

9.23. Hydraulics Engineering	35

0732B&CE-245. Determine Hydrostatic pressure	35

0732B&CE-246. Apply Hydro kinematic Principles	38

9.24. Soil Mechanics	41

0732B&CE-247. Determine moisture content and specific gravity of soil	41

0732B&CE-248. Determine Atterberg’s Limits	43

0732B&CE-249. Identify Type of Soil	45

0732B&CE-250. Conduct soil compaction tests	48

0732B&CE-251. Determine permeability of soil	51

0732B&CE-252. Investigate shear strength of soil	53

0732B&CE-253. Find out bearing capacity of soil	55

9.25. Bridge Engineering	57

0732B&CE-254. Produce sketches of bridges w. r. t. functions	57

0732B&CE-255. Produce sketches of bridges w. r. t. material of construction	59

0732B&CE-256. Produce sketches of bridges w. r. t. type of super structure	61

0732B&CE-257. Produce sketches of bridges w. r. t. floor type	64

0732B&CE-258. Produce sketches of culverts, low level and high-level causeway	65

9.26. Environmental Technology	68

0732B&CE-259. Determine Water Pollution	68

0732B&CE-260. Evaluate Air Pollution	72

0732B&CE-261. Determine Noise Pollution	74

0732B&CE-262. Evaluate Solid Waste Pollution	75

0732B&CE-263. Adopt occupational health and safety	77

0732B&CE-264. Respond to Natural Disaster	82

9.27. Construction Project planning and Management	84

0732B&CE265-	Develop Project Management Life Cycle	84

0732B&CE266-	Develop Preliminary Project Plan	88

0732B&CE267-	Develop Construction Project Plan	90

0732B&CE268-	Conduct on-site supervision of construction projects	94

0732B&CE269-	Perform Basic operation in Primavera P6	97

0732B&CE270-	Perform road Project Activities Scheduling in Primavera P6	99

0732B&CE271-	Perform road Project Resources Costing &Planning in Primavera P6	103

0732B&CE272-	Manage construction Project in Primavera P6	108

0732B&CE273-	Identify Project Risk and Consequences on Project life cycle	110

0732B&CE274-	Perform Evaluation to assess the severity of Risk and formulate the strategy to manage the risk	112

National Diploma in Civil Technology Level-5 Part-II	114

9.28. Transportation Engineering II	114

0732B&CE275-	Plan Route Alignment of Highway	114

0732B&CE276-	Design Highway Geometrics	116

0732B&CE277-	Perform tests on stone metal for pavement construction	119

0732B&CE278-	Perform tests on bitumen	121

0732B&CE279-	Prepare drawings of flexible/rigid pavement	126

0732B&CE280-	Plan highway drainage	129

0732B&CE281-	Develop maintenance plan for roads	131

9.29. Irrigation Engineering	133

0732B&CE282-	Produce sketches of canals and head works	133

0732B&CE283-	Design Canals	138

0732B&CE284-	Produce sketches of Hydraulic structures	141

9.30. R.C.C. Design	144

0732B&CE285-	Design and Draw Reinforcement for Beams	144

0732B&CE286-	Design and Draw Reinforcement for Slabs	146

0732B&CE287-	Design and Draw Reinforcement for Columns	147

0732B&CE288-	Design and Draw Reinforcement for Footings	150

0732B&CE289-	Design and Draw Reinforcement for Stairs	152

9.31. Civil Engineering Project	153

0732B&CE290-	Develop Project for construction of RCC framed Structure Class Room.	153

0732B&CE291-	Develop Project plan for construction of RCC OHR of 10000-gal (UK) capacity	156

0732B&CE292-	Develop Project plan for construction of Water Supply Scheme	159

0732B&CE293-	Develop Project plan for construction of sewerage scheme	162

0732B&CE294-	Develop Project plan for construction of Highway	165

0732B&CE295-	Develop Project plan for construction of Irrigation Canal	168

9.32. Entrepreneurial Skills.	171

0732B&CE296-	Investigate micro business opportunities	171

0732B&CE297-	Develop a micro business proposal	177

0732B&CE298-	Develop a Business Plan & Marketing Plan.	182

0732B&CE299-	Organize finances for the micro business.	188

0732B&CE300-	Manage human resources.	192

0732B&CE301-	Market products and services.	197

0732B&CE302-	Monitor and review business performance.	201

0732B&CE303-	Negotiate for resolving business issues	204

10.	List of Tools, Equipment and Consumable	211

11.	Members of Curriculum Development Committee	219

# Introduction

Civil engineering is considered as the first discipline of the various branches of engineering after military engineering, and includes the designing, planning, construction, and maintenance of the infrastructure. The works include roads, bridges, buildings, dams, canals, water supply and numerous other facilities that affect the life of human beings. Civil engineering is intimately associated with the private and public sectors, including the individual homeowners and international enterprises. It is one of the oldest engineering professions, and ancient engineering achievements due to civil engineering include the pyramids of Egypt and road systems developed by the Romans.

Civil engineering has a significant role in the life of every human being, though one may not truly sense its importance in our daily routine. The function of civil engineering commences with the start of the day when we take a shower, since the water is delivered through a water supply system including a well-designed network of pipes, water treatment plant and other numerous associated services. The network of roads on which we drive while proceeding to school or work, the huge structural bridges we come across and the tall buildings where we work, all have been designed and constructed by civil engineers. Even the benefits of electricity we use are available to us through the contribution of civil engineers who constructed the towers for the transmission lines. In fact, no sphere of life may be identified that does not include the contribution of civil engineering. Thus, the importance of civil engineering may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Civil Technology under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# Purpose of the Qualification:

The purpose of this qualifications is to set high professional standards for civil technology sector. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for civil technology industry

- Enable the existing workforce to capacitate themselves in new technologies and methods

# Overall objectives of training program

The Civil qualification of level 5 consists of both the theoretical and practical details and having the following Occupations

- Civil Surveyor

- Draftsman

- Quantity Surveyor

- Quantity Surveyor after Draftsman

- Materials Technician

- Construction Quality control Supervisor

- Mason

- Plumber

- Domestic Electrician.

- Site supervisor

# Entry level of trainees

| National Vocational Certificate Level 2 (Site Assistant in Civil Technology) | Middle |
| --- | --- |
| National Vocational Certificate Level 3 (Assistant Surveyor in Civil Technology) | Level 2 |
| National Vocational Certificate Level 4 (Site Supervisor in Civil Technology) | Level 3 |
| National Diploma in Civil Technology Level 5 | Level 4 |

# Minimum qualification for teachers

- B. Tech in Civil Technology 4 Years

- B.Sc. / B.E. in Civil Engineering 4 Years

- D. A. E. in Civil Technology with 3 Years teaching experience

- Must be able to communicate effectively

Medium of instruction

English, and Urdu.

# Duration of the course:

The proposed curriculum is composed of 303 modules that will be covered in 3600 Learning hours. Duration of the course is proposed to be Three years. The total weightage for technical modules is 3600 hours. A total of 1200 hours have been reserved for allied subjects i.e. Islamic studies, English, Mathematics, Physics and Chemistry.

The details of technical modules are given as under:

Level - 2 = 6 months.

Level - 3 = 6 months.

Level - 4 = 1 Year. (Part-1 & Part-2)

Level - 5 = 1 Year. (Part-1 & Part-2)

The overall distribution of contact hours and Credit Hours is given below:

Total.		3600 hours. & 360 Credits

Theory.	976 hours (27.11 %)

Practical.	2624 hours (72.89 %)

The distribution of contact hours and credit hours in each level is given below:

Level 1- 2 = 6 months (16 weeks)

Total.		592 hours. & 59.2 Credits

Theory.	128 hours (22 %)

Practical.	464 hours (78 %)

Level - 3 = 6 months (16 weeks)

Total.		608 hours. & 60.8 Credits

Theory.	128 hours (21 %)

Practical.	480 hours (79 %)

Level - 4 = 1 Year (16 weeks + 16 Weeks)

Total.		1200 hours. & 120 Credits

Theory.	240 hours (20 %)

Practical.	960 hours (80 %)

Level - 5 = 1 Year (16 weeks + 16 Weeks)

Total.		1200 hours. & 120 Credits

Theory.	480 hours (40 %)

Practical.	720 hours (60 %)

# Description and structure of the course

Following is the structure of the course:

(National Diploma in Civil Technology Level-5) Part-I

| Code | Competency Standard | Category | Level | Th | Practical | Total | Credit Hour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9.22 Transportation Engineering I |  |  | 64 | 48 | 112 | 11.2 |
| 0732B&CE241 | Develop construction plan of Tunnels | Core | 5 | 8 | 3 | 11 | 1.1 |
| 0732B&CE242 | Produce sketches of an airport | Core | 5 | 8 | 3 | 11 | 1.1 |
| 0732B&CE243 | Produce sketches of permanent Way | Core | 5 | 12 | 12 | 24 | 2.4 |
| 0732B&CE244 | Produce sketches of points and crossings | Core | 5 | 12 | 9 | 21 | 2.1 |
| 0732B&CE245 | Produce sketches of station yards and signals | Core | 5 | 12 | 9 | 21 | 2.1 |
| 0732B&CE246 | Produce sketches of harbour | Core | 5 | 12 | 12 | 24 | 2.4 |
|  | 9.23 Hydraulics Engineering |  |  | 32 | 48 | 80 | 8 |
| 0732B&CE247 | Determine Hydrostatic pressure | Core | 5 | 16 | 21 | 37 | 3.7 |
| 0732B&CE248 | Apply Hydro kinematic Principles | Core | 5 | 16 | 27 | 43 | 4.3 |
|  | 9.24 Soil Mechanics |  |  | 32 | 48 | 80 | 8 |
| 0732B&CE249 | Determine moisture content and specific gravity of soil | Core | 5 | 4 | 6 | 10 | 1 |
| 0732B&CE250 | Determine Atterberg’s Limits | Core | 5 | 4 | 6 | 10 | 1 |
| 0732B&CE251 | Identify type of soil | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE252 | Conduct soil compaction tests | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE253 | Determine permeability of soil | Core | 5 | 4 | 6 | 10 | 1 |
| 0732B&CE254 | Investigate shear strength of soil | Core | 5 | 4 | 6 | 10 | 1 |
| 0732B&CE255 | Find out bearing capacity of soil | Core | 5 | 4 | 6 | 10 | 1 |
|  | 9.25 Bridge Engineering |  |  | 32 | 48 | 80 | 8 |
| 0732B&CE256 | Produce sketches of bridges w.r.t. functions | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE257 | Produce sketches of bridges w.r.t. material of construction | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE258 | Produce sketches of bridges w.r.t. type of super structure | Core | 5 | 8 | 12 | 20 | 2 |
| 0732B&CE259 | Produce sketches of bridges w.r.t. floor type | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE260 | Produce sketches of culverts, low level and high-level causeway | Core | 5 | 6 | 9 | 15 | 1.5 |
|  | 9.26 Environmental Technology |  |  | 32 | 48 | 80 | 8 |
| 0732B&CE261 | Determine Water Pollution | Core | 5 | 6 | 12 | 18 | 1.8 |
| 0732B&CE262 | Evaluate Air Pollution | Core | 5 | 6 | 12 | 18 | 1.8 |
| 0732B&CE263 | Determine Noise Pollution | Core | 5 | 3 | 5 | 8 | 0.8 |
| 0732B&CE264 | Evaluate Solid Waste Pollution | Core | 5 | 4 | 3 | 7 | 0.7 |
| 0732B&CE265 | Adopt occupational health and safety | Generic | 5 | 9 | 9 | 18 | 1.8 |
| 0732B&CE266 | Respond to Natural Disaster | Core | 5 | 4 | 7 | 11 | 1.1 |
|  | 9.27. Construction Project Planning & Management |  |  | 64 | 96 | 160 | 16 |
| 0732B&CE267 | Develop Project Management Life Cycle | Core | 5 | 4 | 6 | 10 | 1 |
| 0732B&CE268 | Develop Preliminary Project Plan | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE269 | Develop Construction Project Plan | Core | 5 | 6 | 9 | 15 | 1.5 |
| 0732B&CE270 | Conduct on-site supervision of construction projects | Core | 5 | 5 | 12 | 17 | 1.7 |
| 0732B&CE271 | Perform Basic operation in Primavera P6 | Core | 5 | 8 | 12 | 20 | 2 |
| 0732B&CE272 | Perform road Project Activities Scheduling in Primavera P6 | Core | 5 | 8 | 12 | 20 | 2 |
| 0732B&CE273 | Perform road Project Resources Costing &Planning in Primavera P6 | Core | 5 | 8 | 12 | 20 | 2 |
| 0732B&CE274 | Manage construction Project in Primavera P6 | Core | 5 | 8 | 12 | 20 | 2 |
| 0732B&CE275 | Identify Project Risk and Consequences on Project life cycle | Core | 5 | 5 | 6 | 11 | 1.1 |
| 0732B&CE276 | Perform Evaluation to assess the severity of Risk and formulate the strategy to manage the risk | Core | 5 | 6 | 6 | 12 | 1.2 |

(National Diploma in Civil Technology Level-5) Part-II

| Code | Competency Standard | Category | Level | Th | Practical | Total | Credit Hour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9.22 Transportation Engineering II |  |  | 48 | 48 | 96 | 9.6 |
| 0732B&CE277 | Plan Route Alignment of Highway | Core | 5 | 4 | 6 | 10 | 1.0 |
| 0732B&CE278 | Design Highway Geometrics | Core | 5 | 4 | 6 | 10 | 1.0 |
| 0732B&CE279 | Perform tests on stone metal for pavement construction | Core | 5 | 4 | 6 | 10 | 1.0 |
| 0732B&CE280 | Perform tests on bitumen | Core | 5 | 12 | 21 | 33 | 3.3 |
| 0732B&CE281 | Prepare drawings of flexible/rigid pavement | Core | 5 | 6 | 3 | 9 | 0.9 |
| 0732B&CE282 | Plan highway drainage | Core | 5 | 6 | 3 | 9 | 0.9 |
| 0732B&CE283 | Develop maintenance plan for roads | Core | 5 | 6 | 3 | 9 | 0.9 |
|  | 9. Irrigation Engineering |  |  | 64 | 48 | 112 | 11 |
| 0732B&CE284 | Produce sketches of canals and head works | Core | 5 | 21 | 18 | 39 | 3.9 |
| 0732B&CE285 | Design Canals | Core | 5 | 22 | 15 | 37 | 3.7 |
| 0732B&CE286 | Produce sketches of Hydraulic structures | Core | 5 | 21 | 15 | 36 | 3.6 |
|  | 9.21 RCC Design |  |  | 80 | 63 | 143 | 14 |
| 0732B&CE287 | Design and Draw Reinforcement for Beams | Core | 5 | 14 | 12 | 26 | 2.6 |
| 0732B&CE288 | Design and Draw Reinforcement for Slabs | Core | 5 | 24 | 15 | 39 | 3.9 |
| 0732B&CE289 | Design and Draw Reinforcement for Columns | Core | 5 | 19 | 11 | 30 | 3 |
| 0732B&CE290 | Design and Draw Reinforcement for Footings | Core | 5 | 8 | 10 | 18 | 1.8 |
| 0732B&CE291 | Design and Draw Reinforcement for Stairs | Core | 5 | 15 | 15 | 30 | 3 |
|  | 9.24. Civil Engineering Project (Any one project to a group of 8 students) |  |  | 0 | 138 | 138 | 13.8 |
| 0732B&CE292 | Develop Project for construction of RCC framed Structure Class Room. | Core | 5 | 0 | 138 | 138 | 13.8 |
| 0732B&CE293 | Develop Project plan for construction of RCC OHR of 10000-gal (UK) capacity. | Core | 5 | 0 | 138 | 138 | 13.8 |
| 0732B&CE294 | Develop Project plan for construction of Water Supply Scheme. | Core | 5 | 0 | 138 | 138 | 13.8 |
| 0732B&CE295 | Develop Project plan for construction of sewerage scheme | Core | 5 | 0 | 138 | 138 | 13.8 |
| 0732B&CE296 | Develop Project plan for construction of Highway | Core | 5 | 0 | 138 | 138 | 13.8 |
| 0732B&CE297 | Develop Project plan for construction of Irrigation Canal | Core | 5 | 0 | 138 | 138 | 13.8 |
|  | 9.32. Entrepreneurial Skills |  |  | 32 | 63 | 95 | 9.5 |
| 0732B&CE298 | Investigate micro business opportunities | Generic | 5 | 3 | 6 | 9 | 0.9 |
| 0732B&CE299 | Develop a micro business proposal | Generic | 5 |  | 8 | 12 | 1.2 |
| 0732B&CE300 | Develop a marketing plan and a business plan | Generic | 5 | 7 | 15 | 22 | 2.2 |
| 0732B&CE301 | Organize finances for the micro business | Generic | 5 | 4 | 8 | 12 | 1.2 |
| 0732B&CE302 | Manage human resources | Generic | 5 | 3 | 6 | 9 | 0.9 |
| 0732B&CE303 | Market products and services | Generic | 5 | 4 | 8 | 12 | 1.2 |
| 0732B&CE304 | Monitor and review business performance | Generic | 5 | 4 | 8 | 12 | 1.2 |
| 0732B&CE305 | Negotiate for resolving business issues | Generic | 5 | 3 | 4 | 7 | 0.7 |

Scheme of Studies.

National Diploma in Civil Technology Level-5 (Part-I)

| Code | Name of Subjects | Category | Numbers of Module | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 000000000 | Transportation Engineering-I | Technical | 06 | 64 | 48 | 112 | 11.2 | 4 | 3 | 7 |
| 000000000 | Hydraulics Engineering | Technical | 02 | 32 | 48 | 80 | 8 | 2 | 3 | 5 |
| 000000000 | Soil Mechanics | Technical | 07 | 32 | 48 | 80 | 8 | 2 | 3 | 5 |
| 000000000 | Bridge Engineering | Technical | 05 | 32 | 48 | 80 | 8 | 2 | 3 | 5 |
| 000000000 | Environmental Technology | Technical | 06 | 32 | 48 | 80 | 8 | 2 | 3 | 5 |
| 00000000 | Construction Project Planning and Management | Technical | 10 | 64 | 96 | 160 | 16 | 4 | 6 | 10 |
| 000000000 |  | Allied |  |  |  |  |  |  |  |  |
| Total |  |  | 36 | 256 | 336 | 592 | 59.2 | 16 | 21 | 37 |

National Diploma in Civil Technology Level-5 (Part-II)

| Code | Name of Subjects | Category | Numbers of Module | Contact Hour |  |  | Credit | Periods per week |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th | Pr | Total |  | Th | Pr | Total |
| 000000000 | Transportation Engineering-II | Technical | 07 | 42 | 48 | 90 | 9.0 | 3 | 3 | 6 |
| 000000000 | Irrigation Engineering | Technical | 03 | 64 | 48 | 112 | 11.2 | 4 | 3 | 7 |
| 000000000 | R.C.C. Design | Technical | 05 | 80 | 63 | 143 | 14.3 | 5 | 4 | 9 |
| 000000000 | Civil Engineering Project | Technical | 06 | 0 | 138 | 138 | 13.8 | 0 | 10 | 10 |
| 000000000 | Entrepreneurial Skill | Generic | 08 | 32 | 63 | 95 | 9.5 | 2 | 4 | 6 |
| 00000000 | Green Skills in Construction | Generic | 01 | 06 | 24 | 30 | 3.0 | 1 | 3 | 4 |
| 00000000 |  | Allied |  |  |  |  |  |  |  |  |
| Total |  |  | 30 | 224 | 384 | 608 | 60.8 | 15 | 27 | 42 |
| Grand Total of Level-5 |  |  | 66 | 480 | 720 | 1200 | 120 | 30 | 45 | 75 |

- Detail of Module Level-5

# National Diploma in Civil Technology Level-5 (Part-I)

# 9.22. Transportation Engineering- I

### 0732B&CE-239. Develop construction plan of Tunnels

Objective: This competency standard covers the skills and knowledge required to demark the alignment of tunnel and draft a construction plan of tunnel

Duration: 11 Hours									Theory: 8 Hours				Practice:3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Demark the alignment of tunnel | Trainee will be able to: - Collect contour map of a hilly area - Identify considerations for demarcation of alignment - Place contour map on drafting table - Mark possible routes considering influencing factors | - Necessity of tunnels - Importance of tunnel engineering - Factors influencing alignment of tunnels Practical Activity: Demark the alignment of tunnel on contour map | Theory-3 Hrs Practice-1.5 Hrs Total- 4.5 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU2. Draft a construction plan of tunnel | Trainee will be able to: - Analyze activities to be performed to construct tunnel - Identify machinery required to execute activities planned before - Draft hierarchy of construction team - Define roles according to positions for construction - Identify hazards and precautionary measures | - Methods of construction of tunnels in soft soils - Methods of construction of tunnels in hard rocks - Machinery required for tunnel construction Practical Activity: - Prepare a construction plan of tunnel | Theory-5 Hrs Practice-1.5 Hrs Total- 6.5 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |

### 0732B&CE-240. Produce sketches of an airport

Objective: This competency standard covers the skills and knowledge required to prepare a general layout plan of an airport, draw cross section of a runway, draw sketches of airport patterns and draw cross section of drainage for sub surface.

Duration: 11 Hours								Theory: 8 Hours				Practice:3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Prepare a general layout plan of an airport | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw layout plan of an airport | - Technical terminologies of airports - Factors influencing site selection of airports - Types of airports Practical Activity: - Sketch the general layout plan of various types of airport | Theory-2 Hrs Practice-0.75 Hrs Total- 2.75 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU2.Draw cross section of a runway | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw cross section of a runway | - Purpose of runway - Runway lengths as ICAO Practical Activity: - Produce cross section of a runway | Theory-2 Hrs Practice-0.75 Hrs Total- 2.75 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU3.Draw sketches of airport patterns | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw sketches of airport patterns | - Purpose of runway patterns - Types of runway patterns - Airport lighting Practical activity: - Produce sketches of airport patterns | Theory-02 Hrs Practice-0.75 Hrs Total- 2.75 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU4.Draw cross section of drainage for sub surface | Trainee will be able to - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw cross section of drainage for sub surface | - Purpose of airport drainage - Types of airport drainage structures Practical activity: - Produce cross section of drainage for sub surface | Theory-0.2 Hrs Practice-0.75 Hrs Total- 2.75 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |

### 0732B&CE-241. Produce sketches of permanent Way

Objective: This competency standard covers the skills and knowledge required to draw rails sections (double headed, bull headed and flat footed), sketches of rail fastenings, sleeper fastenings, cross section of broad gauge rail track.

Duration: 24 Hours							Theory:  12 Hours				Practice:12Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Draw rails sections (double headed, bull headed and flat footed) | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw rails sections (double headed, bull headed and flat footed) - Perform labeling and dimensioning | - Requirements of permanent way - Explanation of components of permanent way, their functions and types - Creep in rails Practical activity: - Produce sketches of rail sections (double headed, bull headed and flat footed) | Theory-3 Hrs Practice-3 Hrs Total- 6 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU2. Draw sketches of rail fastenings | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw sketches of rail fastenings - Label parts of rail fastenings | - Purpose of rail fastenings - Explanation of rail fastenings with their types Practical activity: - Produce sketches of rail fastenings | Theory-3 Hrs Practice-3 Hrs Total- 6 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU3. Draw sketches of sleeper fastenings | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw sketches of sleeper fastenings - Label parts of sleeper fastenings | - Purpose of sleeper fastenings - Explanation of sleeper fastenings with their types Practical activity: - Produce sketches of sleeper fastenings | Theory-3 Hrs Practice-3 Hrs Total- 6 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU4. Draw cross section of broad gauge rail track | Trainee will be able to - Identify drawing tools - Fix drawing sheet on drafting table - Draw cross section of broad gauge rail track of in filling - Draw cross section of broad gauge rail track in cutting - Label components of rail tracks | - Purpose of gauges in rail track - Types of gauges - Selection of gauges - Difficulties in gauges Practical activity: - Produce cross section of broad gauge rail track in cutting & filling | Theory-3 Hrs Practice-3 Hrs Total- 6 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |

### 0732B&CE-242. Produce sketches of points and crossings

Objective: This competency standard covers the skills and knowledge required to draw sketches of switches, crossings, right hand turnout and left hand turnout.

Duration: 21 Hours								Theory: 12 Hours				Practice: 9Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Draw sketches of switches | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw sketch of stub switch - Draw sketch of split switch - Label components of switches | - Explanation of switches, shapes, length of stock and tongue rails, heal clearance, switch angle, through switches - Function of switches - Types of switches Practical activity: - Produce sketches of types of switches | Theory-3 Hrs Practice-2.25 Hrs Total- 5.25 Hrs | - Drawing sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU2.Draw sketches of crossings | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw sketches of crossings (acute angle, obtuse angle, right angle, spring, cross over, diamond, scissors) - Label components of crossings | - Function of crossings - Terminologies of crossings - Types of crossings Practical activity: - Produce sketches of types crossings | Theory-3 Hrs Practice-2.25 Hrs Total- 5.25 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU3.Draw sketch of right hand turnout | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw sketch of right hand turnout - Label components of right hand turnout | - Function of turnout - Description of right hand turnout Practical activity: - Produce sketches of right hand turn out | Theory-3 Hrs Practice-2.25 Hrs Total- 5.25 Hrs | - Drawing sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU4. Draw sketch of left hand turnout | Trainee will be able to - Identify drawing tools - Fix drawing sheet on drafting table - Draw sketch of left hand turnout - Label components of left hand turnout | - Description of left hand turnout Practical activity: - Produce sketches of left hand turn out | Theory-3 Hrs Practice-2.25 Hrs Total- 5.25 Hrs | - Drawing sheets - Led Pencil - Erase T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |

### 0732B&CE-243. Produce sketches of station yards and signals

Objective: This competency standard covers the skills and knowledge required todraw sketch of coaching yard, goods yard, locomotive yard, marshaling yard and sketches of rail signals

Duration: 21 Hours								Theory: 12 Hours				Practice: 9Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Draw sketch of coaching yard | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw the sketch of coaching yard | - Station & its functions - Types of stations & their functions - Function of caching yard Practical activity: - Produce the sketch of coaching yard | Theory-3 Hrs Practice-1.8 Hrs Total- 4.8 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU2. Draw sketch of goods yard | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw the sketch of goods yard | - Function of goods yard Practical activity: - Produce the sketch of goods yard | Theory-2 Hrs Practice-1.8 Hrs Total- 3.8 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU3.Draw sketch of locomotive yard | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw the sketch of locomotive yard | - Function of locomotive yard Practical activity: - Produce the sketch of locomotive yard | Theory-2 Hrs Practice-1.8 Hrs Total- 3.8 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU4. Draw sketch of marshaling yard | Trainee will be able to - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw the sketch of marshaling yard | - Function of marshaling yard Practical activity: - Produce the sketch of marshaling yard | Theory-2 Hrs Practice-1.8 Hrs Total- 3.8 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU5. Draw the sketches of rail signals | Trainee will be able to - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw the sketches of rail signals (semaphore, warning, light disc and dwarf signals) | - Purpose of signals in railway - Types of rail signals & their uses - Necessity of interlocking system in railways - Methods of interlocking Practical activity: - Produce the sketches of rail signals | Theory-3 Hrs Practice-1.8 Hrs Total- 4.8 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |

### 0732B&CE-244. Produce sketches of harbor

Objective: This competency standard covers the skills and knowledge required to prepare layout plan of a typical harbor, draw cross section of breakwater, draw sketches of jetties, piers, lock gates, quay, typical light house and floating signals.

Duration: 24 Hours								Theory: 12 Hours			Practice: 12Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Prepare layout plan of a typical harbor | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw layout plan of a typical harbour | - Classification of harbors - Requirements of a commercial harbor, its location and size Practical activity: - Draw layout plan of a typical harbor | Theory-1.5 Hrs Practice-1.5 Hrs Total- 3Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU2.Draw cross section of breakwater | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw cross section of breakwater | - Explanation of tidal waves - Break water and their classification Practical activity: - Produce cross section of breakwater | Theory-1.5 Hrs Practice-1.5 Hrs Total- 3 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU3.Draw sketches of jetties | Trainee will be able to: - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw sketches of jetties | - Function of jetties - Practical activity: - Produce sketches of jetties | Theory-1.5 Hrs Practice1.5 Hrs Total- 3 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU4.Draw sketches of pier | Trainee will be able to - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw sketches of pier | - Function of piers Practical activity: - Produce sketches of piers | Theory-1.5 Hrs Practice-1.5 Hrs Total- 3 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU5.Draw sketches of lock gates | - Trainee will be able to - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw sketches of lock gates | - Function of lock gates Practical activity: - Produce sketches of lock gates | Theory-1.5 Hrs Practice-1.5 Hrs Total- 3 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU6.Draw sketches of quay | Trainee will be able to - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw sketches of quay | - Function of lock quay Practical activity: - Produce sketches of quay | Theory-1.5 Hrs Practice-1.5 Hrs Total- 3 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU7. Draw sketches of a typical light house | Trainee will be able to - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw sketches of typical light house | - Navigational aids & their purpose - Classification of signals & their uses Practical activity: - Produce sketch of typical light house | Theory-1.5 Hrs Practice-1.5 Hrs Total- 3 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU8. Draw sketches floating signals | Trainee will be able to - Identify drawing tools - Fix drawing sheet on drafting table - Draw border lines and title bar - Draw sketches of floating signals | - Purpose of floating signals - Types of floating signals & their uses Practical activity: - Produce sketches floating signals | Theory-1.5 Hrs Practice-1.5 Hrs Total- 3 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |

# 9.23. Hydraulics Engineering

### 0732B&CE-245. Determine Hydrostatic pressure

Objective: This module covers the knowledge and  skills required to find pressure by pressure gauges, Hydrostatic pressure and centre of pressure of vertically immersed surface.

Duration:  37 Hours							Theory: 16 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Find pressure by Piezometer | Trainee will be able to: - Identify pressure gauge - Measure pressure by Piezometer | - Define Pressure. - What is Piezometer? - What is Intensity of Pressure? - State Pascal’s Law and its simple Application Practical Activity Determine Pressure using Piezometer | Theory-3 Hrs Practical-3 Hrs Total- 6 Hrs | Piezometer Water Tank | Class Room and workshop |
| - Find pressure by manometer | Trainee will be able to: - Identify pressure gauge - Measure pressure by Manometer | - What is Manometer? - What is Pressure Head? - Differentiate between Gauge Pressure & Absolute Pressure Practical Activity Pressure measurement by Manometer. | Theory-3 Hrs Practical-6 Hrs Total- 9 Hrs | Manometer Water Tank | Class Room and workshop |
| - Find out Hydrostatic pressure and center of pressure on vertically Immersed surface. | Trainee must be able to: - Identify apparatus - Immerse apparatus in water tank - Identify the formulae for hydrostatic pressure and center of pressure. - Find the area of immersed surface - Find moment of inertia of the figure - Calculate hydrostatic pressure on surface - Find position of center of pressure | - Define Hydrostatics & Hydrostatic Pressure. - What is center of Pressure? - State Pressure on Immersed Surface - Define Moment of Inertia? Practical Activity Determination of Hydrostatic Pressure & Center of Pressure on vertically Immersed Surface. | Theory-3 Hrs Practical-6 Hrs Total- 9 Hrs | Hydrostatic Apparatus Water Tank | Class Room and workshop |
| - Find out discharge through a pipe with Pitot tube | Trainee must be able to - Identify the instrument - Fix the Pitot tube with pipe - Measure height of liquid in the tube - Find the velocity by using formula. - Find the discharge by using formula | - Define Discharge. - Define Velocity. - What is Specific Weight and Specific Volume? Practical Activity Determination of Discharge through Pipe using Pitot Tube. | Theory-4 Hrs Practical-3 Hrs Total- 7 Hrs | Pitot Tube | Class Room and workshop |
| - Find the Metacentric height of a floating body in Lab. | Trainee must be able to - Identify the apparatus. - Put apparatus in water tank. - Place jokey weight to make the ship symmetrical. - Place the ship in water and adjust the plumb to zero mark. - Move jokey weight across deck to a specified distance. - Measure the angle of inclination. - Calculate the metacentric height of ship by formula. | - What is Metacenter and Metacentric Height? - What is angle of inclination? - Define the terms, buoyancy, floatation, buoyant force and center of buoyancy Practical Activity Calculate the Metacentric height of a floating body in Lab. | Theory-3 Hrs Practical-3 Hrs Total- 6 Hrs | Metacentric height apparatus Water Tank | Class Room and Workshop |

### 0732B&CE-246. Apply Hydro kinematic Principles

Objective: This module covers the knowledge and  skills required to determine coefficient of discharge using venture meter, to calculate energy losses in pipe flow. Determine Hydraulic Co-efficient for circular orifices Calculate hydraulic and energy gradient for pipelines. Calculate flow in open channels. Calculate flows through notch. Calculate flows through weirs.

Duration: 43 Hours							Theory: 16 Hours				Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Determine coefficient of discharge using venture meter | Trainee will be able to: - Check dimensions of venture meter and water tank. - Calculate at inlet and throat - Open inlet valve and take piezometer readings - Measure the head. - Calculate actual discharge. - Calculate theoretical discharge. - Calculate the coefficient of discharge using formulae. | - State discharge and equation of continuity of a liquid flow. - Differentiate between Actual Discharge and Theoretical Discharge - Define Coefficient of discharge - Define Head - Explain Bernoulli's theorem with its formula, limitations and practical application i.e., venture meter Practical Activity Calculate Coefficient of discharge using Venture meter | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Venture meter Water Tank | Class Room and workshop |
| - Calculate energy losses in pipe flow. | Trainee will be able to: - Identify the apparatus. - Open valve and pass water through pipe. - Check piezo metric readings - Draw hydraulic grade line. | - Define Energy Losses - What is Hydraulic Grade Line - Distinguish types of flow in pipes i.e. steady and unsteady flow, uniform and non-uniform flow, turbulent flow. - explain difference between flow through pipes and open channel flow Practical Activity Calculation of Energy losses in Pipe Flow | Theory-3 Hrs Practical-3 Hrs Total- 6 Hrs | Water Tank | Class Room and workshop |
| - Determine hydraulic Co-efficient for circular orifices | Trainee must be able to: - Identify the orifice and adjust with the apparatus - Set constant head in the tank - Measure horizontal and vertical coordinates - Find actual discharge by using a container and stop watch - Use formulae to calculate coefficient of velocity and discharge | - Define Hydraulic Coefficient - Define orifice - State types of orifices - What is co-efficient of discharge and velocity of approach. Practical Activity Calculation of Hydraulic Coefficient for Circular Orifices | Theory-3 Hrs Practical-3 Hrs Total- 6 Hrs | Orifices Stop Watch | Class Room and workshop |
| - Calculate hydraulic and energy gradient for pipe lines. | Trainee must be able to - Prepare pipeline design charts using standard formulae. - Identify the limitations of formulae. - Identify variations in roughness coefficients. - Calculate the pressure in pipeline systems using the hydraulic gradient line. - Calculate the pipe discharge from reservoirs. | - Define Hydraulic Gradient - What is Energy gradient - Chezy’s & Darcy’s formulae for calculation of losses in pipes. - What are major and minor losses of head of water flowing through pipes Practical Activity Calculation of Hydraulic and Energy gradient | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Hydraulic Bench Stop Watch | Class Room and workshop |
| - Calculate flow in open channels. | Trainee must be able to - Identify methods used for measuring flows in open channels. - Identify instruments to be used - Use formulae for calculating flows in open channels. - Find the velocity by current meter and velocity rod - Find discharge. | - State Chezy's, Manning’s & Bazin's formulae for discharge through open channel. Practical Activity Calculate flow in open channels. | Theory-2 Hrs Practical-6 Hrs Total- 8 Hrs | Stop Watch Hydraulic Bench | Class Room and workshop |
| - Calculate flows through Vee and Trapezoidal Notch. | Trainee must be able to - Identify the methods used for measuring flows in notches. - Calculate area of notch. - Measure head of water over the notch. - Use the formulae for calculating flow in notches. | - State discharge through rectangular & trapezoidal channel section and their formulae. Practical Activity Calculation of flows through Vee & Trapezoidal Notch | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Vee and Trapezoidal notch Stop Watch Hydraulic Bench | Class Room and workshop |
| - Calculate flows through weirs | Trainee must be able to - Identify the methods used for measuring flows in notches. - Calculate area of weirs - Measure head of water over the weirs - Use the formulae for calculating flow in weirs | - Define Notches and its types - Define weirs and its types - State the discharge formulae for weirs. Practical Activity Flow Calculation through Weirs. | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Different Types of Weirs Stopwatch Hydraulic Bench | Class Room and workshop |

# 9.24. Soil Mechanics

### 0732B&CE-247. Determine moisture content and specific gravity of soil

Objective: This module covers the knowledge and  skills required to determine soil moisture content by Speedy moisture, establish soil moisture content by oven drying and Identify specific gravity of soil sample by pycnometer

Duration: 10 Hours									Theory: 4 Hours				Practice: 6Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Determine soil moisture content by Speedy moisture | Trainee will be able to: - Identify apparatus - Collect soil sample to be tested - Identify chemical to be used in speedy moisture - Assemble pressure vessel according to specification Record dial reading after shaking the vessel - Interpret moisture content corresponding to the reading of dial gauge | - Definition of soil mechanics - Objectives of soil mechanics - Types of soils - Soil formation - Soil structure - Soil group - moisture content of soil in field Practical Activity Find moisture content of soil of your college ground using speedy moisture apparatus | Theory-2 Hrs Practical-2Hrs Total- 4 Hrs | Speedy moisture test apparatus Empty clean container Moist Sample Balance accurate to 0.01gm Steel ball of 1.25-inch diameter Brush for cleaning the tester Scoop for measuring Calcium carbide | Class Room and Soil workshop |
| - Determine soil moisture content by oven drying | Trainee will be able to: - Determine weight of containers Determine Weight of soil sample with containers - Dry soil sample in oven - Evaluate moisture content | - Three phase diagrams of soil - Physical properties of soil - Laboratory method for moisture content Practical Activity Determine moisture content of college ground soil using oven dry method | Theory-1 Hrs Practical-2 Hrs Total- 3 Hrs | Empty clean container Moist Sample Oven Balance accurate to 0.01gm | Class Room and Soil workshop |
| - Evaluate specific gravity of soil sample by pycnometer | Trainee must be able to: - Identify pycnometer - Assemble apparatus with soil sample - Evaluate specific gravity using formula | - Specific gravity - Types of densities - Interrelationships between specific gravity, pore pressure, density etc. Practical Activity Determine specific gravity of soil sample from your college ground by use of pycnometer | Theory1 Hrs Practical-2 Hrs Total- 3Hrs | Empty clean container Moist Sample Oven Balance accurate to 0.01gm Pycnometer of about 1 liter capacity Glass rod Vacuum pump | Class Room and soil workshop |

### 0732B&CE-248. Determine Atterberg’s Limits

Objective: This module covers the knowledge and  skills required to determine liquid limit of soil, plastic limit of soil and shrinkage limit of soil

Duration: 10 Hours								Theory: 4 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Determine liquid limit of soil | Trainee will be able to: - Identify apparatus - Prepare soil sample - Assemble soil sample in apparatus - Cut groove in sample - Record number of blows - Repeat soil sample trials by varying moisture content - Interpret no. of blows and water content graphically | - Atterberg’s limits - liquid limit - liquid limit of various soils - Consistency of soil Practical Activity - Determine liquid limit of soil of your college ground | Theory-2 Hrs Practical-2 Hrs Total- 4 Hrs | Soil sample Balance Casagrande apparatus Grooving tool Mixing dishes Spatula Oven Squeeze bottle | Class Room and soil workshop |
| - Determine plastic limit of soil | Trainee must be able to: - Identify apparatus - Prepare soil sample - Roll treads of soil with fingers - Complete Rolling and kneading of soil samples - Dry crumbled soil threads in oven - Evaluate soil water content | - plastic limit - plastic limit of various soils - relationship of plastic limit, liquid limit, shrinkage limit etc. Practical Activity - Determine plastic limit of soil available in college ground area | Theory-1 Hrs Practical-2 Hrs Total- 3 Hrs | Soil sample Ground glass plate Plastic rolling device Spatula Dry oven Metallic rod Water content container Balance | Class Room and soil workshop |
| - Determine shrinkage limit of soil | Trainee must be able to: - Identify apparatus - Prepare soil sample - Evaluate volume of wet soil - Evaluate weight of wet soil - Evaluate volume of dry soil after soil shrinkage - Evaluate weight of dry soil after soil shrinkage - Identify shrinkage limit formula | - Shrinkage limit of soil - Shrinkage limit of various soils - Different Indices (Plasticity index, liquidity index, consistency index etc.) Practical Activity - Determine shrinkage limit of soil present in your college ground | Theory-1 Hrs Practical-2 Hrs Total- 3 Hrs | Shrinkage dish Electric oven Mercury Digital balance Sieve#40 Spatula Containers | Class Room and soil workshop |

### 0732B&CE-249. Identify Type of Soil

Objective: This module covers the knowledge and  skills required to perform Sieve analysis for soil sample, execute hydrometer test for fine soil, draft gradation curve Classify soil by AASHTO classification, Classify soil by USCS and Classify soil according to textural formation

Duration: 15 Hours								Theory: 6 Hours				Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform Sieve analysis for soil sample | Trainee will be able to: - Identify sieves sizes - Assemble nest of sieve - Prepare soil sample - Complete soil sample shaking - Compute percentage retained - Compute percentage passing | - classification of soil - types of soil classification system - utilization of soil classification system Practical Activity - Compute percentage passing and retained with respect to sieves for soil present in college ground | Theory-1 Hrs Practical-3 Hrs Total- 4 Hrs | Set of ASTM sieves containing sieve # 4, 10, 20, 40, 60, 100, 200 lid and pan Balance sensitive to 0.1gm Sieve shaker Soft brush for cleaning | Class Room and soil workshop |
| - Execute Hydrometer test for fine soil | Trainee will be able to: - Identify apparatus - Evaluate soil fraction passing sieve#200 - Prepare soil sample - Identify chemical - Complete test - Evaluate soil fraction size | - Fine aggregates - Classification of fine aggregates - Physical properties of fine aggregates Practical Activity - Compute particle size of soil passing sieve #200 gradation present in college ground | Theory-1 Hrs Practical-2 Hrs Total- 3 Hrs | Hydrometer Glass measuring cylinder Stirrer Weigh balance Oven Deflocculating agent Desicator Evaporate dish Beaker Stop watch Wash bottle Sieve | Class Room and soil workshop |
| - Draft gradation curve. | Trainee must be able to: - Identify soil grain size - Identify percentage passing of soil sample - Identify semi-logarithm graph - Plot grain size vs percentage passing of soil sample - Interpret soil type graphically | - Well graded soil - uniformly graded soil, - poorly graded soil, - Effective grain size, - coefficient of uniformity, - coefficient of curvature Practical Activity - Plot gradation curve of given soil sample from college ground | Theory-1 Hrs Practical-1 Hrs Total- 2 Hrs | Semi-log graph paper Pencil Rubber Sharpener | Class Room and Soil workshop |
| - Classify soil by AASHTO classification | Trainee must be able to - Obtain AASHTO classification chart - Identify the required parameters - Compile the data of required parameters of sample - Classify sample using the AASHTO classification chart | - AASHTO soil classification system - Soil groups - Nomenclature of soil Practical Activity - Classify soil available in college vicinity using AASHTO classification system | Theory-1 Hrs Practical-1 Hrs Total- 2 Hrs | Sieve set Atterberg limit test apparatus Weighing balance Drying oven | Class Room and workshop |
| - Classify soil by USCS | Trainee must be able to - Collect USCS tables - Identify the required parameters - Compile the data of required parameters of sample - Classify sample using USCS tables | - Unified soil classification system - Soil groups - Soil nomenclature Practical Activity - Classify soil available in college vicinity using USCS soil classification system | Theory-1 Hrs Practical-1 Hrs Total- 2 Hrs | Set of ASTM sieves containing sieve # 4, 10, 20, 40, 60, 100, 200 lid and pan Balance sensitive to 0.1gm Sieve Shaker Soft brush for cleaning Hydrometer apparatus | Class Room and Soil Workshop |
| - Classify soil according to textural formation | - Collect textural classification chart - Identify the required parameters - Compile the data of required parameters of sample - Classify sample using textural classification chart | - soil Textural classification system - particle size analysis for different soil types Practical Activity - Classify soil available in college vicinity using textural classification system of soil | Theory-1 Hrs Practical-1 Hrs Total- 2 Hrs | Sieve set lid pan Balance sensitive to 0.1gm Sieve shaker Soft brush for cleaning textural classification chart | Class Room and Soil Workshop |

### 0732B&CE-250. Conduct soil compaction tests

Objective: This module covers the knowledge and  skills required to Conduct Standard Proctor test, conduct modified Proctor test, inspect field density by core cutter and inspect soil field density by Sand replacement method

Duration: 15 Hours									Theory: 6 Hours				Practice:9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Conduct standard Proctor test | Trainee will be able to: - Set up apparatus - Prepare soil sample - Identify Weight of proctor mold - Distribute soil sample in three equal parts - Complete soil Compaction - Inspect compacted specimen weight - Dry compacted soil sample in oven - Evaluate bulk density of soil - Evaluate dry density of compacted soil - Interpret water content and dry density graphically | - Compaction - Significance of compaction. - Principles of compaction - Compaction vs. consolidation. - Relation between optimum moisture content and maximum dry density Practical Activity - Determine density of soil available in college vicinity using standard proctor test in lab. | Theory-1 Hrs Practical-2 Hrs Total- 3 Hrs | Apparatus Standard proctor mold Balance Tray rammer of 2.5Kg (5.5 lb) of fall 25.4Cm (12”) moisture container sieve 19 mm (3/4”) and 9.5mm (3/8”) | Class Room and Soil workshop |
| - Conduct modified Proctor test | Trainee will be able to: - Set up Apparatus - Prepare soil sample - Identify Weight of proctor mold - Distribute soil sample in five equal parts - Complete soil Compaction - Identify weight of compacted specimen - Dry compacted soil sample in oven - Evaluate bulk density of soil - Evaluate dry density of compacted soil - Interpret water content and dry density graphically | - Compaction Effort or Energy - Procedure to conduct modified proctor test - Difference between standard proctor test and modified proctor test Practical Activity - Determine density of soil using modified standard proctor test in lab. | Theory-2 Hrs Practical-2 Hrs Total- 4 Hrs | Apparatus Standard proctor mould Balance Tray rammer of 4.54Kg (10 lb), of fall 38Cm (18”) moisture container sieve 19 mm (3/4”) and 9.5mm (3/8”) | Class Room and Soil workshop |
| - Inspect Field density by core cutter | Trainee must be able to: - Identify apparatus - Evaluate weight and volume of core cutter - Perform field soil sampling - Evaluate weight of field sample - Evaluate water content of field sample - Evaluate field density | - field density of soil - Relation between field density and lab density - Methods to determine field density of soil - Procedure to conduct field density of soil using core cutter method Practical Activity - Inspect filed density of soil available in college vicinity using core cutter method | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Apparatus Core cutter Hammer excavating tools oven | Class Room and soil workshop |
| - Inspect soil field density by Sand replacement method | Trainee must be able to - Identify apparatus - Complete apparatus calibration - Evaluate volume of hole on ground - Identify water content of field soil sample - Evaluate field density of soil | - Destructive and non-destructive tests to determine field density of soil. - Types of destructive and non-destructive tests - Procedure to conduct field density by sand replacement method. Practical Activity - Inspect filed density of soil available in college vicinity using sand replacement method | Theory-1 Hrs Practical-2 Hrs Total- 3 Hrs | Sand bottle calibrating container glass plate base plate pan tools for excavating hole clean dry sand passing from No.10 (2.0mm) sand retained on No.20 (850m) sieve Speedy moisture tester apparatus | Class Room and Soil workshop |

### 0732B&CE-251. Determine permeability of soil

Objective: This module covers the knowledge and  skills required to determine permeability using constant head method and falling head method

Duration: 10 Hours								Theory: 4 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Permeability constant head method | Trainee will be able to: - Identify apparatus - Prepare sample - Assemble apparatus - Identify water head level difference - Evaluate permeability of cohesion-less soil | - Permeability - factors affecting permeability. - Permeability of various soils - Darcy's Law Practical Activity - Find permeability of given soil sample | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Constant head Permeameter device (including constant head filter tank and manometer tubes) 500ml beaker Stop watch Thermometer | Class Room and soil workshop |
| - Permeability falling head method | Trainee will be able to: - Identify apparatus - P2. Assemble apparatus - P3. Complete apparatus calibration - P4. Prepare soil sample - P5. Complete test - P6. Evaluate permeability of cohesive soils | - Laboratory permeability tests - Field permeability tests - Permeability test with respect to soil type - Practical Activity - Find the permeability of soil sample available in college vicinity | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Apparatus Falling head Permeameter device Ring stand with test tube clamp Burette (100ml) | Class Room and soil workshop |

### 0732B&CE-252. Investigate shear strength of soil

Objective: This module covers the knowledge and  skills required to perform direct shear test and execute triaxial shear test

Duration: 10 Hours									Theory: 4 Hours				Practice: 6Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform Direct Shear test | Trainee will be able to: - Set up apparatus - Prepare sample - Assemble apparatus - Complete test - Interpret readings - Evaluate shear strength - Produce results graphically - Interpret internal friction angle from graph | - Internal friction angle - cohesion of soil - cohesive, non-cohesive soils - Stresses - Strains - Shear strength of soils. - Coulomb's Law - Mohr’s circle - Procedure for Laboratory measurement of shear strength Practical Activity - Determine internal friction angle of soil sample available in college vicinity | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Shear box Shear box container Base plate with cross groves on its top Porous stones (2 Nos) Plain Grid plates (2Nos) Perforated grid plates (2Nos) Loading pad with steel ball Digital weighing machine Loading frame with loading yoke Proving ring Dial gauges (2 Nos) Weights | Class Room and Soil workshop |
| - Execute Triaxial Shear test | - Trainee will be able to: - Identify apparatus - Identify test - Identify soil sample preparation technique - Assemble apparatus - Complete test depending upon test condition requirement - Interpret readings - Evaluate readings - Produce results graphically - Interpret cohesion and internal friction angle graphically | - Consolidation soils - Unconsolidated soils - Drained soils - Undrained soils - Tri axial compression tests - Types of triaxial tests - Factors affecting shear strength of soil Practical Activity - Determine unconfined compressive strength of soil sample available in college vicinity | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Tampering Rod Spatula Rammer Sampler Distribution Panel Automatic Pressure/Volume Controllers Load Frame Load Cell Displacement transducer Tri axial Cell Pore Pressure Transducer De-aired water supplier Data logger Sample extruder Sample moulds Membranes ‘o’ rings Porous discs Ruler Balance Filter papers | Class Room and Soil workshop |

### 0732B&CE-253. Find out bearing capacity of soil

Objective: This module covers the knowledge and  skills required to Find bearing capacity of soil using Spt test and Identify bearing capacity using California bearing ratio

Duration: 10 Hours								Theory: 4 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Evaluate bearing capacity of soil using SPT test | Trainee will be able to: - Identify apparatus - Identify soil sampler - Assemble apparatus - Complete test - Record number of blows - Evaluate bearing capacity | - bearing capacity of soil. - Factors affecting bearing capacity. - Equations to evaluate bearing capacity of soils - Standard penetration test procedure Practical Activity - Find soil bearing capacity of college playground using SPT | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Drilling Rig. Split spoon (tube). Drop hammer Driving head (anvil). Guiding rod. Tripod. Extension rods Cylindrical mold detachable extension collar detachable perforated base plate | Class Room and Soil workshop |
| - Evaluate bearing capacity using California beraing ratio | - Trainee will be able to: - Identify CBR apparatus - Prepare sample - Complete test - Record readings - Interpret penetration and penetration load graphically - Evaluate CBR from graph - Interpret percentage CBR and dry density of sample graphically - Identify CBR graphically at required degree of compaction | - Procedure for California bearing ratio test - Application of CBR test Practical Activity - Find bearing capacity of sub grades using CBR test | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Spacer disc Metal rammer Weights. Loading Metal penetration piston Two dial gauges reading to 0.01 mm. Sieves. 4.75 mm and 20 mm I.S. Sieves mixing bowl straight edge scales soaking tank or pan drying oven filter paper Containers. | Class Room and Soil workshop |

# 9.25. Bridge Engineering

### 0732B&CE-254. Produce sketches of bridges w. r. t. functions

Objective: This module covers the knowledge and  skills required to draw sketches of foot, highway, railway, aqueduct and road cum railway bridge.

Duration: 15 Hours								Theory: 6 Hours				Practice: 9Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Draw sketch of foot bridge | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of foot bridge - Label the structural components of foot bridge | - foot bridge - Types of foot bridges - Construction materials for highway bridges - Utility of a foot bridges - Components of a foot bridge Practical Activity - Sketch and label components of foot bridge | Theory-1.5 Hrs Practical-2 Hrs Total-3 .5Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of highway bridge | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of highway bridge - Label the structural components of highway bridge | - Highway bridges - construction materials for highway bridges - types of highway bridges - Utility of a highway bridges - Components of a highway bridges Practical Activity - Sketch and label highway bridge | Theory-1.5 Hrs Practical-2 Hrs Total- 3.5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of railway bridge | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of railway bridge - Label the structural components of railway bridge | - Railway bridge - Utility of railway bridge - Components of a railway bridge Practical Activity - Sketch and label railway bridge | Theory-1 Hrs Practical-2 Hrs Total- 3 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of aqueduct bridge | Trainee must be able to - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of aqueduct bridge - Label the structural components of aqueduct bridge | - Aqueduct - Types of aqueduct - Components of an aqueduct Practical Activity - Sketch and label aqueduct bridge | Theory-1 Hrs Practical-2 Hrs Total- 3 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of road cum railway bridge | Trainee must be able to - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of road cum railway bridge - Label the structural components of road cum railway bridge | - Explain a road cum railway bridge - Component of a road cum railway bridge - Utility of a road cum railway bridge - types of bridges with respect to utility Practical Activity - Sketch and label railway bridge | Theory-1 Hrs Practical-1 Hrs Total- 2 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |

### 0732B&CE-255. Produce sketches of bridges w. r. t. material of construction

Objective: This module covers the knowledge and  skills required to draw sketches of timber, masonry, steel, R.C.C and pre stressed concrete bridges.

Duration: 15 Hours							Theory: 6 Hours				Practice:9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Draw sketch of timber bridge | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of timber bridge - Label the structural components of timber bridge | - Components of a timber bridge - Utility of timber bridge - Suitability of construction of timber bridge Practical Activity - Sketch and label timber bridge | Theory-1 Hrs Practical-1 Hrs Total- 2 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of masonry bridge | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of masonry bridge - Label the structural components of masonry bridge | - Major components of a masonry bridges - Necessity and suitability of masonry bridge - Components of a masonry bridges Practical Activity - Sketch and label masonry bridge | Theory-1 Hrs Practical-2 Hrs Total- 3 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of steel bridge | Trainee must be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of steel bridge - Label the structural components of steel bridge | - Utility of steel bridge - Components of steel bridges - Site suitability for steel bridges Practical Activity - Sketch and label steel bridge | Theory-1 Hrs Practical-2 Hrs Total-3 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of R.C.C bridge | Trainee must be able to - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of R.C.C bridge - Label the structural components of R.C.C bridge | - R.C.C bridges - Components of R.C.C bridges - Necessity and suitability - Characteristics of an ideal site for a bridge (River and Banks, foundation design and construction facilities, approaches) - Location of an alignment and site - State Factors governing the choice of a bridge Practical Activity - Sketch and label RCC bridges | Theory-2 Hrs Practical-2 Hrs Total- 4 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of pre stressed concrete bridge | Trainee must be able to - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of pre stressed concrete bridge - Label the structural components of pre stressed concrete bridge | - Components of pre-stressed bridge - Utility of pre-stressed bridges - Advantages of pre-stressed concrete bridges Practical Activity - Sketch and label pre-stressed concrete bridge | Theory-1 Hrs Practical-2 Hrs Total-3 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |

### 0732B&CE-256. Produce sketches of bridges w. r. t. type of super structure

Objective: This module covers the knowledge and  skills required to draw sketches of arch, girder, truss and suspension bridges.

Duration: 20 Hours								Theory: 8 Hours				Practice:12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Draw sketch of arch bridge | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of arch bridge - Label the structural components of arch bridge | - components of arch bridge - Types of arch bridges - Utility of arch bridge Practical Activity - Sketch and label arch bridge | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of girder bridge | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of girder bridge - Label the structural components of girder bridge | - Components of girder bridge - Types of girder bridges - Utility of girder bridge Practical Activity - Sketch and label girder bridge | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of truss bridge | Trainee must be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of truss bridge - Label the structural components of truss bridge | - Components of truss bridge - Types of truss bridges - Utility of truss bridge Practical Activity - Sketch and label truss bridge | Theory-2 Hrs Practical-3 Hrs Total-5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of suspension bridge | Trainee must be able to - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of suspension bridge - Label the structural components of suspension bridge | - Suspension bridge - Components of suspension bridge - Types of suspension bridges - Utility of suspension bridge - Necessity and suitability - Comparison of s various types of bridge with respect to super-structure - Characteristics of an ideal site for a bridge (River and Banks, foundation design and construction facilities, approaches) - Location of an alignment and site - State Factors governing the choice of a bridge Practical Activity - Sketch and label suspension bridge | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |

### 0732B&CE-257. Produce sketches of bridges w. r. t. floor type

Objective: This module covers the knowledge and  skills required to draw sketches of deck, through and semi through bridges

Duration:  15 Hours									Theory: 6 Hours				Practice: 9Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Draw sketch of deck bridge | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of deck bridge - Label the structural components of deck bridge | - Define deck of a bridge - Components of a deck - Material used for deck construction - Utility of deck - Types of decks Practical Activity - Sketch and label deck bridge | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of through bridge | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of through bridge - Label the structural components of through bridge | - Through bridge - Types of through bridge - Utility of through bridge - Site suitability for through bridge Practical Activity - Label and sketch through bridge | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing lab |
| - Draw sketch of semi through bridge | Trainee must be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of semi through bridge - Label the structural components of semi through bridge | - What is semi through bridge - What are different types of semi through bridge - What is the material utilized for construction of semi through bridge? - What is the difference between through and semi through bridge? Practical Activity - Label and sketch semi through bridge | Theory-2 Hrs Practical-3 Hrs Total- 5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener Masking tape | Drawing hall |

### 0732B&CE-258. Produce sketches of culverts, low level and high-level causeway

Objective: This module covers the knowledge and  skills required to draw sketches of culverts (pipe, box, arch and slab), low level and high level causeway.

Duration:  15 Hours								Theory: 6 Hours				Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Draw sketch of low level causeway | - Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of low level causeway - Label the structural components of low level causeway | - What do we mean by causeway? - How many types of causeways are there? - Define low level causeway - Utility of low-level causeway Practical Activity - Label and sketch low level causeway | Theory-1 Hrs Practical-1.5 Hrs Total- 2.5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener | Drawing lab |
| - Draw sketch of high level causeway | - Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of high level causeway - Label the structural components of high level causeway | - Define high level causeway - Difference between high level and low-level causeway Practical Activity - Label and sketch high level causeway | Theory-1 Hrs Practical-1.5 Hrs Total-2.5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener | Drawing lab |
| - Draw sketch of pipe culvert | - Trainee must be able to: - Identify drawing toolsfor drawing - Fix drawing sheet on drafting table - Draw sketch of pipe culvert - Label the structural components of pipe culvert | - define piper culvert - name different types of pipe culverts - why pipe culverts are used? Practical Activity - Label and sketch pipe culvert | Theory-1 Hrs Practical-1.5 Hrs Total- 2.5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener | Drawing lab |
| - Draw sketch of box culvert | - Trainee must be able to - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of box culvert - Label the structural components of box culvert | - Define box culvert - Types of box culverts - Utility of box culverts - Practical Activity - Label and sketch box culvert | Theory-1 Hrs Practical-1.5 Hrs Total- 2.5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener | Drawing lab |
| - Draw sketch of arch culvert | - Trainee must be able to - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of arch culvert - Label the structural components of arch culvert | - Define arch culvert - Different types of arch culvert - Utilization of arch culvert - Differentiate between box and arch culvert Practical Activity - Label and sketch box culvert | Theory-1 Hrs Practical-1.5 Hrs Total- 2.5Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener | Drawing lab |
| - Draw sketch of slab culvert | - Trainee must be able to - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw sketch of slab culvert - Label the structural components of slab culvert | - Define slab culvert - What is the utility of slab culvert? - Components of slab culvert - Difference between a bridge and a culvert. Practical Activity - Label and sketch slab culvert | Theory-1 Hrs Practical-1.5 Hrs Total- 2.5 Hrs | Paper Drafting table Pencil Ruler Rubber Sharpener |  |

# 9.26. Environmental Technology

### 0732B&CE-259. Determine Water Pollution

Objective: This module covers the knowledge and  skills required to determine Dissolved Oxygen in water, turbidity, total suspended solids, PH value, concentration of nitrates and Biochemical oxygen demand.

Duration: 18 Hours							Theory: 06 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Determine Dissolved Oxygen in water | Trainee will be able to: - Identify the equipment (dissolved oxygen meter and sensor) - Calibrate the equipment for measurement - Record the result of dissolved oxygen in water | - Definition of Dissolved Oxygen - Performance to calculate Dissolve Oxygen Practical Activity Determine Dissolved Oxygen in water | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Dissolved Oxygen meter - Measuring beakers - Eraser - Pencil - Sharpener - Scale - Notepad - Charts - Calculator | Class Room and Environmental lab |
| - Determine Turbidity of water | Trainee will be able to: - Identify turbidity meter - Calibrate the equipment for measurement - Evaluate the result of turbidity | - Definition of turbidity - Measurement of turbidity of water - Measurement of turbidity levels Practical Activity Determine Turbidity of water | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Turbidity meter - Measuring beakers - Eraser - Pencil - Sharpener - Scale - Notepad - Charts - Calculator | Class Room and Environmental lab |
| - Determine Total Suspended Solids in water | Trainee must be able to: - Identify the equipment - Record volume of sample - Filter the sample - Dry the filtered material - Weigh the material - Determine the quantitative TSS | - Measurement of Total Suspended Solids in water - Composition of Sewage Practical Activity Determine Total Suspended Solids in water | Theory-01 Hrs. Practical-02 Hrs. Total- 03 Hrs. | - Measuring beakers - China dish - Filter paper - Oven - Weighing Balance - Eraser - Pencil - Sharpener - Scale - Notepad - Charts - Calculator | Class Room and Environmental lab |
| - Determine the pH of water | Trainee must be able to - Identify the pH meter - Identify the litmus paper - Wet the litmus paper with water - Compare the color with the chart - Record the PH value | - Definition of pH - Determination of pH of water Practical Activity Determine the pH of water | Theory-01 Hrs Practical-01 Hr Total- 02 Hr | - pH meter - Litmus paper - Measuring beakers - Eraser - Pencil - Sharpener - Scale - Notepad - Charts - Calculator | Class Room and Environmental lab |
| - Find out the concentration of Nitrates in water | Trainee must be able to - Identify Spectrophotometer - Prepare standard NO3 nitrate solution - Record absorbance or transmittance against redistilled water set at zero absorbance - Obtain absorbance of standard nitrate solution - Draw the relationship between concentration and absorbance - Calculate the factor m ( slope of line) - Calculate the concentration of nitrate from the standard curve or from equation (conc. = m* abs) | - Determination of Nitrates in water - Quantity of Nitrates in water Practical Activity Find out the concentration of Nitrates in water | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Spectrophotometer - Measuring beakers - Eraser - Pencil - Sharpener - Scale - Notepad - Charts - Calculator | Class Room and Environmental lab |
| - Determine Biochemical Oxygen Demand for 5 days | Trainee must be able to - Determine the amount of sample to be analyzed - Place a clean, calibrated thermometer into the constant temperature chamber - Turn on the constant temperature chamber to allow the controlled temperature - Turn on the DO instrument - Aerate dilution water before adding nutrient solutions - Add to dilution water - Shake the container of dilution water for about 1 minute - Place the dilution water in the constant temperature chamber - The initial and final (after 5 days ± 4 hours) DO tests of the dilution water is determined and recorded | - Definition of Biochemical Oxygen Demand - Determination of Biochemical Oxygen Demand - Guidelines of Biochemical Oxygen Demand of water Practical Activity Determine Biochemical Oxygen Demand for 5 days | Theory-01Hrs Practical-03 Hrs Total- 04Hrs | - Calibrated thermometer - Oven - Dissolve Oxygen instrument - Measuring beakers - Eraser - Pencil - Sharpener - Scale - Notepad - Charts - Calculator | Class Room and Environmental lab |

### 0732B&CE-260. Evaluate Air Pollution

Objective: This module covers the knowledge and  skills required to analyze sources of air pollution, analyze methods of detection/measurement of air pollution, evaluate environment quality standards in ambient air, analyze methods to control air pollution and evaluate pathological effects of air pollution on human life.

Duration: 18 Hours								Theory: 06 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Analyze sources of air pollution | Trainee will be able to: - Identify the air pollutants - Identify sources of air pollutants - Gather information about mechanism of emission of pollutants - Compile data in a presentable form | - Sources of air pollution - Definition of air pollution - Definition of air pollutants - Mechanism of emission of pollutants Practical Activity Analyze sources of air pollution | Theory-1.5 Hr Practical-02 Hrs Total- 3.5 Hrs | - Eraser - Pencil - Sharpener - Scale - Notepad - Charts | Class Room and Environmental lab |
| - Analyze methods of detection/measurement of air pollution | Trainee will be able to: - Identify the air pollutants - Identify methods for detection and measurement of air pollutants - Enlist merits and demerits of methods for measurement of air pollution | - Measurement of air pollutants - Methods for detection and measurement of air pollutants - Identification of air quality meter Practical Activity Analyze methods of detection/measurement of air pollution | Theory-1.5 Hrs Practical-02 Hrs Total- 3.5 Hrs | - Air Quality Index - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room and Environmental lab |
| - Evaluate environment quality standards in ambient air | Trainee must be able to: - Collect EQS in ambient air of Pakistan - Collect EQS of developed countries - Compare EQS of Pakistan and developed countries | - Information of environment quality standards Practical Activity Evaluate environment quality standards in ambient air | Theory-01 Hrs Practical-03 Hr Total- 04 Hrs | - Air Quality Index - Eraser - Pencil - Sharpener - Scale - Notepad - Computer | Class Room and environmental Lab |
| - Analyze methods to control air pollution | Trainee must be able to - Identify methods to control air pollution - Enlist merits and demerits of methods to control air pollution | - Controlling measures of air pollution Practical Activity Analyze methods to control air pollution | Theory-01 Hrs Practical-02 Hrs Total- 03 Hr | - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room and environmental Lab |
| - Evaluate pathological effects of air pollution on human life | Trainee must be able to - Collect information about pathological effects of air pollutants - Compile data in a presentable form | - Pathological effects of air pollutants on human life Practical Activity Evaluate pathological effects of air pollution on human life | Theory-01 Hrs Practical-03 Hrs Total- 04 Hr | - Eraser - Pencil - Sharpener - Scale - Notepad - Charts | Class Room and environmental Lab |

### 0732B&CE-261. Determine Noise Pollution

Objective: This module covers the knowledge and  skills required to measure the noise pollution, compare the standards and control noise pollution.

Duration: 08 Hours								Theory: 03 Hours				Practice: 05 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure the noise pollution | Trainee will be able to: - Identify the sound level meter - Measure the sound level - Compare the sound level with standards - Identify the sound level as noise | - Measurement of sound level by sound level meter Practical Activity Measure the noise pollution | Theory-01 Hr Practical-02 Hrs Total- 03 Hr | - Sound level meter | Class Room and Environmental lab |
| - Compare the standards | Trainee will be able to: - Collect the standards of developed countries - Collect standards of Pakistan - Compare the sound level values for different zones | - Information of sound level values for different zones Practical Activity Compare the standards | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | - Eraser - Pencil - Sharpener - Scale - Notepad - Computer | Class Room and Environmental lab |
| - Enlist the methods to control noise pollution | Trainee must be able to: - Identify the sources of noise - Calculate the noise - Enlist the methods to control noise pollution | - Measurement of noise pollution Practical Activity Enlist the methods to control noise pollution | Theory-01 Hs Practical-01 Hrs Total- 02 Hrs | - Noise dosimeter. - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room Environmental lab |

### 0732B&CE-262. Evaluate Solid Waste Pollution

Objective: This competency standard covers the skills and knowledge required to evaluate the methods of disposal of solid waste and develop a management plan for disposal of solid waste.

Duration: 7 Hours								Theory: 04 Hours			Practice: 03 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Evaluate the methods of disposal of solid waste | Trainee will be able to: - Identify the sources - Differentiate the waste as organic/inorganic - Enlist the methods of disposal - Enlist the merits and demerits of each method | - Information of Disposal of solid wastes (sanitary, landfill, incineration, compositing, recycling) Practical Activity Methods of Disposal of solid waste | Theory-02 Hr Practical-1.5 Hrs Total- 3.5 Hrs | Pencil Charts Eraser | Class Room and Environmental lab |
| - Develop a management plan for disposal of solid waste | Trainee will be able to: - Identify the sources - Propose suitable method for different types of waste - Develop hierarchy of the organization - Present the plan | - Information of disposal site plan Practical Activity Preparing a plan for solid waste disposal | Theory-02 Hrs Practical-1.5 Hrs Total- 3.5 Hrs | Eraser Pencil Sharpener Scale Notepad Computer | Class Room and Environmental lab |

### 0732B&CE-263. Adopt occupational health and safety

Objective: This module covers the knowledge and  skills required to identify the fire control equipment, use fire extinguisher and develop an emergency exit plan.

Duration: 18 Hours							Theory: 09 Hours				Practice: 09 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Analyze safety and occupational health codes and standards | Trainee will be able to: - Collect occupational health safety codes and standards - Evaluate occupational health safety codes and standards | - Introduction to safety - Definition of occupational health safety - Information of occupational health safety codes and standards Practical activity Analyze safety and occupational health codes and standards | Theory-01 Hr Practical-02 Hrs Total- 03 Hrs | - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room and Environmental Lab |
| - Recognize hazards in construction industry | Trainee will be able to: - Identify hazards in construction industry. - Record hazards in construction industry - Interpret risks for enlisted hazards such as injury, burn, cut etc. | - Information of industrial noise and its control. - Industrial ventilation, exhaust systems, Fire hazards - Fire control systems - Fire control operation - Emergency exit plan - Safety precautionary measures for construction industry Practical activity Recognize hazards in construction industry | Theory-01 Hrs Practical-02Hrs Total- 03 Hrs | - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room and Environmental Lab |
| - Draw construction safety signs | Trainee must be able to: - Identify construction safety signs internationally recognized - Interpret safety signs for specific hazards - Draw construction safety signs | - Information of construction safety signs - Hazards and its safety signs Practical activity Draw construction safety signs | Theory-01 Hrs Practical-02 Hr Total- 03 Hrs | - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room and Environmental Lab |
| - Inspect personal protective equipment | Trainee must be able to - Identify personal protective equipment for different risks such as head injury, falls, burns etc. - Wear personal protective equipment | - Use of personal protective equipment for different risks Practical Activity Inspect personal protective equipment | Theory-01 Hrs Practical-01 Hrs Total- 02Hr | - Eraser - Pencil - Sharpener - Scale - Notepad - Ear Muff, Ear Plug - Safety shoes - Gloves - Safety Goggles - Safety Helmet - Respiratory protective equipment i.e. Masks - Life Jacket - High visibility jacket - Safety harness - Face shield - Rubber shoes/ long shoes - Water Buckets - Fire Blankets - Sand Buckets - First Aid Box - Thermometer and B.P apparatus - Fire Extinguisher - Fire Alarm - Sprinklers | Class Room and Environmental lab |
| - Handle material safely | Trainee must be able to - Identify lifting machinery - Identify hazards related to lifting operations - Interpret risks for enlisted hazards | - Safety Precautionary Measures for scaffolding, formwork, ladder, drilling & blasting, demolition, hot bituminous works, excavation and dewatering Practical Activity Handle material safely | Theory-01 Hrs Practical-0.5 Hrs Total- 1.5 Hr | - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room and Environmental Lab |
| - Identify fire control equipments | Trainee must be able to - Identify the fire control equipment - Diagnose operations of fire control equipment | - Fire hazards in building - Fire resistant construction materials - Fire control systems - Fire control operation Practical Activity Identify fire control equipments | Theory-01 Hrs Practical-0.25 Hrs Total- 1.25 Hr | - Fire control panel - Fire extinguishers - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room and Environmental lab |
| - Perform operation of fire extinguisher | Trainee must be able to - Identify fire extinguisher - Use fire extinguisher | - Name and uses of different fire extinguishers Practical Activity Perform operation of fire extuingisher | Theory-01 Hrs Practical-0.25 Hrs Total- 1.25 Hr | - Fire extinguishers - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room and Environmental Lab |
| - Develop an emergency exit plan | Trainee must be able to - Collect a plan of a building - Propose emergency exits - Draw the emergency plan | - Emergency exit plan Practical Activity Develop an emergency exit plan | Theory-01 Hrs Practical-0.5 Hrs Total- 1.5 Hr | - Eraser - Pencil - Sharpener - Scale - Notepad - Charts | Class Room and Environmental Lab |
| - Provide first aid | Trainee must be able to - Identify elements of first aid kit - Treat for suffocation, cut, injury, electric shock, snake bite, strains and gastrointestinal problems | - Elements of first aid kit Practical Activity Provide first aid | Theory-01 Hrs Practical-0.5 Hrs Total- 1.5 Hr | - An up-to-date first-aid manual. - A list of emergency phone numbers. - Sterile gauze pads of different sizes. - Adhesive tape. - Adhesive bandages (Band-Aids) in several sizes. - Elastic bandage. - A splint. - Antiseptic wipes. | Class Room and Environmental lab |

### 0732B&CE-264. Respond to Natural Disaster

Objective: This module covers the knowledge and  skills required to prepare structure for disaster risk management, develop a response strategy for pre disaster phase, develop a response strategy for disaster phase and develop a response strategy for post disaster phase.

Duration: 12 Hours							Theory: 04 Hours				Practice: 07 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare structure for disaster risk management | Trainee will be able to: - Identify the authority at national level - Identify authorities at provincial level - Identify different governing bodies at sub sequent levels - Develop structure for disaster risk management | - Introduction to disasters - Causes of natural disasters - Effects of natural disasters - Remedial measures for natural disasters - Disaster risk management Practical Activity Prepare structure for disaster risk management | Theory-01 Hr Practical-02 Hrs Total- 03 Hr | - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room and Environmental Lab |
| - Develop a response strategy for pre disaster phase | Trainee will be able to: - Identify risks of disasters - Enlist activities for immediate response - Identify tools to aware people of risk - Propose an immediate response plan | - Tools and techniques to aware people of risk - Denial, deliberation and the decisive moment. Practical Activity Develop a response strategy for pre disaster phase | Theory-01 Hrs Practical-01 Hrs Total- 02 Hrs | - Eraser - Pencil - Sharpener - Scale - Notepad - Charts | Class Room and Environmental Lab |
| - Develop a response strategy for disaster phase | Trainee must be able to: - Identify the measures - Enlist the activities for disaster response - Make a chart for disaster relief activities | - Information of the activities for disaster response i.e. Mitigation, Preparedness, Response, and Recovery Practical Activity Develop a response strategy for disaster phase | Theory-01 Hrs Practical-02 Hr Total- 03 Hrs | - Eraser - Pencil - Sharpener - Scale - Notepad | Class Room and Environmental Lab |
| - Develop a response strategy for post disaster phase | Trainee must be able to - Identify the measures - Enlist the tools to assess the damage - Propose a plan to rehabilitation/reconstruction - Propose a plan for sustainable development | - Tools to assess damage - Resources to incorporate sustainable development Practical Activity Develop a response strategy for post disaster phase | Theory-01 Hrs Practical-02 Hrs Total- 03 Hr | - Eraser - Pencil - Sharpener - Scale - Notepad - Chart | Class Room and Environmental Lab |

# 9.27. Construction Project planning and Management

- Develop Project Management Life Cycle

Objective: This module covers the knowledge and  skills required to perform operations for initiation of project, support project planning and design processes, assist with project control and execution Assist with project finalization-closure.

Duration: 20.00 Hours							Theory: 8.00	Hours				Practice: 12.00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform operations for initiation of project. | Trainee will be able to: - Prepare documents for project. - Establish relationship between the project and broader organisational strategies and goals - Assist in negotiating and documenting project objectives, outcomes and benefits - Assist in establishing the project governance structure-Induct requisite staff. - Assist in drafting a project charter for approval- feasibility report. - Prepare conditions applicable for project. - Open project office. | - Types of civil engineering projects - Objectives and functions of project management - Stages and activities involved in construction projects i.e. planning stage, designing stage, tendering stage and execution stage. - Classification of works -original, major, minor & petty work, annual repair and special repair works. - Parties/persons involved in a construction project- scope of duties & responsibilities of construction team - Importance and role of each member of construction team - Organizations-line staff, direct and functional organization, their features, merits and demerits giving merits and demerits of each - Engineering departments of government - Organizational chart of C&W department Irrigation & power department, public Health Engineering department - Duties of different officers of works departments - Power of sanction of various officers of works departments - Classes of establishments in works department - Qualities of project Manager - Project management life cycle i.e. project initiating, planning, execution and project closer. - Procedure for project initiating, planning, execution and project closer. Practical Activity Draft operations for initiation of a road project | Theory-3 Hrs Practice-4 Hrs Total- 7 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Support project planning and design processes | Trainee will be able to: - Support breakdown of project objectives into achievable project deliverables - Assist in identifying project stages, and key requirements for stage completion-resources, budget, standards, and communication. - Identify project milestones and map clearly against time and objectives - Assist in consolidating associated plans and baselines in project management plan - Support negotiation with relevant stakeholders and project authority to gain approval of project plan | - Procedure for project initiating, planning, execution and project closer. Practical Activity Draft operations for planning and design of a road project | Theory-2 Hrs Practice-2 Hrs Total- 4Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Assist with project control and execution | Trainee will be able to: - Maintain and update records against project deliverables and plans at required intervals - Prepare status reports on project progress and identified issues - Assist with undertaking an impact analysis of proposed changes to the project - Maintain relevant project logs and registers accurately and regularly to assist with project audit - Update associated plans to reflect project progress against baselines and approved changes | - Procedure for project initiating, planning, execution and project closer. Practical Activity Draft reports and documents for control and execution of a road project | Theory-2 Hrs Practice-3 Hrs Total- 5 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Assist with project finalization-closure. | Trainee will be able to - Assist with project finalisation activities where required - Prepare project products and associated documentation for handover to client - Assist in completing financial, legal and contractual obligations where required - Contribute perspectives on project performance to assist with project review assessments | - Procedure for project initiating, planning, execution and project closer. Practical Activity Draft operations for finalization of a road project. | Theory-1 Hrs Practice-3 Hrs Total- 4 Hrs | Lead Pencil Paper Eraser Rules | Class Room |

- Develop Preliminary Project Plan

Objective: This module covers the knowledge and  skills required to prepare flow chart for tendering process, prepare tender documents, tender notice and collect proposals.

Duration:  20.00 Hours							Theory: 8.00	Hours				Practice: 12.00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform Survey of Area | Trainee will be able to: - Perform reconnaissance survey of area for project. - Perform topographic survey of area for project. - Prepare map of area for project. | - Importance of preliminary planning - Procedure for performing preliminary Survey. Practical Activity Carryout Preliminary survey for a road project. | Theory-2 Hrs Practice-2 Hrs Total- 4 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Allocate fund & select site of project | Trainee will be able to: - Prepare rough cost estimate. - Get Administrative approval of project. - Mark options on the prepared map. - Select most suitable and economic site. | - Procedure for preparation of estimate - Procedure for selection of site of project Practical Activity Get Admin approval & mark site on map for road project. | Theory-2 Hrs Practice-2 Hrs Total- 4 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Design the project features. | Trainee will be able to: - Perform Detailed Survey. - Prepare structural design of components of project. - Prepare geometric deign / drawings of project. - Prepare working drawings of project. | - Procedure for detailed Survey. - Procedure for design- geometric & structural. - Working drawings Practical Activity Perform Detailed Survey & Design Project features. | Theory-2 Hrs Practice-4 Hrs Total- 6 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Prepare detailed estimate of project- fund allocation. | Trainee will be able to: - Work out quantities of items of work. - Prepare cost estimate. - Get technical sanction from the authority. | - Detailed estimate of project. - Technical Sanction procedure Practical Activity Get technical sanction on detailed estimate. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Conduct feasibility study for execution of project. | Trainee will be able to: - Draft construction and system feasibility- resources for project. - Identify budget allocation and earning from project- economy. - Identify social benefits from the project-operational. - Estimate quantity of work, time, and available resources for project. - Decide the feasibility and prepare feasibility report of project. | - Pre-feasibility study. - Data to be collected and aspects to be considered in feasibility report - Types of feasibility study. - Steps involved in feasibility study. - Aspects to be considered during preparation of project report - Feasibility report and project report Practical Activity Conduct feasibility study for execution of project. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Lead Pencil Paper Eraser Rules | Class Room |

- Develop Construction Project Plan

Objective: This module covers the knowledge and  skills required to specify the individual activities, determine the sequence of those activities, draw a network diagram, estimate the completion time for each activity, level the resources, apply constraints, identify the critical path (longest path through the network), update the CPM diagram as the project progresses.

Duration:  20.00 Hours						Theory: 8.00	Hours				Practice: 12.00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Specify the individual activities. | Trainee will be able to: - Collect data from estimate of construction project. - Identify pre-construction tasks. - Identify time dependency of activities- Dummy activities. - Divide the work into smaller parts. - Define the activities depending on resources. - Prepare the list of activities along with normal duration | - Objectives of scheduling - Activities in constructions work. Practical Activity Work out activities in road construction project. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Determine the sequence of those activities. | Trainee will be able to: - Identify the activities of construction project. - Find out predecessor and successor of each activity. - Arrange them in sequence. | - Procedure of making constructions schedule i.e. sequencing and time computation of each activity - Need of material, equipment and Labor schedules. - Methods of procurement for Labor, materials and equipment Practical Activity Perform sequencing of activities in road construction project. | Theory-1.5 Hrs Practice-2 Hrs Total- 3.5 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Draw a network diagram. | Trainee will be able to: - Represent each activity with arc/ line with an arrow. - Draw ellipse/ circles at start and end of activities. - Observe the direction of arrows. | - Planning by bar chart-time and progress chart Gantt Chart - Bar chart and its limitations - Advantages of project planning by network analysis-critical path method. - Procedure to draw C.P.M. network for a construction project Practical Activity Perform sequencing of activities in road construction project. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Estimate the completion time for each activity. | Trainee will be able to: - Examine the resources for each activity. - Estimate the workability of workforce - Examine the productivity of machinery. - Determine the activities normal duration. - Write down time required to complete each activity. | - Calculation of network time. - Preparation of work progress charts. - Construction machinery- characteristics, operations and safety. - Cost of owning and operating of construction machinery - Factors affecting selection of construction machinery - Productivity of different construction machinery, e.g. Bulldozer, Excavator, etc Practical Activity Perform sequencing of activities in road construction project. | Theory-1.5 Hrs Practice-1 Hrs Total- 2.5 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Identify the critical path (longest path through the network) | Trainee will be able to: - Perform forward pass by formula at each project activities. - Write early start time and early finish time for each activity. - Perform backward pass by formula at each project activities. - Write late finish time and late start time for each activity. - Level the resources of project. - Apply constraints due to resources, time, environment, season. - Calculate float/ slack time for each activity. - Decide the critical path of construction project- activities with zero float/ slack time. - Calculate the duration of project for completion. | - Performance of forward pass and backward pass. - Calculation for network free float and total float - Calculation for network critical path. - Scheduling and leveling of Resources. Practical Activity Work out critical path and apply resources along with constraints. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Expedite/ crash progress of work | Trainee will be able to: - Enhance workforce force and machinery for activities. - Enhance daily working hours. - Assess the impact of each on cost. - Calculate critical durations for activities. - Calculate time for completion of crash programming. | - Reviewing of network and crash programming - Cash programming for network. - Impact of crash programming on cost. Practical Activity Perform Crash programming for construction project. | Theory-1 Hrs Practice-1 Hrs Total- 2 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Update the CPM diagram as the project progresses. | Trainee will be able to: - Record actual time duration during construction on network. - Assess the actual cost. | - Monitoring of project progress Practical Activity Perform monitoring of construction project. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Lead Pencil Paper Eraser Rules | Class Room |

- Conduct on-site supervision of construction projects

Objective: This module covers the knowledge and  skills required to prepare map of proposed site, prepare contract documents, prepare Working Drawings, draft Specifications, Design of RCC slabs, lintels, T & L beams, columns and column footings, prepare estimates- Cost, Materials, Time (labor, machinery), layout Structures and conduct Planning for project for framed structure class room.

Duration:  20.00 Hours							Theory: 8.00	Hours				Practice: 12.00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Determine quality requirements | Trainee will be able to: - Determine quality objectives, standards and levels, with input from stakeholders and guidance of a higher project authority, to establish the basis for quality outcomes and a quality management plan - Select and use established quality management methods, techniques and tools to determine preferred mix of quality, capability, cost and time - Identify quality criteria, obtain agreement from a higher project authority and communicate to stakeholders, to ensure clarity of understanding and achievement of quality and overall project objectives - Include agreed quality requirements in the project plan and implement as basis for performance measurement | - Need for inspection of works - Duties of various inspecting officers - Quality management theory, techniques, tools and methodologies - Roles and responsibilities in project management- sub-engineer regarding works, store and account Practical Activity Draft Quality Requirements for a road project. | Theory-3 Hrs Practice-3 Hrs Total- 6 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Implement quality assurance processes | Trainee will be able to: - Measure and document results of project activities and product performance throughout the project life cycle to determine compliance with agreed quality standards - Identify causes of unsatisfactory results, in consultation with client, and recommend appropriate actions to a higher project authority to enable continuous improvement in quality outcomes - Conduct inspections of quality processes and quality control results to determine compliance of quality standards to overall quality objectives - Maintain a quality management system to enable effective recording and communication of quality issues and outcomes to a higher project authority and stakeholders | - Use of site order book - Principles of supervision - Need and methods of quality control - Points to be considered in enforcing specifications - Necessity for sampling and testing of materials Practical Activity Carry out Quality Assurance Processes for a road project. | Theory-3 Hrs Practice-6 Hrs Total- 9 Hrs | Lead Pencil Paper Eraser Rules | Class Room |
| - Implement project quality improvements | Trainee will be able to: - Review processes and implement agreed changes continually throughout the project life cycle to ensure continuous quality improvement - Review project outcomes against performance criteria to determine the effectiveness of quality management processes and procedures - Identify and document lessons learned and recommended improvements, and pass on to higher project authority for application in future projects | - Methods for managing and improving performance - Relevant legislation codes and national standards: Practical Activity Carry out agreed Quality improvements for a road project. | Theory-2 Hrs Practice-3 Hrs Total- 5 Hrs | Lead Pencil Paper Eraser Rules | Class Room |

- Perform Basic operation in Primavera P6

Objective: This module covers the knowledge and  skills required to Load Primavera Software, Unload Primavera Software, Recognize Toolbars, Customize P6 Screen and Display Gantt Chart.

Duration:  20.00 Hours						Theory: 8.00	Hours				Practice: 12.00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Load & unload primavera P6 Software. | - Trainee will be able to: - Locate P6 icon & double click on it - Exit from software by menu/ close button/ short key. | - Uses of Primavera Software - Loading / Unloading procedure of P6 software - Practical Activity - Perform loading and unloading of Primavera P6 | Theory-2 Hrs Practice-3 Hrs Total- 5 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Prepare interface of software | - Trainee will be able to: - Identify parts of application window. - Identify parts of working window. | - Interface of Primavera P6. - Benefits of true Enterprise Project Management systems - Practical Activity - Draw interface of Primavera P6 | Theory-2 Hrs Practice-3 Hrs Total- 5 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Customize P6 Screen Layout | - Trainee will be able to: - Identify the ‘Layout Options Bar’ on the top in Activities category - Add required number of columns to the layout. - Add created columns in existing layout. - Move columns to the left and right for adjustment. - Change the order of columns as per requirement. | - Names and function of all Views, Layouts, Toolbars and Icons in P6 - Process of Customizing Layouts - Practical Activity - Display screen layouts and change the same in Primavera P6 | Theory-2 Hrs Practice-3 Hrs Total- 5 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Display Gantt Chart | - Trainee will be able to: - Open Gantt chart window. - Select ‘Layout Options Bar’ ‘Group and Sort…’ - Select ‘Group by’ dropdown list ‘WBS’ - Select other required parameters. - Use of snip tool for screen shot. | - Process of displaying Gantt chart. - Process of taking screen shot. - Practical Activity - Display Gantt chart and take screen shots in Primavera P6 | Theory-2 Hrs Practice-3 Hrs Total- 5 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |

- Perform road Project Activities Scheduling in Primavera P6

Objective: This module covers the knowledge and  skills required to Add Project, Create WBS, Create Activities, Create Relationships, Create Schedule.

Duration: 20.00 Hours						Theory: 8.00	Hours				Practice: 12.00Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Add Project in Primavera | Trainee will be able to: - Open ‘Projects’ screen - Select the EPS for the new project - Enter Project ID and Project Name - Enter Planned Start Date for the project and target deadline in ‘Must Finish by’ - Select the ‘Responsible Manager’ - Select ‘Assignment rate Type’ - Check ‘no do not run project architecture’ - Open Projects Screen and click on the newly created project - Open ‘Notebook’ tab in the opened ‘Project Detail’ window  ‘Add’  Select the type of note to be added - Open ‘Dates’ tab  Add relevant dates | - Project, its types. - Activity, activity types, duration and dependencies. - Activity relationships, constraints, float, critical path, forward & backward pass. - Resource and enlist its types - Gantt Chart Practical Activity Add a new road project in Primavera. | Theory-2 Hrs Practice-2 Hrs Total- 4 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Create WBS of project in Primavera. | Trainee will be able to: - Click ‘WBS’ tab on the Directory Bar on the left to open WBS Window - Click ‘Add’ from the command bar on the right - Assign a ‘Code’ and ‘Name’ to the newly added WBS Element - Add more WBS elements - Take a WBS element to a higher hierarchy level by clicking on the LEFT arrow key on the command bar - Take a WBS element to a lower hierarchy level by clicking on the RIGHT arrow key on the command bar - Move a WBS element to a higher head by clicking the UP arrow key on the command bar - Move a WBS element to a lower head by clicking the DOWN arrow key on the command bar - Create a detailed WBS | - Work Breakdown Structure - Adding WBS elements in different elements. - Coding of WBS elements. Practical Activity Add new WBS elements for road project in Primavera. | Theory-2 Hrs Practice-1 Hrs Total- 3 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Create Activities of project in Primavera. | Trainee will be able to: - Click the ‘Activities’ icon on the directory bar on the left to open Activities Screen - Click ‘Add’ button on the command bar on the right - Type activity name in the relevant box - Click ‘Activity Type’ tab of the activity - Select ‘Start Milestone’ for the first activity - Add expected activity duration in the ‘original duration’ column - Add activities to other WBS levels - Click on ‘Show/Hide Activity Detail’ icon on the toolbar to open Detail View - Add other parameters in the tabs shown | - Activity attributes. - Task dependent activity, resource dependent activity, start milestone activity, finish milestone activity, level of effort activity - Duration Types. - Purpose of using Activity Codes. Practical Activity Add Activities and their details for road project in Primavera. | Theory-2 Hrs Practice-3 Hrs Total- 5Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Create Relationships between activities of project in Primavera. | Trainee will be able to: - Click ‘Activity Network’ icon on the toolbar - Select a WBS Node - Select simultaneously the activities whose relationship is to be defined. - Click ‘Edit’ dropdown menu Click ‘Link Activities’ to create an automatic Start-to-Finish relationship between the two. - FROM GANTT CHART VIEW - Click the ‘Gantt Chart’ icon on the toolbar - Click the Activity name from the Activity Table Window on the left - Click the ‘Relationships’ tab on the Activity Detail window on the bottom - In the ‘Predecessors’ and ‘Successors’ window, click ‘Assign’ Select relevant activity from the pop-up window. - Click the “Assign’ button on the right of ‘Assign Successor/Predecessor’ pop-up screen Click on the Close button to exit. - Click on the cell in the ‘Relationship Type’ column to change the activity relationship from FS to either SS, or SF or FF, as required. - TO VIEW ACTIVITY RELATIONS GRAPHICALLY - Click ‘Relationship Lines’ icon on the toolbar - Identify the relationship arrows now visible in the Gantt Chart on the right | - Relationships & their Types - Forward and Backward Pass, Practical Activity Create Relationships between activities for road project in Primavera. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Create Schedule of activities of project in Primavera. | Trainee will be able to: - Click on the ‘Schedule’ icon on the Toolbar OR Press ‘F9’ Key - Add all the relevant parameters in the ‘Schedule’ pop-up window, Click ‘Schedule’ button on the right. | - Parameters of activities - Scheduling of activities Practical Activity Create schedule of activities for road project in Primavera. | Theory-1 Hrs Practice-4Hrs Total- 5 Hrs | Paper USB Printer Cartridge Equipment Computer | Class Room and computer Lab |

- Perform road Project Resources Costing &Planning in Primavera P6

Objective: This module covers the knowledge and  skills required to add Constraints, Create & Assign Calendar, Add, Assign & Analyze Resources, Add Cost and Baseline the Project.

Duration: 20.00 Hours						Theory: 8.00	Hours				Practice: 12.00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Add constraints of activities of project in Primavera. | Trainee will be able to: - ADDING ACTIVITY CONSTRAINTS - Select the activity - Click on the ‘Status’ tab in the Activity Detail Window. - In the ‘Constraints’ box, select Constraint types and required dates - Schedule the project - ADDING PROJECT CONSTRAINTS - Click on the ‘Projects’ icon in the directory bar to open Projects Screen - Select the Project - Click on the ‘Dates’ tab in the Project Detail Window. - Add a ‘Must Finish by’ date in the Schedule Dates box. - Run Scheduler | - Activity Constraints. - Adding constraints Practical Activity Add constraints of activities for road project. | Theory-1 Hrs Practice-1 Hrs Total- 2 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Create Calendar for activities of project in Primavera. | Trainee will be able to: - Go to ‘Enterprise’ menu Click ‘Calendars’ - Click ‘Add’ on the left in the ‘Calendar’ pop-up screen - Select the appropriate existing calendar to copy from. - Enter the name for the new calendar and press Enter - Select the created calendar and click ‘Modify’ on the left side - Display ‘detailed work hours/day’ by selecting that option on the top - Made necessary modifications - TO ASSIGN NON-WORKING/WORKING DAY - Select the required day of the week Click ‘Work’/” Nonwork’ as required - TO CHANGE WORKING DAYS PER WEEK - Click ‘Workweek’ on the left Select the day to be marked working/non-working Select the hours of the day to be marked Click the appropriate button (‘Work’/’Nonwork’) | - Work calendar - Work /non-work days Practical Activity Add constraints of activities for road project. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Assign Calendars to activities of project in Primavera. | Trainee will be able to: - Add ‘Calendar’ column to the layout - Click Layout Options bar Click ‘Show on Top’ Click ‘Activity Usage Spreadsheet’ (OR Display Activity Usage Spreadsheet) - Double click the ‘Calendar’ field of the required activity - Select the required calendar from the ‘Select Activity Calendar’ pop-up Click ‘Assign’ button on the left - OR - Go to the Activity Detail window ‘General’ ‘Activity Calendar’ Select the required Calendar. - Run Scheduler | - Assigning of calendars. Practical Activity Add constraints of activities for road project. | Theory-1 Hrs Practice-1 Hrs Total- 2 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Add Resources to activities of project in Primavera. | Trainee will be able to: - Click ‘Resources’ icon on the Directory bar to open ‘Resource Dictionary’ - Click the Display bar ‘Details’ to display the ‘Resource Details’ window - Click ‘Add’ button on the right side of the screen - Add resource details in the Resource Detail tabs: - - Give Resource ID and Name in ‘General’ tab - Define Resource Type and Working Hours in ‘Details’ tab - Add “effective date’, ‘Max units/time’ and ‘Hourly Rates’ | - Resources and types. - Adding resources to activities. - Working hours Practical Activity Add constraints of activities for road project. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Assign Resources of activities of project in Primavera. | Trainee will be able to: - Select the activity to which resources are to be assigned - Go to Activity Detail window ‘Resources’ tab ‘Add Resource’ Select the appropriate resource from the ‘Assign Resources’ pop-up window - Indicate the Primary Resource from the list of resources added. | - Assigning of resources to activities. Practical Activity Add constraints of activities for road project. | Theory-1 Hrs Practice-1 Hrs Total- 2 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Add Cost of activities of project in Primavera. | Trainee will be able to: - Select the required activity - Open ‘Expenses’ tab in the Activity Detail window Click ‘Add’ Give details as required | - Roles and Hourly Rates. - Costing for activities Practical Activity Add constraints of activities for road project. | Theory-1 Hrs Practice-1 Hrs Total- 2 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Analyse Resources of activities of project in Primavera. | Trainee will be able to: - Click ‘Resource Usage Profile’ from the toolbar to display ‘Resource Usage Profile Graph’ - Click on each project resource listed on the left to display their respective working hours/week graphs - Identify work hours per week, working limit and over-allocation from the bar chart on the right - Select the ‘Resource’ box to display only the activities involving the selected resource. - Identify overlapping activities in the Gantt Chart - Click ‘Resources’ button on the right to re-allocate the activity to another resource Click ‘Replace Resource’ button on the right Select the appropriate resource as a replacement. - Confirm the selected resource is not over-allocated as a result. | - Resource usage profile graph. - Working hours, limit - Overlapping of activities - Over-allocation Practical Activity - Add constraints of activities for road project. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Perform Baseline process for Project. | Trainee will be able to: - CREATE BASELINE - Click ‘Project’ menu ‘Maintain Baselines…’ ‘Add’ Select ‘Save a copy of the current project as a new baseline’ - Give Baseline Name in the appropriate Box - Categorize the Baseline type by selecting the appropriate type from the ‘Baseline Type’ dropdown menu - ASSIGN BASELINE - Click ‘Project’ menu ‘Assign Baselines…’ - Select the required ‘Project Baseline’ from the dropdown menu - DISPLAY THE BASELINE GRAPHICALLY - Click Layout Options Bar Click ‘Bars...’ - Select the box before ‘Project Baseline’ and ‘Project Baseline Milestone’ in the ‘Bars’ window - Observe the modified bars of actual performance in the Gantt Chart and compare them to the baseline | - Baselines. - Assigning baseline. - Usage of base line in evaluating project performance Practical Activity Display performance of project with Baseline for the road project. | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |

- Manage construction Project in Primavera P6

Objective: This module covers the knowledge and  skills required to Status the Project and Mitigate the schedule resulting in monitoring of planning.

Duration: 20.00 Hours						Theory: 8.00	Hours				Practice: 12.00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Status the Project | Trainee will be able to: - Click the ‘Progress Spotlight’ button in the toolbar Drag the spotlight edge in the Gantt chart to increase/decrease the spotlight timeline. - Select the required activity - Open the ‘Status’ tab in the Activity Display window - Check ‘Started’ and ‘Finished’ box in the ‘Status’ section Give appropriate dates - Click ‘Expenses’ tab Add Actual Activity Cost - Click ‘Schedule’ button on the toolbar Add ‘Data Date’ in the ‘Current Data Date’ ‘Schedule’ | - Progress spotlight timeline. - Dates for status of project. - Actual Activity Cost. Practical Activity Perform actual costing expenses along with time. | Theory-4Hrs Practice-6 Hrs Total- 10 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |
| - Mitigate the schedule | Trainee will be able to: - Identify the activities deviating the most from the project baseline - Click on that activity - Perform any/all of the following: - - Add more resources to the activity - Modify the calendar for resources working below their work/hour limit - Remove constraints causing delay to the activity - Schedule the project | - Deviation of activities. - Crash programming for project progress. - Monitoring and control Practical Activity Perform actual costing expenses along with time. | Theory-4 Hrs Practice-6 Hrs Total-10 Hrs | Paper USB Printer Cartridge Equipment Computer Lab Equipment | Class Room and computer Lab |

- Identify Project Risk and Consequences on Project life cycle

Objective: This Module covers the skills and knowledge required to identify Risks, hazard, Risks Classification and its consequences on the Project life cycle.

Duration: 11 Hours								Theory: 05 Hours				Practice: 06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify potential Risks in Construction Projects, identify the Project phases | Trainee will be able to: - Prepare Construction Project life cycle - Prepare Construction Project Risks analysis team - Enlist the Project Phases - Identify Risks Management Process | - Define the Risk and Hazards. - Define the Risk Management. - Explain the Project Life Cycle and Project phases. - Explain Project triangle, Cost, time and Quality. Practical Activity - Prepare the project Phases - Prepare Risks assessment model flow diagram | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | Calculator Ruler Pencil Drawing lab equipment | Class Room |
| - Prepare Classification of identified risks and breakdown structure | Trainee will be able to: - Plan to identify the Risks in each Phase of Project - Prepare the Classification of Identified Risks - Prepare Risk Break down Structure (RBS) | - Explain the procedure to identify potential Risks. - Explain the Classification of Risks and Define the Risk Beak Down Structure. Practical Activity - Prepare the list of potential risks in each phase of the project. - Prepare the Risk break down structure. | Theory-02 Hrs Practical-02 Hrs Total- 04 Hrs | Calculator Ruler Pencil Drawing lab equipment | Class Room |
| - Determine the likelihood and impact of identified risks | Trainee will be able to: - Prepare the data and information from previous project history - Prepare the scoring criteria to assess the likelihood and Impact of the identified risks. | - Explain brain storming. - Define the Likelihood and Impact of Risk. - Explain the Scoring Criteria to assess the Likelihood and impact of risks. Practical Activity - Prepare Scoring Criteria to assess the risks. | Theory-02 Hrs Practical-02 Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |

- Perform Evaluation to assess the severity of Risk and formulate the strategy to manage the risk

Objective: This Module covers the skills and knowledge required to Evaluate the risks Levels in term of (Very high, high, moderate, Low) and strategy formulation to manage the risks effectively and rationally.

Duration: 12 Hours								Theory: 04 Hours				Practice: 08 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Evaluation of Risk and prepare the severity of Risks. | Trainee will be able to: - Identify Approach selecting for Risks analysis method - Perform the Qualitative Semi Qualitative and Quantitative methods to evaluate the risks. - Perform Risks analysis methods and Quantification. - Identify Risks management process | - Define Risk analysis - Explain the approach selecting the Risks analysis method. - Explain Qualitative, Semi Qualitative and Quantitative methods to evaluate the Risks. - Explain the procedure to perform Risks analysis method and Quantification. Practical Activity - Prepare Risks evaluation model. | Theory-02 Hrs Practical-04 Hrs Total- 06 Hrs | Calculator Ruler Pencil Drawing Lab | Class Room |
| - Formulate the Strategy to Mitigate and Manage the Risks. | Trainee will be able to: - Prepare the data management and matrixes to present the severity and Risk Level. - Prepare Risk Model to Manage the risk - Apply the Risk Model on each Risk to mitigate the adverse consequences of risk - Prepare risks monitoring process - Prepare the reference for future. | - Explain the Data management and risks matrixes. - Explain the Risk management model to avoid, mitigate, transfer and accept the Risks. - Explain monte carlo simulation - Define the party who can bear the risk. - Identify the risks cannot be transferred. - Explain Risk monitoring Process. Practical Activity - Prepare Risks Matrixes for Cost, time and Quality. - Prepare Risks Management model flow diagram | Theory-02 Hrs Practical-04 Hrs Total- 06 Hrs | Calculator Ruler Pencil Drawing Lab | Class Room |

# National Diploma in Civil Technology Level-5 Part-II

# 9.28. Transportation Engineering II

- Plan Route Alignment of Highway

Objective: This competency standard covers the skills and knowledge required to analyze considerations for road alignment and demark the road alignment on given contour map.

Duration: 12 Hours								Theory: 6 Hours				Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Analyze road alignment | Trainee will be able to: - Identify factors affecting alignment of road - Evaluate factors identified as per specifications | - Necessity, importance and modes of transportation - Highway terminology’s - Purpose of road alignment - Fundamental principles of road alignment - Factors influencing selection of alignment for a road in plain and hilly area Practical Activity: - Evaluate road alignment factors | Theory-3 Hrs Practice-3 Hrs Total- 6 Hrs | - Paper - Pencil | Class Room and Drafting Lab |
| LU2.Mark the road alignment on given contour map | Trainee will be able to: - Identify drawing tools - Fix the contour map on drafting table - Mark the alignment of road | - Surveys required for fixing alignment - Drawings for road alignment - Practical Activity: - Locate the alignment of road on given contour map | Theory-3 Hrs Practice-3 Hrs Total- 6 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and Drafting Lab |

- Design Highway Geometrics

Objective: This competency standard covers the skills and knowledge required to draw cross section of typical geometric design including stopping sight distance, overtaking sight distance, super elevation and widening of road at curve.

Duration: 12 Hours									Theory:  6 Hours				Practice: 6Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Draw cross sections of road - National, Urban and Motorway | Trainee will be able to: - Identify national standards for national highway, urban roads and motorway - Fix drawing sheet on drafting table - Draw cross section of National Highway as per local standards - Draw cross section of urban roads as per local standards - Draw cross section of Motorway - Label components of highway geometrics | - Description of highway geometrics - Types of roads Practical Activity: - Produce cross sections of various types of roads | Theory-1 Hrs Practice-2 Hrs Total- 3 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| - Find out stopping sight distance | Trainee will be able to: - Identify given parameters for stopping sight distance - Calculate stopping sight distance | - Explanation of stopping sight distance. - Explanation of the factors affecting stopping sight distance Practical Activity: - Calculate stopping sight distance from given data | Theory-1 Hrs Practice-1 Hrs Total- 2 Hrs | - Drawing Sheets - Led Pencil - Eraser | Class Room and drafting lab |
| - Find out overtaking sight distance | Trainee will be able to: - Identify given parameters for overtaking sight distance - Calculate overtaking sight distance | - Explanation of overtaking sight distance. - Explanation of the factors affecting overtaking sight distance Practical Activity: Calculate overtaking sight distance from given data | Theory-1 Hrs Practice-1 Hrs Total- 2 Hrs | - Drawing Sheets - Led Pencil - Eraser | Class Room and drafting lab |
| - Design super elevation | Trainee will be able to - Identify given parameters for super elevation - Calculate super elevation | - Super elevation & its necessity - Method of introduction of super elevation Practical Activity: - Calculate super elevation from given data | Theory-2 Hrs Practice-1 Hrs Total- 3 Hrs | - Drawing Sheets - Led Pencil - Eraser | Class Room and drafting lab |
| - Design widening of road at curve | Trainee will be able to - Identify given parameters for widening of road at curve - Calculate widening of road at curve | - Widening pf curve & its necessity - Method of introduction of widening of curve Practical Activity: - Calculate widening of road at curve from given data | Theory-1 Hrs Practice-1 Hrs Total- 2 Hrs | - Drawing Sheets - Led Pencil - Eraser | Class Room and drafting lab |

- Perform tests on stone metal for pavement construction

Objective: This competency standard covers the skills and knowledge required to inspect field density by sand replacement method and perform Los-Angeles Abrasion test.

Duration:  12 Hours									Theory: 6 Hours				Practice: 6Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Inspect field density by sand replacement method | Trainee will be able to: - Identify apparatus - Evaluate volume of cone by filling sand of known specific gravity - Evaluate volume of hole on ground - Calculate field density of soil/sub-base/base | - Purpose of field density - Methods for finding field density - Procedure of sand replacement test Practical Activity: - Perform sand replacement test | Theory-3 Hrs Practice-3 Hrs Total- 6 Hrs | - Soil sample - Sand T & E: - Sand bottle - calibrating container - glass plate - base plate - pan - tools for excavating hole - Electronic digital balance - Set of sieves | Class room, Material testing lab and College ground |
| - Perform Los Angeles Abrasion test | Trainee will be able to: - Identify equipment - Prepare sample for test - Place the charges and sample in drum - Operate the machine according to specified revolutions - Determine mass loss after revolutions - Calculate the percentage of wear | - Purpose of Los Angeles Abrasion test - Instruments for Los Angeles Abrasion test - Procedure of Los Angeles Abrasion test Practical Activity: - Perform Los Angeles Abrasion test | Theory-3 Hrs Practice-3 Hrs Total- 6 Hrs | - Coarse aggregates - Bitumen T & E: - Los Angeles abrasion apparatus Spares: - Set of 12 abrasive steel ball | Class room and Material testing lab |

- Perform tests on bitumen

Objective: This competency standard covers the skills and knowledge required to draw structural components of flexible pavement, perform Ductility Test on Bitumen, perform softening point test for bitumen, perform Flash and Fire point test for bitumen / cutback, perform Marshal stability test for bituminous mixture and extract quantitative bitumen from bituminous paving.

Duration: 33 Hours							Theory: 12 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Perform Ductility Test on Bitumen | Trainee will be able to: - Identify the equipment - Prepare sample for test - Place the sample in water bath - Measure the distance covered during test | - Purpose of Ductility test - Instruments for Ductility test - Procedure of Ductility test Practical Activity: - Perform Ductility test | Theory-2 Hrs Practice-3 Hrs Total- 5 Hrs | - Bitumen T & E: - Ductilometer ASTM D113 Accessories: - Briquette mould - Base plate - Complete refrigerating system | Class room, Material testing lab and College ground |
| LU2.Perform softening point test for bitumen | Trainee will be able to: - Identify Ring and Ball apparatus - Prepare sample for test - Assemble the apparatus to perform test - Record temperature at which balls touch the bottom of cylinder | - Purpose of softening point test - Instruments for softening point test - Procedure of softening point test Practical Activity: - Perform softening point test | Theory-2 Hrs Practice-3 Hrs Total- 5 Hrs | - Bitumen T & E: - Ring and ball apparatus ASTM D36 Accessories: - Thermometer 0-100°C - Thermometer 0-200°C - Thermometer ASTM 15 - Thermometer ASTM 16C - Magnetic stirrer, with heating - Bunsen burner - Asbestos Mesh, 100 x 100 mm - Spare Parts: Test ball - Centering guides - Brass ring | Class room, Material testing lab and College ground |
| LU3.Perform Flash and Fire point test for bitumen / cutback | Trainee will be able to: - Assemble the apparatus - Heat the bitumen sample and stir the bitumen to maintain uniform temperature in sample - Apply test flame to sample - Record the temperature for flash point - Record the temperature for fire point | - Purpose of Flash and Fire point test - Instruments for Flash and Fire point test - Procedure of Flash and Fire point test Practical Activity: - Perform Flash and Fire point test | Theory-2 Hrs Practice-3Hrs Total- 5 Hrs | - Bitumen T & E: - Cleveland open cup apparatus ASTM D92 Accessories: - Electric heater - aluminum cup - Thermometer from - 6 to + 400°C - Electric thermo regulator - Heating plate, Rod and clip for the thermometer - Power supply 220 V. 50 Hz. 700 W. | Class room, Material testing lab and College ground |
| LU4.Perform Marshal stability test for bituminous mixture | Trainee will be able to - Identify the equipment - Prepare sample for test - Place the mould in Marshal test setup - Load the mould in Marshal test setup - Note the reading for vertical deformation of specimen - Vary the bitumen content for next trial - Compile data in tabular form - Compute required parameters - Plot the graph using the predetermined parameters - Determine optimum bitumen content | - Purpose of Marshal stability test - Instruments for Marshal stability test - Procedure of Marshal stability test Practical Activity: - Perform Marshal stability test for bituminous mixture | Theory-2 Hrs Practice-4 Hrs Total- 6 Hrs | - Bitumen T & E: - Marshall stability test machines ASTM-D-1559 Accessories: - Load ram - Extruder - Breaking head stability mould - Dial gauge - Bracket device | Class room, Material testing lab and College ground |
| LU5.Extract quantitative bitumen from bituminous paving | Trainee will be able to - Identify centrifuge apparatus - Prepare sample for test - Assemble sample in apparatus according to specifications - Add solvent to disintegrate materials - Extract bitumen form sample - Determine the amount of bitumen | - Purpose of quantitative extraction of bitumen - Instruments of quantitative extraction of bitumen - Procedure of quantitative extraction of bitumen Practical Activity: - Perform quantitative extraction of bitumen | Theory-2 Hrs Practice-4 Hrs Total- 6 Hrs | - Bitumen T & E: - Asphalt centrifuge extractors ASTM D 2172 Accessories: - Filter discs, box of 250 units - Basket & cover, capacity 3000g - Container with 25l.of trichloroethylene - TIn with 200l.of trichloroethylene | Class room, Material testing lab and College ground |
| LU6.Determine consistency of | Trainee will be able to - Identify penetrometer - Prepare sample for test - Examine and clean the needle - Place the sample in assembly to perform test - Record the reading from dial gauge | - Purpose of consistency test of bitumen - Instruments of consistency test of bitumen - Procedure of consistency test of bitumen Practical Activity: - Perform consistency test of bitumen | Theory-2 Hrs Practice-4 Hrs Total- 6 Hrs | - Bitumen T & E: - Standard penetrometer - ASTM D5, D217 Accessories: - Thermostatic bath - Mirror Spare parts: - Penetration needle - Transfer dish - Sample cup | Class room, Material testing lab and College ground |

- Prepare drawings of flexible/rigid pavement

Objective: This competency standard covers the skills and knowledge required to draw structural components of flexible/rigid pavement, draw cross section of rigid pavement showing longitudinal joints and draw cross section of rigid pavement showing transverse joints.

Duration: 9 Hours								Theory: 6 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Draw structural components of flexible pavement | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw cross section of flexible pavement - Label the structural components of flexible pavement | - Structural components of flexible pavement - Construction procedure of flexible pavement - Construction detail of surfacing cold and hot process bituminous macadam, premix asphalt macadam, bituminous concrete pavements and sheet asphalt pavement Practical Activity: - Produce labeled cross section of flexible pavement | Theory-2 Hrs Practice-0.75 Hrs Total- 2.75 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU2.Draw structural components of rigid pavement | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw cross section of rigid pavement - Label the structural components of rigid pavement | - Structural components of rigid pavement - Construction procedure of rigid pavement - Constructional detail of bounded concrete, cement bound macadam, sandwich type macadam, colloidal concrete road Practical Activity: - Produce labeled cross section of rigid pavement | Theory-2 Hrs Practice-0.75 Hrs Total- 2.75 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU3.Draw cross section of rigid pavement showing longitudinal joints | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw cross section of rigid pavement showing longitudinal joints | - Purpose of longitudinal joints in rigid pavement Practical Activity: - Produce cross section of rigid pavement showing longitudinal joints | Theory-1 Hrs Practice-0.75 Hrs Total- 1.75 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU4.Draw cross section of rigid pavement showing transverse joints | Trainee will be able to - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw cross section of rigid pavement showing transverse joints | - Purpose of transverse joints in rigid pavement Practical Activity: - Produce cross section of rigid pavement showing transverse joints | Theory-1 Hrs Practice-0.75 Hrs Total- 1.75 Hrs | Drawing Sheets Led Pencil Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |

- Plan highway drainage

Objective: This competency standard covers the skills and knowledge required to draw cross section of surface drainage and draw cross section of sub surface drainage.

Duration: 9 Hours								Theory: 6 Hours				Practice:3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Draw cross section of surface drainage | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw cross section of surface drainage | - Necessity and importance of highway drainage - Purpose of surface drainage - Surface drainage structures in plain areas - Surface drainage structures in hilly area Practical Activity: - Produce cross section of surface drainage | Theory-3 Hrs Practice-1.5 Hrs Total- 4.5 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU2. Draw cross section of sub surface drainage | Trainee will be able to: - Identify drawing tools for drawing - Fix drawing sheet on drafting table - Draw cross section of sub surface drainage | - Purpose of sub surface drainage - Sub surface drainage structure in plain area - Sub Surface drainage structures in hilly area - Methods of providing sub surface drainage Practical Activity: - Produce cross section of sub surface drainage | Theory-3 Hrs Practice-1.5 Hrs Total- 4.5 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |

- Develop maintenance plan for roads

Objective: This competency standard covers the skills and knowledge required to identify the defects in road and draft a management plan for road maintenance

Duration: 9 Hours								Theory: 6 Hours				Practice:3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify defects in road | Trainee will be able to: - Enlist defects in roads - Analyze causes of road defects - Draw defects in roads | - Explanation of defects in bitumen roads - Causes of defects in bitumen roads - Explanation of defects in concrete roads - Causes of defects in concrete roads Practical Activity: - Draw & identify defects in roads | Theory-3 Hrs Practice-1.5 Hrs Total- 4.5 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |
| LU2. Draft a management plan for road maintenance | Trainee will be able to: - Identify roles of individuals in maintenance of roads - Propose positions for different roles - Draft hierarchy of organization roles according to positions for maintenance of roads | - Remedial measures for defects- scraping of existing layers, patch repair, replacing with reinforced layers, resurfacing etc - Recycling procedure for road used materials Practical Activity: - Prepare hierarchy of organization roles according to positions for maintenance of roads | Theory-3 Hrs Practice-1.5 Hrs Total- 4.5 Hrs | - Drawing Sheets - Led Pencil - Eraser T & E: - Drawing board - T square - Pair of Set squares - Drawing Box (Compasses, divider, protractor, rule) - Sharpener/ Sand paper - Handkerchief - French Curves - Set of Card board scales | Class Room and drafting lab |

# 9.29. Irrigation Engineering

- Produce sketches of canals and head works

Objective: This module covers the knowledge and  skills required to  draw a skeleton map of Pakistan showing rivers, main and link canal, Head works and barrages, Draw the plan of a multipurpose irrigation project, Draw a plan showing general layout of river training and protection works, Draw cross section of dams, Draw a plan showing the general layout of head works of a perennial canal, Draw cross-sections of a weir floors, Draw the plan and section of a silt ejector.

Duration:  39 Hours							Theory: 21 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Draw a skeleton map of Pakistan showing rivers, main and link canal, Head works and barrages | Trainee will be able to: - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Draw map of Pakistan. - Show rivers on map. - Draw barrages, main canals, head works and link canals | - Define Irrigation - State necessity and scope of irrigation. - What are various sources of irrigation water. - Describe the irrigation network in Pakistan. - What are types of irrigation i.e. flow irrigation and lift irrigation. - State the important barrages of Pakistan. - State the characteristics of Pakistan’s rivers. Practical Activity Draw a skeleton map of Pakistan showing rivers, main and link canal, Head works and barrages | Theory-3 Hrs Practical-3 Hrs Total- 6 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room and workshop |
| - Draw the plan of a multipurpose irrigation project | Trainee will be able to: - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Draw plan of project. - Show all details on plans | - State Indus Basin Project. - Explain need and details of replacement works in Indus Basin Project. - Know IRSA and PIDA Practical Activity Draw the plan of a multipurpose irrigation projects | Theory-3 Hrs Practical-2 Hrs Total- 5 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room and workshop |
| - Draw a plan showing river training and Protection works. | Trainee must be able to: - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Draw plan of training and Protection works. - Show all details on plans | - Explain with sketches the different types of protective and river training works. - Objects of River Training. Practical Activity Draw plan showing River Training and Protection works | Theory-3 Hrs Practical-2 Hrs Total- 5 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room and workshop |
| - Draw cross section of dams | Trainee must be able to - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Draw x-section of dam. - Label all necessary details on plan. | - Define Dam - State classification of dams i.e. based on function and use, structural design, materials of construction, shape of X-section - State factors to be considered during choice of dam for hydro project - Explain the construction details of earthen dams - State the causes of failure of earthen dams and their remedies - Explain methods to control seepage in earthen dams - Explain methods & necessity of raising of dams. Practical Activity Draw Cross section of Dams | Theory-3 Hrs Practical-2 Hrs Total- 5 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room and workshop |
| - Draw a plan showing the general layout of head works of a perennial canal | Trainee must be able to - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Select suitable scale. - Draw layout of head works. - Label all necessary details on plan. | - What are Head Works? - Define Perennial Canal. - Explain head regulator and its types, i.e. flume, meter flume. Practical Activity Draw plan showing the general Layout of Head Works of a Perennial Canal | Theory-3 Hrs Practical-3 Hrs Total- 6 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room And Workshop |
| - Draw cross-sections of a weir floors | Trainee must be able to - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Select suitable scale. - Draw weir’s section. - Label all necessary details. | - Define weir and barrages. - Distinguish between barrages and weirs and explain their purposes. - Describe with sketches the components/parts of a barrage. - State the factors governing the site selection of a barrage. - State the types of weirs. - Explain surface flow at weirs. Practical Activity Draw Cross section of Weir Floors | Theory-3 Hrs Practical-3 Hrs Total- 6 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room And Workshop |
| - Draw the plan and section of a silt ejector | Trainee must be able to - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Select suitable scale. - Draw plan and section. - Label all necessary details. | - What is Silt Ejector? - Describe the necessity and importance of regulation and silt controlling works. - Describe with sketches the silt ejector, silt vanes, silt excluder and skimming platform. Practical Activity Draw the Plan and Cross Section of a Silt Ejector. | Theory-3 Hrs Practical-3 Hrs Total- 6 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room And Workshop |

- Design Canals

Objective: This module covers the knowledge and  skills required to find design discharge of Karif and Rabi crops for given area, Draw the alignment of a canal, minor distributary and watercourses on contour map, Design a canal by Kennedy Critical velocity theory, Design a canal by Lacey's Regime velocity and  Draw the typical X-sections of channels.

Duration: 37 Hours							Theory: 22 Hours				Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Find design discharge of Kharif and Rabi crops for given area | Trainee will be able to: - Identify area of crops (Kharif or Rabi) - Identify delta of each crop - Calculate duty - Estimate total water requirement | - Describe the term water requirement of crops. - Enlist factors affecting water requirement - Define the terms crop period, base period, Kharif-Rabi ratio, core watering, cash crop, crop rotation, and delta. - Explain duty of water, factors affecting and its significance. - State relation between duty and delta. Practical Activity Find design discharge of Karif and Rabi crops for given area | Theory-3 Hrs Practical-3 Hrs Total- 6 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room and workshop |
| - Draw the alignment of a canal, minor distributary and watercourses on contour map | Trainee will be able to: - Arrange drawing instruments - Draw a contour map with two-meter interval - Draw alignment of canal, distributary and water course on some scale | - State the types of canal. - Explain basic methods of water distribution to field i.e. Surface irrigation method, Furrow method, Contour method, Flooding method. - Explain Sprinkler irrigation method - Explain Sprinkler irrigation method | Theory-4 Hrs Practical-3 Hrs Total- 7 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils | Class Room and workshop |
| - Design a canal by Kennedy Critical velocity theory | Trainee must be able to: - Calculate area of cross-section and wetted perimeter - Assume depth and calculate critical velocity - Calculate C and R - Find velocity by using formulae - Calculate CVR - Find design parameters. | - State classification of canals - Explain with sketches, components of a canal section - Explain the factors governing alignment of canal. - Describe Kennedy's critical velocity for design of canals - What is Wetted Perimeter? Practical Activity Design of Canal by Kennedy Critical Velocity Theory | Theory-6 Hrs Practical-3 Hrs Total- 9 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room and workshop |
| - Design a canal by Lacey's Regime velocity | Trainee must be able to - Identify formulae for design - Calculate f, V, A, Pw. Assume depth and calculate critical velocity - Calculate C and R - Find velocity by using formulae - Calculate CVR - Find design parameters. | - Define Critical Velocity - What is CVR? - Describe Lacy's regime velocity for design of canals - Describe keep up and maintenance of canals. Practical Activity Design of canal by Lacey's Regime velocity | Theory-6 Hrs Practical-3 Hrs Total- 9 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room and workshop |
| - Draw the typical X-sections of channels | Trainee must be able to - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards. - Select suitable scale. - Draw sections of channel - Label all necessary details. | - Define Channel? - Explain types of Channel? Practical Activity Draw the Typical X-sections of Channel | Theory-3 Hrs Practical-3 Hrs Total- 6 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room and Work Shop |

- Produce sketches of Hydraulic structures

Objective: This module covers the knowledge and  skills required to draw sketches of various canal falls. Draw the plane and L-section of a masonry flume. Prepare drawings of an Aqueduct crossing, prepare drawings of a pipe outlet and prepare drawings of an A.P.M Outlet.

Duration: 36 Hours								Theory: 21 Hours				Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Draw sketches of various canal falls | Trainee will be able to: - Identify falls (Ogee, rapids, stepped, trapezoidal, sarda and glace) - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards - Draw sketches of falls | - Define canal falls. - What are the basic requirements of canal fall? - Describe the types of canal falls. - Explain the factors governing the site selection of canal falls. Practical Activity Draw sketches of various canal falls | Theory-4 Hrs Practical-3 Hrs Total- 7 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils | Class Room and workshop |
| - Draw the plane and L-section of a masonry flume | Trainee will be able to: - Identify flume structures - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards - Select scale as per requirement - Make drawing of masonry flume | - What is Flume? - Define Head Regulators. - Explain the types of Head Regulators, i.e. flume, meter flume. Practical Activity Draw the plane and L-section of a masonry flume | Theory-4 Hrs Practical-3 Hrs Total-7 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils | Class Room and workshop |
| - Prepare drawings of an Aqueduct crossing | Trainee must be able to: - Identify cross drainage works (aqueduct, super passage, level crossing, tail escape) - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards - Select scale as per requirement - Make drawing of aqueduct | - What is cross drainage works? - Describe with sketches the different types of cross drainage work. - Explain Aqueduct, super passage, level crossing, tail escape Practical Activity Prepare drawings of an Aqueduct crossing | Theory-5 Hrs Practical-3 Hrs Total- 8 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room and workshop |
| - Prepare drawings of a pipe outlet | Trainee must be able to - Identify outlets - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards - Select scale as per requirement - Make drawing of pipe outlet | - Define outlet. - State essential requirements of an outlet. - Explain the characteristics of outlets. Practical Activity Prepare drawings of a pipe outlet | Theory-4 Hrs Practical-3 Hrs Total- 7 Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils | Class Room and workshop |
| - Prepare drawings of an A.P.M Outlet | Trainee must be able to - Identify outlets - Arrange drawing instruments - Select drawing sheet - Set sheet on drawing table as per standards - Select scale as per requirement - Make drawing of A.P.M. outlet | - Describe with sketches the types of outlets (modular, semi modular and non-modular). - Design of outlet, modular, semi-modular and non-modular Practical Activity Prepare drawings of an A.P.M Outlet | Theory-4 Hrs Practical-3 Hrs Total- Hrs | Drawing board Drawing sheet Pair of Set squares T-Square Eraser Drawing Box (Compasses, divider, protractor, rule) HB, H, 2H, 3H Pencils Sharpener/ Sand paper Scotch tape | Class Room and Work Shop |

# 9.30. R.C.C. Design

- Design and Draw Reinforcement for Beams

Objective: This module covers the skills and knowledge required to design and draw Simple rectangular R.C.C beam with UDL, Design and Draw Shear force reinforcement for rectangular beam and check for bond, design cantilever beam, design and draw doubly reinforced beam, Design and draw simply supported Tee and Ell Beam,

, Duration:  24:00 Hours						Theory: 14:00 Hours				Practice: 12:00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Design a single reinforced rectangular beam and Draw reinforcement diagram | Trainee will be able to - Identify the type of beam. - Calculate the factored load. - Calculate shear force and bending moment. - Calculate maximum bending moment - Calculate the area of steel. - Draw the reinforcement detail. | - Basic Purpose and concept of Beams - Types of beams - Rectangular Beams - Types of loads - Factored loads - Bending moment and shear force diagrams - Use of Bending moment diagram. - Reinforcement Diagram Practical Activity - Design and drawing of reinforcement for simple rectangular R. C. C. beam of 10 ft span with U. D.L. | Theory-3 Hrs Practical-2 Hrs Total- 5 Hrs | Calculator Pen Drawing sheet Pencil Scotch tape Eraser Pencil Sharpener Note book | Class Room |
| - Design and draw reinforcement for doubly reinforced beam | Trainee will be able to - Calculate the factored loads. - Find out moments. - Calculate the area of steel in tension and compression - Select the size of bar. - Draw the reinforcement detail. | - Definition, necessity and use, limitation of doubly beam - Elements and assumption related to design, design steps. - Design of simply supported doubly reinforced beam Practical Activity - Design and drawing of reinforcement for doubly reinforced beam of 10ft span. | Theory-2.5 Hrs Practical-2 Hrs Total- 4.5 Hrs |  | Class Room |
| - Design and Draw Shear reinforcement for rectangular beam and check for bond. | Trainee will be able to - Identify the type of beam. - Calculate the live and dead loads. - Calculate shear force and bending moment - Find out the areas of steel by using formula. - Calculate the spacing - Draw the reinforcement detail. | - Shear stresses in beam - Types of shear reinforcements and stirrups - Design for horizontal shear, bent up bars, spacing of stirrups, zones of shear reinforcement in beams. - Bond between concrete and reinforcement, bond stresses, development length, checking bond stresses in beam Practical Activity - Solve problem on simply supported rectangular beam for different span. | Theory-2.5 Hrs Practical-2Hrs Total- 4.5 Hrs |  | Class Room |
| - Design and draw reinforcement for cantilever beam. | Trainee will be able to - Calculate the factored loads. - Find out moments. - Calculate the area of steel. - Select the size of bar. - Draw the reinforcement detail. | - Definition, necessity and, advantages of cantilever beam - S.F.D & B.M.D of cantilever beams under various loading conditions - Reinforcement details. Practical Activity - Solve problem on simply supported Cantilever beam having different span. | Theory-2 Hrs Practical-2 Hrs Total- 4 Hrs |  | Class Room |
| - Design and draw reinforcement for Tee beam | - Trainee will be able to - Calculate the factored loads. - Find out moments. - Calculate the area of steel. - Select the size of bar. - Draw the reinforcement detail. | - Definition, necessity and, advantages of Tee beam - S.F.D & B.M.D of Tee beams under various loading conditions - Reinforcement details. Practical Activity - Solve problem on simply supported Tee beam having different span. | Theory-2 Hrs Practical-2 Hrs Total- 4 Hrs |  | Class Room |
| - Design and draw reinforcement for Ell beam. | - Trainee will be able to - Calculate the factored loads. - Find out moments. - Calculate the area of steel. - Select the size of bar. - Draw the reinforcement detail. | - Definition, necessity and, advantages of Ell beam - S.F.D & B.M.D of Tee beams under various loading conditions - Reinforcement details. Practical Activity - Solve problem on simply supported Ell beam having different span. | Theory-2 Hrs Practical-2Hrs Total- 4 Hrs |  | Class Room |

- Design and Draw Reinforcement for Slabs

Objective: This module covers the skills and knowledge required to design and draw simply supported and overhanging one way slab, Design and draw reinforcement for 2 way slab.

, Duration: 40:00 Hours					Theory: 24:00 Hours				Practice: 16:00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Design and Draw reinforcement for simply supported and overhanging one-way slab. | Trainee will be able to - Identify the type of slab. - Calculate the factored load. - Select the preliminary depth of slab - Calculate bending moments and shear forces - Find out the areas of steel - Select diameter and spacing of reinforcing bars. - Draw the reinforcement detail. | - Basic purpose of slab. - Types of loads - Factored loads - Design methods - Identification of one way and two-way slab. - S.F.D & B.M.D of slab under various loading conditions - Reinforcement details. - Practical Activity - Solve problem of one-way slab of different span and size. | Theory-12Hrs Practical- 09 Hrs Total- 21 Hrs | Calculator Drawings sheet Pen, Pencil Eraser | Class Room |
| - Design and draw reinforcement for simply supported 2-way slab | Trainee will be able to - Identify the type of slab. - Select the preliminary depth of slab - Find out and check the effective and total depths of slabs - Find out areas of steel - Select diameter and spacing of reinforcing bars. - Draw the reinforcement detail. | - Identification of one way and two-way slab. - Design methods - Loads, bending moment coefficients and its use - S.F.D & B.M.D of slab under various loading conditions - Reinforcement detailing. Practical Activity - Solve problem of two-way slab of different span and size. | Theory-12 Hrs Practical-07Hrs Total- 19 Hrs | Calculator Drawings sheet Pen, Pencil Eraser | Class Room |

- Design and Draw Reinforcement for Columns

Objective: This module covers the skills and knowledge required to design and draw reinforcement for RCC column

, Duration: 30:00 Hours					Theory: 19:00 Hours				Practice: 11:00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Design and draw R.C.C. rectangular axially loaded short column as per ACI - code | Trainee will be able to - Identify type of column. - Calculate load on column. - Calculate size of column. - Calculate steel area. - Calculate size and spacing of rings. - Draw the reinforcement detail. | - Basic Purpose and concept of Columns - Types of Columns - Short and long Columns. - Code provisions - Types of loads - Factored loads. - Reinforcement Diagram Practical Activity - Solve numerical problem of short column in lab and draw reinforcement detail. | Theory- 05 Hrs Practical-03 Hrs Total- 08 Hrs | Calculator Drawings sheet Pen, Pencil Eraser | Class Room |
| - Design and draw R.C.C. rectangular axially loaded long column as per ACI - code. | Trainee will be able to - Identify type of column. - Calculate load on column. - Select load reduction factor - Calculate size of column. - Calculate steel area. - Calculate size and spacing of rings. - Draw the reinforcement detail. | - Basic Purpose and concept of long Columns - Types of loads - Factored loads. - Reinforcement Diagram Practical Activity - Solve numerical problem of long column in lab and draw reinforcement detail. | Theory-5 Hrs Practical-3 Hrs Total- 8 Hrs | Calculator Drawings sheet Pen, Pencil Eraser | Class Room |
| - Design and draw reinforcement for R.C.C. axially loaded circular column with lateral ties as per ACI-code | Trainee will be able to - Identify type of column. - Calculate load on column. - Calculate area of column - Calculate diameter of column. - Calculate steel area. - Calculate size and spacing of lateral ties. - Draw the reinforcement detail. | - Basic Purpose and concept of circular Columns - Use of circular Columns - Lateral Ties - Types of loads - Factored loads. - Reinforcement Diagram Practical Activity - Solve numerical problem of circular column with lateral ties in lab and draw reinforcement detail. | Theory- 05Hrs Practical- 03 Hrs Total- 08 Hrs | Calculator Drawings sheet Pen, Pencil Eraser | Class Room |
| - Design and draw reinforcement for R.C.C. axially loaded circular column with helical bindings as per ACI-code | Trainee will be able to - Identify type of column. - Calculate load on column. - Calculate area of column - Calculate diameter of column. - Calculate steel area. - Calculate size and spacing of lateral ties. - Draw the reinforcement detail. | - Basic Purpose and concept of circular Columns - Helical Binding. - Types of loads - Factored loads. - Reinforcement Diagram Practical Activity - Solve numerical problem of circular column with helical bindings in lab and draw reinforcement detail. | Theory- 04Hrs Practical- 02 Hrs Total- 06 Hrs | Calculator Drawings sheet Pen, Pencil Eraser | Class Room |

- Design and Draw Reinforcement for Footings

Objective: This module covers the skills and knowledge required to design and draw Footing reinforcement

, Duration: 18:00 Hours					Theory: 08:00 Hours				Practice:  10:00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Design and draw reinforcement for isolated footing with bearing capacity as per locality. | Trainee will be able to - Calculate load on column - Calculate the thickness of footing due to punching shear. - Calculate the thickness of footing due to bending moment. - Calculate effective depth - Calculate the steel area in both directions. - Calculate the spacing of steel. - Draw the reinforcement detail. | - Types of footings - Basic purpose regarding isolated footings - Concept of isolated footing and its uses. - Steel detailing Practical Activity - Solve numerical problem for design of isolated footing in lab and draw reinforcement detail. | Theory-4 Hrs Practical- 05 Hrs Total-15 Hrs | Calculator Drawings sheet | Class Room |
| - Design and draw reinforcement for combine footing with bearing capacity as per locality. | Trainee will be able to - Calculate load on column - Calculate the thickness of footing due to punching shear - Calculate the thickness of footing due to bending moment. - Calculate effective depth - Calculate the steel area in both directions. - Calculate the spacing steel. - Draw the reinforcement detail. | - Basic purpose of combine footings - Concept of combine footing and its uses. - Steel detailing Practical Activity - Solve numerical problem for design of combine footing in lab and draw reinforcement detail. | Theory-4 Hrs Practical- 05 Hrs Total- 15 Hrs | Calculator Drawings sheet | Class Room |

- Design and Draw Reinforcement for Stairs

Objective: This module covers the skills and knowledge required to design and draw reinforcement for the stairs.

Duration: 30:00 Hours						Theory: 15:00 Hours				Practice:  15:00 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Design and draw reinforcement for stair spanning vertically. | Trainee will be able to - Identify type of Stair - Calculate load on stair. - Calculate thickness of waist. - Calculate steel area - Calculate the spacing of bars. - Draw the reinforcement detail. | - Types of stairs and its common uses - Component of stairs - Relation of riser and tread - Concept of Spanning vertically Practical Activity - Solve numerical problem for design of stair case of house spanning vertically in lab and draw reinforcement detail. | Theory-10 Hrs Practical- 09 Hrs Total- 19 Hrs | Calculator Drawings sheet | Class Room |
| - Design and draw reinforcement for stair spanning horizontally | Trainee will be able to - Identify type of Stair - Calculate load on stair. - Calculate thickness of waist. - Calculate steel area - Calculate the spacing of bars. - Draw the reinforcement detail. | - Concept of Spanning vertically Practical Activity - Solve numerical problem for design of stair case of house spanning horizontally in lab and draw reinforcement detail. | Theory- 05 Hrs Practical- 06 Hrs Total- 11 Hrs | Calculator Drawings sheet | Class Room |

# 9.31. Civil Engineering Project

Note: All the 06 competencies of this cluster should be divided to 06 groups of the class. That is each group will perform one competency. The all 06 competencies should be compiled and provided to all students.

- Develop Project for construction of RCC framed Structure Class Room.

Objective: This module covers the knowledge and  skills required to prepare map of proposed site, prepare contract documents, prepare  Working Drawings, draft Specifications, Design of RCC slabs, lintels, T & L beams, columns and column footings, prepare estimates-Cost, Materials, Time(labor, machinery), layout Structures and conduct planning for RCC framed structure class room project.

Duration: 160 Hours							Theory:00 Hours				Practice: 160 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Conduct Location Survey and preparation of map of proposed site of framed Structure class room | Trainee will be able to: - Perform survey of area. - Perform contouring, x-sectioning & long sectioning. - Prepare the map. | - Describe procedure of carrying out site survey along with production of site map. (Already learnt in Basic Engineering Surveying) | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Survey Lab Equipment Calculator Pencil Ruler Drawing lab equipment AutoCAD/ Computer Lab Equipment | Class Room, Drawing and Survey workshop |
| - Design structural elements of framed Structure class | Trainee will be able to: - Design RCC slab and lintels. - Design RCC T & L beams. - Design RCC Columns & column footings. | - Design of RCC slabs, lintels, T & L beams, columns and column footings. (Already learnt) | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil | Class Room and Concrete Workshop |
| - Prepare Contract Documents for framed Structure class room. | Trainee must be able to: - Identify & get formed procurement team. - Download latest bidding documents from PEC web site. - Identify & Prepare tender document observing PEC directions- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and Drawings. | - Prepare tender documents- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and drawings. (Already learnt) | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil Computer Lab | Class Room and Computer Lab |
| - Prepare Working Drawings for framed Structure class room. | Trainee must be able to - Identify the requirements of client. - Observe the standards for size and orientation of building. - Prepare plans, elevation, sections, and schedules. | - Prepare Working Drawings, procedure of layout and planning of framed structure class room. (Already learnt) | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil AutoCAD/ Computer Lab | Class Room and workshop |
| - Document Specifications for framed Structure class components. | Trainee must be able to - Write down general specifications as per class. - Draft detailed & special specifications of building. | - Compressive study of framed structure as per building codes. (Already learnt) | Theory-00 Hrs Practice-12 Hrs Total- 12 Hrs | Calculator Pencil | Class Room and computer lab |
| - Prepare Estimates-Cost, Materials, Time(Labor, machinery) of framed Structure class. | Trainee must be able to - Prepare abstract of quantities, BoQ & cost as per CSR - Prepare materials statement for project. - Calculate the time for labor & machinery required to be employed. | - Calculate construction materials, time of machinery & labour to be employed. (Already learnt) | Theory-00 Hrs Practice-32 Hrs Total- 32 Hrs | Calculator Pencil | Class Room |
| - Layout Structures and conduct Planning of framed Structure class. | Trainee must be able to - Read layout drawing of project. - Use instrument for layout as per present circumstances. - Establish control points & set out framed structure class room elements. | - Describe the procedure of layout and planning of framed structure class room. (Already learnt) | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil Computer Lab | Class Room, drawing workshop and Survey workshop |

- Develop Project plan for construction of RCC OHR of 10000-gal (UK) capacity

Objective: This module covers the knowledge and  skills required to prepare map of proposed site, perform structural design, prepare contract documents, prepare Working Drawings, draft Specifications, Design of RCC slabs, T & L beams, columns and column footings prepare estimates- Cost, Materials, Time(Labor, machinery), layout Structures and conduct Planning for project for RCC OHR 10000 gal (UK) capacity.

Duration: 160 Hours							Theory: 00 Hours				Practice: 160 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Conduct Location Survey and preparation of map of proposed site of OHR. | Trainee will be able to: - Perform survey of area. - Perform contouring, x-sectioning & long sectioning. - Prepare the map. | - Describe procedure of carrying out site survey along with production of site map of OHR. | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Survey Lab Equipment Calculator Pencil Ruler Drawing lab equipment AutoCAD/ Computer Lab Equipment | Class Room, Drawing and Survey workshop |
| - Design structural elements of OHR. | Trainee will be able to: - Design RCC slabs - Design RCC T & L beams. - Design RCC Columns & column footings. | - Design of RCC slabs, T & L beams, columns and column footings for OHR. | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil | Class Room |
| - Prepare of Contract Documents for OHR. | Trainee must be able to: - Identify & get formed procurement team. - Download latest bidding documents from PEC web site. - Identify & Prepare tender document observing PEC directions- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and Drawings. | - Prepare tender documents for OHR- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and drawings. | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil Computer Lab | Class Room and Computer Lab |
| - Prepare Working Drawings for OHR. | Trainee must be able to - Identify the requirements of area. - Prepare plans, elevation, sections, and schedules. | - Prepare Working Drawings,procedure of layout and planning of OHR. | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil Ruler Drawing lab equipment | Class Room and drawing workshop |
| - Document Specifications for OHR components. | Trainee must be able to - Write down general specifications as per design. - Draft detailed & special specifications of building. | - Compressive study OHR Component as per codes. | Theory-00 Hrs Practice-12 Hrs Total- 12 Hrs | Calculator Pencil | Class Room |
| - Prepare Estimates-Cost, Materials, Time(Labor, machinery) for OHR. | Trainee must be able to - Prepare abstract of quantities, BoQ & cost as per CSR. - Prepare materials statement for OHR - Calculate the time for labor & machinery required to be employed. | - Calculate construction materials, time of machinery & labour to be employed for OHR. | Theory-00 Hrs Practice-32 Hrs Total- 32 Hrs | Calculator Pencil | Class Room |
| - Layout Structures and conduct planning for OHR. | Trainee must be able to - Read layout drawing of OHR - Use instruments for layout as per present circumstances. - Establish control points & set out OHR elements | - Describe the procedure of layout and planning of OHR. | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil Computer Lab | Class Room, drawing workshop and Survey workshop |

- Develop Project plan for construction of Water Supply Scheme

Objective: This module covers the knowledge and  skills required to to prepare map of proposed site, design water supply scheme-mark water supply lines & prepare hydraulic statement, prepare contract documents, prepare Working Drawings, draft Specifications, prepare estimates-Cost, Materials, Time(Labor, machinery), layout Structures and conduct Planning for project for construction of water supply scheme for a small town(200 housesin25acres)forpopulationof1000persons.

Duration: 160 Hours							Theory: 00 Hours				Practice: 160 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Conduct Location Survey and preparation of map of proposed site of water supply scheme. | Trainee will be able to: - Perform survey of area. - Perform long sectioning. - Prepare the map. | - Describe procedure of carrying out site survey along with production of site map.(Already learnt in Basic Engineering Surveying) | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Survey Lab Equipment Calculator Pencil Ruler Drawing lab equipment AutoCAD/ Computer Lab Equipment | Class Room and workshop |
| - Design structural elements of water supply scheme. | Trainee will be able to: - Mark the location of water supply lines on map. - Interpret head losses in pipes. - Prepare hydraulic statement of water supply scheme. | - Design water Supply scheme- mark pipe lines & prepare hydraulic statement. | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil | Class Room |
| - Prepare of Contract Documents for water supply scheme. | Trainee must be able to: - Identify & get formed procurement team. - Download latest bidding documents from PEC web site. - Identify & Prepare tender document observing PEC directions- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and Drawings. | - Describe contracting procedure water supply scheme. - Prepare tender documents- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and drawing - Define tender, contract, tender notice and tender documents. - Describe contracting procedure water supply scheme. | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil Computer Lab | Class Room and computer Lab |
| - Prepare Working Drawings for water supply scheme. | Trainee must be able to - Identify the requirements of area. - Prepare plans, elevation, sections, and schedules. | - Preparation of working Drawings for water supply scheme as per standard | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil AutoCAD/ Computer Lab | Class Room and drawing workshop |
| - Document Specifications for water supply scheme components. | Trainee must be able to - Write down general specifications as per month. - Draft detailed & special specifications of water supply scheme. | - Preparation of detailed & special specifications of water supply scheme as per international standard. | Theory-00 Hrs Practice-12 Hrs Total- 12 Hrs | Calculator Pencil | Class Room |
| - Prepare Estimates-Cost, Materials, Time (Labor, machinery) for water supply scheme. | Trainee must be able to - Prepare abstract of quantities, BoQ & cost as per CSR. - Prepare materials statement for water supply scheme. - Calculate the time for labour & machinery required to be employed. | - Calculate construction materials, time of machinery & labour to be employed. | Theory-00 Hrs Practice-32 Hrs Total- 32 Hrs | Calculator Pencil | Class Room |
| - Layout Structures and conduct planning for water supply scheme. | Trainee must be able to - Read layout drawing of water supply scheme. - Use instrument for layout as per present circumstances. - Establish control points & set out water supply scheme elements. | - Describe the procedure of layout and planning of water supply scheme. | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil Computer Lab | Class Room, drawing workshop and Survey workshop |

- Develop Project plan for construction of sewerage scheme

Objective: This module covers the knowledge and  skills required to prepare map of proposed site, mark sewerage line & prepare hydraulic statement, design septic tank & soakage pit, prepare contract documents, prepare Working Drawings, draft Specifications, prepare estimates-Cost, Materials, Time(Labor, machinery), layout Structures and conduct Planning for a colony(200housesin 25acres)forpopulationof1000persons.

Duration: 160 Hours							Theory:00 Hours				Practice: 160 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Conduct Location Survey and preparation of map of proposed site of sewerage scheme. | Trainee will be able to: - Perform survey of area. - Perform contouring, x-sectioning & long sectioning. - Prepare the map. | - Describe procedure of carrying out site survey along with production of site map. (Already learnt in Basic Engineering Surveying) | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Survey Lab Equipment Calculator Pencil Ruler Drawing lab equipment AutoCAD/ Computer Lab Equipment | Class Room and workshop |
| - Design structural elements of sewerage scheme. | Trainee will be able to: - Mark the location of sewerage scheme lines on map. - Interpret velocity standards for type of pipe. - Prepare hydraulic statement of sewerage scheme. - Design Septic Tank for scheme. - Design soakage pit for scheme. | - Design of Septic tank, soakage and prepare hydraulic statement | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil | Class Room and workshop |
| - Prepare of Contract Documents for sewerage scheme. | Trainee must be able to: - Identify & get formed procurement team. - Download latest bidding documents from PEC web site. - Identify & Prepare tender document observing PEC directions- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and Drawings. | - Define tender, contract, tender notice and tender documents sewerage scheme. - Describe contracting procedure sewerage scheme. - Prepare tender documents- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and drawings. | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil Computer Lab | Class Room and Computer Lab |
| - Prepare Working Drawings for sewerage scheme. | Trainee must be able to - Identify the requirements as per design. - Prepare plans, elevation, sections, and schedules. | - Prepare Working Drawings for sewerage scheme. including plans, elevation, sections, and schedules | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil Ruler Drawing lab equipment | Class Room and drawing workshop |
| - Document Specifications for sewerage scheme components | Trainee must be able to - Write down general specifications as per design. - Draft detailed & special specifications of design of scheme. | - Preparation of detailed & special specifications of sewerage design as per standard. | Theory-00 Hrs Practice-12 Hrs Total- 12 Hrs | Calculator Pencil | Class Room |
| - Prepare Estimates-Cost, Materials, Time(Labor, machinery) for sewerage scheme. | Trainee must be able to - Prepare abstract of quantities, BoQ & cost as per CSR. - Prepare materials statement for project. - Calculate the time for labor & machinery required to be employed. | - Calculate construction materials, time of machinery & labour to be employed. | Theory-00 Hrs Practice-32 Hrs Total- 32 Hrs | Calculator Pencil | Class Room |
| - Layout Structures and conduct planning for sewerage scheme. | Trainee must be able to - Read layout drawing of sewerage scheme. - Use instrument for layout as per present circumstances. - Establish control points & set out sewerage scheme elements. | - Describe the procedure of layout and planning of sewerage scheme. | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil Computer Lab | Class Room, drawing workshop and Survey workshop |

- Develop Project plan for construction of Highway

Objective: This module covers the knowledge and  skills required to prepare topographic map of proposed site, Design the geometric & structural components, prepare contract documents, prepare Working Drawings, draft Specifications, prepare estimates-Cost, Materials, Time(Labor, machinery), layout structures-alignment and curves, conduct planningfor2km long road

Duration: 160 Hours							Theory: 00 Hours				Practice: 160 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Conduct Location Survey and preparation of map of proposed site of 2km long road. | Trainee will be able to: - Perform survey along route of road. - Perform contouring, x-sectioning & long sectioning. - Prepare the map. | - Describe procedure of carrying out site survey along with production of site map. (Already learnt in Basic Engineering Surveying) | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Survey Lab Equipment Calculator Pencil Ruler Drawing lab equipment AutoCAD/ Computer Lab Equipment | Class Room and workshop |
| - Design structural elements of 2km long road. | Trainee will be able to: - Design geometric components of road. - Perform tests for structural design of road components. - Design sub-base, base& surfacing of road | - Design of geometric & structural components of road. | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil | Class Room and workshop |
| - Prepare of Contract Documents for 2km long road. | Trainee must be able to: - Identify & get formed procurement team. - Download latest bidding documents from PEC web site. - Identify & Prepare tender document observing PEC directions- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and Drawings. | - Define tender, contract, tender notice and tender documents. - Describe contracting procedure. - Prepare tender documents for road- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and drawings. | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil Computer Lab | Class Room and Computer Lab |
| - PrepareWorkingDrawingsfor2km long road. | Trainee must be able to - Identify the requirements of traffic. - Observe the standards for lane and land width. - Prepare plans, long & cross sections, schedules. | - Prepare Working Drawings for2km long road including plans, long & cross sections, and schedules as per standard. | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil Ruler Drawing lab equipment | Class Room and drawing workshop |
| - DocumentSpecificationsfor2km long road components. | Trainee must be able to - Write down general specifications as per class. - Draft detailed & special specifications of building. | - Preparation of detailed & special specifications for2km long road components as per standard. | Theory-00 Hrs Practice-12 Hrs Total- 12 Hrs | Calculator Pencil | Class Room |
| - Prepare Estimates-Cost, Materials, Time (Labor, machinery) for 2km long road. | Trainee must be able to - Prepare abstract of quantities, BoQ & cost as per CSR. - Prepare materials statement for road - Calculate the time for labor & machinery required to be employed. | - Calculate construction materials, time of machinery & labour to be employed. | Theory-00 Hrs Practice-32 Hrs Total- 32 Hrs | Calculator Pencil | Class Room |
| - Layout Structures and conduct planningfor2km long road. | Trainee must be able to - Read layout drawing of road. - Use instrument for layout as per present circumstances. - Establish control points & set out road elements | - Describe the procedure of layout and planning of 2 km long road. | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil Computer Lab | Class Room, drawing workshop and Survey workshop |

- Develop Project plan for construction of Irrigation Canal

Objective: This module covers the knowledge and  skills required to prepare map of proposed route, design the canal, prepare contract documents, and prepare Working Drawings, draft Specifications, footings prepare estimates-Cost, Materials, Time(Labor, machinery), layout canal and conduct planning for irrigation canal 2 RD long

Duration: 160 Hours							Theory: 00 Hours				Practice: 160 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Conduct Location Survey and preparation of map of proposed site of 2 RD long canal. | Trainee will be able to: - Perform survey for 2 RD long canal. - Perform contouring, x-sectioning & long sectioning. - Prepare map 2 RD long canal. | - Describe procedure of carrying out site survey along with production of site map. (Already learnt in Basic Engineering Surveying) | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Survey Lab Equipment Calculator, Pencil, Ruler Drawing lab equipment AutoCAD/ Computer Lab Equipment | Class Room and workshop |
| - Design structural elements of 2 RD long canal | Trainee will be able to: - Design RCC slabs - Design RCC T & L beams. - Design RCC Columns & column footings. | - Design of irrigation channel. | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil | Class Room and workshop |
| - PrepareofContractDocumentsfor2 RD long canal | Trainee must be able to: - Identify & get formed procurement team. - Download latest bidding documents from PEC web site. - Identify & Prepare tender document observing PEC directions- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and Drawings. | - Define tender, contract, tender notice and tender documents 2 RD long canal. - Describe contracting procedure 2 RD long canal. - Prepare tender documents- Instructions to Bidders, Bidding Data, General Conditions of Contract Part-I (GCC), Particular Conditions of Contract Part-II (PCC), Specifications - Special Provisions, Specifications - Technical Provisions, Form of Bid & Appendices to Bid, Bill of Quantities, Form of Bid Security, Form of Agreement, Form of Performance Security/Bond & Mobilization Advance Guarantee/Bond, and drawings. | Theory-00 Hrs Practice-36 Hrs Total- 36 Hrs | Calculator Pencil Computer Lab | Class Room and Computer Lab |
| - PrepareWorkingDrawingsfor2 RD long canal. | Trainee must be able to - Identify the requirements of client. - Observe the standards for size and orientation of building. - Prepare plans, elevation, sections, and schedules. | - Prepare Working Drawings for2 RD long canal. Including plans, elevation, sections, and schedules as per standard. | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil Ruler Drawing lab equipment | Class Room and drawing workshop |
| - DocumentSpecificationsfor2 RD long canal. | Trainee must be able to - Write down general specifications as per class. - Draft detailed & special specifications of building. | - Preparation of detailed & special specifications for2 RD long canal. as per standard. | Theory-00 Hrs Practice-12 Hrs Total- 12 Hrs | Calculator Pencil | Class Room |
| - Prepare Estimates-Cost, Materials, Time(Labor, machinery) for 2 RD long canal. | Trainee must be able to - Prepare abstract of quantities, BoQ & cost as per CSR. - Prepare materials statement for project. - Calculate the time for labor & machinery required to be employed. | - Calculate construction materials, time of machinery & labour for 2 RD long canal. | Theory-00 Hrs Practice-32 Hrs Total- 32 Hrs | Calculator Pencil | Class Room |
| - Layout Structures and conduct Planningfor2 RD long canal. | Trainee must be able to - Read layout drawing of project. - Use instrument for layout as per present circumstances. - Establish control points & set out the project elements. | - Describe the procedure of layout and planning of 2 RD long canal. | Theory-00 Hrs Practice-18 Hrs Total- 18 Hrs | Calculator Pencil Computer Lab | Class Room, drawing workshop and Survey workshop |

# 9.32. Entrepreneurial Skills.

- Investigate micro business opportunities

Objective: This module describes the performance outcomes, skills and knowledge required to develop business ideas, and to investigate market needs and factors affecting potential markets.

Duration: 09 Hours								Theory: 03 Hours				Practice: 06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Describe business ideas | Trainee will be able to: - Gather information for business ideas from appropriate sources - List details of business ideas and opportunities - Research alternative business ideas in light of the resources available - Specify and list products and services to match business ideas - Identify and research potential customer information for business ideas - Identify and take into account financial, business and technical skills available when researching business opportunities | - Define entrepreneurship. - Explain the concept of entrepreneurship - Explain the various types of enterprise that exist in the community - Identify and interpret the terms and elements involved in the concept of enterprise - Appreciate that the advancement of individual and society in general when entrepreneurship is adopted - Explain various motivational factors that entrepreneurs possess and utilize. - Exhibit the skills needed to assess and evaluate a risk Practical Activity - Prepare a list of four rewards and four risks of being an entrepreneur - Prepare a list the types of business found in your community - Prepare a list of each type of business - Designing - Manufacturing - Supply and distribution - Selling - Retailing - Repairing - Service industry | Theory-02 Hrs Practical-01 Hrs Total- 03 Hrs | Calculator Ruler Pencil Drawing lab equipment | Class Room |
| - Identify market needs | Trainee will be able to: - Collect information regarding market size and potential from appropriate sources - Investigate market trends and developments to identify market needs relative to business ideas - Gather market information from primary and secondary sources to identify possible market needs in relation to business ideas - Identify ethical and cultural requirements of the market and their impact on business ideas - Identify new and emerging markets and document their features - Identify and organize information on expected market growth or decline and associated risk factors | - Describe the outline of small enterprise - Describe the creativity and innovation - Apply the techniques for developing creative abilities - Explain the resources of business idea - Explain the collective and creative thinking - Explain how to generate a business idea - Appreciate the importance of, and possess techniques for identifying and assessing business opportunities. Practical Activity To succeed as a new small business owner, you do not necessarily need to invent something new or create something unique. You can find an unfilled service or commodity in your community and look for a business that fills this vacancy. In this activity you will answer the following questions about your business idea: - Draft the business idea you want to start. - List other businesses in your community who provide similar services or products: - Select the location where you will have the business. Will it be in a separate location or will it be in your home? | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | Calculator Ruler Pencil Drawing lab equipment | Class Room |
| - Investigate factors affecting the market | Trainee must be able to: - Identify projected changes in population, economic activity and the labour force that may affect business ideas - Identify movements in prices and projected changes in availability of resources - Review trends and developments and identify their potential impact on business ideas | - Identify the various entrepreneurial characteristics - Access personal potential for becoming future entrepreneurs. - Identify leadership qualities which are essential to the success of entrepreneurs - Identify self- management skills and how they are important to be enterprising - Apply a rational approach to make personal and business decisions - Explain the steps for decision making and rating of decision making skills - Apply the rules of negotiation for resolving business issues Practical Activity Using your business idea, answer the following questions: - Prepare a list of interested (needs) customers of your product or service. Incorporate the answers the following questions: - Why do they like or dislike my product or service? Are they willing to buy it (give money or another product for it)? - Will they always want to buy the product regularly or seasonally (sustainable)? Or will they stop buying it completely (unsustainable)? - How many of my product or hours of my service will they need? - Where should the product be available (location of sales points)? | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | Calculator Ruler Pencil Drawing lab equipment | Class Room |

- Develop a micro business proposal

Objective: This module describes the performance outcomes, skills and knowledge required to develop an identified business idea, to research the feasibility of the business opportunity and to present a business idea in formats that suit a range of stakeholders

Duration: 12 Hours								Theory: 04 Hours				Practice: 08 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Evaluate business opportunities | Trainee will be able to: - Identify and research key factors that influence viability of business ideas - Analyze business ideas in terms of personal or family needs and commitments - Evaluate impacts of emerging or changing technology, including e-commerce, on the business - Determine viability of business opportunity in line with perceived risks, resources available, financial returns and other outcomes sought Assess and match personal skills/attributes against those perceived as necessary for a particular business opportunity - Identify and assess business risks according to resources available and personal preferences | - factors that influence viability of business ideas - Analysis business ideas - Impacts of emerging or changing technology, including e-commerce, on the business - Assessment of business risks Practical Activity - Prepare a list of facilities and incentives that are available to you for starting a new business in your community? | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | Calculator Ruler Pencil | Class Room |
| - Detail the business idea | Trainee will be able to: Develop an accurate description of the business idea for key stakeholders Develop an accurate summary of the major products and/or services required to suit personal needs and requirements | - Difference between a business idea and a business opportunity? - Creative-thinking techniques used to generate ideas. - Name four common ways of turning business ideas into opportunities. - Practical methods for determining the feasibility of a business idea. Practical Activity Imagine that you are planning to survey students at your school about a potential bookstore to be located on school grounds. The proposed store will be run by students with teachers’ guidance. Profits will go to a charitable group chosen by the students. Working in a small group, develop a questionnaire for the survey with no more than 20 questions. Make sure the questions focus on the most important issues involved in this startup. | Theory-01 Hrs Practical-02 Hrs Total- 03 Hrs | Calculator Ruler Pencil | Class Room |
| - Prepare the business overview to suit different stakeholders | Trainee must be able to: - Present an accurate list of key stakeholders and their information requirements - Determine an acceptable method of presentation of information for each stakeholder - Provide accurate customized information to target audiences | - Difference between limited and unlimited liability. - Sole proprietorship different from general partnership. - Partnership agreement. - Advantages and disadvantages of a corporation over a sole proprietorship. Practical Activity - Working as a team, write a partnership agreement as if you were going into business together. Assume that some partners can work 40 hours per week and others can only work 20 hours. Also assume that some partners can contribute Rs. 5, 000 each to setting up the business and others can contribute Rs. 1,000. Be certain to specify in the agreement how the profits will be divided among the partners. | Theory-02 Hrs Practical-02 Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |

- Develop a Business Plan & Marketing Plan.

Objective: This module describes the performance outcomes, skills and knowledge required to research, develop and present a marketing plan for an entrepreneurship business

Duration: 22 Hours								Theory: 07 Hours				Practice: 15Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Devise marketing strategies | Trainee will be able to: - Evaluate marketing opportunity options that address organizational objectives, and evaluate their risks and returns in the selection process - Develop marketing strategies that address strengths and opportunities within the organization’s projected capabilities and resources - Develop strategies which increase resources or organizational expertise where gaps exist between current capability and marketing objectives - Develop feasible marketing strategies and communicate reasons that justifies their selection - Ensure strategies align with organization’s strategic direction - Develop a marketing performance review strategy, incorporating appropriate marketing metrics to review of organizational performance against marketing objectives. | - Describe the market & marketing - Differentiate between sellers and buyers’ market - Describe the five ‘w’ of market - Explain the procedure for assessing the market size and demand - Explain the major factors to be considered when selecting a location for a business Practical Activity | Theory-02 Hrs Practical-02 Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| - Plan marketing tactics | Trainee will be able to: - Detail tactics to implement each marketing strategy in terms of scheduling, costing, accountabilities and persons responsible - Identify coordination and monitoring mechanisms for scheduled activities - Ensure tactics are achievable within organization’s projected capabilities and budget - Ensure tactics meeting legal and ethical requirements - Ensure tactics provide for ongoing review of performance against objectives and budgets, and allow marketing targets to be adjusted if necessary | - Main elements in a marketing mix. - Define branding and explain its potential advantages. - Describe the three basic distribution strategies. - Basic strategies used to determine prices. Practical Activity - Work with two partners. Each will focus on a distribution strategy (exclusive, intensive, or selective). Research the advantages and disadvantages of your strategy. When finished, assemble your research results into a PowerPoint presentation or other method of displaying information. | Theory-01 Hrs Practical-03 Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| - Prepare and present a marketing plan | Trainee must be able to: - Ensure marketing plan meets organizational, as well as marketing, objectives and incorporates marketing approaches and a strategic marketing mix - Ensure marketing plan contains a rationale for objectives and information that supports the choice of strategies and tactics - Present marketing plan for approval in the required format and timeframe - Adjust marketing plan in response to feedback from key stakeholders and disseminate for implementation within the required timeframe | Practical Activity: Prepare a marketing plan of your proposed idea | Theory-00 Hrs Practical-03 Hrs Total- 03 Hrs | Calculator Ruler Pencil | Class Room |
| - Determine scope of business | - Determine scope of the business plan and associated systems is determined in consultation with specialist personnel. - Access accurate information for inform business plan development - Account for and incorporate trends and seasonal variations into the business plan. - Account for strategic goals, targets and directions of the enterprise in the development of the business plan - Comply Legal obligations in developing the business plan. | - What is a business plan? - Main types of business plans. - Parts of a business plan - Questions do you need to consider early in the process of developing a business plan. Practical Activity Working in groups of four, use the Internet to research three business plan models. Record the parts of each plan and the order in which the parts are presented. What are the differences between the plan in the text and the plans you researched? What are the similarities? | Theory-02 Hrs Practical-01 Hrs Total- 03 Hrs | Calculator Ruler Pencil | Class Room |
| - Prepare business plan | Trainee will be able to: - Develop operational goals and targets to meet the enterprise strategic plan. - Identify and incorporate supply chains into the business plan. - Identify risk management needs are within the business plan. - Incorporate trial systems in order to test budgetary impact and operational potential prior to full implementation of the business plan. - Set clear and measureable indicators of operational performance to allow for realistic analysis of performance. | Practical Activity - Prepare a business plan of your proposed and finalized idea incorporating the following information’s: - title page (name of your business, your name and contact address) - summary of business objectives - table of contents - structure of your proposed business - the product or service I intend to sell - marketing plan (based on a simple market survey) - financial plan - what you expect your business to earn other supporting documents, such as letters of recommendation, certificates of education and training, community service documents, etc. | Theory-00 Hrs Practical-03 Hrs Total- 03 Hrs | Calculator Ruler Pencil | Class Room |
| - Document and review business plan | Trainee must be able to: - Include fiscal and operational systems that enhance performance management and suit enterprise requirements. - Incorporate resource considerations the business plan. - Document accurately and clearly communicate business Plan to all relevant parties. - Monitor to identify strengths, weaknesses and areas for improvement performance against the business plan Make recommendations to improve the business plan and associated systems as required. | - Business plan drivers - Things to remember while preparing a business plan - Perspectives to be considered in business plan preparation - Who should prepare the business plan Practical Activity - Prepare power point of your business plan and discuss in the class | Theory-02 Hrs Practical-03 Hrs Total- 05 Hrs | Calculator Ruler Pencil | Class Room |

- Organize finances for the micro business.

Objective: This module describes the performance outcomes, skills and knowledge required to investigate the financial capacity to enter into a micro business, to determine the projected cash flow, to source finances and to monitor the profitability of the business.

Duration: 12 Hours								Theory: 04 Hours				Practice: 08 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU 1. Ascertain own financial position and the ability to provide capital/equity for the business | Trainee will be able to: - Realistically detail personal, family or community financial situation in terms of funds available and commitments already incurred - Determine equity finance and assets available for micro business from personal, family or community sources | - Components of a start-up investment Practical Activity - Form two teams. The first will plan for a new business that sells bakery item like donuts (made at another location and purchased wholesale). The second team will plan for a new business that sells umbrellas (made at another location and purchased wholesale). Each team will determine the business’s required start-up investment. Research costs on the Internet. Each team will present its analysis to the class. | Theory-01 Hrs Practical-02 Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| LU.2 Determine projected cash flow for the business | Trainee will be able to: - Determine the level of forecast business activity over a year and the business mix - Estimate establishment costs for the business and repayment schedule for borrowings - Calculate the monthly variable and fixed costs needed to conduct business activity over a year - Estimate personal drawings needed to be taken from the business - Estimate the monthly income generated by the business for a year based on price per unit item or hourly charge rate for labor - Develop a cash flow budget for the first year of business operation - Seek professional advice to estimate goods and services tax and operating finance required for the business | - Define debt financing? - Define equity financing? - Four specialized sources of financing that may provide either debt or equity funding. - Effects of debt financing and equity financing on the balance sheet. - Define and explain cash flow statement. Practical Activity - Prepare a proposed balance sheet and Income statement of your proposed business | Theory-02 Hrs Practical-02 Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| LU.3 Source the required funds to establish the business | Trainee must be able to: - Estimate required funding to establish and run the business based on expected sales and activity levels, available finances and commitments - Investigate methods of accessing alternative sources of finance - Identify strategies for meeting financial obligations | Practical Activity - Form groups of three. Each group will investigate a different investment companies and Banks. Each will present a short presentation describing the selected company. Make sure to indicate the types of businesses in which the company invests and whether most of the investments are debt or equity financing. | Theory-02 Hrs Practical-02 Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| LU.4 Monitor profitability of the business | Trainee must be able to - Maintain and review monthly expenditure and income records - Compare equity at beginning and end of a year to estimate business performance - Assess the financial viability of the business after a year of operation - Seek professional advice on depreciation, insurance and tax implications of the business | Practical Activity | Theory-02 Hrs Practical-02 Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |

- Manage human resources.

Objective: This module covers the skills and knowledge required to manage human resources and to manage and develop human resources to achieve organization’s operational objectives

Duration: 09 Hours								Theory: 03 Hours				Practice: 06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Lead and motivate people | Trainee will be able to: - Establish goals for people and teams to optimize achievement in work tasks. - Take into account the capabilities of people and teams. - Provide advice and support sensitive to the individual's needs to people in the performance of their duties. - Undertake activities to achieve commitment to common goals. - Recognize and encourage initiative and innovation - P6. Recognize and communicate achievements within the organization. | Practical Activity | Theory-02 Hrs Practical-02Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| - Undertake human resource planning | Trainee will be able to: - Determine human resource needs within the anticipated operational needs and allocated budget. - Analyze alternatives to staffing levels which clearly demonstrate returns to the organization. - Develop contingency plans for staffing which meet key provisions of the human resources plan. - Compare existing competencies of staff with the needs of the work group. - Plan staffing levels and negotiate with stakeholders within the organizational framework to achieve maximum efficiency of operations. | Practical Activity | Theory-02 Hrs Practical-02Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| - Prepare record for transfer of store. | Trainee must be able to: - Negotiate performance criteria individuals, teams and work groups. - Review performance criteria as circumstances change. - Conduct performance appraisal based on clearly established and agreed performance criteria. - Identify and propose the total performance development system strategies to rectify performance shortfalls and recognize success. - Address performance problems confidentially and in a constructive and timely manner, in line with relevant organizational procedures. - Make selections, transfers and promotions in accordance with organization policies and supported with documented information. - Develop and implement mechanisms for the identification of human resource development needs within the work group taking account of the strategic plan for the organization. | Practical Activity | Theory-02 Hrs Practical-02Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| - Prepare receipt record of tools & plants- supplies | Trainee must be able to - Make information on planned training events widely available throughout the organization. - Include training, education and development plans as part of individual/team performance plans. - Facilitate individual/team access to, and participation in, training, education and development opportunities. - Contribute coaching and mentoring effectively to the training, education and development of personnel in an environment of change. - Enhance training, education and development opportunities of individual, team and organizational performance. - Create workplace environment is which facilitates training, education and development | Practical Activity | Theory-02 Hrs Practical-02 Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |

- Market products and services.

Objective: This module covers the knowledge and  skills required to prepare record of store (stock) debited to suspense account, prepare record of store directly charged to work, prepare record for transfer of store & prepare receipt record of tools & plants- supplies.

Duration: 09urs								Theory: 03 Hours				Practice: 06 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Analyze market information | Trainee will be able to: - Identify, research and analyze existing or new markets for existing or new products or services using techniques to ensure reliable data - Analyze past trends and developments to determine market variability and associated risks - Develop gross margin budgets to account for market variability - Identify and evaluate competing products to determine strengths and weaknesses of own products - Monitor market environment to ensure information is current and reliable - Identify the legal, ethical and environmental constraints of the markets and their effect on the enterprise - Identify product specifications that suit market requirements and price advantage at the time - Present clear and concise information to the enterprise management team. | Practical Activity | Theory-02 Hrs Practical-02Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| - Prepare record of store directly charged to work. | Trainee will be able to: - Identify stores debited to debit to final head of account-Tools & plants, road metals, materials charged to work. - Identify stock articles requiring no prior sanction- materials charged to work. - Identify stock articles requiring prior sanction- tools & plants. - Record sales Account on Form DFR DW-10 | Practical Activity | Theory-02 Hrs Practical-02Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| - Prepare record for transfer of store. | Trainee must be able to: - Get issued indent book (Form PFR-26). - Record items required for consumption on indent book. - Get approval of authority. - Receive store and record on consumable Stock registers. - Receive tools & plants and record on durable goods Stock register. | Practical Activity | Theory-02 Hrs Practical-02Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |
| - Prepare receipt record of tools & plants- supplies | Trainee must be able to - Record delivery of items-Inward gate pass, Goods Delivery challan - Get inspected store items-Technical Evaluation committee, Goods Inspection Report - Record accepted store items in register-Goods receiving Note, Stock Register. - Get prepared Stock Entry Certificate for store items | Practical Activity | Theory-02 Hrs Practical-02Hrs Total- 04 Hrs | Calculator Ruler Pencil | Class Room |

- Monitor and review business performance.

Objective: This module covers the knowledge and  skills required to prepare record of store (stock) debited to suspense account, prepare record of store directly charged to work, prepare record for transfer of store & prepare receipt record of tools & plants- supplies.

Duration: 12urs								Theory: 04 Hours				Practice: 08 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Evaluate commercial performance | Trainee will be able to: - Gather and analyze data relating to enterprise performance to identify historical and current performance. - Review and analyze operational structures to determine the suitability of organizational processes to enterprise objectives. - Evaluate enterprise strengths and weaknesses against market conditions to determine current and future capacities. - Evaluate enterprise objectives are to identify variations and scope for future development. | Practical Activity | Theory-01 Hrs Practical-02Hrs Total- 03 Hrs | Calculator Ruler Pencil | Class Room |
| - Allocate and co-ordinate business resources | Trainee will be able to: - Identify and communicate roles and responsibilities of personnel. - Identify resource requirements for enterprise and cost them using standard financial analysis techniques. - Calculate costs of ensuring sustainability of enterprise operations and factor into business planning for the enterprise. | Practical Activity | Theory-01 Hrs Practical-02Hrs Total- 03 Hrs | Calculator Ruler Pencil | Class Room |
| - Identify performance requirements | Trainee must be able to: - Develop realistic performance indicators within available timeframes and resources - Identify and minimize factors inhibiting performance against objectives. - Monitor and assess market conditions based on relevant data. - Prepare and incorporate strategies and programs to promote the sustainability of operations into enterprise procedures. | Practical Activity | Theory-01 Hrs Practical-02Hrs Total- 03Hrs | Calculator Ruler Pencil | Class Room |
| - Review business performance | Trainee must be able to - Review regularly enterprise operations to identify opportunities for improvements in performance. - Monitor and anticipate impact of natural conditions on enterprise to assess sustainability of resource use. - Compare costs and estimates with resource allocation. - Determine operational plans to determine schedule of activities | Practical Activity | Theory-01Hrs Practical-02Hrs Total- 03 Hrs | Calculator Ruler Pencil | Class Room |

- Negotiate for resolving business issues

Objective: This module covers the knowledge and  skills required to prepare record of store (stock) debited to suspense account, prepare record of store directly charged to work, prepare record for transfer of store & prepare receipt record of tools & plants- supplies.

Duration: 7urs								Theory: 03 Hours				Practice: 04 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Develop and implement issue resolution and grievance procedures | Trainee will be able to: - Establish problem solving/issue resolution procedures within legislative requirements and organization’s guidelines. - Manage grievances and complaints are in a timely and caring way to optimize likelihood of a favorable outcome for all parties and in line with organizational objectives and procedures. - Document and communicate individual's rights and obligations under industrial awards/agreements and legislation are in a clear and concise manner and in appropriate language. - Conduct hearings, interviews and meetings within the principles of industrial democracy and participative, consultative processes. - Identify and invite all relevant industrial parties to participate in the resolution process. | - Importance of negotiation in business - Ways to prepare for negotiation - Techniques for conducting negotiation - Benefits of negotiated agreements Practical Activity | Theory-02 Hrs Practical-04Hrs Total- 06 Hrs | Calculator Ruler Pencil | Class Room |
| - Manage disciplinary matters | Trainee will be able to: - Implement disciplinary matters in accordance with organization’s procedures. - Conduct investigations in a caring and confidential manner to maintain performance and morale. - Provide feedback promptly. - Institute appeals processes in accordance with organizational procedures. - Recognize and recommend deficiencies in procedures for changes made | Practical Activity | Theory-02 Hrs Practical-04Hrs Total- 06 Hrs | Calculator Ruler Pencil | Class Room |

0732B&CE306 Perform Basic Green Skills in Construction

Objective: After the completion of this module, the Trainee will be able to develop skill and competence required to use green skills in construction..

Duration: 30 Hours 								Theory: 06 Hours				Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Apply green skills in construction sector | Trainee will be able to: - Identify building as per zone plan - Identify climate condition of the construction site - Avoid environmental degradation - Select eco-friendly material - Identify construction waste - Apply waste reduction techniques - Dispose unusable waste as per SOP - Transport recycling material to designated area. | - Describe Importance of green skills - Basic knowledge of green energy resources (solar, biogas, natural light, rainwater, wind energy etc.) Practical Activity: Mock exercise of identification of environmental friendly material at site. | Theory-01 Hrs Practical-06Hrs Total- 07 Hrs | - Notebooks - Pen - White board PPES - Multimedia - Internet - Computer system | Class Room Working Site |
| - Develop green zones in urban area | Trainee will be able to: - Prepare plantation plan at soft area and bank of canals - Arrange green resources - Develop plans for park installation - Manage resources efficiently - Arrange eco-friendly garbage disposal | - Importance of urban planning - Types of urban plans - Zonal Plans Practical Activity: Mock exercise of developing green zones in an urban area | Theory-02 Hrs Practical-06Hrs Total- 08Hrs | - Notebooks - Pen - White board PPES - Multimedia - Internet Computer system | Class Room Working Site |
| - Manage sustainability of rebar fixing materials | - Select sustainable materials as per requirement - Follow standard procedure to manage systems (waste, energy) - Perform impact quantification of used material in steel fixing | - Environmental degradation of rebars - Types of rebar and fixing materials Practical Activity: Mock exercise of identification of different size of rebars at site. | Theory-01 Hrs Practical-06Hrs Total- 07 Hrs | - Notebooks - Pen - White board PPES - Multimedia - Internet - Computer system | Class Room Working Site |
| - Perform safe storage and disposal of waste | 1. Identify different types of waste material 2. Identify types of containers to store the different types of waste material 3. Use required labels on storage containers 4. Store the waste materials according to standards 5. Identify types of waste bins 6. Dispose-off waste material according to green and safety procedures | • Define types of waste • Differentiate between reusable and recyclable materials • Describe waste reduction techniques • Explain the concept of 6 R approach (Reduce, Reuse, Recycle, Repair, Renew, and Rethink) • Methods for disposal of unusable materials Activity: Perform mock exercise of collecting waste material at site, sort reusable and recyclable waste material and store/dispose of waste material | Theory-02 Hrs Practical-06Hrs Total- 08 Hrs | - Notebooks - Pen - White board PPES - Multimedia - Internet - Computer system | Class Room Working Site |

- List of Tools, Equipment and Consumable

18: Transportation Engineering

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1. | Drawing board | 25 Nos | 1 | Drawing Sheets | 100 Nos |
| 2. | T square | 25 Nos | 2 | Graph paper | 100 Nos |
| 3. | Pair of Set squares | 25 Nos | 3 | Scotch Tape | 50 Nos |
| 4. | Drawing Box (Compasses, divider, protractor, rule) | 25 Nos | 4 | Handkerchief | 100 Nos |
| 5. | French Curves | 25 Nos | 5 | Bitumen | 200 Kg |
| 6. | Set of Card board scales | 25 Nos | 6 | Coarse aggregate | 50 cft. |
| 7. | Sand cone Apparatus | 25 Nos | 7 | Sand | 50 cft. |
| 8. | calibrating container | 25 Nos | 8 | Ballast | 50 cft |
| 9. | glass plate | 25 Nos | 9 | Sharpener/ Sand paper | 100 Nos |
| 10. | base plate | 25 Nos |  |  |  |
| 11. | pan | 25 Nos |  |  |  |
| 12. | tools for excavating hole | 25 Nos |  |  |  |
| 13. | Electronic digital balance | 05 Nos |  |  |  |
| 14. | Set of sieves | 25 Nos |  |  |  |
| 15. | Los Angeles abrasion apparatus | 05 Nos |  |  |  |
| 16. | Set of sieves | 25 Nos |  |  |  |
| 17. | Sieve shaker | 15 Nos |  |  |  |
| 18. | Digital stop watch | 25 Nos |  |  |  |
| 19. | Bitumen ductility test apparatus | 25 Nos |  |  |  |
| 20. | Thermostatic lab oven | 25 Nos |  |  |  |
| 21. | Equipment for flash and fire test apparatus | 25 Nos |  |  |  |
| 22. | Centrifuge | 25 Nos |  |  |  |
| 23. | Spatula | 25 Nos |  |  |  |
| 24. | Standard Penetration Test Apparatus | 25 Nos |  |  |  |
| 25. | Marshal Stability Test (30KN Capacity) | 25 Nos |  |  |  |
| 26. | Say bolt Viscometer | 25 Nos |  |  |  |
| 27. | Softening Point Test (Ring & Ball Apparatus) | 25 Nos |  |  |  |
| 28. | Penetration of Bituminous Material | 25 Nos |  |  |  |
| 29. | Extraction of Bitumen | 25 Nos |  |  |  |

19: Hydraulics and Irrigation Engineering

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1. | Piezometer. | 05 Nos | 1. | Drawing sheet | 100 Nos |
| 2. | Water tank. | 05 Nos | 2. | Eraser | 100 Nos |
| 3. | Hydraulic Bench | 02 Nos | 3. | HB, H, 2H, 3H Pencils | 50 Nos each |
| 4. | Metacentric Height Measuring Apparatus: | 25 Nos | 4. | Scotch tape | 100Nos |
| 5. | Hydrostatic Pressure App. | 25 Nos |  |  |  |
| 6. | Venturi meter App. | 05 Nos |  |  |  |
| 7. | Manometer (U-Tube) | 05 Nos |  |  |  |
| 8. | Flow Over Weir Apparatus | 05 Nos |  |  |  |
| 9. | Pitot Tube | 05 Nos |  |  |  |
| 10. | Energy Losses in Pipe Apparatus | 05 Nos |  |  |  |
| 11. | Current Meter | 05 Nos |  |  |  |
| 12. | Friction in pipes apparatus. | 05 Nos |  |  |  |
| 13. | Orifices. | 05 Nos |  |  |  |
| 14. | Vee and trapezoidal notches. | 05 Nos |  |  |  |
| 15. | Weirs | 05 Nos |  |  |  |
| 16. | Drawing board | 25 Nos |  |  |  |
| 17. | T square | 25 Nos |  |  |  |
| 18. | Pair of Set squares | 25 Nos |  |  |  |
| 19. | Drawing Box (Compasses, divider, protractor, rule) | 50 Nos |  |  |  |
| 20. | Sharpener/ Sand paper | 100 Nos |  |  |  |

20: Soil Mechanics and Bridge Engineering

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | scales soaking tank or pan | 05 Nos | 1. | Soil samples | 25 Nos |
| 2 | Empty clean container | 05 Nos | 2. | clean dry sand passing from No.10 (2.0mm) sand retained on No.20 (850mm) sieve | 50 cft |
| 3 | Oven | 05 Nos | 3. | Drawing Sheet | 100 Nos |
| 4 | Balance accurate to 0.01gm | 05 Nos | 4. | Drafting table | 25 Nos |
| 5 | Speedy moisture tester | 25 Nos | 5. | Pencil | 50 Nos |
| 6 | Steel ball of 1.25 Inch diameter | 25 Nos | 6. | Ruler | 50 Nos |
| 7 | Brush for cleaning the tester | 25 Nos | 7. | Rubber | 50 Nos |
| 8 | Scoop for measuring Calcium carbide reagent | 05 Nos | 8. | Sharpener | 50 Nos |
| 9 | Pycnometer of about 1liter capacity | 25 Nos | 9. | Masking tape | 50 Nos |
| 10 | Glass rod | 25 Nos |  |  |  |
| 11 | Vacuum pump | 05 Nos |  |  |  |
| 12 | Set of ASTM sieves containing sieve # 4, 10, 20, 40, 60, 100, 200 | 5 Nos each |  |  |  |
| 13 | lid and pan | 25 Nos |  |  |  |
| 14 | Sieve shaker | 05 Nos |  |  |  |
| 15 | Soft brush for cleaning | 25 Nos |  |  |  |
| 16 | Evaporating dish | 25 Nos |  |  |  |
| 17 | Mixing Bowl | 25 Nos |  |  |  |
| 18 | Standard proctor Test Apparatus with rammer of 2.5Kg (5.5 lb) of fall 25.4Cm (12”) | 25 Nos |  |  |  |
| 19 | Balance 5 kg capacity | 05 Nos |  |  |  |
| 20 | GI Sheet Tray | 25 Nos |  |  |  |
| 21 | Standard proctor Test Apparatus with rammer 4.54Kg (10 lb), of fall 38Cm (18”) | 25 Nos |  |  |  |
| 22 | sieve 19 mm (3/4”) and 9.5mm (3/8”) | 25 Nos |  |  |  |
| 23 | straight edge | 25 Nos |  |  |  |
| 24 | Apparatus Core cutter | 25 Nos |  |  |  |
| 25 | Hammer | 25 Nos |  |  |  |
| 26 | excavating tools | 25 Nos |  |  |  |
| 27 | Sand Cone Apparatus | 25 Nos |  |  |  |
| 28 | calibrating container | 25 Nos |  |  |  |
| 29 | Spoon | 25 Nos |  |  |  |
| 30 | Ladle | 25 Nos |  |  |  |
| 31 | Spade | 25 Nos |  |  |  |
| 32 | Trowel | 25 Nos |  |  |  |
| 33 | Beater | 25 Nos |  |  |  |
| 34 | Drawknife | 25 Nos |  |  |  |
| 35 | Sample Bags Chisel | 25 Nos |  |  |  |
| 36 | Flat Scoop | 25 Nos |  |  |  |
| 37 | Small Curve Scoop | 25 Nos |  |  |  |
| 38 | Medium Curve Scoop | 25 Nos |  |  |  |
| 39 | Large Curve Scoop | 25 Nos |  |  |  |
| 40 | Flat Brush | 25 Nos |  |  |  |
| 41 | Canvas 2 X 2 M. | 25 Nos |  |  |  |
| 42 | glass plate | 25 Nos |  |  |  |
| 43 | Constant head Permeameter device (including constant head filter tank and manometer tubes) | 25 Nos |  |  |  |
| 44 | Graduated Cylinder 1000 Ml Capacity | 25 Nos |  |  |  |
| 45 | Graduated Cylinder 2000 Ml Capacity | 25 Nos |  |  |  |
| 46 | 500 ml beaker | 25 Nos |  |  |  |
| 47 | 1000 ml beaker | 25 Nos |  |  |  |
| 48 | Vernier Caliper | 25 Nos |  |  |  |
| 49 | Desiccator | 25 Nos |  |  |  |
| 50 | Canvas Gloves | 25 Nos |  |  |  |
| 51 | Gloves, Latex Rubber | 25 Nos |  |  |  |
| 52 | Plastic Bottle, Cap. 2 L. | 25 Nos |  |  |  |
| 53 | Plastic Bottle, Cap. 5 L. | 25 Nos |  |  |  |
| 54 | Rubber Bucket, Cap. 12 L. | 25 Nos |  |  |  |
| 55 | Rubber Bucket, Cap. 18 L. | 25 Nos |  |  |  |
| 56 | Stainless Steel Bowls | 25 Nos |  |  |  |
| 57 | Steel Hammer | 25 Nos |  |  |  |
| 58 | Pick Hammer | 25 Nos |  |  |  |
| 59 | Rubber Hammer | 25 Nos |  |  |  |
| 60 | Stop watch | 25 Nos |  |  |  |
| 61 | Thermometer | 25 Nos |  |  |  |
| 62 | Apparatus Falling head Permeameter device | 25 Nos |  |  |  |
| 63 | Ring stand with test tube clamp | 25 Nos |  |  |  |
| 64 | Burette (100ml) | 25 Nos |  |  |  |
| 65 | Shear box | 25 Nos |  |  |  |
| 66 | Shear box container | 25 Nos |  |  |  |
| 67 | Base plate with cross groves on its top | 25 Nos |  |  |  |
| 68 | Porous stones (2 Nos) | 25 Nos |  |  |  |
| 69 | Plain Grid plates (2Nos) | 25 Nos |  |  |  |
| 70 | Perforated grid plates (2Nos) | 25 Nos |  |  |  |
| 71 | Loading pad with steel ball | 25 Nos |  |  |  |
| 72 | Digital weighing machine | 25 Nos |  |  |  |
| 73 | Loading frame with loading yoke | 25 Nos |  |  |  |
| 74 | Proving ring | 25 Nos |  |  |  |
| 75 | Dial gauges (2 Nos) | 25 Nos |  |  |  |
| 76 | Weights | 25 Nos |  |  |  |
| 77 | Tampering Rod | 25 Nos |  |  |  |
| 78 | Spatula | 25 Nos |  |  |  |
| 79 | Triangle Spatula | 25 Nos |  |  |  |
| 80 | Sampler | 25 Nos |  |  |  |
| 81 | Distribution Panel | 05 Nos |  |  |  |
| 82 | Automatic Pressure/Volume Controllers | 05 Nos |  |  |  |
| 83 | Load Frame | 25 Nos |  |  |  |
| 84 | Load Cell | 25 Nos |  |  |  |
| 85 | Displacement transducer | 25 Nos |  |  |  |
| 86 | Tri axial Cell | 25 Nos |  |  |  |
| 87 | Pore Pressure Transducer | 25 Nos |  |  |  |
| 88 | De-aired water supplier | 25 Nos |  |  |  |
| 89 | Data logger | 25 Nos |  |  |  |
| 90 | Sample extruder | 25 Nos |  |  |  |
| 91 | Sample moulds | 25 Nos |  |  |  |
| 92 | Membranes | 25 Nos |  |  |  |
| 93 | ‘o’ rings | 25 Nos |  |  |  |
| 94 | Porous discs | 25 Nos |  |  |  |
| 95 | Ruler | 25 Nos |  |  |  |
| 96 | Filter papers | 25 Nos |  |  |  |
| 97 | Drilling Rig. | 25 Nos |  |  |  |
| 98 | Split spoon (tube). | 25 Nos |  |  |  |
| 99 | Drop hammer | 25 Nos |  |  |  |
| 100 | Driving head (anvil). | 25 Nos |  |  |  |
| 101 | Guiding rod. | 25 Nos |  |  |  |
| 102 | Tripod. | 25 Nos |  |  |  |
| 103 | Extension rods | 25 Nos |  |  |  |
| 104 | Cylindrical mould | 25 Nos |  |  |  |
| 105 | detachable extension collar | 25 Nos |  |  |  |
| 106 | detachable perforated base plate | 25 Nos |  |  |  |
| 107 | Spacer disc | 25 Nos |  |  |  |
| 108 | Metal rammer | 25 Nos |  |  |  |
| 109 | Loading frame | 25 Nos |  |  |  |
| 110 | Metal penetration piston | 25 Nos |  |  |  |
| 111 | Two dial gauges reading to 0.01 mm. | 02 Nos |  |  |  |
| 112 | Sieves. 4.75 mm and 20 mm I.S. Sieves | 25 Nos |  |  |  |

21: RCC Design

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Calculator | 25 Nos | 1. | Pencil | 25 Nos |
| 2 | Ruler | 25 Nos | 2. | Eraser | 25 Nos |
| 3 | Drawing lab equipment | 25 Nos |  |  |  |

22: Environment Technology

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
|  | Dissolved Oxygen meter and sensor | 25 Nos | 1 | Eraser | 50 Nos |
|  | Turbidity meter | 25 Nos | 2 | Pencil | 50 Nos |
|  | Litmus Paper | 100 Nos |  |  |  |
|  | Spectrophotometer | 25 Nos |  |  |  |
|  | Thermometer | 25 Nos |  |  |  |
|  | Containers | 25 Nos |  |  |  |
|  | Sound level meter | 05 Nos |  |  |  |
|  | Charts | 25 Nos |  |  |  |
|  | Ear Muff, Ear Plug | 25 Nos |  |  |  |
|  | Safety shoes | 25 Nos |  |  |  |
|  | Gloves | 25 Nos |  |  |  |
|  | Safety Goggles | 25 Nos |  |  |  |
|  | Safety Helmet | 25 Nos |  |  |  |
|  | Respiratory protective equipment i.e. Masks | 25 Nos |  |  |  |
|  | Life Jacket | 25 Nos |  |  |  |
|  | High visibility jacket | 25 Nos |  |  |  |
|  | Safety harness | 25 Nos |  |  |  |
|  | Face shield | 25 Nos |  |  |  |
|  | Rubber shoes/ long shoes | 25 Nos |  |  |  |
|  | Water Buckets | 05 Nos |  |  |  |
|  | Fire Blankets | 05 Nos |  |  |  |
|  | Sand Buckets | 05 Nos |  |  |  |
|  | First Aid Box | 05 Nos |  |  |  |
|  | Thermometer and B.P apparatus | 05 Nos |  |  |  |
|  | Fire Extinguisher | 05 Nos |  |  |  |
|  | Fire Alarm | 05 Nos |  |  |  |
|  | Sprinklers | 25 Nos |  |  |  |
|  | Computer system | 25 Nos |  |  |  |

23: Construction Project Planning and Management

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1. | Calculator | 25 Nos | 1. | Pencil | 50 Nos |
| 2. | Ruler | 25 Nos | 2. | Eraser | 50 Nos |
| 3. | Computer with mouse & keyboard | 25 Nos | 3. | Printer Toner | 02 Nos |
| 4. | Primavera P6 Software installed | 1 no | 4. | Printer paper | 02 Rims |

24: Civil Engineering Project

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1. | Calculator | 25 Nos | 1. | Pencil | 50 Nos |
| 2. | Ruler | 25 Nos | 2. | Eraser | 50 Nos |
| 3. | Drawing lab equipment | 25 Nos | 3. | Materials related to labs mentioned in equipment list | 25 Nos |
| 4. | Survey Lab Equipment | 25 Nos |  |  |  |
| 5. | AutoCAD/ Computer Lab Equipment | 25 Nos |  |  |  |
| 6. | Materials Testing Lab Equipment | 25 Nos |  |  |  |

25: Entrepreneurial Skills

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Calculator | 25 Nos | 1. | Pencil | 50 Nos |
| 2 | Ruler | 25 Nos | 2. | Eraser | 50 Nos |
| 3 | Notebook | 25 Nos |  |  |  |

- Members of Curriculum Development Committee

The following members participated in the Curriculum development Committee:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr. Azhar Iqbal Shad | Chief Instructor (Civil) | GCT Raiwnd Road, LHR |
| 2 | Mr. Imtiaz Ahmed Awan | Sr. Instructor (Civil) | GCT Rasul (Mandi Bahauddin) |
| 3 | Mr. Muhammad Amjad Rafique | Sr. Instructor (Civil) | GCT Rasul (Mandi Bahauddin) |
| 4 | Engr. Zuneera Ashfaq | Assistant Director | PITAC Lahore |
| 5 | Engr. Mohsin Jahanzeb | Instructor (Civil) | GSTC, Faisalabad |
| 6 | Engr. Norheen Amina | Consultant | Allied Engineering Services Lahore |
| 7 | Engr. Hira Ishtiaq | Consultant. | Allied Engineering Services Lahore |
| 8 | Engr. Habiba Mohsin | Design Engineer | JERS Engineering Consultants Lahore |
| 9 | Engr. Rana Haroon Mujahid | Deputy Director | C&W Department Govt. of Punjab |
| 10 | Engr. Arsalan Hameed Khan | Assistant Manager (Project) | Planning & Project department Lahore Waste Management Company. |
| 11 | Mr. Syed Fayyaz Mustafa Naqvi | Junior Instructor | GSTC Faisalabad |
| 12 | Engr. Inayat-ur-Rehman | DACUM, Facilitator | Peshawar |
| 13 | Mr. Muhammad Ishaq | Director HR | NAVTTC, Islamabad |