Instruction Sheet for the Candidate

| Qualification | Construction Machinery (Level-2) Workshop Practice-I |
| --- | --- |
| Competency Standard | Perform different welding joints |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Practice of Making Butt Joint - Practice of making Lap Joint - Practice of making Tee Joint |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Practice of Making Butt Joint - Take M.S Flat as per drawing - Straiten it with the help of hammer and anvil - Place the work piece on welding table - Hold the electrode in electrode holder - Set the current on welding machine as per standard - Hold the electrode at right angle - Set the current at 120-125 ampere - Tack both pieces of M.S Flat for Butt Joint - Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap - Lean the electrode at 70º to the base metal and complete the bead to form a square butt joint. Practice of making Lap Joint - Mark parallel line on both pieces with the help of scriber for positioning of lap joint - Place the work piece on welding table - Place the Bottom piece on work table and place the Top plate along the marked line - Hold the electrode in electrode holder - Set the current on welding machine as per standard Practice of making Tee Joint. - Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other - Place the Bottom piece on work table and place the Top plate at 90* to each other as per drawing - Hold the electrode in electrode holder - Set the current on welding machine as per standard - Tack both work pieces of M.S Flat - Lean the electrode and move along traverse direction - Complete the bead as per standard |

Self-Assessment Checklist.

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Construction Machinery (Level-2) Workshop Practice-I |
| Competency Standard | Perform different welding joints |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Set Welding Equipment - Prepare materials for welding - Cut and Prepare Edge/s of Base Materials |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Take M.S Flat as per drawing |  |  |
| - Straiten it with the help of hammer and anvil |  |  |
| - Place the work piece on welding table |  |  |
| - Hold the electrode in electrode holder |  |  |
| - Set the current on welding machine as per standard |  |  |
| - Hold the electrode at right angle |  |  |
| - Set the current at 120-125 ampere |  |  |
| - Tack both pieces of M.S Flat for Butt Joint. |  |  |
| - Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap. |  |  |
| - Lean the electrode at 70º to the base metal and complete the bead to form a square butt joint. |  |  |
| - Mark parallel line on both pieces with the help of scriber for positioning of lap joint |  |  |
| - Place the work piece on welding table |  |  |
| - Place the Bottom piece on work table and place the Top plate along the marked line |  |  |
| - Hold the electrode in electrode holder |  |  |
| - Set the current on welding machine as per standard. |  |  |
| - Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other. |  |  |
| - Place the Bottom piece on work table and place the Top plate at 90* to each other as per drawing. |  |  |
| - Hold the electrode in electrode holder |  |  |
| - Set the current on welding machine as per standard |  |  |
| - Tack both work pieces of M.S Flat. |  |  |
| - Lean the electrode and move along traverse direction. |  |  |
| - Complete the bead as per standard. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Construction Machinery (Level-2) Workshop Practice-I |
| --- | --- |
| Competency Standard | Perform different welding joints |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: _________________________________________________ Registration/Roll Number: ________________Signature: _____________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor ____________________________________ Assessor’s code: ___________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Set Welding Equipment - Prepare materials for welding - Cut and Prepare Edge/s of Base Materials |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Take M.S Flat as per drawing |  |  |  |  |  |
|  | Straiten it with the help of hammer and anvil |  |  |  |  |  |
|  | Place the work piece on welding table |  |  |  |  |  |
|  | Hold the electrode in electrode holder |  |  |  |  |  |
|  | Set the current on welding machine as per standard |  |  |  |  |  |
|  | Hold the electrode at right angle |  |  |  |  |  |
|  | Set the current at 120-125 ampere |  |  |  |  |  |
|  | Tack both pieces of M.S Flat for Butt Joint. |  |  |  |  |  |
|  | Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap. |  |  |  |  |  |
|  | Lean the electrode at 70º to the base metal and complete the bead to form a square butt joint. |  |  |  |  |  |
|  | Mark parallel line on both pieces with the help of scriber for positioning of lap joint |  |  |  |  |  |
|  | Place the work piece on welding table |  |  |  |  |  |
|  | Place the Bottom piece on work table and place the Top plate along the marked line |  |  |  |  |  |
|  | Hold the electrode in electrode holder |  |  |  |  |  |
|  | Set the current on welding machine as per standard. |  |  |  |  |  |
|  | Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other. |  |  |  |  |  |
|  | Place the Bottom piece on work table and place the Top plate at 90* to each other as per drawing. |  |  |  |  |  |
|  | Hold the electrode in electrode holder |  |  |  |  |  |
|  | Set the current on welding machine as per standard |  |  |  |  |  |
|  | Tack both work pieces of M.S Flat. |  |  |  |  |  |
|  | Lean the electrode and move along traverse direction. |  |  |  |  |  |
|  | Complete the bead as per standard. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Construction Machinery (Level-2) Workshop Practice-I |
| --- | --- |
| Competency Standard | Perform different welding joints |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: __________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ____ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: _______________________________________ Assessor’s code: ________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is Electrical supply AC and DC? |  |  |
|  | Define motion of electrode in ARC welding |  |  |
|  | Define importance of gap between electrode and base metal |  |  |
|  | Define importance of cleanliness of surface to be welded. |  |  |
|  | What is WPS? |  |  |
|  | Define Welding codes. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________