Instruction Sheet for the Candidate

| Qualification | Construction Machinery Level-3 (Automotive Electrics & Electronics L-3) |
| --- | --- |
| Competency Standard | Repair Battery Charging System |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Repair charging system - Replace alternator |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Repair Charging System - Inspect vehicle charging system and diagnose the following: Battery warning light indication on instrument panel, test output voltage of alternator, drive belt condition, alternator bearing noise, wire harness, fuse, and proper insulated wires. - Dismantle/ service of alternator as following; Repair connector, replace carbon brushes, Replace Rectifier Bridge, replace voltage regulator and replace stator winding. Replace Alternator - Perform safe working practice to remove the alternator from the vehicle and refit under the specified procedure. - Perform test bench of alternator to confirm the operation. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Construction Machinery Level-3 (Automotive Electrics & Electronics L-3) |
| Competency Standard | Repair Battery Charging System |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Repair Charging System - Replace Alternator |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Inspect vehicle charging system and diagnose the following: Battery warning light indication on instrument panel, test output voltage of alternator, drive belt condition, alternator bearing noise, wire harness, fuse, and proper insulated wires. |  |  |
| - Dismantle/ service of alternator as following; Repair connector, replace carbon brushes, Replace Rectifier Bridge, replace voltage regulator and replace stator winding. |  |  |
| - Perform safe working practice to remove the alternator from the vehicle and refit under the specified procedure. |  |  |
| - Perform test bench of alternator to confirm the operation. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Construction Machinery Level-3 (Automotive Electrics & Electronics) |
| --- | --- |
| Competency Standard | Repair Battery Charging System |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Repair Charging System - Replace Alternator |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Inspected vehicle charging system and diagnosed the following: Battery warning light indication on instrument panel, tested output voltage of alternator, drive belt condition, alternator bearing noise, wire harness, fuse, and proper insulated wires. |  |  |  |  |  |
|  | Dismantled/ serviced alternator as following; Repaired connector, replaced carbon brushes, Replaced Rectifier Bridge, replaced voltage regulator and replaced stator winding. |  |  |  |  |  |
|  | Performed safe working practice to remove the alternator from the vehicle and refitted under the specified procedure. |  |  |  |  |  |
|  | Performed test bench of alternator to confirm the operation. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Construction Machinery Level-3 (Automotive Electrics & Electronics L-3) |
| --- | --- |
| Competency Standard | Repair Battery Charging System |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe the Purpose and function of Alternator’s following parts; Rotor, stator windings, Voltage regulator, Rectifier Bridge, Internal cooling fan, Bearings, carbon brush, mounting/ adjustment bolts), drive belt and its types, ratio alternator/ crank pulleys, RPM, Alternator input/ output voltages. |  |  |
|  | Describe the functions and uses of the following: Semiconductor and its types, Rectification (Half wave/ full wave), transformer and its types, capacitor and its types. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________