Instruction Sheet for the Candidate

| Qualification | Construction Machinery Level-4 ( Problems in IC Engine) |
| --- | --- |
| Competency Standard | Diagnose Engine Cooling System Faults and Repair |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): CU1. Cooling system service CU2. Repair Cooling system |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Cooling system service - Inspect engine cooling system’s main components and diagnose the performance of (Radiator cap, Radiator fan, water body/ pump, drive belt/ fan belt, thermostat Valve, radiator, Drain plug, hoses/ clamps, and gasket/ seals) - Flush the cooling system. - Clean the radiator core with air by revising the direction of rotation of fan motor. - Inspect/ service the radiator and top up with the coolant/ antifreeze as per specified ratio in the manual. Repair Cooling system - Change the water body under specified procedure and start the engine to confirm correct operation. - Adjust fan belt tension under specified procedure and start the engine to confirm correct operation. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Construction Machinery Level-4 ( Problems in IC Engine) |
| Competency Standard | Diagnose Engine Cooling System Faults and Repair |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Cooling system service - Repair Cooling system |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Inspect engine cooling system’s main components and diagnose the performance of (Radiator cap, Radiator fan, water body/ pump, drive belt/ fan belt, thermostat Valve, radiator, Drain plug, hoses/ clamps, and gasket/ seals) |  |  |
| - Flush the cooling system. |  |  |
| - Clean the radiator core with air by revising the direction of rotation of fan motor. |  |  |
| - Inspect/ service the radiator and top up with the coolant/ antifreeze as per specified ratio in the manual. |  |  |
| - Change the water body under specified procedure and start the engine to confirm correct operation. |  |  |
| - Adjust fan belt tension under specified procedure and start the engine to confirm correct operation. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Construction Machinery Level-4 ( Problems in IC Engine) |
| --- | --- |
| Competency Standard | Diagnose Engine Cooling System Faults and Repair |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Cooling system service - Repair Cooling system |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Inspected engine cooling system’s main components and diagnose the performance of (Radiator cap, Radiator fan, water body/ pump, drive belt/ fan belt, thermostat Valve, radiator, Drain plug, hoses/ clamps, and gasket/ seals) |  |  |  |  |  |
|  | Flushed the cooling system. |  |  |  |  |  |
|  | Cleaned the radiator core with air by revising the direction of rotation of fan motor. |  |  |  |  |  |
|  | Inspected/ serviced the radiator and top up with the coolant/ antifreeze as per specified ratio in the manual. |  |  |  |  |  |
|  | Changed the water body under specified procedure and started the engine to confirm correct operation. |  |  |  |  |  |
|  | Adjusted fan belt tension under specified procedure and started the engine to confirm correct operation. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Construction Machinery Level-4 ( Problems in IC Engine) |
| --- | --- |
| Competency Standard | Diagnose Engine Cooling System Faults and Repair |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe the operating principles and terminologies of cooling system as following. Air cooled engine, liquid cooled engine, operating temperature, thermal efficiency, pressurized system, heat exchange method (conduction, radiation, and convection), corrosion, and inhibitors/ antifreeze) |  |  |
|  | Describe the functions and purpose of engine cooling system components as following. Electrical motor operated fan with thermo switch, belt operated fan, temperature controlled/viscous coupling fan, pressure cap, water body/pump, drive belt, thermostat, radiator, core plug, hoses/clamps, and gasket/ seals. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________