Instruction Sheet for the Candidate

| Qualification | Construction Machinery (Level-4) Operation of Construction Machinery |
| --- | --- |
| Competency Standard | Operate Bulldozer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Operate Controls Levers/joystick - Strip and stockpile surface materials - Cut and fill material - Create slopes - Create ditches - Spread ballast - Rip dense materials - Clear land - Push scraper |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Operate Controls Levers/joystick - Operate control levers/joystick smoothly and safely - Operate different operating control levers/joystick simultaneously as required - React to changing conditions/situations Strip and stockpile surface materials - Distinguish waste layer from structural layer - Remove waste layer - Move full blade load with optimum capacity - Clean up windrows and any remaining waste material - Stockpile waste materials. Cut and fill material - Estimate the height of cuts and fills - Observe grade by coordination with surveyors - Cut “humps” and create enough loose material to fill blade before pushing to haulage distance - Push material to fill depressions - Match blade load with available power and traction - Perform rough leveling of ground - Eliminate windrows and clean up. Create slopes - Interpret stakes/specifications - Apply grade checking instruments - Cut the slope next to each row of stakes - Perform heavy cuts down hill - Match blade load with available power and traction - Apply safe practices regarding stability issues - Grade area to a given slope and eliminate windrows and clean up. Create ditches - Identify the required profile using grade checking instruments - Create ditch of specified dimensions - Stockpile or blend in material - Level the ground roughly, eliminate windrows and clean up. Spread ballast - Identify dumping location and pattern - Match blade load with available power and traction - Spread material - Grade to requisite level Rip dense materials - Rip hard strata - Balance ripper load depth & load to available power and traction Clear land - Work around obstructions and hazards - Clear land in accordance with job specifications. Push scraper - Balance engine power to load and traction - Minimize wear & tear impact, track spinning - Assess grade and level. - Remove obstacles and rocks. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Construction Machinery (Level-4) Operation of Construction Machinery |
| Competency Standard | Operate Bulldozer |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Operate Controls Levers/joystick - Strip and stockpile surface materials - Cut and fill material - Create slopes - Create ditches - Spread ballast - Rip dense materials - Clear land - Push scraper |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Operate control levers/joystick smoothly and safely |  |  |
| - Operate different operating control levers/joystick simultaneously as required |  |  |
| - React to changing conditions/situations |  |  |
| - Distinguish waste layer from structural layer |  |  |
| - Remove waste layer |  |  |
| - Move full blade load with optimum capacity |  |  |
| - Clean up windrows and any remaining waste material |  |  |
| - Stockpile waste materials. |  |  |
| - Estimate the height of cuts and fills |  |  |
| - Observe grade by coordination with surveyors |  |  |
| - Cut “humps” and create enough loose material to fill blade before pushing to haulage distance |  |  |
| - Push material to fill depressions |  |  |
| - Match blade load with available power and traction |  |  |
| - Perform rough leveling of ground |  |  |
| - Eliminate windrows and clean up. |  |  |
| - Interpret stakes/specifications |  |  |
| - Apply grade checking instruments |  |  |
| - Cut the slope next to each row of stakes |  |  |
| - Perform heavy cuts down hill |  |  |
| - Match blade load with available power and traction |  |  |
| - Apply safe practices regarding stability issues |  |  |
| - Grade area to a given slope and eliminate windrows and clean up. |  |  |
| - Identify the required profile using grade checking instruments |  |  |
| - Create ditch of specified dimensions |  |  |
| - Stockpile or blend in material |  |  |
| - Level the ground roughly, eliminate windrows and clean up. |  |  |
| - Identify dumping location and pattern |  |  |
| - Match blade load with available power and traction |  |  |
| - Spread material |  |  |
| - Grade to requisite level |  |  |
| - Rip hard strata |  |  |
| - Balance ripper load depth & load to available power and traction |  |  |
| - Work around obstructions and hazards |  |  |
| - Clear land in accordance with job specifications. |  |  |
| - Balance engine power to load and traction |  |  |
| - Minimize wear & tear impact, track spinning |  |  |
| - Assess grade and level. |  |  |
| - Remove obstacles and rocks |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Construction Machinery (Level-4) Operation of Construction Machinery |
| --- | --- |
| Competency Standard | Operate Bulldozer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Operate Controls Levers/joystick - Strip and stockpile surface materials - Cut and fill material - Create slopes - Create ditches - Spread ballast - Rip dense materials - Clear land - Push scraper |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Operated control levers/joystick smoothly and safely |  |  |  |  |  |
|  | Operated different operating control levers/joystick simultaneously as required |  |  |  |  |  |
|  | Reacted to changing conditions/situations |  |  |  |  |  |
|  | Distinguished waste layer from structural layer |  |  |  |  |  |
|  | Removed waste layer |  |  |  |  |  |
|  | Moved full blade load with optimum capacity |  |  |  |  |  |
|  | Cleaned up windrows and any remaining waste material |  |  |  |  |  |
|  | Stockpiled waste materials. |  |  |  |  |  |
|  | Estimated the height of cuts and fills |  |  |  |  |  |
|  | Observed grade by coordination with surveyors |  |  |  |  |  |
|  | Cut “humps” and create enough loose material to fill blade before pushing to haulage distance |  |  |  |  |  |
|  | Pushed material to fill depressions |  |  |  |  |  |
|  | Matched blade load with available power and traction |  |  |  |  |  |
|  | Performed rough leveling of ground |  |  |  |  |  |
|  | Eliminated windrows and clean up. |  |  |  |  |  |
|  | Interpreted stakes/specifications |  |  |  |  |  |
|  | Applied grade checking instruments |  |  |  |  |  |
|  | Cut the slope next to each row of stakes |  |  |  |  |  |
|  | Performed heavy cuts down hill |  |  |  |  |  |
|  | Matched blade load with available power and traction |  |  |  |  |  |
|  | Applied safe practices regarding stability issues |  |  |  |  |  |
|  | Grade area to a given slope and eliminate windrows and clean up. |  |  |  |  |  |
|  | Identified the required profile using grade checking instruments |  |  |  |  |  |
|  | Created ditch of specified dimensions |  |  |  |  |  |
|  | Stockpile or blend in material |  |  |  |  |  |
|  | Leveled the ground roughly, eliminate windrows and clean up. |  |  |  |  |  |
|  | Identified dumping location and pattern |  |  |  |  |  |
|  | Matched blade load with available power and traction |  |  |  |  |  |
|  | Spread material |  |  |  |  |  |
|  | Grade to requisite level |  |  |  |  |  |
|  | Rip hard strata |  |  |  |  |  |
|  | Balanced ripper load depth & load to available power and traction |  |  |  |  |  |
|  | Worked around obstructions and hazards |  |  |  |  |  |
|  | Cleared land in accordance with job specifications. |  |  |  |  |  |
|  | Balanced engine power to load and traction |  |  |  |  |  |
|  | Minimized wear & tear impact, track spinning |  |  |  |  |  |
|  | Assessed grade and level. |  |  |  |  |  |
|  | Removed obstacles and rocks |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Construction Machinery (Level-4) Operation of Construction Machinery |
| --- | --- |
| Competency Standard | Operate Bulldozer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define Bulldozer |  |  |
|  | Enlist type of Bulldozer |  |  |
|  | Describe blade |  |  |
|  | Define ripper |  |  |
|  | Enlist types of Track Chain |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________