Instruction Sheet for the Candidate

| Qualification | Construction Machinery (Level-4) Operation of Construction Machinery |
| --- | --- |
| Competency Standard | Operate Motor Grader |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Operate Control levers/joystick - Apply Grading Fundamentals - Form and handle windrows - Strip surface materials - Cut and fill material - Maintain access roads - Create slopes - Create ditches - Create shouldering - Form sub-Grade - Place aggregates to specified elevations (finish grading) - Clear snow and ice |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Operate Control levers/joystick - Operate control levers/joystick smoothly and safely. - Operate different operating control levers/joystick simultaneously as required - React to changing conditions/situations. Apply Grading Fundamentals - Apply wheel lean control - Apply frame articulation fundamentals - Select gear and engine speed - Apply grading tips Form and handle windrows - Choose gear and engine speed - Choose blade position - Cut material to form a windrow - Move material back over area Strip surface materials - Distinguish waste layer from structural layer - Strip waste materials (usually organic) - Finish windrows of stripped material - Enter and exit machine Cut and fill material - Estimate the height of cut and fill - Choose blade tilt, angel and position - Cut heights - Match blade load to available power and traction - Move material to low areas - Grade area to desired profile Maintain access roads - Identify drainage structures, culverts and obstacles - Adjust windrow to allow traffic to continue - Choose blade position, wheel lean, articulation, gear and speed - Reshape and recover materials for the road surface - Cut shoulders and move material to center or from one side to another Create slopes - Identify required slope - Apply grade checking instruments - Choose blade position, wheel lean, articulation, gear and speed - Smooth the area at the base of the slope for smooth working platform - Start at the top of slope - Shape the shoulder accurately Create ditches - Identify the required profile using grade checking instruments - Choose blade position, wheel lean, articulation, gear and speed - Shape ditch by repeated passes. Create shouldering - Choose blade position, wheel lean, articulation, gear and speed - Position grader with outer tires on pavement, and inner tires just off pavement on shoulder for left side shoulder - Position grader with inner tires on pavement, and outer tires just off pavement on shoulder for right side shoulder - Move only enough material to pavement edge to dress the shoulder - Roll the windrow back away from the pavement edge - Shape the shoulder accurately Form sub-Grade - Choose blade tilt, angel and position - Match blade load to available power and traction - Remove unsuitable material - Cut and fill load bearing soils to create desired profile - Shape for drainage and ditch as required Place aggregates to specified elevations (finish grading) - Identify the required profile using grade checking instruments - Get the correct volume in the efficient placement - Position for efficient spreading - Get correct volume of aggregates - Shift the circle and blade towards the piles - Cut out windrows only as large as the machine can handle without tire spinning - Angle the blade as appropriate - Precise control to achieve elevations and shape to very accurate tolerances Clear snow and ice - Choose proper attachment, as chains, V-plow, wing plow, skid shoes and wing gates - Identify snow type, moisture content, density, weight, depth of snow, underlying surface, weather, visibility, traffic, obstacles and hidden structures - Mount chain on tires carefully - Drive the machine in higher speed to move snow across and off the blade |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Construction Machinery (Level-4) Operation of Construction Machinery |
| Competency Standard | Operate Motor Grader |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Operate Control levers/joystick - Apply Grading Fundamentals - Form and handle windrows - Strip surface materials - Cut and fill material - Maintain access roads - Create slopes - Create ditches - Create shouldering - Form sub-Grade - Place aggregates to specified elevations (finish grading) - Clear snow and ice |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Operate control levers/joystick smoothly and safely. |  |  |
| - Operate different operating control levers/joystick simultaneously as required |  |  |
| - React to changing conditions/situations |  |  |
| - Apply wheel lean control |  |  |
| - Apply frame articulation fundamentals |  |  |
| - Select gear and engine speed |  |  |
| - Apply grading tips |  |  |
| - Choose gear and engine speed |  |  |
| - Choose blade position |  |  |
| - Cut material to form a windrow |  |  |
| - Move material back over area |  |  |
| - Distinguish waste layer from structural layer |  |  |
| - Strip waste materials (usually organic) |  |  |
| - Finish windrows of stripped material |  |  |
| - Enter and exit machine |  |  |
| - Estimate the height of cut and fill |  |  |
| - Choose blade tilt, angel and position |  |  |
| - Cut heights |  |  |
| - Match blade load to available power and traction |  |  |
| - Move material to low areas |  |  |
| - Grade area to desired profile |  |  |
| - Identify drainage structures, culverts and obstacles |  |  |
| - Adjust windrow to allow traffic to continue |  |  |
| - Choose blade position, wheel lean, articulation, gear and speed |  |  |
| - Reshape and recover materials for the road surface |  |  |
| - Cut shoulders and move material to center or from one side to another |  |  |
| - Identify required slope |  |  |
| - Apply grade checking instruments |  |  |
| - Choose blade position, wheel lean, articulation, gear and speed |  |  |
| - Smooth the area at the base of the slope for smooth working platform |  |  |
| - Start at the top of slope |  |  |
| - Shape the shoulder accurately |  |  |
| - Identify the required profile using grade checking instruments |  |  |
| - Choose blade position, wheel lean, articulation, gear and speed |  |  |
| - Shape ditch by repeated passes. |  |  |
| - Choose blade position, wheel lean, articulation, gear and speed |  |  |
| - Position grader with outer tires on pavement, and inner tires just off pavement on shoulder for left side shoulder |  |  |
| - Position grader with inner tires on pavement, and outer tires just off pavement on shoulder for right side shoulder |  |  |
| - Move only enough material to pavement edge to dress the shoulder |  |  |
| - Roll the windrow back away from the pavement edge |  |  |
| - Shape the shoulder accurately |  |  |
| - Choose blade position, wheel lean, articulation, gear and speed |  |  |
| - Choose blade tilt, angel and position |  |  |
| - Match blade load to available power and traction |  |  |
| - Remove unsuitable material |  |  |
| - Cut and fill load bearing soils to create desired profile |  |  |
| - Shape for drainage and ditch as required |  |  |
| - Identify the required profile using grade checking instruments |  |  |
| - Get the correct volume in the efficient placement |  |  |
| - Position for efficient spreading |  |  |
| - Get correct volume of aggregates |  |  |
| - Shift the circle and blade towards the piles |  |  |
| - Cut out windrows only as large as the machine can handle without tire spinning |  |  |
| - Angle the blade as appropriate |  |  |
| - Precise control to achieve elevations and shape to very accurate tolerances |  |  |
| - Choose proper attachment, as chains, V-plow, wing plow, skid shoes and wing gates |  |  |
| - Identify snow type, moisture content, density, weight, depth of snow, underlying surface, weather, visibility, traffic, obstacles and hidden structures |  |  |
| - Mount chain on tires carefully |  |  |
| - Drive the machine in higher speed to move snow across and off the blade |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Construction Machinery (Level-4) Operation of Construction Machinery |
| --- | --- |
| Competency Standard | Operate Motor Grader |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Operate Control levers/joystick - Apply Grading Fundamentals - Form and handle windrows - Strip surface materials - Cut and fill material - Maintain access roads - Create slopes - Create ditches - Create shouldering - Form sub-Grade - Place aggregates to specified elevations (finish grading) - Clear snow and ice |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Operated control levers/joystick smoothly and safely. |  |  |  |  |  |
|  | Operated different operating control levers/joystick simultaneously as required |  |  |  |  |  |
|  | Reacted to changing conditions/situations |  |  |  |  |  |
|  | Applied wheel lean control |  |  |  |  |  |
|  | Applied frame articulation fundamentals |  |  |  |  |  |
|  | Selected gear and engine speed |  |  |  |  |  |
|  | Applied grading tips |  |  |  |  |  |
|  | Choose gear and engine speed |  |  |  |  |  |
|  | Choose blade position |  |  |  |  |  |
|  | Cu material to form a windrow |  |  |  |  |  |
|  | Moved material back over area |  |  |  |  |  |
|  | Distinguished waste layer from structural layer |  |  |  |  |  |
|  | Striped waste materials (usually organic) |  |  |  |  |  |
|  | Finished windrows of stripped material |  |  |  |  |  |
|  | Entered and exit machine |  |  |  |  |  |
|  | Estimated the height of cut and fill |  |  |  |  |  |
|  | Choose blade tilt, angel and position |  |  |  |  |  |
|  | Cut heights |  |  |  |  |  |
|  | Matched blade load to available power and traction |  |  |  |  |  |
|  | Moved material to low areas |  |  |  |  |  |
|  | Graded area to desired profile |  |  |  |  |  |
|  | Identified drainage structures, culverts and obstacles |  |  |  |  |  |
|  | Adjusted windrow to allow traffic to continue |  |  |  |  |  |
|  | Choose blade position, wheel lean, articulation, gear and speed |  |  |  |  |  |
|  | Reshape and recover materials for the road surface |  |  |  |  |  |
|  | Cut shoulders and move material to center or from one side to another |  |  |  |  |  |
|  | Identified required slope |  |  |  |  |  |
|  | Applied grade checking instruments |  |  |  |  |  |
|  | Choose blade position, wheel lean, articulation, gear and speed |  |  |  |  |  |
|  | Smooth the area at the base of the slope for smooth working platform |  |  |  |  |  |
|  | Started at the top of slope |  |  |  |  |  |
|  | Shaped the shoulder accurately |  |  |  |  |  |
|  | Identified the required profile using grade checking instruments |  |  |  |  |  |
|  | Choose blade position, wheel lean, articulation, gear and speed |  |  |  |  |  |
|  | Shaped ditch by repeated passes. |  |  |  |  |  |
|  | Choose blade position, wheel lean, articulation, gear and speed |  |  |  |  |  |
|  | Positioned grader with outer tires on pavement, and inner tires just off pavement on shoulder for left side shoulder |  |  |  |  |  |
|  | Positioned grader with inner tires on pavement, and outer tires just off pavement on shoulder for right side shoulder |  |  |  |  |  |
|  | Moved only enough material to pavement edge to dress the shoulder |  |  |  |  |  |
|  | Rolled the windrow back away from the pavement edge |  |  |  |  |  |
|  | Shaped the shoulder accurately |  |  |  |  |  |
|  | Choose blade position, wheel lean, articulation, gear and speed |  |  |  |  |  |
|  | Choose blade tilt, angel and position |  |  |  |  |  |
|  | Matched blade load to available power and traction |  |  |  |  |  |
|  | Removed unsuitable material |  |  |  |  |  |
|  | Cut and fill load bearing soils to create desired profile |  |  |  |  |  |
|  | Shaped for drainage and ditch as required |  |  |  |  |  |
|  | Identified the required profile using grade checking instruments |  |  |  |  |  |
|  | Get the correct volume in the efficient placement |  |  |  |  |  |
|  | Positioned for efficient spreading |  |  |  |  |  |
|  | Get correct volume of aggregates |  |  |  |  |  |
|  | Shifted the circle and blade towards the piles |  |  |  |  |  |
|  | Cut out windrows only as large as the machine can handle without tire spinning |  |  |  |  |  |
|  | Angled the blade as appropriate |  |  |  |  |  |
|  | Precise control to achieve elevations and shape to very accurate tolerances |  |  |  |  |  |
|  | Choose proper attachment, as chains, V-plow, wing plow, skid shoes and wing gates |  |  |  |  |  |
|  | Identified snow type, moisture content, density, weight, depth of snow, underlying surface, weather, visibility, traffic, obstacles and hidden structures |  |  |  |  |  |
|  | Mounted chain on tires carefully |  |  |  |  |  |
|  | Drive the machine in higher speed to move snow across and off the blade |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Construction Machinery (Level-4) Operation of Construction Machinery |
| --- | --- |
| Competency Standard | Operate Motor Grader |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define motor grader |  |  |
|  | Enlist types of motor grader |  |  |
|  | Enlist parts of motor grader |  |  |
|  | Enlist function of motor grader |  |  |
|  | Define bank cutting |  |  |
|  | Define mixing |  |  |
|  | Define spreading |  |  |
|  | Define ditching |  |  |
|  | Differentiate between rough and fine leveling |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________