Instruction Sheet for the Candidate

| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| --- | --- |
| Competency Standard | Adjust the Track chain tension |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Adjust the Track chain tension - Align the track chain |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Adjust the Track chain tension - Arrange Proper Tools for adjust the track chain tension - Check track chain tension - Pump grease through grease nipple with the grease pump.(if needed) - Check track tension; continue pump grease until tension is according to requirement. - Loose lubricator to release the grease if track is tight. - Test the machine for above adjustment by moving a machine forward and reverse direction. - Readjust the Track tension (if needed). Align the track chain - Arrange Proper Tools to Align the track chain - Inspect the deviation of track with respect to center of sprocket and rollers. - Adjust Idler Clearance in Sideways direction. - Test the machine for above adjustment by moving a machine forward and reverse direction. - Readjust the Idler Clearance (if Needed). |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| Competency Standard | Adjust the Track chain tension |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Adjust the Track chain tension - Align the track chain |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Arrange Proper Tools for adjust the track chain tension |  |  |
| - Check track chain tension |  |  |
| - Pump grease through grease nipple with the grease pump.(if needed) |  |  |
| - Check track tension; continue pump grease until tension is according to requirement. |  |  |
| - Loose lubricator to release the grease if track is tight. |  |  |
| - Test the machine for above adjustment by moving a machine forward and reverse direction. |  |  |
| - Readjust the Track tension (if needed). |  |  |
| - Arrange Proper Tools to Align the track chain |  |  |
| - Inspect the deviation of track with respect to center of sprocket and rollers. |  |  |
| - Adjust Idler Clearance in Sideways direction. |  |  |
| - Test the machine for above adjustment by moving a machine forward and reverse direction. |  |  |
| - Readjust the Idler Clearance (if Needed). |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| --- | --- |
| Competency Standard | Adjust the Track chain tension |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Adjust the Track chain tension - Align the track chain |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Arranged Proper Tools for adjust the track chain tension |  |  |  |  |  |
|  | Checked track chain tension |  |  |  |  |  |
|  | Pumped grease through grease nipple with the grease pump.(if needed) |  |  |  |  |  |
|  | Checked track tension; continue pump grease until tension is according to requirement. |  |  |  |  |  |
|  | Loosed lubricator to release the grease if track is tight. |  |  |  |  |  |
|  | Tested the machine for above adjustment by moving a machine forward and reverse direction. |  |  |  |  |  |
|  | Readjusted the Track tension (if needed). |  |  |  |  |  |
|  | Arranged Proper Tools to Align the track chain |  |  |  |  |  |
|  | Inspected the deviation of track with respect to center of sprocket and rollers. |  |  |  |  |  |
|  | Adjusted Idler Clearance in Sideways direction. |  |  |  |  |  |
|  | Tested the machine for above adjustment by moving a machine forward and reverse direction. |  |  |  |  |  |
|  | Readjusted the Idler Clearance (if Needed). |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| --- | --- |
| Competency Standard | Adjust the Track chain tension |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe how to adjust track chain tension. |  |  |
|  | Explain function of recoil spring assembly. |  |  |
|  | State components of Recoil spring assembly |  |  |
|  | Explain procedure to adjustment of track tension of track chain of Bulldozer |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________