Instruction Sheet for the Candidate

| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| --- | --- |
| Competency Standard | Disassemble and assemble the final drive |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Disassemble the final drive - Assemble the final Drive |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Disassemble the final drive - Arrange Proper tools to Disassemble the final drive - Drain oil of final drive. - Remove drive shaft and final drive assembly. - Dismantle Sprocket hub assembly with the help of SST. - Remove wear guard of sprocket assembly. - Remove Cover Assembly. - Remove Carrier Assembly. - Remove Planetary Gear set. - Remove hub and case assembly. - Remove Cage assembly. - Remove Gear Assembly. - Remove outer race and oil seal from case. Assemble the final Drive - Inspect, clean and replace faulty parts of final drive assembly - Press fit oil seal and outer race to case. - Install Gear Assembly. - Install Cage assembly. - Install hub and case assembly. - Install Planetary Gear set. - Install Carrier Assembly. - Install Cover Assembly. - Install wear guard of sprocket assembly - Assemble Sprocket hub assembly with the help of SST. - Install drive shaft and final drive assembly - Refill oil of final drive Assembly. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| Competency Standard | Disassemble and assemble the final drive |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Disassemble the final drive - Assemble the final Drive |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Arrange Proper tools to Disassemble the final drive |  |  |
| - Drain oil of final drive. |  |  |
| - Remove drive shaft and final drive assembly. |  |  |
| - Dismantle Sprocket hub assembly with the help of SST. |  |  |
| - Remove wear guard of sprocket assembly. |  |  |
| - Remove Cover Assembly. |  |  |
| - Remove Carrier Assembly. |  |  |
| - Remove Planetary Gear set. |  |  |
| - Remove hub and case assembly. |  |  |
| - Remove Cage assembly. |  |  |
| - Remove Gear Assembly. |  |  |
| - Remove outer race and oil seal from case. |  |  |
| - Inspect, clean and replace faulty parts of final drive assembly |  |  |
| - Press fit oil seal and outer race to case. |  |  |
| - Install Gear Assembly. |  |  |
| - Install Cage assembly. |  |  |
| - Install hub and case assembly. |  |  |
| - Install Planetary Gear set. |  |  |
| - Install Carrier Assembly. |  |  |
| - Install Cover Assembly. |  |  |
| - Install wear guard of sprocket assembly |  |  |
| - Assemble Sprocket hub assembly with the help of SST. |  |  |
| - Install drive shaft and final drive assembly |  |  |
| - Refill oil of final drive Assembly. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| --- | --- |
| Competency Standard | Disassemble and assemble the final drive |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Disassemble the final drive - Assemble the final Drive |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Arranged Proper tools to Disassemble the final drive |  |  |  |  |  |
|  | Drained oil of final drive. |  |  |  |  |  |
|  | Removed drive shaft and final drive assembly. |  |  |  |  |  |
|  | Dismantled Sprocket hub assembly with the help of SST. |  |  |  |  |  |
|  | Removed wear guard of sprocket assembly. |  |  |  |  |  |
|  | Removed Cover Assembly. |  |  |  |  |  |
|  | Removed Carrier Assembly. |  |  |  |  |  |
|  | Removed Planetary Gear set. |  |  |  |  |  |
|  | Removed hub and case assembly. |  |  |  |  |  |
|  | Removed Cage assembly. |  |  |  |  |  |
|  | Removed Gear Assembly. |  |  |  |  |  |
|  | Removed outer race and oil seal from case. |  |  |  |  |  |
|  | Inspected, cleaned and replaced faulty parts of final drive assembly |  |  |  |  |  |
|  | Pressed fit oil seal and outer race to case. |  |  |  |  |  |
|  | Installed Gear Assembly. |  |  |  |  |  |
|  | Installed Cage assembly. |  |  |  |  |  |
|  | Installed hub and case assembly. |  |  |  |  |  |
|  | Installed Planetary Gear set. |  |  |  |  |  |
|  | Installed Carrier Assembly. |  |  |  |  |  |
|  | Installed Cover Assembly. |  |  |  |  |  |
|  | Installed wear guard of sprocket assembly |  |  |  |  |  |
|  | Assembled Sprocket hub assembly with the help of SST. |  |  |  |  |  |
|  | Installed drive shaft and final drive assembly |  |  |  |  |  |
|  | Refilled oil of final drive Assembly. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| --- | --- |
| Competency Standard | Disassemble and assemble the final drive |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define function of Final Drive system |  |  |
|  | Define Single reduction Final Drive |  |  |
|  | Define Double reduction Final Drive |  |  |
|  | Define planetary type Final Drive |  |  |
|  | What is the disassembling procedure of final drive |  |  |
|  | What is the assembling procedure of final drive |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________