Instruction Sheet for the Candidate

| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| --- | --- |
| Competency Standard | Disassemble & assemble inter-axle differential assembly |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Disassemble inter-axle differential assembly - Assemble inter-axle differential assembly |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Disassemble inter-axle differential assembly - Arrange Proper tools to Disassemble inter-axle differential assembly - Drain oil from differential and final Drive Case. - Dismantle Wheel Assembly. - Pull out Drive shaft - Remove inter-axle differential assembly. - Remove Coupling - Remove Bevel Pinion and cage assembly. - Remove Differential gear Assembly from Differential case. - Dismantle the differential gear assembly. Assemble inter-axle differential assembly - Inspect, clean and replace faulty parts of inter-axle differential assembly - Assemble the differential gear assembly. - Install Differential gear Assembly on Differential case. - Adjust preload on bearings. - Install Bevel Pinion and cage assembly. - Install Coupling of differential. - Install inter axle differential assembly on machine. - Refit Drive shaft - Assemble Wheel Assembly. - Refill oil in differential and final Drive Case. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| Competency Standard | Disassemble & assemble inter-axle differential assembly |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Disassemble inter-axle differential assembly - Assemble inter-axle differential assembly |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Arrange Proper tools to Disassemble inter-axle differential assembly |  |  |
| - Drain oil from differential and final Drive Case. |  |  |
| - Dismantle Wheel Assembly. |  |  |
| - Pull out Drive shaft |  |  |
| - Remove inter-axle differential assembly. |  |  |
| - Remove Coupling |  |  |
| - Remove Bevel Pinion and cage assembly. |  |  |
| - Remove Differential gear Assembly from Differential case. |  |  |
| - Dismantle the differential gear assembly. |  |  |
| - Inspect, clean and replace faulty parts of inter-axle differential assembly |  |  |
| - Assemble the differential gear assembly. |  |  |
| - Install Differential gear Assembly on Differential case. |  |  |
| - Adjust preload on bearings. |  |  |
| - Install Bevel Pinion and cage assembly. |  |  |
| - Install Coupling of differential. |  |  |
| - Install inter axle differential assembly on machine. |  |  |
| - Refit Drive shaft |  |  |
| - Assemble Wheel Assembly. |  |  |
| - Refill oil in differential and final Drive Case. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| --- | --- |
| Competency Standard | Disassemble & assemble inter-axle differential assembly |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Disassemble inter-axle differential assembly - Assemble inter-axle differential assembly |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Arranged Proper tools to Disassemble inter-axle differential assembly |  |  |  |  |  |
|  | Drained oil from differential and final Drive Case. |  |  |  |  |  |
|  | Dismantled Wheel Assembly. |  |  |  |  |  |
|  | Pulled out Drive shaft |  |  |  |  |  |
|  | Removed inter-axle differential assembly. |  |  |  |  |  |
|  | Removed Coupling |  |  |  |  |  |
|  | Removed Bevel Pinion and cage assembly. |  |  |  |  |  |
|  | Removed Differential gear Assembly from Differential case. |  |  |  |  |  |
|  | Dismantled the differential gear assembly. |  |  |  |  |  |
|  | Inspected, cleaned and replaced faulty parts of inter-axle differential assembly |  |  |  |  |  |
|  | Assembled the differential gear assembly. |  |  |  |  |  |
|  | Installed Differential gear Assembly on Differential case. |  |  |  |  |  |
|  | Adjusted preload on bearings. |  |  |  |  |  |
|  | Installed Bevel Pinion and cage assembly. |  |  |  |  |  |
|  | Installed Coupling of differential. |  |  |  |  |  |
|  | Installed inter axle differential assembly on machine. |  |  |  |  |  |
|  | Refitted Drive shaft |  |  |  |  |  |
|  | Assembled Wheel Assembly. |  |  |  |  |  |
|  | Refilled oil in differential and final Drive Case. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | construction Machinery (Level-5) Final Drive Undercarriage and Power Line |
| --- | --- |
| Competency Standard | Disassemble & assemble inter-axle differential assembly |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Explain inter axle differential assembly |  |  |
|  | Explain structure of inter axle differential assembly |  |  |
|  | Explain function of inter axle differential assembly |  |  |
|  | Explain testing of inter axle differential assembly |  |  |
|  | Explain adjustment of inter axle differential assembly |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________