Instruction Sheet for the Candidate

| Qualification | Construction Machinery (Level-5) Steering and Brake System of Construction Machinery |
| --- | --- |
| Competency Standard | Test and adjust of steering system of wheel loader. |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Measure steering wheel play - Test and adjust the free play of steering wheel of wheel loader. - Measure operating force of steering wheel. - Measure operating time of steering wheel - Measure steering relief pressure. - Adjust steering relief pressure |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Measure steering wheel play - Confirm the neutral position of steering wheel put mark 1 on the wheel. - Turn steering wheel clock wise up till tires on road begin to turn put mark 2 on steering wheel. - Turn steering wheel anti clock wise up till tires on road begin to turn put mark 3 on steering wheel. - Measure the straight distance between mark 2 and 3. Test and adjust the free play of steering wheel of wheel loader. - Inspect the steering linkages for looseness and free play. - Loose lock nut of steering gear box. - Adjust by turning adjustment screw in or out. Measure operating force of steering wheel. - Hook push –pull scale to the knob of steering wheel. - Raise the bucket 400mm and remove safety bar. - Pull push-pull scale tangentially to steering wheel. - Read the value when steering wheel is moving smoothly. Measure operating time of steering wheel - Start the engine and raise bucket about 400mm and remove the safety bar. - Turn steering wheel up till machine is turned fully right or left at low idling. - Turn steering wheel up till machine is turned fully right or left at high idling. - Check steering oil pressure. Measure steering relief pressure. - Fit safety bar on the frame. - Remove plug from takeoff port of right turn of steering circuit. - Install pressure gauge. - Start the engine and run it at high idling. - Turn the steering wheel to the right and measure pressure when relief valve is actuated - Turn the steering wheel to the left and measure pressure when relief valve is actuated Adjust steering relief pressure - Stop the engine. - Remove cap nut of relief valve. - Loose lock nut. - Turn adjustment screw to adjust. - Tighten screw to increase pressure - Loosen screw to decrease pressure |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Construction Machinery (Level-5) Steering and Brake System of Construction Machinery |
| Competency Standard | Test and adjust of steering system of wheel loader. |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Measure steering wheel play - Test and adjust the free play of steering wheel of wheel loader. - Measure operating force of steering wheel. - Measure operating time of steering wheel - Measure steering relief pressure. - Adjust steering relief pressure |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Confirm the neutral position of steering wheel put mark 1 on the wheel. |  |  |
| - Turn steering wheel clock wise up till tires on road begin to turn put mark 2 on steering wheel. |  |  |
| - Turn steering wheel anti clock wise up till tires on road begin to turn put mark 3 on steering wheel. |  |  |
| - Measure the straight distance between mark 2 and 3. |  |  |
| - Inspect the steering linkages for looseness and free play. |  |  |
| - Loose lock nut of steering gear box. |  |  |
| - Adjust by turning adjustment screw in or out. |  |  |
| - Hook push –pull scale to the knob of steering wheel. |  |  |
| - Raise the bucket 400mm and remove safety bar. |  |  |
| - Pull push-pull scale tangentially to steering wheel. |  |  |
| - Read the value when steering wheel is moving smoothly. |  |  |
| - Start the engine and raise bucket about 400mm and remove the safety bar. |  |  |
| - Turn steering wheel up till machine is turned fully right or left at low idling. |  |  |
| - Turn steering wheel up till machine is turned fully right or left at high idling. |  |  |
| - Check steering oil pressure. |  |  |
| - Fit safety bar on the frame. |  |  |
| - Remove plug from takeoff port of right turn of steering circuit. |  |  |
| - Install pressure gauge. |  |  |
| - Start the engine and run it at high idling. |  |  |
| - Turn the steering wheel to the right and measure pressure when relief valve is actuated |  |  |
| - Turn the steering wheel to the left and measure pressure when relief valve is actuated |  |  |
| - Stop the engine. |  |  |
| - Remove cap nut of relief valve. |  |  |
| - Loose lock nut. |  |  |
| - Turn adjustment screw to adjust. |  |  |
| - Tighten screw to increase pressure |  |  |
| - Loosen screw to decrease pressure |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Construction Machinery (Level-5) Steering and Brake System of Construction Machinery |
| --- | --- |
| Competency Standard | Test and adjust of steering system of wheel loader. |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Measure steering wheel play - Test and adjust the free play of steering wheel of wheel loader. - Measure operating force of steering wheel. - Measure operating time of steering wheel - Measure steering relief pressure. - Adjust steering relief pressure |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Confirmed the neutral position of steering wheel put mark 1 on the wheel. |  |  |  |  |  |
|  | Turned steering wheel clock wise up till tires on road begin to turn put mark 2 on steering wheel. |  |  |  |  |  |
|  | Turned steering wheel anti clock wise up till tires on road begin to turn put mark 3 on steering wheel. |  |  |  |  |  |
|  | Measured the straight distance between mark 2 and 3. |  |  |  |  |  |
|  | Inspected the steering linkages for looseness and free play. |  |  |  |  |  |
|  | Loosed lock nut of steering gear box. |  |  |  |  |  |
|  | Adjusted by turning adjustment screw in or out. |  |  |  |  |  |
|  | Hooked push –pull scale to the knob of steering wheel. |  |  |  |  |  |
|  | Raised the bucket 400mm and remove safety bar. |  |  |  |  |  |
|  | Pulled push-pull scale tangentially to steering wheel. |  |  |  |  |  |
|  | Read the value when steering wheel is moving smoothly. |  |  |  |  |  |
|  | Started the engine and raise bucket about 400mm and remove the safety bar. |  |  |  |  |  |
|  | Turned steering wheel up till machine is turned fully right or left at low idling. |  |  |  |  |  |
|  | Turned steering wheel up till machine is turned fully right or left at high idling. |  |  |  |  |  |
|  | Checked steering oil pressure. |  |  |  |  |  |
|  | Fitted safety bar on the frame. |  |  |  |  |  |
|  | Removed plug from takeoff port of right turn of steering circuit. |  |  |  |  |  |
|  | Installed pressure gauge. |  |  |  |  |  |
|  | Started the engine and run it at high idling. |  |  |  |  |  |
|  | Turned the steering wheel to the right and measure pressure when relief valve is actuated |  |  |  |  |  |
|  | Turned the steering wheel to the left and measure pressure when relief valve is actuated |  |  |  |  |  |
|  | Stopped the engine. |  |  |  |  |  |
|  | Removed cap nut of relief valve. |  |  |  |  |  |
|  | Loosed lock nut. |  |  |  |  |  |
|  | Turned adjustment screw to adjust. |  |  |  |  |  |
|  | Tightened screw to increase pressure |  |  |  |  |  |
|  | Loosed screw to decrease pressure |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Construction Machinery (Level-5) Steering and Brake System of Construction Machinery |
| --- | --- |
| Competency Standard | Test and adjust of steering system of wheel loader. |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe the testing procedure of steering wheel play of wheel loader |  |  |
|  | Describe the adjusting procedure of steering wheel play of wheel loader |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________