Instruction Sheet for the Candidate

| Qualification | Construction Machinery (Level-4) Work Shop Practice - II (Level-4) |
| --- | --- |
| Competency Standard | - Carry out operation of Head surface grinding machine - Carry out operation of Honing Machine - Carry out operation of Main line boring machine - Carry out operation of Connecting Rod Boring Machine - Carry out operation of Crankshaft grinding machine |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name___________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Select appropriate tools and measuring devices - Carry out different machining operation (Annexure-I) |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Operate Honing Machine - Centring of sleeve - Select grinding/Honing stone according to dia of sleeve - Fit the grinding/Honing stone in machine - Switch on lubrication oil circulation - Operate Tool UP/Down - Operate tool in circular motion Operate Main line boring machine - Fasten the Cylinder Block to the machine - Fix & center the cutting tool - Set the cutting depth - Set the desired speed - Switch on the machine Operate Crankshaft grinding machine - Balancing & setting of crankshafts on the machine - Fix grinding wheel & set the distance. - Fix the sand paper for polishing |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Construction Machinery (Level-4) Work Shop Practice - II (Level-4) |
| Competency Standard | - Carry out operation of Head surface grinding machine - Carry out operation of Honing Machine - Carry out operation of Main line boring machine - Carry out operation of Connecting Rod Boring Machine - Carry out operation of Crankshaft grinding machine |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | - Carry out different machining operation |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Centring of sleeve |  |  |
| - Select grinding/Honing stone according to dia of sleeve |  |  |
| - Fit the grinding/Honing stone in machine |  |  |
| - Switch on lubrication oil circulation |  |  |
| - Operate Tool UP/Down |  |  |
| - Operate tool in circular motion |  |  |
| - Fasten the Cylinder Block to the machine |  |  |
| - Fix & center the cutting tool |  |  |
| - Set the cutting depth |  |  |
| - Set the desired speed |  |  |
| - Switch on the machine |  |  |
| - Balancing & setting of crankshafts on the machine |  |  |
| - Fix grinding wheel & set the distance. |  |  |
| - Fix the sand paper for polishing |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Construction Machinery (Level-4) Work Shop Practice - II (Level-4) |
| --- | --- |
| Competency Standard | - Carry out operation of Head surface grinding machine - Carry out operation of Honing Machine - Carry out operation of Main line boring machine - Carry out operation of Connecting Rod Boring Machine - Carry out operation of Crankshaft grinding machine |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Carry out different machining operation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Centred of sleeve |  |  |  |  |  |
|  | Selected grinding/Honing stone according to diameter of sleeve |  |  |  |  |  |
|  | Fitted the grinding/Honing stone in machine |  |  |  |  |  |
|  | Switched on lubrication oil circulation |  |  |  |  |  |
|  | Operated Tool UP/Down |  |  |  |  |  |
|  | Operated tool in circular motion |  |  |  |  |  |
|  | Fastened the Cylinder Block to the machine |  |  |  |  |  |
|  | Fixed & center the cutting tool |  |  |  |  |  |
|  | Set the cutting depth |  |  |  |  |  |
|  | Set the desired speed |  |  |  |  |  |
|  | Switched on the machine |  |  |  |  |  |
|  | Balancing & setting of crankshafts on the machine |  |  |  |  |  |
|  | Fixed grinding wheel & set the distance. |  |  |  |  |  |
|  | Fixed the sand paper for polishing |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Construction Machinery (Level-4) Work Shop Practice - II (Level-4) |
| --- | --- |
| Competency Standard | - Carry out operation of Head surface grinding machine - Carry out operation of Honing Machine - Carry out operation of Main line boring machine - Carry out operation of Connecting Rod Boring Machine - Carry out operation of Crankshaft grinding machine |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define Honing |  |  |
|  | Define grinding |  |  |
|  | Define boring |  |  |
|  | Define ovality |  |  |
|  | Differentiate between operations of connecting rod boring machine, crank shaft grinding machine. |  |  |
|  | Enlist 4 uses of gas analyzers. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________

(Annexure-I)

Practical Activity: Perform the following two activities;

- Select appropriate gauge and note 3 readings, measure the inner diameter and ovality of engine liner.

- De-mount a cylinder head and perform surface grinding of cylinder head using head surface grinding machine. Perform surface precisely abrasiveness of inner surface of cylinder using honing machine. Note before and after readings for both machining operations