National Certificate (Level-2) in Mechanical Technology with Specialization in Construction Machinery

Level-2 Equivalent to G-III

Competency Standards

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

Contents

1.	Introduction	4

2.	Purpose of the qualification	5

3.	Summary of Competency Standards	5

4.	Date of validation	6

5.	Codes of Qualifications	6

6.	Qualifications Development Committee	7

7.	Qualifications Validation Committee	8

8.	Entry Requirements	8

9.	Packaging of Qualifications	9

10.	Detail of Competency Standards	11

National Certificate Level-2 in Construction Machinery………………………………….11

10.1. Workshop Practice-I	11

10.1.1	Maintain Tools & Equipment.	11

10.1.2	Apply technique of Hand and Power Tools	13

10.1.3	Perform Metal/Bench Work	15

10.1.4	Perform cutting on Metal using Circular/Power Heck Saw	17

10.1.5	Perform grinding operations	18

10.1.6	Perform Tool & Cutter Grinding	21

10.1.7	Perform Lathe Machine Operations	23

10.1.8	Perform Shaper Machining Operations	27

10.1.9	Perform CNC Lathe Operations	29

10.1.10	Perform Pre-Welding Operations	32

10.1.11	Perform Shielded Metal Arc Welding (SMAW)/electric welding	35

10.1.12	Perform different welding joints	38

10.1.13	Perform Forging Operations	40

10.2. Basic Engineering Drawing and CAD/CAM	42

10.2.1	Perform Basic Manual Drawing	42

10.2.2	Construct Different Engineering Curves	46

10.2.3	Construct Multi-view drawings	50

10.2.4	Install CAD software.	53

10.2.5	Manage display in CAD software	55

10.2.6	Perform function of CAD general commands	57

10.2.7	Perform Different Cad operations	60

10.3	. Occupational Health, Safety and Environment	63

10.3.1	Read and Develop Career Professionalism	63

10.3.2	Apply basic Occupational Health & Safety regarding heavy machinery	65

10.3.3	Perform general health, safety and environment practices	67

11.	List of Machinery, Tools & Equipment.	71

11.1. Workshop Practice-…...……………………………………………………………………………..71

11.2. Basic Engineering Drawing and CAD/CAM……………………………………………………….72

11.3. Occupational Health, Safety and Environment……………………………………………………72

# Introduction

Heavy Machine Operators and Technicians are in demand across the country and abroad. Their services are required for everything from road and bridge construction, back hoe operation, bulldozing, loading and grading, dredging and heavy paving, to excavating and much, much more. This is a good career opportunity for a reliable and responsible individual with a strong work ethic. Heavy Machine Operators and Technicians not only work on regular construction building jobs, but also on infrastructure projects (roads, bridges, Dams and ports, otherwise called non-building construction), and in mining and timber operations.

In order to build the capacity of technical and vocational training institutes in Pakistan through provision of demand driven competencies-based trainings in construction sector the NAVTTC, and TEVT Sector Support Program (TSSP) have joined hands together to develop qualifications for construction sector. These qualifications will not only build the capacity of existing workers of this sector but also support the youth to acquire skills best fit for this sector. The benefits and impact of development of these qualifications will be on both demand and supply side.

Based upon this demand of industry these competency‐based qualifications for Heavy Construction Machinery technology are developed under National Vocational Qualification Framework (NVQF) (Level 5).  The qualifications mainly cover competencies along with related knowledge and professional skills which are essential for getting a job or self-employed.

The National Vocational & Technical Training Commission (NAVTTC) has approved the Qualification Development Committee (QDC). The QDC consists experts from the relevant industries from different geographical locations across Pakistan and academicians who were consulted during the development process to ensure input and ownership of all the stakeholders. The National Competency Standards could be used as a referral document for the development of curricula to be used by training institutions.

Heavy Machine Operator and Technician curriculum will prepare students to efficiently operate heavy machines such as Dozers, Loaders, Excavator and Graders and to perform basic preventive maintenance on most types of heavy equipment. Coursework includes Safety, Heavy Machines operations, grades, Legal & environmental concerns & equipment maintenance. Graduates of this program may find employment with state and local government agencies and private contractors engaged in highway or other construction activities.

In fact, no sphere of life may be identified that does not include the contribution of construction machinery. Thus, the importance of construction machinery may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechanical technology with Specialization in Construction Machinery under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# Purpose of the qualification

The purpose of these qualifications is to set high professional standards for Construction Machinery. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for Construction Machinery.

- Enable the existing workforce to capacitate themselves in new technologies and method so that train persons in this field can maintain construction machinery for its longer life.

# Summary of Competency Standards

The Construction Technology qualifications of level 25 consists 20% Theory and 80% Practical. The Core competencies of the qualification are as follows:

Level-2

Workshop Practice-1

| Code | Competency Standards | Level | TH | PR | Total | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-01 | Maintain Tools & Equipment. | 3 | 8 | 22 | 30 | 3 | Technical |
| CS-02 | Apply technique of Hand and Power Tools | 3 | 8 | 22 | 30 | 3 | Technical |
| CS-03 | Perform Metal/Bench Work | 3 | 6 | 34 | 40 | 4 | Technical |
| CS-04 | Perform cutting on Metal Circular/Power Hack Saw | 3 | 4 | 26 | 30 | 3 | Technical |
| CS-05 | Perform Grinding Operations | 3 | 4 | 16 | 20 | 2 | Technical |
| CS-06 | Perform Tool & Cutter Grinding | 5 | 4 | 16 | 20 | 2 | Technical |
| CS-07 | Perform Lathe Machine Operations | 3 | 10 | 50 | 60 | 6 | Technical |
| CS-08 | Perform Shaper Machining Operations | 3 | 6 | 34 | 40 | 4 | Technical |
| CS-09 | Perform CNC Lathe Operations | 5 | 8 | 32 | 40 | 4 | Technical |
| CS-10 | Perform Pre-Welding Operations | 3 | 8 | 22 | 30 | 3 | Technical |
| CS-11 | Perform Shielded Metal Arc Welding (SMAW)/Electric welding | 3 | 8 | 22 | 30 | 3 | Technical |
| CS-12 | Perform different welding joints | 3 | 8 | 22 | 30 | 3 | Technical |
| CS-13 | Perform Forging Operations | 3 | 4 | 16 | 20 | 2 | Technical |

Engineering Drawing and CAD-I

| Code | Competency Standards | Level | TH | PR | Total | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-14 | Perform Basic Manual Drawing | 3 | 8 | 22 | 30 | 3 | Allied |
| CS-15 | Construct different Engineering Curves | 5 | 8 | 22 | 30 | 3 | Allied |
| CS-16 | Construct multi-view drawings | 5 | 6 | 34 | 40 | 4 | Allied |
| CS-17 | Install CAD software | 3 | 4 | 16 | 20 | 2 | Allied |
| CS-18 | Manage display in CAD software | 5 | 4 | 16 | 20 | 2 | Allied |
| CS-19 | Perform function of CAD general commands | 5 | 8 | 32 | 40 | 4 | Allied |
| CS-20 | Perform Different CAD operations | 5 | 8 | 32 | 40 | 4 | Allied |

Generic

| Code | Competency Standards | Level |  | TH | PR | Total | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS-21 | Read and Develop Career Professionalism |  | 1 | 2 | 8 | 10 | 1 | Generic |
| CS-22 | Apply basic Occupational Health & Safety regarding heavy machinery |  | 1 | 2 | 8 | 10 | 1 | Generic |
| CS-23 | Perform general health, safety and environment practices |  | 1 | 2 | 8 | 10 | 1 | Generic |
|  | Total |  |  | 138 | 532 | 670 | 67 |  |

# Date of validation

The level-2 of National qualification on Construction Technology has been validated by the Qualifications Development Committee (QDC) members on ………… and will remain in currency until …………………………

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Construction Technology level-5 |  |
| --- | --- |
| Code | Description |
| 000000000 | National Certificate of level-2, in “Construction Technology” |
| 000000000 | National Certificate of level-3, in “Construction Technology” |
| 000000000 | National Certificate of level-4, in “Construction Technology” |
| 000000000 | National Certificate of level-5, in “Construction Technology” |

# Qualifications Development Committee

The following members participated in the qualification’s development workshop 18th March 2019

To 22nd March 2019 at Grand Palm Hotel, Lahore:

| S.No. | Name & Designation | Organization |
| --- | --- | --- |
|  | Muhammad Ishaq, Dy Director (TE) | NAVTTC, HQ |
|  | Muzammal Hussain, Instructor (Auto & Diesel) | GCT, Rawalpindi, TEVTA |
|  | Muhammad Ejaz, HOD, Mechanical Department | C.T.T.I, Islamabad |
|  | Hassan Raza, Jr. Instructor | C.T.T.I, Islamabad |
|  | Umer Farooq, Instructor, Mechanical | G.S.P.C.T Gujrat |
|  | Muhammad Rizwan Sarwar, Sr. Instructor (Auto Mechanic) | G.T.T.I, Shiekhupura |
|  | Hafiz Ghulam Mohyudin, Asst. Manager Services | Dynamic Equipment & Control, Lahore |
|  | Mehr Ali Qureshi, Sr. Service Engineer | Murshid Builders (PVT) Limited, Lahore |
|  | Syed Hussein-el-Edroos, Sr. Service Engineer | Independent Consultant |
|  | Waqar Ahmed, Service Manager | Jaffer Brothers (PVT) Limited, Lahore |
|  | Ghulam Dastgir, Incharge Projects | Ex-DESCON Engineering, Lahore |
|  | Zain-ul-Abideen, Instructor | G.T.T.I, Mughalpura, Lahore |
|  | Atif Waheed Abbasi, Heavy Machine Operator | National Logistic Cell Mandra, Islamabad |
|  | Sikander Mehmood Jr. Instructor | C.T.T.I, Islamabad |
|  | Engr. Mohsin Jahanzeb Instructor | GSTC,Faisalabad |
|  | Muhammad Amjad Iqbal Service Engineer | Greaves Pakistan Ltd,Lahore |
|  | Sayed Fayyaz Mustafa Naqvi J. Instructor Civil | GSTC,Faisalabad |
|  | Saba Sadiq, MS Mechanical Engineer/ DACUM Facilitator, | University of Lahore, Lahore |

# Qualifications Validation Committee

The following members participated in the qualification development and validation workshop on:

| S.No. | Name & Designation | Organization |
| --- | --- | --- |
|  | Mr. Muhammad Ejaz | HOD, CTTI, Islamabad |
|  | Mr. Hassan Raza | Instructor, CTTI, Islamabad |
|  | Mr. Muzammal Hussain | Instructor (Auto & Diesel) , GCT, Rawalpindi |
|  | Mr. Sikandar Masood | Director, NAVTTC, Islamabad |
|  | Mr. Abdur Rehman | Representative from PBTE, Lahore |
|  | Mr. Zafar Iqbal | Representative from KP-TEVTA |
|  | Mr. Rafique Ahmed Shaikh | Representative from Singh-TEVTA |
|  | Mr. Naveed Ahmed Ajmal | Representative from P-TEVTA |
|  | Mr. Irfan Khalil | Representative from Sector Skill Council/ ABAD Karachi |
|  | Saba Sadiq | DACUM, Facilitator, Lahore (UOL) |

# Entry Requirements

The entry for National Certificate level-2, in Construction Technology are

- A person having Matric certificate with Science subjects

# Packaging of Qualifications

The National Vocational Qualifications have been packaged as detailed below:

| Level | Name of Occupation/Nomenclature | Generic Competencies | Allied competencies require | Core /Technical Competencies required |
| --- | --- | --- | --- | --- |
| - Diesel Engine Technician |  |  |  |  |
| Level-2 | Pro Diesel Mechanic | 150,151,152,153,154 |  | 1,2,3,4,5,6,7 |
| Level-3 | Engine Rebuilder | 150,151,152,153,154 |  | 44,45,46,47,48,49,50,51,52 |
| Level-3 | Generator Technician | 150,151,152,153,154 |  | 61,62,63,64,65,66 |
| Level-3 | Engine Technician | 149,150,151,152,153,154 |  | 53,54,55,56,57,58,59,60 |
| Level-3 | Pro Technician Diesel Pump | 149,150,151,152,153,154 |  | 67,68,69,70,71,72 |
| Level-4 | Diesel Pump Technician | 140,141,142,143,144,145,146,147,148, 149,150,151,152,153,154 |  | 128,129,130,131,132,133 |
| Level-4 | CNC Machine operator | 150,151,152,153,154 |  | 15,16,17,18,19,20,21,22,23,24,25,26,27 |
| Level-4 | Thermo Man | 150,151,152,153,154 |  | 39,40,41,42,43 |
| - Electrician |  |  |  |  |
| Level-2 | Auto Electrician | 150,151,152,153,154 |  | 28,29,30,31,32,33,34,35,36,37,38 |
| - Engineering Drawing |  |  |  |  |
| Level-3 | Draft Man | 152,153,154 |  | 8,9,10,11,12,13,14 |
| - Heavy Machine Operator |  |  |  |  |
| Level-3 | Excavator Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,78 |
| Level-3 | Bulldozer Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,80 |
| Level-3 | Dump truck Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,79 |
| Level-3 | Wheel Loader Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,81 |
| Level-3 | Wheel Mounted Loading Crane Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,83 |
| Level-4 | Master operator (Excavator, Bulldozer, Dump truck, Wheel Loader, Grader, Crane and asphalt paver) | 149,150,151,152,153,154,155 |  | 73,74,75,76,77,78,79,80,81,82,83,84,85 |
| - Heavy Machinery Mechanic |  |  |  |  |
| Level-5 | Construction Machinery Transmission Technician | 149, 150,151,152,153,154 |  | 86,87,88,89,90,91,92,93 |
| Level-5 | Steering and Brake Mechanic of Construction Machinery | 149, 150,151,152,153,154 |  | 94,95,96,97,98,99,100,101,102,103 |
| level-5 | Under Carriage Mechanic of Construction Machinery | 149, 150,151,152,153,154 |  | 104,105,106,107,108,109,110 |
| Level-5 | Hydraulic Mechanic of Construction Machinery | 149, 150,151,152,153,154 |  | 111,112,113,114, 115,116,117,118 |
| Level-5 | Hydraulic Excavator Mechanic of Construction Machinery | 140,141,142,143,144,145,146,147,148,149, 150,151,152,153,154 |  | 119,120,121,122,123,124,125,126,127 |
| Level-5 | Construction field Manager | 140,141,142,143,144,145,146,147,148,149, 150,151,152,153,154 |  | 134,135,136,137,138,139 |

# Detail of Competency Standards

## National Certificate Level-2 in Construction Machinery

# Workshop Practice-I

    - Maintain Tools & Equipment.

Overview: This competency standard covers the skills and knowledge required to Arrange Tools and Equipment, Maintain Tool Box, Insulate Tools and Equipment Calibrate measuring tools and Manage Inventory of tools and equipment.

| Competency Units | Performance Criteria |
| --- | --- |
| - Arrange Tools and Equipment | - Identify tools and equipment - Interpret job card - Prepare list of tools and equipment as per requirement - Collect tools and equipment from store. - Identify insulated tools and equipment |
| - Maintain Toolbox | - Check physical conditions of tools and equipment before use - Perform preventive maintenance as per standards - Perform corrective maintenance of tools as per requirements. - Clean tools and equipment after use - Place tools and equipment at appropriate place. |
| - Calibrate measuring tools | - Check calibration status of the measuring tools - Perform calibration of measuring tools as per standards - Record calibration test results |
| - Manage Inventory of tools and equipment | - Check tools and equipment as per record - Report for faulty tools and equipment to supervisor - Generate demand for deficit tools and equipment - Maintain all records of tools and Equipment |

Knowledge & Understanding

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Define Machine shop tool their working and safety procedure during work.

- Define Marking tools.

- Describe the safety procedure using marking tools.

- Describe different type of measuring tools.

- Describe Proper Usage of Measuring Tools

- Describe maintenance and its types.

- Describe the safety procedure during maintenance.

Tools and Equipment

Measuring Tool

Marking Tool

Calibrating tool

Cutting Tool

Holding Devices

Tool boxes

Bench vice,

Hammer,

Tri-square,

Hand hacksaw,

    - Apply technique of Hand and Power Tools

Overview: This competency standard covers the skills and knowledge required to determine job requirements, sequence of operations and apply technique of hand and power tool.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Use of hand tools | - Select hand tools appropriate to the task requirements. - Use hand tools to produce desired outcomes to job specifications which may include finish, tension, size or shape. - Adhere all safety requirements before, during and after use. - Identify and mark unsafe or faulty tools for repair according to designated procedures before, during and after use. - Maintain tools, including hand sharpening according to standard operational procedures, principles and techniques. - Store hand tools safely in appropriate location according to standard operational procedures and manufacturers’ recommendations. |
| CU2. Use of power tools | - Select power tools appropriate to the task requirements. - Use power tools for a determined sequence of operations-which may include clamping, alignment and adjustment to produce desired outcomes-to job specifications which may include finish, size or shape. - Adhere all safety requirements to before, during and after use. - Identify and mark unsafe or faulty tools for repair according to designated procedures before, during and after use. - Maintain tools, including hand sharpening according to standard operational procedures, principles and techniques. - Store power tools safely in appropriate location according to standard operational procedures and manufacturers’ recommendations. |

Knowledge & Understanding

- Applications of different hand and power tools.

- Common faults and/or defects in hand and power tools

- Procedures for marking unsafe or faulty tools for repair

- Routine maintenance requirements for a range of hand and power tools

- Storage location and procedures for a range of hand and power tools

- Hazards and control measures associated with using hand and power tools

- Benefits and limits of cutting and shaping metal with auxiliary equipment

- Environmental benefits of maintaining auxiliary equipment

- Clamping /securing methods

- Adjustment/alignments to a range of power tools

- Tool sharpening techniques for a range of power tools

Tools and Equipment

Work bench, Bench vice, Hammer, Tri-square, Hand hacksaw, Scriber, Vernier caliper, Flat File, Number/alphabet punch

Work bench, Bench vice, Hammer, Tri-square, Hand hacksaw, Scriber, Vernier caliper, Flat File, Number/alphabet punch

    - Perform Metal/Bench Work

Overview: Overview: This competency standard covers the skills and knowledge required to Develop Name Plate manually, Carry out Sawing, Prepare Inside Calliper, Prepare Bottle Opener, Prepare Dovetail Joint, Prepare Tri Square (small size), Cut Threads on Work Piece, Prepare Funnel, Prepare Drawer Handle, Cut Pipe Threads and Prepare spanner (small size).

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Develop Name Plate manually | - Select marking tools - Hold the sheet in vice. - Cut sheet as per drawing - Perform surface finishing with file - Level the surface with tri-square - Mark the plate as per name requirements - Punch the marked area - Perform finishing with sand paper |
| CU2. Prepare Dovetail Joint | P1. Select marking tools P2. Cut sheet as per drawing P3. Perform surface finishing with file P4. Level the surface of both work pieces with tri-square P5. Mark both work pieces according to drawing P6. Create outer notch on work piece using flat file and hacksaw P7. Create inner notch using hacksaw and chisel P8. Compare both pieces by inserting outer notch into inner notch. P9. Perform finishing with sandpaper |
| CU3. Cut Threads on Work Piece with tap and die | P1. Identify different kind of taps & die according to requirement P2. Identify the work piece clamping method. P3. Apply tap and die alignment. P4. Apply lubricants while threading. P5. Avoid unwanted engraving and slips. P6. Interpret proper threading procedure |
| CU4. Cut Pipe Threads | P1. Select marking tools P2. Cut pipe as per drawing P3. Select die as per pipe size P5. Set die into die holder P6. Select relevant vice for pipe clamping P7. Perform pipe threading using appropriate method P8. Perform finishing with sand paper |
| CU5. Prepare Funnel | P1. Select marking tools P2. Cut sheet as per drawing P3. Perform surface finishing with file P5. Mark the sheet according to drawing P6. Cut the sheet with hand shear P7. Create radius of funnel using appropriate tools P8. Perform flat lock seam bend using bench vice P9. Perform finishing with sand paper. |

Knowledge & Understanding

- Applications of different hand tools.

- Common faults and/or defects in hand tools.

- Describe cutting tools.

Tools and Equipment

Punching tools, Work bench, Bench vice, Pipe vice, Tri-square, Vernier calliper, Hammer, Die handle, Lubricant, Scriber, Flat File, Round file, Hand shear, Hand Hacksaw

    - Perform cutting on Metal using Circular/Power Heck Saw

Overview: This competency standard covers the skills and knowledge required to Prepare the job for Sawing and Carry out Sawing at differet angles.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare the job for Sawing and Carry out Sawing operation | P1. Mark the job according to given drawing P2. Select appropriate blade according to job requirement P3. Set blade in frame of hacksaw as per procedure P4. Ensure the work piece is clamped firmly and properly P5. Adopt methods and techniques for sawing that is appropriate to job requirement. P6. Follow marked line during sawing to ensure accuracy. |
| CU2. Carry out Sawing at differet angles | - Mark the job according to given drawing - Select appropriate blade according to job requirement - Set blade in frame of metal circular saw as per procedure. - Check the blade tightness and rotating side. - Check the work piece is clamped firmly and properly - Adopt methods and techniques for sawing that is appropriate to job requirement - Follow marked line during sawing to ensure accuracy. |

Knowledge & Understanding

K1. Describe basic measurement

K2. Describe types of hacksaw frames

K3. Describe basic measuring /Marking /cutting tools

K4. Describe clamping/holding methods

K5. Define methods and techniques of sawing.

Tools and Equipment

Work bench, Bench vice, Tri-square, Hand hacksaw with blade, Scriber, Flat File, Vernier caliper, punching tools

    - Perform grinding operations

Overview: Overview: This competency standard covers the skills and knowledge required to Adjust Grinding wheel, Adjust tool Rest and Prepare a single point cutting tool for Basic machining purposes.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Adjust Grinding wheel | - Select the right tool for job - Remove the safety guard of grinding wheel. - Select the wheel type according to material/speed. - Mount the wheel on the spindle. - Balance the wheel - Put back the safety covers. - Ensure that wheel is properly mounted. |
| CU2. Adjust tool Rest | - Select the right tool for job - Remove the safety guard of grinding wheel. - Mount the tool rest. - Balance the wheel - Put back the safety covers - Ensure that tool rest is properly mounted and balanced. |
| CU3. Prepare a single point cutting tool for Basic machining purposes | P1. Select marking tools P2. Cut the material as per requirement. P3. Perform surface finishing with file P4. Level the surface with tri-square P5. Give the angle to the tool according to cutting requirement. |
| - Dress the Wheel | - Select dressing tools - Ensure that wheel is properly mounted. - Use dressing tool to remove the material from grinding wheel which is creating problems. - Balance the grinding wheel. |
| CU5 Perform Angle Grinding for finishing | P1. Select marking tools P2. Cut the material as per requirement. P3. Perform surface finishing with angle grinder machine P4. Level the surface with tri-square |
| CU6 Operate surface grinding | P1. Select the suitable size and type of grinding wheel. P2. Mount the work piece over the holding devices to ensure proper clamping. P3. Dress the wheel with diamond tip if required. P4. Identify reference points on work piece before grinding. P5. Adjust depth of cut according to speed of machine table. P6. Use coolant continuously to avoid over heating of the job. P7. Observe personal and workplace safety. |

Knowledge & Understanding

K1.	Define types of grinding wheels

K2.	 Describe basic measuring

K3.	Describe the Grinding machine types.

K4.	Describe clamping/holding methods.

K1. Type and size of wheels and abrasive.

K2. Method of dressing of grinding wheel.

K3. Work holding methods which include:

•	Magnet Table

•	Vice

•	Angle Plate

•	Machine base

K4. Importance of using coolant.

K5. Methods and techniques for surface grinding.

K6. Selecting right standing position during grinding.

K7. Specific safety precautions and guidelines

K1. Describe basic measurement

K2. Describe basic measuring /Marking /cutting tools

K3. Describe clamping/holding methods

K4. Knowledge of different cutting tool materials.

K5. Describe safety during handling the angle grinder

Tool and Equipment

Offhand Grinding Machine

Bench vices

Hammer

Scriber

Vernier calliper

Set of spanners

Angle Grinding Machine

Grinding

Machine

Holding Devices

Wheel Dresser

Grinding Wheels

Wheel Dresser Stand

Measuring Tools

Adjustable Wrench

Allen Key Set

    - Perform Tool & Cutter Grinding

Overview: This competency standard covers the skills and knowledge required to Adjust Grinding wheel, adjust tool Rest and Operate the Tool and cutter grinding.

| Competency Units | Performance Criteria |
| --- | --- |
| - Adjust Grinding wheel | - Select the right tool for job - Remove the safety guard of grinding wheel. - Select the wheel type according to material/speed. - Mount the wheel on the spindle. - Balance the wheel - Put back the safety covers. - Ensure that wheel is properly mounted. |
| - Adjust tool Rest | - Select the right tool for job - Remove the safety guard of grinding wheel. - Mount the tool rest. - Balance the wheel - Put back the safety covers - Ensure that tool rest is properly mounted and levelled. |
| - Operate the Tool and cutter grinding | - Select the suitable size, type and shape of grinding wheel. - Mount work piece onto correct attachment for required procedure. - Adjust the attachments according to different types of tools and cutter grinding. - Perform grinding on damaged drill bit to make it right. - Perform grinding on damaged cutter of milling Machine to make it right. - Follow procedure for sharpening of tools and cutter that is safe and appropriate. - Observe personal and safety precautions. |

Knowledge & Understanding

- Define types of grinding wheels

- Describe basic measuring

- Describe the Grinding machine types.

- Describe clamping/holding methods.

- Define types of grinding wheels

- Define safe distance from grinding wheel for tool rest

- Types, sizes and shapes of grinding wheels.

- Types of attachments and their use.

- Procedure of mounting of work-piece on to related attachments.

- Different tools and cutter angles.

- Procedure of sharpening of tools and cutters.

Tool and Equipment

Tool and cutting grinding Machine, Bench vices, Hammer, Scriber, Vernier caliper, Set of spanners, Offhand Grinding Machine, Diamond dresser tool , Grinding attachment, Universal bevel , Protector, Tool and Cutter

    - Perform Lathe Machine Operations

Overview: This competency standard covers the skills and knowledge required to perform centering, turning, facing and knurling operations.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Perform centering operations | P1. Select facing tools according to job requirement. P2. Mount and set the required work-holding devices, work piece and cutting tools. P3. Follow the correct specifications for the part or component to be produced. P4. Select safe procedures and tools to accomplish the work. P5. Adjust the operating parameters (e.g. speed and feed) of machine tool for centering the job. P6. Ensure all safety mechanisms are in place. |
| CU2. Perform facing Operations | P1. Select facing tools according to job requirement. P2. Mount and set the required work-holding devices, work piece and cutting tools. P3. Follow the correct specifications for the part or component to be produced. P4. Select safe procedures and tools to accomplish the work. P5. Adjust the operating parameters (e.g. speed and feed) of machine tool to achieve the work specification. P6. Ensure all safety mechanisms are in place. |
| CU3. Perform turning Operations | P1. Obtain and follow work specifications, drawings or sketches to accomplish the work. P2. Set up and adjust the machine as per work specifications and procedures. P3. Ensure the components produced have the required quality and within the specified dimensional accuracy by turning operation. P4. Shut down the machine and equipment on conclusion of the machining activities. |
| CU4. Perform drilling or boring operations | P1. Select drill or boring tools according to drawings. P2. Mount and set the required work-holding devices, work piece and cutting tools. P3. Adjust the RPM of machine according to the cutting speed. P4. Perform the boring operation according to the drawing. P5. Check quality of the component produced at different intervals. P6. Observe personal and workplace safety. |
| CU5. Perform step turning operations | P1. Mount and set the required work-holding devices, work piece and cutting tools. P2. Select and adjust appropriate speeds and feeds of turning machine. P3. Produce a component which matches the work specifications using step turning methods and techniques. P4. Check quality of the component produced at different intervals. P5. Follow safety precautions to ensure safe work and to avoid any injury. |
| CU6. Perform knurling Operations | P1. Select the knurling tool according to drawing. P2. Set the tool and work piece in the machine according to procedure. P3. Adapt right methods and techniques to produce proper knurling on work piece. P4. Select and adjust appropriate speeds and feeds of lathe machine. Use coolants during knurling to achieve smooth impression on work piece. P6. Observe personal and workplace safety. |
| CU7. Taper turning by tail stock off-set method | - Loosen tailstock clamp out. - Offset tailstock required amount. - Centre cutting tool. - Setup cutting tool for parallel turning. - Starting at small diameter take excessive cuts until the taper is .05 to .06 in oversize. - Check taper for accuracy using a taper ring gauge. - Finish turn the taper to size and fit required. |
| CU8. Taper turning by plain taper turning attachment | - Remove the binding screw that cross slide to cross feed screw nut. - Tighten the lock screw and set cutting tool on center. - Set workpiece in lathe and mark length of taper. - Use binding screw to connect sliding block to side of taper attachment. - Select depth of feed cut by compound rest feed handle. - Take a light cut and recheck taper fit. - Check taper for accuracy using a taper ring gauge. |
| CU9. Taper turning by telescopic taper turning attachment | - Clean and oil the guide bar. - Loose lock screws and offset end of guide bar, - Set the bar to required taper in degrees. - Tighten the lock screw and set cutting tool on center. - Set workpiece in lathe and mark length of taper and tighten connecting screw on sliding block. - Move carriage until center of attachment is opposite to length of taper. - Lock anchor bracket to lathe bed. - Take a cut and select depth of cut. - Readjust the taper attachment, take a light cut and recheck taper fit. - Check taper for accuracy using a taper ring gauge. |
| CU10. Perform Internal and External threading Operations | - Mount and set the required work-holding devices, work piece and cutting tools. - Select and adjust appropriate speeds and feeds of turning machine. - Produce a component which matches the work specifications using appropriate methods and techniques. - Check quality of the component produced at different intervals. - Use Proper cutting tool to cut the threads with required dimensions. - Follow safety precautions to ensure safe work and to avoid any injury. |

Knowledge & Understanding

- Calculation of speed and feed.

- Safety precautions involved in work.

- Methods and techniques of mounting and setting of work-piece.

- Methods and techniques of adjusting operating parameters of machine tool.

- Procedure of adjusting speed and feed.

- Calculation of speed and feed.

- Use of cutting tool

- Reading and interpreting work specifications, drawings and sketches.

- Method and technique of setting up and adjusting the machine.

- Techniques to check quality of component produced.

- Procedure of shutting down of machine and equipment after closure of activities.

- Safety precautions and procedures need to be observed during work.

- Types of drilling or boring tools and their function.

- Procedure of mounting and setting up of work-holding devices, work pieces and cutting tools.

- Method and technique of adjusting RPM of lathe machine.

- Safe boring procedures.

- Calculation of RPM.

- Types of knurling tools and Methods of knurling.

- Knowledge of lathe operations

- Use of dial indicator

- Calculations for taper turnings

- Types of threading tool.

- Types and methods of threading.

- Procedure of setting tools and work piece in the machine.

- Procedure of adjusting speeds and feeds of lathe machine. Importance of using coolants during knurling.

- Safety precautions and guidelines.

Tool and Equipment

Measuring Tools, Cutting Tools, Vernier Caliper, Checking gauges, Knurling Tools, Lathe Machine, Threading Tools, Personal Protective Equipment, Files

    - Perform Shaper Machining Operations

Overview: This competency standard covers the skills and knowledge required to Produce a squared shape work piece, Produce V shaped work piece and Machine Irregular Surfaces.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Produce a squared shape work piece | - Identify the parts and their functions of shaper Machine. - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Leveled the machine vice according to job requirement. - Select point cutting tool and set machine as per requirements. - Mount cutting tool and work piece in the machine. - Check quality of the component at suitable intervals. - Shut down the machine at safe position after finishing the work. |
| CU2. Produce V shaped work piece | P1. Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. P2. Dial the machine vice according to job requirement. P3. Select point cutting tool and set machine according to job requirements. P4. Mount cutting tool and work piece in the machine. P5. Check quality of the component at suitable intervals. P6. Shut down the machine in safe position after finishing the work |
| CU3. Perform T-slot Machining | P1. Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. P2. Dial the machine vice according to job requirement. P3. Select point cutting tool and set machine according to job requirements. P4. Mount cutting tool and work piece in the machine. P5. Check quality of the component at suitable intervals. P6. Shut down the machine in safe position after finishing the work |
| CU4. Machine Irregular Surfaces | - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Use Different feed and speed of cutting and different points according to Instructions. - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work |

Knowledge & Understanding

- List safety hazards related with the shaper machine operations.

- Use of Dial indicator

- Method of mounting of cutting tool

- Checking of right angle with the tri square.

- Explain square facing procedure.

- Safety guidelines and procedures.

- Safety checks for operating shaper machine.

- Interpreting information given in the engineering drawings and job specifications

- Checking of angle with the bevel protector.

- V-Shape cutting procedure.

- Interpreting information given in the engineering drawings and job specifications.

- List safety hazards related with the shaper machine operations.

Tool and Equipment

Shaper, Planar or Slotter, Machine Vice, Tri square/bevel, protector, Vernier Caliper, Dial indicator with magnet stand, Point cutting tools, Personal Protective Equipment

    - Perform CNC Lathe Operations

Overview: This competency standard covers the skills and knowledge required to mount the Job, Generate the Program, Run Simulation, Feed the Program and Perform CNC Lathe Operations.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Mount the Job | - Mount the work-piece by considering the working capacity of machine as well as job requirement according to the drawing/design - Select appropriate work holding device(s) in order to achieve dimensional accuracy and clamp the job firmly as per standard practice - Attain proper alignment of tool/ cutter and workpiece e.g. concentricity of rotating jobs as per set practice - Keep safe measures while mounting the workpiece so that unwanted operation by machine may not be initiated as per safety precautions |
| CU2. Generate the Program | - Interpret job requirements, calculate extra material to be removed and define reference point as per drawing/design - Define absolute or incremental coordinates system, tool path strategies, machining features and tool compensation for generating the tool path as per standard procedure - Use appropriate part programming credentials (Sequence, G-codes, M-codes, coordinates, feed, speed, tooling information etc.) according to the CNC machine control unit - Keep record of generated part program in soft/hard form in order to feed into machine control unit as per standard procedure |
| CU3. Run Simulation | - Feed the generated part program into appropriate simulation platform and run simulation for checking the tool gouge according to safety measures - Run simulation and verify movements of tool/cutter to get same results as per defined sequence - Identify occurrence of errors and modify the program as per defined procedure |  |
| CU4. Feed the Program | - Ensure proper synchronization between machine control unit and part program file as per standard operating procedure - Switch machine to receiving mode and feed the desired part program file into machine control unit for further execution as per standard operating procedure - Select the desired part program file for execution as per standard operating procedure |  |
| CU5. Perform CNC Lathe Operations | - Ensure to control the safe operation of working on CNC machines before executing part program according to the safety measures - Control the feeds, speeds and override of machine before operating according to the prescribed procedure - Switch machine to execution mode (single block or auto) and press cycle start to run the machining sequence as per prescribed method - Compare the block-wise movements of machining sequence thoroughly during operating of machine according to the part program file - Complete the job and inspect its accuracy and precision according to the drawing/design |  |

Knowledge & Understanding

- Work place safety and health considerations

  - Use of Personal Protective Equipment

  - Reading Drawing

  - Mechanism of working of CNC lathe machine

- Use of control panel and commands

- Program debugging techniques

- Use of Simulation software

- Use of portable devices for CNC lathe

- Use of Turret / Magazine and their sequence of tool mounting

- Possible accidents and their counteractions

- Coolant types along with benefits and uses

- Methods of calculating Coordinates techniques

- G codes and M codes.

- Feed and speed concepts in Lathe Machine

- Lathe operations such as Facing, Turning, Drilling, Grooving, Threading, Knurling, Boring etc.

- Use of Lathe tools and their types with respect to operations and materials

- Feed and speed concepts in Lathe Machine

Tools and Equipment

1.    CNC Lathe Machine or Turning Centre along with Standard Accessories

2.    Lathe Tooling (Assorted Range)

3.    CNC Programming Manual

4.    CAM Software with Simulation Module

5.   Measuring Instruments (Vernier, Inside/Outside Calipers, Micrometer, Steel Rule, Tri-Square, Bevel Protractor etc.)

6.    Work Holding Devices

7.    Measuring Gauges

8.    Tooling Catalogue

9.    CNC Manual

10.   Complete Set of Computer System with Multimedia Projector

11.   Personal Protective Equipment (PPE)

    - Perform Pre-Welding Operations

Overview: This competency standard covers the skills and knowledge required to Setting Welding Equipment, prepare materials for welding, Cut and Prepare Edge/s of Base Materials, Prepare Welding Consumables

| Competency Units | Performance Criteria |
| --- | --- |
| - Set Welding Equipment | - Adjust pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch Make carburizing flame by increasing acetylene gas quantity - Make neutral flame by adjusting both gases at same quantity - Make oxidizing flame by increasing oxygen gas quantity - Adjust pressure of both gas cylinders with the help of regulator - Select the correct size of the nozzle - Set the both gas flame of welding torch as per standard |
| - Prepare Tools and materials for welding | - Select and obtain required material/s as per job requirements - Select appropriate marking tools as per job requirements - Mark the area to be cut as per drawing/job requirements - Select relevant welding consumables as per job requirements/ WPS. - Prepare consumables in accordance with required specifications |
| - Cut and Prepare Edge/s of Base Materials | - Select appropriate cutting equipment as per job requirements - Set-up cutting equipment as per manufacturer’s instructions/job requirements - Cut the base material as per job specifications and dimensions provided in the drawing - Prepare edges of the base materials as per drawing/WPS - Check dimensions of the prepared edges as per drawing/WPS - Select proper tools and chemicals for cleaning Clean the edges of the base materials as per job requirements. |

Knowledge & Understanding

- Describe about welding torch

- Identify Gas pressure regulators

- Explain temperature and its units

- Describe pre heating

- Explain importance of pre heating

- Explain metal properties

- Describe malleability

- Describe types of grinder

- Explain use of tri square

- Describe importance of filing

- Describe the filler rod

- Describe electrode baking oven

- Describe purpose of flux

Tools and Equipment

- Oxygen cylinder

- Acetylene gas cylinder

- Pressure regulators

- Cylinder key

- Welding torch

- Rubber house pipe

- Back fire arrester

- Flash back arrester

- Spark lighter

- Steel wire brush

- Welding electrode

- Metal Filler rod

- Welding flux

- Work bench

- Bench vice

- Hammer

- Tri-square

- Hand hacksaw

- Scriber

- Vernier calliper

- Flat File

- Pedestal grinder

- Disk grinder

- Pin grinder

    - Perform Shielded Metal Arc Welding (SMAW)/electric welding

Overview: This Competency Standard is designed to gain basic knowledge and skills required to perform Shielded Metal Arc Welding (SMAW) operations in Flat (1F, 1G) and Horizontal (2F, 2G) positions at workplace. The standard covers specific knowledge of performing Shielded Metal Arc Welding (SMAW) by selecting and setting up welding equipment, installing consumables, adjusting welding parameters and making fillet and groove welds in Flat (1F, 1G) and Horizontal (2F, 2G) positions of plate. The standard also covers post welding operations comprising cleaning, measuring, inspecting and repairing welds at workplace

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare Welding Machine and Accessories for SMAW | - Identify welding requirements from the job, welding procedure specifications and technical drawings - Prepare SMAW welding machine in accordance with welding procedure specifications/ manufacturer instructions - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions - Connect welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications. |
| CU2 . Make Fillet Welds on C Steel Plate (1F, 1G) | - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Flat (1F) and Flat (1G) positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects. |
| CU3. Make Groove Welds on Carbon Steel Plate (2F, 2G) | - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Horizontal (2F) and Horizontal (2G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects |
| CU4. Make Fillet Welds on Carbon Steel Plate (3F, 3G) | - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Vertical (3F) and Vertical (3G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects |
| CU5. Make Groove Welds on Carbon Steel Plate (4F, 4G) | - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Carry out welding in Overhead (4F) and Overhead (4G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects |
| CU6. Perform Post Welding Operations | - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects, as required - Carry out repair work in accordance with approved procedures, as required - Clean work area in accordance with workplace safety practices - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines |

Knowledge & Understanding

- Understanding of technical drawings

- Electrical supply AC and DC

- Specifications/ classification of electrode/s required for the job

- Electrical parameters like (voltage, current etc.) and their effects on weld

- Welding techniques as per WPS/instruction sheet

- Welding procedure specifications (WPS)

- Method of Pre- heating of base metal

- Polarity setting according to standard specifications

- Visual welding defects

- Welding codes and standards

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform SMAW fillet weld at 1F, 2F positions, groove weld at 1G, 2G positions as per given job specification

    - Perform different welding joints

Overview: This Competency Standard is designed to gain basic knowledge and skills required to Practice of Making Butt Joint, Practice of making Lap Joint and Practice of making Tee Joint.

| Competency Units | Performance Criteria |
| --- | --- |
| C1. Practice of Making Butt Joint | P1. Take M.S Flat as per drawing P2. Straiten it with the help of hammer and anvil P3. Place the work piece on welding table P4. Hold the electrode in electrode holder P5. Set the current on welding machine as per standard P6. Hold the electrode at right angle P7. Set the current at 120-125 ampere P8. Tack both pieces of M.S Flat for Butt Joint P9. Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap P10. Lean the electrode at 70º to the base metal and complete the bead to form a square butt joint |
| C2. Practice of making Lap Joint | P1. Take M.S Flat as per drawing P2. Straiten it with the help of hammer and anvil P3. Mark parallel line on both pieces with the help of scriber for positioning of lap joint P4. Place the work piece on welding table P5. Place the Bottom piece on work table and place the Top plate along the marked line P6. Hold the electrode in electrode holder P7. Set the current on welding machine as per standard P8. Tack both work pieces of M.S Flat P9. Complete the bead by back hand welding technique as per standard |
| C3. Practice of making Tee Joint | P1. Take M.S Flat as per drawing P2. Straiten it with the help of hammer and anvil P3. Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other P4. Place the Bottom piece on work table and place the Top plate at 90* to each other as per drawing P5. Hold the electrode in electrode holder P6. Set the current on welding machine as per standard P7. Tack both work pieces of M.S Flat P8. Lean the electrode and move along traverse direction P9. Complete the bead as per standard |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain positions of electrode according to work

- Knowledge of setting the current on welding machine

- Describe motion of electrode in ARC welding

- Explain importance of gap between electrode and base metal

- Describe use of tri square

- Describe importance of cleanliness of surface to be welded.

    - Perform Forging Operations

Overview: This Competency Standard is designed to gain basic knowledge and skills required to Perform main operation of forging (Drawing down and Swaging). The standard covers specific knowledge of performing 0f Forging in drawing down and swaging.

| Competency Units | Performance Criteria |
| --- | --- |
| - Practice of drawing | - Make fire in available forge furnace - Put the stock in forge furnace. - Heat the stock up to bright red condition (Plastic State) - Grip the heated stock by tong and put on anvil. - Apply firm blows of sledge hammer on both sides of stock. - Put the stock again in forge furnace and repeat process - Always wear safety dress before start the performance. |
| - Practice of Swaging | - Make fire in forge furnace - Put the round stock in forge furnace. - Heat the stock up to bright red condition. - Fix the bottom swage in hardy hole. - Put the red-hot round bar in the bottom swage. - Put the top swage over round bar with the help of handle. - Apply firm blows of sledge hammer and move round bar longitudinally and circumferentially. - Always wear safety dress before start the performance. |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the Knowledge of:

- Understanding of techniques of drawing down

- Proper preparation and selection of stock.

- Understand the use of proper equipment.

- Using proper safety dress before starting performance.

- Complete the drawing down and swaging process rapidly.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in

- Prepare the job through drawing down process.

- Prepare the job through Swaging process

Tools and equipment required

- Forge furnace

- sledge hammer

- Safety Dress

- Dies

- Hacksaw

- Drill set

- Drill Machine

- Welding Machine

- Lathe Machine

- Milling Machine

# Basic Engineering Drawing and CAD/CAM

- Perform Basic Manual Drawing

Overview: This competency standard covers the skills and knowledge required to Draw single stroke capital vertical lettering, Draw single stroke capital inclined lettering, Draw horizontal, vertical and inclined lines, Use of compass, circles, half circles, radius, Drawing Centre   lines, centres, curves, and crossing of lines, Construction of parallel-lines, perpendicular, bisects line, angles and equal division of lines, Draw round corners, circles elements, quadrilaterals inside and outside circle and Construction of angles and triangles.

| Competency Units | Performance Criteria |
| --- | --- |
| - Practice of Lettering 5 mm height | - Prepare Drawing sheet. - Select the tools. - Use Proper pencil for lettering with holding techniques. - Draw Boundary lines as per standards. - Make title bar - Draw upper and lower lines for lettering on distance of 5mm - Start Writing Vertical Lettering with different style like Gothic, Roman and free hand lettering |
| - Practice of Lettering 3 mm height | - Prepare Drawing sheet. - Select the tools. - Use Proper pencil for lettering with holding techniques. - Draw Boundary lines as per standards. - Make title bar - Draw upper and lower lines for lettering on distance of 3mm - Start Writing Vertical Lettering with different style like Gothic, Roman and free hand lettering |
| - Use Tee Square and Set Squares for drawing horizontal, vertical and inclined lines | - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw horizontal lines at 0 degree using standard tools - Draw inclined lines at 30, 45, 60 and 120 degree angles using standard tools - Draw vertical lines at 90 degree using standard tools |  |
| - Carry out use of Tee square and for drawing centres, crossing of lines | - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different angle curves. - Sketch Centre line according to standard. - Draw crossing line |  |
| - Draw circles, half circles and radius | - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia meter circles and half circles using compass - Make different Radius using Compass and Protector |  |
| - Draw round corners, figure inside and outside circle | - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia circles. - Make diagrams inside the circle that touch the circle at the tangent points - Make diagrams outside the circle that touch the circle at the tangent points |  |
| - Draw Plane geometry angles and triangles | - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Equilateral Triangle, Isosceles triangle, Scalene Triangle, Right Triangle, Obtuse Triangle, Acute Triangle. |  |
| - Sketch Plane Geometry quadrilateral, square, rhombus and parallelogram | - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Square and Rectangle as per drawing - Draw Rhombus using protector and set square as per drawing - Draw Parallelogram as per drawing |  |
| - Plane Geometry parallel-lines, perpendicular, bisect line and angle | - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw parallel-lines of different length - Draw perpendicular lines as per drawing - Draw bisect lines as per drawing |  |
| - Sketch Plane geometry equal division of line and some radio with the help of compass and set square | - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Draw lines of different lengths as per drawing - Open compass more than one half of drawn line - Use compass to draw an arc from one end of drawn line - Repeat this step on other end of line - Draw a perpendicular line where both arc meets - Check the half of line using tri square |  |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Importance of Technical Drawing.

- Language of engineering terminology.

- Uses of Technical Drawing

- Type of Drawing

- Application of Technical drawing

- Drawing Pencil, their grading, sharpening and using techniques.

- Style of letters.

- General rules for lettering

- Introduction to geometry

- Introduction to sketching techniques.

- Techniques of sketching straight lines in different directions.

- Define Triangles, Quadrilateral, and Polygons

Tool and Equipment.

Graph and drawing sheet, Drawing Board/table, T-Square, Set Square, Templets and

Geometry Box.

- Construct Different Engineering Curves

Overview:This competency standard covers the skills and knowledge required to Construct inscribe and circumscribe figures, Construct a pentagon, Hexagon and Octagon by circumscribe method, Construct a pentagon, Hexagon and Octagon by inscribe method, Construct a Tangents of circles (Inside & Outside)When the centre of the given circle is known and when the circle of centre is not known, Construct an Ellipse by Concentric Circle Method, Rectangle Method, Oblong Method, Arcs of Circle Method, Rhombus Method and Basic Locus Method, Construct a parabola curve by Rectangle Method, Method of Tangents( Triangle Method) and Basic Locus Method, Construct a hyperbola curve, Construct a Archimedean Spiral curve, Construct a involutes curve of square rectangle hexagon and circle and Construct of cycloid, epicycloids, and hypocycloid.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw inscribe and circumscribe figures. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw square, triangle and hexagon according to dimension. |
| - Draw a pentagon, Hexagon and Octagon by circumscribe method. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon . |
| - Draw pentagon, Hexagon and Octagon by inscribe method | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon. |
| - Draw Tangents of circles (Inside & Outside) | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Tangents Inside of a circle When the centre of the circle is known. - Draw Tangents Inside of a circle When the centre of the circle is unknown - Draw Tangents outside of a circle When the centre of the circle is known - Draw Tangents outside of a circle When the centre of the circle is unknown |
| - Construct Ellipse | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw an Ellipse by Concentric Circle. - Draw an Ellipse by Rectangle Method - Draw an Ellipse by Oblong Method - Draw an Ellipse by Arcs of Circle Method - Draw an Ellipse by Rhombus Method. - Draw an Ellipse by Basic Locus Method |
| - Draw parabola curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a parabola curve by Rectangle - Draw a parabola curve by Method of Tangents (Triangle Method) - Draw a parabola curve by Basic Locus Method |
| - Draw hyperbola curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a hyperbola curve. |
| - Make Archimedean Spiral curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw spiral curve. |
| - Make involute curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw involute curve by square - Draw involute curve by rectangle - Draw involute curve by hexagon - Draw involute curve by circle. |
| - Draw cycloid, epicycloid, and hypocycloid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw the generating circle and the base line equal to the circumference of the generating circle - Divide the circle and the base line in to equal number of parts - Complete the cycloid, epicycloids, and hypocycloid. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

K1.	Techniques of sketching straight lines in different directions.

K2.	Define Triangles, Quadrilateral and Polygons.

K3.	Describe circular arc using different line method.

K4.	Describe circular arc

K5.	Types of Geometric Shape

K6.	Two-dimensional shapes

K7.	Three-dimensional shapes

K8.	Types of Geometric Shape

K9.	Regular Polyhedrons

K10.	Methods of drawing Tangents & Normal

K11.	Describe ellipse

K12.	Describe different methods of sketching ellipse

K13.	Describe parabola

K14.	Describe different methods of parabola

K15.	Describe spiral curve

Tool and Equipment.

Graph and drawing sheet, Drawing Board/Table, Tee-Square, Set Square, Templets, Geometry Box

- Construct Multi-view drawings

Overview: This competency standard covers the skills and knowledge required to Sketch Orthographic projection 1st angle, Sketch Orthographic projection 3rd angle, Sketch Oblique Drawing, Construct multi view drawing of Simple Bearing, Construct multi view drawing of Open Bearing, Sketch prism, Sketch cone and Draw pyramid.

| Competency Units | Performance Criteria |
| --- | --- |
| - Sketch Orthographic projection in 1st angle of Projection | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view |
| - Sketch Orthographic projection 3rd angle of Projection | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view |
| - Sketch Oblique Drawing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts - Draw the front or side view of the object. |
| - Construct multi view drawing of Simple Bearing. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of simple bearing - Draw front view of simple bearing - Draw side view of simple bearing |
| - Multi view drawing of Open Bearing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of open bearing - Draw front view of open bearing - Draw side view of open bearing |
| - Sketch prism | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch prism |
| - Sketch cone | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Start with a horizontal oval - draw the two sides of a triangle which meets at a common point |
| - Draw pyramid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch pyramid |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain Orthographic projection 1st angle.

- Explain Orthographic projection 3rd angle.

- Explain Oblique Drawing.

- Explain Multi view drawing of Simple Bearing.

- Explain Prism, Cone and pyramid

Tool and Equipment.

Graph and drawing sheet, Drawing Board/Table, T-Square, Set Square, Templets, Geometry Box

- Install CAD software.

Overview: This competency standard covers the skills and knowledge required to create new file and create basic drawing, Practice loading CAD software into computer memory and Practice CAD abbreviations.

| Competency Units | Performance Criteria |
| --- | --- |
| - Practice loading CAD software into computer memory | - Download the installer files based on the download methods available for your product. - Find and double-click the setup file to start the installation - Click Install/Uninstall on this Computer - Select any one of the following options - Typical - Full - Compact - Custom - Perform necessary steps to Install CAD software |
| - Practice CAD abbreviations, auto CAD release 12 of latest (e.g. A for Arc, C for circle, E for Erase etc.). | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Click on command window - Type alphabet A and select method of arc to be used - Type alphabet C and select method of circle to be used - Type alphabet E and select erase method to be used |
| - Practice function keys for shortcuts | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Select function key F1 to display help - Select function key F3 to turn on/off Object Snap - Select function key F4 to turn on/off 3D Object Snap - Select function key F7 to turn on/off Grid Display - Select function key F8 to lock cursor movement to Horizontal/Vertical |

Knowledge and skills

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Installation of software.

- Basic Drawing Settings

- Unit setting

- Limits setting

- User coordinate system Workspace setting

- Object Snap Settings

- Basic Commands and Concepts Angles and lines in AutoCAD.

- Differentiate between absolute, relative and polar system

- DIMSTYLE and MTEXT commands

- HATCHING concepts in AutoCAD

- Differentiate between CHAMFER and FILLET command

- Types of Array

- OFFSET, CIRCLE and ROTATE short commands

- Zooming options

- Tools palettes window

- Design center

- Scale and paper sizes

- Modify dimension style and text size according to paper size

- Backup file

Tool and Equipment.

Computer with all accessories, AutoCAD software disk and Models

- Manage display in CAD software

Overview: This competency standard covers the skills and knowledge required to Practice to draw two points using Cartesian Notation on graph paper and Set-up drawing area using CAD software.

| Competency Units | Performance Criteria |
| --- | --- |
| - Practice to draw two points using Cartesian Notation on graph paper | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw A1,1 point on x and y axis keeping origin as reference - Draw B4,2 point on x and y axis keeping origin as reference |
| - Practice to draw straight line using polar coordinates on graph paper | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Select line command from draw tool bar - Select point 1,1 on graph paper - Type @5<30 in command prompt and press enter - Type @60<0 in command prompt and press enter - Type @20<90 in command prompt and press enter - Press ESC to exit command |
| - Set-up drawing area using CAD software | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Set-up drawing area as per standard operating procedure |
| - Practice for Turning GRID ON and Off and SNAP ON and OFF | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Select Status Bar and click on GRID icon to turn on or turn off grid - Select Status Bar and click on SNAP icon to turn on or turn off snap |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

This competency standard will provide knowledge related to

- Basic Drawing Settings

- Unit setting

- Limits setting

- User coordinate system Workspace setting

- Object Snap Settings

- Basic Commands and Concepts Angles and lines in AutoCAD.

- Differentiate between absolute, relative and polar system

- DIMSTYLE and MTEXT commands

- HATCHING concepts in AutoCAD

- Differentiate between CHAMFER and FILLET command

- Types of Array

- OFFSET, CIRCLE and ROTATE short commands

- Zooming options

- Tools palettes window

- Design center

- Scale and paper sizes

- Modify dimension style and text size according to paper size

- Backup file

Tool and Equipment.

Computer with all accessories, AutoCAD software disk Models

- Perform function of CAD general commands

Overview: This competency standard covers the skills and knowledge required to Draw a line with LINE command, Widen Border lines with PEDIT and Create layers and move border to its own layer.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw a line with LINE command | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Click on LINE icon from draw tool bar - Give dimension of line in Command window OR P8. Specify Start Point and End Point |
| - Widen Border lines with PEDIT | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Type the command PEDIT - Select Width and type a width factor |
| - Save Border Template (QSAVE) | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Type the command CNTL+S to quickly save border template |
| - Create layers and move border to its own layer | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw different layers as per standard - Draw border as per standard - Select MOVE command - Select border as per standard - Draw border on selected layer |
| - Create a layer for Title Block | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Create a layer as per standard - Draw title block using lines as per standard |
| - Create Title Block | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Create title block as per standard - Specify cell alignment of title block - Specify scale of title block |
| - Practice for Zoom command | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Type ZOOM in command bar - Select required zoom type for example - All - Center - Dynamic - Extents - Previous - Scale - Window - Object |

- Perform Different Cad operations

Overview: This competency standard covers the skills and knowledge required to Practice of 3D commands, Practice for filling Title Block, Start a New drawing and practice different shapes.

| Competency Units | Performance Criteria |
| --- | --- |
| - Practice of 3D commands | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Draw objects as per standard - use 3D commands to convert objects into 3D as per standards |
| - Practice for filling Title Block | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Create title block as per standard - Specify cell alignment of title block - Specify scale of title block - Select edit command to fill title block |
| - Practice for plotting the drawing on plotter or printer | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Select print command - Select available plotter or printer as per standard |
| - Start a New drawing | - Start auto cad program - Press CTRL+ N or select New in file menu - Select units as per requirements - Select drawing limits |
| - Practice with edit command to make changes in the drawing | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Type command DEDIT - Select line whose dimension needs to be edited |
| - Draw an Angled line | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Type command ALIGNED LINE and type magnitude and angle of line - Type LINE command and type @20<45 in command bar - Select LINE icon and specify first and second point with mouse cursor on desired angle |
| - Practice with Undo Command and Redo command | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Select CIRCLE command and draw a circle of desired dia - Press CTRL+Z to undo that circle - Press CTRL+Y to redo that circle |
| - Draw a circle with circle command | - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Select CIRCLE command - Select circle type e.g. center radius, center diameter, 2-point, 3-point and draw a circle of desired dia or radius |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Basic Drawing Settings

- Unit setting

- Limits setting

- User coordinate system Workspace setting

- Object Snap Settings

- Basic Commands and Concepts Angles and lines in AutoCAD.

- Knowledge of LINE command

- Knowledge of CIRCLE command

- Knowledge of POLYLINE command

- Knowledge of RECTANGLE command

- Knowledge of ELLIPSE command

- Knowledge of POLYGON command

- Knowledge of MODIFYING commands

- Knowledge of absolute coordinates system

- Knowledge of relative coordinates system

- Knowledge of polar coordinates system

- Command windows

- The ribbon

- Quick access toolbar

- View cube

# Occupational Health, Safety and Environment

- Read and Develop Career Professionalism

Overview: This unit covers the knowledge, skills and attitudes in promoting career growth and advancement

| ##### Unit of Competency | ##### Performance Criteria |
| --- | --- |
| - Integrate personal objectives with organizational goals | - Personal growth and work plans are pursued towards improving the qualifications set for the profession - Maintain the Intra- and interpersonal relationships in the course of managing oneself based on performance evaluation. - Demonstrate the commitment to the organization and its goal in the performance of duties. |
| - Set and meet work priorities | - Competing demands are prioritized to achieve personal, team and organizational goals and objectives. - Manage work priorities and commitments in order to utilize the resources efficiently and effectively. - Practices along economic use and maintenance of equipment and facilities are followed as per established procedures |
| - Maintain professional growth and development | - Identify the trainings and career opportunities and avail on the bases of job requirements. - Recognitions are sought/received and demonstrated as proof of career advancement - Obtained and renewed the licenses and/or certifications relevant to job and career. |

##### Knowledge & Understanding

- Goal alignment: Set objective which are in line with business priorities. Goals must be smart and time bound

- Self-Development: Identify your training needs to develop competencies in order to perform given work profile

- Evaluation: Performance Appraisal, Psychological Profile, Aptitude Tests.

- Resources: Human, Financial, Technology, Hardware, Software

- Trainings and career opportunities: Participation in training programs, Technical, Supervisory, Managerial, Continuing Education, Serving as Resource Persons in conferences and workshops.

- Recognitions: Recommendations, Citations, Certificate of Appreciations, Commendations, Awards, Tangible and Intangible Rewards.

- Licenses and/or certifications: National Certificates, Certificate of Competency, Support Level Licenses, Professional Licenses

##### Critical Evidence(s) Required

- Capable to Integrate personal objectives with organizational goals

- Capable to set and meet work priorities

- Maintain professional growth and development

- Apply basic Occupational Health & Safety regarding heavy machinery

Overview: This competency standard covers the skills and knowledge required to Adopt Health & Safety regulations, encourage primary safety program, Ensure Personal protective equipment (PPE), Ensure Safety documentation common to Heavy Machines, Assess Worksite hazards, Judge Dangerous Trees, protect wildlife, Adopt Safety during fire season and Perform Map orientation.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Adopt Health & Safety regulations. | P1. Identify rights & responsibilities regarding safety P2. Interpret regulations & guidelines specific to Heavy Machines. P3. Interpret common safety rules and tips. P4. Identify employer safety rules and policies. |
| CU2. Encourage primary safety program | P1. Motivate by regulation. P2. Motivate by ethics, legitimate concern P3. Motivate by cost of lost time and injury Claims. P4. Motivate by liability |
| CU3. Ensure Safety documentation common to Heavy Machines. | P1. Construct safety network flag person certification. P2. Ensure Workplace Hazardous Materials Information System. (WHMIS) P3. Prepare Material Safety Data Sheets (MSDS) P4. Ensure symbols identifying dangerous goods. P5. Follow safety while transporting dangerous goods. |
| CU4.Assess Worksite hazards | P1. Ensure safety during Steep terrain movement. P2. Ensure safe terrain stability P3. Evaluate overhead High voltage wires. P4. Estimate overhead obstacles, overhangs & demolition |
| CU5. Judge Dangerous trees. | P1. Identify Snags. (Snag is any dry standing Tree) P2. Down all snags. P3. Flagg Huge snag if left standing. |
| CU6. Protect wildlife | P1. Identify Osprey-nest. P2. Mark nest on the map. P3. Leave the trees around for animals / bear in a den. |
| CU7. Adopt Safety during fire season | P1. Ensure designated fire equipment with the machine. P2. Ensure fire watch, fire trucks are ready to go. P3. Shut down the operation when humidity becoming lower than the temperature. P4. Calculate degree of hazards depends on the level of humidity versus temperature. |
| CU8. Perform Map orientation | P1. Identify hazards on map like Rocks, construction, pipelines, boundaries, power lines, soft ground, quarries and tree lines. P2. Follow map and evaluate all the issues like terrain of an area, slope percentage. |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Define Worksite hazards.

- Learn about the safety responsibilities of workers and employers according to Govt. of Pakistan (Pakistan Factory Act 1934)

- Learn about the safe work method of heavy machinery

- Learn common safety guidelines of the industry

- Observe and identify Worksite hazards

- Financial, social and moral impact of incidents

- Enlist various types of hazardous materials.

- Enlist various types of personal protective equipment.

- DefineMaterial Safety Data Sheets (MSDS).

- Describe Workplace Hazardous Materials Information System. (WHMIS)

Critical Evidence(s)

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Adopt Health & Safety regulations.

- Ensure Personal protective equipment (PPE)

- Ensure Safety documentation common to Heavy Machines.

- Assess Worksite hazards

- Judge Dangerous Trees.

- Protect wildlife

- Adopt Safety during fire season

- Perform Map orientation

- Perform general health, safety and environment practices

Overview: This competency standard covers the skills and knowledge required to Ensure personal protective equipment (PPE), Protect Tools and Equipment, Maintain First aid Box, Ensure Safeguard of Machines, Adopt Environmental Regulation, Adopt company policies and procedures, Follow federal, provincial/ territorial, and municipal legislation, Attain health & safety training, Prepare for emergencies, Respond to emergencies, Monitor activities of people, vehicles, and other equipment in area.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify hazards relevant to your task | - Identify hazards correctly in accordance with OHS standards - Identify safety signs and symbols - Identify unsafe act and conditions |
| CU2. Ensure personal protective equipment (PPE) | P1. Arrange PPEs as per requirement P2. Wear proper PPE as per nature of job P3. Store PPE at appropriate place after use |
| CU3. Protect Tools and Equipment | P1. Ensure insulation of tools and equipment P2. Store tools and equipment safely P3. Clean tools on a regular basis before stacking |
| CU4. Maintain First aid Box | P1. Ensure availability of first aid box P2. Check first aid box for requisite emergency medicines P3. Check expiry date of medicines P4. Perform first aid treatment against electric shock P5. Perform first aid treatment / bandages against minor injuries |
| CU5. Ensure Safeguard of Machines | P1. Check safety guards of machine P2. Check brake of machines P3. Check controlling devices of machine P4. Perform test operation on machine |
| CU6. Adopt Environmental Regulation | P1. Locate applicable permits on job site P2. Ensure work friendly environment P3. Adopt environmental regulations |
| CU7. Adopt company policies and procedures | P1. Ensure company policy and procedures P2. Adopt company procedures |
| CU8. Follow federal, provincial/ territorial, and municipal legislation | P1. Locate relevant section and legislation P2. Seek clarification of legislation P3. Adopt regulation of the area |
| CU9. Attain health & safety training | P1. Take required health and safety training P2. Implement work place hazardous materials information system (WPHMIS) P3. Adopt first aid, cardio for respiratory, resuscitation, and CPR |
| CU10. Prepare for emergencies | P1. Take emergency response training P2. Ensure emergency response exercises P3. Adopt first aid, cardio for respiratory, resuscitation, and CPR |
| CU11. Respond to emergencies | P1. Follow emergency plan P2. Communicate instructions P3. Assess risk and determine course of action P4. Operate emergency equipment and supplies |
| CU12. Monitor activities of people,vehicles, and other equipment in area | P1. Identify movement of others in work area P2. Respond to signals or traffic control person P3. Communicate with site person |
| CU13. Investigate incident at workplace | - Identify incidents causes - Collect relevant data for evidences - Analyze the accident and plan a control measure - Implement the plan |

Knowledge & Understanding

- Define Hazard.

- Describe types of hazard.

- Knowledge and proper use of Personal Protective Equipment (PPE).

- Knowledge about cleaning of workplace with regards to Safety

- Describe Machine & equipment safety. Develop knowledge about Lock-Out Tag Out of machine before making any intervention

- Build competency by acquiring relevant training of HSE

- Describe Occupational Health & Safety Regulations.

- Describe Typical worksite Hazards.

- Describe factors affecting Health & Safety in the workplace.

- Knowledge about cleaning of workplace with regards to Safety

- Describe Machine & equipment safety. Develop knowledge about Lock-Out Tag Out of machine before making any intervention

- Build competency by acquiring relevant training of HSE

- Knowledge about First-Aid-Box.

- Usage of first aid box

- Accident history in different types of industries

- Environment safety.

Tool and Equipment

1. Steel-toed footwear,

2.  Hard hat,

3.  Safety gloves,

4. Appropriate safety glasses,

5. High visibility vest,

6.  Hearing protection,

7.  Breathing apparatus,

8. De-electric boots and gloves for protection from electrical shock.

9. Fall protection, and other applicable PPE

10. Site emergency response plan,

11. Fire extinguishers,

12. Fire blankets,

13. Respirators, masks,

14. Fire hoses,

15. First aid kits, stretchers,

16. WHMIS book,

#### Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare a list of PPEs

- Demonstrate the use of PPEs in front of assessor as per assessors’ directions

- Perform first aid treatment against minor injury.

- Explain safety procedure at workplace

- Differentiate between safe and unsafe tools

- Should be able to perform risk assessment of work site before commencement of work

- List of Machinery, Tools & Equipment.

(FOR A CLASS OF 25 STUDENTS)

## 11.1. Workshop Practice-I

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Work bench | 05 Nos | 1 | Cutting oil | 25 Liters |
| 2 | Bench vices | 25 Nos | 2 | Grese | 10 Kg |
| 3 | Hammer | 25 Nos | 3 | Welding electrode Size 12 No and10 No | 5 Packet each |
| 4 | Tri-square | 25 Nos | 4 | Metal Filler rod | 5 Kg |
| 5 | Hand hacksaw | 25 Nos | 5 | Welding flux | 5 Packet |
| 6 | Scriber | 25 Nos | 6 | Solder wire | 0.5 kg |
| 7 | Vernier caliper | 25 Nos | 7 | Copper Filler rod | 1 kg |
| 8 | Flat File | 25 Nos | 8 | Cotton waste | 10 kg |
| 9 | Number/alphabet punch | 25 Nos | 9 | Kerosene Oil | 10 liters |
| 10 | Round file | 25 Nos |  |  |  |
| 11 | Metal working chisel | 25 Nos |  |  |  |
| 12 | Punching tools | 25 Nos |  |  |  |
| 13 | Tap & Die set | 25 Nos |  |  |  |
| 14 | Pipe cutter | 25 Nos |  |  |  |
| 15 | Pipe vice | 25 Nos |  |  |  |
| 16 | D-type bevel protector | 25 Nos |  |  |  |
| 17 | Grinding Machine | 02 Nos |  |  |  |
| 18 | Personal Protective Equipment | 25 Nos |  |  |  |
| 19 | Wheel Dresser stand | 05 Nos |  |  |  |
| 20 | Dresser | 05 Nos |  |  |  |
| 21 | Lathe Machine | 20 Nos |  |  |  |
| 22 | Cutting Tools | 20 Nos |  |  |  |
| 23 | Measuring Tools | 25 Nos |  |  |  |
| 24 | Files | 25 Nos |  |  |  |
| 25 | Thread gauges | 25 Nos |  |  |  |
| 26 | Threading Tools | 25 Nos |  |  |  |
| 27 | Drilling Machines | 02 Nos |  |  |  |
| 28 | Drill chuck with Key | 05 Nos |  |  |  |
| 29 | Machine Vice | 25 Nos |  |  |  |
| 30 | Marking Tools | 25 Nos |  |  |  |
| 31 | Drill Sleeve and Socket | 05 Nos |  |  |  |
| 32 | Counter drill | 05 Nos |  |  |  |
| 33 | Shaper, Planar or Slotter | 1 No each |  |  |  |
| 34 | Dial indicator with magnet stand | 25 Nos |  |  |  |
| 35 | Point cutting tools | 25 Nos |  |  |  |
| 36 | Bevel protector | 25 Nos |  |  |  |
| 37 | Layout tools. | 25 Nos |  |  |  |
| 38 | Measuring devices. | 25 Nos |  |  |  |
| 39 | Hand held calculator. | 25 Nos |  |  |  |
| 40 | Hand drill machine | 25 Nos |  |  |  |
| 41 | Disk grinder | 05 Nos |  |  |  |
| 42 | Pin grinder | 05 Nos |  |  |  |
| 43 | Oxygen cylinder | 10 Nos |  |  |  |
| 44 | Acetylene gas cylinder | 10 Nos |  |  |  |
| 45 | Pressure regulators | 10 Nos |  |  |  |
| 46 | Cylinder key | 10 Nos |  |  |  |
| 47 | Welding torch | 10 Nos |  |  |  |
| 48 | Rubber house pipe | 10 Nos |  |  |  |
| 49 | Back fire arrester | 10 Nos |  |  |  |
| 50 | Spark lighter | 10 Nos |  |  |  |
| 51 | Steel wire brush | 10 Nos |  |  |  |
| 52 | Pedestal grinder | 02 Nos |  |  |  |
| 53 | Solder gun | 25 Nos |  |  |  |
| 54 | Marking tools | 25 Nos |  |  |  |

## 11.2. Basic Engineering Drawing and CAD/CAM

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Drawing Board/Table. | 25 Nos | 1 | Graph and drawing sheet. | 500 No each |
| 2 | T-Square | 25 Nos | 2 | Templets. | 25 Nos |
| 3 | Set Square. | 25 Nos | 3 |  |  |
| 4 | Geometry Box. | 25 Nos | 4 |  |  |
| 5 | Tee-square and compass. | 25 Nos | 5 |  |  |
| 6 | Computer with all accessories | 25 Nos | 6 |  |  |
| 7 | AutoCAD software disk | 25 Nos | 7 |  |  |
| 8 | Models | 25 Nos | 8 |  |  |

## 11.3. Occupational Health, Safety and Environment

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Steel-toed footwear, | 25 Nos |  |  |  |
| 2 | Hard hat, | 25 Nos |  |  |  |
| 3 | Safety gloves, | 25 Nos |  |  |  |
| 4 | Appropriate safety glasses, | 25 Nos |  |  |  |
| 5 | High visibility vest, | 25 Nos |  |  |  |
| 6 | Hearing protection, | 25 Nos |  |  |  |
| 7 | Breathing apparatus, | 05 Nos |  |  |  |
| 8 | De-electric boots and gloves for protection from electrical shock. | 25 Nos |  |  |  |
| 9 | Site emergency response plan, | 05 Nos |  |  |  |
| 10 | Fire extinguishers, | 05 Nos |  |  |  |
| 11 | Fire blankets, | 25 Nos |  |  |  |
| 12 | Respirators, masks, | 25 Nos |  |  |  |
| 13 | Fire hoses, | 05 Nos |  |  |  |
| 14 | First aid kits, stretchers, | 05 Nos |  |  |  |
| 15 | Safety standard books | 05 Nos |  |  |  |