National Certificate (Level-4) in Mechanical Technology with Specialization in Construction Machinery

Level-4 Equivalent to G-1

Competency Standards

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

Contents

1.	Introduction	5

2.	Purpose of the qualification	6

3.	Summary of Competency Standards	6

4.	Date of validation	9

5.	Codes of Qualifications	9

6.	Qualifications Development Committee	9

7.	Qualifications Validation Committee	10

8.	Entry Requirements	11

9.	Packaging of Qualifications	11

10.	Detail of Competency Standards	13

National Certificate Level-4 Part-1 in Construction Machinery………………………13

10.7.	Problems in IC Engines	13

10.7.1	Diagnose and Adjust Engine Fault	13

10.7.2	Diagnose and adjust faults of engine exhaust system	15

10.7.3	Diagnose faults of engine cylinder head	16

10.7.4	Diagnose faults of engine cylinder block	17

10.7.5	Perform Complete Overhauling of Diesel Engine	18

10.7.6	Diagnose Engine Cooling System Faults and Repair	20

10.7.7	Diagnose Engine Lubrication System Faults and Repair	21

10.7.8	Perform testing of engine with the help of Work Bench	22

10.8.	Workshop Practice – II	23

10.8.1	Recognize Types of tools	23

10.8.2	Identify Gauges	25

10.8.3	Demonstrate Gas Analyser	27

10.8.4	Carry out operation of Head surface grinding machine	29

10.8.5	Carry out operation of Honing Machine	30

10.8.6	Carry out operation of Main line boring machine	31

10.8.7	Carry out operation of Connecting Rod Boring Machine	32

10.8.8	Carry out operation of Crankshaft grinding machine	33

10.9.	Fuel System-I	34

10.9.1	Recognize INLINE Fuel System	34

10.9.2	Recognize PT (Pressure Time) Fuel System	36

10.9.3	Recognize DPA (Distributor Pump Assembly) Fuel System	38

10.9.4	Recognize CRI (Common Rail Injection) Fuel System	40

10.9.5	Recognize MEUI (mechanical electrical unit injector) fuel system	42

10.9.6	Recognize HEUI (hydraulic electrical unit injector) fuel system	44

10.10.	Generic	46

10.10.1	Lead Small Teams	46

10.10.2	Develop Negotiation Skills	48

10.10.3	Solve Problems Related to Work Activities	50

National Certificate Level-4 Part-2 in Construction Machinery………………………52

10.11.	Fuel System-II	52

10.11.1	Maintain inline fuel system	52

10.11.2	Maintain PT (pressure time) fuel system	54

10.11.3	Maintain DPA (distributor pump assembly) fuel system	56

10.11.4	MAINTAIN CRI (Common Rail Injection) Fuel System	58

10.11.5	MAINTAIN MEUI (mechanical electrical unit injector) fuel system	60

10.11.6	Maintain HEUI (hydraulics electrical unit injector) fuel system	62

10.12.	Applied Thermodynamics	64

10.12.1	Analyse Thermodynamic Performance	64

10.12.2	Solve problems related to fundamentals of thermodynamics	66

10.12.3	Solve problems of Laws and properties of perfect gases	67

10.12.4	Derive Thermodynamics Processes and Cycles	68

10.13.	Operate Construction Machinery	70

10.13.1	Identify Machines & Its Attachments	70

10.13.2	Maintain Machines (with Engine Off)	73

10.13.3	Maintain Machines (with Engine Running)	77

10.13.4	Perform Parking of Machines	79

10.13.5	Perform Transportation of Machines	81

10.13.6	Operate Excavator	83

10.13.7	Conduct dump truck operations	87

10.13.8	Operate Bulldozer	91

10.13.9	Operate Wheel Loader	94

10.13.10	Operate Road Roller	96

10.13.11	Operate a Wheel-mounted loading crane	99

10.13.12	Operate Motor Grader	102

10.13.13	Conduct asphalt paver operations	106

10.14.	Generic	109

10.14.1	Develop Communication Skill	109

10.14.2	Develop Administrative Skills	112

11.	List of Tools, Equipment, Machinery and Consumable	113

11.1. Level-4 Part-1 in Construction Machinery………………………………………..113

11.1.1. Problems in IC Engines	113

11.1.2. Workshop Practice – II	113

11.1.3. Fuel System-I	114

11.2. Level-4 Part-2 in Construction Machinery………………………………………..115

11.2.1. Fuel System-II……………………………………………………………………….115

11.2.2. Operation of Construction Machinery…………………………………………..115

11.2.3. Machines and its Attachments…………………………………………………...116

10.2.4. Specification of Machines & Consumable……………………………………..117

# Introduction

Heavy Machine Operators and Technicians are in demand across the country and abroad. Their services are required for everything from road and bridge construction, back hoe operation, bulldozing, loading and grading, dredging and heavy paving, to excavating and much, much more. This is a good career opportunity for a reliable and responsible individual with a strong work ethic. Heavy Machine Operators and Technicians not only work on regular construction building jobs, but also on infrastructure projects (roads, bridges, Dams and ports, otherwise called non-building construction), and in mining and timber operations.

In order to build the capacity of technical and vocational training institutes in Pakistan through provision of demand driven competencies-based trainings in construction sector the NAVTTC, and TEVT Sector Support Program (TSSP) have joined hands together to develop qualifications for construction sector. These qualifications will not only build the capacity of existing workers of this sector but also support the youth to acquire skills best fit for this sector. The benefits and impact of development of these qualifications will be on both demand and supply side.

Based upon this demand of industry these competency‐based qualifications for Heavy Construction Machinery technology are developed under National Vocational Qualification Framework (NVQF) (Level 4).  The qualifications mainly cover competencies along with related knowledge and professional skills which are essential for getting a job or self-employed.

The National Vocational & Technical Training Commission (NAVTTC) has approved the Qualification Development Committee (QDC). The QDC consists experts from the relevant industries from different geographical locations across Pakistan and academicians who were consulted during the development process to ensure input and ownership of all the stakeholders. The National Competency Standards could be used as a referral document for the development of curricula to be used by training institutions.

Heavy Machine Operator and Technician curriculum will prepare students to efficiently operate heavy machines such as Dozers, Loaders, Excavator and Graders and to perform basic preventive maintenance on most types of heavy equipment. Coursework includes Safety, Heavy Machines operations, grades, Legal & environmental concerns & equipment maintenance. Graduates of this program may find employment with state and local government agencies and private contractors engaged in highway or other construction activities.

In fact, no sphere of life may be identified that does not include the contribution of construction machinery. Thus, the importance of construction machinery may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechanical technology with Specialization in Construction Machinery under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# Purpose of the qualification

The purpose of these qualifications is to set high professional standards for Construction Machinery. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for Construction Machinery.

- Enable the existing workforce to capacitate themselves in new technologies and method so that train persons in this field can maintain construction machinery for its longer life.

# Summary of Competency Standards

The Construction Technology qualifications of level-4 consists 20% Theory and 80% Practical. The Core competencies of the qualification are as follows:

Level-4 (Part-1)

Problems in Internal Combustion Engines

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-45 | Diagnose and Adjust Engine Fault | 4 | 7 | 21 | 28 | 2.8 | Technical |
| CS-46 | Diagnose and adjust faults of engine exhaust system | 4 | 4 | 15 | 19 | 1.9 | Technical |
| CS-47 | Diagnose faults of engine cylinder head | 4 | 6 | 21 | 27 | 2.7 | Technical |
| CS-48 | Diagnose faults of engine cylinder block | 4 | 4 | 15 | 19 | 1.9 | Technical |
| CS-49 | Perform Complete Overhauling of Diesel Engine | 4 | 6 | 39 | 45 | 4.5 | Technical |
| CS-50 | Diagnose Engine Cooling System Faults and Repair | 4 | 7 | 18 | 25 | 2.5 | Technical |
| CS-51 | Diagnose Engine Lubrication System Faults and Repair | 4 | 7 | 18 | 25 | 2.5 | Technical |
| CS-52 | Perform Testing of engine with the help of Work Bench | 4 | 7 | 30 | 37 | 3.7 | Technical |
|  | Total |  | 48 | 177 | 225 | 22.5 |  |

Workshop Practice-II

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-53 | Recognize types of Tools | 4 | 6 | 21 | 27 | 2.7 | Technical |
| CS-54 | Identify Gauges | 4 | 6 | 18 | 24 | 2.4 | Technical |
| CS-55 | Demonstrate Gas Analyzer | 4 | 6 | 21 | 27 | 2.7 | Technical |
| CS-56 | Carry out operation of Head surface grinding machine | 4 | 4 | 24 | 28 | 2.8 | Technical |
| CS-57 | Carry out operation of Honing Machine | 4 | 4 | 21 | 25 | 2.5 | Technical |
| CS-58 | Carry out operation of Main line boring machine | 4 | 4 | 24 | 28 | 2.8 | Technical |
| CS-59 | Carry out operation of Connecting Rod Boring Machine | 4 | 4 | 21 | 25 | 2.5 | Technical |
| CS-60 | Carry out operation of Crankshaft grinding machine | 4 | 4 | 21 | 25 | 2.5 | Technical |
|  | Total |  | 38 | 171 | 209 | 20.9 |  |

Fuel System - I

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-61 | Recognize Inline Fuel system | 4 | 5 | 18 | 23 | 2.3 | Technical |
| CS-62 | Recognize PT (Pressure Time) Fuel System | 4 | 5 | 18 | 23 | 2.3 | Technical |
| CS-63 | Recognize DPA (Distributor Pump Assembly) Fuel System | 4 | 6 | 18 | 24 | 2.4 | Technical |
| CS-64 | Recognize CRI (Common Rail Injection) Fuel System | 4 | 6 | 18 | 24 | 2.4 | Technical |
| CS-65 | Recognize MEUI (Mechanical Electrical Unit Injector) Fuel System | 4 | 6 | 18 | 24 | 2.4 | Technical |
| CS-66 | Recognize HEUI (Hydraulic Electric Unit Injector) Fuel System | 4 | 6 | 18 | 24 | 2.4 | Technical |
|  | Total |  | 34 | 108 | 142 | 14.2 |  |

Generic

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-67 | Lead Small Teams | 4 | 2 | 6 | 8 | 0.8 | Generic |
| CS-68 | Develop Negotiation Skills | 4 | 2 | 6 | 8 | 0.8 | Generic |
| CS-69 | Solve Problems Related to Work Activities | 4 | 2 | 6 | 8 | 0.8 | Generic |
| Total |  |  | 6 | 18 | 24 | 2.4 |  |
| Grand Total of Level-4 (Part-1) |  |  | 126 | 474 | 600 | 60 |  |

Level-4 (Part-2)

Fuel System - II

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-70 | Maintain Inline Fuel System | 4 | 3 | 21 | 24 | 2.4 | Technical |
| CS-71 | Maintain PT (Pressure Time) Fuel System | 4 | 3 | 21 | 24 | 2.4 | Technical |
| CS-72 | Maintain DPA (Distributor Pump Assembly) Fuel System | 4 | 3 | 21 | 24 | 2.4 | Technical |
| CS-73 | Maintain CRI (Common Rail Injection) Fuel System | 4 | 3 | 30 | 33 | 3.3 | Technical |
| CS-74 | Maintain MEUI (Mechanical Electrical Unit Injector) Fuel System | 4 | 3 | 27 | 30 | 3 | Technical |
| CS-75 | Maintain HEUI (Hydraulic Electric Unit Injector) Fuel System | 4 | 4 | 33 | 37 | 3.7 | Technical |
|  | Total |  | 19 | 153 | 172 | 17.2 |  |

Thermodynamics

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-76 | Analyze Thermodynamic Performance | 4 | 8 | 12 | 20 | 2 | Functional |
| CS-77 | Solve Problems Related To Fundamentals of Thermodynamics | 4 | 8 | 12 | 20 | 2 | Functional |
| CS-78 | Solve Problems of Laws And Properties of Perfect Gases | 4 | 8 | 12 | 20 | 2 | Functional |
| CS-79 | Derive Thermodynamics Processes and Cycles | 4 | 8 | 21 | 29 | 2.9 | Functional |
|  | Total |  | 32 | 57 | 89 | 8.9 |  |

Operation of Construction Machinery

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-80 | Identify Machines & its Attachments | 4 | 3 | 12 | 15 | 1.5 | Technical |
| CS-81 | Maintain Machines (with Engine Off) | 4 | 2 | 9 | 11 | 1.1 | Technical |
| CS-82 | Maintain Machines (with Engine Running) | 4 | 2 | 6 | 8 | 0.8 | Technical |
| CS-83 | Perform Parking of Machines | 4 | 2 | 9 | 11 | 1.1 | Technical |
| CS-84 | Perform Transportation of Machines | 4 | 2 | 6 | 8 | 0.8 | Technical |
| CS-85 | Operate Excavator | 4 | 6 | 36 | 42 | 4.2 | Technical |
| CS-86 | Conduct dump truck operations | 4 | 3 | 12 | 15 | 1.5 | Technical |
| CS-87 | Operate Bulldozer | 4 | 3 | 33 | 36 | 3.6 | Technical |
| CS-88 | Operate wheel Loader | 4 | 3 | 33 | 36 | 3.6 | Technical |
| CS-89 | Operate Road Roller | 4 | 4 | 33 | 37 | 3.7 | Technical |
| CS-90 | Operate a wheel-mounted loading crane | 4 | 4 | 33 | 37 | 3.7 | Technical |
| CS-91 | Operate Motor Grader | 4 | 6 | 36 | 42 | 4.2 | Technical |
| CS-92 | Conduct asphalt paver operations | 4 | 4 | 21 | 25 | 2.5 | Technical |
|  | Total |  | 44 | 279 | 323 | 32.3 |  |

Generic

| Code | Competency Standards | Level | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS-93 | Develop Communication Skill | 4 | 2 | 6 | 8 | 0.8 | Generic |
| CS-94 | Develop Administrative Skills | 4 | 2 | 6 | 8 | 0.8 | Generic |
| Total |  |  | 4 | 12 | 16 | 1.6 |  |
| Grand Total of Level-4 (Part-1) |  |  | 126 | 474 | 600 | 60 |  |
| Grand Total of Level-4 (Part-2) |  |  | 99 | 501 | 600 | 60 |  |
| Grand Total of Level-4 |  |  | 225 | 975 | 1200 | 120 |  |

# Date of validation

The level 4 of National qualification on Construction Technology has been validated by the Qualifications Development Committee (QDC) members on ………… and will remain in currency until …………………………

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Construction Machinery level-5 |  |
| --- | --- |
| Code | Description |
| 000000000 | National Certificate of level-2, in “Construction Machinery” |
| 000000000 | National Certificate of level-3, in “Construction Machinery” |
| 000000000 | National Certificate of level-4, in “Construction Machinery” |
| 000000000 | National Certificate of level-5, in “Construction Machinery” |

# Qualifications Development Committee

The following members participated in the qualification’s development workshop 18th March 2019

To 22nd March 2019 at Grand Palm Hotel, Lahore:

| S.No. | Name & Designation | Organization |
| --- | --- | --- |
|  | Muhammad Ishaq, Dy Director (TE) | NAVTTC, HQ |
|  | Muzammal Hussain, Instructor (Auto & Diesel) | GCT, Rawalpindi, TEVTA |
|  | Muhammad Ejaz, HOD, Mechanical Department | C.T.T.I, Islamabad |
|  | Hassan Raza, Jr. Instructor | C.T.T.I, Islamabad |
|  | Umer Farooq, Instructor, Mechanical | G.S.P.C.T Gujrat |
|  | Muhammad Rizwan Sarwar, Sr. Instructor (Auto Mechanic) | G.T.T.I, Shiekhupura |
|  | Hafiz Ghulam Mohyudin, Asst. Manager Services | Dynamic Equipment & Control, Lahore |
|  | Mehr Ali Qureshi, Sr. Service Engineer | Murshid Builders (PVT) Limited, Lahore |
|  | Syed Hussein-el-Edroos, Sr. Service Engineer | Independent Consultant |
|  | Waqar Ahmed, Service Manager | Jaffer Brothers (PVT) Limited, Lahore |
|  | Ghulam Dastgir, Incharge Projects | Ex-DESCON Engineering, Lahore |
|  | Zain-ul-Abideen, Instructor | G.T.T.I, Mughalpura, Lahore |
|  | Atif Waheed Abbasi, Heavy Machine Operator | National Logistic Cell Mandra, Islamabad |
|  | Sikander Mehmood Jr. Instructor | C.T.T.I, Islamabad |
|  | Engr. Mohsin Jahanzeb Instructor | GSTC,Faisalabad |
|  | Muhammad Amjad Iqbal Service Engineer | Greaves Pakistan Ltd,Lahore |
|  | Sayed Fayyaz Mustafa Naqvi J. Instructor Civil | GSTC,Faisalabad |
|  | Saba Sadiq, MS Mechanical Engineer/ DACUM Facilitator, | University of Lahore, Lahore |

# Qualifications Validation Committee

The following members participated in the qualification development and validation workshop on:

| S.No. | Name & Designation | Organization |
| --- | --- | --- |
|  | Mr. Muhammad Ejaz | HOD, CTTI, Islamabad |
|  | Mr. Hassan Raza | Instructor, CTTI, Islamabad |
|  | Mr. Muzammal Hussain | Instructor (Auto & Diesel) , GCT, Rawalpindi |
|  | Mr. Sikandar Masood | Director, NAVTTC, Islamabad |
|  | Mr. Abdur Rehman | Representative from PBTE, Lahore |
|  | Mr. Zafar Iqbal | Representative from KP-TEVTA |
|  | Mr. Rafique Ahmed Shaikh | Representative from Singh-TEVTA |
|  | Mr. Naveed Ahmed Ajmal | Representative from P-TEVTA |
|  | Mr. Irfan Khalil | Representative from Sector Skill Council/ ABAD Karachi |
|  | Saba Sadiq | DACUM, Facilitator, Lahore (UOL) |

# Entry Requirements

The entry for National Certificate level-4, in Construction Technology are

- A person having National Vocational Certificate level-3, in Construction Technology.

# Packaging of Qualifications

The National Vocational Qualifications have been packaged as detailed below:

| Level | Name of Occupation/Nomenclature | Generic Competencies | Allied competencies require | Core /Technical Competencies required |
| --- | --- | --- | --- | --- |
| - Diesel Engine Technician |  |  |  |  |
| Level-2 | Pro Diesel Mechanic | 150,151,152,153,154 |  | 1,2,3,4,5,6,7 |
| Level-3 | Engine Rebuilder | 150,151,152,153,154 |  | 44,45,46,47,48,49,50,51,52 |
| Level-3 | Generator Technician | 150,151,152,153,154 |  | 61,62,63,64,65,66 |
| Level-3 | Engine Technician | 149,150,151,152,153,154 |  | 53,54,55,56,57,58,59,60 |
| Level-3 | Pro Technician Diesel Pump | 149,150,151,152,153,154 |  | 67,68,69,70,71,72 |
| Level-4 | Diesel Pump Technician | 140,141,142,143,144,145,146,147,148, 149,150,151,152,153,154 |  | 128,129,130,131,132,133 |
| Level-4 | CNC Machine operator | 150,151,152,153,154 |  | 15,16,17,18,19,20,21,22,23,24,25,26,27 |
| Level-4 | Thermo Man | 150,151,152,153,154 |  | 39,40,41,42,43 |
| - Electrician |  |  |  |  |
| Level-2 | Auto Electrician | 150,151,152,153,154 |  | 28,29,30,31,32,33,34,35,36,37,38 |
| - Engineering Drawing |  |  |  |  |
| Level-3 | Draft Man | 152,153,154 |  | 8,9,10,11,12,13,14 |
| - Heavy Machine Operator |  |  |  |  |
| Level-3 | Excavator Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,78 |
| Level-3 | Bulldozer Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,80 |
| Level-3 | Dump truck Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,79 |
| Level-3 | Wheel Loader Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,81 |
| Level-3 | Wheel Mounted Loading Crane Operator | 149,150, 151,152,153,154 |  | 73,74,75,76,77,83 |
| Level-4 | Master operator (Excavator, Bulldozer, Dump truck, Wheel Loader, Grader, Crane and asphalt paver) | 149,150,151,152,153,154,155 |  | 73,74,75,76,77,78,79,80,81,82,83,84,85 |
| - Heavy Machinery Mechanic |  |  |  |  |
| Level-5 | Construction Machinery Transmission Technician | 149, 150,151,152,153,154 |  | 86,87,88,89,90,91,92,93 |
| Level-5 | Steering and Brake Mechanic of Construction Machinery | 149, 150,151,152,153,154 |  | 94,95,96,97,98,99,100,101,102,103 |
| level-5 | Under Carriage Mechanic of Construction Machinery | 149, 150,151,152,153,154 |  | 104,105,106,107,108,109,110 |
| Level-5 | Hydraulic Mechanic of Construction Machinery | 149, 150,151,152,153,154 |  | 111,112,113,114, 115,116,117,118 |
| Level-5 | Hydraulic Excavator Mechanic of Construction Machinery | 140,141,142,143,144,145,146,147,148,149, 150,151,152,153,154 |  | 119,120,121,122,123,124,125,126,127 |
| Level-5 | Construction field Manager | 140,141,142,143,144,145,146,147,148,149, 150,151,152,153,154 |  | 134,135,136,137,138,139 |

# Detail of Competency Standards

## National Certificate Level-4 Part-1 in Construction Machinery

# Problems in IC Engines

- Diagnose and Adjust Engine Fault

Overview: These competency standards designed to enable the learner to find out the faults and rectify accordingly, learner will be able to overhaul the engine under the specified format in the service manual.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Diagnose Engine fault | - Start the engine and identify the condition of engine rpm fluctuating, stalling, and lack of power, excessive smoke (Black, blue and white), excessive fuel consumption, excessive oil consumption, and leakages. - Use the compression tester and test engine compression to diagnose fault in piston rings or valve seats/burnt valve under specified procedure. |
| - Adjust Engine fault | - Perform engine adjustment/setting of Engine Timing, fuel injector problem, valve adjustment, drive belt/fan belt adjustment. - Change the engine cylinder head gasket, Crankshaft oil seals, Engine timing belt, manifold gasket, and exhaust muffler. - Check Lubricating systems. - Check fuel systems - Check cooling systems. |

Knowledge & Understanding

- Describe the following.

Valve timing, Injector advanced timing and retard timing, overlap, timing diagrams, valve, clearances/ adjustment/ adjustment methods.

- Terms: Top Dead Centre (TDC), Bottom Dead Centre (BDC), Bore, Stroke, Capacity, Clearance Volume, Swept Volume, Compression Ratio, Crank Throw, Torque at Revs/Min, Brake power at Revs/min, Specific fuel consumption (SFC), Volumetric Efficiency, OHV, and OHC.

- Understand different troubleshooting chart of different problem of engine

Critical Evidence(s) Required

- Capable of adjustments.

- Capable of changing faulty components.

List of Tools and Equipment

1	Tools trolley (complete set of tools)

2	Compression testers, compression gauge)

3	Measuring tools (Vernier caliper, micrometer, bore gauge, dial gauge, Steel rule, filler gauge, try square)

4	Torque wrenches

5	Impact guns

6	Oil can

7	Belt tensioners

8	Spray gun

9	Valve lapping tool

10	Timing gun

11	Engine analyzer

12	Tachometer

13	Scraper

Consumable Items:

1	Fuel (petrol/diesel)

2	Cotton rag

3	WD 40 spray

4	Carb cleaner spray

5	Kerosene oil

6	Engine oil

7	Engine coolant

8	fan belt

9	Timing belt

10	oil seals

11	Gasket

12	Silicone

13	Emery paste

- Diagnose and adjust faults of engine exhaust system

Overview: These competency standards designed to enable the learner to find Trace Faults of exhaust system and adjust faults exhaust system.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Trace Faults of exhaust system | - Inspect exhaust manifold. - Inspect diesel particulate filter. - Inspect exhaust pipe. - Inspect muffler assembly. - Inspect tail pipe. - Identify the factors influencing power of engine |
| - Adjust faults exhaust system | - Troubleshoot exhaust manifold. - Troubleshoot diesel particulate filter. - Troubleshoot exhaust pipe. - Troubleshoot muffler assembly. - Troubleshoot tail pipe. |

Knowledge & Understanding

- Describe exhaust system components.

- Describe exhaust system problems.

- Describe influencing power of engine.

Critical Evidence(s) Required

- Capable inspect exhaust system components

- Capable to troubleshoot exhaust system components

List of Tools and Equipment

1	Tools trolley (complete set of tools)

2	Special service tools

- Diagnose faults of engine cylinder head

Overview : These competency standards designed to enable the learner to inspect cylinder head components and troubleshoot cylinder head components.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Inspect cylinder head components | - Inspect cylinder valve - Measure clearance of valve guide. - Measure clearance of valve seat. - Measure tension of valve spring - Inspect rocker arm - Inspect rocker arm shaft. - Inspect push rod - Measure tappets cover clearance. - Inspect cam shaft - Inspect cam lob. - Inspect cylinder head intake port. - Inspect cylinder head exhaust port. |
| - Troubleshoot cylinder head components | - Troubleshoot/replace cylinder valve - Replace valve guide. - Replace valve seat. - Replace valve spring - Troubleshoot/replace rocker arm - Troubleshoot/replace rocker arm shaft. - Troubleshoot/replace push rod - Troubleshoot/replace tappet cover - Troubleshoot/replace cam shaft - Troubleshoot cylinder head intake port. - Troubleshoot cylinder head exhaust port. |

Knowledge & Understanding

- Describe valve problems.

- Describe camshaft problems.

- Describe engine sounds

- Describe cylinder head problems

- Describe cylinder head gasket problems.

Critical Evidence(s) Required

- Capable of inspect cylinder head components.

- Able to troubleshoot cylinder head components.

- Able to replace cylinder head components.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (complete set of tools) |
| 2 | Engine |

- Diagnose faults of engine cylinder block

Overview : These competency standards designed to enable the learner to inspect cylinder block components and troubleshoot cylinder block components.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Inspect cylinder block components | - Inspect crank shaft - Measure crank shaft end play - Measure crank shaft ovality - Inspect connecting rod - Inspect piston pin - Inspect piston head - Inspect piston skirt - Inspect compression rings - Inspect oil ring - Inspect cylinder sleeve - Measure cylinder sleeve dia - Measure cylinder sleeve ovality - Measure cylinder sleeve tapperness |
| - Troubleshoot cylinder block components | - Rebuild/Replace crank shaft - Rebuild/Replace connecting rod - Replace piston pin - Replace piston - Replace compression rings - Replace oil ring - Replace cylinder sleeve |

Knowledge & Understanding

- Describe piston problems.

- Describe piston ring problems

- Describe sleeves problems.

- Describe connecting rod problems.

- Describe crank shaft problems.

- Describe piston pin problems.

Critical Evidence(s) Required

- Capable of inspect cylinder block components.

- Able to replace cylinder block components.

- Able to rebuild cylinder head components.

List of Tools and Equipment

| 1 | Tools trolley (complete set of tools) |
| --- | --- |

- Perform Complete Overhauling of Diesel Engine

Overview: These competency standards designed to enable the learner to find out the faults and rectify accordingly, learner will be able to overhaul the engine under the specified format in the service manual.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Engine sub-assemblies | - Make the engine sub-assemblies and inspect the performance for (setting piston rings, piston pin and connecting road), - Make the engine sub-assemblies and inspect the performance for (valve train and test for the leakage from valve seats), - Make the engine sub-assemblies and inspect the performance for (fitting crankshaft with main bearings and pistons with the big end bearings), - Perform the straightness inspect of engine head and block straightness/warpage level and make decision for correctness. |
| - Engine measurement | - Use appropriate measuring tool and Measure piston bore, measure crankshaft main journals (dia and ovality), crankshaft big end journals, - Use appropriate measuring tool and Measure Camshaft main journals (dia and ovality), Cam loops, - Use appropriate measuring tool and Measure engine thrust washer, and piston connecting pin. - Assemble the engine block with the engine head and adjust tappets, engine timing, and injector distributor of diesel in engine under specified procedure. - Check cylinder liners ovality. |

Knowledge & Understanding

- The possible causes of high fuel consumption as following.

Overload the engine, excessive oil reducing, faulty air filter, carburetor poor setting, engine over cooled and hard/jam running of engine, faulty fuel injectors, faulty fuel pump/fuel filter.

Critical Evidence(s) Required

- Capable to take decision for engine overhauling.

- Capable of knowing the engine faults

- Capable to assemble the engine.

- Capable to measure the engine components

List of Tools and Equipment

1	Tools trolley (complete set of tools)

2	Compression tester, compression gauge)

3	Measuring tools (Vernier caliper, micrometer, bore gauge, dial indicator gauge, Steel rule, filler gauge, try square, mercer gauge)

4	Torque wrench

5	Impact gun

6	Oil can

7	Belt tensioner

8	Spray gun

9	Valve lapping tool

10	Timing gun

11	Engine analyzer

12	Tachometer

13	Scraper

Consumable Items:

1	Fuel (petrol/diesel)

2	Cotton rag

3	WD 40 spray

4	Carb cleaner spray

5	Kerosene oil

6	Engine oil

7	Engine coolant

8	fan belt

9	Timing belt

10	oil seals

11	Gasket

12	Silicone

13	Emery paste

- Diagnose Engine Cooling System Faults and Repair

Overview: These competency standards designed to enable the learner to diagnose the fault related to the vehicle cooling system and troubleshoot accordingly.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Cooling system service | - Inspect engine cooling system’s main components and diagnose the performance of (Radiator cap, Radiator fan, water body/ pump, drive belt/ fan belt, thermostat Valve, radiator, Drain plug, hoses/ clamps, and gasket/ seals) - Flush the cooling system. - Clean the radiator core with air by revising the direction of rotation of fan motor. - Inspect/ service the radiator and top up with the coolant/ antifreeze as per specified ratio in the manual. |
| - Repair Cooling system | - Change the water body under specified procedure and start the engine to confirm correct operation. - Adjust fan belt tension under specified procedure and start the engine to confirm correct operation. |

Knowledge & Understanding

- Describe the operating principles and terminologies of cooling system as following.

Air cooled engine, liquid cooled engine, operating temperature, thermal efficiency, pressurized system, heat exchange method (conduction, radiation, and convection), corrosion, and inhibitors/ antifreeze)

- Describe the functions and purpose of engine cooling system components as following.

Electrical motor operated fan with thermo switch, belt operated fan, temperature controlled/viscous coupling fan, pressure cap, water body/pump, drive belt, thermostat, radiator, core plug, hoses/clamps, and gasket/ seals.

Critical Evidence(s) Required

- Capable of diagnose, adjust and rectify the cooling system faults.

List of Tools and Equipment

1	Hand tools trolley

2	Radiator leak tester

3	Vehicle/ simulator with a cooling system

5	Drive belt

Consumable Items:

1.		Sealant

2.		Hose pipe

3.		Thermostat valve

- Diagnose Engine Lubrication System Faults and Repair

Overview

These competency standards designed to enable the learner to diagnose the fault related to the lubrication system and troubleshoot accordingly.

| Units of Competency | Performance Criteria |
| --- | --- |
| - Lubrication system | - Inspect engine lubrication system’s main components and check (engine oil level) - Inspect engine lubrication system’s main components and check (oil leakage) - Inspect engine lubrication system’s main components and check (oil warning switch) - Inspect engine lubrication system’s main components and check (PCV valve) - Check the remaining hours of engine oil and filter from monitor screen. |
| - Repair Lubrication system | - Inspect/replace of oil warning switch, PCV valve, oil sump, oil sump seal/gasket, and oil pump under the specified procedure. - Carryout safe procedure to change the engine oil and oil filter. |

Knowledge & Understanding

- Describe the functions/operating principle and main components of engine lubrication system as following.

Wet sump, dry sump, total loss, Oil pumps and its types, pressure relief valves, oil filter, oil cooler, ventilation/ PCV Valve, hoses, and oil level indicators.

Lubricant (oils viscosity, viscosity index, oil classifications, Reduction of frictional forces, cooling effect, cleaning effect, corrosion resistance, and noise reduction).

- Describes the procedure to change the engine oil and oil filter.

Critical Evidence(s) Required

- Capable of diagnose, adjust and rectify the lubrication system faults.

- Capable to change the engine oil and filter.

List of Tools and Equipment

| S No. | Description |
| --- | --- |
| 1 | Hand tools trolley |
| 2 | Lifter |
| 3 | Vehicle/ simulator with a cooling system |
| 4 | Sealant/gasket |

- Perform testing of engine with the help of Work Bench

Overview These competency standards designed to diagnose and troubleshoot different types of engines as well as conceptualize the fuel injection system mechanism in the engine.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Work bench of 200 HP engine | - Check the Engine Torque, oil Temperature, coolant temperature, fuel temperature and fuel pressure. - Check the periodic duration of fuel filter, oil filter and air filter. - Diagnose the electrical faults related to engine. |
| - Work bench of C9 engine | - Attach the ET (Electronic Technician) with the Engine with the described in the manual. - Check the Engine Torque, oil Temperature, coolant temperature, fuel temperature and fuel pressure. - Check the periodic duration of fuel filter, oil filter and air filter. - Diagnose the electrical faults related to engine. - Diagnose the fault of fuel injectors. - Check the Fuel delivery amount. - Preform calibration of injector. |

Knowledge & Understanding

- Describe the Purpose and function of workbench.

- Describe Nomenclature of engine.

- Micron rating of fuel filter, air filter and oil filter.

- Describe the function of TECH-2.

- Describe the Purpose and function of ET tester.

- Describe Temperature, pressure , velocity and torque

Critical Evidence(s) Required

- Troubleshot the vehicle Electrical system.

- Capable to adjust the injector timing

- Capable to service the fuel system.

- Capable to use TECH_2 Tester

- Capable to use ET Tester.

List of Tools and Equipment

| S No. | Description |
| --- | --- |
|  | - Engine/ Simulator - TECH-2 tester - ET Tester |
|  | - Digital Multimeter (DMM). |
|  | - Tools Box |

# Workshop Practice – II

- Recognize Types of tools

Overview: This competency standard identifies the competencies you need to identify standard tools and identify special service tools.

| Competency Units | Performance Criteria |
| --- | --- |
| - Identify standard tools | - Identify different types of spanners. - Identify different types of sockets. - Identify different types of wrenches. - Identify different types of screw drivers. - Identify different types of pillars. - Identify different types of snap ring extractors. - Identify different types of snap ring installers. - Identify different types of hammers. - Identify different types of bars. - Identify different types of hexagonal Wrenches. |
| - Identify special service tools | - Identify different types of pullers. - Identify different types of cranes. - Identify different types of hydraulic jacks. - Identify different components of flaring tools. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe different sizes of spanners.

- Describe different sizes of sockets.

- Describe different sizes of wrenches.

- Describe different sizes of screw drivers.

- Describe different sizes of pillars.

- Describe different sizes of snap ring extractors.

- Describe different sizes of snap ring installers.

- Describe different sizes of hammers.

- Describe different sizes of bars.

- Describe different sizes of hexagonal Wrenches.

- Describe different types of Cranes.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard.

- Capable to understand Spanners.

- Capable to understand sockets.

- Capable to understand wrenches.

- Capable to understand screw drivers.

- Capable to understand pillars.

- Capable to understand snap ring extractors and installers.

- Capable to understand hammers.

- Capable to understand bars

- Capable to understand flaring Tools.

- Capable to understand cranes.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Special Service Tools |

- Identify Gauges

Overview: This competency standard identifies the competencies you need to maintain Compression Gauge, vacuum gauge and mercer gauge.

| Competency Units | Performance Criteria |
| --- | --- |
| - Maintain Compression Gauge | - Identify different types of Compression gauge. - Remove Spark Plug/glow plug. - Select the adopter as per requirement. - Install compression gauge on engine. - Crank the engine. - Get three reading and get average. - Compare reading with chat. |
| - Maintain Vacuum Gauge | - Remove take of port adopter from intake manifold. - Select the adopter as per requirement. - Install vacuum gauge on intake manifold. - Crank the engine. - Get three reading and get average. - Compare reading with chat. |
| - Maintain Mercer Gauge | - Identify rod. - Identify dial indicator. - Identify extension. - Identify extension rod. - Identify adopter. - Identify shims. - Identify contact point. - Check cylinder bore Dia. - Check cylinder bore ovality. - Check cylinder bore tapperness. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe compression gauge.

- Describe tests of compression gauge.

- Describe vacuum gauge.

- Describe procedure of vacuum gauge.

- Describe mercer gauge.

- Describe procedure to check cylinder bore dia.

- Describe procedure to check cylinder bore ovality.

- Describe procedure to check cylinder bore tapperness.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard.

- Capable to check the compression of engine.

- Capable to check the vacuum of engine.

- Capable to check the cylinder bore dia.

- Capable to check the cylinder bore ovality.

- Capable to check the cylinder bore tapper-ness.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Compression Gauge |
| 3 | Vacuum Gauge |
| 4 | Mercer Gauge |
| 5 | Dial Indicator |

- Demonstrate Gas Analyser

Overview: This competency standard identifies the competencies you need to maintain four gas analyse and handy smoke tester.

| Competency Units | Performance Criteria |
| --- | --- |
| - Demonstrate Four Gas Analyzer | - Identify transformer. - Identify primary filters. - Identify secondary filters. - Identify probe. - Point out main screen. - Connect probe with gas analyzer. - Connect transformer with gas analyzer. - Plug in transformer wire in electric socket. - Put probe in tail pipe of vehicle. - Start the engine. - Get readings. |
| - Demonstrate Handy Smoke Tester | - Identify main feed pump. - Identify paper filters. - Identify probe. - Identify handle. - Identify suction holes. - Put probe in tail pipe. - Put the paper in taster. - Start the engine. - Put engine on high idle. - Pull the handle of tester to suck the exhaust gasses. - Remove the filter. - Match the color of filter paper with chat. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe four gas analyzer.

- Describe the procedure to use four gas analyzer.

- Describe handy smoke tester.

- Describe the procedure to use handy smoke tester.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard.

- Capable to use four gas analyzer.

- Capable to use handy smoke tester.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Four Gas Analyzer |
| 3 | Handy Smoke Tester |

- Carry out operation of Head surface grinding machine

Overview: This competency standard identifies the competencies you need to perform Identify the machine and operate the machine.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify the machine | - Point out Magnetic Table - Locate Grinding Wheel - Locate Wheel Flange - Point out Balancing Mill - Point out Dressing Apparatus - Point out Extracting Device - Locate Parallel Stands - Locate Clamping device - Locate Lightening Lamp - Point out Cooling Unit - Point out Hydraulic Unit - Locate Tool Set |
| CU2. Operate the machine | - Clamp/Bolt the head on the machine - Set the Dial Gauge to Zero for alignment of head - Fix the grinding stone - Switch on the Cooling unit - Set the desired grinding depth |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Learn to identify common faults

- Learn to identify Parts of machines

- Capable to perform daily check-ups

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard.

- Capable to perform daily check ups

- List of Tools and Equipment

1	Tools trolley (Complete set of Hand Tools)

2	Head Surface Grinding Machine

3	Grinding Tools

- Carry out operation of Honing Machine

Overview: This Competency Standard identifies the competencies required to Identify  and operate the honing machine.

| Competency Units | Performance Criteria |
| --- | --- |
| C1. Identify the machine | - Locate Honing head - Point out Head Extension Spindle & Sleeve - Point out Hydraulic Clamping apparatus - Locate Bore Centring flange - Locate Moveable Head Holder - Point out Control Levers - Point out Lubrication Tank - Locate On-Off Switches |
| C2. Operate the machine | - Centring of sleeve - Select grinding/Honing stone according to dia of sleeve - Fit the grinding/Honing stone in machine - Switch on lubrication oil circulation - Operate Tool UP/Down - Operate tool in circular motion |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Learn to identify common faults

- Learn to identify Parts of machines

- Capable to perform daily check-ups.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to identify common faults

- Capable to identify Parts of machines

- Capable to perform daily check-ups.

List of Tools and Equipment

1	Tools trolley (Complete set of Hand Tools)

2	Honing Machine

3	Honing Tool

- Carry out operation of Main line boring machine

Overview: This competency standard identifies the competencies you need to perform identify and operate the boring machine.

| Competency Units | Performance Criteria |
| --- | --- |
| D1. Identify the machine | - Point out Spindle - Locate Joint - Point out Cutter adjusting micrometer - Identify Centering apparatus - Point out Parallel Support - Point out Leveling apparatus - Locate Block clamping apparatus |
| D2. Operate the machine | - Setting of the machine - Fasten the Cylinder Block to the machine - Fix & centre the cutting tool - Align the Cylinder Block - Set the cutting depth - Set the desired speed - Switch on the machine |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Learn to identify common faults

- Learn to identify Parts of machines

- Capable to perform daily check-ups.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to identify common faults

- Capable to identify Parts of machines

- Capable to perform daily check-ups.

- List of Tools and Equipment’s

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Main Line Boring Machine |
| 3 | Boring Tools |

- Carry out operation of Connecting Rod Boring Machine

Overview: This Competency Standard identifies the competencies required to identify and operate the machine.

| Competency Units | Performance Criteria |  |
| --- | --- | --- |
| B1. Identify the machine | - Identify the spindle - Identify the clamping apparatus - Identify the work piece centring apparatus - Identify the feed apparatus - Identify the Lubrication Pump - Identify the drive motor |  |
| B2. Operate the machine |  | - Setting of the machine - Fasten the connecting rod on the machine - Fix & centre the cutting tool - Centring of Connecting Rod - Set the cutting depth - Switch on the machine |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Knowledge of machine component

- Knowledge of machine operation

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to perform work on connecting rod boring machine

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Connecting Rod Boring Machine |
| 3 | Cutting Tools |

- Carry out operation of Crankshaft grinding machine

Overview: This competency standard identifies the competencies you need to identify and operate the machine.

| Competency Units | Performance Criteria |
| --- | --- |
| E1. Identify the machine | - Point out Crankshaft measuring device - Locate Wheel dresser apparatus - Point out Crankshaft resetting device - Point out Shaft Centering Apparatus - Locate Steady Supporting base - Locate Wheel balancing Plate & Shaft - Locate Grinding Wheel - Point out Cooling Unit - Identify Hydraulic System - Point out Steady Tail Stock - Point out Front Weight - Locate Rear Weight - Locate Wheel Extracting Key - Locate Chuck Key |
| E2. Operate the machine | - Fasten the crankshaft on the machine - Balancing & setting of crank shaft on the machine - Fix grinding wheel & set the distance. - Fix the sand paper for polishing - Change lever position for desired working direction |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Learn to identify common faults

- Learn to identify Parts of machines

- Learn to perform daily check-ups.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to identify common faults

- Capable to identify Parts of machines

- Capable to perform daily check-ups.

List of Tools and Equipment

1	Tools trolley (Complete set of Hand Tools)

2	Crack Shaft Grinding Machine

3	Grinding Tools

# Fuel System-I

- Recognize INLINE Fuel System

Overview: This Competency Standard identifies the competencies required to Identify and inspect inline fuel Pump Circuit.

| Competency Units | Performance Criteria |
| --- | --- |
| A1. Identify Inline fuel Pump Circuit | You must be able to: - Locate the fuel tank. - Point out fuel priming pump. - Point out Fuel injection pump. - Locate fuel filters, high pressure lines, return line. - Point out Fuel Injection Nozzles. - Locate adopters. - Identify different types of Inline Pump. - Figure out different components of hand priming pump. - Identify different components of feed pump. - Point out internal components of pumping element of Inline Pump from cut away model. - Identify different types of inline pump governors. - Point out different components of inline pump Governor from cut away model. |
| A2. Inspect Inline Fuel Pump Circuit | You must be able to: - Analyze fuel tank - Analyze fuel tank cap. - Dissemble primary pump. - Analyze primary fuel pump as per standards. - Dissemble fuel injection pump. - Analyze Fuel injection pump according to set standards. - Inspect fuel filter, high pressure Lines and return line. - Remove injection nozzles. - Dissemble injection nozzle. - Analyze Injection nozzles. - Inspect adaptors. - Dissemble inline pump governor. - Inspect the governor of Inline Fuel Pump. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe of Inline Fuel system.

- Describe of Priming Pump

- Describe of Feed Pump.

- Describe of Governor.

- Describe of Inline Pump

- Describe of Fuel Injectors/Nozzles. State the micron rating of Fuel Filter

- State the cetane rating of Diesel.

- Harmful substances in Exhaust gases and their effect on environment.

- Sulphur contents in Fuel.

- Fuel internal chemical composition.

- Types of fuels.

- Emission control standards.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to point out/inspect Inline fuel system fuel lines.

- Capable to identify/inspect Inline Fuel Components.

- Capable to identify/inspect priming pump.

- Capable to identify/inspect feed pump.

- Capable to identify/inspect pumping element.

- Capable to identify/inspect governor of inline pump.

List of Tools and Equipment

1	Tools trolley (Complete set of Hand Tools)

2	Fuel Filter

3	Inline pump

4	Inline pump injection nozzle

- Recognize PT (Pressure Time) Fuel System

Overview: This Competency Standard identifies the competencies required to identify and inspect PT (Pressure Time) Pump circuit.

| Competency Units | Performance Criteria |
| --- | --- |
| A1. Identify PT (Pressure Time) Pump circuit | You must be able to: - Identify the Main fuel tank - Point out Float tank - Point out fuel filters and fuel lines. - Locate magnetic strainer. - Point out pulsation damper. - Locate PT pump - Point out return line. - Point out fuel Injectors. - Identify different types of PT Pump. - Point out PT pumps governors. - Point out shut off solenoid. - Identify the internal components of PT Pump from cut away model. - Identify the AFC (Air Fuel Control Unit) Unit. |
| A2. Inspect PT Fuel Pump circuit | You must be able to: - Analyze main fuel tank - Analyze Float tank. - Analyze fuel filter and fuel Lines. - Disassemble PT pump. - Inspect Gear pump as per standards. - Inspect PTG (pressure time gear) Governor. - Analyze MVS (Mechanical variable speed) Governor. - Analyze VS (variable speed) Governor. - Analyze Strainer of PT pump. - Inspect the AFC (Air Fuel Control Unit) Unit - Analyze Fuel Shut OFF valve. - Remove fuel injector. - Dissemble fuel injector. - Inspect Fuel Injectors. - Analyze fuel Return Line |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe of PT Fuel system.

- Describe of PT Pump.

- Describe PTG Governor.

- Describe MVS Governor.

- Describe VS Governor.

- Describe of PT Fuel Injectors.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to point out/inspect PT fuel system fuel lines.

- Capable to identify/inspect PT Fuel pump Components.

- Capable to point /analyze the PT Injectors.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | PT Pump |
| 3 | Fuel filter |
| 4 | PT injector |

- Recognize DPA (Distributor Pump Assembly) Fuel System

Overview: This Competency Standard identifies the competencies required to Identify and inspect DPA (Distributor Pump Assembly) Fuel Circuit.

| Competency Units | Performance Criteria |
| --- | --- |
| A1. Identify of DPA (Distributor Pump Assembly) Fuel Circuit | You must be able to: - Locate the Main fuel tank - Point out fuel supply pump - Locate Fuel Filters - Point out Pressure Regulator Valve and fuel Lines. - Locate DPA fuel pump - Point out Fuel Cut OFF solenoid - Locate Return Line. - Identify the components of DPA Pump from cut away model. - Locate electric circuit for supply pump. - Point out the different components of DPA Pump Governor from cut away model. - Locate the injection nozzles. - Identify different types of injection nozzles. |
| A2. Inspect of DPA (Distributor Pump Assembly) Fuel Circuit | You must be able to: - Analyze the fuel tank. - Analyze Fuel tank cap. - Inspect fuel supply pump connectors. - Inspect fuel supply pump. - Analyze Fuel Filters and Pressure Regulator Valve - Inspect high pressure fuel Lines - Disassemble DPA pump - Inspect DPA pump components. - Inspect Fuel Cut OFF solenoid and connector - Analyze fuel Return Line. - Inspect the components of DPA Pump Governor. - Remove injection nozzle. - Disassemble fuel injection nozzle. - Inspect the injection nozzle. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe of DPA Fuel System.

- Describe Fuel Filter.

- Describe of Fuel Supply Pump.

- Describe of DPA Pump.

- Describe DPA Pump Governor.

- Describe Fuel Cut OFF Solenoid and valve.

- Describe of Fuel Injection Nozzles.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to identify/inspect DPA fuel system.

- Capable to identify/inspect DPA Fuel Pump components.

- Capable to point out/analyze Fuel Supply pump.

- Capable to point out/inspect DPA pump Governor Components.

- Capable to identify/analyze the DPA Injection Nozzles components.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Special Service tools |
| 3 | DPA Pump |
| 4 | Injection nozzles |
| 5 | Fuel filter |

- Recognize CRI (Common Rail Injection) Fuel System

Overview: This Competency Standard identifies the competencies required to Identify and inspect CRI (Common Rail injection) Fuel Circuit.

| Competency Units | Performance Criteria |
| --- | --- |
| A1. Identify of CRI (Common Rail injection) Fuel Circuit | You must be able to: - Locate the fuel tank. - Point out Electromagnetic fuel pump. - Point out main CRI Pump. - Locate feed pump - Locate fuel filters and fuel lines. - Locate common rail and pressure regulate valve. - Point out injection nozzle sensor and electrical circuit. - Locate fuel Injection Nozzles. - Point out ECM (Electronic Control Module) - Point out crank shaft position, cam shaft position, Fuel temperature, Fuel pressure and limit sensor. - Point out the Main components of CRI Pump from cut away model. - Locate SCV (Suction control valve) Sensor. - Locate SCV(Suction control valve) - Locate fuel return line. |
| A2. Inspect of CRI (Common Rail injection) Fuel Circuit | You must be able to: - Analyze the fuel tank. - Inspect fuel tank Cap. - Inspect Electromagnetic fuel pump. - Analyze main CRI Pump. - Analyze feed pump - Inspect fuel filters and fuel lines. - Analyze common rail and pressure regulate valve. - Identify the injection nozzle sensor and electrical circuit. - Inspect Electrical Grips/connectors. - Remove injection nozzle. - Disassemble injection nozzle. - Inspect Injection Nozzles. - Analyze ECM (Electronic Control Module). - Inspect crank shaft position, cam shaft position, Fuel temperature, Fuel pressure and limit sensor. - Disassemble main CRI pump. - Analyze the Main components of CRI Pump - Inspect SCV (Suction control valve) Sensor. - Analyze SCV(Suction control valve) - Analyze return line |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe of CRI Fuel system.

- Describe of Electromagnetic Pump

- Describe of CRI Pump.

- Describe of Common rail.

- Describe of injection nozzle.

- Describe different sensors.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to identify/inspect CRI fuel system fuel lines.

- Capable to identify/inspect CRI Fuel system Components.

- Capable to point out/inspect electromagnetic pump.

- Capable to identify/inspect common rail.

- Capable to identify/inspect solenoids.

- Capable to point out/inspect injection nozzles.

- Capable to identify/inspect sensors.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Special Service tools |
| 3 | Multi meter |
| 4 | Tech-2 and Doctor ZX |
| 5 | CRI Pump |
| 6 | Common Rail |
| 7 | Injection Nozzle |

- Recognize MEUI (mechanical electrical unit injector) fuel system

Overview: This Competency Standard identifies the competencies required to Inspect of MEUI (MECHANICAL ELECTRICAL UNIT INJECTOR) Fuel System.

| Competency Units | Performance Criteria |
| --- | --- |
| A1. Identify of MEUI(MECHANICAL ELECTRICAL UNIT INJECTOR) Circuit | You must be able to: - Point out main fuel tank. - Point out primary fuel pump - Locate Fuel injection pump. - Point out fuel filters and fuel lines. - Locate fuel return line. - Locate regulator valve and Solenoids. - Point out fuel Injectors. - Locate fuel injector connectors. - Identify different types of MEUI Pump. - Identify the components of MEUI Pump from cut away model. |
| A2. Inspect of MEUI (MECHANICAL ELECTRICAL UNIT INJECTOR) Fuel System | You must be able to: - Analyze fuel tank. - Inspect fuel tank cap. - Analyze primary fuel pump. - Analyze Fuel injection pump - Inspect fuel filter and fuel Lines. - Analyze regulator valve and Solenoid. - Remove fuel injector. - Disassemble fuel injector. - Analyze fuel Injectors. - Dissemble MEUI Pump. - Inspect MEUI pump according to set standards. - Inspect connectors of fuel injector. - Inspect the solenoids of fuel Injector. - Inspect the electrical circuit of fuel Injector. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe of MEUI Fuel system.

- Describe of Priming Pump.

- Describe of MEUI Pump.

- Describe of Fuel Injectors.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to identify/analyze MEUI fuel system fuel lines.

- Capable to identify/analyze MEUI Fuel Components.

- Capable to s identify/analyze MEUI pump.

- Capable to identify/analyze the MEUI Injectors.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Special Service tools |
| 3 | MEUI Pump |
| 4 | MEUI Fuel Injector |

- Recognize HEUI (hydraulic electrical unit injector) fuel system

Overview: This Competency Standard identifies the competencies required to Inspect HEUI (mechanical electrical unit injector) fuel system.

| Competency Units | Performance Criteria |
| --- | --- |
| 1. Identify HEUI(hydraulic electrical unit injector) fuel circuit | You must be able to: - Locate the fuel tank - Point out primary fuel pump - Point out HEUI pump. - Point out fuel filters and fuel lines. - Locate regulator valve. - Locate Solenoids and ECM (Electronic control Module). - Point out Injectors. - Identify the Main components of HEUI Pump from cut away model. - Identify the oil Pump. - Identify the oil cooler. - Identify the oil filter. - Identify the components of HEUI Injector from cut away model. |
| A2. Inspect of HEUI (hydraulic electrical unit injector) fuel circuit | You must be able to: - Inspect the fuel tank. - Inspect fuel tank cap. - Disassemble primary fuel tank. - Inspect primary fuel pump. - Disassemble HEUI pump. - Inspect HEUI pump. - Inspect fuel filters and fuel lines. - Inspect regulator valve. - Inspect Solenoids and ECM (Electronic control Module). - Remove Injectors from engine. - Disassemble Injectors. - Inspect Injectors. - Inspect the oil Pump. - Inspect the oil cooler. - Inspect the oil filter. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe of HEUI Fuel system.

- Describe of Priming Pump.

- Describe of HEUI Pump.

- Describe of Lubrication system.

- Describe of Fuel Injectors.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to identify and inspect HEUI fuel system fuel lines.

- Capable to identify and inspect HEUI Fuel Components.

- Capable to identify and inspect priming pump.

- Capable to identify and inspect HEUI pump.

- Capable to identify and inspect the HEUI Injectors.

- Capable to identify and inspect the electrical harness of injector and solenoid.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Special Service tools |
| 3 | Multi meter |
| 4 | ET Tester |

# Generic

- Lead Small Teams

Overview

This unit covers the knowledge, skills and attitudes to lead small teams including setting and maintaining team and individual performance standards.

| ##### Unit of Competency | ##### Performance Criteria |
| --- | --- |
| - Provide team leadership | - Identify the work requirements and present it to team members - Communicate the reasons for instructions and requirements to team members. - Deal with the team members and discuss the queries and concerns. |
| - Assign responsibilities | - Identify the duties, and responsibilities are allocated having regard to the skills, knowledge and aptitude required to properly undertake the assigned task and according to company policy - Identify the duties are allocated having regard to individual preference, domestic and personal considerations, whenever possible |
| - Set performance expectations for team members | - Understand the performance expectations are established based on client needs and according to assignment requirements - Demonstrate the Performance expectations are based on individual team members duties and area of responsibility - Demonstrate Performance expectations are discussed and disseminated to individual team members |
| - Supervised team performance | - Demonstrate the monitoring of performance takes place against defined Performance Criteria and/or assignment instructions and corrective action taken. - Provide a feedback by the Team members, positive support and advice on strategies to overcome any deficiencies - Identify the performance issues which cannot be rectified or addressed within the team are referenced to appropriate personnel according to employer policy - Inform the team members and keep informed of any changes in the priority allocated to assignments or tasks which might impact on client/customer needs and - satisfaction - identify the team operations are monitored to ensure that employer/client needs and requirements are met - Follow-up communication is provided on all issues affecting the team - Identify the all relevant documentation is completed in accordance with company procedures |

##### Knowledge & Understanding

- Work requirements: Client Profile, Assignment instructions.

- Team member’s concerns: Roster/shift details.

- Monitor performance: Formal process, Informal process.

- Feedback: Formal process, Informal process.

- Performance issues: Work output, Work quality, Team participation, Compliance with workplace protocols, Safety, Customer service.

##### Knowledge & Understanding

- Describe the different hydraulic symbol

- Describe the different hydro pneumatic symbol.

- Describe the pneumatic symbol.

- Describe the different electro pneumatic symbols.

##### Critical Evidence(s) Required

- Capable to identify the hydraulic symbol.

- Capable to identify the pneumatic symbol.

- Develop Negotiation Skills

##### Overview

This unit covers the skills, knowledge and attitudes required to collect information in order to negotiate to a desired outcome and participate in the negotiation.

| ##### Unit of Competency | ##### Performance Criteria |
| --- | --- |
| - Plan negotiations | - Identify the Information on preparing for negotiation and included in the plan. - Perform positive negotiating and create Information of nonverbal environments and included in the plan. - Identify the information on active listening is identified and included in the plan - Identify the information on different questioning techniques and included in the plan - Check the Information to ensure it is correct and up-to- date. |
| - Participate in negotiations | - Identify the criteria for successful outcome are agreed upon by all parties - Considered Appropriate language is used on desired outcome of all parties throughout the negotiation. - Use a variety of questioning techniques are used The issues and processes are documented and agreed upon by all parties - Assess possible solutions that are discussed and their viability. - Identify the areas for agreement is confirmed and recorded Follow-up action is agreed upon by all parties. |

##### Knowledge & Understanding

- Preparing for negotiation: Background information on other parties to the

- Negotiation: Good understanding of topic to be negotiated, Clear understanding of desired outcome/s,

- Personal attributes, (self-awareness, self-esteem, objectivity, empathy, respect for others)

- Interpersonal skills, (listening/reflecting, nonverbal communication, assertiveness, behavior labeling, testing understanding, seeking information, self-disclosing

- Analytic skills (observing differences between content and process, identifying bargaining information, applying strategies to manage process, applying steps in negotiating process, strategies to manage conflict, steps in negotiating process, options within organization and externally for resolving conflict.

- Nonverbal environments: Friendly reception, Warm and welcoming room, Refreshments offered, lead in conversation before negotiation begins.

- Active listening: Attentive, Don’t interrupt, Good posture, Maintain eye contact, Reflective listening.

- Questioning techniques: Direct, Indirect, Open-ended

##### Knowledge & Understanding

- Describe the different hydraulic symbol

- Describe the different hydro pneumatic symbol.

- Describe the pneumatic symbol.

- Describe the different electro pneumatic symbols.

##### Critical Evidence(s) Required

- Capable to identify the hydraulic symbol.

- Capable to identify the pneumatic symbol.

- Solve Problems Related to Work Activities

Overview

This unit covers the skills, knowledge and attitudes required to collect information in order to solve problems to a desired outcome and participate in the work activities.

| ##### Unit of Competency | ##### Performance Criteria |
| --- | --- |
| - Identify the problem | - Identify the Variances from normal operating parameters; and product quality Extent, - Identify the cause and nature are of the problem are defined through observation, investigation and analytical techniques - Demonstrate the Problems are clearly stated and specified |
| - Determine fundamental causes of the problem | - Identify the Possible causes that are based on experience and the use of problem solving tools / analytical techniques. - Develop possible cause statements based on findings - Identify the fundamental causes as per results of investigation conducted. |
| - Determine corrective action | - Report on recommendations are prepared - Recommendations are presented to appropriate personnel. - Recommendations are followed-up, if required |

##### Knowledge & Understanding

- Analytical techniques: Brainstorming, Intuitions/Logic, Cause and effect diagrams, Pareto analysis, SWOT analysis, Gant chart, Pert CPM and graphs, Scatter grams.

- Problem: Non – routine process and quality problems, Equipment selection, availability and failure, Teamwork and work allocation problem, Safety and emergency situations and incidents.

- Action plans: Priority requirements, Measurable objectives, Resource requirements,  Timelines, Co-ordination and feedback requirements, Safety requirements, Risk assessment, Environmental requirements

##### Knowledge & Understanding

- Describe the different hydraulic symbol

- Describe the different hydro pneumatic symbol.

- Describe the pneumatic symbol.

- Describe the different electro pneumatic symbols.

##### Critical Evidence(s) Required

- Capable to identify the hydraulic symbol.

- Capable to identify the pneumatic symbol.

## National Certificate Level-4 Part-2 in Construction Machinery

# Fuel System-II

- Maintain inline fuel system

Overview: This Competency Standard identifies the competencies required to troubleshoot inline fuel pump system.

| Competency Units | Performance Criteria |
| --- | --- |
| A1. Troubleshoot Inline Fuel Pump system | - Troubleshoot fuel tank - Troubleshoot Fuel Tank cap. - Replace primary fuel pump. - Replace fuel filter. - Troubleshoot/replace fuel Lines. - Remove Fuel injection pump from engine. - Disassemble Fuel injection pump. - Troubleshoot pumping element components as per requirement. - Troubleshoot governor components as per requirement. - Remove injection nozzles from engine. - Disassemble injection nozzle. - Troubleshoot/replace Injection nozzles. - Assemble injection nozzle. - Troubleshoot the feed pump as per requirement. - Assembling of Inline Pump. |
| A2. Adjust Inline Fuel Pump | - Perform Phasing of Inline Pump. - Perform Calibration of Inline Pump. - Perform Idling Spring Adjustment. - Perform full Load stopper Adjustment. - Mount Inline Pump on Engine. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe of troubleshoot/adjustment of Inline Fuel system.

- Describe of troubleshoot/adjustment Priming Pump

- Describe of troubleshoot/adjustment Feed Pump.

- Describe of troubleshoot/adjustment Governor.

- Describe of troubleshoot/adjustment Inline Pump

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to service Inline fuel system fuel lines.

- Capable to service Inline Fuel Components.

- Capable to service priming pump.

- Capable to service feed pump.

- Capable to service pumping element.

- Capable to service governor of inline pump.

- Capable to adjust the inline pump.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Special Service tools |
| 3 | Inline Pump test Bench |
| 4 | Injector Tester |

- Maintain PT (pressure time) fuel system

Overview: This Competency Standard identifies the competencies required to troubleshoot of PT fuel pump system.

| Competency Units | Performance Criteria |  |
| --- | --- | --- |
| CU1. Troubleshoot of PT Fuel Pump system |  | - Troubleshoot fuel tank. - Troubleshoot/replace Float valve from float tank. - Replace fuel filter. - Trouble shoot/replace fuel Lines - Remove PT pump from engine. - Disassemble PT pump. - Dissemble PTG (Pressure Time Gear) Governor. - Dissemble MVS (Mechanical Variable Speed) governor or VS (Variable speed) Governor. - Troubleshoot/replace Components of PT Pump as per standards. - Disassemble AFC (Air Fuel Control) Unit. - Troubleshoot/replace AFC Unit. - Troubleshoot/replace Fuel cut off solenoid. - Assemble PT Pump. - Remove PT Injectors from Engine. - Disassemble PT Injectors. - Troubleshoot/replace PT injectors. - Assembling of PT Pump. - Troubleshoot/replace Fuel Return Line. |
| CU2. Adjust of PT pump and Injector |  | - Place dial Indicator on injector. - Check the tension of injector spring. - Adjust the height test of injector. - Adjust the components of PT pump. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe disassembling/troubleshoot of PT Fuel system.

- Describe of replace Priming Pump.

- Describe disassembling/troubleshoot of PT Pump.

- Describe of disassembling/troubleshoot of Fuel Injectors.

- Describe Height test of injector.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to service PT fuel system fuel lines.

- Capable to service PT Fuel Components.

- Capable to service priming pump.

- Capable to service PT pump.

- Capable to adjust the PT Injectors.

List of Tools and Equipments

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Special Service tools |
| 3 | PT test bench |
| 4 | PT injector tester |

- Maintain DPA (distributor pump assembly) fuel system

Overview: This Competency Standard identifies the competencies required to troubleshoot DPA (distributor pump assembly) fuel system.

| Competency Units | Performance Criteria |  |
| --- | --- | --- |
| CU1. Troubleshoot DPA (Distributor Pump Assembly) Fuel System |  | - Troubleshoot/replace Fuel Filters. - Troubleshoot/replace Pressure Regulator Valve. - Troubleshoot/replace Fuel Cut OFF solenoid, - Troubleshoot/replace Return Line. - Troubleshoot/replace fuel supply pump. - Disassemble DPA Pump. - Troubleshoot/replace the Main components of DPA Pump as per standards. - Troubleshoot of DPA Pump Governor as per standards. - Remove Injection Nozzle from engine. - Disassemble Injection nozzle. - Troubleshoot/replace the injection nozzles as per standers. |
| CU2. Adjust DPA pump and Injector |  | - Adjust governor components of DPA pump. - Adjust plunger of DPA Pump. - Adjust pressure of Injection Nozzle. - Adjust the spring tension of Injection Nozzle. - Install Injection Nozzles on Engine. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe disassemble/replace of DPA Fuel System.

- Describe disassemble/replace of Fuel Supply Pump.

- Describe disassemble/replace of DPA Pump.

- Describe disassemble/replace of DPA Pump Governor.

- Describe disassemble/replace of Fuel Injection Nozzles.

- Describe the adjustment of injection nozzles.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to service DPA fuel system.

- Capable to service DPA Fuel Pump.

- Capable to service Fuel Supply pump.

- Capable to service DPA pump Governor.

- Capable to service the DPA Injection Nozzles.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Special Service tools |
| 3 | DPA Pump Test Bench |
| 4 | Injector Tester |

- MAINTAIN CRI (Common Rail Injection) Fuel System

Overview: This Competency Standard identifies the competencies required to Troubleshoot CRI (Common Rail injection)Fuel System.

| Competency Units | Performance Criteria |  |
| --- | --- | --- |
| A1. Troubleshoot CRI (Common Rail injection)Fuel System |  | - Troubleshoot the fuel tank. - Troubleshoot/replace fuel Tank cap. - Troubleshoot / replace Electromagnetic fuel pump. - Troubleshoot/replace feed pump. - Troubleshoot/replace fuel filters and fuel lines. - Troubleshoot/replace common rail and pressure regulate valve. - Troubleshoot Injection Nozzles. - Replace ECM (Electronic Control Module). - Replace crank shaft position, cam shaft position. - Replace Fuel temperature, Fuel pressure, Mass air Flow and limit sensor. - Disassemble CRI pump. - Troubleshoot/replace the Main components of CRI Pump as per standers. - Troubleshoot / replace SCV (Suction control valve) Sensor. - Troubleshoot /replace SCV (Suction control valve). - Troubleshoot/replace return line. - Assemble CRI Pump. - Replace injection nozzle sensor. - Disassemble Injection nozzle. - Troubleshoot/Replace injection nozzle. |
| A2. Adjust CRI (Common Rail injection)Fuel System |  | - Enter injection nozzle model number/QR Code in ECM (Electronic control module) by using specific tool. - Adjust/Check the injection timing. - Adjust/Check the fuel pressure. - Adjust/Check the fuel pressure. - Adjust CRI Pressure. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe problems of CRI Fuel system.

- Describe troubleshoot of Electromagnetic Pump

- Describe disassemble/troubleshoot CRI Pump.

- Describe troubleshoot of Common rail.

- Describe disassemble/troubleshoot of injection nozzle.

- Describe different sensors.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to service CRI fuel system fuel lines.

- Capable to service CRI Fuel system Components.

- Capable to service electromagnetic pump.

- Capable to service common rail.

- Capable to service solenoids.

- Capable to service injection nozzles.

- Capable to replace sensors.

List of Tools and Equipment

1	Tools trolley (Complete set of Hand Tools)

2	Special Service tools

3	Multi meter

4	Tech-2 and Doctor ZX

- MAINTAIN MEUI (mechanical electrical unit injector) fuel system

Overview: This Competency Standard identifies the competencies required to troubleshoot MEUI (mechanical electrical unit injector) fuel pump and system

| Competency Units | Performance Criteria |  |
| --- | --- | --- |
| A1. Troubleshoot MEUI (mechanical electrical unit injector) fuel pump and system |  | - Troubleshoot fuel tank. - Troubleshoot/replace Float valve from float tank. - Replace fuel filter. - Trouble shoot/replace fuel Lines. - Troubleshoot/replace Fuel cut off solenoid. - Disassemble MEUI Pump. - Troubleshoot/replace the MEUI pump as per requirement. - Assemble MEUI Pump. - Install MEUI pump on engine. - Remove injector from engine. - Troubleshoot the solenoid of Injector. - Disassemble fuel injector. - Troubleshoot the electrical circuit of Injector. - Troubleshoot/replace Fuel Return Line. |
| A2. Adjust MEUI (mechanical electrical unit injector) fuel pump and system |  | - Place dial Indicator on injector. - Check the tension of injector spring. - Adjust the height test of injector. - Adjust the components of MEUI pump. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe troubleshoot of Fuel system.

- Describe disassembling/troubleshoot of MEUI Pump

- Describe disassembling/troubleshoot of Fuel Injectors.

- Describe height test of fuel injector.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to service MEUI fuel system fuel lines.

- Capable to service MEUI Fuel Components.

- Capable to service priming pump.

- Capable to service MEUI pump.

- Capable to adjust the MEUI Injectors.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Special Service tools |
| 3 | MEUI injector tester |

- Maintain HEUI (hydraulics electrical unit injector) fuel system

Overview: This Competency Standard identifies the competencies required to troubleshoot HEUI (hydraulically electrical unit injector) fuel system.

| Competency Units | Performance Criteria |  |
| --- | --- | --- |
| A1. Troubleshoot HEUI (hydraulically electrical unit injector) fuel system |  | - Troubleshoot fuel tank. - Troubleshoot/replace fuel tank cap. - Troubleshoot/replace primary fuel pump. - Troubleshoot/replace fuel filters and fuel lines. - Troubleshoot/replace IAP (Injection Actuating Pressure) Sensor and Valve. - Troubleshoot/replace the ECM (Electronic control Module). - Troubleshoot the Oil cooler. - Replace the Oil filter. - Disassemble HEUI Pump. - Troubleshoot/replace the HEUI pump as per requirement. - Assemble HEUI Pump. - Replace the solenoid of Injector. - Troubleshoot the electrical circuit of Injector. |
| A2. Adjust HEUI (hydraulically electrical unit injector) fuel system |  | - Check QR code of Injector by using Specific Tool. - Adjust/check the fuel pressure. - Check the fuel temperature. - Adjust oil pump pressure. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe troubleshoot of Fuel system.

- Describe disassembling/troubleshoot of HEUI Pump.

- Describe troubleshoot of Lubrication system.

- Describe disassembling/troubleshoot of Fuel Injectors.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Capable to service HEUI fuel system fuel lines.

- Capable to service HEUI Fuel Components.

- Capable to service HEUI pump.

- Capable to adjust the HEUI Injectors.

- Capable to service the electrical harness of injector and solenoid.

List of Tools and Equipment

| S No. | Descriptions |
| --- | --- |
| 1 | Tools trolley (Complete set of Hand Tools) |
| 2 | Special Service tools |
| 3 | Multi meter |
| 4 | ET Tester |
| 5 | HEUI Injector Tester |

# Applied Thermodynamics

- Analyse Thermodynamic Performance

Overview: This Competency Standard covers the analysis of heating, ventilating, air conditioning and refrigeration (HVAC/R) systems to provide solution to thermodynamic performance issues. It encompasses working safely, apply extensive knowledge of thermodynamic parameters, gathering and analyzing data, applying problem solving techniques, developing and documenting results and solutions for use in design work to provide you the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare to analyze the thermodynamic performance | - Observe safe work practices and personal protective OHS processes and procedures for a given work area are identified, obtained and understood. - Establish OHS risk control measures and procedures are followed in preparation for the work. - Determine extent of the thermodynamic issues from performance specifications, situation reports and in consultations with relevant persons - Plan activities to meet scheduled timelines in consultation with others involved in the work - Form effective strategies to ensure solution development and implementation to carry out efficiently. |
| - Analyze the thermodynamic performance of Heavy Construction Machinery | - Apply thermodynamic principles to analytical solutions on refrigeration and air conditioning systems. - Obtain Parameters, specifications and performance requirements in relation to refrigeration and air conditioning systems in accordance with established procedures. - Carry out approaches to analyzing thermodynamic parameters to provide the most effective solution. - Deal unplanned events with safely and effectively, consistent with regulatory requirements and enterprise policy. - Monitor quality of work against personal performance agreement and/or establish organizational & professional standards |
| - Report and action on the results of thermodynamic performance analysis | - Evaluate to determine their effectiveness of solutions for thermodynamic issues and modify where necessary. - Report the analysis including details of all findings, calculations and assumptions. - Report analysis to appropriately personnel to establish appropriate action to be taken based on findings. - Take actions regarding equipment, documented for inclusion in work/project or development records in accordance with professional standards and manufacturers specifications. |

Knowledge and Understanding

This describes the essential skills and knowledge and their level, required for this unit. Evidence shall show that knowledge has been acquired of safe working practices and analyzing the thermodynamic performance of HVAC/R systems.

K-1.	Understanding the thermodynamic performance issues

K-2.	Forming effective strategies for analyzing refrigeration and air conditioning systems performance

K-3.	Obtaining thermodynamic performance parameters, specifications and performance requirements appropriate to each situation.

K-4.	Evaluating the results of the analysis

K-5.	Documenting analysis details of all findings, calculations and assumptions.

- Solve problems related to fundamentals of thermodynamics

Overview: The user/ individual on the job needs to know and understand how to Solve problems of fundamentals of thermodynamics, heat, mass & weight, force, work done and power. Develop skill, mathematical attitudes and logical perception in the use of mathematical instruments as required in the automobile fields.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1 Preform the conversation of Temperature scales | P1. Measuring the temperature and convert to K to C. P2. Solve the problem with the help formula. P3. Measuring the temperature with appropriate tool. |
| CU-2 Measuring the vacuum and pressure gauge | P1. Measure the vacuum use with vacuum gauge on engine. P2. Measure the pressure use with the pressure gauge in different scales. P3. Perform the conversation of vacuum and pressure in different scales. P4. Solve the problems of mathematical attitudes and logical perception in the use of mathematical instruments |
| CU-3 Understand and Solve the problems of Different Energies | P1. Solve the problems of following energies: - Potential Energy - Kinetic Energy - Thermal Energy - Chemical Energy P2. Drive the law of - Laws of thermodynamics - Law of conservation of energy |
| CU-4 Derive the mathematical relations | P1. Drive the following laws: - Boyles Law - Charles Law - Joules Law P2. Solve the mathematical problems applying the laws |

- Solve problems of Laws and properties of perfect gases

Overview: The user/ individual on the job needs to know and understand how to the knowledge of fundamentals of thermodynamics, laws and properties of gases, thermodynamic processes and cycles, steam and Gas turbines, I.C. Engines.

| Competency Units | Performance Criteria |
| --- | --- |
| CS-1 Applying Gas Laws in problems | P1. Derive the following mathematical relations: - General gas equation - Characteristic Gas equation - Universal Gas equation P2. Understand the specific heats of a gas and derive its mathematical relations. |
| CS-2 Derive the mathematical equation | P1. Derive the mathematical equation of Enthalpy of a Gas. |

Understanding and Knowledge:

- Perfect gas and its property

- Derive the mathematical relations.

- Boyle’s law.

- Charles’s law

- Joule’s law

- Derive the mathematical relations.

- General gas equation

- Characteristic Gas equation

- Universal Gas equation

- Describe the following.

The two specific heats of a gas and derive its mathematical relations.

  - K.5 State the following.

  - Enthalpy of a Gas

- Derive Thermodynamics Processes and Cycles

Overview: The user/ individual on the job needs to know and understand how to prepare the properties of steam, steam boilers and their performance , steam and Gas  turbines , I.C. Engines, Air compressors and their problems.

| Competency Units | Performance Criteria |
| --- | --- |
| CU-1 Draw the different Cycles | P1 Explain CARNOT CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation. P2 OTTO CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation P3. DIESEL CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation. P4. DUAL COMBUSTION CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation. |
| CU-2 Evaluate the performance of steam boilers | P1. Working and general construction of a boiler, Classification of boilers, Selection factors of a good steam boiler, Important terms used for steam boilers. P2. The construction and working of Simple Vertical Boiler with the help of neat sketch. P-3. The construction and working of COCHRAN Boiler (Multi tubular boiler) with the help of neat sketch. P-4 The construction and working of Babcock and Wilcox Boiler with the help of neat sketch. |
| CU-3 Derived and solved the mathematically problems of I.C engines | P1. Solve and Derived following - Torque and its unit in SI system. - Mean effective pressure. - Indicated power and its formula. - Brake Horse power and its formula. - Measurement of Brake Horse power P2. Solve and Derived following - Friction Horse power. - Mechanical efficiency. - Thermal efficiency. |

Understanding and Knowledge:

K.1 Explain CARNOT CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation.

K.2 Explain OTTO CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation.

K.3 Explain DIESEL CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation.

K.4 Explain DUAL COMBUSTION CYCLE with the help of P-V diagram; also derive its mathematical relations for Air Standard Efficiency during the cycle of operation.

K.5 Solve the mathematical expressions of following:

    - Performance of boiler

    - Equivalent evaporation of boiler

    - Efficiency of boiler

K.6 Derived formula of Comparison between Water tube and Fire tube boilers.

# Operate Construction Machinery

- Identify Machines & Its Attachments

Overview: This competency standard covers the skills and knowledge required to identify machines its types and various attachments associated with the machines.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify Machine and its sizes. | - Enlist name of different heavy machines - Enlist crawler machines - Enlist wheeled/tire machines - Check specifications of crawler machines - Check specifications of wheeled/tire machines |
| CU2. Identify components & Attachments | - Enlist different components - Enlist different attachments |
| CU3. Identify capacities& capabilities of Machine | P1. Check capacity as per manufacturers specifications P2. Check capability as per manufacturers specifications P3. Ensure proper capacity of machine P4. Ensure proper capability of machine |
| CU4.Identify basic tools and supplies associated with Machines | P1. Check standard tools supplied with machines P2. Check spares/consumable materials P3. Adopt manufacturers specifications for tools and supplies |
| CU5. Manage Inventory of tools and equipment. | P1. Check number of tools and equipment as per record P2. Report for faulty tools and equipment P3. Generate demand for defective tools and equipment P4. Maintain record of all tools and equipment. |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Define common Heavy Machines & Industry terminologies.

- Describe Common Heavy Machine Capabilities, Advantages & Limitations.

- Describe Heavy Machines Attachments, their Purpose and Capabilities.

- Describe basic Tools, Supplies & Lubricants associated with Heavy Machines.

- Describe Inventory management procedure of Tools & Equipment.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Identify Machines and its related Attachments

- Identify Capacity of machine before use

- Ensure Capability of attachments as per standards

Machines and Its Attachments

A.	Bulldozer Attachments: -

1. Blades.

2. Ripper

B.	Excavator (Wheel & Crawler) Attachments: -

1. 	Buckets.

2. 	Grappler.

3. 	Coupler.

4. 	Thumbs.

5. 	Pulverize.

6. 	Lifting.

7. 	Rakes.

8. 	Chuck

9. 	Blades.

10. Ripper.

11. Forks.

12. Adapter.

13. Hammer.

14. Auger.

15. Compactor.

16. Stump Harvester.

C.	Motor Grader Attachments: -

1. 	Angle Blade.

2. 	Lift Group.

3. 	One way Plow.

4. 	Snow Gate.

5. 	Snow Wing.

6. 	Straight Blade

7. 	UV Angle Blade.

8. 	V-Plow

D.	Wheel Loader Attachments: -

1.	Coupler.

2. 	Dozer Blade.

3. 	Boom Poles.

4. 	Bucket.

5. 	Fork.

6. 	Grappler.

7. 	Snow Blade

8. 	Trailer Hitches.

9. 	Rotary Sweeper.

10. Broadcast Spreader

- Maintain Machines (with Engine Off)

Overview: This competency standard covers the skills and knowledge required to Inspect and service Lubrication, Electrical, Hydraulic, Cooling, Fuel, and braking system of the Heavy Machines.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Inspect and service lubrication system | P1. Locate components to be inspected P2. Identify low oil levels, dirty filler cap P3. Select appropriate tools P4. Adjust oil levels P5. Identify and report leakages |
| CU2. Inspect and service electrical system | P1. Locate components to be inspected P2. Identify service needs, defects and hazardous conditions through visual/physical inspection P3. Select appropriate tools for rectification of minor defects P4. Check water level of batteries P5. Replace batteries. |
| CU3. Inspect and service hydraulic system | P1. Locate components to be inspected P2.Identify service needs, defects and hazardous conditions through visual/physical inspection P3. Identify and report leakages and noise of the hydraulic system P4. Check hydraulic oil levels P5. Replace hoses/pipes |
| CU4. Inspect and service cooling system | P1. Adopt appropriate safety measures. P2. Ensure unobstructed airflow through radiator P3. Locate components to be inspected P4. Adjust coolant level P5. Replace belts and hoses |
| CU5. Inspect and service air intake system | P1. Locate components to be inspected P2. Check air service indicators P3. Select appropriate tools P4. Clean primary air filter. P5. Replace intake hoses and clamps |
| CU6. Inspect and service fuel system | P1. Locate components to be inspected P2. Identify and read fuel gauges and level indicator P3. Select appropriate tools P4. Identify service needs, defects and hazardous conditions through visual/physical inspection P5. Perform basic maintenance such as cleaning of fuel strainer P6. Report fuel leakage |
| CU7. Inspect and service suspension system | P1. Locate components to be inspected P2. Select appropriate tools P3. Check gashes or bulges and tires P4. Grease , bearings, bush and pins P5. Change damaged grease fittings |
| CU8. Inspect and service drive train | P1. Locate components to be inspected P2. Select appropriate tools P3. Identify service needs, defects and hazardous conditions through visual/physical inspection P4. Check wear, leaks and damage to components P5. Identify defective undercarriage components |
| CU9. Inspect and service braking system | P1. Locate components to be inspected P2. Select appropriate tools P3. Identify service needs, defects and hazardous conditions through visual/physical inspection P4. Top-up fluid reservoir P5. Identify defective components of braking system |
| CU10. Inspect and service operator station/Cab | P1. Locate and identify controls inside operator station/Cab P2. Identify missing or defective components or controls P3. Clean front/rear wind screen, windows and mirrors P4. Adjust mirrors P5. Replace broken mirror/frame P6. Adjust seat and seat belt P7. Check nobs of all lights and indicators |
| CU11. Inspect safety equipment | P1. Ensure safety equipment is securely mounted P2. Replace expired fire extinguisher P3. Ensure wearing of PPE |
| CU12. Inspect and service Attachments | P1. Identify service needs, defects and hazardous conditions through visual/physical inspection P2. Select appropriate tools P3. Perform basic maintenance such as greasing, bushing and pins P4. Report worn teeth |
| CU13. Inspect and service supporting pneumatic (Air-filled) system | P1. Locate components to be inspected P2. Select appropriate tools P3. Identify service needs, defects and hazardous conditions through visual/physical inspection P4. Perform basic maintenance, such as choked drain valves P5. Replace air lines |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Describe Functions of Mechanical Systems in Heavy Machines.

- Explain Inspection & Maintenance of Heavy Machines, and their associated Attachments.

- Describe Inspection & Maintenance procedure of Mechanical Systems in Heavy Machines.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Inspect and service braking system.

- Inspect and service hydraulic system.

- Inspect and service lubrication system.

- Inspect and service cooling system.

Tools and Equipment

- Basic tools, such as grease gun,

- Hammer,

- Screwdrivers,

- Pliers,

- Self-locking pliers,

- Adjustable wrench,

- Assorted other wrenches,

- Basic supplies, such as grease, oil, window cleaner, rags, ice scraper, whisk broom.

- Maintain Machines (with Engine Running)

Overview: This competency standard covers the skills and knowledge required to monitor warning systems, engine warm up, cycle equipment functions, requirement of scheduled maintenance and operate Logbook.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Start engine monitor warning systems | P1. Identify leaks and burnt lights P2. Select appropriate tools P3. Replace fuses and tighten loose fittings |
| CU2. Warm up engine | P1. Monitor instrument panel P2. Warm up engine according to manufacturer’s instructions |
| CU3. Troubleshoot basic equipment functions | P1. Activate all functions, such as brakes, steering, lights, wipers and hydraulic functions P2. Identify problems with functions P3. Perform required service |
| CU4. Comply with Scheduled Maintenance Requirements | P1. Comply with safety requirements P2. Read indicators/warning signals and remove the problem P3. Maintain record and documentation. P4. Perform scheduled maintenance |
| CU5. Maintain Logbook | P1.Record fuel consumption P2. Record oil change P3. Record time period/mileage of vehicle for schedule maintenance. |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Describe warning systems of Machines.

- Describe different types of machine warning

- Describe procedure of engine warming up

- Describe cycle equipment functions

- Enlist Techniques of Scheduled Maintenance.

- Describe procedure to maintain Logbook

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Start engine monitor warning systems

- Warm up engine

- Maintain Logbook.

Tools and Equipment

- Utility documentation.

- Logbooks.

- Service Manuals.

- Operational Manuals.

- Perform Parking of Machines

Overview: This competency standard covers the skills and knowledge required to Clean under carriage and attachments before parking, Park equipment in appropriate location, Shut down and secures equipment, Perform housekeeping tasks, and Perform visual inspection.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Clean under carriage and attachments before parking | P1. Clean machine body, wheels, & undercarriage P2. Clean attachments according to manufacturer’s specifications and company policy and procedure |
| CU2. Park equipment in appropriate location | P1. Identify appropriate parking location P2. Park equipment according to company policy and procedure P3. Lower the attachments to the ground level P4. Put paddings under the attachment if the machine is to be parked for longer duration |
| CU3. Shut down and secure equipment | P1. Shut down equipment according to the manufacturers specifications P2. Secure equipment against movement and damage |
| CU4. Perform housekeeping tasks | P1. Clean wind shields, side rails, steps and instrument panel P2. Sweep floor P3. Remove garbage P4. Apply glass/mirror covers |
| CU5. Perform visual inspection | P1. Check parked equipment visually P2. Identify existing or potential problems P3. Communicate to appropriate personnel such as supervisor/mechanic |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Read Manufacturers’ specifications

- Read Company policies and procedures

- Define importance of cleaning tracks, wheels, rollers, and attachments.

- Describe suitable and safe parking locations, such as dry and clean surface, level, away from fuel storage or water courses, secure area.

Critical Evidence(s)

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Park machine at appropriate location.

- Switch off and secure machine.

- Perform housekeeping tasks.

- Perform visual inspection.

Tools and equipment:

•	Bulldozer

•	Excavator

•	Motor Grader

•	Wheel Loader

PPE,s:

•	Coverall

•	Safety shoes

- Perform Transportation of Machines

Overview: This competency standard covers the skills and knowledge required to Prepare to load machine and attachments, Load or assist with loading machine and attachments, Assist with securing machine and attachments. Unload or assist with unloading machine and attachments, Prepare rubber-tired machine for road travel, and Drive rubber tired machine on Public roads

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare to load machine and attachments | P1. Assess hazards, such as ground and utility lines P2. Prepare machines and attachments for transport, such as clean tracks or wheels or disassemble for transport |
| CU2. Load or assist with loading machine and attachments | P1. Avoid hazards, such as uneven ground and utility lines P2. Load or assist with loading of machines and attachments P3. Respond to hand signals |
| CU3. Assist with securing machine and attachments | P1. Protect equipment from damage, such as cover windshield and exhaust pipe P2. Secure attachments, such as bucket P3. Assist transport vehicle driver as required, such as secure machines, attach warning flags and reflectors. |
| CU4. Unload or assist with unloading machine and attachments | P1. Assess and adjust to hazards, such as overhead obstructions, narrow landing areas P2. Unload or assist with unloading machines and attachments. P3. Assist transport vehicle driver as required, such as remove tie-down, warning flags and reflectors P4. Clean equipment. |
| CU5. Prepare rubber-tired machine for road travel | P1. Secure attachments in proper position for road travel. P2. Complete inspection, such as check brakes, steering, lights, tires and back-up warnings. P3. Clean equipment. |
| CU6. Drive rubber tired machine on public roads | P1. Comply with legislation, such as traffic laws. P2. Possess appropriate and valid driver license. P3. Read maps P4. Follow route to destination P5. Adjust to road and weather conditions, such as adjust speed. P6. Recognize and avoid potential hazards. |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Define Loading/unloading techniques

- Estimate carrying capacities of transport vehicles

- Describe Road, weather and deck conditions

- Define Hazards and blocking

- Describe Hand signals.

- Describe Tie-down points.

- Explain limitations on public roads, such as speed and blind spots

- Define applicable legislation, such as traffic laws

- Explain Route and destination

- Define Proper positioning of attachments for road travel

Critical Evidence(s)

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Prepare to load machine and attachments.

- Load or assist with loading machine and attachments.

- Assist with securing machine and attachments.

- Unload or assist with unloading machine and attachments.

- Prepare rubber-tired machine for road travel.

- Drive rubber tired machine on public roads.

Tools & Equipment’s:

- Dump Truck

- Excavator

- Motor Grader

- Wheel Loader

- PPE,s:

- Coverall

- Safety shoes

- Helmet.

- Operate Excavator

Overview: This competency standard covers the skills and knowledge required to Comply with safety requirements, Sets up equipment, Install attachments, Operate controls of Wheel Excavator, Operate controls of Crawler Excavator, Create Slopes, Build, excavate, and maintain haul roads and ramps, Create mass Excavation, Excavate trenches, Excavate ditches  Load Trucks, Cut and fills materials, Stock piles materials, Excavate Back fills trenches, Hoist objects, Clear land, Demolish buildings and other structures, and Ensure performance of equipment

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Comply with safety requirements | P1. Operate safety controls and equipment. P2. Respond to caution, warning and hazard signs, lights and symbols. |
| CU2. Sets up equipment | P1. Adjust to factors affecting safe operation of equipment. P2. Maintain stability of equipment. P3. Position equipment correctly. P4. Communicate with traffic control person. |
| CU3. Install attachments | P1. Select appropriate tools. P2. Position equipment and attachment for installation. P3. Respond to hand signals. P4. Install attachments safely. |
| CU4. Operate control Levers/joystick of Wheel Excavator | P1. Operate control levers/joystick smoothly and safely P2. Operate different operating control levers/joystick simultaneously as required. P3. React to changing conditions/situations. |
| CU5. Operate control Levers/joystick of Crawler Excavator | P1. Operate control levers/joystick smoothly and safely P2. Operate different operating control levers/joystick simultaneously as required. P3. React to changing conditions/situations. |
| CU6. Create Slopes | P1. Interpret specifications of slope. P2. Practice grade checking instruments P3. Fill cuts in the slope with a partial bucket technique. |
| CU7. Build, excavate, and maintain haul roads and ramps | P1. Work around obstructions and hazards. P2. Practice grade checking devices. P3. Protect existing structures and utilities. P4. Build, excavate or maintain haul roads and ramps in accordance with job specifications. |
| CU8. Create mass Excavation. | P1. Adopt laser location for line of sight as excavation progresses. P2. Perform straight edges and stable sides. P3. Dig offset from footing location. P4. Keep the machine level. P5. Level to very fine tolerance. P6. Adopt partial bucket technique. |
| CU9. Excavate trenches | P1. Work around site obstructions and hazards. P2. Maintain equipment in stable position and correct location for job. P3. Practice grade checking devices. P4. Excavate trench in accordance with job specifications. P5. Respond to hand signals. |
| CU10. Excavate ditches | P1. Work around site obstructions and hazards. P2. Maintain equipment in stable position and correct location for job. P3. Practice grade checking devices. P4. Excavate ditches in accordance with job specifications. P5. Respond to hand signals. |
| CU11. Load Trucks | P1. Work around obstructions and hazards. P2. Direct loading vehicle operators. P3. Align according to the position of truck. P4. Load transport vehicles in accordance with job specifications. P5. Respond to hand signals. |
| CU12. Cut and fills materials | P1. Work around site obstructions and hazards. P2. Position equipment correctly. P3. Practice grade checking devices. P4. Cut and fill material in accordance with job specifications P5. Tamp the filled material P6. Respond to hand signals. |
| CU13. Stock piles materials | P1. Work around site obstructions and hazards. P2. Stockpile material in accordance with jobs specifications. |
| CU14. Excavate and Back fills trenches | P1. Work around site obstructions and hazards. P2. Ensure that structures or utility lines are not damaged during backfilling. P3. Maintain stability of equipment P4. Level or layer the material P5. Practice grade checking devices. P6. Backfill trenches/excavations in accordance with job specifications. P7. Respond to hand signals. |
| CU15. Hoist objects | P1. Inspect rigging (ropes) components visually. P2. Identify and discard worn or damaged rigging components. P3. Communicate with appropriate personnel to replace worn or damaged components. P4. Work around obstructions and hazards. P5. Set up equipment in stable position and correct location for jobs. P6. Hoist materials in accordance with manufacturer’s specifications, job specifications and legislation. P7. Respond to hand signals. |
| CU16. Clear land | P1. Work around obstructions and hazards. P2. Install attachments. P3. Maintain haul roads as required. P4. Clear land in accordance with job specifications. |
| CU17. Demolish buildings and other structures | P1. Work around obstructions and hazards. P2. Position equipment safely while demolition and always have means of exist. P3. Demolish structures and remove demolished materials in accordance with job specifications. P4. Respond to hand signals. |
| CU18. Monitor performance of machines | P1. Interpret information from gauges and symbols. P2. Monitor performance using own senses. P3. Identify equipment problems. |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Describe types and sizes of Machines.

- Describe components and functions.

- Explain capacities & capabilities.

- Describes attachments and purposes.

- Define basic tools and supplies.

- Explain safety equipment.

- Describe daily maintenance work

- Define Machine emergency shut-down procedure.

- Describe basic operating functions.

- Explain Pre-start checks, startup and shutdown procedures.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Install attachments

- Operate controls of Wheel Excavator

- Operate controls of Crawler Excavator

- Create Slopes

- Create mass Excavation.

- Excavate trenches

- Excavate ditches

- Load Trucks

- Cut and fills materials

- Stock piles materials

- Excavate and back fill trenches

- Hoist objects

- Clear land

- Demolish buildings and other structures.

Tools & Equipment’s:

- Excavator

- PPE,s:

- Coverall

- Safety shoes

- Conduct dump truck operations

Overview: This competency standard covers the skills and knowledge required to plan and prepare, Conduct machine pre-operational checks, Operate truck, Load, transport and tip materials, Carry out driver maintenance and Clean up.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Plan and prepare | - Obtain, confirm and apply work instructions, including plans, specifications, quality requirements and operational details to the allotted task - Obtain, confirm and apply work safety requirements from the site safety plan and organisational policies and procedures to the allotted task - Identify, obtain and implement signage requirements from the project traffic management plan - Select vehicle, tools and equipment to carry out tasks consistent with the requirements of the job, check for serviceability and rectify or report any faults - Identify, confirm and apply environmental protection requirements from the project environmental management plan to the allotted task |
| CU2. Conduct machine pre-operational checks | - Carry out Pre-start, start up, park and shut down procedures in accordance with manufacturers' and/or site specific requirements - Check dump truck controls and functions, including tray, articulation, brakes and manoeuvrability for serviceability and rectify or report any faults |
| CU3. Operate truck | - Identify site hazards associated with dump truck operations and use safe operating techniques to minimise risk - Manage engine power to ensure efficiency of truck movements and to minimise damage to the engine and gears - Coordinate engine power with gear selection ensuring smooth transition and operation within torque range - Operate dump truck to work instructions under varied site and weather conditions in accordance with safe work practices and operating procedures - Monitor road/traffic conditions constantly taking into account of road standards, traffic flow, distance and load, ensuring no injury to people or damage to property, equipment, loads and facilities - Bring vehicle to a halt without injury to personnel or damage to property, equipment and loads, through the use of engine retarder, gears and brakes using straight line braking techniques - Assume responsibility for self-direction to achieve finished product to job/design specification |
| CU4. Load, transport and tip materials | - Position vehicle at load and discharge points with a minimum of manoeuvre - Control dump truck movements including the raising and lowering of the tray smoothly - Assess weight and distribution of load for type of material and size of vehicle to ensure it is within vehicle capacity - Maintain safety and security of load, including load cover requirements, from loading site to discharge site - Discharge load on slope and/or over face at fill site in accordance with standard procedures - Dump/spread material evenly in accordance with standard procedures - Clear, lower and secure tray before resuming travel in accordance with manufacturers' instructions |
| CU5. Carry out driver maintenance | - Park dump truck safely, prepare for maintenance and shut down in accordance with manufacturers' manual and organisational requirements - Conduct inspection and fault finding in accordance with manufacturers' specifications and/or organisational requirements - Remove and replace defective parts safely and effectively according to manufacturers' manual and organisational requirements - Carry out regular programmed maintenance tasks in accordance with manufacturers' and/or organisational requirements |
| CU6.Clean up | - Clear work area and dispose of or recycle materials in accordance with project environmental management plan - Clean, check maintain and store vehicle, tools and equipment in accordance with manufacturers' recommendations and standard work practices |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Dump truck types, characteristics, technical capabilities and limitations

- Basic principles of soil technology for civil works

- Site and equipment safety requirements

- Dump truck operational techniques related to essential tasks

- Processes for interpreting engineering drawings and sketches

- Operational, maintenance and basic diagnostic procedures

- Site isolation and traffic control responsibilities and authorities

- Materials Safety Data Sheets and materials handling methods

- Project quality requirements

- Civil construction terminology

- Methods of changing machine attachments

- Safe operating techniques in all terrain

- Basic earthworks calculations

- Civil construction activity sequences of road construction, earthworks and drainage

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Tools & Equipment’s:

- Dump Truck

- PPE,s:

- Coverall

- Safety shoes

- Helmet

- Operate Bulldozer

Overview: This competency standard covers the skills and knowledge required to Operate Control levers/joystick, Strip and stockpiles surface materials, Cut and fill material, Create slopes, Create ditches, Spread ballast, Rip dense materials, Clear land, and Push scraper

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Operate Controls Levers/joystick | P1. Operate control levers/joystick smoothly and safely P2. Operate different operating control levers/joystick simultaneously as required P3. React to changing conditions/situations |
| CU2. Strip and stockpile surface materials | P1. Distinguish waste layer from structural layer P2. Remove waste layer P3. Move full blade load with optimum capacity P4. Clean up windrows and any remaining waste material P5. Stockpile waste materials. |
| CU3. Cut and fill material | P1. Estimate the height of cuts and fills P2. Observe grade by coordination with surveyors P3. Cut “humps” and create enough loose material to fill blade before pushing to haulage distance P4. Push material to fill depressions P5. Match blade load with available power and traction P6. Perform rough leveling of ground P7. Eliminate windrows and clean up. |
| CU4. Create slopes | P1. Interpret stakes/specifications P2. Apply grade checking instruments P3. Cut the slope next to each row of stakes P4. Perform heavy cuts down hill P5. Match blade load with available power and traction P6. Apply safe practices regarding stability issues P7. Grade area to a given slope and eliminate windrows and clean up. |
| CU5. Create ditches | P1. Identify the required profile using grade checking instruments P2. Create ditch of specified dimensions P3. Stockpile or blend in material P4. Level the ground roughly, eliminate windrows and clean up. |
| CU6. Spread ballast | P1. Identify dumping location and pattern P2. Match blade load with available power and traction P3. Spread material P4. Grade to requisite level |
| CU7. Rip dense materials | P1. Rip hard strata P2. Balance ripper load depth & load to available power and traction |
| CU8. Clear land | P1. Work around obstructions and hazards P2. Clear land in accordance with job specifications. |
| CU9. Push scraper | P1. Balance engine power to load and traction P2. Minimize wear & tear impact, track spinning P3. Assess grade and level. P4. Remove obstacles and rocks. |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Describe types and sizes of Machines.

- Describe components and functions of Machines.

- Define capacities & capabilities of Machines.

- Describe attachments and its purposes.

- Describe basic tools and supplies.

- Describe Safety equipment.

- Describe daily maintenance work

- Describe Machine emergency shut-down procedure.

- Define basic operating functions.

- Explain Pre-start checks, Startup/Shutdown Procedures.

Critical Evidence(s)

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Operate Controls.

- Strip and stockpile surface materials.

- Cut and fill material.

- Create slopes.

- Create ditches.

- Spread ballast.

- Rip dense materials.

- Clear land.

- Push scraper.

Tools and Equipment

- Color-code cards,

- Utility documentation.

- Logbooks.

- Service Manuals,

- O. H. S Regulation,

- Operate Wheel Loader

Overview: This competency standard covers the skills and knowledge required to Operate Controls, Dig, Carry (tram) & Stockpile Materials. Place and Spread materials, Backfill trenches & Excavate, and Load Trucks.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Operate Control Levers/joystick | P1. Operate control levers/joystick smoothly and safely P2. Operate different operating control levers/joystick simultaneously as required P3. React changing conditions/situations |
| CU2. Dig, Carry (tram) & Stockpile Materials | P1. Fill bucket in loose material P2. Carry loose material to a short distance P3. Place material in a stockpile P4. Maintain smooth pit floor/running surface |
| CU3. Place and Spread materials | P1. Load bucket quickly and fully in loose material P2. Carry loose material to a short distance P3. Spread material P4. Maintain smooth pit floor/running surface |
| CU4. Backfill trenches & Excavate | P1. Place backfill material P2. Manage piles of imported aggregates to minimize waste P3. Spread materials at work area P4. Excavate soft soil strata |
| CU5. Load Trucks | P1. Arrange the loading site P2. Maintain the pit floor, level, smooth and clear of obstructions P3. Load smoothly and gently P4. Communicate with signaler P5. Load truck as per capacity |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Describe types and sizes of Machines.

- Describe components and their functions.

- Describe capacities & capabilities.

- Describe attachments and purposes.

- Describes basic tools and supplies.

- Describe Safety equipment.

- Explain daily maintenance work

- Describe Machine emergency shut-down procedure.

- Describe basic operating functions.

- Define Pre-start checks, startup/shutdown procedures.

Critical Evidence(s)

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Install attachments.

- Operate controls.

- Dig, carry (tram) & stockpile materials.

- Lift and place, spread, & grade materials.

- Backfill trenches & excavate.

- Load trucks.

Tools & Equipment’s:

- Wheel Loader

- PPE,s:

- Coverall

- Safety shoes

- Helmet

- Operate Road Roller

Overview: This competency standard covers the skills and knowledge required to plan and prepare roller/compactor operations, operate roller/compactor, select, relocate the roller, carryout operator maintenance and conduct housekeeping activities.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Plan for roller/compactor operations | - Access, interpret and apply mobile plant and equipment documentation and ensure the work activity is compliant - Obtain, read, interpret, clarify and confirm work requirements |
| CU2. Prepare for roller/compactor operations | - Select and wear personal protective equipment appropriate for work activities - Identify and address potential risks, hazards and environmental issues and implement control measures - Identify, obtain and implement traffic management signage requirements - Select, and check for faults, equipment and/or attachments and ensure ready for work activities - Obtain and interpret emergency procedures, and be prepared for fire/accident/emergency |
| CU3. Operate roller/compactor | - Carry out start-up, park, shutdown and secure equipment procedures - Coordinate activities with others at the site prior to commencement of, during and on completion of the work activity - Drive and operate equipment within recommended speed, engine capability and limitations - Engage and disengage the compacting device - Assess materials and site conditions, and apply appropriate operating techniques - Identify, remove or manage contaminants - Compact to required degree of compaction - Monitor and manage equipment performance using appropriate indicators, and ensure efficiency of operations - Continually monitor hazards and risks, and ensure safety of self, other personnel, plant and equipment |
| CU4.Relocate the roller | - Prepare the roller for relocation - Move the machine between worksites, observing relevant codes and traffic management requirements - Load and unload machine from float/trailer |
| CU5. Carry out operator maintenance | - Prepare machine for maintenance - Conduct inspection and fault finding - Carry out scheduled maintenance tasks - Return machine to service - Process written maintenance records |
| CU6. Conduct housekeeping activities | - Clear work area and dispose of or recycle materials - Manage/report hazards, and maintain a safe working environment - Process records |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Describe types and sizes of Machines.

- Explain types of soils w.r.t. cohesion

- Explain selection of type of rollers for specific soil conditions

- Describe components and functions.

- Define capacities & capabilities.

- Describe attachments and its purposes.

- Describe basic tools and supplies.

- Describe safety equipment.

- Explain daily maintenance work

- Describe Machine emergency shut-down procedure.

- Define basic operating functions.

- Explain Pre-start checks, start-up and shutdown procedures

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Conducting prestart checks prior to commencing operations and shutdown procedures on completion of operations

- Carrying out vehicle refuelling requirements and procedures

- Applying appropriate lifting techniques to remove items from path of plant

- Driving and operating equipment under variable ground conditions and grades

- Engaging and disengaging the compacting device

- Identifying, removing and managing contaminants

- Applying safe work practices and identifying and reporting all potential hazards, risks and environmental issues

- Applying problem solving and troubleshooting techniques

- Manoeuvring the roller compactor safely and smoothly

- Adjusting driving and operations techniques to suit material type and ground conditions

- Completing a minimum of 500 metres roller/compactor operations of tailings and haul road

- Achieving the required degree of compaction of the surface

Tools & Equipment’s:

- Road Roller

- PPE,s:

- Coverall

- Safety shoes

- Helmet

- Operate a Wheel-mounted loading crane

Overview: This competency standard covers the skills and knowledge required to Position and stabilise crane, Operate vehicle-mounted crane, Monitor lift conditions, Pack up crane and Conduct housekeeping activities.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Position and stabilise crane | - Drive crane to position as per job plan to ensure safe operation in accordance with applicable standards, codes of practice, manufacturers specifications and, where applicable, regulatory and local government requirements - Use appropriate, barriers, fencing, temporary boundaries, signage and the like to isolate working area in accordance with safe working practice and lift requirements - Check ground to ensure it is firm enough to bear the load - Use appropriate plates or packing correctly to adequately distribute the load - Deploy and position any outriggers and stabilizers correctly in accordance with manufacturer’s instructions and the appropriate standard and other relevant statutory regulations or local authority requirements - Check outrigger packing for adequacy prior to and after load is taken |
| CU2. Operate vehicle-mounted crane | - Implement planned hazard control strategies - Give, interpret and follow required signals are correctly in accordance with appropriate standards - Assess load mass and correlate with lifting capacity of crane throughout proposed radii of operation - Select appropriate lifting gear and load securely - Hoist and lower load into position using crane movements in accordance with the appropriate standard - Operate crane controls smoothly - Shut down and secure crane during periods of non-operation according to manufacturers specifications and workplace procedures |
| CU3. Monitor lift conditions | - Monitor load constantly to ensure load and structural stability - Identify and monitor conditions which may affect the continuing stability of the crane - Respond to unplanned situations in line with workplace procedures in a manner that minimises risk to personnel and equipment - Seek advice from supervisor where there is doubt about correct response to unanticipated conditions, or conflict with customer request - Advise supervisor/allocator of any concern about completing the job within timeframe - Implement shut-down procedures in accordance with manufacturer’s instructions in the event of an emergency - Apply relevant motion locks and brakes - Shut down crane using the correct sequence of procedures in accordance with manufacturers specifications and workplace procedures - Carryout routine post-operational equipment checks in accordance with manufacturers specifications |
| CU4. Pack up crane | - Check all lifting equipment and crane components for any signs of deterioration or damage in accordance with the appropriate standard - Segregate damaged or worn equipment is and report to an authorised person for testing/repair/destruction - Stow and secure crane equipment correctly in accordance with manufacturer’s instructions and the appropriate standard - Immobilise and secure crane for travel in accordance with manufacturer’s instructions, workplace guidelines and regulatory requirements |
| CU5. Conduct housekeeping activities | - Clear work area and dispose of or recycle materials - Manage/report hazards, and maintain a safe working environment - Process records |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Relevant road rules, regulations, permit and licence requirements pertaining to mobile crane operation

- Relevant OH&S and environmental procedures and regulations

- Mobile crane applications, capacities, configurations, safety hazards and control mechanisms

- Operational procedures for crane crews

- Prioritising and multi-tasking work

- Company work procedures concerning the setting up and rigging of a mobile crane at a work site

- Problems that may arise when operating a vehicle-mounted loading crane and actions that should be taken to prevent or solve them

- Focus of operation of work systems and equipment

- Application of relevant agreements, codes of practice or other legislative requirements

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Tools & Equipment’s:

- Wheel Mounted Loading Crane

- PPE,s:

- Coverall

- Safety shoes

- Helmet

- Operate Motor Grader

Overview: This competency standard covers the skills and knowledge required to Operate Controls, Apply Grading Fundamentals, Form and handle windrows, Strip surface materials, Cut and fill material, Maintain access roads, Create slopes, Create ditches, Create shouldering, Form sub-grade, Place aggregates to specified elevations (finish grading), and Clear snow .

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Operate Control levers/joystick | P1. Operate control levers/joystick smoothly and safely. P2. Operate different operating control levers/joystick simultaneously as required P3. React to changing conditions/situations. |
| CU2. Apply Grading Fundamentals | P1. Apply wheel lean control P2. Apply frame articulation fundamentals P3. Select gear and engine speed P4. Apply grading tips |
| CU3. Form and handle windrows | P1. Choose gear and engine speed P2. Choose blade position P3. Cut material to form a windrow P4. Move material back over area |
| CU4. Strip surface materials | P1. Distinguish waste layer from structural layer P2. Strip waste materials (usually organic) P3. Finish windrows of stripped material P4. Enter and exit machine |
| CU5. Cut and fill material | P1. Estimate the height of cut and fill P2. Choose blade tilt, angel and position P3. Cut heights P4. Match blade load to available power and traction P5. Move material to low areas P6. Grade area to desired profile |
| CU6. Maintain access roads | P1. Identify drainage structures, culverts and obstacles P2. Adjust windrow to allow traffic to continue P3. Choose blade position, wheel lean, articulation, gear and speed P4. Reshape and recover materials for the road surface P5. Cut shoulders and move material to center or from one side to another |
| CU7. Create slopes | P1. Identify required slope P2. Apply grade checking instruments P3. Choose blade position, wheel lean, articulation, gear and speed P4. Smooth the area at the base of the slope for smooth working platform P5. Start at the top of slope P6. Shape the shoulder accurately |
| CU8. Create ditches | P1. Identify the required profile using grade checking instruments P2. Choose blade position, wheel lean, articulation, gear and speed P3. Shape ditch by repeated passes. |
| CU9. Create shouldering | P1. Choose blade position, wheel lean, articulation, gear and speed P2. Position grader with outer tires on pavement, and inner tires just off pavement on shoulder for left side shoulder P3. Position grader with inner tires on pavement, and outer tires just off pavement on shoulder for right side shoulder P4. Move only enough material to pavement edge to dress the shoulder P5. Roll the windrow back away from the pavement edge P6. Shape the shoulder accurately |
| CU10. Form sub-Grade | P1. Choose blade tilt, angel and position P2. Match blade load to available power and traction P3. Remove unsuitable material P4. Cut and fill load bearing soils to create desired profile P5. Shape for drainage and ditch as required |
| CU11. Place aggregates to specified elevations (finish grading) | P1. Identify the required profile using grade checking instruments P2. Get the correct volume in the efficient placement P3. Position for efficient spreading P4. Get correct volume of aggregates P5. Shift the circle and blade towards the piles P6. Cut out windrows only as large as the machine can handle without tire spinning P7. Angle the blade as appropriate P8. Precise control to achieve elevations and shape to very accurate tolerances |
| CU12. Clear snow and ice | P1. Choose proper attachment, as chains, V-plow, wing plow, skid shoes and wing gates P2. Identify snow type, moisture content, density, weight, depth of snow, underlying surface, weather, visibility, traffic, obstacles and hidden structures P3. Mount chain on tires carefully P4. Drive the machine in higher speed to move snow across and off the blade |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Describe types and sizes of Machines.

- Describe components and functions.

- Define capacities & capabilities.

- Describe attachments and its purposes.

- Describe basic tools and supplies.

- Describe safety equipment.

- Explain daily maintenance work

- Describe Machine emergency shut-down procedure.

- Define basic operating functions.

- Explain Pre-start checks, startup and shutdown procedures

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Operate Controls.

- Strip surface materials.

- Cut and fill material.

- Maintain access roads.

- Create slopes.

- Create ditches.

- Create shouldering.

- Form road base (sub-grade).

- Place aggregates to specified elevations (finish grading).

Tools & Equipment’s:

- Motor Grader

- PPE,s:

- Coverall

- Safety shoes

- Helmet

- Conduct asphalt paver operations

Overview: This competency standard covers the skills and knowledge required to plan and prepare, Set up asphalt paver, Operate asphalt paver, Carry out operator maintenance, Relocate paver and clean up.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Plan and prepare the machine | - Access, interpret and apply compliance documentation relevant to the work activity - Obtain and confirm safety requirements from the site safety plan and organisational policies and procedures, and apply to the allotted task - Identify, obtain and implement signage requirements from the project traffic management plan - Determine material to be laid and handling procedures to be employed according to specifications - Select plant, tools and equipment to carry out asphalt paver tasks consistent with the requirements of the job, check for serviceability and rectify or report any faults - Identify environmental protection requirements from the project environmental management plan, and confirm and apply to the allotted task |
| CU2. Set up asphalt paver | - Carry out start-up, park, and shutdown procedures in accordance with manufacturer's and/or site specific requirements - Adjust, change or inflate tyres using safe handling procedures - Set equipment to correct levels to enable the laying of materials to specifications - Set heating controls for the screed board to specifications - Install feeder bin where required - Check materials spreading controls for correct operation |
| CU3. Operate asphalt paver | - Engage delivery vehicles smoothly without bumping - Maintain appropriate uniform speed during spreading operations - Monitor and maintain asphalt mix according to job specifications - Maintain communication with screed hand to ensure job is progressing satisfactorily and that materials are being spread to specifications - Monitor movement of the plant to ensure safety of asphalt crew |
| CU4.Carry out machine maintenance | - Park paver safely, prepare for maintenance and shutdown as per manufacturer's manual and organisational requirements - Conduct inspection and fault finding - Carry out routine operational servicing and lubrication tasks - Carry out minor maintenance - Record performance of machine constantly to enable timely repair of equipment |
| CU5.Relocate paver | - Prepare paver for relocation - Drive paver safely on highways and construction sites, observing highway code and local safety requirements |
| CU6. Clean up the machine and work area | - Clear work area and dispose of or recycle materials in accordance with project environmental management plan - Clean, check, maintain and store plant, tools and equipment |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

- Site and equipment safety requirements

- Asphalt paving techniques

- Asphalt paving operations

- Asphalt strength and performance characteristics

- Edge and joint treatments

- Equipment types, characteristics, technical capabilities and limitations

- Operational, maintenance and basic diagnostic procedures

- Site isolation and traffic control responsibilities and authorities

- Processes for the calculation of material requirements, mix, application rates, uniformity and travel speed

- Materials safety data sheets and materials handling methods

- Project quality requirements

- Civil construction terminology

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Tools & Equipment’s:

- Asphalt Paver

- PPE,s:

- Coverall

- Safety shoes

- Helmet

# Generic

- Develop Communication Skill

Overview: The user/ individual on the job needs to know and understand how to writing skills, reading skills and oral communications skills (Listening and speaking)

| ##### Unit of Competency | ##### Performance Criteria |
| --- | --- |
| - Writing Skills | The user/ individual on job needs to know and understand how to: - Document instructions, itinerary and task lists - Maintain documents and reports pertaining to security operations, emergency response, personnel and equipment - Document activities and incidents in a chronological order - Write letters/ memos, feedback and reports - Use computers to generate Management Information System (MIS), emails and other reports - Write in English and at least in one vernacular language proficiently |
| - Speaking and Listening Skills | The user/ individual on job needs to know and understand how to: - Read and assimilate organizational procedures, site and security instructions, and correspondence, as applicable - Read security registers, documents, feedback, reports and applications Oral Communication (Listening and Speaking skills)The user/individual on the job needs to know and understand how to: - Speak clearly and emphatically - Receive briefings and instructions from superiors and ask queries - Brief and instruct subordinates and receive debriefings - Reply to the queries from stakeholders - Ask questions from visitors - Raise alarm/ make announcements/ speak over phone/ radio |
| - Professional Skills | Decision Making The user/individual on job needs to know and understand how to: SB1 take decisions pertaining to security operations, training, administration and emergency situations Plan and Organize. The user/individual on the job needs to know and understand how to: - Plan security operations as per site instructions and resources available assess and assign tasks to teams and individuals - Conduct training, rehearsals and mock drills Customer Centricity The user/individual on the job needs to know and understand how to: - Manage relationships with stakeholders and visitors Problem Solving The user/individual on the job needs to know and understand how to: - Think through the problem, evaluate the possible solution(s) and adopt a best possible solution(s) - Handle disruption in security operations Analytical Thinking The user/individual on the job needs to know and understand how to: - Anticipate and identify potential risk and threats and take suitable actions - Observe people, activities and movements keenly Critical Thinking The user/individual on the job needs to know and understand how to: - Analyze and evaluate a situation and available information to take appropriate action |

##### Knowledge & Understanding

- Analytical techniques: Brainstorming, Intuitions/Logic, Cause and effect diagrams, Pareto analysis, SWOT analysis, Gant chart, Pert CPM and graphs, Scatter grams.

- Problem: Non – routine process and quality problems, Equipment selection, availability and failure, Teamwork and work allocation problem, Safety and emergency situations and incidents.

- Action plans: Priority requirements, Measurable objectives, Resource requirements,  Timelines, Co-ordination and feedback requirements, Safety requirements, Risk assessment, Environmental requirements

##### Knowledge & Understanding

- Describe the different hydraulic symbol

- Describe the different hydro pneumatic symbol.

- Describe the pneumatic symbol.

- Describe the different electro pneumatic symbols.

##### Critical Evidence(s) Required

- Capable to identify the hydraulic symbol.

- Capable to identify the pneumatic symbol.

- Capable to

- Develop Administrative Skills

Overview: The user/ individual on the job need to know and understand how to develop administrative skills.

| ##### Unit of Competency | ##### Performance Criteria |
| --- | --- |
| - Decision making | - Perform decisions pertaining to security operations, training and administration - Perform emergency situations |
| - Make Plan and Organize the workshop | The user/individual on job needs to know and understand how to: - Plan security operations as per site instructions and resources available assess and assign tasks to teams and individuals - Conduct training, rehearsals and mock drills - Manage relationships with stakeholders and visitors |
| - Evaluate the possible solution | - Think through the problem, evaluate the possible solution(s) and adopt a best possible solution(s) - Handle disruption in security operations - Anticipate and identify potential risk and threats and take suitable actions - Observe people, activities and movements keenly - Analyze and evaluate a situation and available information to take appropriate action |

##### Knowledge & Understanding

- Analytical techniques: Brainstorming, Intuitions/Logic, Cause and effect diagrams, Pareto analysis, SWOT analysis, Gant chart, Pert CPM and graphs, Scatter grams.

- Problem: Non – routine process and quality problems, Equipment selection, availability and failure, Teamwork and work allocation problem, Safety and emergency situations and incidents.

- Action plans: Priority requirements, Measurable objectives, Resource requirements, Timelines, Co-ordination and feedback requirements, Safety requirements, Risk assessment, Environmental requirements

# List of Tools, Equipment, Machinery and Consumable

## 11.1. Level-4 Part-1 in Construction Machinery

### 11.1.1. Problems in IC Engines

| S# | Tools and Equipment | Quantity | S# | Consumable Material |  | Quantity |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Belt tensioner | 25 Nos |  | Carb cleaner spray | 25 Nos |  |
| 2 | Compression testers, compression gauge) | 25 Nos |  | Cotton rag | 10 kg |  |
| 3 | Digital Multimeter (DMM). | 25 Nos |  | Emery paste | 05 packet |  |
| 4 | Drive belt | 25 Nos |  | Engine coolant | 10 liters |  |
| 5 | Engine | 05 Nos |  | Engine oil | 10 liters |  |
| 6 | Engine analyzer | 01 No |  | fan belt | 25 Nos |  |
| 7 | Engine Simulator | 01 No |  | Fuel (petrol/diesel) | 20 liters each |  |
| 8 | ET Tester | 05 Nos |  | Gasket | 25 Nos |  |
| 9 | Hand tools trolley | 05 Nos |  | Hose pipe | 25 Nos |  |
| 10 | Impact gun | 25 Nos |  | Kerosene oil | 20 liters |  |
| 11 | Lifter | 05 Nos |  | oil seals | 25 Nos |  |
| 12 | Measuring tools (Vernier caliper, micrometer, bore gauge, dial indicator gauge, Steel rule, filler gauge, try square, mercer gauge) | 25 Nos each |  | Sealant/gasket | 25 Nos |  |
| 13 | Oil can | 25 Nos |  | Silicone | 10 packets |  |
| 14 | Radiator leak tester | 25 Nos |  | Thermostat valve | 25 Nos |  |
| 15 | Scraper | 25 Nos |  | Timing belt | 25 Nos |  |
| 16 | Spray gun | 25 Nos |  | WD 40 spray | 20 Nos |  |
| 17 | Tachometer | 25 Nos |  |  |  |  |  |
| 18 | TECH-2 tester | 25 Nos |  |  |  |  |  |
| 19 | Timing gun | 25 Nos |  |  |  |  |  |
| 20 | Tools Box | 25 Nos |  |  |  |  |  |
| 21 | Tools trolley (complete set of tools) | 05 Nos |  |  |  |  |  |
| 22 | Torque wrench | 25 Nos |  |  |  |  |  |
| 23 | Valve lapping tool | 25 Nos |  |  |  |  |  |
| 24 | Vehicle/ simulator with a cooling system | 02 Nos |  |  |  |  |  |

### 11.1.2. Workshop Practice – II

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Belt tensioner | 25 Nos |  | Carb cleaner spray | 25 Nos |
| 2 | Compression Gauge | 05 Nos |  | Cotton rag | 10 kg |
| 3 | Compression testers | 05 Nos |  | Kerosene oil | 20 liters |
| 4 | Connecting Rod Boring Machine | 01 No |  | Boring Tools | 05 Nos |
| 5 | Crack Shaft Grinding Machine | 01 No |  | Cutting Tools | 05 Nos |
| 6 | Dial Indicator | 25 Nos |  | Drive belt | 05 Nos |
| 7 | Digital Multimeter (DMM). | 25 Nos |  | Cutting oil | 20 liters |
| 8 | Engine | 01 No |  |  |  |
| 9 | Engine analyzer | 01 No |  |  |  |
| 10 | Engine Simulator | 01 No |  |  |  |
| 11 | ET Tester | 05 Nos |  |  |  |
| 12 | Four Gas Analyzer | 05 Nos |  |  |  |
| 13 | Grinding Tools | 05 Nos |  |  |  |
| 14 | Handy Smoke Tester | 05 Nos |  |  |  |
| 15 | Head Surface Grinding Machine | 01 No |  |  |  |
|  | Honing Machine | 01 No |  |  |  |
|  | Honing Tool | 05 Nos |  |  |  |
|  | Impact gun | 05 Nos |  |  |  |
|  | Lifter | 05 Nos |  |  |  |
|  | Main Line Boring Machine | 01 No |  |  |  |
|  | Measuring tools (Vernier caliper, micrometer, bore gauge, dial indicator gauge, Steel rule, filler gauge, try square, mercer gauge) | 25 Nos each |  |  |  |
|  | Mercer Gauge | 05 Nos |  |  |  |
|  | Oil can | 05 Nos |  |  |  |
|  | Special Service Tools | 05 Nos |  |  |  |
|  | Tools trolley (Complete set of Hand Tools) | 05 Nos |  |  |  |
|  | Vacuum Gauge | 05 Nos |  |  |  |

### 11.1.3. Fuel System-I

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Common Rail | 05 Nos |  | Carb cleaner spray | 25 Nos |
| 2 | CRI Pump | 05 Nos |  | Cotton rag | 10 kg |
| 3 | DPA Pump | 05 Nos |  | Kerosene oil | 20 liters |
| 4 | ET Tester | 05 Nos |  | Boring Tools | 05 Nos |
| 5 | Fuel filter | 05 Nos |  | Fuel (petrol/diesel) | 20 liters each |
| 6 | Injection nozzles | 05 Nos |  |  |  |
| 7 | Inline pump | 05 Nos |  |  |  |
| 8 | MEUI Fuel Injector | 05 Nos |  |  |  |
| 9 | MEUI Pump | 05 Nos |  |  |  |
| 10 | Multi meter | 05 Nos |  |  |  |
| 11 | PT injector | 05 Nos |  |  |  |
| 12 | PT Pump | 05 Nos |  |  |  |
| 13 | Special Service tools | 05 Nos |  |  |  |
| 14 | Tech-2 and Doctor ZX | 05 Nos |  |  |  |
| 15 | Tools trolley (Complete set of Hand Tools) | 05 Nos |  |  |  |

## 11.2. Level-4 Part-2 in Construction Machinery

## 11.2.1. Fuel System-II

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | DPA Pump Test Bench | 05 Nos |  | Carb cleaner spray | 25 Nos |
| 2 | ET Tester | 05 Nos |  | Cotton rag | 10 kg |
| 3 | HEUI Injector Tester | 05 Nos |  | Kerosene oil | 20 liters |
| 4 | Injector Tester | 05 Nos |  | Fuel (petrol/diesel) | 05 Nos |
| 5 | Inline Pump test Bench | 05 Nos |  |  | 20 liters each |
| 6 | MEUI injector tester | 05 Nos |  |  |  |
| 7 | Multi meter | 05 Nos |  |  |  |
| 8 | PT injector tester | 05 Nos |  |  |  |
| 9 | PT test bench | 05 Nos |  |  |  |
| 10 | Special Service tools | 05 Nos |  |  |  |
| 11 | Tech-2 and Doctor ZX | 05 Nos |  |  |  |
| 12 | Tools trolley (Complete set of Hand Tools) | 05 Nos |  |  |  |

## 11.2.2. Operation of Construction Machinery

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Adjustable wrench, | 25 Nos |  | Carb cleaner spray | 05 Nos |
| 2 | Asphalt Paver | 05 Nos |  | Cotton rag | 10 kg |
| 3 | Assorted wrenches, | 25 Nos |  | Kerosene oil | 20 liters |
| 4 | Grease gun, | 25 Nos |  | Grease, | 05 Nos |
| 5 | Color-code cards, | 25 Nos |  | Fuel (diesel) | 100 liters each |
| 6 | Coverall | 25 Nos |  | window cleaner, | 10 Packet |
| 7 | Hammer, | 25 Nos |  | ice scraper, | 05 Nos |
| 8 | Helmet | 25 Nos |  | whisk broom | 05 Nos |
| 9 | Logbooks. | 25 Nos |  |  |  |
| 10 | O. H. S Regulation, | 25 Nos |  |  |  |
| 11 | Operational Manuals. | 25 Nos |  |  |  |
| 12 | Pliers, | 25 Nos |  |  |  |
| 13 | Safety shoes | 25 Nos |  |  |  |
| 14 | Screwdrivers, | 25 Sets |  |  |  |
| 15 | Self-locking pliers, | 25 Nos |  |  |  |
| 16 | Service Manuals. | 25 Nos |  |  |  |
| 17 | Utility documentation. | 25 Nos |  |  |  |

## 11.2.3. Machines and its Attachments

A. Bulldozer Attachments: -

      - Blades.

      - Ripper

B. Excavator (Wheel & Crawler) Attachments: -

- Buckets.

- Grappler.

- Coupler.

- Thumbs.

- Pulverize.

- Lifting.

- Rakes.

- Chuck

- Blades.

- Ripper.

- Forks.

- Adapter.

- Hammer.

- Auger.

- Compactor.

- Stump Harvester.

C. Motor Grader Attachments: -

- Angle Blade.

- Lift Group.

- One way Plow.

- Snow Gate.

- Snow Wing.

- Straight Blade

- UV Angle Blade.

- V-Plow

D. Wheel Loader Attachments: -

- Coupler.

- Dozer Blade.

- Boom Poles.

- Bucket.

- Fork.

- Grappler.

- Snow Blade

- Trailer Hitches.

- Rotary Sweeper.

- Broadcast Spreader

- Dump Truck

## 10.2.4. Specification of Machines & Consumable

- Bulldozer Specification & Consumable

| S.# | Length (mm) | D50A-17 | D65A-8 | D85-18/D85A | D155A-1 |
| --- | --- | --- | --- | --- | --- |
| 1. | Overall Length | 4765 | 5135 | 5750 | 6880 |
| 2. | Overall Width | 2145 | 3970 | 3725 | 4130 |
| 3. | Overall, Height | 2900 | 3020 | 3395 | 3720 |
| 4. | Overall Op Weight | 12240 | 15890 | 23510 | 33690 |
| 5. | Ground Clearance | 315 | 400 | 400 | 500 |
| 6. | Track Shoes Width | 460 | 460 | 560 | 560 |
| 7. | Grade Ability (degree) | 30 | 30 | 30 | 30 |
| 8. | Ground Pressure (kg/cm2) | 0.62 | 0.67 | 0.62 | 0.77 |
| 9. | Horse Power | 120 | 165 | 220 | 320 |
| 10. | Type of Dozer | Angle | Tilt | Tilt | Tilt |
| 11. | Fuel (LT) | 250 | 320 | 450 | 660 |
| 12. | Engine Oil-SAE 30 (LT) | 30 | 30 | 43 | 71 |
| 13. | Hydraulic Oil (LT) | 87 | 108 | 110 | 164 |
| 14. | Transmission Oil (LT) | 18 | 52 | 122 | 185 |
| 15. | Cooling Water (LT) | 52 | 63 | 79 | 165 |
| 16. | Steering Oil (LT) | 63 | 70 | Nil | Nil |
| 17. | Final Drive Case Oil (LT) | 52 (26 each side) | 62 (31 each side) | 72 (36 each side) | 110 (55 each side) |

- Excavator Specification & Consumable.

| S# | Specification | PC 120 | PC 150 | PC 200 |
| --- | --- | --- | --- | --- |
| 1 | Bucket Capacity (m3) | 0.50 m3 | 0.55 m3 | 0.7 m3 |
| 2 | Operating Weight (Kg) | 12030 kg | 14500 kg | 18000 kg |
| 3 | Overall Length (mm) | 7050 | 8350 | 9380 |
| 4 | Overall Width (mm) | 2500 | 2550 | 2740 |
| 5 | Overall, Height (mm) | 2700 | 2900 | 2940 |
| 6 | Swing Speed (rpm) | 20 | 19.6 | 13 |
| 7 | Travel Speed (Km/h) | 3 km/h | 3.2 km/h | 3.5k m/h |
| 8 | Grade ability (Degree) | 25 % to 30% | 35% | 35% |
| 9 | Ground Pressure (Kg/cm2) | 0.45 Kg/cm2 | 0.47 Kg/cm2 | 0.47 Kg/cm2 |
| 10 | Max. Excavation (mm) | 3060 | 5400 | 6550 |
| 11 | Max. Stockpile (mm) | 4420 mm | 5530 mm | 6255 mm |
| 12 | Max. Stretch (mm) | 7050 mm | 8440 mm | 9850 mm |
| 13 | Horsepower (HP) | 85.4 HP | 86 HP | 106 HP |
| 14 | Fuel Capacity (LT) | 230 LT | 280 LT | 540 LT |
| 15 | Engine Oil (LT) | 11 LT | 24 LT | 24 LT |
| 16 | Hydraulic Oil (LT) | 100 LT | 250 LT | 250 LT |
| 17 | Swing case Oil (LT) | 2.5 LT | 7 LT | 8 LT |
| 18 | Water (Lt) | 15.7 LT | 24 LT | 24 LT |
| 19 | Track Chain Pulley | 20 to 25 mm | 20 to 25 mm | 60 to 100 mm |
| 20 | Final Drive | Each side | Each side | Each side |
|  |  | 2.5 LTR | 2.5 LTR | - LTR |

- Motor Grader Specification & Consumable.

| S.# | Detail | MG 200 | MG 330 | MG 430 | GD-605-A3 |
| --- | --- | --- | --- | --- | --- |
| 1. | Heaped Blade Capacity | 3.06 m3 | 3.9 m3 | 1.01 m3 | 3.9 m3 |
| 2. | HP (Horse Power) | 115 hp | 135 hp | 155 hp | 145 hp |
| 3. | Op/Weight | 9885 kg | 10920 kg | 12220 kg | 12870 kg |
| 4. | Fuel | 210 ltr | 230 ltr | 275 ltr | 250 ltr |
| 5. | Engine Oil | 12 ltr | 13 ltr | 13 ltr | 24 ltr |
| 6. | Hydraulic Oil | 70 ltr | 67 ltr | 67 ltr | 60 ltr R/Fel |
| 7. | Transmission | 40 ltr | 23 ltr | 32 ltr | 30 ltr |
| 8. | Water | 21 ltr | 34 ltr | 46 ltr | 45 ltr |
| 9. | Tire Pressure | 2.25 | 1.8 | 2.6 | 2.45kg |
| 10. | Gear Oil/Final Drive | 2.5 ltr | 3.4 ltr | 3.5 ltr | 26 CTR |
| 11. | Tandem Oil | 48 ltr | 73 ltr | 85 ltr | - tr 36X36 |

- Wheel Loader Specification & Consumable

| S# | Items | WA 450 | WA 320 | WA 200 | 966 F cat | 928 F cat |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Horsepower (HP) | 237 | 165 | 110 | 220 | 120 |
| 2 | Operating Weight (Kg) | 19100 | 13450 | 92100 | 20905 | 11148 |
| 3 | Bucket Capacity (m3) | 3.5 | 2.8 | 1.7 | 3.8 | 2 |
| 4 | Grade ability (Degree) | 30 | 35 | 33 | 35 | 35 |
| 5 | Speed/Hour | 34-38 | 38 | 37 | 48 | 45 |
| 6 | Fuel (LT) | 330 | 228 | 170 | 304 | 189 |
| 7 | Engine Oil (LT) | 32 | 19.5 | 24 | 28 | 20 |
| 8 | Hydraulic Oil (LT) | 230 | 89 | 83 | 205 | 100 |
| 9 | Transmission Oil (LT) | 61 | 74 | 35 | 59 | 30 |
| 10 | Cooling Water (LT) | 65 | 20 | 38 | 48 | 41 |
| 11 | F/R Axle oil (LT) | 120 | 48 | 34 | 47 | 50 |
| 12 | Tire Pressure (Kg/cm2) | 2.8 | 2.5 | 2.5 | 2.8 | 2.5 |